3GPP TSG-CT WG1 Meeting #131-e	C1-214596

- E-meeting, 19-27 August 2021

Source:	Qualcomm Incorporated

Title:	5G_ProSe update on UE-to-network relay configuration

Spec:	3GPP TS 24.554

Agenda item:	17.2.18

Document for:	Agreement

1. Introduction

Currently stage-2 requirement for 5G ProSe Layer-3 UE-to-Network Relay with N3IWF support is not spcecified in TS 24.554.

2. Reason for Change

For RSC for N3IWF support, according to TS 23.304, under Figure 6.5.1.2.1-1

2-4.   A 5G ProSe Layer-3 UE-to-Network Relay and 5G ProSe Remote UE follow the procedures described in steps 3-5 in clause 6.5.1.1 using the RSC configured for making the Remote UE access to 5GC via N3IWF.

In addition, in 6.5.4 of TS 23.304

If configured in the ProSe Policy, the 5G ProSe Remote UE may attempt the discovery of a Relay Service Code corresponds to 5G ProSe Layer-3 UE-to-Network Relay with N3IWF support in the discovery procedure.

The remote UE selects RSC for U2N relay discovery based on the URSP and ProSe policy. In order to make the remote UE to select the RSC for U2N relay with N3IWF support, the ProSe policy for the remote UE needs to indicate which RSC is for the U2N relay with N3IWF support.

In addition, the remote UE needs to be configured with following information to support N3IWF selection:

TS 23.304, section 5.6.1.2.2

To support the N3IWF selection, a 5G ProSe Remote UE is configured by HPLMN with following information:

1.	N3IWF identifier configuration (either FQDN or IP address) in the HPLMN.

2.	5G ProSe Layer-3 UE-to-Network Relay access node selection information - a prioritized list of PLMNs for N3IWF selection. It also indicates if selection of an N3IWF in a PLMN should be based on Tracking Area Identity FQDN or on Operator Identifier FQDN.

(…)

N3IWF identifier configuration and the 5G ProSe Layer-3 UE-to-Network Relay access node selection information are provided to the 5G ProSe Remote UE in the ProSe Policy.

Therefore, the configuration information to the remote UE needs to include N3IWF identifier configuration and U2N relay node selection information.

3. Proposal

It is proposed to agree the following changes to 3GPP TS <TS 24.554>

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS); Stage 2".

[3]	IETF RFC 7230: "Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[4]	IETF RFC 7231: "Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content".

[5]	3GPP TS 24.526: "UE policies for 5G System (5GS); Stage 3".

[6]	OMA-WAP-TS-PushOTA-V2_1-20110405-A: "Push Over the Air".

[7]	OMA-AD-Push-V2_2-20110809-A: "Push Architecture".

[8]	WAP-168-ServiceLoad-20010731-a: "Service Loading".

[9]	3GPP TS 29.555: "Inter-5G Direct Discovery Name Management Function (DDNMF) signalling aspects; Stage 3".

[10]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[11]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[12]	3GPP TS 23.003: "Numbering, addressing and identification".

[13]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[14]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[15]	3GPP TS 38.304: "User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[16]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[17]	3GPP TS 24.555: "Proximity-services (ProSe) in 5G System (5GS); User Equipment (UE) policies; Stage 3".

[18]	3GPP TS 24.587: "Vehicle-to-Everything (V2X) services in 5G System (5GS); Protocol aspects; Stage 3".

[19]	3GPP TS 29.343: "Proximity-services (ProSe) function to ProSe application server aspects (PC2); Stage 3".

[20]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[21]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[r24502]	3GPP TS 24.502: "Access to the 5G System (5GS) via non-3GPP access networks; Stage 3".

* * * Second Change * * * *

### 5.2.5	Configuration parameters for 5G ProSe UE-to-network Relay

The configuration parameters for the role of a ProSe UE-to-network relay over PC5 reference point consist of:

a)	a validity timer for the validity of the configuration parameter for 5G ProSe UE-to-network Relay over PC5 interface;

b)	a list of PLMNs in which the UE is authorised to relay traffic for Remote UEs when the UE is served by NG-RAN, and in each PLMN;

1)	an indication of whether the UE is authorized to be a 5G ProSe UE-to-Network Relay;

Editor's note:	It is FFS whether the indication distinguishes L2 or L3 for U2N Relay, i.e., a separate indication for L2 or L3 U2N Relay.

c)	a User Info ID for the UE-to-Network Relay discovery;

d)	one or more Relay Service Code(s) for the UE-to-Network Relay discovery, and for each Relay Service Code:

1)	security related content for ProSe Relay Discovery; and

2)	for Layer 3 ProSe UE-to-Network Relay, a set of PDU session parameters:

i)	PDU Session type;

ii)	DNN;

iii)	SSC Mode;

iv)	S-NSSAI; or

v)	Access Type Preference.

Editor's note:	Details of security related content are FFS and will be determinated by SA WG3.

e)	For Layer 3 ProSe UE-to-Network Relay, mapping rules between a 5QI value and a 5G ProSe PQI value over PC5 for traffic relayed over the PC5 interface.

f)	the radio parameters of the 5G ProSe Relay Discovery applicable per geographical area with an indication of whether these radio parameters are "operator managed" or "non-operator managed" when the UE is not served by NG-RAN; and

g)	the radio parameters of the 5G ProSe direct communication applicable per geographical area with an indication of whether these radio parameters are "operator managed" or "non-operator managed" when the UE is not served by NG-RAN.

The configuration parameters for the role of a 5G ProSe Remote UE consist of:

a)	a validity timer for the validity of the configuration parameters for 5G ProSe Remote UE;

b)	a list of PLMNs in which the UE is authorized to use a ProSe UE-to-Network Relay:

1)	an indication of whether the UE is authorized to be a 5G ProSe Remote UE;

Editor's note:	It is FFS whether the indication distinguishes L2 or L3 for U2N Relay, i.e., a separate indication for L2 or L3 Remote UE.

c)	a User Info ID for the UE-to-Network Relay discovery;

d)	one or more Relay Service Code(s) for the UE-to-Network Relay discovery, and for each Relay Service Code:

1)	security related content for ProSe Relay Discovery; and

2)	for Layer 3 ProSe UE-to-Network Relay, a set of PDU session parameters:

i)	PDU Session type;

ii)	DNN;

iii)	SSC Mode;

iv)	S-NSSAI; or

v)	Access Type Preference.

3)	for communication with 5G ProSe Layer 3 UE-to-Network Relays, an indication that the Relay Service Code is corresponding to 5G ProSe Layer 3 UE-to-Network Relay with N3IWF support.

Editor's note:	A need for explicit indication for Relay Service Code corresponding to 5G ProSe Layer 3 UE-to-Network Relay with N3IWF will be aligned once stage-2 requirement is clarified.

Editor's note:	Details of security related content are FFS and will be determinated by SA WG3.

e)	the radio parameters of the 5G ProSe Relay Discovery applicable per geographical area with an indication of whether these radio parameters are "operator managed" or "non-operator managed" when the UE is not served by NG-RAN;

f)	the radio parameters of the 5G ProSe direct communication applicable per geographical area with an indication of whether these radio parameters are "operator managed" or "non-operator managed" when the UE is not served by NG-RAN;

NOTE:	Whether a frequency band is "operator managed" or "non-operator managed" in a given Geographical Area is defined by local regulations.

g)	for communication with 5G ProSe Layer 3 UE-to-Network Relays, N3IWF identifier configuration (either FQDN or IP address) in the HPLMN; and

h)	for communication with 5G ProSe Layer 3 UE-to-Network Relays, access node selection information consists of:

- 	a prioritized list of PLMNs for N3IWF selection and an indication that the selection of an N3IWF in a PLMN should be based on Tracking Area Identity FQDN or on Operator Identifier FQDN as specified in clause 7.2.4 of TS 24.502 [r24502].

* * * End of Change * * * *