3GPP TSG-CT WG1 Meeting #131-e	C1-21aabb

- E-meeting, 19-27 August 2021							was C1-214304

Source:	MediaTek Inc.

Title:	IoT NTN for EPS

Agenda item:	17.1.2

Document for:	Discussion and agreement

# 1 	Introduction

RAN#92e approved a new Rel-17 WI to introduce minimum essential functionality to support IoT NTN into EPS only ([1] RP-211601), acknowledging the urgent market need identified in [2] ([3]) for such technology.

SA2 has previously responded to RAN that they assume the impact on aligning SA2 specs with RAN will be small and that if the need for more substantial impacts were found, a new WI would be defined.

This contribution discusses the related work in CT1 required to support IoT NTN for EPS in Rel-17.

# 2	Discussion

## 2.1	General

In view of the approved WI by TSG RAN [1] (of which the objectives can be found below in appendix) it is proposed that CT1 (and other CT working groups), follow RAN approach and focus on minimum essential functionality to support IoT NTN in EPS in Rel-17, similar to the proposal [2] made in SA2, and align its specifications based on coming SA2 and RAN(+WGs) decisions.

Proposal#1: CT1 focuses on minimum essential functionality to support IoT NTN in EPS in Rel-17.

Proposal#2: Support for IoT NTN in EPS is introduced using 5GSAT_ARCH as a baseline i.e. 5GSAT_ARCH functionality is imported and adapted into EPS as necessary given EPS and CIoT characteristics, and in accordance with Proposal 1 (i.e. minimum essential functionality).

Proposal#3: CT1 aligns specifications (e.g. TS 24.301) according to stage2 (SA2) and RAN WGs decisions. CT1 updates TS 23.122 for possible impacts on PLMN selection procedure and analyses need for NAS retransmission timer extensions. Technical discussions start at October CT1 meeting.

It is anticipated that SA2#146e meeting will decide on the scope of the work for introducing IoT NTN in EPS in Rel-17. CT work should therefore align with such decision. Furthermore, although a dedicated CT WI is not strictly needed (i.e. alignment work can be done using TEI17 and referring to RAN WI code), should companies prefer a dedicated WI to track CRs, MediaTek volunteers to provide a WI at the earliest at CT#93e plenary, the scope of which would then be aligned with SA2#146e anticipated decision.

Proposal#4: The scope of the work in CT1 (and other CT WGs) aligns with the scope of the work in SA2 (to be decided in SA2#146e).

Proposal#5: Draft WID for CT is provided earliest at CT#93 and latest at October CT1 meeting (CT1#132e).

# 3	Conclusion

Proposal#1: CT1 will focus on minimum essential functionality to support IoT NTN in EPS in Rel-17.

Proposal#2: Support for IoT NTN in EPS is introduced using 5GSAT_ARCH as a baseline i.e. 5GSAT_ARCH functionality is imported and adapted into EPS as necessary given EPS and CIoT characteristics, and in accordance with Proposal 1 (i.e. minimum essential functionality).

Proposal#3: CT1 will align specifications (e.g. TS 24.301) according to stage2 (SA2) and RAN decisions. CT1 updates TS 23.122 for possible impacts on PLMN selection procedure and analyses need for NAS retransmission timer extensions. Technical discussions start at October CT1 meeting.

Proposal#4: The scope of the work in CT1 (and other CT WGs) aligns with the scope of the work in SA2 (to be decided in SA2#146e).

Proposal#5: Draft WID for CT is provided earliest at CT#93 and latest at October CT1 meeting (CT1#132e).

# References

[1]	RP-211601, "WID on NB-IoT/eMTC support for NTN", MediaTek Inc., Eutelsat et al., RAN#92e, June 2021

[2]	S2-2106273, " EPS Support for IoT NTN", MediaTek Inc. et al., SA2#146e, August 2021

# Appendix A – RAN WI Objectives (RP-211601)

# 4	Objective

## 4.1	Objective of Core part WI

The objective of this Work Item is to specify support of NB-IoT and eMTC over NTN. Work on both NB-IoT and eMTC will start in August 2021 meetings.

Enhancements shall be specified as described hereafter with the following assumptions:

- Standalone deployment for NB-IoT / eMTC (i.e. operating in carrier(s) used only for NB-IoT NTN (resp. eMTC NTN)) for support in Rel-17 timeframe will be prioritized.

- GNSS capability in the UE is taken as a working assumption for both NB-IoT and eMTC devices. With this assumption, UE can estimate and pre-compensate timing and frequency offset with sufficient accuracy for UL transmission. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed.

- NB-IoT/eMTC design for terrestrial networks shall be reused as much as possible.

- Transparent payload

### 4.1.1	RAN1

Specify the following time and frequency synchronization enhancements, using NR_NTN_solutions WI  agreements as baseline, according to Section 8 in TR 36.763:

-	UE pre-compensation including ephemeris format (orbital / Position -Velocity)

-	UE pre-compensation for UL synchronization in RRC_IDLE and RRC_CONNECTED states based at least on its GNSS-acquired position and the serving satellite ephemeris

-	Timing advance formula (granularity of the timing advance may be different)

-	Combination of Open (i.e. UE autonomous TA estimation, and common TA estimation) and Closed TA (i.e., received TA commands) control loops in RRC_CONNECTED state

Agreements on the above are up to the decision in NR_NTN_Solutions WI and will be used for IoT NTN with minimum changes, if any.

Specify the following time and frequency synchronization enhancements that are not covered by NR_NTN_Solutions WI agreements, according to Section 8 in TR 36.763:

-	Long PUSCH and PRACH Transmission enhancements: segmented UE pre-compensations, new UL gaps and/or implementation solutions, time units and duration of segments.

-	Validity timer for UL synchronization: satellite ephemeris, and potentially other aspects

-	DL synchronization enhancements: A single solution will be selected between: new channel raster, (part of) ARFCN-indication-in-MIB.

-	GNSS Measurements: Validity of a GNSS position fix and details of acquiring a GNSS position fix, duration of validity, in RRC CONNECTED mode for sporadic short transmission

Specify the following  IoT NTN specific timing relationships enhancements according to Section 8 in TR 36.763:

-	Timing relationships for NB-IoT / eMTC: as listed in Section 6.6.3 in TR 36.763

-	UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD

-	Signalling aspects in UE-specific TA maintenance and reporting, techniques to reduce the signalling load and determination of the UE-specific TA.

### 4.1.2	RAN2

All cellular IoT features specified up to Rel-16 are supported for IoT NTN unless problems are found.

Specify the following enhancements re-using NR_NTN_Solutions WI agreements as a baseline, according to Section 8 in TR 36.763:

-	User Plane:

-	Enhancements to ra-ResponseWindowSize, mac-ContentionResolutionTimer, HARQ RTT timer, UL HARQ RTT timer, and sr-ProhibitTimer.

-	Enhancements to RLC t-Reordering timer.

-	Others:

-	Provisioning of ephemeris

Specify the following IoT NTN specific enhancements not covered by NR_NTN_Solutions WI agreements, according to Section 8 in TR 36.763:

-	Architecture:

-	Support for EPC

-	Mobility and Tracking Area:

-	Enhancements to tracking area management using the earth-fixed TA concept, considering both hard-switch and soft-switch options, where in the soft-switch option the network may broadcast more than one Tracking Area Code per PLMN.

-	Support of legacy (Rel-16) cell selection/reselection mechanisms without major enhancements. Minor adjustments to existing mobility mechanisms, such as a new parameter values, change to timing etc. can be considered to adapt functionality to NTN.

-	Support of legacy (Rel-16) Handover and RLF/reestablishment mechanisms without major enhancements. For eMTC, Rel-16 LTE CHO procedure can be considered without major enhancements. Minor adjustments to existing mobility mechanisms, such as a new parameter values, change to timing etc. can be considered to adapt functionality to NTN.

-	Others:

-	Support of discontinuous coverage without excessive UE power consumption and without excessive failures / recovery actions. Minor enhancements to the existing power saving mechanisms e.g. DRX, PSM, eDRX, relaxed monitoring, and (G)WUS can be considered, and if found needed, specified, to support discontinuous coverage;

### 4.1.3	RAN3

NB-IoT and eMTC NTN support for E-UTRAN (i.e. including S1 interface) will be specified by re-using NR NTN functionality as a baseline, e.g.

-	Support for cell identity and TA corresponding to Earth-fixed area in relevant network interfaces (taking Rel-17 NR NTN as baseline where appropriate)

-	Support for country-specific CN routing (taking Rel-17 NR NTN as baseline where appropriate)

-	Support for identification and restriction of satellite access (following Rel-17 NR NTN, and if confirmed by SA2)

-	OAM requirements (taking Rel-17 NR NTN as baseline where appropriate).

Where needed, adjustments will be considered for IoT NTN specific alignments in line with functionality defined in other WGs.