3GPP TSG-CT WG1 Meeting #131-e	C1-21xxxx

- E-meeting, 19-27 August 2021													(Revision of C1-214470)

Source:	OPPO, InterDigital

Title:	Update direct discovery configuration parameters

Spec:	3GPP TS 24.555 v 0.3.0

Agenda item:	17.2.18

Document for:	Agreement

1. Introduction

There are the configuration parameters for direct discovery.

2. Reason for Change

In the TS 23.304 v 1.0.0, the configuration parameters for direct discovery was updated to remove the discovery group ID. The configuration parameters for direct discovery in TS 24.555 should be updated.

3. Conclusions

The configuration parameters for direct discovery in TS 24.555 are updated.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.555 v 0.3.0.

* * * First Change * * * *

### 5.3.2	Information elements coding

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | ProSeP info type = {UE policies for 5G ProSe direct discovery} |  |  |  | octet k |
| Spare |  |  |  |  |  |  |  |  |
| Length of ProSeP info contents |  |  |  |  |  |  |  | octet k+1 octet k+2 |
| Validity timer |  |  |  |  |  |  |  | octet k+3 octet k+7 |
| Served by NG-RAN |  |  |  |  |  |  |  | octet k+8 octet o1 |
| Not served by NG-RAN |  |  |  |  |  |  |  | octet o1+1 octet o2 |
| ProSe direct discovery UE ID |  |  |  |  |  |  |  | octet o2+1 octet o2+3 |
| Group member discovery parameters |  |  |  |  |  |  |  | octet o2+4 octet o3 |
| ProSe application identifiers |  |  |  |  |  |  |  | octet o3+1 octet o4 |
| ProSe application identifier to default destination Layer-2 ID for initial discovery signalling mapping rules |  |  |  |  |  |  |  | octet o4+1 octet o5 |
| Security parameters used for direct discovery |  |  |  |  |  |  |  | octet o5+1 octet l |

Figure 5.3.1.1: ProSeP Info = {UE policies for 5G ProSe direct discovery}

Editor's note:	How to define the security parameters used for direct discovery depends on SA3 final requirements.

Table 5.3.1.1: ProSeP Info = {UE policies for 5G ProSe direct discovery}

ProSeP info type (bit 1 to 4 of octet k) shall be set to "0001" (UE policies for 5G ProSe direct discovery)

Length of Length of ProSeP info contents (octets k+1 to k+2) indicates the length of ProSeP info contents.

Validity timer (octet k+3 to k+7): The validity timer field provides the expiration time of validity of the UE policies for 5G ProSe direct discovery. The validity timer field is a binary coded representation of a UTC time, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds).

Served by NG-RAN (octet k+8 to o1): The served by NG-RAN field is coded according to figure 5.3.1.2 and table 5.3.1.2, and contains configuration parameters for 5G ProSe direct discovery when the UE is served by NG-RAN.

Not served by NG-RAN (octet o1+1 to o2): The not served by NG-RAN field is coded according to figure 5.3.1.6 and table 5.3.1.6, and contains configuration parameters for 5G ProSe direct discovery when the UE is not served by NG-RAN.

ProSe Direct Discovery UE ID (octet o2+1 to o2+3): The ProSe Direct Discovery UE ID is a 24-bit long bit string.

Group member discovery parameters (octet o2+4 to o3): The group member discovery parameters field is coded according to figure 5.3.1.12 and table 5.3.1.12 and contains group member discovery parameters.

ProSe application identifiers (octet o3+1 to o4): The ProSe application identifiers field is coded according to figure 5.3.1.14 and table 5.3.1.14 and contains ProSe application identifiers.

ProSe application identifier to default destination Layer-2 ID for initial discovery signalling mapping rules (octet o4+1 to o5): The ProSe application identifiers to default destination Layer-2 ID for initial discovery signalling mapping rules field is coded according to figure 5.3.1.15 and table 5.3.1.15 and contains ProSe application identifiers to default destination Layer-2 ID for initial discovery signalling mapping rules.

Security parameters used for direct discovery (octet o5+1 to l): The security parameters used for direct discovery field is coded according to figure 5.3.1.x and table 5.3.1.x and contains Security parameters used for direct discovery.

If the length of ProSeP info contents field is bigger than indicated in figure 5.3.1.1, receiving entity shall ignore any superfluous octets located at the end of the ProSeP info contents.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of served by NG-RAN contents |  |  |  |  |  |  |  | octet k+8 octet k+9 |
| Authorization for direct discovery info 1 |  |  |  |  |  |  |  | octet k+10 octet o50 |
| Authorization for direct discovery info 2 |  |  |  |  |  |  |  | octet o50+1 octet o51 |
| … |  |  |  |  |  |  |  | octet o51+1 octet o52 |
| Authorization for direct discovery info n |  |  |  |  |  |  |  | octet o52+1 octet o1 |

Figure 5.3.1.2: Served by NG-RAN

Table 5.3.1.2: Served by NG-RAN

Authorization for direct discovery info: The authorization for direct discovery info field is coded according to figure 5.3.1.3 and table 5.3.1.3.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of authorization for direct discovery info contents |  |  |  |  |  |  |  | octet o50+1 octet o50+2 |
| 0 spare | 0 spare | 0 spare | 0 spare | 0 spare | Role | Model | DDT | octet o50+3 |
| Authorized PLMN info |  |  |  |  |  |  |  | octet o50+4 octet o51 |

Figure 5.3.1.3: Authorization for direct discovery info

Table 5.3.1.3: Authorization for direct discovery info

Direct discovery type (DDT) (octet o50+3 bit 1): Bit 1 0 Open 1 Restricted

Model (octet o50+3 bit 2): Bit 2 0 A 1 B

If Model bit is set to "A", Role (octet o50+3 bit 3): Bit 3 0 Announcing 1 Monitoring

If Model bit is set to "B", Role (octet o50+3 bit 3): Bit 3 0 Discoverer 1 Discoveree

Authorized PLMN info (octet o50+4 to o51): The authorized PLMN info field is coded according to figure 5.3.1.4 and table 5.3.1.4, or figure 5.3.1.4B and table 5.3.1.4B.

If the length of authorization for direct discovery info field is bigger than indicated in figure 5.3.1.3, receiving entity shall ignore any superfluous octets located at the end of the authorization for direct discovery info.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of authorized PLMN info contents |  |  |  |  |  |  |  | octet o50+4 octet o50+5 |
| Authorized PLMN and range 1 |  |  |  |  |  |  |  | octet (o50+6)* octet (o50+9)* |
| Authorized PLMN and range 2 |  |  |  |  |  |  |  | octet (o50+10)* octet (o50+13)* |
| ... |  |  |  |  |  |  |  | octet (o50+14)* octet o150* |
| Authorized PLMN and range n |  |  |  |  |  |  |  | octet (o150+1)* octet o51* |

Figure 5.3.1.4: Authorized PLMN info when the Role bit is not "monitoring"

Table 5.3.1.4: Authorized PLMN info when the Role bit is not "monitoring"

Authorized PLMN and range: The authorized PLMN and range field is coded according to figure 5.3.1.4A and table 5.3.1.4A.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PLMN ID |  |  |  |  |  |  |  | octet o50+6 octet o50+8 |
| Range |  |  |  |  |  |  |  | octet o50+9 |

Figure 5.3.1.4A: Authorized PLMN and range

Table 5.3.1.4A: Authorized PLMN and range

PLMN ID: The PLMN ID field is coded according to figure 5.3.1.5 and table 5.3.1.5.

Range: Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 1 Short 0 0 0 0 0 0 1 0 Medium 0 0 0 0 0 0 1 1 Long The other values are reserved.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of authorized PLMN info contents |  |  |  |  |  |  |  | octet o50+4 octet o50+5 |
| Authorized PLMN 1 |  |  |  |  |  |  |  | octet (o50+6)* octet (o50+8)* |
| Authorized PLMN 2 |  |  |  |  |  |  |  | octet (o50+9)* octet (o50+11)* |
| ... |  |  |  |  |  |  |  | octet (o50+12)* octet o150* |
| Authorized PLMN n |  |  |  |  |  |  |  | octet (o150+1)* octet o51* |

Figure 5.3.1.4B: Authorized PLMN info when the Role bit is "monitoring"

Table 5.3.1.4B: Authorized PLMN

Authorized PLMN: The authorized PLMN field is coded according to figure 5.3.1.5 and table 5.3.1.5.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet o50+6 |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet o50+7 |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet o50+8 |

Figure 5.3.1.5: PLMN ID

Table 5.3.1.5: PLMN ID

Mobile country code (MCC) (octet o50+5, octet o50+6 bit 1 to 4): The MCC field is coded as in ITU-T Recommendation E.212 [5], annex A.

Mobile network code (MNC) (octet o50+6 bit 5 to 8, octet o50+7): The coding of MNC field is the responsibility of each administration but BCD coding shall be used. The MNC shall consist of 2 or 3 digits. If a network operator decides to use only two digits in the MNC, MNC digit 3 shall be coded as "1111".

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of not served by NG-RAN contents |  |  |  |  |  |  |  | octet o1+1 octet o1+2 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | PDNNI | octet o1+3 |
| NR radio parameters per geographical area list |  |  |  |  |  |  |  | octet (o1+4)* octet o2* |

Figure 5.3.1.6: Not served by NG-RAN

Table 5.3.1.6: Not served by NG-RAN

5G ProSe direct discovery when not served by NG-RAN indicator (PDNNI) (octet o1+3 bit 1): The PDNNI bit indicates whether the UE is authorized to perform 5G ProSe direct discovery when not served by NG-RAN. Bit 1 0 Not authorized 1 Authorized

NR radio parameters per geographical area list (octet o1+4 to o2): If PNNI bit is set to "Authorized", the NR radio parameters per geographical area list field is present otherwise the NR radio parameters per geographical area list field is absent. It is coded according to figure 5.3.1.7 and table 5.3.1.7.

If the length of not served by NG-RAN contents field is bigger than indicated in figure 5.3.1.6, receiving entity shall ignore any superfluous octets located at the end of the not served by NG-RAN contents.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of radio parameters per geographical area list contents |  |  |  |  |  |  |  | octet o1+4 octet o1+5 |
| Radio parameters per geographical area info 1 |  |  |  |  |  |  |  | octet (o1+6)* octet o6* |
| Radio parameters per geographical area info 2 |  |  |  |  |  |  |  | octet (o6+1)* octet o7* |
| ... |  |  |  |  |  |  |  | octet (o7+1)* octet o8* |
| Radio parameters per geographical area info n |  |  |  |  |  |  |  | octet (o8+1)* octet o2* |

Figure 5.3.1.7: Radio parameters per geographical area list

Table 5.3.1.7: Radio parameters per geographical area list

Radio parameters per geographical area info: The radio parameters per geographical area info field is coded according to figure 5.3.1.8 and table 5.3.1.8.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of radio parameters per geographical area contents |  |  |  |  |  |  |  | octet o6+1 octet o6+2 |
| Geographical area |  |  |  |  |  |  |  | octet o6+3 octet o9 |
| Radio parameters |  |  |  |  |  |  |  | octet o9+1 octet o7-1 |
| MI | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | 0 Spare | octet o7 |

Figure 5.3.1.8: Radio parameters per geographical area info

Table 5.3.1.8: Radio parameters per geographical area info

Geographical area (octet o6+3 to o9): The geographical area field is coded according to figure 5.3.1.9 and table 5.3.1.9.

Radio parameters (octet o9 to o7-1): The radio parameters field is coded according to figure 5.3.1.11 and table 5.3.1.11, applicable in the geographical area indicated by the geographical area field when not served by NG-RAN.

Managed indicator (MI) (octet o7 bit 8): The Managed indicator indicates how the radio parameters indicated in the radio parameters field in the geographical area indicated by the geographical area field are managed. Bit 8 0 Non-operator managed 1 Operator managed

If the length of radio parameters per geographical area contents field is bigger than indicated in figure 5.3.1.8, receiving entity shall ignore any superfluous octets located at the end of the radio parameters per geographical area contents.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of geographical area contents |  |  |  |  |  |  |  | octet o6+3 octet o6+4 |
| Coordinate 1 |  |  |  |  |  |  |  | octet (o6+5)* octet (o6+10)* |
| Coordinate 2 |  |  |  |  |  |  |  | octet (o6+11)* octet (o6+16)* |
| ... |  |  |  |  |  |  |  | octet (o6+17)* octet (o6-2+6*n)* |
| Coordinate n |  |  |  |  |  |  |  | octet (o6-1+6*n)* octet (o6+4+6*n)* = octet o9* |

Figure 5.3.1.9: Geographical area

Table 5.3.1.9: Geographical area

Coordinate: The coordinate field is coded according to figure 5.3.1.10 and table 5.3.1.10.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Latitude |  |  |  |  |  |  |  | octet o6+11 octet o6+13 |
| Longitude |  |  |  |  |  |  |  | octet o6+14 octet o6+17 |

Figure 5.3.1.10: Coordinate area

Table 5.3.1.10: Coordinate area

Latitude: The latitude field is coded according to clause 6.1 of 3GPP TS 23.032 [6].

Longitude: The longitude field is coded according to clause 6.1 of 3GPP TS 23.032 [6].

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of radio parameters contents |  |  |  |  |  |  |  | octet o9+1 octet o9+2 |
| Radio parameters contents |  |  |  |  |  |  |  | octet o9+3 octet o7-1 |

Figure 5.3.1.11: Radio parameters

Table 5.3.1.11: Radio parameters

Radio parameters contents: Radio parameters are defined as SL-PreconfigurationNR in clause 9.3 of 3GPP TS 38.331 [7].

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of groupcast parameters contents |  |  |  |  |  |  |  | octet o2+4 octet o2+5 |
| Application layer group info 1 |  |  |  |  |  |  |  | octet (o2+6)* octet (o51)* |
| Application layer group info 2 |  |  |  |  |  |  |  | octet (o51+1)* octet (o52)* |
| … |  |  |  |  |  |  |  | octet (o52+1)* octet (o53)* |
| Application layer group info n |  |  |  |  |  |  |  | octet (o53+1)* octet o3 |

Figure 5.3.1.12: Groupcast parameters

Table 5.3.1.12: Groupcast parameters

Application layer group info: The application layer group info field is coded according to figure 5.3.1.13 and table 5.3.1.13.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of application layer group info contents |  |  |  |  |  |  |  | octet o51+1 octet o51+2 |
| Application layer group identifier |  |  |  |  |  |  |  | octet o51+3 octet o151 |
| ProSe Layer-2 group identifier |  |  |  |  |  |  |  | octet o151+1 octet o151+3 |
| User info ID |  |  |  |  |  |  |  | octet o151+4 octet (o151+9) = octet o52 |

Figure 5.3.1.13: Application layer group info

Table 5.3.1.13: Application layer group info

Application layer group identifier (octet o51+3 to o151): The first octet of application layer group identifier field is the length of application group identifier. The value of application group identifier field is a bit string. The format of application group identifier parameter is out of scope of this specification.

ProSe Layer-2 group identifier (octet o151+1 to o151+3)

The ProSe Layer-2 group identifier field is a binary coded layer-2 identifier.

User info ID (octet o151+4 to o52)

The value of the User Info ID parameter is a 48-bit long bit string. The format of the User Info ID parameter is out of scope of this specification.

If the length of application layer group info contents field is bigger than indicated in figure 5.3.1.13, receiving entity shall ignore any superfluous octets located at the end of the application layer group info contents.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of ProSe application identifiers contents |  |  |  |  |  |  |  | octet o3+1 octet o3+2 |
| ProSe application identifier 1 |  |  |  |  |  |  |  | octet (o3+3)* octet (o3+6)* |
| ProSe application identifier 2 |  |  |  |  |  |  |  | octet (o3+7)* octet (o3+10)* |
| ... |  |  |  |  |  |  |  | octet (o3+11)* octet (o3+n*4)* |
| ProSe application identifier n |  |  |  |  |  |  |  | octet (o3-1+n*4)* octet (o3+2+n*4)* = octet o4* |

Figure 5.3.1.14: ProSe application identifiers

Table 5.3.1.14: ProSe application identifiers

ProSe application identifier:

Editor's note:	It is FFS on how to define the ProSe application identifier.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of ProSe application identifier to default destination Layer-2 ID for initial discovery signalling mapping rules contents |  |  |  |  |  |  |  | octet o4+1 octet o4+2 |
| ProSe application identifier to default destination Layer-2 ID for initial discovery signalling mapping rule 1 |  |  |  |  |  |  |  | octet (o4+3)* octet o54* |
| ProSe application identifier to default destination Layer-2 ID for initial discovery signalling mapping rule 2 |  |  |  |  |  |  |  | octet (o54+1)* octet o55* |
| ... |  |  |  |  |  |  |  | octet (o55+1)* octet o56* |
| ProSe application identifier to default destination Layer-2 ID for initial discovery signalling mapping rule n |  |  |  |  |  |  |  | octet (o56+1)* octet o46* |

Figure 5.3.1.15: ProSe application identifier to default destination Layer-2 ID for initial discovery signalling mapping rules

Table 5.3.1.15: ProSe application identifier to default destination Layer-2 ID for initial discovery signalling mapping rules

ProSe application identifier to destination Layer-2 ID for broadcast mapping rule: The ProSe application identifier to destination Layer-2 ID for broadcast mapping rule field is coded according to figure 5.3.1.16 and table 5.3.1.16.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of ProSe application identifier to default destination Layer-2 ID for initial discovery signalling mapping rule contents |  |  |  |  |  |  |  | octet o54+1 octet o54+2 |
| ProSe application identifiers |  |  |  |  |  |  |  | octet o54+3 octet o154 |
| Destination Layer-2 ID for initial discovery signalling |  |  |  |  |  |  |  | octet o154+1 octet (o154+3) = octet o55 |

Figure 5.3.1.16: ProSe application identifier to default destination Layer-2 ID for initial discovery signalling mapping rule

Table 5.3.1.16: ProSe application identifier to default destination Layer-2 ID for initial discovery signalling mapping rule

ProSe application identifiers (octet o54+3 to o154): The ProSe application identifiers field is coded according to figure 5.3.1.14 and table 5.3.1.14.

Destination Layer-2 ID for initial discovery signalling (octet o154+1 to o55): The destination Layer-2 ID for initial discovery signalling field is a binary coded layer-2 identifier.

If the length of ProSe application identifier to default destination Layer-2 ID for initial discovery signalling mapping rule contents field is bigger than indicated in figure 5.3.1.16, receiving entity shall ignore any superfluous octets located at the end of the ProSe application identifier to default destination Layer-2 ID for initial discovery signalling mapping rule contents.

* * * End of Changes * * * *