3GPP TSG-CT WG1 Meeting #131-e	C1-214809

- E-meeting, 19-27 August 2021

Title:	LS on UE configuration for warning messages reception when using a PLMN subscription to access an SNPN

Release:	Release 17

Work Item:	eNPN

Source:	CT1

To:	SA1

Cc:	CT6

Contact person:	Lena Chaponniere

lguellec@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None.

# 1	Overall description

CT1 is in the process of updating CT1 specifications to support PWS in NPN in Release 17 as per SA Plenary’s decision in SP-210586.

For reception of warning messages in PLMNs, USIM file EFPWS is used to configure the UE on whether to ignore all warning messages received in its HPLMN or in a PLMN equivalent to it, and on whether to ignore all warning messages received in a VPLMN or in a PLMN equivalent to it. For reception of warning messages in SNPNs, a similar configuration is needed. UEs in SNPN access operation mode may or not have a USIM. Hence, for the case that the UE is not using a PLMN subscription to access an SNPN, the configuration needs to be stored in the ME.

CT1 would like to ask SA1 the following question:

Question: For the case when the UE is using a PLMN subscription to access an SNPN so a USIM is used, should be  the configuration for warning messages reception stored in the ME, or should it be stored in the USIM?

# 2	Actions

To SA1

ACTION: 	3GPP TSG CT WG1 asks SA1 to answer the question above.

3	Dates of next TSG CT WG1 meetings

CT1#132e	11th-15th October 2021	electronic meeting

CT1#133e	11th-19th November 2021	electronic meeting