3GPP TSG-CT WG1 Meeting #132-e	C1-215611

- E-meeting, 11-15 October 2021

Source:	vivo

Title:	Determinate destination L2 ID of group member discovery

Spec:	3GPP TS 24.554 v0.4.0.

Agenda item:	17.2.18

Document for:	Agreement

1. Introduction

According to TS 23.304 clause 5.8.1.2, it has been specified that a UE can convert the application layer group ID into the destination layer-2 ID as following:

For Group member discovery:

-	If an Application Layer Group ID has a configured Layer-2 Group ID, which is provisioned as specified in clause 5.1.2.1, the UE uses this Layer-2 Group ID as the Destination Layer-2 ID,

-	otherwise, the UE converts the Application Layer Group ID into a Destination Layer-2 ID.

NOTE:	The mechanism for converting the application layer provided Application Layer Group ID to the Destination Layer-2 ID is defined in Stage 3.

However, in 24.554 the destination L2 ID can only be set to layer-2 group ID. It’s proposed to add the converted destination L2 ID in group member discovery.

Besides, it is also proposed to shift the determination of the souce L2 ID into a single bullet to make it more readable in group member discovery procedure.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 24.554 v0.4.0.

* * * First Change * * * *

###### 6.2.15.2.1.2	Announcing UE procedure for group member discovery initiation

The UE is authorised to perform the announcing UE procedure for group member discovery if:

a)	the following is true:

1)	the UE is not served by NG-RAN, is authorised to perform 5G ProSe direct discovery using announcing procedure when the UE is not served by NG-RAN, and is configured with the radio parameters to be used for 5G ProSe direct discovery when not served by NG-RAN;

2)	the UE is served by NG-RAN, and is authorised to perform 5G ProSe direct discovery using announcing in the PLMN indicated by the serving cell; or

3)	the UE is:

i)	in 5GMM-IDLE mode, in limited service state as specified in 3GPP TS 23.122 [14], and the reason for the UE being in limited service state is one of the following:

A)	the UE is unable to find a suitable cell in the selected PLMN as specified in 3GPP TS 38.304 [15];

B)	the UE received a REGISTRATION REJECT message or a SERVICE REJECT message with the 5GMM cause #11 "PLMN not allowed" as specified in 3GPP TS 24.501 [11]; or

C)	the UE received a REGISTRATION REJECT message or a SERVICE REJECT message with the 5GMM cause #7 "5GS services not allowed " as specified in 3GPP TS 24.501 [11]; and

ii)	authorised to perform 5G ProSe direct discovery using announcing when the UE is not served by NG-RAN, and:

A)	configured with the radio parameters to be used for 5G ProSe direct discovery when not served by NG-RAN; or

B)	the lower layers indicate that the UE does not need to request resources for 5G ProSe direct discovery procedure; and

NOTE:	When the lower layers indicate that the UE does not need to request resources for 5G ProSe direct discovery procedure, the serving cell broadcasts a common radio resources pool for 5G ProSe discovery transmission and the UE can use this common radio resources pool while in limited service state.

b)	the UE is configured with the application layer group ID identifying the application layer group to be announced and with the User info ID for the group member discovery parameter;

otherwise, the UE is not authorised to perform the announcing UE procedure for group member discovery procedure.

Figure 6.2.15.2.1.2.1 illustrates the interaction of the UEs in the announcing UE procedure for group member discovery.

Figure 6.2.15.2.1.2.1: Announcing UE procedure for group member discovery

When the UE is triggered by an upper layer application to announce availability in a discovery group, if the UE is authorised to perform the announcing UE procedure for group member discovery, then the UE:

a)	if the UE is served by NG-RAN, and the UE in 5GMM-IDLE mode needs to request resources for sending PROSE PC5 DISCOVERY messages as specified in 3GPP TS 36.331 [TBD], shall perform a service request procedure as specified in 3GPP TS 24.501 [11];

b)	shall obtain a valid UTC time for the discovery transmission from the lower layers and generate the UTC-based counter corresponding to this UTC time;

c)	shall generate a PROSE PC5 DISCOVERY message for group member discovery announcement. In the PROSE PC5 DISCOVERY message for group member discovery announcement, the UE:

1)	shall set the announcer info parameter to the User info ID for the group member discovery parameter; and

2)	shall set the application layer group ID parameter to the application layer group ID parameter identifying the discovery group to be announced; and

3)	shall set the UTC-based counter LSB parameter to include the eight least significant bits of the UTC-based counter;

d)	shall apply the DUIK, DUSK, or DUCK with the associated Encrypted Bitmask, along with the UTC-based counter to the PROSE PC5 DISCOVERY message for whichever security mechanism(s) configured to be applied, e.g., integrity protection, message scrambling or confidentiality protection of one or more above parameters, as specified in 3GPP TS 33.303[TBD];

Editor’s note:	Details of security related content in d) are FFS and will be determinated by SA WG3.

e)	shall apply one of the following to determine the destination layer-2 ID:

1)	if the application layer group ID has a configured layer-2 group ID as specified in clause 5.2.3, set the destination layer-2 ID to the layer-2 group ID; or

2)	otherwise, convert the application layer group ID into a destination layer-2 ID as following:

i)	to use the group identifier as the input to the SHA-256 hashing algorithm as specified in ISO/IEC 10118-3:2018 [28]; and

ii)	to use the 24 least significant bits of the 256 bits of the output as destination layer-2 ID;

NOTE:	SHA-256 hashing algorithm is implemented in the ME.

f)	shall self-assign a source layer-2 ID for sending the group member discovery announcement message; and

g)	shall pass the resulting PROSE PC5 DISCOVERY message for group member discovery announcement along with the source layer-2 ID and the destination layer-2 ID to the lower layers for transmission over the PC5 interface.

The announcing UE shall ensure that it keeps on passing the same PROSE PC5 DISCOVERY message to the lower layers for transmission until the announcing UE is triggered by an upper layer application to stop announcing availability in a discovery group, or until the UE stops being authorised to perform the announcing UE procedure for group member discovery.

* * * Next Change * * * *

###### 6.2.15.2.1.4	Monitoring UE procedure for group member discovery initiation

The UE is authorised to perform the monitoring UE procedure for group member discovery if:

a)	the following is true:

1)	the UE is not served by NG-RAN, is authorised to perform 5G ProSe direct discovery using monitoring when the UE is not served by NG-RAN, and is configured with the radio parameters to be used for 5G ProSe direct discovery when not served by NG-RAN;

2)	the UE is served by NG-RAN, and is authorised to perform 5G ProSe direct discovery monitoring in at least one PLMN; or

3)	the UE is:

i)	in 5GMM-IDLE mode, in limited service state as specified in 3GPP TS 23.122 [14], and the reason for the UE being in limited service state is one of the following:

A)	the UE is unable to find a suitable cell in the selected PLMN as specified in 3GPP TS 38.304 [15];

B)	the UE received a REGISTRATION REJECT message or a SERVICE REJECT message with the 5GMM cause #11 "PLMN not allowed" as specified in 3GPP TS 24.501 [11] ; or

C)	the UE received a REGISTRATION REJECT message or a SERVICE REJECT message with the 5GMM cause #7 "5GS services not allowed" as specified in 3GPP TS 24.501 [11]; and

ii)	authorised to perform 5G ProSe direct discoveryusing monitoring when the UE is not served by NG-RAN, and:

A)	configured with the radio parameters to be used for 5G ProSe direct discovery when not served by NG-RAN; or

B)	the lower layers indicate that the UE does not need to request resources for 5G ProSe direct discovery procedure; and

NOTE:	When the lower layers indicate that the UE does not need to request resources for 5G ProSe direct discovery procedure, the serving cell broadcasts a common radio resources pool for 5G ProSe discovery transmission and the UE can use this common radio resources pool while in limited service state.

b)	the UE is configured with the application layer group ID parameter identifying the discovery group to be monitored;

otherwise, the UE is not authorised to perform the monitoring UE procedure for group member discovery.

Figure 6.2.15.2.1.4.1 illustrates the interaction of the UEs in the monitoring UE procedure for group member discovery.

Figure 6.2.15.2.1.4.1: Monitoring UE procedure for group member discovery

When the UE is triggered by an upper layer application to monitor proximity of other UEs in a discovery group, and if the UE is authorised to perform the monitoring UE procedure for group member discovery, then the UE shall instruct the lower layers to start monitoring for PROSE PC5 DISCOVERY message as following:

a)	if the application layer group ID has a configured layer-2 group ID as specified in clause 5.2.3, the UE shall monitor for PROSE PC5 DISCOVERY message with the layer-2 group ID as specified in clause 5.2.3; or

b)	otherwise, the UE shall convert the application layer group ID into a destination layer-2 ID, and shall monitor for PROSE PC5 DISCOVERY message with the converted destination layer-2 ID. The UE shall convert the application layer group ID into a destination layer-2 ID as following:

1)	to use the group identifier as the input to the SHA-256 hashing algorithm as specified in ISO/IEC 10118-3:2018 [28]; and

2)	to use the 24 least significant bits of the 256 bits of the output as destination layer-2 ID.

NOTE:	SHA-256 hashing algorithm is implemented in the ME.

Upon reception of a PROSE PC5 DISCOVERY message for group member discovery announcement, for the target application layer group ID of the discovery group to be monitored, the UE shall use the associated DUSK, if configured, and the UTC-based counter obtained during the monitoring operation to unscramble the PROSE PC5 DISCOVERY message as described in 3GPP TS 33.303 [TBD]. Then, if a DUCK is configured, the UE shall use the DUCK and the UTC-based counter to decrypt the configured message-specific confidentiality-protected portion, as described in 3GPP TS 33.303 [TBD]. Finally, if a DUIK is configured, the UE shall use the DUIK and UTC-based counter to verify the MIC field in the unscrambled PROSE PC5 DISCOVERY message for group member discovery announcement.

NOTE 2:	The use of an erroneous UTC-based counter for processing received PROSE PC5 DISCOVERY messages at the ProSe-enabled UE can cause MIC check failure after DUIK is used for integrity check, and malformed contents after DUSK is used for unscrambling or DUCK is used for deciphering. How a ProSe-enabled UE ensures the accuracy of the UTC-based counter is left to UE implementation.

Then if the application layer group ID parameter of the PROSE PC5 DISCOVERY message for group member discovery announcement is the same as the configured application layer group ID parameter as specified in clause 5, the UE shall consider that other UE in the discovery group the UE seeks to monitor has been discovered.

Editor’s note:	Details of Monitoring UE procedure upon reception of a PROSE PC5 DISCOVERY message for group member discovery announcement are FFS and will be determinated by cooperation with SA WG3.

* * * Next Change * * * *

###### 6.2.15.2.2.2	Discoverer UE procedure for group member discovery initiation

The UE is authorised to perform the discoverer UE procedure for group member discovery if:

a)	the following is true:

1)	the UE is not served by NG-RAN, is authorised to perform 5G ProSe direct discovery discoverer operation when the UE is not served by NG-RAN, and is configured with the radio parameters to be used for 5G ProSe direct discovery when not served by NG-RAN;

2)	the UE is served by NG-RAN, and is authorised to perform 5G ProSe direct discovery discoverer operation in the PLMN indicated by the serving cell; or

3)	the UE is:

i)	in 5GMM-IDLE mode, in limited service state as specified in 3GPP TS 23.122 [14], and the reason for the UE being in limited service state is one of the following:

A)	the UE is unable to find a suitable cell in the selected PLMN as specified in 3GPP TS 38.304 [15];

B)	the UE received a REGISTRATION REJECT message or a SERVICE REJECT message with the 5GMM cause #11 "PLMN not allowed" as specified in 3GPP TS 24.501 [11]; or

C)	the UE received a REGISTRATION REJECT message or a SERVICE REJECT message with the 5GMM cause #7 "5GS services not allowed" as specified in 3GPP TS 24.501 [11]; and

ii)	authorised to perform 5G ProSe direct discovery discoverer operation when the UE is not served by NG-RAN, and:

A)	configured with the radio parameters to be used for 5G ProSe direct discovery use when not served by NG-RAN; or

B)	the lower layers indicate that the UE does not need to request resources for 5G ProSe direct discovery procedure; and

NOTE:	When the lower layers indicate that the UE does not need to request resources for 5G ProSe direct discovery procedure, the serving cell broadcasts a common radio resources pool for 5G ProSe discovery transmission and the UE can use this common radio resources pool while in limited service state.

b)	the UE is configured with the application layer group ID parameter identifying the discovery group to be solicited and with the User info ID for the group member discovery parameter;

otherwise, the UE is not authorised to perform the discoverer UE procedure for group member discovery.

Figure 6.2.15.2.2.2.1 illustrates the interaction of the UEs in the discoverer UE procedure for group member discovery.

Figure 6.2.15.2.2.2.1: Discoverer UE procedure for group member discovery

When the UE is triggered by an upper layer application to solicit proximity of other UEs in a discovery group, and if the UE is authorised to perform the discoverer UE procedure for group member discovery, then the UE:

a)	if the UE is served by NG-RAN, and the UE in 5GMM-IDLE mode needs to request resources for sending PROSE PC5 DISCOVERY messages as specified in 3GPP TS 36.331 [TBD], shall perform a service request procedure as specified in 3GPP TS 24.501 [11];

b)	shall obtain a valid UTC time for the discovery transmission from the lower layers and generate the UTC-based counter corresponding to this UTC time;

c)	shall generate a PROSE PC5 DISCOVERY message for group member discovery solicitation. In the PROSE PC5 DISCOVERY message for group member discovery solicitation, the UE:

1)	shall set the Discoverer info parameter to the User info ID for the group member discovery parameter;

2)	shall set the application layer group ID parameter to the application layer group ID parameter identifying the discovery group to be solicited;

3)	shall set either the target user info parameter or the target group info parameter to the target info according to the target information provided by the upper layer application; and

4)	shall set the UTC-based counter LSB parameter to include the 8 least significant bits of the UTC-based counter;

d)	shall apply the DUIK, DUSK, or DUCK with the associated Encrypted Bitmask, along with the UTC-based counter to the PROSE PC5 DISCOVERY message for whichever security mechanism(s) configured to be applied, e.g. integrity protection, message scrambling or confidentiality protection of one or more above parameters, as specified in 3GPP TS 33.303 [TBD];

Editor’s note:	Details of security related content in d) are FFS and will be determinated by SA WG3.

e)	shall apply one of the following to determine the destination layer-2 ID:

1)	if the application layer group ID has a configured layer-2 group ID as specified in clause 5.2.3, set the destination layer-2 ID to the layer-2 group ID; or

2)	otherwise, convert the application layer group ID into a destination layer-2 ID as following:

i)	to use the group identifier as the input to the SHA-256 hashing algorithm as specified in ISO/IEC 10118-3:2018 [28]; and

ii)	to use the 24 least significant bits of the 256 bits of the output as destination layer-2 ID;

NOTE:	SHA-256 hashing algorithm is implemented in the ME.

f)	shall self-assign a source layer-2 ID for sending the group member discovery solicitation message; and

g)	shall pass the resulting PROSE PC5 DISCOVERY message for group member discovery solicitation along with the source layer-2 ID and destination layer-2 ID to the lower layers for transmission over the PC5 interface.

The UE shall ensure that it keeps on passing the same PROSE PC5 DISCOVERY message to the lower layers for transmission with an indication that the message until the UE is triggered by an upper layer application to stop soliciting proximity of other UEs in a discovery group, or until the UE stops being authorised to perform the discoverer UE procedure for group member discovery.

Upon reception of a PROSE PC5 DISCOVERY message for group member discovery response, for the target application layer group ID of the discovery group to be discovered, the UE shall use the associated DUSK, if configured, and the UTC-based counter obtained during the monitoring operation to unscramble the PROSE PC5 DISCOVERY message as described in 3GPP TS 33.303 [TBD]. Then, if a DUCK is configured, the UE shall use the DUCK and the UTC-based counter to decrypt the configured message-specific confidentiality-protected portion, as described in 3GPP TS 33.303 [TBD]. Finally, if a DUIK is configured, the UE shall use the DUIK and UTC-based counter to verify the MIC field in the unscrambled PROSE PC5 DISCOVERY message for group member discovery response.

Then if the application layer group ID parameter of the PROSE PC5 DISCOVERY message for group member discovery response is the same as the application layer group ID parameter of the PROSE PC5 DISCOVERY message for group member discovery solicitation, the UE shall consider that other UE in the discovery group the UE seeks to discover has been discovered.

Editor’s note:	Details of Discoverer UE procedure upon reception of a PROSE PC5 DISCOVERY message for direct discovery response are FFS and will be determinated by cooperation with SA WG3.