3GPP TSG-CT WG1 Meeting #132-e	C1-215578

- E-meeting, 11-15 October 2021

Source:	CATT

Title:	Cell ID announcement request procedure

Spec:	3GPP TS 24.554 v040

Agenda item:	17.2.18

Document for:	Agreement

1. Introduction

Cell ID announcement procedure is defined be SA2 in clause 6.5.1.3, TS 23.304 v110. But the procedure is not reflected to Stage 3.

2. Reason for Change

Change TS 24.554 to align with SA2’s conclusion.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.554 v040.

* * * First Change * * * *

### 8.2.x	5G ProSe Cell ID announcement procedure

#### 8.2.x.1	General

The purpose of the 5G ProSe cell ID announcement procedure is for the 5G ProSe layer-3 remote UE to obtain NCGI of the cell serving the 5G ProSe layer-3 UE-to-network relay UE.

The remote UE in this procedure shall be a 5G ProSe-enabled UE and is authorised to act as a 5G ProSe layer-3 remote UE towards a 5G ProSe layer-3 UE-to-network relay UE based on the service authorisation procedure as specified in clause 5. The 5G ProSe layer-3 UE-to-network relay UE in this procedure shall be a 5G ProSe-enabled UE and is authorised to act as a 5G layer-3 ProSe UE-to-network relay UE based on the service authorisation procedure as specified in clause 5.

* * * Next Change * * * *

#### 8.2.x.2	5G ProSe Cell ID announcement procedure initiation by the 5G ProSe layer-3  remote UE

Before initiating the 5G ProSe cell ID announcement procedure, a direct link has been successfully established between the 5G ProSe layer-3 remote UE and the 5G ProSe layer-3 UE-to-network relay UE.

The 5G ProSe layer-3 remote UE shall initiate a 5G ProSe cell ID announcement procedure:

a)	when the 5G ProSe layer-3 remote UE is triggered by an upper layer application to report NCGI of the serving cell to the application server, but cannot receive the PROSE PC5 DISCOVERY message for relay discovery additional information from the 5G ProSe layer-3 UE-to-network relay UE, or the NCGI is not included in the PROSE PC5 DISCOVERY message for relay discovery additional information from the 5G ProSe layer-3 UE-to-network relay UE; or

b)	when the NCGI announcement request refresh timer T5106 expires and the 5G ProSe layer-3 remote UE still needs to obtain NCGI of the cell serving the 5G ProSe layer-3 UE-to-network relay.

The 5G ProSe layer-3 remote UE shall generate a PROSE CELL ID ANNOUNCEMENT REQUEST message and pass this message to the lower layers for transmission along with the 5G ProSe layer-3 remote UE's layer-2 ID (for unicast communication) and the 5G ProSe layer-3 UE-to-network relay UE's layer-2 ID (for unicast communication).

Figure 8.2.x.2.1: 5G ProSe Cell ID announcement procedure

* * * Next Change * * * *

#### 8.2.x.3	5G ProSe Cell ID announcement procedure accepted by the 5G ProSe layer-3 UE-to-network relay UE

Upon receiving a PROSE CELL ID ANNOUNCEMENT REQUEST message, the 5G ProSe layer-3 UE-to-network relay UE shall allocate a NCGI announcement request refresh timer T5106 to the remote UE, and start a timer T5107. The timer T5107 shall be longer than the NCGI announcement request refresh timer T5106.

Then the 5G ProSe layer-3 UE-to-network relay UE shall respond a PROSE CELL ID ANNOUNCEMENT RESPONSE message with a NCGI announcement request refresh timer T5106 IE set to the T5106 timer value assigned by the 5G ProSe layer-3 UE-to-network relay UE.

* * * Next Change * * * *

#### 8.2.x.4	5G ProSe Cell ID announcement procedure completion by the 5G ProSe layer-3 remote UE

Upon receiving a PROSE CELL ID ANNOUNCEMENT RESPONSE message, the UE shall start the NCGI announcement request refresh timer T5106 with the received value.

* * * Next Change * * * *

#### 8.2.x.5	Abnormal cases

##### 8.2.x.5.1	Abnormal cases in the 5G ProSe layer-3 remote UE

If there is no response from the 5G ProSe layer-3 UE-to-network relay UE after the PROSE CELL ID ANNOUNCEMENT REQUEST message has been successfully delivered, the 5G ProSe layer-3 remote UE shall retransmit the PROSE CELL ID ANNOUNCEMENT REQUEST message.

NOTE:	The timer to trigger retransmission and the maximum number of allowed retransmissions are UE implementation specific.

* * * Next Change * * * *

### 10.3.x	ProSe cell ID announcement request

#### 10.3.x.1	Message definition

This message is sent by the 5G ProSe layer-3 remote UE to 5G ProSe layer-3 UE-to-network relay UE to initiate the ProSe cell ID announcement request procedure. See table 10.3.x.1.1.

Message type:	PROSE CELL ID ANNOUNCEMENT REQUEST

Significance:	dual

Direction:		UE to peer UE

Table 10.3.x.1.1: PROSE CELL ID ANNOUNCEMENT REQUEST content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE CELL ID ANNOUNCEMENT REQUEST identity | Message Type 11.3.1 | M | V | 1 |
|  | Sequence Number | Sequence Number 11.3.2 | M | V | 1 |

* * * Next Change * * * *

### 10.3.y	ProSe cell ID announcement response

#### 10.3.y.1	Message definition

This message is sent by the 5G ProSe layer-3 UE-to-network relay UE to the 5G ProSe layer-3 remote UE to acknowledge and respond to the ProSe cell ID announcement request. See table 10.3.y.1.1.

Message type:	PROSE CELL ID ANNOUNCEMENT RESPONSE

Significance:	dual

Direction:		UE to peer UE

Table 10.3.y.1.1: PROSE CELL ID ANNOUNCEMENT RESPONSE content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE CELL ID ANNOUNCEMENT RESPONSE identity | ProSe PC5 signalling message type 11.3.1 | M | V | 1 |
|  | Sequence Number | Sequence number 11.3.2 | M | V | 1 |
|  | NCGI announcement request refresh timer T5106 | Binary 11.3.y | M | V | 2 |

* * * Next Change * * * *

### 11.3.1	ProSe PC5 signalling message type

The purpose of the ProSe PC5 signalling message type information element is to indicate the type of messages used in ProSe PC5 signalling protocol.

The value part of the ProSe PC5 signalling message type information element used in the ProSe PC5 signalling messages is coded as shown in table 11.3.1.1.

The ProSe PC5 signalling message type is a type 3 information element, with the length of 1 octet.

Table 11.3.1.1: ProSe PC5 signalling message type

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | PROSE DIRECT LINK ESTABLISHMENT REQUEST |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |  | PROSE DIRECT LINK ESTABLISHMENT ACCEPT |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |  | PROSE DIRECT LINK ESTABLISHMENT REJECT |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |  | PROSE DIRECT LINK MODIFICATION REQUEST |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 |  | PROSE DIRECT LINK MODIFICATION ACCEPT |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |  | PROSE DIRECT LINK MODIFICATION REJECT |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |  | PROSE DIRECT LINK RELEASE REQUEST |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |  | PROSE DIRECT LINK RELEASE ACCEPT |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 |  | PROSE DIRECT LINK KEEPALIVE REQUEST |
| 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 |  | PROSE DIRECT LINK KEEPALIVE RESPONSE |
| 0 | 0 | 0 | 0 | 1 | 0 | 1 | 1 |  | PROSE DIRECT LINK AUTHENTICATION REQUEST |
| 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 |  | PROSE DIRECT LINK AUTHENTICATION RESPONSE |
| 0 | 0 | 0 | 0 | 1 | 1 | 0 | 1 |  | PROSE DIRECT LINK AUTHENTICATION REJECT |
| 0 | 0 | 0 | 0 | 1 | 1 | 1 | 0 |  | PROSE DIRECT LINK SECURITY MODE COMMAND |
| 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |  | PROSE DIRECT LINK SECURITY MODE COMPLETE |
| 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |  | PROSE DIRECT LINK SECURITY MODE REJECT |
| 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |  | PROSE DIRECT LINK REKEYING REQUEST |
| 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 |  | PROSE DIRECT LINK REKEYING RESPONSE |
| 0 | 0 | 0 | 1 | 0 | 0 | 1 | 1 |  | PROSE DIRECT LINK IDENTIFIER UPDATE REQUEST |
| 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 |  | PROSE DIRECT LINK IDENTIFIER UPDATE ACCEPT |
| 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 |  | PROSE DIRECT LINK IDENTIFIER UPDATE ACK |
| 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 |  | PROSE DIRECT LINK IDENTIFIER UPDATE REJECT |
| 0 | 0 | 0 | 1 | 0 | 1 | 1 | 1 |  | PROSE DIRECT LINK AUTHENTICATION FAILURE |
| 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 |  | PROSE CELL ID ANNOUNCEMENT REQUEST |
| 0 | 0 | 0 | 1 | 1 | 0 | 0 | 1 |  | PROSE CELL ID ANNOUNCEMENT RESPONSE |

* * * Next Change * * * *

### 11.3.y	NCGI announcement request refresh timer T5106

This parameter is used to carry the value of NCGI announcement request refresh timer. It is an integer in the 1-1440 range representing the timer value in unit of minutes.

The NCGI announcement request refresh timer T5106 is a type 3 information element with a length of 2 octets.

The NCGI announcement request refresh timer T5106 IE is coded as shown in figure 11.3.y.1 and table 11.3.y.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NCGI announcement request refresh timer T5106 IEI |  |  |  |  |  |  |  | octet 1 |
| NCGI announcement request refresh timer T5106 contents |  |  |  |  |  |  |  | octet 2 octet 3 |

Figure 11.3.y.1: NCGI announcement request refresh timer T5106 information element

Table 11.3.y.1: NCGI announcement request refresh timer T5106 information element

NCGI announcement request refresh timer T5106 (octet 1 to 2) This field contains the 16-bit NCGI announcement request refresh timer T5106.

* * * Next Change * * * *

## 12.x	Timers of 5G ProSe cell ID announcement procedure

Table 12.x: 5G ProSe Cell ID announcement timers

| TIMER NUM. | TIMER VALUE | CAUSE OF START | NORMAL STOP | ON EXPIRY |
| --- | --- | --- | --- | --- |
| T5106 | NOTE 1 | Upon receiving a T5106 value in a PROSE CELL ID ANNOUNCEMENT RESPONSE message as described in subclause 10.3.y. | Upon receiving one or more new T5106 timer values in a PROSE CELL ID ANNOUNCEMENT_RESPONSE message. | Re-initiate the cell ID announcement request procedure, if the upper layer application still needs to obtain NCGI of the cell serving the 5G ProSe layer-3 UE-to-network relay. |
| T5107 | NOTE 2 | Upon sending a PROSE CELL ID ANNOUNCEMENT RESPONSE message as described in subclause 10.3.y. | Upon sending a new PROSE CELL ID ANNOUNCEMENT RESPONSE message. | Stop the cell ID announcement in the PROSE PC5 DISCOVERY message for relay discovery additional Information. |
| NOTE 1: The value of this timer is provided by the 5G ProSe layer-3 UE-to-network relay UE during the 5G ProSe cell ID announcement procedure. NOTE 2: The value of this timer is assigned by the 5G ProSe layer-3 UE-to-network relay UE during the 5G ProSe cell ID announcement procedure. |  |  |  |  |

* * * End of Change * * * *