3GPP TSG-CT WG1 Meeting #132-e	C1-21xxxx

- E-meeting, 11-15 October 2021

Source:	ZTE

Title:	Clarification on N3IWF selection information for 5G ProSe layer-3 remote UE

Spec:	3GPP TS 24.554 v0.4.0

Agenda item:	17.2.18

Document for:	Agreement

1. Introduction

According to TS 23.304, N3IWF selection information for 5G ProSe Layer-3 Remote UE is included in UE policies for 5G ProSe remote UE. It contains two parts:

-	N3IWF identifier configuration for 5G ProSe Layer-3 Remote UE (either FQDN or IP address) in the HPLMN.

-	5G ProSe Layer-3 UE-to-Network Relay access node selection information - a prioritized list of PLMNs for N3IWF selection. It also indicates if selection of an N3IWF in a PLMN should be based on Tracking Area Identity FQDN or on Operator Identifier FQDN.

2. Reason for Change

"N3IWF identifier configuration in the HPLMN" needs to be modified to "N3IWF identifier configuration for 5G ProSe layer-3 remote UE" to align with TS 23.304.

The encoding of N3IWF selection information for 5G ProSe Layer-3 Remote UE is defined in TS 24.555.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.554 v0.4.0.

* * * First Change * * * *

### 5.2.5	Configuration parameters for 5G ProSe UE-to-network relay

The configuration parameters for the role of a ProSe UE-to-network relay UE over PC5 reference point consist of:

a)	a validity timer for the validity of the configuration parameter for 5G ProSe UE-to-network relay over PC5 interface;

b)	a list of PLMNs in which the UE is authorised to relay traffic for 5G ProSe layer-3 remote UEs when the UE is served by NG-RAN, and in each PLMN;

c)	a list of PLMNs in which the UE is authorised to relay traffic for 5G ProSe layer-2 remote UEs when the UE is served by NG-RAN, and in each PLMN;

d)	the default destination layer-2 ID(s) for sending and receiving the initial UE-to-network relay discovery signaling;

Editor's note:	It is FFS whether there is more than one default destination layer-2 ID for the initial discovery message e.g., default destination layer-2 ID per relay service code.

e)	a User info ID for the UE-to-network relay discovery;

f)	one or more relay service code(s) for the UE-to-network relay discovery, and for each relay service code:

1)	security related content for 5G ProSe relay discovery; and

2)	for 5G ProSe layer-3 UE-to-network relay UE, a set of PDU session parameters:

i)	PDU Session type;

ii)	optionally, DNN;

iii)	optionally, SSC Mode;

iv)	optionally, S-NSSAI; or

v)	optionally, access type preference.

Editor's note:	Details of security related content are FFS and will be determinated by SA WG3.

g)	for 5G ProSe layer-3 UE-to-network relay UE, QoS mapping rules including:

1)	a mapping between a 5QI value and a 5G ProSe PQI value over PC5 for traffic relayed over the PC5 interface;

2)	a PDB adjustment factor of the standardized PDB identified by the PQI; and

3)	optionally, the relay service code(s) associated with the QoS mapping rule;

h)	the radio parameters of the 5G ProSe UE-to-network relay discovery applicable per geographical area with an indication of whether these radio parameters are "operator managed" or "non-operator managed" when the UE is not served by NG-RAN;

i)	for 5G ProSe layer-3 UE-to-network relay UE, for Ethernet and Unstructured traffic using IP type PDU session, a list of ProSe Service(s) to ProSe application server address mapping rule. Each mapping rule contains one or more ProSe Service(s) and IP address/FQDN and transport layer port number; and

Editor's Note: How to identify ProSe Service(s) is up to stage-2 decision.

j)	the radio parameters of the 5G ProSe direct communication applicable per geographical area with an indication of whether these radio parameters are "operator managed" or "non-operator managed" when the UE is not served by NG-RAN.

The configuration parameters for the role of a 5G ProSe remote UE consist of:

a)	a validity timer for the validity of the configuration parameters for 5G ProSe remote UE;

b)	a list of PLMNs in which the UE is authorized to use a 5G ProSe layer-3 UE-to-network relay UE;

c)	a list of PLMNs in which the UE is authorized to use a 5G ProSe layer-2 UE-to-network relay UE;

d)	default destination layer-2 ID for sending and receiving the initial UE-to-network relay discovery signalling;

e)	a User info ID for the UE-to-network relay discovery;

f)	one or more relay service code(s) for the UE-to-network relay discovery, and for each relay service code:

1)	security related content for 5G ProSe relay discovery;

2)	for 5G ProSe remote UE using 5G ProSe layer-3 UE-to-network relays, a set of PDU session parameters:

i)	PDU Session type;

ii)	optionally, DNN;

iii)	optionally, SSC Mode;

iv)	optionally, S-NSSAI; or

v)	optionally, access type preference; and

3)	for communication with 5G ProSe layer-3 UE-to-network relays, an indication that the relay service code is corresponding to 5G ProSe layer-3 UE-to-network relay with N3IWF support;

Editor's note:	A need for explicit indication for relay service code corresponding to 5G ProSe layer-3 UE-to-network relay with N3IWF will be aligned once stage-2 requirement is clarified.

Editor's note:	Details of security related content are FFS and will be determinated by SA WG3.

g)	the radio parameters of the 5G ProSe Relay Discovery applicable per geographical area with an indication of whether these radio parameters are "operator managed" or "non-operator managed" when the UE is not served by NG-RAN;

h)	the radio parameters of the 5G ProSe direct communication applicable per geographical area with an indication of whether these radio parameters are "operator managed" or "non-operator managed" when the UE is not served by NG-RAN; and

NOTE:	Whether a frequency band is "operator managed" or "non-operator managed" in a given Geographical Area is defined by local regulations.

i)	the N3IWF selection information for 5G ProSe layer-3 remote UE:

1)	N3IWF identifier configuration (either FQDN or IP address); and

2)	5G ProSe layer-3 UE-to-network relay access node selection information consists of

a prioritized list of PLMNs for N3IWF selection and an indication that the selection of an N3IWF in a PLMN should be based on Tracking Area Identity FQDN or on Operator Identifier FQDN.

* * * Next Change * * * *

#### 8.2.7.3	N3IWF selection for 5G ProSe layer-3 remote UE

As specified in clause 6.5.1.2.2 of 3GPP TS 23.304 [2], the 5G ProSe layer-3 remote UE selects the N3IWF using following information included in UE policies for 5G ProSe UE-to-network remote UE as defined in clause 5.6.2 of 3GPP TS 24.555 [17]:

a)	N3IWF identifier configuration for 5G ProSe layer-3 remote UE; and

b)	5G ProSe layer-3 UE-to-network relay access node selection information.

The 5G ProSe layer-3 remote UE shall perform the N3IWF selection procedure as specified in clause 7.2.4.3 of 3GPP TS 24.502 [26].

Editor's note:	It is FFS what additional procedure is needed. It depends on the definition of the N3IWF identifier configuration in the HPLMN and the access node selection information in UE policies for 5G ProSe layer-3 UE-to-network relay.

* * * End of Changes * * * *