3GPP TSG-CT WG1 Meeting #133-bis-e	C1- 220056

# E-meeting, 17-21 January 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.282 | CR | 0281 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving Editor’s Note related to MCData Server to Server API security mechanism |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCData3 |  |  |  |  |  | Date: |  |  | 2022-01-10 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To resolve the following two ENs in subcaluse 21.1: - “Editor's note: [eMCData2, CR 0168, C1-204022] The security mechanism for communication from the MCData server acting as an HTTP client and the Message store function acting as an HTTP server is FFS.” and - “Editor's note: [eMCData3, CR 0254, C1-215635] The security mechanism for communication from the MCData store function acting as an HTTP client and the MCData notification server acting as an HTTP server is FFS.” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding an statement to 21.1 which referrences SA3’s 33.180 spec where the security mechanisms for two MCData network functions (i.e. servers) in the network is specified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 3 spec, TS 24.282, won’t be complete and the spec will be confusing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 21.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

# 21	MCData Message Store

## 21.1	General

This clause defines procedures for communication between the MCData message store client and the MCData message store function as well as the MCData server and the MCData message store function as specified in clause 7.13.3 of 3GPP TS 23.282[2].

Additionally, this clause defines procedures for communication between the Message notification client and the MCData notification server as well as the MCData message store function and the MCData notification server as specified in clause 7.13.3 of 3GPP TS 23.282[2].

The communication between the MCData message store client and the MCData message store function as well as between the Message notification client and the MCData notification server shall use HTTP over TLS as specified in annex A of 3GPP TS 24.482 [24].

The MCData message store function shall act as an HTTP server as defined in annex A of 3GPP TS 24.482 [24].

The MCData message store client and the Message notification client in the role of an HTTP client shall include the MCData access token (with the "Bearer" authentication scheme) in the Authorization header field of an HTTP request as specified in 3GPP TS 24.482 [24].

Editor's note:	[eMCData3, CR 0254, C1-215635] How the MCData client is aware of the hostname of the MCData message store and the MCData norification server is FFS.

The HTTP server (i.e. the MCData message store and the MCData notification server) shall validate the MCData access token as specified in 3GPP TS 24.482 [24].

NOTE 1:	In procedures for communication between the MCData message store client and the MCData message store function as well as communication between the Message notification client and the MCData notification server, the MCData ID which is the identity of the MCData user is part of MCData access token as specified in 3GPP TS 24.482 [24]. Additionally, the MCData ID can be used as the value for userId variable while generating the HTTP request URL.

NOTE 1A:	In procedures for communication between MCData server and MCData message store function, the MCData ID which is the identity of the MCData user is used as the value of the resource URL variable, "boxId" as specified in clause 5.2 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66].

The interface between the MCData message store client and the MCData message store function (i.e. MCData-7) as well as the interface between the MCData server and the MCData message store function (i.e. MCData-8) shall be based on the RESTful API as specified in OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66].

The interface between the Message notification client and the MCData notification server (i.e. MCData-10) shall be based on the RESTful API as specified in OMA-TS-REST_NetAPI_NotificationChannel-V1_0-20200319-C [76].

the MCData message store function uses HTTP POST method to push notifications to the MCData notification server (i.e. MCData-11) at a CallBack URL provided by the MCData message store client during notification subscription creation procedure (as defined in the following clauses).

The HTTP communications (i.e. RESTful API invocations) between the MCData server and the MCData message store function (i.e. MCData-8) as well as between the MCData message store function and the MCData notification server (i.e. MCData-11) are authenticated/authorizated as per security mechanisms described in 3GPP TS 33.180 [26].

NOTE 2:	Procedures defined for communication between the MCData message store client and the MCData message store function as well as the MCData server and the MCData message store function in the following sections reference clause 6 "Detailed specification of the resources" of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66]. Additional information related to RESTful resources, data types and sequence diagrams are found in clause 5 and JSON examples in appendix D of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66].

NOTE 3:	Procedures defined for communication between the Message notification client and the MCData notification server in the following sections reference clause 6 "Detailed specification of the resources" of OMA-TS-REST_NetAPI_NotificationChannel-V1_0-20200319-C [76]. Additional information related to RESTful resources, data types and sequence diagrams are found in clause 5 and JSON examples in appendix D of OMA-TS-REST_NetAPI_NotificationChannel-V1_0-20200319-C [76].

* * * * * * * END CHANGES * * * * * * *