3GPP TSG-CT WG1 Meeting #133-bis-e	C1-220417

# E-meeting, 17-21 January 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.282 | CR | 0283 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Common procedure to retrieve the file from functional entity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCData3 |  |  |  |  |  | Date: |  |  | 2022-01-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per TS 23.282 subclauses 7.13.3.20.2 & 7.13.3.1.41 “MCData retrieve file to store locally request” operation can be invoked by the message store client which would lead to having the MCData message store retrieving the file from the MCData content server. Accordingly, stage 3, TS 24.282, needs to be updated to address the newly introduced feature in stage 2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Subsclause 6.7 “Stored files operational procedures” procedure is specified to address the newly introduced feature in stage 2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 3 spec TS 24.282 won’t be able to support the newly introduced feature in stage 2, TS 23.282 (subclauses 7.13.3.20.2 & 7.13.3.1.41). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7 (NEW), 6.7.1 (NEW), 6.7.2 (NEW), 6.7.2.1 (NEW), 6.7.2.2 (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

## 6.7	Stored files operational procedures

### 6.7.1	General

This clause describes the various operational procedures (e.g. retrieval of a file, retrieval of a file’s metadata, checking the availability of a file) of the stored files for the general clients. The following procedures are common for the functional entities which require to fulfil the operational requirements using the HTTP interface and support the role of both HTTP Client and HTTP Server as defined in annex A of 3GPP TS 24.482 [24].

### 6.7.2	Retrieve the stored file procedure

#### 6.7.2.1	General client procedures

In order to retrieve a file from the functional entity acting as HTTP server, the functional entity in the network, acting as an HTTP client:

1)	shall generate an HTTP GET request as specified in IETF RFC 7230 [22] and IETF RFC 7231 [23] with the following clarifications:

a)	a Request-URI set to an absolute URI identifying the URL of the file being requested to download; and

b)	the Host header field shall be set to the functional entity acting as HTTP server; and

c)	shall include a valid access token in the Authorization header; and

2)	shall send the HTTP GET request as specified for the HTTP client in the network entity in annex A of 3GPP TS 24.482 [24].

On receipt of an HTTP 200 OK response containing the requested file, the HTTP client shall store the file for further processing.

#### 6.7.2.2	General server procedures

On receipt of an HTTP GET request with a Request-URI identifying a file, the functional entity acting as an HTTP server:

1)	shall handle the HTTP request as specified for the HTTP server in annex A of 3GPP TS 24.482 [24] with the following clarifications:

a)	shall validate the access token received in the Authorization header of the request as specified in 3GPP TS 24.482 [24];

b)	if the HTTP client is not allowed to download files due to operator policy, shall return an HTTP 403 Forbidden response; and

c)	if the requested file is not available to download, shall return an HTTP 404 Not Found; and

2)	shall process the HTTP GET request by following the procedures in IETF RFC 7230 [22] and IETF RFC 7231 [23], and shall return a HTTP 200 OK response containing the requested file.

* * * * * * * END CHANGES * * * * * * *