3GPP TSG-CT WG1 Meeting #134-e	C1-221206

# E-Meeting, 17th – 25th February 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.282 | CR | 0303 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Interconnect – MCData Emergency Alert procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | FirstNet, Airbus |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCSMI_CT |  |  |  |  |  | Date: |  |  | 2022-02-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As specified in TS 23.280, when MCVideo systems in different trust domains are interconnected, MCVideo gateway servers are inserted in the path between the controlling and participating or non-controlling functions that are in the different domains. This is done transparently to the involved non-controlling, participating and controlling servers, by use of the PSI of the gateway servers in place of the PSI of the targetted function in the partner system, so that SIP request are routed through the MCVideo gateway server. How those PSIs are determined or configured is out of the scope of the present specification (see existing NOTEs). |  |  |  |  |  |  |  |  |
| Summary of change: |  | When the controlling server determines the PSI of the participating function or of the non-controlling function, NOTEs are added to explain that in case of interconnection, that PSI may be the PSI of the MCVideo gateway server that is the entry point in the the partner MCVideo system in a different trust domain and that the request can be routed through an exit MCVideo gateway server in the primary system. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interconnection of MCVideo systems in different trust domains will not be possible |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*******

* * * First Change * * * *

#### 16.2.2.1	Receipt of a SIP MESSAGE request for emergency notification from the served MCData client

Editor’s note: In the current release, support for emergency groups and emergency group communications may be absent, partial or limited, namely only provided to the extent of facilitating emergency alert functionality.

Upon receipt of a "SIP MESSAGE request for emergency notification for originating participating MCData function", the participating MCData function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP MESSAGE request with a SIP 500 (Server Internal Error) response. The participating MCData function may include a Retry-After header field in the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [4] and skip the rest of the steps;

NOTE 1:	if the SIP MESSAGE request contains an emergency indication set to a value of "true" or an alert indication set to a value of "true", the participating MCData function can, according to local policy, choose to accept the request.

2)	shall determine the MCData ID of the calling user from the public user identity in the P-Asserted-Identity header field of the SIP MESSAGE request, and shall authorise the calling user;

NOTE 2:	The MCData ID of the calling user is bound to the public user identity at the time of service authorisation, as documented in clause 7.3.

3)	if the MCData user is not affiliated with the MCData group as determined by clause 8.3.2.11, shall perform the actions specified in clause 8.3.2.12 for implicit affiliation;

4)	if the actions for implicit affiliation specified in step 3) above were performed but not successful in affiliating the MCData user due to the MCData user already having N2 simultaneous affiliations, shall reject the "SIP MESSAGE request for emergency notification for originating participating MCData function" with a SIP 486 (Busy Here) response with the warning text set to "102 too many simultaneous affiliations" in a Warning header field as specified in clause 4.9 and skip the rest of the steps;

NOTE 3:	N2 is the total number of MCData groups that an MCData user can be affiliated to simultaneously as specified in 3GPP TS 23.282 [2].

NOTE 4:	As this is a request for MCData emergency services, the participating MCData function can choose to accept the request.

5)	shall determine the public service identity of the controlling MCData function associated with the group identity in the received SIP MESSAGE request;

6)	shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6];

7)	shall set the Request-URI of the outgoing SIP MESSAGE request to the public service identity of the controlling MCData function associated with the group identified by the <mcdata-request-uri> element contained in the <mcdatainfo> element containing the <mcdata-Params> element of the application/vnd.3gpp.mcdata-info+xml MIME body in the incoming SIP MESSAGE request;

NOTE 5:	The public service identity can identify the controlling MCData function in the primary MCData system or in a partner MCData system.

NOTE 6:	If the controlling MCData function is in a partner MCData system in a different trust domain, then the public service identity can identify the MCData gateway server that acts as an entry point in the partner MCData system from the primary MCData system.

NOTE 7:	If the controlling MCData function is in a partner MCData system in a different trust domain, then the primary MCData system can route the SIP request through an MCData gateway server that acts as an exit point from the primary MCData system to the partner MCData system

NOTE 8:	How the participating MCData function determines the public service identity of the controlling MCData function serving the target MCData ID or of the MCData gateway server in the partner MCData system is out of the scope of the present document.

NOTE 9:	How the primary MCData system routes the SIP request through an exit MCData gateway server is out of the scope of the present document.

8)	shall copy the contents of the application/vnd.3gpp.mcdata-info+xml MIME body in the received SIP MESSAGE request into an application/vnd.3gpp.mcdata-info+xml MIME body as specified in clause D.1 included in the outgoing SIP MESSAGE request;

9)	shall set the <mcdata-calling-user-id> element of the <mcdatainfo> element containing the <mcdata-Params> element to the MCData ID determined in step 2) above;

10)	if the received SIP MESSAGE request contains an application/vnd.3gpp.mcdata-location-info+xml MIME body as specified in clause D.4, shall copy the contents of the application/vnd.3gpp.mcdata-location-info+xml MIME body in the received SIP MESSAGE request into an application/vnd.3gpp.mcdata-location-info+xml MIME body included in the outgoing SIP MESSAGE request;

11)	shall set the P-Asserted-Identity in the outgoing SIP MESSAGE request to the public user identity in the P-Asserted-Identity header field contained in the received SIP MESSAGE request;

12)	if the received SIP MESSAGE request contains an application/vnd.3gpp.mcdata-info+xml MIME body that contains a <functional-alias-URI> element, shall check if the status of the functional alias is activated for the MCData ID. If the functional alias status is activated, then the participating MCData function shall set the <functional-alias-URI> element of the application/vnd.3gpp.mcdata-info+xml MIME body in the outgoing SIP MESSAGE request to the received value, otherwise shall not include a <functional-alias-URI> element; and

13)	shall send the SIP MESSAGE request as specified in 3GPP TS 24.229 [5].

Upon receipt of a SIP 2xx response in response to the SIP MESSAGE request sent in step 12):

1)	shall generate a SIP 200 (OK) response as specified in 3GPP TS 24.229 [5] with the follow clarifications:

a)	shall include the public user identity received in the P-Asserted-Identity header field of the incoming SIP 2xx response into the P-Asserted-Identity header field of the outgoing SIP 200 (OK) response;

2)	if the procedures of clause 8.3.2.12 for implicit affiliation were performed in the present subclause, shall complete the implicit affiliation by performing the procedures of clause 8.3.2.13; and

3)	shall send the SIP 200 (OK) response to the MCData client according to 3GPP TS 24.229 [5].

Upon receipt of a SIP 4xx, 5xx or 6xx response to the sent SIP MESSAGE request and if the implicit affiliation procedures of clause 8.3.2.12 were invoked in the present subclause, the participating MCData function shall perform the procedures of subclause 8.3.2.14.

* * * End of Changes * * * *