3GPP TSG-CT WG1 Meeting #134-e	C1-221487

# E-Meeting, 17th – 25th February 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.587 | CR | 0229 | rev | - | Current version: | 17.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Lower layers Consideration for the destination Layer-2 ID in determining the PC5 DRX parameters for broadcast and groupcast modes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC_Ph2 |  |  |  |  |  | Date: |  |  | 2022-02-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The following is specified in RAN2 RRC running CR R2-2111417 for NR Sidelink enhancements for Rel-17, related to the DRX configuration: ### 6.3.5 Sidelink information elements SL-DRX-Config-GC-BC field descriptions sl-DRX-GC-BC-PerDest-List List of one or multiple sidelink DRX configurations for groupcast and broadcast communication, which are set based on Destination Layer-2 ID. sl-DRX-GC-Generic Indicates a sidelink DRX configuration, which is applicable to any QoS profile or any Destination Layer-2 ID. SL-DRX-GC-BC-Dest This field indicates the sidelink DRX related parameter(s) for groupcast and broadcast communication, which are set based on Destination Layer-2 ID. It is clear from the above that the Destination Layer-2 ID is taken into consideration by lower layers in calculating some of the DRX configuration parameters, together with the PC5 QoS parameters. The same is confirmed by another RAN2 running CR (R2-2201808) which states: #### 16.9.5.3 Groupcast/Broadcast For groupcast/broadcast, SL DRX is configured commonly among multiple UEs based on QoS profile and Destination L2 ID. Also, stage-2 spec TS 23.287 is aligned with that design, as it states the following in clause 5.9: When the PC5 DRX operation is needed, e.g. based on the NR Tx Profile in case of broadcast or groupcast, the AS layer determines the PC5 DRX parameter values for V2X communication over PC5 reference point, taking into account, e.g., PC5 QoS parameters and/or destination Layer-2 ID provided by the V2X layer. Hence, it is required to align stage-3 spec with that design, as currently in TS 24.587 it is not indicated that the destination Layer-2 ID is also considered in determining the PC5 DRX for broadcast and groupcast modes by lower layers, which can create misunderstanding. This CR proposes to fix that. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Indicating in a NOTE that, destination Layer-2 ID is also used with PC5 QoS parameters for determining the PC5 DRX parameters by lower layers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong understanding that the determination of PC5 DRX parameters depends only on the PC5 QoS parameters and doesn't depend on destination Layer-2 ID, and misalignment with stage-2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

#### 6.1.3.3	Reception of broadcast mode V2X communication over PC5

The UE may be configured by upper layers with one or more destination layer-2 ID(s) for reception of V2X messages over PC5. The receiving UE shall determine the PC5 QoS parameters for this broadcast V2X service in the same way described in clause 6.1.3.2.1.2 and shall determine the NR Tx Profile as described in clause 5.2.3, and shall provide the PC5 QoS parameters, the NR Tx Profile if available and the destination layer-2 ID(s) to lower layers. For each received protocol data unit over PC5, the receiving UE shall check if the destination layer-2 ID of the received protocol data unit matches one of the configured destination Layer-2 IDs. If yes, the UE shall then check whether the protocol data unit type as defined 3GPP TS 38.323 [10] provided by the lower layers for the received packet is set to IP packet or non-IP packet, and pass the protocol data unit to the corresponding upper layer entity.

NOTE:	When the PC5 DRX operation is needed based on the provided NR Tx Profile if any, the lower layers use PC5 QoS parameters and the destination layer-2 ID(s) to determine the PC5 DRX parameter values for reception operation over PC5 reference point.

***** End of changes *****