3GPP TSG-CT WG1 Meeting #134-e	C1-221496

# E-Meeting, 17th – 25th February 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	confidentiality protection of the UE Identity transmitted during the PC3a control protocol procedures for 5G ProSe direct discovery

Spec:	3GPP TS TS 24.554 V1.1.0

Agenda item:	17.2.18

Document for:	Agreement

1. Reason for Change

Currently in TS 24.554 for the 5G ProSe direct discovery procedures running over PC3a interface, the UE Identity sent in the DISCOVERY_REQUEST message is set to the UE's SUPI, i.e. it is not concealed. For example:

6.2.2.2	Announce request procedure initiation

(…)

The UE shall initiate the announce request procedure for open ProSe direct discovery by sending a DISCOVERY_REQUEST message with:

(…)

d)	the UE identity set to the UE's SUPI;

(…)

It was not clear if and how such UE Identity is protected. Stage-2 security spec TS 33.503 resolves this issue where it states the following:

5.2.3.6	The privacy issue in PC3a interface

PC3a interface will be used to transfer the configuration data that is used to perform 5G Direct discovery. According to clause 6.3.1.4 of TS 23.304 [2], the UE Identity is included in the Discovery Request message. Privacy of UE identity is ensured by the confidentiality protection over PC3a interface.

It is proposed to capture this clarification as a NOTE in stage-3 to resolve any ambiguity.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 24.554 V1.1.0.

* * * First Change * * * *

### 6.2.1	Types of 5G ProSe direct discovery procedures

The following PC3a control protocol procedures are defined:

a)	announce request;

b)	monitor request;

c)	discoveree request;

d)	discoverer request;

e)	match report;

f)	network initiated direct discovery update; and

g)	announcing alert request.

In the following descriptions of PC3a control protocol procedures, the terms "request" and "response" refer to the corresponding PC3a control protocol messages, not to the HTTP request or response. The following procedure descriptions use a single PC3a control protocol message for illustration purposes.

NOTE 1:	A single HTTP request message can contain multiple PC3a control protocol requests and a single HTTP response message can contain multiple PC3a control protocol responses.

NOTE 2:	The privacy of the UE identity included in the PC3a control protocol procedures for 5G ProSe direct discovery is ensured by the confidentiality protection of those procedures.

* * * End of Changes * * * *