3GPP TSG-CT WG1 Meeting #134-e	C1-221514

# E-Meeting, 17th – 25th February 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 27.007 | CR | 0768 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to missing TS reference |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | V2XAPP |  |  |  |  |  | Date: |  |  | 2022-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The specification contains missing TS references. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The missing references are corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect reference exists in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.2.1, 16.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 16.2	Commands specific to VAE layer

### 16.2.1	VAE layer configuration in MT +CVAEACT

Table 16.2.1-1: +CVAEACT parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| +CVAEACT=[<state>[,<V2X_UE_id>]] | +CME ERROR: <err> |
| +CVAEACT? | +CVAEACT: <state>,<setup_cfg> |
| +CVAEACT=? | +CVAEACT: <result>[,<service_discovery_data>] |

Description

Set command activates or deactivates the VAE layer support.

When VAE layer support is activated the MT performs the V2X service discovery procedure according to 3GPP TS 24.486 [172] subclause 6.6.

When the MT announces that the MT's network registration status is "not registered, MT is not currently searching an operator to register" to EPS or "unknown" by issuing the unsolicited result code +CEREG to the TE, the TE deactivates the VAE layer support if it is activated.

When the MT announces that the MT's network registration status is registered or attache to 5GS by issuing the unsolicited result code +C5GREG to the TE, the TE deactivates the VAE layer support if it is activated.

If the V2X service discovery procedure fails, then +CME ERROR is returned. Refer to subclause 9.2 for possible <err> values.

The read command returns the current activation states for all the defined PDP contexts.

Read command returns the state of the VAE layer support and the current VAE layer configuration information in the MT according to 3GPP TS 24.486 [172] (e.g. the V2X UE service discovery data).

If the requested state for any specified context cannot be achieved, an ERROR or +CME ERROR response is returned. Extended error responses are enabled by the +CMEE command. If the MT is not PS attached when the activation form of the command is executed, the MT first performs a PS attach and then attempts to activate the specified contexts. If the attach fails then the MT responds with ERROR or, if extended error responses are enabled, with the appropriate failure-to-attach error message. Refer subclause 9.2 for possible <err> values.

Defined values

<state>: integer type; specifies if VAE layer support is to be enabled or disabled in the MT.

0	activate/active VAE layer support in MT. When VAE layer support is disabled MT is not receiving or consuming any VAE layer service

1	deactivate/deactivated VAE layer support in MT

<V2X_UE_id>: string type; indicates the V2X UE identity to be used for enabling the VAE layer support. The <V2X_UE_id> is encoded as the value part of theV2X-UE-id element in 3GPP TS 24.486 [172], subclause 8.5. This parameter shall not be subject to conventional character conversion as per +CSCS.

<result>: string type; indicates the result of enabling the VAE layer. The <result> is encoded as the value part of result element in 3GPP TS 24.486 [172], subclause 8.5. This parameter shall not be subject to conventional character conversion as per +CSCS.

<service_discovery_data>: string type; indicates the service discovery data information when enabling the VAE layer support has been successful. The <service_discovery_data> is encoded as the value part of the service-discovery-data element in 3GPP TS 24.486 [172], subclause 8.5, and each V2X service identifier is encoded as the value part of theV2X-service-id element as specified in subclause 8.5, and each V2X application server address is encoded as the value part of the V2X-app-server-address element as specified in subclause 8.5. This parameter shall not be subject to conventional character conversion as per +CSCS.

Implementation

Optional. This command is only applicable to UEs supporting EPS.

* * * Next Change * * * *

### 16.2.2	VAE layer registration +CVAEREG

Table 16.2.2-1: +CVAEREG parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| +CVAEREG=<V2X_UE_id>,<V2X_service_id>[,<V2X_service_id>[,...]] | +CME ERROR: <err> +CVAEREG: <result> |
| +CVAEREG=? |  |

Description

Set command enables the TE to request the registration of V2X service identifier(s) by using the VAE layer.

When VAE layer support is active the MT performs the V2X UE registration procedure according to 3GPP TS 24.486 [172] subclause 6.2.

When the MT announces that the MT's network registration status is "not registered, MT is not currently searching an operator to register" to EPS or "unknown" by issuing the unsolicited result code +CEREG to the TE, the TE deactivates the VAE layer support if it is activated by using +CVAEACT.

When the MT announces that the MT's network registration status is registered or attache to 5GS by issuing the unsolicited result code +C5GREG to the TE, the TE deactivates the VAE layer support if it is activated by using +CVAEACT.

If this command is issued when VAE layer support is not active, +CME ERROR is returned. Refer to subclause 9.2 for possible <err> values.

Test command returns values supported as a compound value.

Defined values

<V2X_UE_id>: string type; indicates the V2X UE identity to be used for the registration of V2X service identifier(s). The <V2X_UE_id> is encoded as the value part of theV2X-UE-id element in 3GPP TS 24.486 [172], subclause 8.5. This parameter shall not be subject to conventional character conversion as per +CSCS.

<V2X_service_id>: string type; indicates the V2X service identifiers to be registgered. The <V2X_service_id> is encoded as the value part of theV2X-service-id element in 3GPP TS 24.486 [172], subclause 8.5. This parameter shall not be subject to conventional character conversion as per +CSCS.

<result>: string type; indicates the result of the registration of a V2X service which indicates a value either "success" or "fail". The <result> is encoded as the value part of result element in 3GPP TS 24.486 [172], subclause 8.5. This parameter shall not be subject to conventional character conversion as per +CSCS.

Implementation

Optional. This command is only applicable to UEs supporting EPS.

* * * End of Changes * * * *