3GPP TSG-CT WG1 Meeting #134-e	C1-221525

# E-Meeting, 17th – 25th February 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.544 | CR | 0030 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of CoAP Group subscription and notification procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To introduce support for CoAP for SEAL Group Management, it is proposed to add CoAP alternative for Group subscription and notification procedure, corresponding to HTTP and SIP alternatives. |  |  |  |  |  |  |  |  |
| Summary of change: |  | CoAP alternative for Group subscription and notification procedure is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SEAL GM does not support CoAP as required by stage 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.8.1.x (new), 6.2.8.1.x.1 (new), 6.2.8.1.x.2 (new), 6.2.8.1.x.3 (new), 6.2.4.y (new), 6.2.8.2.x (new), 6.2.8.2.x.1 (new), 6.2.8.2.x.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.2.8.1.X	CoAP based procedures

###### 6.2.8.1.X.1	General

CoAP based procedures shall use the mechanisms to observe a resource as specified in IETF RFC 7641 [14].

NOTE:	CoAP "observe" mechanism uses the principle of eventual consistency where an intermediate state change can be lost when UDP is used. If it is critical for the client to receive every change in the resource state (and not just the latest state), TCP can be used to avoid missing notifications.

* * * Next Change * * * *

###### 6.2.8.1.X.2	Create a subscription

The CoAP resource representation defines in clause X.2.1.2 the following observable resources:

a)	VAL Group Documents resource which represents a collection resource of VAL group documents, and which may be observed by the SGM-C with the purpose to be notified when the VAL user is defined as a group member;

b)	Individual VAL Group Document which represents a single VAL group document, and which may be observed by the SGM-C with the purpose to be notified when the VAL group document is modified. By means of the content filters the SGM-C may choose the part of the VAL group document to be included in the notification.

In order to subscribe to changes of an observable resource the SGM-C shall send an extended CoAP GET request with the CoAP URI set to the URI of the observable resource and with the Observe option set to 0 (Register) as specified in IETF RFC 7641 [14].

Upon reception of such an extended CoAP request from SGM-C where the CoAP URI of the request points at an observable resource and with the Observe option set to 0 (Register), the SGM-S:

a)	shall determine the identity of the sender of the received CoAP GET request as specified in clause 6.2.1.X, and:

1)	if the sender is not authorized to fetch the requested resource, shall respond with a 4.03 (Forbidden) response to the CoAP GET request and skip rest of the steps;

b)	shall support handling a CoAP GET request from a SGM-C according to procedures specified in IETF RFC 7252 [12];

c)	shall register the SGM-C as an observer as per IETF RFC 7641 [14]; and

d)	shall send a CoAP 2.05 (Content) response including the current content of the resource and the Observer option with the initial sequence number of the notification.

* * * Next Change * * * *

###### 6.2.8.1.X.3	Delete a subscription

In order to unsubscribe from changes of an observable resource the SGM-C shall send a CoAP GET request matching the CoAP GET request used to create the subscription but with the Observe option set to 1 (Deregister) as specified in IETF RFC 7641 [14].

Upon reception of a CoAP GET that matches an active subscription but with the Observe option set to 1 (Deregister), the SGM-S:

a)	shall perform the steps as for a normal CoAP GET request for the observable resource;

b)	shall deregister the SGM-C as an observer as per IETF RFC 7641 [14]; and

c)	shall send a CoAP 2.05 (Content) response including the current content of the resource and shall not include the Observer option.

* * * Next Change * * * *

##### 6.2.8.2.X	CoAP based procedures

###### 6.2.8.2.X.1	Client procedure

Upon receiving a CoAP 2.05 (Content) response that matches the extended CoAP GET request which initiated the subscription, and which contains the Observe option, the SGM-C:

a)	shall handle the response according to IETF RFC 7641 [14]; and

b)	shall handle the modification according to the observable resource.

* * * Next Change * * * *

###### 6.2.8.2.X.2	Server procedure

In order to send a notification when the resource which is being observed is modified, the SGM-S shall send a CoAP 2.05 (Content) response to SGM-C containing the modified resource and the Observe option according to IETF RFC 7641 [14]. The Content-Format specified in a 2.xx notification shall be the same as the one used in the initial response to the GET request received for the subscription.

* * * End of Changes * * * *