3GPP TSG-CT WG1 Meeting #135-e	C1-222543

# E-Meeting, 6th – 12th April 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.587 | CR | 0234 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Requesting V2XP, ProSeP or both during registration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe, eV2XARC_Ph2 |  |  |  |  |  | Date: |  |  | 2022-03-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1) If the UE includes the UE POLICY PROVISIONING REQUEST message (with the requested UE policies) in the REGISTRATION REQUEST message, retransmission mechanisms specified for the REGISTRATION REQUEST message ensure that the REGISTRATION REQUEST message (carrying the UE POLICY PROVISIONING REQUEST message) is reliably transported to the AMF. Thus, there is no need to retransmit the UE POLICY PROVISIONING REQUEST message transported in REGISTRATION REQUEST message. However, total duration of the procedure needs to be adjusted to maximum duration of the registration procedure and the PCF needs to provide the network's response to UE's request for V2XP, ProSeP or both reliabily, i.e. using network-requested UE policy management procedure. 2) Upon reception of a UE STATE INDICATION message with UE's request for V2XP, ProSeP or both, or UE POLICY PROVISIONING REQUEST message, if the PCF determines that the UE is authorized to obtain the requested UE policies but the requested UE policies are already up-to-date in the UE, then it is not clear which the UPDS cause to use as: - #32 and #34 are not appropriate since the service option is supported and working. - #35, and #91 - #111 are not appropriate since the UE STATE INDICATION or UE POLICY PROVISIONING REQUEST message is correct from protocol point of view. - #31 is possible but does not convey the actual reason for rejection (i.e. requested UE policies are already up-to-date in the UE). |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) If the UE includes the UE POLICY PROVISIONING REQUEST message (with the requested UE policies) in the REGISTRATION REQUEST message, the UE POLICY PROVISIONING REQUEST message is not retransmitted, the total duration of the procedure is adjusted to maximum duration of the registration procedure and network-requested UE policy management procedure is used to provide errors. 2) Upon reception of a UE STATE INDICATION message with UE's request for V2XP, ProSeP or both or a UE POLICY PROVISIONING REQUEST message, if the PCF determines that the UE is authorized to obtain the requested UE policies but the requested UE policies are already up-to-date in the UE, then the PCF uses UPDS cause set to "no changes of UE policies needed". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1) If the network does not support ProSe and V2X, the UE will unnecessarily send five times the UE POLICY PROVISIONING REQUEST. 2) not clear which cause to use upon reception of a UE STATE INDICATION message with UE's request for V2XP, ProSeP or both, or UE POLICY PROVISIONING REQUEST message, if the PCF determines that the UE is authorized to obtain the requested UE policies but the requested UE policies are already up-to-date in the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.4, 5.3.2.6,8.3.1, 10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** change *****

#### 5.3.2.4	UE-requested V2X policy provisioning procedure not accepted by the network

Upon receipt and rejecting of the UE POLICY PROVISIONING REQUEST message not sent during registration, the PCF shall create a UE POLICY PROVISIONING REJECT message.

The PCF shall set the UPDS cause IE of the UE POLICY PROVISIONING REJECT message to indicate reason for rejecting the UE-requested V2X policy provisioning procedure.

The UPDS cause IE typically indicates one of the following UPDS cause values:

#31	request rejected, unspecified;

#32	service option not supported;

#34	service option temporarily out of order;

#35	PTI already in use;

#XX	no changes of UE policies needed; or

#95 – 111	protocol errors.

The PCF shall transport the UE POLICY PROVISIONING REJECT message to the UE via the AMF using the procedure specified in 3GPP TS 23.502 [4].

Upon receipt of the UE POLICY PROVISIONING REJECT message, the UE shall stop timer T5040.

Upon receipt and rejecting of the UE POLICY PROVISIONING REQUEST message sent during registration, the PCF shall behave as described in clause D.2.1 of 3GPP TS 24.501 [6].

If the UE POLICY PROVISIONING REQUEST message was sent during registration, then upon reception of the MANAGE UE POLICY COMMAND message including the same PTI as included in the UE POLICY PROVISIONING REQUEST message, the UE shall:

a)	stop timer T5040;

b)	if the MANAGE UE POLICY COMMAND message includes the Requested UE policies UPDS cause IE, consider that UE's request for UE policies in the Requested UE policies IE of the UE POLICY PROVISIONING REQUEST message is rejected; and

c)	handle the MANAGE UE POLICY COMMAND message as specified in clause D.2.1.

***** change *****

#### 5.3.2.6	Abnormal cases on the UE

The following abnormal cases can be identified:

a)	T5040 expired.

If the UE POLICY PROVISIONING REQUEST message was transported in the REGISTRATION REQUEST message, the UE shall abort the procedure and release the allocated PTI.

If the UE POLICY PROVISIONING REQUEST message was not transported in the REGISTRATION REQUEST message, the UE shall, on the first expiry of the timer T5040, retransmit the UE POLICY PROVISIONING REQUEST message and shall reset and start timer T5040. This retransmission is repeated four times, i.e. on the fifth expiry of timer T5040, the UE shall abort the procedure and release the allocated PTI.

***** change *****

### 8.3.1	UPDS cause

The purpose of the UPDS cause information element is to indicate the reason why a UPDS request is rejected.

The UPDS cause information element is coded as shown in figure 8.3.1.1 and table 8.3.1.1.

The UPDS cause is a type 3 information element with 2 octets length.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UPDS cause IEI |  |  |  |  |  |  |  | octet 1 |
| Cause value |  |  |  |  |  |  |  | octet 2 |

Figure 8.3.1.1: UPDS cause information element

Table 8.3.1.1: UPDS cause information element

| Cause value (octet 2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 |  | Request rejected, unspecified |
| 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |  | Service option not supported |
| 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 |  | Service option temporarily out of order |
| 0 | 0 | 1 | 0 | 0 | 0 | 1 | 1 |  | PTI already in use |
| x | x | x | x | x | x | x | x |  | No changes of UE policies needed |
| 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |  | Semantically incorrect message |
| 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |  | Invalid mandatory information |
| 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 |  | Message type non-existent or not implemented |
| 0 | 1 | 1 | 0 | 0 | 0 | 1 | 0 |  | Message type not compatible with the protocol state |
| 0 | 1 | 1 | 0 | 0 | 0 | 1 | 1 |  | Information element non-existent or not implemented |
| 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 |  | Conditional IE error |
| 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 |  | Protocol error, unspecified |
| Any other value received by the UE shall be treated as 0010 0010, "service option temporarily out of order". Any other value received by the network shall be treated as 0110 1111, "protocol error, unspecified". |  |  |  |  |  |  |  |  |  |

***** change *****

## 10.2	Timers of provisioning of parameters for V2X configuration procedures

Timers of provisioning of parameters for V2X configuration are shown in table 10.2.1.

Table 10.2.1: Timers of provisioning of parameters for V2X configuration – UE side

| TIMER NUM. | TIMER VALUE | CAUSE OF START | NORMAL STOP | ON THE 1st, 2nd, 3rd, 4th EXPIRY |  |
| --- | --- | --- | --- | --- | --- |
| T5040 | NOTE 4 | Transmission of UE POLICY PROVISIONING REQUEST message | MANAGE UE POLICY COMMAND with UE policies for V2X communication or UE POLICY PROVISIONING REJECT message received | NOTE 5 |  |
| T5041 | NOTE 1 | Start using the new UE policies for V2X communication over PC5 received in MANAGE UE POLICY COMMAND message | Stop using the old UE policies for V2X communication over PC5 | Initiate the UE-requested V2X policy provisioning procedure (NOTE 3) |  |
| T5042 | NOTE 2 | Start using the new UE policies for V2X communication over Uu received in MANAGE UE POLICY COMMAND message | Stop using the old UE policies for V2X communication over Uu | Initiate the UE-requested V2X policy provisioning procedure (NOTE 3) |  |
| NOTE 1: The value of this timer is the validity timer value which is one of the configuration parameters for V2X communication over PC5 (see clause 5.2) and it is specified in 3GPP TS 24.588 [7] clause 5.3. NOTE 2: The value of this timer is the validity timer value which is one of the configuration parameters for V2X communication over Uu (see clause 5.2) and it is specified in 3GPP TS 24.588 [7] clause 5.4. NOTE 3: The timers expire only once. NOTE 4: If the UE POLICY PROVISIONING REQUEST message is transported in the REGISTRATION REQUEST message, the value of this timer is the value of timer T3510 as specified in 3GPP TS 24.501 [6] increased with 1s, otherwise the value of this timer is 16s. NOTE 5: If the UE POLICY PROVISIONING REQUEST message is transported in the REGISTRATION REQUEST message, the timer expire only once and expiration of the timer results into procedure being considered complete and UE's request for UE policies in the Requested UE policies IE considered as rejected, otherwise expiration of the timer results into retransmission of UE POLICY PROVISIONING REQUEST message. |  |  |  |  |  |