3GPP TSG-CT WG1 Meeting #136-e	C1-22xxxx

# E-Meeting, 12th – 20th May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.554 | CR | 0011 | rev | 2 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General parts for procedures for PC8 interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Qualcomm Incorporated, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2022-05-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SA3 specified for stage-2 requirements for security for 5G ProSe Communication via 5G ProSe UE-to-Network Relay, over user plane. Those stage-2 requirements are not addressed in stage-3 yet. |  |  |  |  |  |  |  |  |
| Summary of change: |  | General parts for procedures specified for stage-2 requirements for security for 5G ProSe Communication via 5G ProSe UE-to-Network Relay, over user plane. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage-2 requirements for security for 5G ProSe Communication via 5G ProSe UE-to-Network Relay, over user plane not addressed in stage-3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 8.2.x (new), 8.2.X.1 (new), 8.2.X.1.1 (new), 8.2.X.1.2 (new), 8.2.X.1.2.1 (new), 8.2.X.2 (new), 8.2.X.2.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 2: - subclause 8.2.X.1.2.2 removed from scope of this CR, to avoid conflict with TS 24.554 CR 0085 (C1-223551 and its revisions) |  |  |  |  |  |  |  |  |

***** change *****

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5G DDNMF	5G Direct Discovery Name Management Function

5G ProSe	5G Proximity-based Services

DN	Data Network

DUCK	Discovery User Confidentility Key

DUIK	Discovery User Integrity Key

DUSK	Discovery User Scrambling Key

GFBR	Guaranteed Flow Bit Rate

LSB	Least Significant 8 Bits

MSB	Most Significant 8 Bits

MFBR	Maximum Flow Bit Rate

MIC	Message Integrity Check

NCGI	NG-RAN Cell Global ID

PC5 LINK-AMBR	PC5 Link Aggregated Bit Rate

PDUID	ProSe Discovery UE ID

PKMF	ProSe Key Management Function

PQI	PC5 5QI

ProSeP	5G ProSe Policy

PRUK	Prose Remote User Key

PSDK	Public Safety Discovery Key

RQI	Reflective QoS Indication

RPAUID	Restricted ProSe Application User ID

RSC	Relay Service Code

TTL	Time-To-Live

***** change *****

# 8.2.X	5G ProSe security procedures over PC8 interface

## 8.2.X.1	General

### 8.2.X.1.1	Transport protocol for PC8 messages

The UE and 5G PKMF shall use HTTP 1.1 as specified in IETF RFC 7230 [3] and IETF RFC 7231 [4] as the transport protocol for PC8 messages over the PC8 interface. The PC8 messages described here shall be included in the body of either an HTTP request message or an HTTP response message.

### 8.2.X.1.2	Handling of UE-initiated procedures

#### 8.2.X.1.2.1	General

The following rules apply for UE-initiated procedures:

a)	the UE initiates 5G ProSe transactions with an HTTP request message containing the PC8 request(s);

b)	the 5G PKMF responds to the requests with an HTTP response message containing the PC8 response(s) for the PC8 request(s); and

c)	HTTP POST methods are used for 5G ProSe procedures over PC8 interface.

The UE may use UE local configuration or URSP, as defined in 3GPP TS 24.526 [5], to establish a PDU session for reaching the HPLMN 5G PKMF:

a)	if a PDU session for reaching the HPLMN 5G PKMF is not established yet, the UE shall establish the PDU session for reaching the HPLMN 5G PKMF and shall send the HTTP request message via the PDU session for reaching the HPLMN 5G PKMF; and

b)	if a PDU session for reaching the HPLMN 5G PKMF is already established (e.g., either due to other 5G ProSe feature or due to other application), the UE shall send the HTTP request message via the PDU session for reaching the HPLMN 5G PKMF.

## 8.2.X.2	Procedures

### 8.2.X.2.1	Types of 5G ProSe procedures over PC8 interface

The following procedures are defined:

a)	5G ProSe UE-to-network relay discovery security parameters request procedure;

b)	5G ProSe remote user key request procedure; and

c)	Key request procedure.

In the following descriptions of 5G procedures over PC8 interface, the terms "request" and "response" refer to the corresponding PC8 messages, not to the HTTP request or response. The following procedure descriptions use a single PC8 message for illustration purposes.

The 5G ProSe procedures over PC8 interface shall be integrity protected and confidentiality protected using the security procedures using GBA or AKMA, see clause 5.2.5.3 and clause 5.2.5.4 in 3GPP TS 33.503 [34].

NOTE 1:	A single HTTP request message can contain multiple PC8 requests and a single HTTP response message can contain multiple PC8 responses.

NOTE 2:	The privacy of the UE identity included in the PC8 messages of 5G ProSe procedures over PC8 interface is ensured by the confidentiality protection of those procedures.