3GPP TSG-CT WG1 Meeting #139	C1-227010

# Toulouse, France, 14th – 18th November 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 4702 | rev | 3 | Current version: | 18.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NAS operation for network slice-based random access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Apple, Huawei, HiSilicon, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NRslice |  |  |  |  |  | Date: |  |  | 2022-11-18 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | How the NAS operates for NSAG-aware random access needs to be specified. The stage 2 CR having formal linkage to this CR specifies the foloiwng requirement. For Network Slice based Random Access, different Random Access resources may be assigned to different NSAG(s). The UE determines Random Access configuration among NSAGs that are published in SIB for Random Access and that are associated to the S-NSSAIs triggering the access. If the signalling transaction triggering the access attempt is related to more than one network slice, and the S-NSSAIs of these network slices are associated with more than one NSAG for Random Access, the NSAG with the highest priority is selected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The NAS layer provides one or more S-NSSAIs associated with an access attempt. Backward compatibility analysis Backward compatible as it only impacts internal operation in the UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Rel-17 WI NRslice is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 4.6.2.6	Provision of NSAG information to lower layers

NSAG information provided by the network and stored in the UE includes a list of NSAGs each of which contains:

a)	an NSAG ID;

b)	a list of S-NSSAI(s), which are associated with the NSAG and shall be part of the configured NSSAI;

c)	optionally a list of TAIs in which the NSAG is valid. If it is not provided by the network, the NSAG is valid in the PLMN which has sent the NSAG information; and

d)	a priority value that is associated with the NSAG.

The UE NAS layer shall provide the lower layers with:

a)	the most recent NSAG information stored in the UE (see subclause 4.6.2.2);

b)		the allowed NSSAI or the requested NSSAI for the purpose of network slice-based cell reselection (see 3GPP TS 23.501 [8]); and

c)	one or more S-NSSAIs related to an access attempt for the purpose of network slice-based random access, when the access attempt is made by the UE in 5GMM-IDLE mode or 5GMM-CONNECTED mode with RRC inactive indication, determined as follows:

i)	if the UE is in 5GMM-IDLE mode, S-NSSAI(s) triggering the initial NAS message (requested NSSAI (if any) in case of the REGISTRATION REQUEST message and S-NSSAI(s) associated with all the PDU sessions included in the Uplink data status IE (if any), PDU session status IE (if any), and Allowed PDU session status IE (if any) in case of the (CONTROL PLANE) SERVICE REQUEST message); or

ii)	if the UE is in 5GMM-CONNECTED mode with RRC inactive indication and the transition to 5GMM-CONNECTED mode is needed due to:

A)	an uplink user data packet to be sent for a PDU session with suspended user-plane resources or an UL NAS TRANSPORT which carries a 5GSM message for a PDU session associated with an S-NSSAI, the S-NSSAI associated with the PDU session;

B)	a trigger to initiate a registration procedure or a service request procedure, S-NSSAI(s) triggering the initial NAS message (requested NSSAI in case of the REGISTRATION REQUEST (if any) message and S-NSSAI(s) associated with all the PDU sessions included in the Uplink data status IE (if any), PDU session status IE (if any), and Allowed PDU session status IE (if any) in case of the (CONTROL PLANE) SERVICE REQUEST message); or

C)	other reason than those specified in bullets A) and B), the allowed NSSAI.

* * * End of Changes * * * *