3GPP TSG-CT WG1 Meeting #141-e	C1-232441

# E-Meeting, 17th – 21st April 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.581 | CR | 0096 | rev | - | Current version: | 14.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction and clarification to transmission control SSRC in MCVideo – media plane |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Research America |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCImp-MCVIDEO-CT |  |  |  |  |  | Date: |  |  | 2023-04-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The SSRC of granted transmission participant is sent in the both Transmission Granted message and Media transmission notification. If the transmission is granted for implicit transmission request as a part of call initialization then the SSRC of granted transmission participant value needs to be aware of before populating the field. This SSRC field value should be same as that of the SSRC of the transmission participant in the Transmission Control messages to identify the sender of the Transmission Control messages. In this CR, a unqiue value of SSRC’s are generated and assigned to each transmission participant are communicated during call establishment. This value will be used in the Transmission Control messages. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New "mc_txp_ssrc" fmtp attribute is introduced to provide the SSRC value generated by the transmission control server to be used by the transmission participants in Transmission Control messages. The Semantics and Syntax are added for the newly added “mc_txp_ssrc" fmtp attribute |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will not be clear which value to be used in the Transmission Control messages for the transmission participant identifier (SSRC) and can result into implementation errors. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.2.2, 12.1.2.3, new (14.2.X, 14.3.Y) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 12.1.2.2	Semantics

In an SDP offer and answer, the "mc_queueing" fmtp attribute is used to indicate support of the Transmission Request message queueing mechanism, as defined in the present specification.

In an SDP offer, the "mc_priority" fmtp attribute indicates (using an integer value between '1' and '255') the maximum transmission priority that the offerer requests to be used with Transmission Request messages sent by the offerer. In an SDP answer, the attribute parameter indicates the maximum priority level that the answerer has granted to the offerer. The value must be equal or less than the value provided in the associated SDP offer.

NOTE 1:	If the "mc_priority" fmtp attribute is not used within an SDP offer or answer, a default priority value is assumed.

In an SDP offer, the "mc_reception_priority" fmtp attribute indicates (using an integer value between '1' and '255') the maximum reception priority that the offerer requests to be used with Reception Request messages sent by the offerer. In an SDP answer, the attribute parameter indicates the maximum reception priority level that the answerer has granted to the offerer. The value must be equal or less than the value provided in the associated SDP offer.

NOTE 2:	If the "mc_reception_priority" fmtp attribute is not used within an SDP offer or answer, a default reception priority value is assumed.

In an SDP offer, the "mc_granted" fmtp attribute parameter indicates that the offerer supports the procedure where the answerer indicates, using the fmtp attribute in the associated SDP answer, that the permission to transmit has been granted to the offerer.

NOTE 3:	When the "mc_granted" fmtp attribute is used in an SDP offer, it does not indicate an actual request for the media transmission. The SDP "mc_implicit_request" fmtp attribute can be used to request the media transmission. In an SDP answer, the attribute indicates that the permission to Transmission has been granted to the offerer.

NOTE 4:	Once the offerer has been granted the permission to Transmission, the offerer can perform media transmission until it receives a Transmission Revoked message, or until the offerer itself ends the media transmission by sending a Transmission end request message, as described in the present specification.

In an SDP offer, the "mc_implicit_request" fmtp attribute indicates that the offerer implicitly requests for media transmission (without the need to send a Transmission Request message). In an SDP answer, the attribute parameter indicates that the answerer has accepted the implicit Transmission Request. Once the answerer grants the permission to Transmission to the offerer, the answerer will send a Transmission Granted message.

NOTE 5:	The usage of the "mc_implicit_request" fmtp attribute in an SDP answer does not mean that the answerer has granted the permission to Transmission to the offerer, only that the answerer has accepted the implicit Transmission Request.

In an SDP offer, the "mc_txp_ssrc" fmtp attribute indicates the ssrc of the transmission participant (server generated and assigned ssrc value to be used) that the offerer requests to be used with Transmission Control messages by the receiving transmission participant. In SDP answer, the "mc_txp_ssrc" fmtp attribute indicates the ssrc of the transmission participant (server generated and assigned ssrc value to be used) that the answerer requests to be used with Transmission Control messages by the receiving transmission participant.

* * * Next Change * * * *

#### 12.1.2.3	Syntax

Table 12.1.2.3-1: SDP "fmtp" attribute for the MCVideo media plane control channel

fmtp-attr-mpcp      = "a=fmtp:" "MCVideo" SP attr-param-list

attr-param-list     =  attr-param *(COLON attr-param)

attr-param          =  mc_queueing / mc_priority / mc_reception_priority / mc_granted / mc_implicit_request / mc_txp_ssrc

mc_queueing          =  "mc_queueing"

mc_priority         =  "mc_priority=" 1*2(DIGIT)

mc_reception_priority = "mc_reception_priority=" 1*2(DIGIT)

mc_granted          =  "mc_granted"

mc_implicit_request =  "mc_implicit_request"

mc_txp_ssrc      =  "mc_txp_ssrc=" 1*(DIGIT)

Editor's Note:	IANA registration may be required.

* * * Next Change * * * *

### 14.2.X	"mc_txp_ssrc" fmtp attribute

When inviting an MCVideo user to an MCVideo session, the MCVideo server shall assign unique SSRC identifier to be used in the media transmission control messages by an invited MCVideo user and shall include in the "mc_txp_ssrc" fmtp attribute with the assigned unique SSRC identifier of an invited MCVideo user in the SDP offer.

* * * Next Change * * * *

### 14.3.Y	"mc_txp_ssrc" fmtp attribute

The MCVideo server shall assign unique SSRC identifier to be used in the media transmission control messages by an inviting MCVideo user of an MCVideo session and shall include in the "mc_txp_ssrc" fmtp attribute with the assigned unique SSRC identifier of an inviting MCVideo user in the SDP answer.

* * * End of Changes * * * *