3GPP TSG-CT WG1 Meeting #141e	C1-233910

# Online 17– 21 April 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.554 | CR | 0297 | rev | 3 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update 5G ProSe link modification messages for the L3 UE-to-UE relay reselection procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Following updates were proposed in CR C1-232735: - Corresponding IEs added to the link modification messages - New PROSE DIRECT LINK MODIFICATION ACK message is added. The definition of IE “Initiating UE IPv6 address” is missing, which is included when Relay sends LMR to the target end UE, to share the source end UE's IP addr. (step 6, 23.304, 6.7.4.3). Target end UE also includes this IE but on the LM Accept so the correction is made accordingly. A few other clean up and corrections are proposed for clarity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In this CR, the definition of IE “Initiating UE IPv6 address” is missing, which is included, and few clean ups and corrections are proposed for clarity in addition to what is agreed in C1-232735. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 updated of 5G ProSe UE-to-UE relay reselection procedure is not implemented |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.6.1,10.3.6.a, 10.3.6.b, 10.3.6.c, 10.3.6.d, 10.3.7.1, 10.3.7.a, 10.3.7.b, 10.3.7.c, 10.3.7.d,10.3.x.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

#### 10.3.6.1	Message definition

This message is sent by the UE to another peer UE to initiate the direct link modification procedure. See table 10.3.6.1.1.

Message type:	PROSE DIRECT LINK MODIFICATION REQUEST

Significance:	dual

Direction:	UE to peer UE

Table 10.3.6.1.1: PROSE DIRECT LINK MODIFICATION REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE DIRECT LINK MODIFICATION REQUEST message identity | ProSe PC5 signalling message type 11.3.1 | M | V | 1 |
|  | Sequence number | Sequence number 11.3.2 | M | V | 1 |
|  | Link modification operation code | Link modification operation code 11.3.19 | M | V | 1 |
|  | QoS flow descriptions | PC5 QoS flow descriptions 11.3.5 | M | LV-E | 5-65537 |
| 7C | QoS rules | PC5 QoS rules 11.3.29 | O | TLV-E | 7-65538 |
| XY | Source end UE info | User info ID 11.3.xz | O | TLV | 3-257 |
| XZ | Target end UE info | User info ID 11.3.xz | O | TLV | 3-257 |
| ZZ | Target end UE layer-2 ID | Layer-2 ID 11.3.25 | O | TV | 4 |
| AA | Relay reselection indication | Relay reselection indication 11.3.xx | O | TV | 1 |
| AB | List of candidates U2U relay UE info | User info ID 11.3.xz | O | TLV | 3-257 |
| AC | Initiating end UE IPv6 address | Link local IPv6 address 11.3.7 | O | TLV | 17 |
| AD | List of target end UE IPv6 address | Link local IPv6 address 11.3.7 | O | TLV | 17-257 |
| AF | List of candidates U2U relay UE layer-2 ID | Layer-2 ID 11.3.25 | O | TV | 4-257 |

#### 10.3.6.a	Relay reselection indication

The UE shall include this IE if the 5G ProSe direct link modification request is to trigger UE-to-UE relay reselection, and if:

a)	the UE acts as a source 5G ProSe end UE and the 5G ProSe direct link is between the source 5G ProSe end UE and the 5G ProSe UE-to-UE relay UE to indicate the Relay reselection indication to the 5G ProSe UE-to-UE relay UE; or

b)	the UE acts as a 5G ProSe UE-to-UE relay UE and the 5G ProSe direct link is between the 5G ProSe UE-to-UE relay UE and the target 5G ProSe end UE to indicate the Relay reselection indication to the target 5G ProSe end UE.

#### 10.3.6.b	List of candidates U2U relay UE user info

The UE shall include this IE if relay reselection is indicated, and if:

a)	the UE acts as a source 5G ProSe end UE and the 5G ProSe direct link is between the source 5G ProSe end UE and the 5G ProSe UE-to-UE relay UE to indicate the user info ID of the candidate 5G ProSe UE-to-UE relay UEs; or

b)	the UE acts as a 5G ProSe UE-to-UE relay UE and the 5G ProSe direct link is between the 5G ProSe UE-to-UE relay UE and the target 5G ProSe end UE to indicate the user info ID of the candidate 5G ProSe UE-to-UE relay UEs.

#### 10.3.6.c	List of target UE IPv6 address

The UE shall include this IE if relay reselection is indicated, and if:

a)	the UE acts as a source 5G ProSe end UE and the 5G ProSe direct link is between the source 5G ProSe end UE and the 5G ProSe UE-to-UE relay UE to indicate the list of link local IPv6 addresses of the target 5G ProSe end UEs.

#### 10.3.6.d	List of candidates U2U relay UE layer-2 ID

The UE may include this IE if relay reselection is indicated, and if:

a)	the UE acts as a source 5G ProSe end UE and the 5G ProSe direct link is between the source 5G ProSe end UE and the 5G ProSe UE-to-UE relay UE to indicate the layer-2 ID of the 5G ProSe UE-to-UE relay UE; or

b)	the UE acts as a 5G ProSe UE-to-UE relay UE and the 5G ProSe direct link is between the 5G ProSe UE-to-UE relay UE and the target 5G ProSe end UE to indicate the layer-2 ID of the 5G ProSe UE-to-UE relay UE.

#### 10.3.6.e	Initiating end UE IPv6 address

The UE shall include this IE if relay reselection is indicated, and if:

a)	the UE acts as a 5G ProSe UE-to-UE relay UE and the 5G ProSe direct link is between the 5G ProSe UE-to-UE relay UE and the target 5G ProSe end UE to indicate the link local IPv6 address of the source end UE.

***** Next change *****

#### 10.3.7.1	Message definition

This message is sent by the UE to another peer UE to indicate that the link modification request is accepted. See table 10.3.7.1.1.

Message type:	PROSE DIRECT LINK MODIFICATION ACCEPT

Significance:	dual

Direction:	UE to peer UE

Table 10.3.7.1.1: PROSE DIRECT LINK MODIFICATION ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE DIRECT LINK MODIFICATION ACCEPT message identity | ProSe PC5 signalling message type 11.3.1 | M | V | 1 |
|  | Sequence number | Sequence number 11.3.2 | M | V | 1 |
| 79 | QoS flow descriptions | PC5 QoS flow descriptions 11.3.5 | O | TLV-E | 6-65538 |
| 7C | QoS rules | PC5 QoS rules 11.3.29 | O | TLV-E | 7-65538 |
| XY | Source end UE info | User info ID 11.3.xz | O | TLV | 3-257 |
| XZ | Target end UE info | User info ID 11.3.xz | O | TLV | 3-257 |
| BA | Relay reselection indication | Relay reselection indication 11.3.xx | O | TV | 1 |
| BB | New 5G ProSe UE-to-UE relay UE info | User info ID 11.3.xz | O | TLV | 3-257 |
| BC | Initiating end UE IPv6 address | Link local IPv6 address 11.3.7 | O | TLV | 17 |
| BD | Target end UE IPv6 address | Link local IPv6 address 11.3.7 | O | TLV | 17 |

#### 10.3.7.a	Relay reselection indication

The UE shall include this IE if the 5G ProSe direct link modification request for UE-to-UE relay reselection is accepted, and if:

a)	the UE acts as a target 5G ProSe end UE and the 5G ProSe direct link is between the target 5G ProSe end UE and the 5G ProSe UE-to-UE relay UE to indicate the Relay reselection indication to the 5G ProSe UE-to-UE relay UE; or

b)	the UE acts as a 5G ProSe UE-to-UE relay UE and the 5G ProSe direct link is between the 5G ProSe UE-to-UE relay UE and the source 5G ProSe end UE to indicate the Relay reselection indication to the source 5G ProSe end UE.

#### 10.3.7.b	New 5G ProSe UE-to-UE relay UE info

The UE shall include this IE if relay reselection is indicated, and if:

a)	the UE acts as a target 5G ProSe end UE and the 5G ProSe direct link is between the target 5G ProSe end UE and the 5G ProSe UE-to-UE relay UE to indicate the user info ID of the new 5G ProSe UE-to-UE relay UE; or

b)	the UE acts as a 5G ProSe UE-to-UE relay UE and the 5G ProSe direct link is between the 5G ProSe UE-to-UE relay UE and the source 5G ProSe end UE to indicate the user info ID of the new 5G ProSe UE-to-UE relay UEs.

#### 10.3.7.c	Initiating end UE IPv6 address

The UE shall include this IE if relay reselection is indicated, and if:

a)	the UE acts as a target 5G ProSe end UE and the 5G ProSe direct link is between the target 5G ProSe end UE and the 5G ProSe UE-to-UE relay UE to indicate the link local IPv6 address of the source 5G ProSe end UE; or

#### 10.3.7.d	Target end UE IPv6 address

The UE shall include this IE if relay reselection is indicated, and if:

a)	the UE acts as a target 5G ProSe end UE and the 5G ProSe direct link is between the target 5G ProSe end UE and the 5G ProSe UE-to-UE relay UE to indicate the link local IPv6 address of the target 5G ProSe end UE for communication via the newly selected 5G ProSe UE-to-UE relay UE; or

b)	the UE acts as a 5G ProSe UE-to-UE relay UE and the 5G ProSe direct link is between the 5G ProSe UE-to-UE relay UE and the source 5G ProSe end UE to indicate the link local IPv6 address of the target end UE for communication via the newly selected 5G ProSe UE-to-UE relay UE.

***** Next change *****

#### 10.3.x.1	Message definition

This message is sent by the UE to another peer UE to ack the direct link modification procedure. See table 10.3.x.1.1.

Message type:	PROSE DIRECT LINK MODIFICATION ACK

Significance:	dual

Direction:	UE to peer UE

Table 10.3.x.1.1: PROSE DIRECT LINK MODIFICATION ACK message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE DIRECT LINK MODIFICATION ACK message identity | ProSe PC5 signalling message type 11.3.1 | M | V | 1 |
|  | Sequence number | Sequence number 11.3.2 | M | V | 1 |
|  | Relay reselection indication | Relay reselection indication 11.3.xx | M | TV | 1 |
|  | Initiating end UE IPv6 address | Link local IPv6 address 11.3.7 | M | TV | 17 |
|  | Target end UE IPv6 address | Link local IPv6 address 11.3.7 | M | TV | 17 |

***** End of changes *****