3GPP TSG-CT WG1 Meeting #142	C1-234010

# Bratislava 22– 26 May 2023

Source:	Xiaomi

Title:	Unicast mode V2X communication procedures for ranging and sidelink positioning

Spec:	3GPP TS 24.514 v0.1.0

Agenda item:	18.2.23

Document for:	Agreement

1. Introduction

This p-CR provides content for Unicast mode V2X communication procedures for ranging and sidelink positioning.

2. Reason for Change

For V2X capable UEs discovery over PC5 interface, it is agreed in stage 2 as specified in clause 5.2&6.4.2.3 in 3GPP TS 23.586 to support Ranging and sidelink positioning UE discovery with V2X capable UE using the procedures of Layer-2 link establishment over PC5 reference point in clause 6.3.3 of TS 23.287, so the related procedure needs to be defined.

To transport RSPP over PC5 for V2X capable UEs, it is agreed in stage 2 as specified in clause 5.3 in 3GPP TS 23.586 that the procedures defined in clause 6.3 of TS 23.287 are used to support the corresponding communicated modes and the RSPP message is carried using "Non-IP" PDCP SDU type for V2X capable UEs as defined in clause 6.1.1 of TS 23.287.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.514 v0.1.0.

* * * First Change * * * *

6.2.3	V2X capable UEs discovery

This clause describes the procedures for ranging and sidelink positioning UE discovery with V2X capable UEs over PC5 interface.

To perform ranging and sidelink positioning UE discovery procedure with V2X capable UE over PC5 interface, the procedure of V2X direct link establishment procedure for ranging and sidelink positioning in clause 7.2.2.x2 is applied.

* * * Next Change * * * *

7.2.1	General

Depending on type of the UE (e.g. V2X capable or 5G ProSe capable), V2X communication procedures as defined in TS 24.587 [4] or 5G ProSe direct communication procedures as defined in 3GPP TS 24.554 [6] are used for ranging and sidelink positioning communication over PC5.

For the RSPP transport between UEs over PC5, the following modification is applied:

a)	For V2X capable UEs, V2XP is used to determine the corresponding transport configurations for the RSPP signalling. The V2X service identifier shall take the value(s) defined for "ranging and sidelink positioning Protocol".

b)	For 5G ProSe capable UEs, ProSeP is used to determine the corresponding transport configuration for the RSPP signalling. The ProSe identifier shall take the value(s) defined for "ranging and sidelink positioning Protocol".

NOTE:	For a UE with both V2X capability and 5G ProSe capability, separate RSPP transport links per the capability are used, i.e., the RSPP transport link over V2X communication is independent with the RSPP transport link over 5G ProSe direct communication.

The RSP layer supports broadcast mode, groupcast mode, and unicast mode PC5 communication depending on the policy and parameter configuration in the UE.

"Non-IP" layer-3 protocol data unit type for V2X capable UEs and "Unstructured" layer-3 protocol data unit type for 5G ProSe capable UEs are used for the transport of RSPP payload.

* * * Next Change * * * *

7.2.2	Unicast mode ranging and sidelink positioning direct communication over PC5

7.2.2.x2	Unicast mode communication over PC5 with V2X capable UEs

This clause describes the PC5 signalling protocol procedures between two UEs for unicast mode of V2X communication for ranging and sidelink positioning. The following PC5 signalling protocol procedures are supported:

a)	PC5 unicast link establishment procedure with V2X capable UEs as specified in 3GPP TS 24.587 [4] clause 6.1.2.2;

b)	PC5 unicast link modification procedure with V2X capable UEs as specified in 3GPP TS 24.587 [4] clause 6.1.2.3;

c)	PC5 unicast link release with procedure V2X capable UEs as specified in 3GPP TS 24.587 [4] clause 6.1.2.4;

d)	PC5 unicast link identifier update procedure with V2X capable UEs as specified in 3GPP TS 24.587 [4] clause 6.1.2.5; and

e)	PC5 unicast link keep-alive procedure with V2X capable UEs as specified in 3GPP TS 24.587 [4] clause 6.1.2.8.

* * * End of Changes * * * *