3GPP TSG-CT WG1 Meeting #143	C1-236002

# Goteborg, 21– 25 August 2023

# Title:	LS on MBSR authorization state

# Release:	Rel-18

# Work Item:	VMR

Source:	CT1

To:	SA2

Cc:

Contact Person:

Name:	Ivo Sedlacek

E-mail Address:	ivo dot sedlacek at ericsson dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

CT1 discussed how to inform MBSR node about change of MBSR authorization state in the following cases:

Case-1)	MBSR node is registered and operates as an MBSR and MBSR authorization state changes from "authorized" to "not-authorized" and the MBSR node is allowed to operate as a UE.

Case-2)	MBSR node is registered and operates as an MBSR and MBSR authorization state changes from "authorized" to "not-authorized" and the MBSR node is not allowed to operate as a UE.

Case-3)	MBSR node is registered and operates as a UE and MBSR authorization state changes from "not-authorized" to "authorized".

Case-4)	MBSR node is performing initial registration and indicates the request to operate as a MBSR, and MBSR authorization is "authorized" and the MBSR node is allowed to operate as an MBSR.

Case-5)	MBSR node is performing initial registration and indicates the request to operate as a MBSR, and MBSR authorization is "not-authorized" and the MBSR node is allowed to operate as a UE.

Case-6)	MBSR node is performing initial registration and indicates the request to operate as a MBSR, and MBSR authorization is "not-authorized" and the MBSR node is not allowed to operate as a UE.

For Case-1 and Case-3, CT1 agreed that generic UE configuration update procedure is used to inform the UE about the MBSR authorization state change. Deregistration procedure was not extended. Deregistration procedure with "re-registration required" indication without any extensions can be used.

For Case-2, CT1 agreed that deregistration procedure with existing cause can be used so there is no need of extensions.

For Case-4 and Case-5, CT1 agreed that registration accept is used to inform the UE about the MBSR authorization state.

For Case-6, CT1 agreed that registration reject with existing cause can be used so there is no need of extensions.

2. Actions:

To SA2 group.

ACTION: 	CT1 would like to ask SA2 to take the above into consideration.

3. Date of Next CT1 Meetings:

CT1#144	9th   – 13th October 2023 		Xiamen

CT1#145	13th   – 17th November 2023 		Chicago