3GPP TSG-CT WG1 Meeting #143	C1-237322

# Goteborg, 21– 25 August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.558 | CR | 0057 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EDGE 5 API implementation guidance. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 3GPP TS 23.558 CR#0432(see S6-232639) agreed in 3GPP SA6#56 meeting describes EDGE-5 APIs exposed by EEC towards the AC that includes AC registration, EAS Discovery, ACR request, AC Subscription and notification. TS 24.558 needs to be aligned with the above SA6 requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | EDGE-5 methods are defined as guidance in new clause 10) that shall be exposed by EEC to AC. AC shall invoke these methods to consume edge services offered over EDGE-1 and EDGE-4 interface. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with the stage 2 requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4, 4.2(new clause), 10(new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 4	Overview

In order to support the edge applications over the 3GPP systems, various features are defined to ensure the efficient use and deployment of edge applications, some of which include, registration, discovery, service provisioning, capability exposure and support for service continuity.

The present document specifies the APIs in detail, needed to support the services offered by the EES over EDGE-1 interface and offered by the ECS over EDGE-4 interface for enabling the edge applications over 3GPP network. Also provides guidance for that API defined for services offered by EDGE-5 interface from the EEC to AC.

## 4.1	Information applicable to APIs over EDGE-1 and EDGE-4

The APIs as specified in this document allow secure access to the capabilities provided by the EES and ECS functional entity.

The stage-2 level requirements and signalling flows are defined in 3GPP TS 23.558 [2].

The usage of HTTP, content type, URI structure definition, notifications, error handling, feature negotiation, HTTP headers and Conventions for Open API specification files, as specified in clauses 7.3, 7.4, 7.5, 7.6, 7.7, 7.8, 7.9 and 7.10 of 3GPP TS 29.558 [4] respectively, shall be applicable for the APIs in the current specification.

## 4.2 Information applicable to APIs over EDGE-5

The stage-2 level requirements are defined in 3GPP TS 23.558 [2].

* * * Next Change * * * *

# 10 EDGE-5 APIs

## 10.1 General

This clause serves as the guidance to the design of the UE internal APIs exposed by the Edge Enabler Client (EEC) that shall be invoked by the Application Client (AC) to consume the edge services offered over EDGE-1 and EDGE-4 interface.

As described in clause 8.14 of TS 23.558 [2], when a AC has to perform edge specific procedure it shares relevant data with the EEC via EDGE-5 reference point through API requests and shall receive the results over response callback. In case the APIs are related to subscribe, then results will be shared over notification callback as depicted in figure 10.1.1. Further each APIs exposed from EDGE-5 interface along with callback are detailed in clause 10.2.

Figure 10.1.1: UE architecture for EDGE API request, response and notification callbacks via EDGE-5 interface

## 10.2 Methods

### 10.2.1 Overview

Table 10.2.1-1 provides an overview of the methods and notifications, namely:

Table 10.2.1-1: Methods invoked by the Application Client on the Edge Enabler Client

| Method name | Description |
| --- | --- |
| Eeec_ACRegistration | AC registers with the EEC, including a response callback listener as specified in 10.2.1-2 for receiving response. (NOTE) |
|  | AC updates the existing registration with the EEC. (NOTE) |
|  | AC deregisters with the EEC. (NOTE) |
| Eeec_EASDiscovery | AC requests discovery of the EAS. (NOTE) |
| Eeec_ACRTrigger | AC sends an ACR trigger request to EEC. (NOTE) |
| Eeec_Services | AC requests EEC to subscribe for services, including a notification callback as specified in 10.2.1-2 for receiving event notifications. (NOTE 1) |
|  | AC requests to update existing subscription for services. (NOTE 1) |
|  | AC requests to unsubscribe for services. (NOTE 1) |
| Eeec_UEId | AC requests to determine the associated Edge UE ID or AF-specific UE ID(s). (NOTE) |
| NOTE: The response for these requests will be provided over response callback. NOTE 1: The notifcation for these requests will be provided over notification callback. |  |

Table 10.2.1-2: Notification from Edge Enabler Client to Application Client

| Method name | Description |
| --- | --- |
| updateResponseCallback | EEC provides the response with relevant information for the outcome of the request triggered by AC. |
| updateNotificationCallback | EEC notifies the event with relevant information for those services it subscribed. |

* * * End of Changes * * * *