3GPP TSG-CT WG1 Meeting #144	C1-237915

# Xiamen, China 9– 13 October 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 5639 | rev | 4 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to discontinuous coverage overload control for mobility registration update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph2 |  |  |  |  |  | Date: |  |  | 2023-10-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS 23.501 states the following in section 5.4.13.5: “In order to avoid a large number of UEs causing excessive signalling load on the network when leaving coverage or re-gaining coverage after being out of coverage, the AMF may determine a Maximum Time Offset for the overload controlling when UEs are allowed to initiate NAS signalling with the network, as described in this clause. …… When the UE knows a later time at which coverage will be lost and when the UE does not send Mobility Registration Update to the AMF in advance (see clause 5.4.13.1), the UE determines a random value up to and including the latest Maximum Time Offset for this PLMN and RAT type and determines an earlier time equal to the later time when coverage will be lost less the random value. The UE shall send the Mobility Registration Update at that earlier time to the AMF indicating the loss of coverage.” The text in 24.501 includes the case of overload control when re-gaining coverage after being out of coverage. However, it does not include the case of overload control for UE leaving from the coverage. This CR aims to include the overload control for mobility registration update to achieve the stage 2 requirement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add discontinuous coverage overload control for mobility registration update. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements from stage 2 are not met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.26 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****** START CHANGE ******

5.3.26	Support for unavailability period

If the UE and network support unavailability period and if the use of unavailability period is not due to NR satellite access discontinous coverage and an event is triggered in the UE making the UE unavailable for a certain period of time, the UE may store its 5GMM and 5GSM context in USIM or non-volatile memory in the ME to be able to reuse it after the unavailability period.

NOTE 1:	How the UE stores its contexts is UE implementation specific.

To activate the unavailability period, the UE provides an unavailability period duration, an indication and type of unavailability, the start of the unavailability period if known during the registration procedure or during the de-registration procedure (see 3GPP TS 23.501 [8] and 3GPP TS 23.502 [9]). The support for the unavailability period is negotiated in the registration procedure. If the UE is registered to a PLMN via a satellite NG-RAN cell, the AMF may provide the unavailability period duration of the UE during the registration procedure. The AMF may consider the unavailability period duration provided by the UE if available and determine the unavailability period duration of the UE as described in subclause 5.5.1.3.4.

If the UE provided an unavailability period duration in the last registration procedure or de-registration procedure, the AMF considers the UE unreachable until the UE re-register for a normal service without providing an unavailability period duration. If the UE did not include a start of the unavailability period, the AMF shall consider the start of the unavailability period to be the time at which AMF received the REGISTRATION REQUEST message from the UE. During the registration procedure, the AMF may determine the value of the periodic registration update timer (T3512) provided to the UE based on the unavailability period duration and the start of the unavailability period based on their availability. The AMF releases the N1 signalling connection after the completion of the registration procedure in which the UE provided an unavailability period duration without providing the start of the unavailability period.

NOTE 2: 	If the UE supports MUSIM and the UE and the networks the UE is registered with support the unavailability period, then the UE can indicate a different unavailability period to each registered network.

If for discontinuous coverage the UE has stored a discontinuous coverage maximum time offset as described in subclause 5.4.4.3, 5.5.1.2.4, and 5.5.1.3.4, the UE shall set the discontinuous coverage maximum time offset value to a random value up to and including the stored discontinuous coverage maximum time offset for this PLMN and satellite NG-RAN RAT Type, determine a time point equal to the time when the UE is about to lose satellite coverage minus the discontinuous coverage maximum time offset value, and send the REGISTRATION REQUEST message for mobility registration update to the AMF indicating out of coverage at the determined time point.

If for discontinuous coverage the UE has stored a discontinuous coverage maximum time offset as described in subclause 5.4.4.3, 5.5.1.2.4, and 5.5.1.3.4, upon returning in coverage after being out of coverage due to discontinuous coverage, the UE sets the discontinuous coverage maximum time offset value to a random value up to and including the stored discontinuous coverage maximum time offset for this PLMN and NR satellite access, and starts this timer. The UE shall not initiate any NAS signalling on that NR satellite access and PLMN while the discontinuous coverage maximum time offset timer is running. The UE shall stop the discontinuous coverage maximum time offset timer and initiate NAS signalling if the UE receives paging message, has pending emergency services, is establishing an emergency PDU session or is performing emergency services fallback procedure or when the UE enters a TAI outside the registration area.

When the unavailability period is activated, all NAS timers are stopped and associated procedures aborted except for timers T3512, T3346, T3447, T3396, T3584, T3585, any back-off timers, T3247, and the timer T controlling the periodic search for HPLMN or EHPLMN or higher prioritized PLMNs (see 3GPP TS 23.122 [5]).

When the UE activates the unavailability period using registration procedure without providing the start of the unavailability period, then after successful completion of the procedure the UE shall enter the state 5GMM-REGISTERED.NO-CELL-AVAILABLE. Otherwise, if the UE provided the start of unavailability period in the registration procedure, the UE shall enter the state 5GMM-REGISTERED.NO-CELL-AVAILABLE only after the UE activates the unavailability period.

When the UE activates the unavailability period using the de-registration procedure, then after successful completion of the procedure the UE shall enter the state 5GMM-DEREGISTERED.NO-CELL-AVAILABLE.

****** END CHANGE ******