3GPP TSG-CT WG1 Meeting #144	C1-238118

# Xiamen, China, 09– 13 October 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.538 | CR | 0080 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update schema of regisration request/response |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2023-10-1 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The schema of registration request is also can be used by the contrained UE. However, some IEs used by the contrained UE has not been defined in the existing schema. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the schema of registration request to add some IEs used by the contrained UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage2 requirement cannot be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

7.3.3.1	MSGin5G UE Registration structure

The schema is based on JSON Schema Draft-07 [8]. For reducing the overhead of the message used in MSGin5G service, the properties are defined as shorten form and the relationships between the properties and elements used in clause 6.3.1.1.1 and clause 6.3.4.2.1 are described in the description of the properties. The JSON schema of the CoAP POST request for the MSGin5G registration is defined below:

{

"$schema": "http://json-schema.org/draft-07/schema#",

"$id": "http://www.3gpp.org/MSGin5G/MSGin5G_Registration_request_schema",

"title": "MSGin5G Registration Request",

"type": "object",

"properties": {

"msgIden": {

"type": "string",

"format": "uri",

"description": "Refer to Service identifier of MSGin5G service"

},

"msgType": {

"type": "string",

"enum": [

"REG"

],

"description": "Refer to the usage of this message. The value REG refers to MSGin5G Registration"

},

"urgentTag": {

"type": "boolean",

"default": false,

"description": "Refer to registration not urgent"

},

"oriAddr": {

"type": "object",

"properties": {

"oriAddrType": {

"enum": [

"UE"

]

},

"addr": {

"type": "string"

}

},

"description": "Refer to Originating UE Service ID"

},

"cliProfile": {

"type": "object",

"properties": {

"triInfo": {

"type": "object",

"properties": {

"ueId": {

"type": "string",

"format": "uri",

"description": "Refer to MSGin5G UE ID"

},

"cliPort": {

"type": "string",

"description": "Refer to MSGin5G Client Ports"

}

},

"required": [

"ueId",

"cliPort"

],

"description": "Refer to MSGin5G Client Triggering Information"

},

"comAvail": {

"type": "object",

"properties": {

"schTime": {

"type": "string",

"format": "date-time",

"description": "Refer to Scheduled Communication Time"

},

"durTime": {

"type": "string",

"format": "date-time",

"description": "Refer to Communication Duration Time"

},

"periIndi": {

"type": "boolean",

"default": false,

"description": "Refer to Periodic Communication Indicator"

},

"periInterval": {

"type": "string",

"format": "date-time",

"description": "Refer to Periodic Communication Interval"

},

"dataSize": {

"type": "string",

"description": "Refer to Data Size Indication"

},

"storeForward": {

"type": "string",

"description": "Refer to Store and Forward Option"

}

},

"description": "Refer to MSGin5G Client Communication Availability"

}

},

"description": "Refer to MSGin5G Client Profile"

},

"reqExpTime": {

"type": "integer",

"description": "Refer to allowed maximum processing time in seconds"

}

},

"required": [

"msgIden",

"oriAddr ",

"secCred"

]

}

The schema is based on JSON Schema Draft-07 [8]. For reducing the overhead of the message used in MSGin5G service, the properties are defined as shorten form and the relationships between the properties and elements used in clause 6.3.1.2.1 are described in the description of the properties. The JSON schema of CoAP 2.01 (Created) response or CoAP 2.04 (Change) response for the MSGin5G registration is defined below:

{

"$schema": "http://json-schema.org/draft-07/schema#",

"$id": "http://www.3gpp.org/MSGin5G/MSGin5G_Registration_response_schema",

"title": "MSGin5G Registration Response",

"type": "object",

"properties": {

"oriAddr": {

"type": "object",

"properties": {

"oriAddrType": {

"enum": [

"UE"

]

},

"addr": {

"type": "string"

}

},

"description": "Refer to Originating UE Service ID"

},

"result": {

"type": "boolean",

"default": true,

"description": "Refer to Registration result. The value true refers to succcess"

}

},

"required": [

"oriAddr",

"result"

]

}

* * * End of Changes * * * *