3GPP TSG-CT WG1 Meeting #145	C1-239156

# Chicago, US , 13– 17 November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.109 | CR | 0075 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reference to obsoleted IETF HTTP RFCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | IMSProtoc18 |  |  |  |  |  | Date: |  |  | 2023-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CT1 agreed to replace references to obsoleted IETF HTTP RFCs in accordance with CT4 LS C1-237127. This specification refers to the obsoleted HTTP/1.1 RFCs 7230, 7231, 7232, 7233, 7234 and 7235, and RFC 2818 "HTTP over TLS". RFC 9112 "HTTP/1.1" obsoletes RFC 7230. RFC 9110 "HTTP Semantics" obsoletes existing RFCs 2818, 7230, 7231, 7232, 7233 and 7235. RFC 9111 "HTTP Caching" obsoletes RFC 7234. Changes between the obsoleted RFC 7230 and new RFC 9112 are listed in Appendix C of RFC 9112. Those changes do not impact the current text in this specification. Changes between the obsoleted RFCs 2818, 7230, 7231, 7232, 7233 and 7235 and new RFC 9110 are listed in Appendix B of RFC 9110. There are no changes from RFCs 2818 and 7235. Changes from RFCs 7230, 7232 and 7233 do not impact the current text in this specification. Changes from RFC 7231 include support of status codes "421 Misdirected Request" and "422 Unprocessable Content" that should be added in tables 6.2.4-1 and 6.3.2-1. Other changes do not impact the current text in this specification. Changes between the obsoleted RFC 7234 and new RFC 9111 are listed in Appendix B of RFC 9111. Those changes do not impact the current text in this specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RFC 7230 replaced with RFC 9112. RFC 7231 replaced with RFC 9110. RFC 7234 replaced with RFC 9111. References to RFCs 2818, 7232, 7233 and 7235 made void in clause 2, in clause 5.2.1 deleted since a reference to RFC 9110 already exists due to replacement of RFC 7231 with RFC 9110, and in other clauses replaced with a reference to RFC 9110. Status codes 421, 422 and missing 426 (i.e. also specified by RFC 7231) added in tables 6.2.4-1 and 6.3.2-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification would refer to obsoleted IETF RFCs, will not be aligned with other CT1 TSs and would then be misleading or potentially cause interoperability problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.2, 5.2.1, 5.2.3, 5.3.2.2, 5.3.5, 6.2.1, 6.2.2, 6.2.4, 6.3.1, 6.3.2, A.2.2, I.2, K.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

# 2	References

The following documents contain provisions, which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic bootstrapping architecture".

[2]	3GPP TR 33.919: "Generic Authentication Architecture (GAA); System description".

[3]	3GPP TS 29.109: "Generic Authentication Architecture (GAA); Zh and Zn Interfaces based on the Diameter protocol; Protocol details".

[4]	3GPP TS 33.221: "Generic Authentication Architecture (GAA); Support for Subscriber Certificates".

[5]	3GPP TS 33.222: "Generic Authentication Architecture (GAA); Access to network application functions using Hypertext Transfer Protocol over Transport Layer Security (HTTPS)".

[6]	IETF RFC 3310: "Hypertext Transfer Protocol (HTTP) Digest Authentication Using Authentication and Key Agreement (AKA)".

[7]	3GPP TS 23.003: "Numbering, addressing and identification".

[8]	IETF RFC 3023: "XML Media Types".

[9]	Void.

[10]	Void.

[11]	Void.

[12]	Void.

[13]	3GPP TS 24.228 Release 5: "Signalling flows for the IP multimedia call control based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[14]	Void.

[15]	Void.

[16]	PKCS#10 v1.7: "Certification Request Syntax Standard".

NOTE:	ftp://ftp.rsasecurity.com/pub/pkcs/pkcs-10/pkcs-10v1_7.pdf

[17]	WAP Forum: "WPKI: Wireless Application Protocol; Public Key Infrastructure Definition"

NOTE:	http://www1.wapforum.org/tech/documents/WAP-217-WPKI-20010424-a.pdf.

[18]	Void.

[19]	Open Mobile Alliance: "ECMAScript Crypto Object"

NOTE:	http://www.openmobilealliance.org.

[20]	Open Mobile Alliance: "WPKI"

NOTE:	http://member.openmobilealliance.org/ftp/public_documents/SEC/Permanent_documents/.

[21]	3GPP TS 33.203: "3G security; Access security for IP-based services".

[22]	IETF RFC 2234: "Augmented BNF for Syntax Specifications: ABNF".

[23]	Void.

[24]	3GPP TS 33.223: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA) Push function".

[25]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[26]	Open Mobile Alliance Push Enabler Release v2.2: "Push Over the Air".

NOTE:	http://www.openmobilealliance.org/Technical/release_program/push_v2_2.aspx.

[27]	Open Mobile Alliance Push Enabler Releasev 2.2: "Push Message Specification".

NOTE:	http://www.openmobilealliance.org/Technical/release_program/push_v2_2.aspx.

[28]	Open Mobile Alliance Device Management Enabler Release v1.2: "Enabler Release Definition for OMA Device Management".

NOTE:	http://www.openmobilealliance.org/Technical/release_program/dm_v1_2.aspx.

[29]	3GPP TS 33.303: "Proximity-based Services (ProSe); Security aspects".

[30]	IETF RFC 9112: "HTTP/1.1".

[31]	IETF RFC 9110: "HTTP Semantics".

[32]	Void.

[33]	Void.

[34]	IETF RFC 9111: "HTTP Caching".

[35]	Void.

[36]	IETF RFC 7616: "HTTP Digest Access Authentication".

[37]	IETF RFC 4648: "The Base16, Base32, and Base64 Data Encodings".

[38]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

# *** Next Change ***

## 4.2	Bootstrapping procedure

The UE shall initiate the bootstrapping procedure when:

a)	the UE wants to interact with a NAF and bootstrapping is required;

b)	a NAF has requested bootstrapping required indication as described in clause 5.2.4 or bootstrapping renegotiation indication as described in clause 5.2.5; or

c)	the lifetime of the key has expired in the UE if one or more applications are using that key.

A UE and the BSF shall establish bootstrapped security association between them by running bootstrapping procedure. Bootstrapping security association consists of a bootstrapping transaction identifier (B-TID) and key material Ks. Bootstrapping session on the BSF also includes security related information about subscriber (e.g. user's private identity). Bootstrapping session is valid for a certain time period, and shall be deleted in the BSF when the session becomes invalid.

Bootstrapping procedure shall be based on HTTP Digest AKA as described in 3GPP TS 33.220 [1] and in RFC 3310 [6] with the modifications described below.

The BSF address is derived from the IMPI or IMSI according to 3GPP TS 23.003 [7].

A UE shall indicate to the BSF that it supports the use of TMPI as defined in 3GPP 33.220 [1] by including a "product" token in the "User-Agent" header field (cf. RFC 9110 [31]) that is set to a static string "3gpp-gba-tmpi" in HTTP requests sent to the BSF.

A BSF shall indicate to the UE that it supports the use of  TMPI as defined in 3GPP 33.220 [1] by including a "product" token in the "Server" header field (cf. RFC 9110 [31]) that is set to a static string "3gpp-gba-tmpi" in HTTP responses sent to the UE.

In the bootstrapping procedure, Authorization, WWW-Authenticate, and Authentication-Info HTTP headers shall be used as described in RFC 3310 [6] with following exceptions:

a)	the "realm" parameter shall contain the network name where the username is authenticated;

b)	the quality of protection ("qop") parameter shall be "auth-int"; and

c)	the "username" parameter shall contain user's private identity (IMPI).

NOTE:	If the UE does not have an ISIM application with an IMPI, the IMPI will be constructed from IMSI, according to 3GPP TS 23.003 [7].

In addition to RFC 3310 [6], the following apply:

a)	In the initial request from the UE to the BSF, the UE shall include Authorization header with following parameters:

-	the username directive, set to

1)	the value of the TMPI if one has been associated with the private user identity as described in 3GPP 33.220 [1]; or

2)	the value of the private user identity;

-	the realm directive, set to the BSF address derived from the IMPI or IMSI according to 3GPP TS 23.003 [7];

-	the uri directive, set to either absoluteURL "http://<BSF address>/" or abs_path "/", and which one is used is specified in RFC 7616 [36];

-	the nonce directive, set to an empty value; and

-	the response directive, set to an empty value;

b)	In the challenge response from the BSF to the UE, the BSF shall include parameters to WWW-Authenticate header as specified in RFC 3310 [6] with following clarifications:

-	the realm directive, set to the BSF address derived from the IMPI or IMSI according to 3GPP TS 23.003 [7];

c)	In the message from the BSF to the UE, the BSF shall include bootstrapping transaction identifier (B-TID) and the key lifetime to an XML document in the HTTP response payload. The BSF may also include additional server specific data to the XML document. The XML schema definition of this XML document is given in annex C.

d)	When responding to a challenge from the BSF, the UE shall include an Authorization header containing a realm directive set to the value as received in the realm directive in the WWW-Authenticate header.

e)	Authentication-Info header shall be included into the subsequent HTTP response after the BSF concluded that the UE has been authenticated. Authentication-Info header shall include the "rspauth" parameter.

After successful bootstrapping procedure the UE and the BSF shall contain the key material (Ks) and the B-TID. The key material shall be derived from AKA parameters as specified in 3GPP TS 33.220 [1]. In addition, BSF shall also contain a set of security specific attributes related to the UE.

An example flow of successful bootstrapping procedure can be found in clause A.3.

# *** Next Change ***

### 5.2.1	General

The HTTP Digest authentication model as described in RFC 7616 [36] can be used with bootstrapped security association as the authentication and integrity protection method, if the application protocol used over Ua interface between UE and NAF is based on HTTP (see RFC 9112 [30], RFC 9110 [31] and RFC 9111 [34]). The HTTP Digest authentication may be used for all protocols that have adopted the HTTP authentication framework to mutually authenticate the UE and the NAF, and also optionally integrity protect any payload being transferred between them.

The UE shall indicate to an application server (i.e. a NAF) that it supports 3GPP-bootstrapping based HTTP Digest authentication by including a "product" token to the "User-Agent" header (cf. RFC 9110 [31]) in outgoing HTTP requests. If the UE supports

a)	AKA-based authentication, then the "product" token is a static string "3gpp-gba" if the HTTP client application resides in the ME, or "3gpp-gba-uicc" if the HTTP client application resides in the UICC; or

b)	GBA_Digest, then the "product" token is "3gpp-gba-digest".

The UE may indicate multiple GBA modes by inserting multiple "product" tokens in the User-Agent header field. The User-Agent header field with GBA related "product" tokens shall be added to each outgoing HTTP request if the UE supports GBA-based authentication using HTTP Digest.

Upon receiving  GBA related "product" tokens, the application server if it supports NAF functionality may decide to authenticate the UE using GBA-based shared secret by executing the authentication procedure. If multiple GBA modes have been indicated in the User-Agent header field, then the NAF selects one GBA mode and indicates the selected mode in responses to the UE in the "realm" parameter of the WWW-Authenticate header field. In the selection of the GBA mode by the UE, AKA-based modes shall take priority over GBA_Digest.

The protocol stack of the Ua interface when HTTP Digest authentication is used is presented in figure 5.2-1. The details are defined in the following clauses.

NOTE:	HTTP is not the only protocol that can be used. Other protocols can also be used as long as the protocol has adopted the HTTP authentication framework.

Figure 5.2-1: Protocol stack of Ua interface with HTTP Digest authentication

# *** Next Change ***

### 5.2.3	Authentication failures

Authentication failures are handled as they are described in RFC 7616 [36] and RFC 9110 [31].

# *** Next Change ***

#### 5.3.2.2	Authentication failures

Server authentication failures are handled in TLS as they are described in the RFC for TLS which is defined in annex E of 3GPP TS 33.310 [25]. Client authentication failures are handled in HTTP Digest as they are described in RFC 7616 [36] and RFC 9110 [31].

# *** Next Change ***

### 5.3.5	Integrity protection

Integrity protection is provided by using authenticated TLS tunnel as described in RFC 9110 [31].

# *** Next Change ***

### 6.2.1	Enrolment procedure

The UE shall generate a PKCS#10 certification request [16] according to 3GPP TS 33.221 [4]. The UE shall send the PKCS#10 certification request to the PKI portal in the HTTP payload in a HTTP POST request. The Request-URI shall indicate the desired response type. Upon successful enrolment, PKI portal shall return the enrolled subscriber certificate in the desired format.

The UE populates the HTTP POST request as follows:

-	the HTTP version shall be 1.1 which is specified in RFC 9112 [30];

-	the base of the Request-URI is extracted from the full PKI portal URI (e.g. if the full PKI portal URI is "http://pki-portal.operator.com/enrol" then the Request-URI shall be "/enrol".

NOTE 1:	In case a proxy is used between the UE and the PKI portal, then the full Request-URI will be used in the HTTP Post request.

-	the Request-URI shall contain an URI parameter "response" that shall be set to "single", "pointer", or "chain" depending on the UE's desired response type (e.g. Request-URI may take the form of "/enrol?response=single" for certificate delivery);

NOTE 2:	The PKI portal might ignore the UE's desired response type, and the UE should be capable of receiving the issued subscriber certificate in any of the response types.

-	the UE may add additional URI parameters to the Request-URI;

NOTE 3:	The PKI portal might ignore the additional URI parameters.

-	the HTTP header Content-Type shall be "application/x-pkcs10";

-	the HTTP header Content-Length shall be the length of the Base64 encoded PKCS#10 certification request in Octets; and

-	the HTTP payload shall contain the Base64 encoded PKCS#10 certification request and optionally surrounded by "----- BEGIN CERTIFICATE REQUEST -----" and "----- END CERTIFICATE REQUEST -----" tags;

-	the UE may add additional HTTP headers to the HTTP POST request.

The UE sends the HTTP POST request to the PKI portal. The PKI portal checks that the HTTP request is valid, and extracts the Base64 encoded PKCS#10 certification request for further processing. The PKI portal shall verify that the subscriber is authorized to receive the particular type of certificate by checking subscriber's user security settings received from the BSF as specified 3GPP TS 33.220 [1].

Upon successful subscriber certificate creation procedure, the PKI portal shall return the subscriber certificate to the UE in the UE's desired format or in the PKI portal's desired format.

The response format type shall be one of the following:

-	the subscriber certificate itself (i.e. desired response type was "single");

-	a pointer to the subscriber certificate (i.e. desired response type was "pointer"); or

-	a certificate chain that contains full certification chain from subscriber certificate to the root certificate (i.e. desired response type was "chain").

If response format type is "single", the PKI portal shall populate HTTP response as follows:

-	the HTTP status code shall be 200;

-	the HTTP header Content-Type shall be "application/x-x509-user-cert";

-	the HTTP header Content-Length shall be the length of the HTTP payload in octets;

-	the HTTP payload shall contain the Base64 encoded subscriber certificate and optionally surrounded by
"----- BEGIN CERTIFICATE -----" and "----- END CERTIFICATE -----" tags;

-	the PKI portal may add additional HTTP headers to the HTTP response.

If response format type is "pointer", the PKI portal shall populate HTTP response as follows:

-	the HTTP status code shall be 200;

-	the HTTP header Content-Type shall be "application/vnd.wap.cert-response";

-	the HTTP header Content-Length shall be the length of the HTTP payload in octets;

-	the HTTP payload shall contain the Base64 encoded CertResponse structure and optionally surrounded by
"----- BEGIN CERTIFICATE RESPONSE -----" and "----- END CERTIFICATE RESPONSE -----" tags;

-	the PKI portal may add additional HTTP headers to the HTTP response.

If response format type is "chain", the PKI portal shall populate HTTP response as follows:

-	the HTTP status code shall be 200;

-	the HTTP header Content-Type shall be "application/pkix-pkipath";

-	the HTTP header Content-Length shall be the length of the HTTP payload in octets;

-	the HTTP payload shall contain the Base64 encoded PkiPath structure;

-	the PKI portal may add additional HTTP headers to the HTTP response.

The PKI portal shall send the HTTP response to the UE. The UE shall check that the HTTP response is valid, and extract the Base64 encoded subscriber certificate, pointer to the subscriber certificate, or certificate chain for further processing.

An example flow of a successful subscriber certificate enrolment procedure can be found in clause E.3.

# *** Next Change ***

### 6.2.2	WIM specific authentication code for key generation

The UE may be equipped with a WIM which may require an authentication code from WIM provider in order to generate a key onboard WIM as specified in OMA ECMAScript [19] and OMA WPKI [20] specifications. In this case, the UE shall request the authentication code from PKI portal using a HTTP GET request. If the PKI portal can acquire authentication code, it is returned to the UE in the corresponding HTTP response.

The UE populates the HTTP GET request as follows:

-	the HTTP version shall be 1.1 which is specified in RFC 9112 [30];

-	the base of the Request-URI is extracted from the full PKI portal URI and appended with "/wim-auth-code" (e.g. if the full PKI portal URI is "http://pki-portal.operator.com/enrol" then the Request-URI shall be "/enrol/wim-auth-code");

-	the Request-URI shall contain an URI parameter "request" that shall be set to the return value received from the WIM;

NOTE 1:	If an authentication code is required, the WIM will return "error:AuthReq:cardSerialNumber:Challenge". The cardSerialNumber and the Challenge are in a hexadecimal format as specified in OMA ECMAScript specification [19].

-	the UE may add additional URI parameters to the Request-URI;

-	the UE may add additional HTTP headers to the HTTP GET request.

The UE sends the HTTP GET request to the PKI portal. The PKI portal acknowledges that this is an authentication code because the Request-URI contains the "/wim-auth-code" and the URI parameter "request". The PKI portal extracts the authentication code derivation parameters from the URI parameter "request", and derives the authentication code.

NOTE 2:	The actual derivation of the authentication code is outside the scope.

Upon successful authentication code derivation, the PKI portal shall return the authentication code to the UE in a HTTP response:

-	the HTTP status code shall be 200;

-	the HTTP header Content-Type shall be "text/plain";

-	the HTTP header Content-Length shall be the length of the HTTP payload in octets;

-	the HTTP payload shall contain the authentication code in a hexadecimal format;

-	the PKI portal may add additional HTTP headers to the HTTP response.

Upon receiving the authentication code from the PKI portal, the UE shall use it to authenticate the procedure of generating the key onboard the WIM.

# *** Next Change ***

### 6.2.4	Error situations

Subscriber certificate enrolment may not be successful for multiple reasons. The error cases are indicated by using 4xx and 5xx HTTP Status Codes as defined in RFC 9110 [31]. The 4xx status code indicates that the UE seems to have erred, and the 5xx status code indicates that the PKI portal is aware that it has erred. Possible error situations during subscriber certificate enrolment and their mappings to HTTP Status Codes are described in table 6.2.4-1.

NOTE:	On the table 6.2.4-1, the "Description" column describes the error situation in PKI portal. The "PKI portal error" column describes the typical reason for the error.

An example flow of a failure in subscriber certificate enrolment procedure can be found in clause E.4.

Table 6.2.4-1: HTTP Status Codes used for enrolment error

| HTTP Status Code | HTTP Error | UE should repeat the request | Description | PKI portal error |
| --- | --- | --- | --- | --- |
| 400 | Bad Request | No | Request could not be understood | PKCS#10 request was missing, or malformed |
| 401 | Unauthorized | Yes | Request requires authentication (cf. clause 5.2) | Authentication pending, cf. clause 5.2 |
| 402 | Payment Required | No | Reserved for future use | - |
| 403 | Forbidden | No | PKI portal understood the request, but is refusing to fulfil it | PKCS#10 request was valid, but subscriber is not allowed to enrol this particular type of certificates or PKCS#10 request contained unacceptable parameters |
| 404 | Not Found | No | PKI portal has not found anything matching the Request-URI | The Request-URI was malformed and PKI portal cannot fulfil the enrolment request |
| 405 to 406 | * | No | Not used by PKI portal | - |
| 407 | Proxy Authentication Required | Yes | PKI portal uses Authentication Proxy and UE shall authenticate itself with the proxy | Authentication Proxy authentication pending, cf. clause 5.2 |
| 408 to 417, 421, 422, 426 | * | No | PKI portal should not use these status codes | - |
| 500 | Internal Server Error | No | PKI portal encountered an unexpected error | PKI portal is mis-configured |
| 501 | Not Implemented | No | PKI portal does not support the required functionality | The server does not contain PKI portal service |
| 502 | Bad Gateway | No | Gateway/Proxy received an invalid response from PKI portal | PKI portal is behind a gateway/proxy and sent an invalid response to the gateway/proxy |
| 503 | Service Unavailable | Yes | PKI portal service is currently unavailable | PKI portal is temporarily unavailable, UE may repeat the request after delay indicated by "Retry-After" header |
| 504 | Gateway Timeout | No | Gateway/Proxy did not receive a timely response from the upstream server | PKI portal is behind a gateway/proxy and did not send a response to the gateway/proxy in time, or was not reachable by the gateway/proxy |
| 505 | HTTP Version Not Supported | No | PKI portal does not support the HTTP protocol version that was used in the request line | UE should use HTTP/1.1 version with PKI portal |

# *** Next Change ***

### 6.3.1	CA certificate delivery procedure

The UE shall populate the HTTP GET request as follows:

-	the HTTP version shall be 1.1 which is specified in RFC 9112 [30];

-	the base of the Request-URI is extracted from the full PKI portal URI (e.g. if the full PKI portal URI is "http://pki-portal.operator.com/getcertificate" then the Request-URI shall be "/getcertificate".

NOTE 1:	In case a proxy is used between the UE and the PKI portal, then the full Request-URI will be used in the HTTP Post request.

-	the Request-URI shall contain an URI parameter "in" that shall be the Base64 encoding of the DER encoded Issuer field of the X.509 certificate;

-	the Request-URI may contain an URI parameter "ki" that shall be the Base64 encoding of the DER encoded the Key Identifier of the X.509 certificate;

NOTE 2:	Key Identifier of the CA certificate can be obtained from the Authority Key Identifier extension of the subscriber certificate.

-	the UE may add additional URI parameters to the Request-URI;

-	the UE may add additional HTTP headers to the HTTP GET request.

The UE sends the HTTP GET request to the PKI portal. The PKI portal checks that the HTTP request is valid, and extracts the "in" parameter and optionally "ki" parameter from the Request-URI. If the PKI portal can verify that the Issuer field parameter is valid, and that the UE may set the CA certificate as a root certificate (i.e. trusted CA certificate), it will then send the CA certificate back to the UE in the corresponding HTTP response.

The PKI portal shall populate the HTTP response as follows:

-	the HTTP status code shall be 200;

-	the HTTP header Content-Type shall be "application/x-x509-ca-cert";

-	the HTTP header Content-Length shall be the length of the HTTP payload in octets;

-	the HTTP payload shall contain the Base64 encoded CA certificate structure and optionally surrounded by "----- BEGIN CERTIFICATE -----" and "----- END CERTIFICATE -----" tags;

-	the PKI portal may add additional HTTP headers to the HTTP response.

The PKI portal shall send the HTTP response to the UE. The UE shall check that the HTTP response is valid, and extract the Base64 encoded CA certificate for further processing. UE shall validate and match the received CA certificate against the parameters supplied in the corresponding request.

An example flow of CA certificate procedure can be found in clause E.5.

# *** Next Change ***

### 6.3.2	Error situations

CA certificate delivery may not be successful for multiple reasons. The error cases are indicates by using 4xx and 5xx HTTP Status Codes as defined in RFC 9110 [31]. The 4xx status code indicates that the UE seems to have erred, and the 5xx status codes indicate that the PKI portal is aware that it has erred. Possible error situations during CA certificate delivery and their mappings to HTTP Status Codes are described in table 6.3.2-1.

NOTE:	On the table 6.3.2-1, the "Description" column describes the error situation in PKI portal. The "PKI portal error" column describes the typical reason for the error.

An example flow of a failure in CA certificate delivery procedure can be found in clause E.6.

Table 6.3.2-1: HTTP Status Codes for CA certificate delivery error

| HTTP Status Code | HTTP Error | UE should repeat the request | Description | PKI portal error |
| --- | --- | --- | --- | --- |
| 400 | Bad Request | No | Request could not be understood | Request could not be understood |
| 401 | Unauthorized | Yes | Request requires authentication (cf. clause 5.2) | Authentication pending, cf. clause 5.2 |
| 402 | Payment Required | No | Reserved for future use | - |
| 403 | Forbidden | No | PKI portal understood the request, but is refusing to fulfil it | CA certificate delivery request was understood but PKI portal refuses to deliver the CA certificate |
| 404 | Not Found | No | PKI portal has not found anything matching the Request-URI | PKI portal does not have the requested CA certificate |
| 405 to 406 | * | No | Not used by PKI portal | - |
| 407 | Proxy Authentication Required | Yes | PKI portal uses Authentication Proxy and UE shall authenticate itself with the proxy | Authentication Proxy authentication pending, cf. clause 5.2 |
| 408 to 417, 421, 422, 426 | * | No | PKI portal should not use these status codes | - |
| 500 | Internal Server Error | No | PKI portal encountered an unexpected error | PKI portal is mis-configured |
| 501 | Not Implemented | No | PKI portal does not support the required functionality | The server does not contain PKI portal service |
| 502 | Bad Gateway | No | Gateway/Proxy received an invalid response from PKI portal | PKI portal is behind a gateway/proxy and sent an invalid response to the gateway/proxy |
| 503 | Service Unavailable | Yes | PKI portal service is currently unavailable | PKI portal is temporarily unavailable, UE may repeat the request after delay indicated by "Retry-After" header |
| 504 | Gateway Timeout | No | Gateway/Proxy did not receive a timely response from the upstream server | PKI portal is behind a gateway/proxy and did not send a response to the gateway/proxy in time, or was not reachable by the gateway/proxy |
| 505 | HTTP Version Not Supported | No | PKI portal does not support the HTTP protocol version that was used in the request line. | UE should use HTTP/1.1 version with PKI portal |

# *** Next Change ***

## A.2.2	Key required to interpret signalling flows

3GPP TS 24.228 [13], clause 4.1.1, specifies the key required to interpret the contents of the SIP methods. This key is used with HTTP based messages (cf. RFC 9110 [31]) as well since SIP and HTTP messages resemble each other in structure. The following key rules are used in addition to those specified in 3GPP TS 24.228 [13]:

a)	The HTTP based messaging is always initiated by the client:

-	HTTP request is generated by the client (i.e. UE);

-	HTTP response is generated by the server as a response to the HTTP request;

-	HTTP proxies may be between the client and the server.

b)	There is only one single HTTP response to the HTTP request.

c)	In order to differentiate between HTTP messages and other protocol messages, the HTTP messages are marked with simple arrow line, and all non-HTTP messages with block arrows.

d)	The flows show the signalling exchanges between the following functional entities in addition to those specified in 3GPP TS 24.228 [13]:

-	Bootstrapping Server Function (BSF);

-	Network Application Function (NAF);

-	PKI portal (PKI portal).

e)	The "(B-TID)" sequence of characters is used to indicate that the bootstrapping transaction identifier (B-TID) needs to be filled in.

# *** Next Change ***

# I.2	GBA_Digest bootstrapping procedure

A UE and the BSF shall establish a bootstrapped security association between them by running the GBA_Digest bootstrapping procedure. The bootstrapping security association consists of a bootstrapping transaction identifier (B-TID) and key material Ks. A Bootstrapping session on the BSF also includes security related information about the subscriber (in particular the user's private identity). A Bootstrapping session is valid for a certain time period, and shall be deleted in the BSF when the session becomes invalid.

The UE shall establish a TLS connection with the BSF prior to sending any HTTP request to the BSF.

A UE shall indicate to the BSF that it intends to run GBA_Digest as defined in 3GPP TS 33.220 [1] by including a "product" token in the "User-Agent" header field (cf. RFC 9110 [31]) that is set to a static string "GBA_Digest" in HTTP requests sent to the BSF. The BSF is configured to either allow or disallow the use of GBA_Digest bootstrapping. If GBA_Digest is disallowed, the BSF shall reject the HTTP request by the UE.

The GBA_Digest Bootstrapping procedure as specified in 3GPP TS 33.220 [1] is further detailed as described below.

-	Authorization, WWW-Authenticate, and Authentication-Info HTTP header fields shall be used as described in RFC 7616 [36] with following exceptions:

a)	the "realm" parameter shall be set to the domain name of the home network;

b)	the quality of protection ("qop") parameter shall be "auth-int";

c)	the "username" parameter shall contain user's private identity;

d)	the "nonce" field shall be populated as specified in 3GPP TS 33.220 [1], annex M with a random number generated by the BSF according to RFC 7616 [36]; and

-	a password, which is called "passwd" and is derived as specified in 3GPP TS 33.220 [1], annex M.

In addition to RFC 7616 [36], the following apply:

a)	in the initial request from the UE to the BSF, the UE shall include an Authorization header field with following parameters:

the uri directive, set to either absoluteURL "https://<BSF address>/" or abs_path "/", and which one is used is specified in RFC 7616 [36];

-	the nonce directive, set to an empty value;

-	the response directive, set to an empty value;

b)	in the HTTP response containing the Digest challenge from the BSF to the UE, the BSF shall include parameters to WWW-Authenticate header field as specified in RFC 7616 [36];

c)	in the HTTP request sent as an answer to the HTTP response in bullet b) the UE shall include an Authorization header field that contains a digest-response, "algorithm", "qop", "cnonce", and "nc" header field parameters as specified in RFC 7616 [36], and

-	the uri directive, set to either absoluteURL "https://<BSF address>/" or abs_path "/", and which one is used is specified in RFC 7616 [36];

-	the nonce directive, set to an empty value; and

d)	n the message from the BSF to the UE, which the BSF shall only send after the BSF concluded that the UE has been authenticated, the BSF shall include an Authentication-Info header with the "rspauth" parameter. Furthermore, the BSF shall include the bootstrapping transaction identifier (B-TID) and the key lifetime to an XML document in the HTTP response payload. The BSF may also include additional server specific data to the XML document. The XML schema definition of this XML document is given in annex C.

After a successful bootstrapping procedure the UE and the BSF shall contain the key material (Ks) and the B-TID. The key material shall be derived from SIP Digest  parameters as specified for GBA_Digest in 3GPP TS 33.220 [1]. In addition, the BSF may also contain a set of security specific attributes (GUSS) related to the UE, depending on the conditions in clause 4.5.2 of 3GPP TS 33.220 [1].

# *** Next Change ***

# K.2	Shared key-based UE authentication with certificate-based AF authentication

The TLS profile for GBA in clause 5.3.2.1 is modified with the AKMA AF taking the role of the NAF from GBA (see 3GPP TS 33.220 [1]) to support AKMA keys as follows:

-	the UE and the AF shall support the TLS version as specified in annex E of 3GPP TS 33.310 [25]. See clause 5.3.1 in 3GPP TS 33.222 [5] for the detailed profiling of TLS.

a)	when the UE starts communication via Ua* reference point with the AF, it shall establish a TLS tunnel with the AF. The AF is authenticated to the UE by means of a public key certificate. The UE shall verify that the server certificate corresponds to the FQDN of the AF with which it established the tunnel. No client authentication is performed as part of TLS (no client certificate necessary).

b)	the UE sends an HTTP request to the AF inside the TLS tunnel (HTTPS, i.e. HTTP over TLS) as described in clause 5.2 with the following changes:

1)	the UE shall indicate to an AF that it supports AKMA based HTTP Digest authentication by including a "product" token, that is a constant string "3gpp-akma", in the "User-Agent" header (see RFC 9110 [31]) in outgoing HTTP requests; and

2)	the AF may decide to authenticate the UE using AKMA-based shared secret by executing the authentication procedure. This is indicated in the "realm" parameter of the WWW-Authenticate header field. The realm attribute shall contain the constant string "3GPP-bootstrapping-akma". If the AF has a choice between GBA_Digest (see 3GPP TS 33.220 [1]) and AKMA keying, then the AF shall select AKMA over GBA_Digest.

NOTE 1:	The choice between AKMA and AKA-based GBA at the UE and the AF, if both are supported, is application dependent.

c)	the UE shall generate the HTTP request and the AF shall authenticate the HTTP request using HTTP Digest. HTTP Digest authentication (see RFC 3310 [6]) shall be used with previously bootstrapped security association as follows:

1)	the "username" parameter shall be the A-KID;

2)	the password used in the digest calculations shall be KAF (AKMA Application Key) with the KAF Base64 encoded as specified in RFC 4648 [37]; and

3)	the "realm" parameter shall contain two parts delimited by "@" sign where the first part is the constant string "3gpp-akma" and the latter part shall be the FQDN of the AF (e.g. "3gpp-akma@af1.operator.com"); and

d)	both the UE and the AF shall verify upon receiving each of the HTTP responses and HTTP requests that the second part of the realm attribute is equal to the FQDN of the AF.

The authentication failures are supported as described in clause 5.3.2.2.

Clauses 5.3.2.3 and 5.3.2.4 are not supported as AKMA does not support deriving a fresh key in the same way as GBA.

NOTE 2:	How a fresh key is derived for AKMA is up to Ua* protocol implementation.

# *** End of Changes ***