3GPP TSG-CT WG1 Meeting #145	C1-239672

# Chicago, US , 13– 17 November 2023

Source:	Lenovo

Title:	UE-to-UE session performance for ADAE services

Spec:	3GPP TS 24.559

Agenda item:	18.2.29

Document for:	Agreement

1. Introduction

<Introduction part (optional)>

2. Reason for Change

The pCR describes on the UE-to-UE session performance.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.559-V0.20.

* * * First Change * * * *

#### 7.1.3.3	Resource: UE-to-UE session performance analytics

##### 7.1.3.3.1	Description

This resource is used by the ADAES to request the ADAEC for the UE-to-UE session performance analytics.

##### 7.1.3.3.2	Resource definition

Resource URI: {apiRoot}/adae-sc/<apiVersion>/ue2ue-session-performance

This resource shall support the resource URI variables defined in the table 7.1.3.3.2-1.

Table 7.1.3.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.2.4 in 3GPP TS 29.122 [X] |

##### 7.1.3.3.3	Resource standard methods

None

##### 7.1.3.3.4	Resource custom operations

###### 7.1.3.3.4.1	Overview

Table 7.1.3.3.4.1-1: Custom operations

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Fetch | /ue2ue-session-performance/fetch | POST | Request for the UE-to-UE session performance analytics |

###### 7.1.3.3.4.2	POST

This method is for the ADAES to request the ADAEC the UE-to-UE session performance analytics and shall support the URI query parameters specified in table 7.1.3.3.4.2-1.

Table 7.1.3.3.4.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 7.1.3.3.4.2-2 and the response data structures and response codes specified in table 7.1.3.3.4.2-3.

Table 7.1.3.3.4.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| Ue2UePerfReq | M | 1 | ADAES requests ADAEC for the UE-to-UE session performance analytics |

Table 7.1.3.3.4.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| Ue2UePerfResp | M | 1 | 200 (OK) | ADAEC responses ADAES the UE-to-UE session performance analytics |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [X] shall also apply. |  |  |  |  |

* * * End of Changes * * * *