3GPP TSG-CT WG1 Meeting #147	C1-240885

# Athens, Greece, 26 February-1 March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 6026 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alternative S-NSSAI supports services the replaced S-NSSAI supports |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As agreed in SP-231512 and captured in TS 23.501 v18.4.0, since there is no guarantee that the alternative S-NSSAI provides similar performance to the replaced S-NSSAI, it is only recommended to use an alternative S-NSSAI that is able to support requirements for the services that the replaced S-NSSAI supports. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add one informative note to suggest that the alternative S-NSSAI is able to support service requirements supported by the replaced S-NSSAI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not aligned with stage 2, unclear whether the alternative S-NSSAI should be able to support minimum requirements for the services that the replaced S-NSSAI supports. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

4.6.2.7	Mobility management based network slice replacement

The support for network slice replacement by a UE or network is optional. If the UE and network support network slice replacement, and the AMF determines that an S-NSSAI included in the allowed NSSAI needs to be replaced with an alternative S-NSSAI, the AMF provides:

a)	the alternative S-NSSAI in the allowed NSSAI, if not included yet;

b)	the alternative S-NSSAI in the configured NSSAI, if not included yet;

c)	the alternative S-NSSAI in the NSAG information, if not included yet and the UE supports NSAG; and

d)	the mapping information between the S-NSSAI to be replaced and the alternative S-NSSAI,

to the UE during the UE configuration update procedure or during the registration procedure as follows:

a)	for non-roaming UE, the AMF provides the mapping information between the S-NSSAI included in the allowed NSSAI and the alternative S-NSSAI to the UE; and

NOTE 1:	In non-roaming scenarios, the alternative S-NSSAI does not have to be part of the subscribed S-NSSAI(s) in the UE subscription.

b)	for roaming UE:

1)	if the S-NSSAI included in the allowed NSSAI needs to be replaced (i.e. the S-NSSAI to be replaced is part of the VPLMN S-NSSAIs), the AMF provides the mapping information between the S-NSSAI included in the allowed NSSAI and the alternative S-NSSAI to the UE; and

2)	if the S-NSSAI included in the mapped S-NSSAI(s) for the allowed NSSAI needs to be replaced (i.e. the S-NSSAI to be replaced is part of the HPLMN S-NSSAIs), the AMF provides the mapping information between the S-NSSAI and the alternative S-NSSAI to the UE.

NOTE 1A:	In roaming scenarios, the alternative S-NSSAI part of the HPLMN S-NSSAIs does not have to be part of the subscribed S-NSSAI(s) in the UE subscription.

NOTE 2:	It is up to AMF local policy to determine when to provide the mapping information between the S-NSSAI to be replaced and the alternative S-NSSAI to the UE, e.g. when the alternative S-NSSAI is available and there is no PDU session associated with the S-NSSAI to be replaced, or wait until the UE establishes the first PDU session associated with the S-NSSAI to be replaced.

If the AMF determines that the S-NSSAI which has been replaced is available, the AMF provides the updated alternative NSSAI excluding the S-NSSAI which has been replaced and the corresponding alternative S-NSSAI to the UE during the UE configuration update procedure or during the registration procedure.

If all the S-NSSAI(s) that were replaced in alternative NSSAI are available, the AMF provides the alternative NSSAI with Length of Alternative NSSAI contents set to 0 in the UE configuration update procedure or registration procedure. The AMF also provides the updated allowed NSSAI and configured NSSAI to the UE.

NOTE 3:	If there is S-NSSAI location validity information for both the S-NSSAI to be replaced and the alternative S-NSSAI, the NS-AoS of the alternative S-NSSAI is the same as or larger than the NS-AoS of the S-NSSAI to be replaced. If the NS-AoS of the alternative S-NSSAI is larger than the NS-AoS of the S-NSSAI to be replaced, the UE can request the establishment of user plane resources of PDU session(s) associated with the alternative S-NSSAI in the cell(s) outside the NS-AoS of the S-NSSAI to be replaced but within the NS-AoS of the alternative S-NSSAI.

NOTE 4:	It is recommended that the alternative S-NSSAI is able to support service requirements supported by the S-NSSAI to be replaced.

* * * End of Change * * * *