3GPP TSG-CT WG1 Meeting #148	C1-242149

# Changsha, China, 15 -19 April 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.577 | CR | 0002 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on IANA media type |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Media type "application/vnd.3gpp.5gsa2x-local-service-information" is registered in IANA. Please see http://www.iana.org/assignments/media-types However, there are typo of media type "vnd.3gpp-5gsa2x-local-service-information" in several places. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct media type terminology to "vnd.3gpp.5gsa2x-local-service-information" |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong media type can be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12A.5.1, 12A.5.6, 12A.5.7, 12A.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** First change **********

### 12A.5.1	Minimum components of A2X AS MBS configuration SDP

The A2X AS MBS configuration SDP shall contain at least the following parameters:

a)	IP multicast address used for A2X application server discovery using MBS;

b)	transport protocol used for A2X application server discovery using MBS, set to UDP;

c)	UDP port number used for A2X application server discovery using MBS;

d)	media type set to "application"; and

e)	media format set to "vnd.3gpp.5gsa2x-local-service-information".

These parameters shall be expressed in SDP syntax (see IETF RFC 4566 [31]) according to the following clauses.

********** Next change **********

### 12A.5.6	Media format

The media format shall be defined according to the <fmt> sub-field of the media announcement field ("m=") of IETF RFC 4566 [31] and shall be set to "vnd.3gpp.5gsa2x-local-service-information".

********** Next change **********

### 12A.5.7	Example of A2X AS MBS configuration SDP

Here is an example of a A2X AS MBS configuration SDP:

v=0

o=user123 2890844526 2890842807 IN IP6 2201:056D::112E:144A:1E24

s=A2X AS MBS configuration SDP example

c=IN IP6 FF15::101
m=application 1234 UDP vnd.3gpp.5gsa2x-local-service-information

********** Next change **********

### 12A.6.2	application/vnd.3gpp.5gsa2x-local-service-information

The MIME type is used to carry information related to the local A2X application server, and optionally the A2X MBS configuration. It shall be coded in binary ASN.1 PER as specified below.

-- ASN1START

A2X-local-service-information-definitions DEFINITIONS AUTOMATIC TAGS ::= BEGIN

A2X-local-service-information ::= SEQUENCE {

a2x-as-info-list				ListOfA2X-as-info,

a2x-mbs-configuration-list		ListOfA2X-MBS-configuration		OPTIONAL,

...

}

ListOfA2X-as-info ::= SEQUENCE OF A2X-as-info

ListOfA2X-MBS-configuration ::= SEQUENCE OF A2X-MBS-configuration

A2X-as-info ::= SEQUENCE {

a2x-service-identifier			OCTET STRING (SIZE (4)),

a2x-as-address					A2X-as-address,

udp-port						INTEGER,

...

}

A2X-MBS-configuration ::= SEQUENCE {

a2x-service-identifier			OCTET STRING (SIZE (4)),

tmgi							OCTET STRING (SIZE (6)),

mbs-service-type				MBS-service-type,

mbs-service-area				List-of-MBS-service-area-parameters			OPTIONAL,

frequency-selection-parameter	Frequency-assistance-information			OPTIONAL,

ip-multicast-address			IP-multicast-address,

udp-port						INTEGER,

...

}

MBS-service-type ::= ENUMERATED {

multicast	(0),

broadcast	(1),

...

}

Frequency-assistance-information ::= SEQUENCE {

frequency		INTEGER (0..3279165)	OPTIONAL,

mbs-fsai-list 	MBS-FSAI-List			OPTIONAL

}

MBS-FSAI-List ::= SEQUENCE OF MBS-FSAI

MBS-FSAI ::= OCTET STRING (SIZE (3))

List-of-MBS-service-area-parameters ::= SEQUENCE OF MBS-service-area-parameter

MBS-service-area-parameter ::= CHOICE {

tai		OCTET STRING (SIZE (6)),

cell-id	OCTET STRING (SIZE (8)),

...

}

IP-multicast-address ::= CHOICE {

ipv4-address		OCTET STRING (SIZE (4)),

ipv6-address		OCTET STRING (SIZE (16)),

ipv4v6-address		IPv4v6-address,

...

}

A2X-as-address ::= CHOICE {

ipv4-address		OCTET STRING (SIZE (4)),

ipv6-address		OCTET STRING (SIZE (16)),

ipv4v6-address		IPv4v6-address,

fqdn				VisibleString,

...

}

IPv4v6-address ::= SEQUENCE {

ipv4-address		OCTET STRING (SIZE (4)),

ipv6-address		OCTET STRING (SIZE (16)),

...

}

END

-- ASN1STOP

********** End of change **********