3GPP TSG-CT WG1 Meeting #149	C1-243305

# Hyderabad, India, 27-31 May 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.502 | CR | 0305 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on optionality of AN-parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., Nokia, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  | Date: |  |  | 2024-05-27 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS 23.502 4.12.2.2 only specified AN-parameter shall be contained in step 5 which is the first EAP-Response/5G-NAS message. If the AN-parameter is not marked as optional then the UE has to include it in the following EAP-Response/5G-NAS messages even though it’s actually not needed. Extended-AN-parameters after R17 is only used when at least one access network parameter is longer than 255 octets which means it’s an optional field. Extensions is optional according to Table 9.3.2.2.2-1. However, the octets for extensions is not marked as optional (*) in Figure 9.3.2.2.2-1. Additionally, Extensions is used to ensure that the receiving entity ignores fields defined in future versions of the specification. Thus, the sending entity is not expected to include Extensions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the AN-parameters in EAP-Response/5G-NAS message IE to optional, therefore the UE can set the AN-parameters length to 0 and send the message without AN-parameter when this field is not needed. The sending entity does not include Extensions. Mark Extended-AN-parameters after R17 and extensions as optional in Figure 9.3.2.2.2-1. Backward compatibility analysis: The CR is backward compatible. For AN-parameters, there is no possible way to decode or encode EAP-Response/5G-NAS message without AN-parameters for entities without implementing this CR. Entities implementing this CR can work fine. For Extended-AN-parameters after R17, it shall be an optional field because it's introduced in R17 and was optional field before R17 (as extension field). The R17 and after UE includes the field only when at least one access network parameter is longer than 255 (decimal) octets according to the current spec. The UE before R17 won’t include it at all. The network allows this field to be optional because it may operate with the old UE (e.g., R15 UE). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE may be unsure about which AN-parameter type field to indicate in the EAP-Response/5G-NAS message when it actually has no additional information to provide. The network doesn’t know how to decode a message without AN-parameters. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** first change *****

9.3.2.2.2	EAP-Response/5G-NAS message

EAP-Response/5G-NAS message is coded as specified in figure 9.3.2.2.2-1 and table 9.3.2.2.2-1.

| Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 | Octets |
| Code |  |  |  |  |  |  |  | 1 |
| Identifier |  |  |  |  |  |  |  | 2 |
| Length |  |  |  |  |  |  |  | 3 - 4 |
| Type |  |  |  |  |  |  |  | 5 |
| Vendor-Id |  |  |  |  |  |  |  | 6 - 8 |
| Vendor-Type |  |  |  |  |  |  |  | 9 - 12 |
| Message-Id |  |  |  |  |  |  |  | 13 |
| Spare |  |  |  |  |  |  |  | 14 |
| AN-parameters length |  |  |  |  |  |  |  | 15-16 |
| AN-parameters |  |  |  |  |  |  |  | 17* - (17+x)* |
| NAS-PDU length |  |  |  |  |  |  |  | y - (y+1) |
| NAS-PDU |  |  |  |  |  |  |  | (y+2) - (y+n) |
| Extensions |  |  |  |  |  |  |  | (y+n+1)* - z* |

Figure 9.3.2.2.2-1: EAP-Response/5G-NAS message

Table 9.3.2.2.2-1: EAP-Response/5G-NAS message

Code field is set to 2 (decimal) as specified in IETF RFC 3748 [9] subclause 4.1 and indicates response.

Identifier field is set as specified in IETF RFC 3748 [9] subclause 4.1.

Length field is set as specified in IETF RFC 3748 [9] subclause 4.1 and indicates the length of the EAP-Response/5G-NAS message in octets.

Type field is set to 254 (decimal) as specified in IETF RFC 3748 [9] subclause 5.7 and indicates the expanded type.

Vendor-Id field is set to the 3GPP Vendor-Id of 10415 (decimal) registered with IANA under the SMI Private Enterprise Code registry.

Vendor-Type field is set to EAP-5G method identifier of 3 (decimal) as specified in 3GPP TS 33.402 [10] annex C.

Message-Id field is set to 5G-NAS-Id of 2 (decimal).

Spare field consists of spare bits.

AN-parameters length indicates the length of the AN-parameters field in octets. If the AN-parameters length field is set to zero value, the AN-parameters field is absent. If the AN-parameters length field is set to a non-zero value, the AN-parameters field is present.

AN-parameters field is coded according to figure 9.3.2.2.2-2 and table 9.3.2.2.2-2.

NAS-PDU length field indicates the length of NAS-PDU field in octets. The NAS-PDU length field is set to a non-zero value.

NAS-PDU field contains a NAS message from the UE as specified in 3GPP TS 24.501 [4].

Extensions field is an optional field and consists of spare bits. The sending entity shall not include the Extensions field.

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AN-parameter 1 |  |  |  |  |  |  |  | octet 17 octet a |
| AN-parameter 2 |  |  |  |  |  |  |  | octet (a+1)* octet b* |
| ... |  |  |  |  |  |  |  | octet (b+1)* octet k* |
| AN-parameter n |  |  |  |  |  |  |  | octet (k+1)* octet (17+x)* |

Figure 9.3.2.2.2-2: AN-parameters field

Table 9.3.2.2.2-2: AN-parameters field

Each AN-parameter field is coded according to figure 9.3.2.2.2.1-3 and table 9.3.2.2.2-3.

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AN-parameter type |  |  |  |  |  |  |  | octet a+1 |
| AN-parameter length |  |  |  |  |  |  |  | octet a+2 |
| AN-parameter value |  |  |  |  |  |  |  | octet (a+3)* octet b* |

Figure 9.3.2.2.2-3: AN-parameter field

Table 9.3.2.2.2-3: AN-parameter field

The AN-parameter length field indicates the length of the AN-parameter value field. If the AN-parameter length field is set to zero value, the AN-parameter value field is absent. If the AN-parameter length field is set to non-zero value, the AN-parameter value field is present.

The AN-parameter type field indicates the type of the AN-parameter value field. Sending entity shall not set the AN-parameter type field to a spare value. Receiving entity shall ignore any AN-parameter field with the AN-parameter type field set to a spare value.

The following AN-parameter type field values are specified: - 01H (GUAMI); - 02H (selected PLMN ID); - 03H (requested NSSAI); - 04H (establishment cause for non-3GPP access); - 05H (selected NID); and - 06H (UE identity). All other values of the AN-parameter type field are spare. Receiving entity shall ignore an AN-parameter field with the AN-parameter type field set to a spare value.

When the AN-parameter type field indicates the GUAMI, the AN-parameter value field is coded as value part (as specified in 3GPP TS 24.007 [22] for type 3 information element) of GUAMI information element as specified in subclause 9.2.1.

When the AN-parameter type field indicates the selected PLMN ID, the AN-parameter value field is coded according to value part of PLMN ID information element as specified in subclause 9.2.3.

When the AN-parameter type field indicates the requested NSSAI, the AN-parameter value field is coded according to value part of NSSAI information element as specified in subclause 9.11.3.37 of 3GPP TS 24.501 [4].

When the AN-parameter type field indicates the establishment cause for non-3GPP access, the AN-parameter field is coded as value part (as specified in 3GPP TS 24.007 [22] for type 3 information element) of the Establishment cause for non-3GPP access information element as specified in subclause 9.2.2.

When the AN-parameter type field indicates the selected NID, the AN-parameter value field is coded according to the value part of the NID information element as specified in subclause 9.2.7.

When the AN-parameter type field indicates the UE identity, the AN-parameter value field is coded according to 5GS mobile identity information element for type of identity 5G-GUTI or for type of identity SUCI as specified in subclause 9.11.3.4 of 3GPP TS 24.501 [4].

***** End of change *****