3GPP TSG-CT WG1 Meeting #149	C1-243413

# Hyderabad, IN, 27th – 31st May 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.587 | CR | 0302 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Size restriction for ASN.1 “VisibleString” type of FQDN in V2X-as-address of encoding of V2X local service information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_MBS4V2X |  |  |  |  |  | Date: |  |  | 2024-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The CP-240239(CR#0286) defined encoding of V2X location service information as below and was agreed. V2X-as-address ::= CHOICE { ipv4-address OCTET STRING (SIZE (4)), ipv6-address OCTET STRING (SIZE (16)), ipv4v6-address IPv4v6-address, fqdn VisibleString, ... } The “fqdn” was defined as “VisibleString” type without size restriction. Sometimes, when using invalid messages, setting clear limitations on the actual messags (e.g. fqdn) type can make the UE / network safer. For example, in ASN.1, even if a value larger than the actual fqdn size is entered, it cannot be handled. If it is an ASN.1 value that is not actually defined, an error return is given, but in this case, this can not be done. Therefore, we want to impose size restrictions on VisibleString type. The ASN.1 VisibleString type supports a subset of ASCII characters that does not include control characters. The VisibleString type can be constrained by a single value, by size constraints, and by permitted alphabet constraints. According to the RFC1035, various objects and parameters in the DNS have size limits. Size limits of names, which means fqdn, is 255 octets or less. Therefore, we would like to change “VisibelString” type to “VisibleString(SIZE (1..255))” in order to use VisibleString type with size restriction. |  |  |  |  |  |  |  |  |
| Summary of change: |  | “VisibelString” type is changed to “VisibleString(SIZE (1..255))” in order to use VisibleString type with size restriction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Even if a value larger than the actual fqdn size is entered, ASN.1 cannot handle it. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.6.2	application/vnd.3gpp-5gsv2x-local-service-information

The MIME type is used to carry information related to the local V2X application server, and optionally the V2X MBS configuration. It shall be coded in binary ASN.1 PER as specified below.

-- ASN1START

V2X-local-service-information-definitions DEFINITIONS AUTOMATIC TAGS ::= BEGIN

V2X-local-service-information ::= SEQUENCE {

v2x-as-info-list				ListOfV2X-as-info,

v2x-mbs-configuration-list		ListOfV2X-MBS-configuration		OPTIONAL,

...

}

ListOfV2X-as-info ::= SEQUENCE OF V2X-as-info

ListOfV2X-MBS-configuration ::= SEQUENCE OF V2X-MBS-configuration

V2X-as-info ::= SEQUENCE {

v2x-service-identifier			OCTET STRING (SIZE (4)),

v2x-as-address					V2X-as-address,

udp-port						INTEGER,

...

}

V2X-MBS-configuration ::= SEQUENCE {

v2x-service-identifier			OCTET STRING (SIZE (4)),

tmgi							OCTET STRING (SIZE (6)),

nid								OCTET STRING (SIZE (6))	OPTIONAL,

mbs-service-type				MBS-service-type,

mbs-service-area				List-of-MBS-service-area-parameters			OPTIONAL,

frequency-selection-parameter	Frequency-assistance-information			OPTIONAL,

ip-multicast-address			IP-multicast-address,

udp-port						INTEGER,

...

}

MBS-service-type ::= ENUMERATED {

multicast	(0),

broadcast	(1),

...

}

Frequency-assistance-information ::= SEQUENCE {

frequency		INTEGER (0..3279165)	OPTIONAL,

mbs-fsai-list 	MBS-FSAI-List			OPTIONAL

}

MBS-FSAI-List ::= SEQUENCE OF MBS-FSAI

MBS-FSAI ::= OCTET STRING (SIZE (3))

List-of-MBS-service-area-parameters ::= SEQUENCE OF MBS-service-area-parameter

MBS-service-area-parameter ::= CHOICE {

tai		OCTET STRING (SIZE (6)),

cell-id	OCTET STRING (SIZE (8)),

...

}

IP-multicast-address ::= CHOICE {

ipv4-address		OCTET STRING (SIZE (4)),

ipv6-address		OCTET STRING (SIZE (16)),

ipv4v6-address		IPv4v6-address,

...

}

V2X-as-address ::= CHOICE {

ipv4-address		OCTET STRING (SIZE (4)),

ipv6-address		OCTET STRING (SIZE (16)),

ipv4v6-address		IPv4v6-address,

fqdn				VisibleString (SIZE (1..255)),

...

}

IPv4v6-address ::= SEQUENCE {

ipv4-address		OCTET STRING (SIZE (4)),

ipv6-address		OCTET STRING (SIZE (16)),

...

}

END

-- ASN1STOP

* * * End of Changes * * * *