3GPP TSG-CT WG1 Meeting #64	C1-101409

# Kyoto (Japan), 10-14 May 2010

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.237 | CR | 0268 | rev | - | Current version: | 9.2.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Infineon Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | IMS_SCC-SPI |  |  |  |  |  |  | Date: |  | 30/04/2010 |
| Category: | D |  |  |  |  |  |  | Release: |  | Rel-9 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change: |  | Editorial errors. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear and hard to read specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.11, A.12, A.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * Changes * * * *

# A.11	Signalling flows for establishment of collaborative session for inter-UE transfer

## A.11.1	Introduction

This clause describes signalling flows for establishing a collaborative session. Two different scenarios have been considered:

-	the first scenario is when the collaborative session is established by transferring a media component from the controller UE to a controllee UE. This scenario is similar to the procedures described in subclause A.12.2 with exception that upon receipt of the SIP REFER request from the controller UE, the SCC AS generates a SIP INVITE request instead of a SIP re-INVITE request and sends it to the controllee UE; and

-	the second scenario is described in subclause A.11.2 and shows an example where the collaborative session is established by adding a new media component on a controllee UE.

## A.11.2	Collaborative session establishment with new media

There is an existing session with audio between controller UE, UE-1, and the remote UE. The controller UE establishes a collaborative session by adding a video media component to the controllee UE, UE-2.

Figure A.11.2: Controller UE establishes a collaborative session by adding a new media on controllee UE

NOTE:	For clarity, the SIP 100 (Trying) responses are not shown in the signalling flow.

1-2.	SIP REFER request (from UE-1 to SCC-AS)

The controller UE, UE-1 sends a SIP REFER request to the SCC AS containing a Refer-To header field containing the GRUU of controllee UE, UE-2 and a body parameter containing an m line for audio set to zero and an m line for video with the port number set to the discard port number "9" since the port number is unknown. The SIP REFER request also includes a Target-dialog header field containing the details of the dialog for the existing session between controller UE, UE-1 and the remote UE.

Table A.11.2-1 SIP REFER request (UE-1 to SCC-AS)

REFER sip:interUEtransfer@sccas1.home1.net SIP/2.0

Via:

To: sip:interUEtransfer@sccas1.home1.net

From: sip:user1_public1@home1.net;tag=24680

Call-ID: cb03a0s09a2sdfglkj490333

CSeq: 93809824 REFER

Max-Forwards: 70

P-Preferred-Identity:

Refer-To: <sip:user1_public2@home2.net;gr=urn:uuid:f81d4fae-7dec-11d0-a762-00a0c91e6bf6?body=m%3Daudio%200%20RTP%2FAVP%2096%0Dm%3Dvideo%209%20RTP%2FAVP%2098%2099>

Require: target-dialog

Target-dialog: cb03a0s09a2sdfglkj13579; remote-tag=abcdef; local-tag=123456

Contact: <sip:user1_public1@home1.net; gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91ewxyz>;+g.3gpp.iut-controller

Allow:

Accept: message/sipfrag

Content-Length: 0

3-4.	SIP 202 (Accepted) response (from SCC AS to UE-1)

The SCC-AS sends a SIP 202 (Accepted) response to controller UE-1 as response to the SIP REFER request.

5-6.	SIP NOTIFY request (from SCC AS to UE-1)

The SCC-AS sends a SIP NOTIFY request to UE-1 notifying implicit subscription to the SIP REFER request.

Table A.11.2-2 SIP NOTIFY request (SCC-AS to UE-1)

NOTIFY

Via:

To: sip:user1_public1@home1.net;tag=24680

From: sip:interUEtransfer@sccas1.home1.net;tag=13579

Call-ID:

CSeq:

Max-Forwards:

P-Asserted-Identity:

Require:

Contact: sip: scc-as@home1.net

Allow:

Event: refer

Subscription-State: active;expires=3600

Content-Type: message/sipfrag;version=2.0

Content-Length: (…)

SIP/2.0 100 Trying

7-8.	SIP 200 (OK) response (from UE-1 AS to SCC-AS)

The controller UE, UE-1, acknowledges the SIP NOTIFY request by sending a SIP 200 (OK) response to the SCC-AS.

9-10.	SIP INVITE request (from SCC-AS to UE-2)

The SCC-AS sends a SIP INVITE request to the controllee UE, UE-2, adding video media and establishing a collaborative session. Since the URI parameters indicate that the port number for the video m-line is set to the discard port number "9", the SCC AS realizes that the port number of the remote UE is unknown and therefore adds an a-line to inactive in the SDP offer to prevent the controllee UE sending media to the remote UE. The SDP offer contains the audio media component on controller UE, UE-1 set to zero. The SIP INVITE request contains a Referred-By header field containing the identity of UE-1.

Table A.11.2-3 SIP INVITE request (SCC-AS to UE-2)

INVITE sip:user2_public1@home1.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91e6bf6 SIP/2.0

Via:

Record-Route: sip:scc-as@home1.net

To: sip:user2_public1@home1.net;

From: sip:user3_public3@home3.net;tag=acegi

Call-ID:

CSeq:

Max-Forwards:

P-Asserted-Identity: "remote user" sip:user3_public3@home3.net

Require:

Referred-By: sip:user1_public1@home1.net

Contact: sip:user3_public3@home3.net;gr=urn:uuid:f81d4fae-17oct-11a1-a678-0054c91eabcd

Allow:

Content-Type: application/sdp

Content-Length:(…)

v=0

o=- 1027933615 1027933615 IN IP4 123.112.67.87

s=-

t=0 0

m=audio 0 RTP/AVP 0

m=video 9 RTP/AVP 98 99

a=inactive

c=123.112.67.87

b=AS:75

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

a=rtpmap:99 MP4V-ES

Editor's note:	The value of c-line is FFS.

11-12.	SIP 200 (OK) response (from UE-2 to SCC-AS)

The controllee UE, UE-2, acknowledges the SIP INVITE request by sending a SIP 200 (OK) response to the SCC-AS.

Table A.11.2-11 SIP 200 (OK) response (UE-2 to SCC-AS)

SIP/2.0 200 OK

Via:

To: sip:user2_public1@home1.net;tag=xyzwv

From: sip:user3_public3@home3.net;tag=acegi

Call-ID:

CSeq:

P-Preferred-Identity:

Contact: sip:user1_public2@home2.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91e6bf6+g.3gpp.iut-controller

Allow: INVITE, PRACK, UPDATE

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 145.23.77.88

s=-

t=0 0

m=audio 0 RTP/AVP 0

m=video 9 RTP/AVP 98

a=inactive

c=145.23.77.88

b=AS:75

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

13-14.	SIP ACK request (from SCC-AS to controllee UE)

The SCC-AS sends a SIP ACK request to the remote UE.

15-16.	SIP re-INVITE request (from SCC-AS to remote UE)

The SCC-AS sends a SIP re-INVITE request to the remote UE.

Table A.11.2-15 SIP re-INVITE request (SCC-AS to remote UE)

INVITE sip:user3_public3@home3.net;gr=urn:uuid:f81d4fae-17oct-11a1-a678-0054c91eabcd SIP/2.0

Via:

To: sip:user3_public3@home3.net;tag=66666

From: sip:user1_public1@home1.net;tag=33333

Call-ID:

CSeq:

Max-Forwards:

P-Asserted-Identity:

Require:

Contact: sip: user1_public1@home1.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91ewxyz

Allow:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 123.45.67.89

s=-

t=0 0

m=audio 1300 RTP/AVP 96 97

c=IN IP4 123.45.67.89

b=AS:25.4

a=rtpmap:96 AMR

a=fmtp:96 mode-set=0,2,5,7; mode-change-period=2

a=rtpmap:97 telephone-event

a=maxptime:20

m=video 1302 RTP/AVP 98

c=IN IP4 145.23.77.88

b=AS:75

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

17-18.	SIP 200 (OK) response (from remote UE to SCC-AS)

The remote UE acknowledges the SIP re-INVITE request by sending a SIP 200 (OK) response to the SCC-AS.

Table A.11.2-17 SIP 200 (OK) response (remote UE to SCC-AS)

SIP/2.0 200 OK

Via:

To:

From:

Call-ID:

CSeq:

P-Asserted-Identity:

Contact: sip:user3_public3@home3.net;gr=urn:uuid:f81d4fae-17oct-11a1-a678-0054c91eabcd

Allow:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 123.112.67.87

s=-

c=IN IP4 123.112.67.87

t=0 0

m=audio 3000 RTP/AVP 96 97

b=AS:25.4

a=rtpmap:96 AMR

a=fmtp:96 mode-set=0,2,5,7; mode-change-period=2

a=rtpmap:97 telephone-event

a=maxptime:20

m=video 3002 RTP/AVP 98

b=AS:75

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

19-20.	SIP ACK request (from SCC-AS to remote UE)

The SCC-AS sends a SIP ACK request to the remote UE.

21-22.	SIP re-INVITE request (from SCC-AS to UE-2)

The SCC-AS sends a SIP re-INVITE request to the controllee UE, UE-2 to inform the controllee UE about the port number for the video media component of the remote UE. The SCC AS adds an a-line set to active in the SDP offer.

NOTE 1:	This SIP re-INVITE request is triggered by the SIP REFER request in steps 1-2. The previous SIP INVITE request was generated by the SCC AS due to third party call control to allow sending this SIP re-INVITE request.

NOTE 2:	Any other changes such as the IP address of the remote UE in case the remote UE uses different IP addresses for different media would also be updated in the SIP re-INVITE request.

Table A.11.2-21 SIP re-INVITE request (SCC-AS to UE-2)

INVITE sip:user1_public2@home2.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91e6bf6 SIP/2.0

Via:

To:

From:

Call-ID:

CSeq:

Contact: sip:user3_public3@home3.net;gr=urn:uuid:f81d4fae-17oct-11a1-a678-0054c91eabcd

Allow:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 123.45.67.87

s=-

c=IN IP4 123.45.67.87

t=0 0

m=audio 0 RTP/AVP 96 97

m=video 3002 RTP/AVP 98

b=AS:75

a=active

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

23-24.	SIP 200 (OK) response (from controllee UE to SCC-AS)

Table A.11.2-23 SIP 200 (OK) response (UE-2 to SCC-AS)

SIP/2.0 200 OK

Via:

To:

From:

Call-ID:

CSeq:

Contact: sip:user1_public2@home2.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91e6bf6

Allow:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 145.23.77.88

s=-

t=0 0

m=audio 0 RTP/AVP 0

m=video 1302 RTP/AVP 98

c=145.23.77.88

b=AS:75

a=active

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

25-26.	SIP ACK request (from SCC-AS to controllee UE)

The SCC-AS sends a SIP ACK request to the controllee UE to acknowledge.

27-28.	SIP NOTIFY request (from SCC-AS to conroller UE, UE-1)

When the media component is added to the controllee UE, UE-2, the SCC-AS sends a SIP NOTIFY request to the controller UE, UE-1 to inform about the success status of adding the media to the controllee UE, UE-2.

Table A.11.2-27 SIP NOTIFY request (SCC-AS to UE-1)

NOTIFY

Via:

To: sip:user1_public1@home1.net;tag = 13579

From: sip:scc-as@home1.net;tag=24680

Call-ID:

CSeq:

Max-Forwards:

P-Asserted-Identity:

Require:

Contact: sip: scc-as@home1.net

Allow:

Event: refer

Subscription-State:terminated; reason=noresource

Content-Type: message/sipfrag ;version=2.0

Content-Length: (…)

SIP/2.0 200 OK

Content-Type: application/sdp

m=audio 0 RTP/AVP 0

m=video 1302 RTP/AVP 98 99

c=145.23.77.88

b=AS:75

a=active

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

29-30.	SIP 200 (OK) response (from controller UE to SCC-AS)

The controller UE acknowledges the NOTIFY request by sending a SIP 200 (OK) response to the SCC-AS.

# A.12	Signalling flows for media transfer within collaborative session for inter-UE transfer

## A.12.1	Introduction

This subclause describes signalling flows for media transfer within collaborative sessions. Two different scenarios are considered in the clause:

-	subclause A.12.2 shows an example where a media component is transferred from the controller UE to the controllee UE; and

-	subclause A.12.3 shows an example where a media component is transferred from one controllee UE to another controllee UE.

## A.12.2	Controller UE initiated media transfer from controller UE to controllee UE

Figure A.12.2: Controller UE transfers a media on controllee UE

NOTE 1:	For clarity, the SIP 100 (Trying) responses are not shown in the signalling flow.

1-2.	SIP REFER request (SIP REFER request from UE-1 to SCC-AS)

There is an existing session with audio and video between controller UE, UE-1 (123.45.67.89), and remote UE (132.54.76.98). The video component is unidirectional from the remote UE to the controller UE, UE1. The Controller UE attempts to transfer the video portion of this session to the controllee UE, UE-2.

Table A.12.2-1 SIP REFER request (UE-1 to SCC-AS)

REFER sip:interUEtransfer@sccas1.home1.net SIP/2.0

Via:

To: sip:interUEtransfer@sccas1.home1.net

From: sip:user1_public1@home1.net; tag=13579sip:user1_pubic1@home1.net;tag = 24680

Call-ID: cb03a0s09a2sdfglkj490333

CSeq: 93809824 REFER

Max-Forwards: 70

P-Preferred-Identity:

Refer-To: <sip:user1_public2@home1.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91e6bf6?body= m%3Daudio%200%20RTP%2FAVP%97%0Dm%3Dvideo%203002%20RTP%2FAVP%2098%2099>

Require: target-dialog

Target-dialog: cb03a0s09a2sdfglkj321576;remote-tag=abcdef;local-tag=123456

Contact: <sip:user1_public1@home1.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91ewxyz>;+g.3gpp.iut-controller

Accept: application/sdp, message/sipfrag

Content-Length: 0

3-4.	SIP 202 (Accepted) response (from SCC AS to UE-1)

The SCC-AS sends a SIP 202 (Accepted) response to controller UE-1 as response to the SIP REFER request.

5-6.	SIP NOTIFY request (from SCC AS to UE-1)

The SCC-AS sends a SIP NOTIFY request to UE-1 to notify implicit subscription to the SIP REFER request results.

Table A.12.2-2 SIP NOTIFY request (SCC-AS to UE-1)

NOTIFY sip:user1_public1@home1.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91ewxyz SIP/2.0

Via:

To: sip:user1_public1@home1.net;tag=24680

From: sip:interUEtransfer@sccas1.home1.net;tag=13579

Call-ID:

CSeq:

Max-Forwards:

P-Asserted-Identity:

Require:

Contact: sip: scc-as@home1.net

Allow:

Event: refer

Subscription-State: active;expires=3600

Content-Type: message/sipfrag ;version=2.0

Content-Length: (…)

SIP/2.0 100 Trying

7-8.	SIP 200 (OK) response (from UE-1 AS to SCC-AS)

The controller UE, UE-1, acknowledges the SIP NOTIFY request by sending a SIP 200 (OK) response to the SCC-AS.

9-10.	SIP INVITE request (from SCC-AS to UE-2)

The SCC-AS sends a SIP INVITE request to the controllee UE, UE-2, to transfer video media.

Editor’s Note:	It is ffs how the controller UE signals what media are to be transferred when the SDP media order is different in controller UE and controllee UE.

Table A.12.2-3 SIP INVITE request (SCC-AS to UE-2)

INVITE sip:user1_public2@home1.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91e6bf6 SIP/2.0

Via:

To: sip:user1_public2@home1.net;

From: sip:scc-as@home1.net; tag=12486

Call-ID:

CSeq:

Max-Forwards:

P-Asserted-Identity:

Require:

Contact:

Allow:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 123.112.67.87

s=-

c=IN IP4 123.112.67.87

t=0 0

m=audio 0 RTP/AVP 97

m=video 3002 RTP/AVP 98 99

b=AS:75

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

a=rtpmap:99 MP4V-ES

11-12.	SIP 200 (OK) response (from UE-2 to SCC-AS)

The controllee UE, UE-2, acknowledges the SIP INVITE request by sending a SIP 200 (OK) response to the SCC-AS.

Table A.12.2-4 SIP 200 OK response (UE-2 to SCC-AS)

SIP/2.0 200 OK

Via:

To: sip:user1_public2@home1.net; tag = xyzwv

From: sip:scc-as@home1.net; tag = 12486

Call-ID:

CSeq:

P-Preferred-Identity:

Contact: sip:user1_public2@home1.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91e6bf6

Allow:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 145.23.77.88

s=-

c=145.23.77.88

t=0 0

m=audio 0 RTP/AVP 97

m=video 1302 RTP/AVP 98 99

b=AS:75

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

a=rtpmap:99 MP4V-ES

13-14.	SIP re-INVITE request (from SCC-AS to remote UE)

The SCC-AS sends a SIP re-INVITE request to the remote UE.

Table A.12.2-5 SIP INVITE request (SCC-AS to remote UE)

INVITE sip:user3_public3@home3.net SIP/2.0

Via:

To: sip:user3_public3@home3.net;tag = 66666

From: sip:scc-as@home1.net; tag=33333

Call-ID:

CSeq:

Max-Forwards:

P-Asserted-Identity:

Require:

Contact: sip:user1_public1@home1.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91ewxyz

Allow:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 123.45.67.89

s=-

t=0 0

m=audio 1300 RTP/AVP 96 97

c=IN IP4 123.45.67.89

b=AS:25.4

a=rtpmap:96 AMR

a=fmtp:96 mode-set=0,2,5,7; mode-change-period=2

a=rtpmap:97 telephone-event

a=maxptime:20

m=video 1302 RTP/AVP 98 99

c=IN IP4 145.23.77.88

b=AS:75

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

a=rtpmap:99 MP4V-ES

15-16.	SIP 200 (OK) response (from remote UE to SCC-AS)

The remote UE acknowledges the SIP re-INVITE request by sending a SIP 200 (OK) response to the SCC-AS.

Table A.12.2-6 SIP 200 (OK) response (remote UE to SCC-AS)

SIP/2.0 200 OK

Via:

To:

From:

Call-ID:

CSeq:

P-Asserted-Identity:

Contact: sip:user3_public3@home3.net

Allow:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 123.112.67.87

s=-

c=IN IP4 123.112.67.87

t=0 0

m=audio 3000 RTP/AVP 96 97

b=AS:25.4

a=rtpmap:96 AMR

a=fmtp:96 mode-set=0,2,5,7; mode-change-period=2

a=rtpmap:97 telephone-event

a=maxptime:20

m=video 3002 RTP/AVP 98 99

b=AS:75

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

a=rtpmap:99 MP4V-ES

17-18.	SIP ACK request (from SCC-AS to remote UE)

The SCC-AS sends a SIP ACK request to the remote UE.

19-20.	SIP ACK request (from SCC-AS to controllee UE; UE-2)

The SCC-AS sends a SIP ACK request to the remote UE.

21-22.	SIP re-INVITE request (from SCC-AS to controller UE; UE-1)

The SCC-AS sends a SIP re-INVITE request to the controller UE.

Table A.12.2-7 SIP INVITE request (SCC-AS to UE-1)

INVITE sip:user1_public1@home1.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91ewxyz SIP/2.0

Via:

To:

From:

Call-ID:

CSeq:

Max-Forwards:

P-Asserted-Identity:

Require:

Contact:

Allow:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 132.54.76.98

s=-

c=IN IP4 132.54.76.98

t=0 0

m=audio 2000 RTP/AVP 96 97

b=AS:25.4

a=rtpmap:96 AMR

a=fmtp:96 mode-set=0,2,5,7; mode-change-period=2

a=rtpmap:97 telephone-event

a=maxptime:20

m=video 0 RTP/AVP 98 99

a=sendonly

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

a=rtpmap:99 MP4V-ES

23-24.	SIP 200 (OK) response (from UE-1 to SCC-AS)

The controller UE acknowledges the SIP re-INVITE request by sending a SIP 200 (OK) response to the SCC-AS.

25-26.	SIP ACK request (from SCC-AS to UE-1)

The SCC-AS sends a SIP ACK request to the controlle1 UE, UE-1 in response to the SIP 200 (OK) response.

27-28.	SIP NOTIFY request (from SCC-AS to conroller UE, UE-1)

The SCC-AS sends a SIP NOTIFY request to controller UE, UE-1 to inform about the success status of the inter-UE transfer.

Table A.12.2-8 SIP NOTIFY request (SCC-AS to UE-1)

NOTIFY sip:user1_public1@home1.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91ewxyz SIP/2.0

Via:

To: sip:user1_public1@home1.net; tag=24680

From: sip:interUEtransfer@sccas1.home1.net;tag=13579

Call-ID:

CSeq:

Max-Forwards:

P-Asserted-Identity:

Require:

Contact: sip: scc-as@home1.net

Allow:

Event: refer

Subscription-State:terminated; reason=noresource

Content-Type: message/sipfrag ;version=2.0

Content-Length: (…)

SIP/2.0 200 OK

Content-Type: application/sdp

v=0

s=-

m=audio 0 RTP/AVP 97

m=video 1302 RTP/AVP 98 99

b=AS:75

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

a=rtpmap:99 MP4V-ES

29-30.	SIP 200 (OK) response (from controller UE to SCC-AS)

The remote UE acknowledges the SIP NOTIFY request by sending a SIP 200 (OK) response to the SCC-AS.

## A.12.3	Controller UE initiated media transfer from controllee UE to another controllee UE with subscription to dialog events

The signalling flow in figure A.12.3 describes the procedures for media transfer from one controllee UE to another controllee UE with presistant subscription to dialog event.

NOTE 1:	For brevity not all SIP NOTIFY requests sent as a result of the subscription to the dialog event package are shown.

Figure A.12.3: Controller UE transfers a media from one controllee UE to another controllee UE

1-2.	SIP SUBSCRIBE request (from controller UE to SCC-AS) - see example in table A.12.3-1

In order that the controller UE fetches the information about the session descriptions of the UEs within the collaborative session, the controller UE subscribes to the existing dialog between the controller UE and the SCC AS.

Table A.12.3-1 SIP SUBSCRIBE request (controller UE to SCC-AS)

SUBSCRIBE sip:scc-as@home1.net SIP/2.0

Via:

To: sip:scc-as@home1.net; tag= 24680

From: sip:user1_public1@home1.net; tag=13579

Event: dialog; call-id=" cb03a0s09a2sdfglkj321576"; from-tag="54321"; to-tag="123456"; include-session-descriptionsip:user1_pubic1@home1.net;tag = 24680

Call-ID: cb03a0s09a2sdfglkj490333

CSeq: 1 SUBSCRIBE

Max-Forwards: 70

P-Preferred-Identity:

Require: target-dialog

Expires: 3600

Target-dialog: cb03a0s09a2sdfglkj321576;remote-tag=abcdef;local-tag=123456

Contact: sip:user1_public1@home1.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91ewxyz

CSeq:

Allow:

Accept: application/dialog-info+xml

Content-Length: 0

3-4.	SIP 200 (OK) response (from SCC-AS to controller UE)

The SCC AS acknowledges the SIP SUBSCRIBE request by sending a SIP 200 (OK) response to the controller UE.

5-6.	SIP NOTIFY request (from SCC-AS to controller UE) - see example in table A.12.3-5

The SCC AS sends a SIP NOTIFY request containing SDP for the remote UE so that the controller UE has the current state of the media for the collaborative session.

Table A.12.3-5 SIP NOTIFY request (SCC-AS to controller UE)

NOTIFY sip:user1_public1@home1.net SIP/2.0

Via:

To: sip:user1_public1@home1.net; tag=13579

From: sip:scc-as@home1.net; tag= 24680sip:user1_pubic1@home1.net;tag = 24680

Call-ID:

CSeq:

Max-Forwards:

P-Asserted-Identity:

Require:

Contact: sip: scc-as@home1.net

Allow:

Event: dialog

Content-Type: application/dialog-info+xml

Content-Length: (…)

<?xml version="1.0"?>

<dialog-info xmlns="urn:ietf:params:xml:ns:dialog-info"

version="0"

state="full"

entity="sip:scc-as@home1.net">

<dialog id="xxxx" call-id="ffafa" local-tag="dd" remote-tag="ee">

<state>confirmed</state>

<local>

<identity display="controllee UE">sip:user2_public1@home1.net</identity>

<target>sip:user2_public1@home1.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91e6bf6</target>

<session-description type="application/sdp">

v=0

o=- 1027933615 1027933615 IN IP4 123.112.67.87

s=-

t=0 0

m=audio 49174 RTP/AVP 96 97

c=IN IP4 123.45.67.89

b=AS:25.4

a=rtpmap:96 AMR

a=fmtp:96 mode-set=0,2,5,7; mode-change-period=2

a=rtpmap:97 telephone-event

a=maxptime:20

m=audio 44552 RTP/AVP 96 97

c=IN IP4 123.45.67.89

b=AS:25.4

a=rtpmap:96 AMR

a=fmtp:96 mode-set=0,2,5,7; mode-change-period=2

a=rtpmap:97 telephone-event

a=maxptime:20

m=video 1009 RTP/AVP 98 99

c=IN IP4 123.112.67.87

a=sendonly

b=AS:75

a=rtpmap:98 H263

a=fmtp:98 H263

a=fmtp:98 profile-level-id=0

a=rtpmap:99 MP4V-ES

</session-description>

</local>

<remote>

<identity display="remote UE">tel:+1-237-555-2222</identity>

<target>sip:user2_public1@home2.net;gr=urn:uuid:2ad8950e-48a5-4a74-8d99-ad76cc7fc74</target>

<session-description type="application/sdp">

v=0

o=- 1027933615 1027933615 IN IP4 123.112.67.87

s=-

t=0 0

m=audio 49174 RTP/AVP 96 97

c= IN IP4 132.54.76.98

b=AS:25.4

a=rtpmap:96 AMR

a=fmtp:96mode-set=0,2,5,7; mode-change-period=2

a=rtpmap:97 telephone-event

a=maxptime:20

m=audio 44552 RTP/AVP 96 97

b=AS:25.4

a=rtpmap:96 AMR

a=fmtp:96mode-set=0,2,5,7; mode-change-period=2

a=rtpmap:97 telephone-event

a=maxptime:20

c= IN IP4 132.54.76.98

m=video 1009 RTP/AVP 98 99

c= IN IP4 132.54.76.98

a=sendonly

b=AS:75

a=rtpmap:98 H263

a=fmtp:98 H263

a=fmtp:98 profile-level-id=0

a=rtpmap:99 MP4V-ES

</session-description>

</remote>

</dialog>

</dialog-info>

7-8.	SIP 200 (OK) response (from controllee UE-2 to SCC-AS)

The controller UE-1 acknowledges the SIP NOTIFY request by sending a SIP 200 (OK) response to the SCC AS.

9-10.	SIP REFER request (UE-1 to SCC-AS) - see example in table A.12.3-9

There is an existing session with audio 1 and audio 2 between UE-2 (123.45.67.89) and the remote UE (132.54.76.98). The video component is unidirectional from the remote UE to the controllee UE, UE-3 (123.112.67.87). The Controller UE attempts to transfer the audio 1 portion of this session to the controllee UE, UE-3.

Table A.12.3-9 SIP REFER request (UE-1 to SCC-AS)

REFER sip:scc-as@home1.net SIP/2.0

Via:

To: sip:scc-as@home1.net; tag= 24680

From: sip:user1_public1@home1.net; tag=13579sip:user1_pubic1@home1.net;tag = 24680

Call-ID: cb03a0s09a2sdfglkj490333

CSeq: 93809824 REFER

Max-Forwards: 70

P-Preferred-Identity:

Refer-To: <sip:user1_public3@home1.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91e6bf6body=m%3Daudio%200%20RTP%2FAVP%200%0Dm%3Daudio%20 49174%20RTP%2FAVP%2096%0Dm%3Dvideo%201009%20RTP%2FAVP%2098%2099>

Require: target-dialog

Target-dialog: cb03a0s09a2sdfglkj321576;remote-tag=abcdef;local-tag=123456

Contact: <sip:user1_public1@home1.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91ewxyz>

Accept: message/sipfrag

Content-Length: 0

11-12.	SIP 202 (Accepted) response (from SCC AS to UE-1)

The SCC-AS sends a SIP 202 (Accepted) response to controller UE-1 as response to the SIP REFER request.

13-14.	SIP NOTIFY request (from SCC AS to UE-1) - see example in table A.13.3-13

The SCC-AS sends a SIP NOTIFY request to UE-1 to notify implicit subscription to the SIP REFER request results.

Table A.12.3-13 SIP NOTIFY request (SCC-AS to UE-1)

NOTIFY

Via:

To: sip:user1_public1@home1.net;tag=24680

From: sip:scc-as@home1.net;tag=13579

Call-ID:

CSeq:

Max-Forwards:

P-Asserted-Identity:

Require:

Contact: sip: scc-as@home1.net

Allow:

Event: refer

Subscription-State: active;expires=3600

Content-Type: message/sipfrag;version=2.0

Content-Length: (…)

SIP/2.0 100 Trying

15-16.	SIP 200 (OK) response (from UE-1 AS to SCC-AS)

The controller UE, UE-1, acknowledges the SIP NOTIFY request by sending a SIP 200 (OK) response to the SCC-AS.

17-18.	SIP INVITE request (from SCC-AS to UE-3) - see example in table A.12.3-17

Since the message 9-10 contains a Refer-to header field addressed to UE-3 and the URI paramaters, listing an audio line which is not currently supported by another controllee UE than UE-2, the SCC AS realizes the procedure is for transferring the media from that controllee UE (UE-2) to controllee UE (UE-3). The SCC-AS sends a SIP INVITE request to the controllee UE, UE-3, to transfer the audio media component. The SDP in the SIP INVITE request lists the media  lines within the collaborative session. In order to avoid UE-3 to start sending audio to the remote UE, the SCC-AS adds an a-line to inactive in the SDP offer.

Table A.12.3-17 SIP INVITE request (SCC-AS to UE-3)

INVITE sip:user1_public3@home1.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91e6bf6 SIP/2.0

Via:

To: sip:user1_public3@home1.net;

From: sip:scc-as@home1.net; tag=12486

Call-ID:

CSeq:

Max-Forwards:

P-Asserted-Identity:

Require:

Contact:

Allow:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 132.54.76.98

s=-

c=IN IP4 132.54.76.98

t=0 0

m=audio 49174 RTP/AVP 96 97

a=rtpmap:0 PCMU/8000

a= sendonly

b=RR:0

b=RS:0m=audio 0 RTP/AVP 0

m=video 1009 RTP/AVP 98 99

a=sendonly

b=AS:75

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

a=rtpmap:99 MP4V-ES

19-20.	SIP 200 (OK) response (from UE-3 to SCC-AS) - see example in table A.12.3-19

The controllee UE, UE-3, acknowledges the SIP INVITE request by sending a SIP 200 (OK) response to the SCC-AS.

Table A.12.3-19 SIP 200 (OK) response (UE-3 to SCC-AS)

SIP/2.0 200 OK

Via:

To: sip:user1_public3@home1.net; tag = xyzwv

From: sip:scc-as@home1.net; tag = 12486

Call-ID:

CSeq:

P-Preferred-Identity:

Contact: sip:user1_public3@home1.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91e6bf6

Allow:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 123.112.67.87

s=-

c=123.112.67.87

t=0 0

m=audio 3002 RTP/AVP 96 97

a=rtpmap:0 PCMU/8000

a=recvonly

m=audio 0 RTP/AVP 0

m=video 1302 RTP/AVP 98 99

a=recvonly

b=AS:75

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

a=rtpmap:99 MP4V-ES

21-22.	SIP ACK request (from SCC-AS to UE-3)

The SCC-AS sends a SIP ACK request to UE-3 to acknowlege.

23-24.	SIP re-INVITE request (from SCC-AS to controllee UE, UE-2) - see example in table A.12.3-23

The SCC AS sends a SIP re-INVITE request to controllee UE, UE-2 to put Audio 1 on hold.

Table A.12.3-23 SIP INVITE request (SCC-AS to UE-2)

INVITE sip:user1_public2@home3.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91e6bf6 SIP/2.0

Via:

To: sip:user1_public2@home1.net;

From: sip:scc-as@home1.net; tag=12386

Call-ID:

CSeq:

Max-Forwards:

P-Asserted-Identity:

Require:

Contact:

Allow:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 123.112.67.87

s=-

c=IN IP4 123.112.67.87

t=0 0

m=audio 49174 RTP/AVP 96 97

a=sendonly

b=RR:0

b=RS:0

m=audio 44552 RTP/AVP 96 97

a=rtpmap:0 PCMU/8000

a=sendonly

b=RR:0

25-26.	SIP 200 (OK) response (from UE-2 to SCC-AS) - see example in table A.12.3-25

The controllee UE, UE-2, acknowledges the SIP INVITE request by sending a SIP 200 (OK) response to the SCC-AS.

Table A.12.3-25 SIP 200 (OK) response (UE-2 to SCC-AS)

SIP/2.0 200 OK

Via:

To: sip:user1_public2@home1.net; tag = xyzwv

From: sip:scc-as@home1.net; tag = 12486

Call-ID:

CSeq:

P-Preferred-Identity:

Contact: sip:user1_public2@home1.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91e6bf6

Allow:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 123.45.67.89

s=-

c=IN IP4 123.45.67.89

t=0 0

m=audio 32324 RTP/AVP 96 97

a=recvonly

m=audio 34002 RTP/AVP 96 97

a=rtpmap:0 PCMU/8000

a=recvonly

27-28.	SIP ACK request (from  SCC-AS to UE-2 )

The SCC –AS sends a SIP ACK request to UE-2 to acknowlege.

29-30	SIP re-INVITE request (from SCC-AS to remote UE) - see example in table A.12.3-29

The SCC-AS sends a SIP re-INVITE request to the remote UE.

Table A.12.3-29 SIP INVITE request (SCC-AS to remote UE)

INVITE sip:user2_public1@home3.net;SIP/2.0

Via:

To: sip:user2_public1@home2.net;tag=66666

From: sip:scc-as@home1.net; tag=33333

Call-ID:

CSeq:

Max-Forwards:

P-Asserted-Identity:

Require:

Contact:sip:user1_public1@home1.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91ewxyz

Allow:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 123.112.67.87

s=-

t=0 0

m=audio 3002 RTP/AVP 96 97

c= IN IP4 123.112.67.87

a=rtpmap:0 PCMU/8000

m=audio 34002 RTP/AVP 96 97

C=IN IP4 123.45.67.89

A=rtpmap:0 PCMU/8000

m=video 1302 RTP/AVP 98 99

c= IN IP4 123.112.67.87

b=AS:75

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

a=rtpmap:99 MP4V-ES

a=recvonly

31-32.	SIP 200 (OK) response (from remote UE to SCC-AS) - see example in table A.12.3-31

The remote UE acknowledges the SIP re-INVITE request by sending a SIP 200 (OK) response to the SCC-AS.

Table A.12.3-31 SIP 200 (OK) response (remote UE to SCC-AS)

SIP/2.0 200 OK

Via:

To:

From:

Call-ID:

CSeq:

P-Asserted-Identity:

Contact: sip:user2_public1@home2.net;

Allow:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 132.54.76.98

s=-

c= IN IP4 132.54.76.98

t=0 0

m=audio 49174 RTP/AVP 96 97

b=AS:25.4

a=rtpmap:96 AMR

a=fmtp:96mode-set=0,2,5,7; mode-change-period=2

a=rtpmap:97 telephone-event

a=maxptime:20

m=audio 44552 RTP/AVP 96 97

a=rtpmap:0 PCMU/8000

m=video 1009 RTP/AVP 98 99

a=sendonly

b=AS:75

a=rtpmap:98 H263

a=fmtp:98 H263

a=fmtp:98 profile-level-id=0

a=rtpmap:99 MP4V-ES

a=sendonly

33-34.	SIP ACK request (from SCC-AS to remote UE)

The SCC-AS sends a SIP ACK request to the remote UE.

35-36.	SIP NOTIFY request (from SCC-AS to controller UE) - see example in table A.12.3-35

The SCC AS sends SIP NOTIFY request containing SDP for the remote UE so that the controller UE can be aware about the change of state of the media for the collaborative session.

Table A.12.3-35 SIP NOTIFY request (SCC-AS to controller UE)

NOTIFY sip:user1_public1@home1.net;

Via:

To: sip:user1_public1@home1.net; tag=13579

From: sip:scc-as@home1.net; tag= 24680sip:user1_pubic1@home1.net;tag = 24680

Call-ID:

CSeq:

Max-Forwards:

P-Asserted-Identity:

Require:

Contact: sip: scc-as@home1.net

Allow:

Event: dialog

Content-Type: application/dialog-info+xml

Content-Length: (…)

<?xml version="1.0"?>

<dialog-info xmlns="urn:ietf:params:xml:ns:dialog-info"

version="1"

state="full"

entity="sip:scc-as@home1.net">

<dialog id="xxxx" call-id="ffafa" local-tag="dd" remote-tag="ee">

<state>confirmed</state>

<local>

<identity display="controllee UE">sip:user2_public1@home1.net</identity>

<target>sip:user2_public1@home1.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91e6bf6</target>

<session-description type="application/sdp">

v=0

o=- 1027933615 1027933615 IN IP4 123.112.67.87

s=-

t=0 0

m=audio 49174 RTP/AVP 96 97

c=IN IP4 123.45.67.89

b=AS:25.4

a=rtpmap:96 AMR

a=fmtp:96mode-set=0,2,5,7; mode-change-period=2

a=rtpmap:97 telephone-event

a=maxptime:20

m=audio 44552 RTP/AVP 96 97

c=IN IP4 123.45.67.89

b=AS:25.4

a=rtpmap:96 AMR

a=fmtp:96mode-set=0,2,5,7; mode-change-period=2

a=rtpmap:97 telephone-event

a=maxptime:20

m=video 1009 RTP/AVP 98 99

c=IN IP4 123.112.67.87

a=sendonly

b=AS:75

a=rtpmap:98 H263

a=fmtp:98 H263

a=fmtp:98 profile-level-id=0

a=rtpmap:99 MP4V-ES

</session-description>

</local>

<remote>

<identity display="remote UE">tel:+1-237-555-2222</identity>

<target>sip:user2_public1@home2.net;gr=urn:uuid:2ad8950e-48a5-4a74-8d99-ad76cc7fc74</target>

<session-description type="application/sdp">

v=0

o=- 1027933615 1027933615 IN IP4 123.112.67.87

s=-

t=0 0

c= IN IP4 132.54.76.98

m=audio 49174 RTP/AVP 96 97

b=AS:25.4

a=rtpmap:96 AMR

a=fmtp:96mode-set=0,2,5,7; mode-change-period=2

a=rtpmap:97 telephone-event

a=maxptime:20

m=audio 44552 RTP/AVP 96 97

b=AS:25.4

a=rtpmap:96 AMR

a=fmtp:96mode-set=0,2,5,7; mode-change-period=2

a=rtpmap:97 telephone-event

a=maxptime:20

m=video 1009 RTP/AVP 98 99

a=sendonly

b=AS:75

a=rtpmap:98 H263

a=fmtp:98 H263

a=fmtp:98 profile-level-id=0

a=rtpmap:99 MP4V-ES

</session-description>

</remote>

</dialog>

</dialog-info>

37-38.	SIP 200 (OK) response (from controllee UE-2 to SCC-AS)

The controller UE-1 acknowledges the SIP NOTIFY request by sending a SIP 200 (OK) response to the SCC AS.

39-40.	SIP re-INVITE request (from SCC-AS to controllee UE, UE-2) - see example in table A.12.3-39

The SCC-AS sends a SIP re_INVITE request to UE-2 to remove the audio 1 media component.

Table A.12.3-39 SIP INVITE request (SCC-AS to UE-2)

INVITE sip:user1_public2@home3.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91e6bf6 SIP/2.0

Via:

To: sip:user1_public2@home1.net;

From: sip:scc-as@home1.net; tag=12386

Call-ID:

CSeq:

Max-Forwards:

P-Asserted-Identity:

Require:

Contact:

Allow:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 123.112.67.87

s=-

c=IN IP4 123.112.67.87

t=0 0

m=audio 0 RTP/AVP 0

m=audio 44552 RTP/AVP 96 97

a=rtpmap:0 PCMU/8000

41-42.	SIP 200 (OK) response (from UE-2 to SCC-AS) - see example in table A.12.3-41

The controllee UE, UE-2, acknowledges the SIP INVITE request by sending a SIP 200 (OK) response to SCC-AS.

Table A.12.3-41 SIP 200 (OK) response (UE-2 to SCC-AS)

SIP/2.0 200 OK

Via:

To: sip:user1_public2@home1.net; tag = xyzwv

From: sip:scc-as@home1.net; tag = 12486

Call-ID:

CSeq:

P-Preferred-Identity:

Contact: sip:user1_public2@home1.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91e6bf6

Allow:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 123.45.67.89

s=-

c=IN IP4 123.45.67.89

t=0 0

m=audio 0 RTP/AVP 0

m=audio 34002 RTP/AVP 96 97

a=rtpmap:0 PCMU/8000

43-44.	SIP ACK request (from SCC-AS to UE-2)

The SCC –AS sends a SIP ACK request to UE-2 to acknowlege.

45-46.	SIP re_INVITE request (from SCC-AS to controllee UE; UE-3) - see example in table A.12.3-45

The SCC-AS sends a SIP re_INVITE request to UE-3 to activate the audio 1 media component.

Table A.12.3-45 SIP re-INVITE request (SCC-AS to UE-3)

INVITE sip:user1_public3@home1.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91e6bf6 SIP/2.0

Via:

To: sip:user1_public3@home1.net;

From: sip:scc-as@home1.net; tag=12486

Call-ID:

CSeq:

Max-Forwards:

P-Asserted-Identity:

Require:

Contact:

Allow:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 123.112.67.87

s=-

c=IN IP4 123.112.67.87

t=0 0

m=audio 49174 RTP/AVP 96 97

a=rtpmap:0 PCMU/8000

a= sendrecv

m=audio 0 RTP/AVP 0

m=video 1009 RTP/AVP 98 99

b=AS:75

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

a=rtpmap:99 MP4V-ES

a=sendonly

NOTE 2:	This SIP re-INVITE request is triggered by the SIP REFER request. The previous SIP INVITE request was generated by the SCC AS due to third party call control to allow sending this SIP re-INVITE request.

47-48.	SIP 200 (OK) response (from controllee UE; UE-3 to SCC AS)

Controllee UE, UE-3 sends a SIP 200 (OK) response to the SCC AS.

49-50.	SIP ACK request (from SCC-AS to UE-3)

The SCC-AS sends a SIP ACK request to the remote UE.

51-52.	SIP NOTIFY request (from SCC-AS to controller UE) - see example in table A.12.3-51

The SCC AS sends a SIP NOTIFY request containing SDP of the SIP 200 (OK) response received from the controllee UE; UE-3.

Table A.12.3-51 SIP NOTIFY request (SCC-AS to controller UE)

NOTIFY sip:user1_public1@home1.net;

Via:

To: sip:user1_public1@home1.net; tag=13579

From: sip:scc-as@home1.net; tag= 24680sip:user1_pubic1@home1.net;tag = 24680

Call-ID:

CSeq:

Max-Forwards:

P-Asserted-Identity:

Require:

Contact: sip: scc-as@home1.net

Allow:

Event: refer

Content-Type: message/sipfrag ;version=2.0 ;

Content-Length: (…)

SIP/2.0 200 OK

To: sip:user1_public3@home1.net; tag = xyzwv

From: sip:scc-as@home1.net; tag = 12486

Call-ID:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 123.112.67.87

s=-

c=123.112.67.87

t=0 0

m=audio 3002 RTP/AVP 96 97

a=rtpmap:0 PCMU/8000

m=audio 0 RTP/AVP 0

m=video 1302 RTP/AVP 98 99

a=recvonly

b=AS:75

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

a=rtpmap:99 MP4V-ES

53-54.	SIP 200 (OK) response (from controller UE to SCC-AS)

The controller UE acknowledges the SIP NOTIFY request by sending a SIP 200 (OK) response to the SCC AS.

## A.12.3A	Controller UE initiated media transfer from controllee UE to another controllee UE

The signalling flow in figure A.12.3A describes the procedures for media transfer from one controllee UE to another controllee UE.

Figure A.12.3A: Controller UE transfers a media from one controllee UE to another controllee UE.

1-2. SIP REFER request (SIP REFER request from UE-1 to SCC-AS)

There is an existing session with audio 1 and audio 2 between UE-2 (123.45.67.89) and the remote UE (132.54.76.98). The video component is unidirectional from the remote UE to the controllee UE, UE-3 (123.112.67.87). The Controller UE attempts to transfer the audio 1 portion of this session to the controllee UE, UE-3.

Table A.12.3A-1 SIP REFER request (UE-1 to SCC-AS)

REFER sip:scc-as@home1.net SIP/2.0

Via:

To: sip:scc-as@home1.net; tag= 24680

From: sip:user1_public1@home1.net; tag=13579sip:user1_pubic1@home1.net;tag = 24680

Call-ID: cb03a0s09a2sdfglkj490333

CSeq: 93809824 REFER

Max-Forwards: 70

P-Preferred-Identity:

Refer-To: <sip:user1_public3@home1.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91e6bf6body=m%3Daudio%200%20RTP%2FAVP%200%0Dm%3Daudio%20 49174%20RTP%2FAVP%2096%0Dm%3Dvideo%201009%20RTP%2FAVP%2098%2099>

Require: target-dialog

Target-dialog: cb03a0s09a2sdfglkj321576;remote-tag=abcdef;local-tag=123456

Contact: <sip:user1_public1@home1.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91ewxyz>

Accept: message/sipfrag

Content-Length: 0

3-4.	SIP 202 (Accepted) response (from SCC AS to UE-1)

The SCC-AS sends a SIP 202 (Accepted) response to controller UE-1 as response to the SIP REFER request.

5-6.	SIP NOTIFY request (from SCC AS to UE-1)

The SCC-AS sends a SIP NOTIFY request to UE-1 to notify implicit subscription to the SIP REFER request results.

Table A.12.3A-2 SIP NOTIFY request (SCC-AS to UE-1)

NOTIFY

To: sip:user1_public1@home1.net;tag=24680

From: sip:scc-as@home1.net;tag=13579

Call-ID: cb03a0s09a2sdfglkj12345

CSeq:

Max-Forwards:

P-Asserted-Identity:

Require:

Contact: sip:scc-as@home1.net

Allow:

Event: refer

Subscription-State:active;expires=3600

Content-Type: message/sipfrag;version=2.0

Content-Length: (…)

SIP/2.0 100 Trying

7-8.	SIP 200 (OK) response (from UE-1 AS to SCC-AS)

The controller UE, UE-1, acknowledges the SIP NOTIFY request by sending a SIP 200 (OK) response to the SCC-AS.

9-10.	SIP INVITE request (from SCC-AS to UE-3)

Since the message 1-2 contains a Refer-to header field addressed to UE-3 and the URI paramaters, listing an audio line which is not currently supported by another controllee UE than UE-2, the SCC AS realizes the procedure is for transferring the media from that controllee UE (UE-2) to controllee UE (UE-3). The SCC-AS sends a SIP INVITE request to the controllee UE, UE-3, to transfer the audio media component. The SDP in the SIP INVITE request lists the media  lines within the collaborative session. In order to avoid UE-3 to start sending audio to the remote UE, the SCC-AS adds an a-line set to sendonly in the SDP offer.

Table A.12.3A-3 SIP INVITE request (SCC-AS to UE-3)

INVITE sip:user1_public3@home1.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91e6bf6 SIP/2.0

Via:

To: sip:user1_public3@home1.net;

From: sip:scc-as@home1.net; tag=12486

Call-ID:

CSeq:

Max-Forwards:

P-Asserted-Identity:

Require:

Contact:

Allow:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 132.54.76.98

s=-

c=IN IP4 132.54.76.98

t=0 0

m=audio 49174 RTP/AVP 96 97

a=rtpmap:0 PCMU/8000

a= sendonly

b=RR:0

b=RS:0m=audio 0 RTP/AVP 0

m=video 1009 RTP/AVP 98 99

a=sendonly

b=AS:75

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

a=rtpmap:99 MP4V-ES

11-12.	SIP 200 (OK) response (from UE-3 to SCC-AS)

The controllee UE, UE-3, acknowledges the SIP INVITE request by sending a SIP 200 (OK) response to the SCC-AS.

Table A.12.3A-4 SIP 200 OK request (UE-3 to SCC-AS)

SIP/2.0 200 OK

Via:

To: sip:user1_public3@home1.net; tag = xyzwv

From: sip:scc-as@home1.net; tag = 12486

Call-ID:

CSeq:

P-Preferred-Identity:

Contact: sip:user1_public3@home1.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91e6bf6

Allow:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 123.112.67.87

s=-

c=123.112.67.87

t=0 0

m=audio 3002 RTP/AVP 96 97

a=rtpmap:0 PCMU/8000

a=recvonly

m=audio 0 RTP/AVP 0

m=video 1302 RTP/AVP 98 99

a=recvonly

b=AS:75

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

a=rtpmap:99 MP4V-ES

13-14. SIP ACK request(from SCC-AS to UE-3)

The SCC-AS sends a SIP ACK request to UE-3 to acknowledge.

15-16.	SIP re-INVITE request (from SCC-AS to controllee UE, UE-2)

The SCC AS sends a SIP re-INVITE request to controllee UE, UE-2 to put Audio 1 on hold.

Table A.12.3A-5 SIP INVITE request (SCC-AS to controllee UE, UE-2)

INVITE sip:user1_public2@home3.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91e6bf6 SIP/2.0

Via:

To: sip:user1_public2@home1.net;

From: sip:scc-as@home1.net; tag=12386

Call-ID:

CSeq:

Max-Forwards:

P-Asserted-Identity:

Require:

Contact:

Allow:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 132.54.76.98

s=-

c=IN IP4 132.54.76.98

t=0 0

m=audio 49174 RTP/AVP 96 97

a=sendonly

b=RR:0

b=RS:0

m=audio 44552 RTP/AVP 96 97

a=rtpmap:0 PCMU/8000

a=sendonly

b=RR:0

17-18. SIP 200 (OK) response (from UE-2 to SCC-AS)

The controllee UE, UE-2 acknowledges the SIP INVITE request by sending a SIP 200 (OK) response to the SCC-AS.

Table A.12.3A-6 SIP 200 (OK) response (UE-2 to SCC-AS)

SIP/2.0 200 OK

Via:

To: sip:user1_public2@home1.net; tag = xyzwv

From: sip:scc-as@home1.net; tag = 12486

Call-ID:

CSeq:

P-Preferred-Identity:

Contact: sip:user1_public2@home1.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91e6bf6

Allow:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 123.45.67.89

s=-

c=IN IP4 123.45.67.89

t=0 0

m=audio 32324 RTP/AVP 96 97

a=recvonly

m=audio 34002 RTP/AVP 96 97

a=rtpmap:0 PCMU/8000

a=recvonly

19-20. SIP ACK request (from SCC-AS to UE-2)

The SCC-AS send a SIP ACK request to UE-2 to acknowledge.

21-22. SIP re-INVITE request (from SCC-AS to remote UE)

The SCC-AS sends a SIP re-INVITE request to the remote UE, UE-4.

Table A.12.3A-7 SIP INVITE request (SCC-AS to remote UE, UE-4)

INVITE sip:user2_public1@home3.net;SIP/2.0

Via:

To: sip:user2_public1@home2.net;tag=66666

From: sip:scc-as@home1.net; tag=33333

Call-ID:

CSeq:

Max-Forwards:

P-Asserted-Identity:

Require:

Contact:sip:user1_public1@home1.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91ewxyz

Allow:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 123.112.67.87

s=-

t=0 0

m=audio 3002 RTP/AVP 96 97

c= IN IP4 123.112.67.87

a=rtpmap:0 PCMU/8000

m=audio 34002 RTP/AVP 96 97

C=IN IP4 123.45.67.89

A=rtpmap:0 PCMU/8000

m=video 1302 RTP/AVP 98 99

c= IN IP4 123.112.67.87

b=AS:75

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

a=rtpmap:99 MP4V-ES

a=recvonly

23-24.	SIP 200 (OK) response (from remote UE, UE-4 to SCC-AS)

The remote UE acknowledges the SIP re-INVITE request by sending a SIP 200 (OK) response to the SCC-AS.

Table A.12.3A-8 SIP 200 OK request (UE-4 to SCC-AS)

SIP/2.0 200 OK

Via:

To:

From:

Call-ID:

CSeq:

P-Asserted-Identity:

Contact: sip:user2_public1@home2.net;

Allow:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 132.54.76.98

s=-

c= IN IP4 132.54.76.98

t=0 0

m=audio 49174 RTP/AVP 96 97

b=AS:25.4

a=rtpmap:96 AMR

a=fmtp:96mode-set=0,2,5,7; mode-change-period=2

a=rtpmap:97 telephone-event

a=maxptime:20

m=audio 44552 RTP/AVP 96 97

a=rtpmap:0 PCMU/8000

m=video 1009 RTP/AVP 98 99

a=sendonly

b=AS:75

a=rtpmap:98 H263

a=fmtp:98 H263

a=fmtp:98 profile-level-id=0

a=rtpmap:99 MP4V-ES

a=sendonly

25-26.	SIP ACK request (from SCC-AS to remote UE)

The SCC-AS sends a SIP ACK request to the remote UE.

27-28.	SIP re-INVITE request (from SCC-AS to controllee UE, UE-2)

The SCC-AS sends a SIP re-INVITE request to UE-2 to remove the audio 1 media component.

Table A.12.3A-9 SIP re-INVITE request (SCC-AS to UE-2)

INVITE sip:user1_public2@home3.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91e6bf6 SIP/2.0

Via:

To: sip:user1_public2@home1.net;

From: sip:scc-as@home1.net; tag=12386

Call-ID:

CSeq:

Max-Forwards:

P-Asserted-Identity:

Require:

Contact:

Allow:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 132.54.76.98

s=-

c=IN IP4 132.54.76.98

t=0 0

m=audio 0 RTP/AVP 0

m=audio 44552 RTP/AVP 96 97

a=rtpmap:0 PCMU/8000

29-30, SIP 200 (OK) response (from UE-2 to SCC-AS)

The controllee UE, UE-2, acknowledges the SIP INVITE request by sending a SIP 200 (OK) response to the SCC-AS.

Table A.12.3A-10 SIP 200 (OK) response (UE-2 to SCC-AS)

SIP/2.0 200 OK

Via:

To: sip:user1_public2@home1.net; tag = xyzwv

From: sip:scc-as@home1.net; tag = 12486

Call-ID:

CSeq:

P-Preferred-Identity:

Contact: sip:user1_public2@home1.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91e6bf6

Allow:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 123.45.67.89

s=-

c=IN IP4 123.45.67.89

t=0 0

m=audio 0 RTP/AVP 0

m=audio 34002 RTP/AVP 96 97

a=rtpmap:0 PCMU/8000

31-32. SIP ACK request (from SCC-AS to UE-2)

The SCC-AS sends a SIP ACK request to UE-2 to acknowledge.

33-34. SIP re-INVITE request (from SCC-AS to controllee UE, UE-3)

The SCC-AS sends a SIP re-INVITE request to UE-3 to activate the audio 1 media component.

NOTE:	The SIP re-INVITE request is triggered by the SIP REFER request. The previous SIP INVITE request was generated by the SCC AS due to third party call cotrol to allow sending this SIP re-INVITE request.

Table A.12.3A-11 SIP re-INVITE request (SCC-AS to UE-3)

INVITE sip:user1_public3@home1.net;gr=urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91e6bf6 SIP/2.0

Via:

To: sip:user1_public3@home1.net;

From: sip:scc-as@home1.net; tag=12486

Call-ID:

CSeq:

Max-Forwards:

P-Asserted-Identity:

Require:

Contact:

Allow:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 123.112.67.87

s=-

c=IN IP4 123.112.67.87

t=0 0

m=audio 49174 RTP/AVP 96 97

a=rtpmap:0 PCMU/8000

a= sendrecv

m=audio 0 RTP/AVP 0

m=video 1009 RTP/AVP 98 99

b=AS:75

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

a=rtpmap:99 MP4V-ES

a=sendonly

35-36. SIP 200 (OK) response (from controllee UE, UE-3 to SCC-AS)

Controllee UE, UE-3 sends a SIP 200 (OK) response to the SCC AS.

37-38. SIP ACK request (from SCC-AS to UE-3)

The SCC-AS sends a SIP ACK request to the remote UE.

39-40. SIP NOTIFY request (from SCC-AS to controller UE, UE-1)

The SCC AS sends a SIP NOTIFY request containing SDP of the SIP 200 (OK) response received from the controllee UE, UE-3.

Table A.12.3A-12 SIP NOTIFY request (SCC-AS to UE-1)

NOTIFY sip:user1_public1@home1.net;

Via:

To: sip:user1_public1@home1.net; tag=13579

From: sip:scc-as@home1.net; tag= 24680sip:user1_pubic1@home1.net;tag = 24680

Call-ID:

CSeq:

Max-Forwards:

P-Asserted-Identity:

Require:

Contact: sip: scc-as@home1.net

Allow:

Event: refer

Content-Type: message/sipfrag ;version=2.0 ;

Content-Length: (…)

SIP/2.0 200 OK

To: sip:user1_public3@home1.net; tag = xyzwv

From: sip:scc-as@home1.net; tag = 12486

Call-ID:

Content-Type: application/sdp

Content-Length: (…)

v=0

o=- 1027933615 1027933615 IN IP4 123.112.67.87

s=-

c=123.112.67.87

t=0 0

m=audio 3002 RTP/AVP 96 97

a=rtpmap:0 PCMU/8000

m=audio 0 RTP/AVP 0

m=video 1302 RTP/AVP 98 99

a=recvonly

b=AS:75

a=rtpmap:98 H263

a=fmtp:98 profile-level-id=0

a=rtpmap:99 MP4V-ES

41-42.	SIP 200 (OK) response (from controller UE to SCC-AS)

The controller UE acknowledges the SIP NOTIFY request by sending a SIP 200 (OK) response to the SCC-AS.

# A.13	Signalling flows for release of collaborative session for inter-UE transfer

## A.13.1	Introduction

The signalling flows for release of Collaborative Session demonstrate how the session is released by the Controller UE or by the remote party UE. The following signalling flows are included:

-	subclause A.13.2 shows an example where the Controller UE initiates the release of a Collaborative Session. It demonstrates how the service control signalling and media path between the Controller UE and remote UE and the media path between the Controllee UE and the remote UE are released as a result of the session release; and

-	subclause A.13.3 shows an example where the remote UE initiates the release of a Collaborative Session.

## A.13.2	Controller UE releases collaborative session

In this example, session release is initiated by the Controller UE (UE 1), which is involved in the Collaborative Session with UE 2 and the remote UE (UE 3). The SCC AS ensures that all Controllee UEs involved in the Collaborative Session receive a request for session release in order to completely release the session ongoing with the remote UE.

Figure A.13.2-1: Release of Collaborative Session initiated by Controller UE

NOTE 1:	For clarity, the SIP 100 (Trying) responses are not shown in the signalling flow.

1.	Collaborative Session currently exists in a dialog with UE 3

A Collaborative Session involving UE 1 and UE 2 exists in a dialog with UE 3. Media paths exist between UE 1 and UE 3 and between UE 2 and UE 3. In this scenario, UE 1 is the Controller UE in the Collaborative Session and thus maintains service control signalling with the SCC AS.

2-3.	SIP BYE request (UE 1 to SCC AS via intermediate IM CN subsystem entities)

UE 1, acting as the Controller UE, initiates session release by sending a SIP BYE request towards the SCC AS. There is no Inter UE transfer specific content in the SIP BYE request.

4-6.	SIP BYE request (SCC AS to UE 3 via intermediate IM CN subsystem entities)

The SCC AS routes the SIP BYE request to UE 3 indicating to the remote UE that the Controller UE requests that the session is to be released.

7-9.	SIP 200 (OK) response (UE 3 to SCC AS via intermediate IM CN subsystem entities)

UE 3 responds to the received SIP BYE request with a SIP 200 (OK) response.

10-11.	SIP BYE request (SCC AS to UE 2 via intermediate IM CN subsystem entities)

The SCC AS, acting as a routing B2BUA, sends a SIP BYE request towards UE 2 to release the dialog it is involved in with UE 3 via the Collaborative Session. There is no Inter UE transfer specific content in the SIP BYE request.

NOTE 1:	The SIP BYE request to UE 2 (step 10) can occur in parallel with the SIP BYE request to UE 3 (step 4). Alternatively, the SCC AS can send the SIP BYE request to UE 2 prior to sending the SIP BYE request to UE 3.

12-13.	SIP 200 (OK) response (UE 2 to SCC AS via intermediate IM CN subsystem entities)

UE 2 responds to the received SIP BYE request with a SIP 200 (OK) response.

14-15	SIP 200 (OK) response (SCC AS to UE 1 via intermediate IM CN subsystem entities)

Upon receiving SIP 200 (OK) responses from UE 2 and UE 3, the SCC AS responds to the SIP BYE request from UE 1 with a SIP 200 (OK) response.

NOTE 2:	The SIP 200 (OK) response in step 14 can be sent earlier by the SCC AS in response to the SIP BYE request received from UE 1(step 3). For example, the SCC AS can send the SIP 200 (OK) response to UE 1 after receiving the SIP 200 (OK) response from UE 3 (step 9).

## A.13.3	Remote UE releases collaborative session

In this example, session release is initiated by the remote UE (UE 3). UE 1 and UE 2 are included in a Collaborative Session with the remote UE. The SCC AS ensures that all Controllee UEs involved in the Collaborative Session receive a request for session release in order to completely release the ongoing session.

Figure A.13.3-1: Release of Collaborative Session initiated by remote UE

NOTE 1:	For clarity, the SIP 100 (Trying) responses are not shown in the signalling flow.

1.	Collaborative Session currently exists in a dialog with UE 3

A Collaborative Session involving UE 1 and UE 2 exists in a dialog with UE 3. Media paths exist between UE 1 and UE 3 and between UE 2 and UE 3. In this scenario, UE 1 is the Controller UE in the Collaborative Session and thus maintains service control signalling with the SCC AS.

2-4.	SIP BYE request (UE 3 to SCC AS via intermediate IM CN subsystem entities)

The remote UE, UE 3, initiates session release by sending a SIP BYE request towards the Controller UE via the SCC AS serving the Controller UE. There is no Inter UE transfer specific content in the SIP BYE request.

5-6.	SIP BYE request (SCC AS to UE 1 via intermediate IM CN subsystem entities)

The SCC AS routes the SIP BYE request to UE 1, indicating to UE 1 that the remote UE requests that the session is to be released.

7-8.	SIP BYE request (SCC AS to UE 2 via intermediate IM CN subsystem entities)

The SCC AS, acting as a routing B2BUA sends a SIP BYE request to UE 2 to release the dialog it is involved in with UE 3 via the Collaborative Session. There is no Inter UE transfer specific content in the SIP BYE request.

NOTE 1:	Step 7 can occur in parallel with step 5. Alternatively, step 7 can occur prior to step 5. The order in which the SCC AS sends the SIP BYE requests is not considered to be important.

9-10.	SIP 200 (OK) response (UE 1 to SCC AS via intermediate IM CN subsystems entities)

UE 1 responds to the received SIP BYE request with a SIP 200 (OK) response.

NOTE 2:	Step 9 can occur immediately after UE1 has received the SIP BYE request in step 6.

11-12.	SIP 200 (OK) response (UE 2 to SCC AS via intermediate IM CN subsystems entities)

UE 2 responds to the received SIP BYE request with a SIP 200 (OK) response.

13-15.	SIP 200 (OK) response (SCC AS to UE 3 via intermediate IM CN subsystem entities)

Upon receiving SIP 200 (OK) responses from UE 1 and UE 2, the SCC AS responds to the SIP BYE request from UE 3 with a SIP 200 (OK) response.

NOTE 3:	Step 13 can occur after the SCC AS has received the SIP BYE request step 4 since it is not necessary to wait for SIP BYE requests to UE1 and UE2.