3GPP TSG-CT WG1 Mission critical ad hoc meeting 	C1ah-190005

# Sophia Antipolis, France, 17-19 July 2019

Source:	L3Harris Technologies

Title:	Discussion on interworking GMS

Agenda item:	16.3.1

Document for:	Information

## 1.	Introduction

The agreed clause 405.6 in TR 24.883 for GMS interface profiling needs to be moved to a normative specification to specify the IWF-3 group management interface.

## 2.	Discussion

## 2.1	Overview

The group management profiling clause in TR 24.883 is given here, in its entirety:

405.6	Interworking Function (IWF)

To be compliant with the procedures in the present document, an IWF:

-	shall support the role of XCAP server as specified in IETF RFC 4825 [88];

-	shall support the role of Group XDMS as specified in OMA OMA-TS-XDM_Group-V1_1_1 [87];

-	shall support the procedure in 3GPP TS 24.481 [31], subclause 6.2.5, with the IWF acting as the GMS;

-	shall support the procedure in 3GPP TS 24.481 [31], subclause 6.3.3.3, with the IWF acting as the GMS;

-	shall support the procedure in 3GPP TS 24.481 [31], subclause 6.3.8.3, with the IWF acting as the GMS;

-	shall support the procedure in 3GPP TS 24.481 [31], subclause 6.3.11.3, with the IWF acting as the GMS;

-	shall support the procedure in 3GPP TS 24.481 [31], subclause 6.3.13.2.3, with the IWF acting as the GMS;

-	shall support the procedure in 3GPP TS 24.481 [31], subclause 6.3.13.3.2.4, with the IWF acting as the GMS, with the clarification: the IWF need not interwork the received information in 3GPP TS 24.481 [31], subclause 6.3.13.2.2;

-	shall support the procedure in 3GPP TS 24.481 [31], subclause 6.3.13.3.3, with the IWF acting as the GMS;

-	shall support the procedure in 3GPP TS 24.481 [31], subclause 6.3.14.3, with the IWF acting as the GMS;

-	shall support the procedure in 3GPP TS 24.481 [31], subclause 6.3.15.3, with the IWF acting as the GMS; and

-	shall support the procedure in 3GPP TS 24.481 [31], subclause 6.3.16.3, with the IWF acting as the GMS.

The possible homes for this information are:

- TS 24.481

- TS 24.379

- A new TS

### 2.2	TS 24.481

Because TS 24.481 is GMS specific, it would be easy to find the information. But, readers might be looking for the information in an IWF specific specification (i.e. 29.xxx).

### 2.3	TS 29.379

Because TS 29.379 is IWF specific, it would be easy to find the information. TS 29.379 refers to GMS actions in its procedures, it would be logical to include the GMS profiling in the same TS.

### 2.4	New TS

Easiest to find the information, it would be a TS that would be both GMS and IWF specific. However, it would be a small amount of extra work to create the new TS. And it would be a very small TS. The new TS could be more generic, for example, to cover MCcore topics. We could add resulting interconnect topics such as user profile retrieval.

## 3.	Summary

|  | 24.481 | 29.379 | New TS |
| --- | --- | --- | --- |
| Ease of locating for the reader |  | Y | Y |
| Ease of documenting | Y | Y |  |
| Good for additional IWF topics (e.g. MCcore related) | N |  | Y |

## 4.	Proposal

Choose the approach in clause 2.3 as the way forward. This approach avoids the task of creating a new TS and keeps the information in a 29 series specification as the reader would expect. GMS profiling in the signalling TS (29.379) rather than the media TS (29.380) should be logical to the reader.