3GPP TS 29.222 V15.1.0 (2018-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

Common API Framework for 3GPP Northbound APIs;

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

5G System

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2018, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Contents	3

Foreword	10

1	Scope	11

2	References	11

3	Definitions and abbreviations	12

3.1	Definitions	12

3.2	Abbreviations	12

4	Overview	12

4.1	Introduction	12

4.2	Service Architecture	12

4.3	Functional Entities	13

4.3.1	API invoker	13

4.3.2	CAPIF core function	14

4.3.3	API exposing function	14

4.3.4	API publishing function	14

4.3.5	API management function	14

5	Services offered by the CAPIF Core Function	15

5.1	Introduction of Services	15

5.2	CAPIF_Discover_Service_API	16

5.2.1	Service Description	16

5.2.1.1	Overview	16

5.2.2	Service Operations	16

5.2.2.1	Introduction	16

5.2.2.2	Discover_Service_API	17

5.2.2.2.1	General	17

5.2.2.2.2	API invoker discovering service API using Discover_Service_API service operation	17

5.3	CAPIF_Publish_Service_API	17

5.3.1	Service Description	17

5.3.1.1	Overview	17

5.3.2	Service Operations	17

5.3.2.1	Introduction	17

5.3.2.2	Publish_Service_API	18

5.3.2.2.1	General	18

5.3.2.2.2	API publishing function publishing service APIs on CAPIF core function using Publish_Service_API service operation	18

5.3.2.3	Unpublish_Service_API	18

5.3.2.3.1	General	18

5.3.2.3.2	API publishing function un-publishing service APIs from CAPIF core function using Unpublish_Service_API service operation	18

5.3.2.4	Get_Service_API	19

5.3.2.4.1	General	19

5.3.2.4.2	API publishing function retrieving service APIs from CAPIF core function using Get_Service_API service operation	19

5.3.2.5	Update_Service_API	19

5.3.2.5.1	General	19

5.3.2.5.2	API publishing function updating published service APIs on CAPIF core function using Update_Service_API service operation	19

5.4	CAPIF_Events_API	20

5.4.1	Service Description	20

5.4.1.1	Overview	20

5.4.2	Service Operations	20

5.4.2.1	Introduction	20

5.4.2.2	Subscribe_Event	20

5.4.2.2.1	General	20

5.4.2.2.2	Subscribing to CAPIF events using Subscribe_Event service operation	20

5.4.2.3	Unsubscribe_Event	20

5.4.2.3.1	General	20

5.4.2.3.2	Unsubscribing from CAPIF events using Unsubscribe_Event service operation	21

5.4.2.4	Notify_Event	21

5.4.2.4.1	General	21

5.4.2.4.2	Notifying CAPIF events using Notify_Event service operation	21

5.5	CAPIF_API_Invoker_Management_API	21

5.5.1	Service Description	21

5.5.1.1	Overview	21

5.5.2	Service Operations	21

5.5.2.1	Introduction	21

5.5.2.2	Onboard_API_Invoker	21

5.5.2.2.1	General	21

5.5.2.2.2	API invoker on-boarding itself as a recognized user of CAPIF using Onboard_API_Invoker service operation	22

5.5.2.3	Offboard_API_Invoker	23

5.5.2.3.1	General	23

5.5.2.3.2	API invoker off-boarding itself as a recognized user of CAPIF using Offboard_API_Invoker service operation	23

5.5.2.4	Notify_Onboarding_Completion	23

5.5.2.4.1	General	23

5.5.2.4.2	Notifying Onboarding completion using Notify_Onboarding_Completion service operation	23

5.6	CAPIF_Security_API	23

5.6.1	Service Description	23

5.6.1.1	Overview	23

5.6.2	Service Operations	24

5.6.2.1	Introduction	24

5.6.2.2	Obtain_Security_Method	24

5.6.2.2.1	General	24

5.6.2.2.2	Request service API security method from CAPIF using Obtain_Security_Method service operation	24

5.6.2.3	Obtain_Authorization	24

5.6.2.3.1	General	24

5.6.2.3.2	Obtain authorization using Obtain_Authorization service operation	25

5.6.2.4	Obtain_API_Invoker_Info	25

5.6.2.4.1	General	25

5.6.2.4.2	Obtain API invoker's security information using Obtain_API_Invoker_Info service operation	25

5.6.2.5	Revoke_Authentication	25

5.6.2.5.1	General	25

5.6.2.5.2	Invalidate authorization using Revoke_Authorization service operation	25

5.7	CAPIF_Monitoring_API	25

5.8	CAPIF_Logging_API_Invocation_API	26

5.8.1	Service Description	26

5.8.1.1	Overview	26

5.8.2	Service Operations	26

5.8.2.1	Introduction	26

5.8.2.2	Log_API_Invocation_API	26

5.8.2.2.1	General	26

5.8.2.2.2	Logging service API invocations using Log_API_Invocation service operation	26

5.9	CAPIF_Auditing_API	26

5.9.1	Service Description	26

5.9.1.1	Overview	26

5.9.2	Service Operations	27

5.9.2.1	Introduction	27

5.9.2.2	Query_Invocation_Logs_API	27

5.9.2.2.1	General	27

5.9.2.2.2	Query API invocation information logs using Query_Invocation_Logs service operation	27

5.10	CAPIF_Access_Control_Policy_API	27

5.10.1	Service Description	27

5.10.1.1	Overview	27

5.10.2	Service Operations	27

5.10.2.1	Introduction	27

5.10.2.2	Obtain_Access_Control_Policy	28

5.10.2.2.1	General	28

5.10.2.2.2	API exposing function obtaining access control policy from the CAPIF core function using Obtain_Access_Control_Policy service operation	28

5.10.3	Related Events	28

6	Services offered by the API exposing function	28

6.1	Introduction of Services	28

6.2	AEF_Authentication_API	28

6.2.1	Service Description	28

6.2.1.1	Overview	28

6.2.2	Service Operations	29

6.2.2.1	Introduction	29

6.2.2.2	Authentication_Initiation_Request_API	29

6.2.2.2.1	General	29

6.2.2.2.2	API invoker initiating authentication using Authentication_Initiation_Request service operation	29

7	CAPIF Design Aspects Common for All APIs	29

7.1	General	29

7.2	Data Types	29

7.2.1	General	29

7.2.2	Referenced structured data types	30

7.2.3	Referenced Simple data types and enumerations	30

7.3	Usage of HTTP	30

7.4	Content type	30

7.5	URI structure	31

7.6	Notifications	31

7.7	Error handling	31

7.8	Feature negotiation	31

7.9	HTTP headers	32

8	CAPIF API Definition	32

8.1	CAPIF_Discover_Service_API	32

8.1.1	API URI	32

8.1.2	Resources	32

8.1.2.1	Overview	32

8.1.2.2	Resource: All published service APIs	32

8.1.2.2.1	Description	32

8.1.2.2.2	Resource Definition	33

8.1.2.2.3	Resource Standard Methods	33

8.1.2.2.3.1	GET	33

8.1.2.2.4	Resource Custom Operations	33

8.1.3	Notifications	33

8.1.4	Data Model	34

8.1.4.1	General	34

8.1.4.2	Structured data types	34

8.1.4.2.1	Introduction	34

8.1.4.2.2	Type: DiscoveredAPIs	34

8.1.4.3	Simple data types and enumerations	34

8.1.5	Error Handling	34

8.1.6	Feature negotiation	34

8.2	CAPIF_Publish_Service_API	35

8.2.1	API URI	35

8.2.2	Resources	35

8.2.2.1	Overview	35

8.2.2.2	Resource: APF published APIs	36

8.2.2.2.1	Description	36

8.2.2.2.2	Resource Definition	36

8.2.2.2.3	Resource Standard Methods	36

8.2.2.2.3.1	POST	36

8.2.2.2.3.2	GET	36

8.2.2.2.4	Resource Custom Operations	37

8.2.2.3	Resource: Individual APF published API	37

8.2.2.3.1	Description	37

8.2.2.3.2	Resource Definition	37

8.2.2.3.3	Resource Standard Methods	37

8.2.2.3.3.1	GET	37

8.2.2.3.3.2	PUT	38

8.2.2.3.3.3	DELETE	38

8.2.2.3.4	Resource Custom Operations	39

8.2.3	Notifications	39

8.2.4	Data Model	39

8.2.4.1	General	39

8.2.4.2	Structured data types	40

8.2.4.2.1	Introduction	40

8.2.4.2.2	Type: ServiceAPIDescription	40

8.2.4.2.3	Type: InterfaceDescription	40

8.2.4.3	Simple data types and enumerations	40

8.2.4.3.1	Introduction	40

8.2.4.3.2	Simple data types	40

8.2.4.3.3	Enumeration: Protocol	41

8.2.4.3.4	Enumeration: DataFormat	41

8.2.4.3.5	Enumeration: CommunicationType	41

8.2.4.3.6	Enumeration: SecurityMethods	41

8.2.5	Error Handling	41

8.2.6	Feature negotiation	41

8.3	CAPIF_Events_API	42

8.3.1	API URI	42

8.3.2	Resources	42

8.3.2.1	Overview	42

8.3.2.2	Resource: CAPIF Events Subscriptions	42

8.3.2.2.1	Description	42

8.3.2.2.2	Resource Definition	43

8.3.2.2.3	Resource Standard Methods	43

8.3.2.2.3.1	POST	43

8.3.2.2.4	Resource Custom Operations	43

8.3.2.3	Resource: Individual CAPIF Events Subscription	43

8.3.2.3.1	Description	43

8.3.2.3.2	Resource Definition	43

8.3.2.3.3	Resource Standard Methods	44

8.3.2.3.3.1	DELETE	44

8.3.2.3.4	Resource Custom Operations	44

8.3.3	Notifications	44

8.3.3.1	General	44

8.3.3.2	Event Notification	45

8.3.3.2.1	Description	45

8.3.3.2.2	Notification definition	45

8.3.4	Data Model	45

8.3.4.1	General	45

8.3.4.2	Structured data types	46

8.3.4.2.1	Introduction	46

8.3.4.2.2	Type: EventSubscription	46

8.3.4.2.3	Type: EventNotification	46

8.3.4.3	Simple data types and enumerations	46

8.3.4.3.1	Introduction	46

8.3.4.3.2	Simple data types	46

8.3.4.3.3	Enumeration: CAPIFEvent	47

8.3.5	Error Handling	47

8.3.6	Feature negotiation	47

8.4	CAPIF_API_Invoker_Management_API	48

8.4.1	API URI	48

8.4.2	Resources	48

8.4.2.1	Overview	48

8.4.2.2	Resource: On-boarded API invokers	49

8.4.2.2.1	Description	49

8.4.2.2.2	Resource Definition	49

8.4.2.2.3	Resource Standard Methods	49

8.4.2.2.3.1	POST	49

8.4.2.2.4	Resource Custom Operations	49

8.4.2.3	Resource: Individual On-boarded API Invoker	50

8.4.2.3.1	Description	50

8.4.2.3.2	Resource Definition	50

8.4.2.3.3	Resource Standard Methods	50

8.4.2.3.3.1	DELETE	50

8.3.2.3.4	Resource Custom Operations	50

8.4.3	Notifications	50

8.4.3.1	General	50

8.4.3.2	Notify_Onboarding_Completion	51

8.4.3.2.1	Description	51

8.4.3.2.2	Notification definition	51

8.4.4	Data Model	51

8.4.4.1	General	51

8.4.4.2	Structured data types	52

8.4.4.2.1	Introduction	52

8.4.4.2.2	Type: APIInvokerEnrolmentDetails	52

8.4.4.2.3	Type: OnboardingNotificationDestination	53

8.4.4.2.4	Type: APIList	53

8.4.4.2.5	Type: OnboardingInformation	53

8.4.4.2.6	Type: OnboardingRequestAck	53

8.4.4.2.7	Type: OnboardingNotification	54

8.4.4.3	Simple data types and enumerations	54

8.4.5	Error Handling	54

8.4.6	Feature negotiation	54

8.5	CAPIF_Security_API	54

8.5.1	API URI	54

8.5.2	Resources	55

8.5.2.1	Overview	55

8.5.2.2	Resource: Trusted API invokers	55

8.5.2.2.1	Description	55

8.5.2.2.2	Resource Definition	55

8.5.2.2.3	Resource Standard Methods	56

8.5.2.2.3.1	POST	56

8.5.2.2.4	Resource Custom Operations	56

8.5.2.3	Resource: Individual trusted API invokers	56

8.5.2.3.1	Description	56

8.5.2.3.2	Resource Definition	56

8.5.2.3.3	Resource Standard Methods	57

8.5.2.3.3.1	GET	57

8.5.2.3.3.2	DELETE	57

8.5.2.3.4	Resource Custom Operations	57

8.5.3	Notifications	58

8.5.3.1	General	58

8.5.3.2	Authorization revoked notification	58

8.5.3.2.1	Description	58

8.5.3.2.2	Notification definition	58

8.5.4	Data Model	59

8.5.4.1	General	59

8.5.4.2	Structured data types	60

8.5.4.2.1	Introduction	60

8.5.4.2.2	Type: ServiceSecurity	60

8.5.4.2.3	Type: SecurityMethod	60

8.5.4.2.4	Type: SecurityNotificationDestination	61

8.5.4.2.5	Type: SecurityNotification	61

8.5.4.3	Simple data types and enumerations	61

8.5.4.3.1	Introduction	61

8.5.4.3.2	Simple data types	61

8.5.4.3.3	Enumeration: Cause	61

8.5.5	Error Handling	61

8.5.6	Feature negotiation	62

8.6	CAPIF_Access_Control_Policy_API	62

8.6.1	API URI	62

8.6.2	Resources	62

8.6.2.1	Overview	62

8.6.2.2	Resource: Access Control Policy List	63

8.6.2.2.1	Description	63

8.6.2.2.2	Resource Definition	63

8.6.2.2.3	Resource Standard Methods	63

8.6.2.2.3.1	GET	63

8.6.2.2.4	Resource Custom Operations	64

8.6.3	Notifications	64

8.6.4	Data Model	64

8.6.4.1	General	64

8.6.4.2	Structured data types	64

8.6.4.2.1	Introduction	64

8.6.4.2.2	Type: AccessControlPolicyList	64

8.6.4.2.3	Type: ApiInvokerPolicy	65

8.6.4.2.4	Type: TimeRangeList	65

8.6.4.3	Simple data types and enumerations	65

8.6.5	Error Handling	65

8.6.6	Feature negotiation	65

8.7	CAPIF_Logging_API_Invocation_API	65

8.7.1	API URI	65

8.7.2	Resources	66

8.7.2.1	Overview	66

8.7.2.2	Resource: Logs	66

8.7.2.2.1	Description	66

8.7.2.2.2	Resource Definition	66

8.7.2.2.3	Resource Standard Methods	67

8.7.2.2.3.1	POST	67

8.7.2.2.4	Resource Custom Operations	67

8.7.3	Notifications	67

8.7.4	Data Model	67

8.7.4.1	General	67

8.7.4.2	Structured data types	68

8.7.4.2.1	Introduction	68

8.7.4.2.2	Type: InvocationLogs	68

8.7.4.2.3	Type: Log	68

8.7.4.3	Simple data types and enumerations	68

8.7.5	Error Handling	68

8.7.6	Feature negotiation	69

8.8	CAPIF_Auditing_API	69

8.8.1	API URI	69

8.8.2	Resources	69

8.8.2.1	Overview	69

8.8.2.2	Resource: All service API invocation logs	70

8.8.2.2.1	Description	70

8.8.2.2.2	Resource Definition	70

8.8.2.2.3	Resource Standard Methods	70

8.8.2.2.3.1	GET	70

8.8.2.2.4	Resource Custom Operations	71

8.8.3	Notifications	71

8.8.4	Data Model	71

8.8.4.1	General	71

8.8.4.2	Structured data types	71

8.8.4.3	Simple data types and enumerations	71

8.8.5	Error Handling	71

8.8.6	Feature negotiation	72

9	AEF API Definition	72

9.1	AEF_Authentication_API	72

9.1.1	API URI	72

9.1.2	Resources	72

9.1.2.1	Overview	72

9.1.2.2	Resource: API invoker authentication profiles	73

9.1.2.2.1	Description	73

9.1.2.2.2	Resource Definition	73

9.1.2.2.3	Resource Standard Methods	73

9.1.2.2.3.1	GET	73

9.1.2.2.4	Resource Custom Operations	73

9.1.3	Notifications	73

9.1.4	Data Model	74

9.1.4.1	General	74

9.1.4.2	Structured data types	74

9.1.4.3	Simple data types and enumerations	74

9.1.5	Error Handling	74

9.1.6	Feature negotiation	74

10	Security	74

10.1	General	74

10.1	CAPIF-1/1e security	74

10.2	CAPIF-2/2e security and securely invoking service APIs	75

Annex A (normative): OpenAPI specification	75

A.1	General	75

A.2	CAPIF_Discover_Service_API	75

A.3	CAPIF_Publish_Service_API	76

A.4	CAPIF_Events_API	81

A.5	CAPIF_API_Invoker_Management_API	83

A.6	CAPIF_Security_API	86

A.7	CAPIF_Access_Control_Policy_API	89

A.8	CAPIF_Logging_API_Invocation_API	90

A.9	CAPIF_Auditing_API	92

A.10	AEF_Authentication_API	94

Annex B (informative): Change history	95

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present specification describes the protocol for the Common API Framework (CAPIF) for 3GPP Northbound APIs. The CAPIF and the related stage 2 architecture and functional requirements are defined in 3GPP TS 23.222 [2].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[3]	Open API Initiative, "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

[4]	IETF RFC 7230: "Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[5]	IETF RFC 7231: "Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content".

[6]	IETF RFC 7232: "Hypertext Transfer Protocol (HTTP/1.1): Conditional Requests".

[7]	IETF RFC 7233: "Hypertext Transfer Protocol (HTTP/1.1): Range Requests".

[8]	IETF RFC 7234: "Hypertext Transfer Protocol (HTTP/1.1): Caching".

[9]	IETF RFC 7235: "Hypertext Transfer Protocol (HTTP/1.1): Authentication".

[10]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[11]	IETF RFC 5246, "The Transport Layer Security (TLS) Protocol Version 1.2".

[12]	IETF RFC 7159: "The JavaScript Object Notation (JSON) Data Interchange Format".

[13]	IETF RFC 6455: "The Websocket Protocol".

[14]	3GPP TS 29.122: "T8 reference point for northbound Application Programming Interfaces (APIs)".

[15]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[16]	3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs"

[17]	IETF RFC 2617: "HTTP Authentication: Basic and Digest Access Authentication".

[18]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

API registry: API registry is a registry maintained by the CAPIF core function to store information about the service APIs based on the data models defined in this specification. The structure of the API registry is out of scope of this specification.

CAPIF administrator: An authorized user with special permissions for CAPIF operations.

PLMN trust domain: The entities protected by adequate security and controlled by the PLMN operator or a trusted 3rd party of the PLMN.

Service API: The interface through which a component of the system exposes its services to API invokers by abstracting the services from the underlying mechanisms.

Subscriber: A functional entity that subscribes to another functional entity for notifications.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AEF	API Exposing Function

AMF	API Management Function

APF	API Publishing Function

AS	Application Server

CAPIF	Common API Framework

CCF	CAPIF Core Function

JSON	JavaScript Object Notation

REST	Representational State Transfer

SCEF	Service Capability Exposure Function

SCS	Service Capability Server

# 4	Overview

## 4.1	Introduction

In 3GPP, there are multiple northbound API-related specifications. To avoid duplication and inconsistency of approaches between different API specifications and to specify common services (e.g. authorization), 3GPP has considered in 3GPP TS 23.222 [2] the development of a common API framework (CAPIF) that includes common aspects applicable to any northbound service APIs.

The present document specifies the APIs needed to support CAPIF.

## 4.2	Service Architecture

3GPP TS 23.222, clause 6 [2] specifies the functional entities and domains of the functional model, which is depicted in Figure 4.2-1, in detail.

Figure 4.2-1: CAPIF Functional Model

CAPIF-1 and CAPIF-1e reference points connect an API invoker inside the PLMN Trust Domain and an API invoker outside the PLMN Trust Domain respectively, with the CAPIF core function.

CAPIF-2 and CAPIF-2e reference points connect an API invoker inside the PLMN Trust Domain and an API invoker outside the PLMN Trust Domain respectively, with the API exposing function.

CAPIF-3 reference point connects an API exposing function inside the PLMN Trust Domain with the CAPIF core function.

CAPIF-4 reference point connects an API publishing function inside the PLMN Trust Domain with the CAPIF core function.

CAPIF-5 reference point connects an API management function inside the PLMN Trust Domain with the CAPIF core function.

NOTE:	The API exposing function, API publishing function and API management function are part the API provider domain which can be implemented by the Service Capability Exposure Function (SCEF) and/or the Network Exposure Function (NEF).

## 4.3	Functional Entities

### 4.3.1	API invoker

The API invoker is typically provided by a 3rd party application provider who has service agreement with PLMN operator. The API invoker may reside within the same trust domain as the PLMN operator network.

The API invoker supports several capabilities such as supporting

-	the authentication and obtaining authorization and discovering using CAPIF-1/CAPIF-1e reference point as defined in 3GPP TS 23.222 [2]; and

-	invoking the Service APIs using CAPIF-2/CAPIF-2e referenced point as defined in 3GPP TS 23.222 [2], e.g. the T8 interface as defined in 3GPP TS 29.122 [14] or the NEF Northbound interface as defined in 3GPP TS 29.522 [15].

### 4.3.2	CAPIF core function

The CAPIF core function (CCF) supports the following capabilities over CAPIF-1/CAPIF-1e reference point as defined in 3GPP TS 23.222 [2]:

-	authenticating the API invoker;

-	providing the authorization information; and

-	service API discovery.

The CAPIF core function supports the following capabilities over CAPIF-3 reference point as defined in 3GPP TS 23.222 [2]:

-	providing the service API access policy;

-	providing the authentication and authorization information of API invoker for validation;

-	logging of service API invocations and

-	charging of service API invocations.

The CAPIF core function supports the following capabilities over CAPIF-4 reference point as defined in 3GPP TS 23.222 [2]:

-	publishing and storing the service APIs information.

The CAPIF core function supports the following capabilities over CAPIF-5 reference point as defined in 3GPP TS 23.222 [2]:

-	providing the service API invocation log for auditing;

-	providing monitoring information the status of service APIs and

-	storing configurations of the API provider policies.

### 4.3.3	API exposing function

The API exposing function (AEF) is the provider of the Service APIs and is also the service communication entry point of the Service API to the API invokers using CAPIF-2/CAPIF-2e reference point as defined in 3GPP TS 23.222 [2]. The API exposing function consists of capabilities such as authenticating the API invoker, validating the authorization provided by the CAPIF core function and logging the Service API invocations at the CAPIF core function using CAPIF-3 reference point as defined in 3GPP TS 23.222 [2].

According to the distributed deployment scenarios specified in 3GPP TS 23.222 [2], it is possible that the CAPIF can be deployed by splitting the functionality of the API exposing function among multiple API exposing function entities, of which one acts as the entry point. The source API exposing function takes the role of API invoker and communicates with the destination API exposing function over CAPIF-2.

### 4.3.4	API publishing function

The API publishing function (APF) enables the API provider to publish the Service APIs information using CAPIF-4 reference point as defined in 3GPP TS 23.222 [2] in order to enable the discovery of Service APIs by the API invoker.

### 4.3.5	API management function

The API management function (AMF) enables the API provider to perform administration of the Service APIs. The API management function supports several capabilities such as querying the Service API invocation log for auditing, monitoring the events, configuring the API provider policies and monitoring the status of the Service APIs using CAPIF-5 reference point as defined in in 3GPP TS 23.222 [2].

# 5	Services offered by the CAPIF Core Function

## 5.1	Introduction of Services

The table 5.1-1 lists the CAPIF Core Function APIs below the service name. A service description subclause for each API gives a general description of the related API.

Table 5.1-1: List of CAPIF Services

| Service Name | Service Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| CAPIF_Discover_Service_API | Discover_Service_API | Request/ Response | AP Invoker |
|  | Event operations (NOTE) | (NOTE) | API Invoker |
| CAPIF_Publish_Service_API | Publish_Service_API | Request/ Response | API Publishing Function |
|  | Unpublish_Service_API | Request/ Response | API Publishing Function |
|  | Update_Service_API | Request/ Response | API Publishing Function |
|  | Get_Service_API | Request/ Response | API Publishing Function |
|  | Event operations (NOTE) | (NOTE) | API Publishing Function |
| CAPIF_Events_API | Subscribe_Event | Request/ Response | API Invoker, API Publishing Function, API Management Function, API Exposing Function |
|  | Notify_Event | Subscribe/Notify | API Invoker, API Publishing Function, API Management Function, API Exposing Function |
|  | Unsubscribe_Event | Request/ Response | API Invoker, API Publishing Function, API Management Function, API Exposing Function |
| CAPIF_API_Invoker_Management_API | Onboard_API_Invoker | Request/ Response | API Invoker |
|  | Offboard_API_Invoker | Request/ Response | API Invoker |
|  | Notify_Onboarding_Completion | Subscribe/Notify | API Invoker |
| CAPIF_Security_API | Obtain_Security_Method | Request/ Response | API Invoker |
|  | Obtain_Authorization | Request/ Response | API Invoker |
|  | Obtain_API_Invoker_Info | Request/ Response | API exposing function |
|  | Revoke_Authorization | Request/ Response | API exposing function |
| CAPIF_Monitoring_API | Event operations (NOTE) | (NOTE) | API Management Function |
| CAPIF_Logging_API_Invocation_API | Log_API_Invocation | Request/ Response | API exposing function |
| CAPIF_Auditing_API | Query_API_Invocation_Log | Request/ Response | API management function |
| CAPIF_Access_Control_Policy_API | Obtain_Access_Control_Policy | Request/Response | API Exposing Function |
| NOTE: The service operations of CAPIF Events API are reused by the CAPIF_Discover_Service_API, CAPIF_Publish_Service_API and CAPIF_Monitoring_API for events related services. |  |  |  |

5.2	CAPIF_Discover_Service_API

### 5.2.1	Service Description

#### 5.2.1.1	Overview

The CAPIF discover service APIs, as defined in 3GPP TS 23.222 [2], allow API invokers via CAPIF-1 and CAPIF-1e reference point to discover service API available at the CAPIF core function.

### 5.2.2	Service Operations

#### 5.2.2.1	Introduction

The service operation defined for CAPIF_Discover_Service_API is shown in table 5.2.2.1-1.

Table 5.2.2.1-1: Operations of the CAPIF_Discover_Service_API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Discover_Service_API | This service operation is used by an API invoker to discover service API available at the CAPIF core function. | API invoker |

#### 5.2.2.2	Discover_Service_API

##### 5.2.2.2.1	General

This service operation is used by an API invoker to discover service API available at the CAPIF core function.

##### 5.2.2.2.2	API invoker discovering service API using Discover_Service_API service operation

To discover service APIs available at the CAPIF core function, the API invoker shall send an HTTP GET message with the API invoker Identifier and query parameters to the CAPIF core function.

Upon receiving the above described HTTP GET message, the CAPIF core function shall:

1.	verify the identity of the API invoker and check if the API invoker is authorized to discover the service APIs;

2.	if the API invoker is authorized to discover the service APIs, the CAPIF core function shall:

a.	search the CAPIF core function (API registry) for APIs matching the query criteria;

b.	apply the discovery policy, if any, on the search results and filter the search results;

c.	return the filtered search results in the response message.

## 5.3	CAPIF_Publish_Service_API

### 5.3.1	Service Description

#### 5.3.1.1	Overview

The CAPIF publish service APIs, as defined in 3GPP TS 23.222 [2], allow API publishing function via CAPIF-4 reference point to publish and manage published service APIs at the CAPIF core function.

### 5.3.2	Service Operations

#### 5.3.2.1	Introduction

The service operations defined for the CAPIF_Publish_Service API are shown in table 5.3.2.1-1.

Table 5.3.2.1-1: Operations of the CAPIF_Publish_Service_API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Publish_Service_API | This service operation is used by an API publishing function to publish service APIs on the CAPIF core function. | API publishing function |
| Unpublish_Service_API | This service operation is used by an API publishing function to un-publish service APIs from the CAPIF core function. | API publishing function |
| Get_Service_API | This service operation is used by an API publishing function to retrieve service APIs from the CAPIF core function. | API publishing function |
| Update_Service_API | This service operation is used by an API publishing function to update published service APIs on the CAPIF core function. | API publishing function |

#### 5.3.2.2	Publish_Service_API

##### 5.3.2.2.1	General

This service operation is used by an API publishing function to publish service APIs on the CAPIF core function.

##### 5.3.2.2.2	API publishing function publishing service APIs on CAPIF core function using Publish_Service_API service operation

To publish service APIs at the CAPIF core function, the API publishing function shall send an HTTP POST message to the CAPIF core function. The body of the HTTP POST message shall include API publishing function Identifier and API Information.

Upon receiving the above described HTTP POST message, the CAPIF core function shall:

1.	verify the identity of the API publishing function and check if the API publishing function is authorized to publish service APIs;

2.	if the API publishing function is authorized to publish service APIs, the CAPIF core function shall:

a.	verify the API Information present in the HTTP POST message and add the service APIs in the CAPIF core function (API registry);

b.	create a new resource as defined in subclause 8.2.3;

c.	send a notification message with the updated service API, to all API Invokers that subscribed to the Service API Update event; and

d.	return the CAPIF Resource URI in the response message.

#### 5.3.2.3	Unpublish_Service_API

##### 5.3.2.3.1	General

This service operation is used by an API publishing function to un-publish service APIs from the CAPIF core function.

##### 5.3.2.3.2	API publishing function un-publishing service APIs from CAPIF core function using Unpublish_Service_API service operation

To un-publish service APIs from the CAPIF core function, the API publishing function shall send an HTTP DELETE message using the CAPIF Resource URI received during the publish operation to the CAPIF core function.

Upon receiving the above described HTTP DELETE message, the CAPIF core function shall

1.	verify the identity of the API publishing function and check if the API publishing function is authorized to un-publish service APIs;

2.	if the API publishing function is authorized to un-publish service APIs, the CAPIF core function shall:

a.	delete the resource pointed by the CAPIF Resource URI;

b.	delete the relevant service APIs from the CAPIF core function (API registry); and

c.	send a notification message with the deleted service API, to all API Invokers that subscribed to the Service API Update event.

#### 5.3.2.4	Get_Service_API

##### 5.3.2.4.1	General

This service operation is used by an API publishing function to retrieve service APIs from the CAPIF core function.

##### 5.3.2.4.2	API publishing function retrieving service APIs from CAPIF core function using Get_Service_API service operation

To retrieve information about the published service APIs from the CAPIF core function, the API publishing function shall send an HTTP GET message with the API publishing function Identifier to the CAPIF core function.

Upon receiving the above described HTTP GET message, the CAPIF core function shall

1.	verify the identity of the API publishing function and check if the API publishing function is authorized to retrieve information about the published service APIs;

2.	if the API publishing function is authorized to retrieve information about the published service APIs, the CAPIF core function shall:

a.	respond with the API Information associated with the CAPIF Resource Identifier mentioned in the HTTP GET message.

#### 5.3.2.5	Update_Service_API

##### 5.3.2.5.1	General

This service operation is used by an API publishing function to update published service APIs on the CAPIF core function.

##### 5.3.2.5.2	API publishing function updating published service APIs on CAPIF core function using Update_Service_API service operation

To update information of published service APIs, the API publishing function shall send an HTTP PUT message with the relevant CAPIF Resource URI and API publishing function Identifier to the CAPIF core function. The body of the HTTP PUT message shall include updated API Information.

Upon receiving the above described HTTP PUT message, the CAPIF core function shall

1.	verify the identity of the API publishing function and check if the API publishing function is authorized to update information of published service APIs;

2.	if the API publishing function is authorized to update information of published service APIs, the CAPIF core function shall:

a.	verify the API Information present in the HTTP PUT message and replace the service APIs in the CAPIF core function (API registry);

b.	replace the existing resource accordingly; and

c.	send a notification message with the updated service API, to all API Invokers that subscribed to the Service API Update event.

## 5.4	CAPIF_Events_API

### 5.4.1	Service Description

#### 5.4.1.1	Overview

The CAPIF events APIs, as defined in 3GPP TS 23.222 [2], allow an API invoker via CAPIF-1 reference point, API exposure function via CAPIF-3 reference point, API publishing function via CAPIF-4 reference point and API management function via CAPIF-5 reference point to subscribe to and unsubscribe from CAPIF events and to receive notifications from CAPIF core function.

NOTE:	The functional elements listed above are referred to as Subscriber in the service operations described in the subclauses below.

### 5.4.2	Service Operations

#### 5.4.2.1	Introduction

The service operations defined for the CAPIF_Events_API are shown in table 5.4.2.1-1.

Table 5.4.2.1-1: Operations of the CAPIF_Events_API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Subscribe_Event | This service operation is used by a Subscriber to subscribe to CAPIF events. | Subscriber |
| Unsubscribe_Event | This service operation is used by a Subscriber to unsubscribe from CAPIF events | Subscriber |
| Notify_Event | This service operation is used by CAPIF core function to send a notification to a Subscriber | CAPIF core function |

#### 5.4.2.2	Subscribe_Event

##### 5.4.2.2.1	General

This service operation is used by a Subscriber to subscribe to CAPIF events.

##### 5.4.2.2.2	Subscribing to CAPIF events using Subscribe_Event service operation

To subscribe to CAPIF events, the Subscriber shall send an HTTP POST message to the CAPIF core function. The body of the HTTP POST message shall include Subscriber's Identifier, Event Type and a Notification Destination URI.

Upon receiving the above described HTTP POST message, the CAPIF core function shall:

1.	verify the identity of the Subscriber and check if the Subscriber is authorized to subscribe to the CAPIF events mentioned in the HTTP POST message;

2.	if the Subscriber is authorized to subscribe to the CAPIF events, the CAPIF core function shall:

a.	create a new resource as defined in subclause 8.3.3; and

b.	return the CAPIF Resource URI in the response message.

#### 5.4.2.3	Unsubscribe_Event

##### 5.4.2.3.1	General

This service operation is used by a Subscriber to un-subscribe from CAPIF events.

##### 5.4.2.3.2	Unsubscribing from CAPIF events using Unsubscribe_Event service operation

To unsubscribe from CAPIF events, the Subscriber shall send an HTTP DELETE message using the CAPIF Resource Identifier to the CAPIF core function.

Upon receiving the HTTP DELETE message, the CAPIF core function shall:

1.	verify the identity of the Unsubscribing functional entity and check if the Unsubscribing functional entity is authorized to Unsubscribe from the CAPIF event associated with the CAPIF Resource URI; and

2.	if the Unsubscribing functional entity is authorized to unsubscribe from the CAPIF events, the CAPIF core function shall delete the resource pointed by the CAPIF Resource URI.

#### 5.4.2.4	Notify_Event

##### 5.4.2.4.1	General

This service operation is used by CAPIF core function to send a notification to a Subscriber.

##### 5.4.2.4.2	Notifying CAPIF events using Notify_Event service operation

To notify CAPIF events, the CAPIF core function shall send an HTTP POST message using the Notification Destination URI received in the subscription request. The body of the HTTP POST message shall include an Event Notification and CAPIF Resource URI.

Upon receiving the HTTP POST message, the Subscriber shall process the Event Notification.

## 5.5	CAPIF_API_Invoker_Management_API

### 5.5.1	Service Description

#### 5.5.1.1	Overview

The CAPIF API invoker management APIs, as defined in 3GPP TS 23.222 [2], allow API invokers via CAPIF-1 and CAPIF-1e reference point to on-board and off-board itself as a recognized user of the CAPIF.

### 5.5.2	Service Operations

#### 5.5.2.1	Introduction

The service operations defined for the CAPIF API Invoker Management API are shown in table 5.5.2.1-1.

Table 5.5.2.1-1: Operations of the CAPIF_API_Invoker_Management_API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Onboard_API_Invoker | This service operation is used by an API invoker to on-board itself as a recognized user of CAPIF | API invoker |
| Offboard_API_Invoker | This service operation is used by an API invoker to off-board itself as a recognized user of CAPIF | API invoker |
| Notify_Onboarding_Completion | This service operation is used by CAPIF core function to send an on-boarding notification to the API invoker. | CAPIF core function |

#### 5.5.2.2	Onboard_API_Invoker

##### 5.5.2.2.1	General

This service operation is used by an API invoker to on-board itself as a recognized user of CAPIF

##### 5.5.2.2.2	API invoker on-boarding itself as a recognized user of CAPIF using Onboard_API_Invoker service operation

To on-board itself as a recognized user of the CAPIF, the API invoker shall send an HTTP POST message to the CAPIF core function. The body of the HTTP POST message shall include API invoker Enrolment Details, API List and a Notification Destination URI for on-boarding notification.

Upon receiving the above described HTTP POST message, the CAPIF core function shall check if it can determine authorization of the request and on-board the API invoker automatically. If the CAPIF core function:

1.	can determine authorization of the request and on-board the API invoker automatically, the CAPIF core function:

a.	shall process the API invoker Enrolment Details and the API List received in the HTTP POST message and determine if the request sent by the API invoker is authorized or not;

b.	if the API invoker's request is authorized, the CAPIF core function shall:

i.	create the API invoker Profile consisting of an API invoker Identifier, Authentication Information, Authorization Information and CAPIF Identity Information;

ii.	verify the API List present in the HTTP POST message and create a API List of APIs the API invoker is allowed to access;

iii.	create a new resource as defined in subclause 8.4.3;

iv.	return the API invoker Profile, API List of APIs the API invoker is allowed to access and the CAPIF Resource URI in the response message.

2.	cannot determine authorization of the request to on-board the API invoker automatically, the CAPIF core function:

a.	shall acknowledge the receipt of the on-boarding request to the API invoker.

b.	shall request the CAPIF administrator to validate the on-boarding request or the API management to validate the on-boarding request by sharing the API invoker Enrolment Details and the API List received in the HTTP POST message;

c.	on receiving confirmation of successful validation of the on-boarding request from the CAPIF administrator or the API management, the CAPIF core function shall:

i.	create the API invoker Profile consisting of an API invoker Identifier, Authentication Information, Authorization Information and CAPIF Identity Information;

ii.	create a new resource as defined in subclause 8.4.3;

iii.	deliver the API invoker Profile, API List of APIs the API invoker is allowed to access and the CAPIF Resource URI to the API invoker in a notification.

NOTE 1:	How the CAPIF core function determines that the CAPIF core function can process the request and on-board the API invoker automatically is out-of-scope of this specification.

NOTE 2:	How the CAPIF core function determines that the API invoker's request to on-board is authorized is specified in 3GPP TS 33.122 [16].

NOTE 3:	Interactions between the CAPIF core function and the CAPIF administrator or the API management is out-of-scope of this specification.

NOTE 4:	The onboarding credential received by the API invoker from the service provider as specified in 3GPP TS 33.122 [16] is included in the Authorization header field of the HTTP request message as described in IETF RFC 2617 [17].

NOTE 5:	After the onboarding operation is completed the API Invoker no longer needs to maintain the Notification Destination URI and may delete it.

#### 5.5.2.3	Offboard_API_Invoker

##### 5.5.2.3.1	General

This service operation is used by an API invoker to stop being as a recognized user of CAPIF

##### 5.5.2.3.2	API invoker off-boarding itself as a recognized user of CAPIF using Offboard_API_Invoker service operation

To off-board itself as a recognized user of the CAPIF, the API invoker shall send an HTTP DELETE message using the CAPIF Resource Identifier received during the on-boarding to the CAPIF core function.

Upon receiving the HTTP DELETE message, the CAPIF core function shall:

1.	determine if the request sent by the API invoker is authorized or not;

2.	if the API invoker's request is authorized, the CAPIF core function shall:

a.	 delete the resource representation pointed by the CAPIF Resource Identifier; and

b.	delete the related API invoker profile.

#### 5.5.2.4	Notify_Onboarding_Completion

##### 5.5.2.4.1	General

This service operation is used by the CAPIF core function to send a notification about the completion of the Onboarding operation to the API Invoker.

##### 5.5.2.4.2	Notifying Onboarding completion using Notify_Onboarding_Completion service operation

When the CAPIF core function cannot immediately authorize the API invoker that issued an Onboarding request (see subclause 5.5.2.2.2) it will send a response acknowledging the request and begin processing it. After completion, the CAPIF core function shall send an HTTP POST message using the Notification Destination URI received in the Onboarding request. The body of the HTTP POST message shall include the API Invoker Profile, API List of the APIs the API invoker is allowed to access and the CAPIF Resource URI.

Upon receiving the HTTP POST message, the API invoker shall process the message in the same manner it would have processed an immediate response to the Onboarding request, and respond to the HTTP POST message with an acknowledgement and no body.

## 5.6	CAPIF_Security_API

### 5.6.1	Service Description

#### 5.6.1.1	Overview

The CAPIF security APIs, as defined in 3GPP TS 23.222 [2], allow:

-	API invokers via CAPIF-1/1e reference points to negotiate the service security method and obtain authorization for invoking service APIs; and

-	API exposing function via CAPIF-3 reference point to obtain authentication information of the API invoker for authentication of the API invoker.

### 5.6.2	Service Operations

#### 5.6.2.1	Introduction

The service operations defined for CAPIF_Authentication_Authorization_API are shown in table 5.6.2.1-1.

Table 5.6.2.1-1: Operations of the CAPIF_Security_API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Obtain_Security_Method | This service operation is used by an API invoker to negotiate and obtain information about service API security method for itself with CAPIF core function. This information is used by API invoker for service API invocations. | API invoker |
| Obtain_Authorization | This service operation is used by an API invoker to obtain authorization to access service APIs. | API invoker |
| Obtain_API_Invoker_Info | This service operation is used by an API exposing function to obtain the authentication or authorization information related to an API invoker. | API exposing function |
| Revoke_Authorization | This service operation is used by an API exposing function to invalidate the authorization of an API invoker. | API exposing function |

#### 5.6.2.2	Obtain_Security_Method

##### 5.6.2.2.1	General

This service operation is used by an API invoker to negotiate and obtain service API security method from the CAPIF core function. The information received by API invoker shall be used for authentication with the API exposing function.

##### 5.6.2.2.2	Request service API security method from CAPIF using Obtain_Security_Method service operation

To negotiate and obtain service API security method information from the CAPIF core function, the API invoker shall send an HTTP POST message to the CAPIF core function. The body of the HTTP POST message shall include Security Method Request and a Notification Destination URI for security related notifications. The Security Method Request from the API invoker contains the unique interface details of the service APIs and may contain a preferred method for each unique service API interface.

Upon receiving the above described HTTP POST message, the CAPIF core function shall:

1.	determine the security method for each service API interface as specified in 3GPP TS 33.122 [16];

2.	store the Notification Destination URI for security related notification;

3.	create a new resource as defined in subclause 8.4.3; and

4.	return the security method information and the CAPIF Resource URI in the response message.

#### 5.6.2.3	Obtain_Authorization

##### 5.6.2.3.1	General

This service operation is used by an API invoker to negotiate and obtain authorization information from the CAPIF core function. The information received by API invoker shall be used for authorization to invoke service APIs exposed by the API exposing function.

##### 5.6.2.3.2	Obtain authorization using Obtain_Authorization service operation

To obtain authorization information from the CAPIF core function to invoke service APIs, the API invoker shall perform the functions of the resource owner, client and redirection endpoints as described in subclause 6.5.2.3 of 3GPP TS 33.122 [16].

#### 5.6.2.4	Obtain_API_Invoker_Info

##### 5.6.2.4.1	General

This service operation is used by an API exposing function to obtain the security information of API Invokers to be able to authenticate them and authorize each service API invocation by them.

##### 5.6.2.4.2	Obtain API invoker's security information using Obtain_API_Invoker_Info service operation

To obtain authentication or authorization information from the CAPIF core function to authenticate or authorize an API invoker, the API exposing function shall send an HTTP GET message to the CAPIF core function with the API invoker ID and an indication to request authentication and authorization information.

Upon receiving the above described HTTP GET message, the CAPIF core function shall:

1.	determine the security information of API invoker for all the service API interfaces of the API exposing function; and

2.	return the security information in the response message.

#### 5.6.2.5	Revoke_Authentication

##### 5.6.2.5.1	General

This service operation is used by an API exposing function to invalidate the authorization of a specified API Invoker to invoke service APIs exposed by the calling API exposing function.

##### 5.6.2.5.2	Invalidate authorization using Revoke_Authorization service operation

To invalidate authorization of an API invoker, the API exposing function shall send an HTTP DELETE message to the CAPIF core function using the API invoker ID.

Upon receiving the above described HTTP DELETE message, the CAPIF core function shall delete the resource representation pointed by the API invoker ID and shall notify the API invoker of the authorization invalidation using the Notification Destination URI received in the Obtain_Security_Method message.

## 5.7	CAPIF_Monitoring_API

The CAPIF monitoring API as defined in 3GPP TS 23.222 [2], allow the API management function via CAPIF-5 reference point to monitor service API invocations and receive such monitoring events from the CAPIF core function.

The CAPIF_Monitoring_API shall use the CAPIF_Events_API as described in subclause 8.3 by setting the CAPIFEvent to "Monitoring service API" as described in subclause 8.3.6.3.3.

## 5.8	CAPIF_Logging_API_Invocation_API

### 5.8.1	Service Description

#### 5.8.1.1	Overview

The Logging API invocations APIs, as defined in 3GPP TS 23.222 [2], allow API exposing functions via CAPIF-3 reference point to log the information related to service API invocations on the CAPIF core function.

### 5.8.2	Service Operations

#### 5.8.2.1	Introduction

Table 5.8.2.1-1: Operations of the CAPIF_Logging_API_Invocation_API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Log_API_Invocation | This service operation is used by an API exposing function to log API invocation information on CAPIF core function. | API exposing function |

#### 5.8.2.2	Log_API_Invocation_API

##### 5.8.2.2.1	General

This service operation is used by an API exposing function to log API invocation information on CAPIF core function.

##### 5.8.2.2.2	Logging service API invocations using Log_API_Invocation service operation

To log service API invocations at the CAPIF core function, the API exposing function shall send an HTTP POST message to the CAPIF core function. The body of the HTTP POST message shall include API exposing function identity information and API invocation log information.

Upon receiving the above described HTTP POST message, the CAPIF core function shall:

1.	verify the identity of the API exposing function and check if the API exposing function is authorized to create service API invocation logs;

2.	if the API exposing function is authorized to create service API invocation logs, the CAPIF core function shall:

a.	process the API invocation log information received in the HTTP POST message and store the API invocation log information in the API repository;

b.	create a new resource as defined in subclause 8.7.3; and

c.	return the CAPIF Resource Identifier in the response message.

## 5.9	CAPIF_Auditing_API

### 5.9.1	Service Description

#### 5.9.1.1	Overview

The Auditing API, as defined in 3GPP TS 23.222 [2], allows API management functions via CAPIF-5 reference point to query the log information stored on the CAPIF core function.

### 5.9.2	Service Operations

#### 5.9.2.1	Introduction

Table 5.9.2.1-1: Operations of the CAPIF_Auditing_API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Query_Invocation_Logs | This service operation is used by an API management function to query API invocation information logs stored on CAPIF core function. | API management function |

#### 5.9.2.2	Query_Invocation_Logs_API

##### 5.9.2.2.1	General

This service operation is used by an API management function to query API invocation information logs stored on CAPIF core function.

##### 5.9.2.2.2	Query API invocation information logs using Query_Invocation_Logs service operation

To query service API invocation logs at the CAPIF core function, the API management function shall send an HTTP GET message with the API management function identity information and the log query to the CAPIF core function.

Upon receiving the above described HTTP GET message, the CAPIF core function shall:

1.	verify the identity of the API management function and check if the API management function is authorized to query the service API invocation logs;

2.	if the API management function is authorized to query the service API invocation logs, the CAPIF core function shall:

a.	search the API invocation logs for logs matching the Log Query criteria; and

b.	return the search results in the response message.

## 5.10	CAPIF_Access_Control_Policy_API

### 5.10.1	Service Description

#### 5.10.1.1	Overview

The CAPIF access control policy APIs allow API exposing function via CAPIF-3 reference point to obtain the service API access policy from the CAPIF core function.

### 5.10.2	Service Operations

#### 5.10.2.1	Introduction

Table 5.3.2.1-1: Operations of the CAPIF_Access_Control_Policy_API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Obtain_Access_Control_Policy | This service operation is used by an API exposing function to obtain the access control policy from the CAPIF core function. | API exposing function |

#### 5.10.2.2	Obtain_Access_Control_Policy

##### 5.10.2.2.1	General

This service operation is used by an API exposing function to obtain the access control policy from the CAPIF core function.

##### 5.10.2.2.2	API exposing function obtaining access control policy from the CAPIF core function using Obtain_Access_Control_Policy service operation

To obtain the access control policy from the CAPIF core function, the API exposing function shall send an HTTP GET message to the CAPIF core function with the API exposing function Identifier and API identification. The GET message may include API invoker ID for retrieving access control policy of the requested API invoker.

Upon receiving the above described HTTP GET message, the CAPIF core function shall

1.	verify the identity of the API exposing function and check if the API exposing function is authorized to obtain the access control policy corresponding to the API identification;

2.	if the API exposing function is authorized to obtain the access control policy, the CAPIF core function shall respond with the access control policy information corresponding to the API identification and API invoker ID (if present) in the HTTP GET message.

### 5.10.3	Related Events

The CAPIF_Access_Control_Policy_API supports the subscription and notification of the status of access control information via the CAPIF_Events_API. The related events are specified in subclause 8.3.6.3.3.

# 6	Services offered by the API exposing function

## 6.1	Introduction of Services

The table 6.1-1 lists the API exposing function APIs below the service name. A service description subclause for each API gives a general description of the related API.

Table 6.1-1: List of AEF Services

| Service Name | Service Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| AEF_Authentication_API | Authentication_Initiation_Request | Request/ Response | API Invoker |

## 6.2	AEF_Authentication_API

### 6.2.1	Service Description

#### 6.2.1.1	Overview

The AEF authentication API, allows an API invokers via CAPIF-2/2e reference points to request API exposing function to ensure that authentication parameters necessary for authentication of the API invoker are available with the API exposing function. If the necessary authentication parameters are not available, the API exposing function fetches necessary authentication parameters from CAPIF core function to authenticate the API invoker.

### 6.2.2	Service Operations

#### 6.2.2.1	Introduction

The service operation defined for AEF_Authentication _API is shown in table 6.2.2.1-1.

Table 6.2.2.1-1: Operations of the AEF_Authentication_API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Authentication_Initiation_Request | This service operation is used by an API invoker to request API exposing function to fetch necessary authentication parameters from CAPIF core function to authenticate the API invoker | API invoker |

#### 6.2.2.2	Authentication_Initiation_Request_API

##### 6.2.2.2.1	General

This service operation is used by an API invoker to initiate authentication with the API exposing function. On receiving the Authentication_Initiation_Request the API exposing function fetches the authentication information of the API invoker from the CAPIF core function, if required.

##### 6.2.2.2.2	API invoker initiating authentication using Authentication_Initiation_Request service operation

To initiate authentication with the API exposing function, the API invoker shall send an HTTP GET message to the API exposing function with the API invoker ID.

Upon receiving the above described HTTP GET message, the API exposing function shall

check if the credentials of the API invoker for authentication are available with the API exposing function. If the credentials of the API invoker for authentication are not available, the API exposing function shall use the service defined in subclause 5.6.2.4.2 to fetch the credentials from the CAPIF core function.

7	CAPIF Design Aspects Common for All APIs

## 7.1	General

CAPIF APIs are RESTful APIs that allow secure access to the capabilities provided by CAPIF.

This document specifies the procedures triggered at different functional entities as a result of API invocation requests and event notifications. The stage-2 level requirements and signalling flows are defined in 3GPP TS 23.222 [2].

Several design aspects, as mentioned in the following subclauses, are specified in 3GPP TS 29.122 [14] and referenced by this specification.

## 7.2	Data Types

### 7.2.1	General

This clause defines structured data types, simple data types and enumerations that are applicable to several APIs defined in the present specification and can be referenced from data structures defined in the subsequent clauses.

In addition, data types that are defined in OpenAPI 3.0.0 Specification [3] can also be referenced from data structures defined in the subsequent clauses.

NOTE:	As a convention, data types in the present specification are written with an upper-case letter in the beginning. Parameters are written with a lower-case letter in the beginning. As an exception, data types that are also defined in OpenAPI 3.0.0 Specification [3] can use a lower-case case letter in the beginning for consistency.

Table 7.2.1-1 specifies data types re-used by the CAPIF from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the CAPIF.

Table 7.2.1-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Uri | 3GPP TS 29.122 [14] |  |
| TestNotification | 3GPP TS 29.122 [14] | Following clarifications apply: - The SCEF is the CAPIF core function; and - The SCS/AS is theSubscriber. |
| WebsockNotifConfig | 3GPP TS 29.122 [14] | Following clarifications apply: - The SCEF is the CAPIF core function; and - The SCS/AS is theSubscriber. |

### 7.2.2	Referenced structured data types

Table 7.2.2-1 lists structured data types defined in this specification referenced by multiple services:

Table 7.2.2-1: Referenced Structured Data Types

| Data type | Reference | Description |
| --- | --- | --- |
| Log | Subclause 8.7.4.2.3 | Individual log entries |
| InterfaceDescription | Subclause 8.2.4.2.3 | Description of the API interface |
| ServiceAPIDescription | Subclause 8.2.4.2.2 | Description of the service API |

### 7.2.3	Referenced Simple data types and enumerations

Following simple data types defined in Table 7.2.3.1-1 are applicable to several APIs in this document:

Table 7.2.3.1-1: Simple data types applicable to several APIs

| Type name | Reference | Description |
| --- | --- | --- |
| CAPIFResourceId | n/a | string chosen by the CAPIF core function to serve as identifier in a resource URI. |
| DataFormat | Subclause 8.2.4.3.4 | Data format used by the API |
| Protocol | Subclause 8.2.4.3.3 | Protocol used by the API |

## 7.3	Usage of HTTP

For CAPIF APIs, support of HTTP/1.1 (IETF RFC 7230 [4], IETF RFC 7231 [5], IETF RFC 7232 [6], IETF RFC 7233 [7], IETF RFC 7234 [8] and IETF RFC 7235 [9]) over TLS (IETF RFC 5246 [11]) is mandatory and support of HTTP/2 (IETF RFC 7540 [10]) over TLS (IETF RFC 5246 [11]) is recommended.

A functional entity desiring to use HTTP/2 shall use the HTTP upgrade mechanism to negotiate applicable HTTP version as described in IETF RFC 7540 [10].

## 7.4	Content type

The bodies of HTTP request and successful HTTP responses shall be encoded in JSON format (see IETF RFC 7159 [12]).

The MIME media type that shall be used within the related Content-Type header field is "application/json", as defined in IETF RFC 7159 [12].

NOTE:	This release only supports the content type JSON.

## 7.5	URI structure

All resource URIs of CAPIF APIs should have the following root structure:

- {apiRoot}/{apiName}/{apiVersion}/

"apiRoot" is configured by means outside the scope of the present document. It includes the scheme ("https"), host and optional port, and an optional prefix string. "apiName" and "apiVersion" shall be set dependent on the API, as defined in the corresponding subclauses below.

All resource URIs in the subclauses below are defined relative to the above root URI.

NOTE 1:	The "apiVersion" will only be increased if the new API version contains backward incompatible changes. Otherwise, the supported feature mechanism defined in subclause 7.8 can be used to negotiate extensions.

NOTE 2:	A different root structure can be used when the resource URI is preconfigured in the API invoking entity.

The root structure may be followed by "apiSpecificSuffixes" that are dependent on the API and are defined separately for each API where they apply:

- {apiRoot}/{apiName}/{apiVersion}/{apiSpecificSuffixes}

## 7.6	Notifications

The functional entities

-	shall support the delivery of notifications using a separate HTTP connection towards an address;

-	may support testing delivery of notifications; and

-	may support the delivery of notification using WebSocket protocol (see IETF RFC 6455 [13]),

as described in 3GPP TS 29.122 [14], with the following clarifications:

-	the SCEF is the CAPIF core function; and

-	the SCS/AS is theSubscriber.

## 7.7	Error handling

Response bodies for error handling, as described in 3GPP TS 29.122 [14], are applicable to all APIs in the present specification unless specified otherwise, with the following clarifications:

-	the SCEF is the CAPIF core function; and

-	the SCS/AS is the functional entity invoking an API.

## 7.8	Feature negotiation

The functional entity invoking an API (i.e. the API invoker, the API exposing function, the API publishing function or the API management function) and the CAPIF core function use feature negotiation procedures defined in 3GPP TS 29.122 [14] to negotiate the supported features, with the following clarifications:

-	The SCEF is the CAPIF core function; and

-	The SCS/AS is the functional entity invoking an API.

## 7.9	HTTP headers

The HTTP headers described in 3GPP TS 29.122 [14] are applicable to all APIs in this document.

# 8	CAPIF API Definition

## 8.1	CAPIF_Discover_Service_API

### 8.1.1	API URI

The request URI used in each HTTP request from the API invoker towards the CAPIF core function shall have the structure as defined in subclause 7.5 with the following clarifications:

-	The {apiName} shall be "service-apis".

-	The {apiVersion} shall be "v1".

-	The {apiSpecificSuffixes} shall be set as described in subclause 8.1.2.

### 8.1.2	Resources

#### 8.1.2.1	Overview

Figure 8.1.2.1-1: Resource URI structure of the CAPIF_Discover_Service_API

Table 8.1.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.1.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| All published service APIs (Store) | {apiRoot} /service-apis/v1 /allServiceApis | GET | Discover published service APIs and retrieve a collection of APIs according to certain filter criteria. |

#### 8.1.2.2	Resource: All published service APIs

##### 8.1.2.2.1	Description

The All published service APIs resource represents a collection of published service APIs on a CAPIF core function. The resource is modelled as a Store resource archetype (see Annex C.3 of 3GPP TS 29.501 [18])

##### 8.1.2.2.2	Resource Definition

Resource URI: {apiRoot}/service-apis/v1/allServiceApis

This resource shall support the resource URI variables defined in table 8.1.2.2.2-1.

Table 8.1.2.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 7.5 |

##### 8.1.2.2.3	Resource Standard Methods

8.1.2.2.3.1	GET

This operation retrieves a list of APIs currently registered in the CAPIF core function, satisfying a number of filter criteria.

Table 8.1.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| apiInvokerId | string | M | 1 | String identifying the API invoker assigned by the CAPIF core function. |
| serviceName | string | O | 0..1 | Name of the service |
| apiName | string | O | 0..1 | API name |
| apiVersion | string | O | 0..1 | API version |
| commType | CommunicationType | O | 0..1 | Communication type used by the API (e.g. request/response or subscribe/notify). |
| interfaceDescription | array(InterfaceDescription) | O | 0..N | Interface details (e.g. domain name, ipv4/6Addr, port). |
| dataFormat | DataFormat | O | 0..1 | Data format used by the API (e.g. serialization protocol JSON used). |

This method shall support the request data structures specified in table 8.1.2.2.3.1-2 and the response data structures and response codes specified in table 8.1.2.2.3.1-3.

Table 8.1.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.1.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(DiscoveredAPIs) | O | 1..N | 200 OK | The response body contains the result of the search over the list of registered APIs. |
| ProblemDetails | M | 1 | 414 URI Too Long | Indicates that the server is refusing to service the request because the request-target is too long. |

##### 8.1.2.2.4	Resource Custom Operations

None.

8.1.3	Notifications

None.

### 8.1.4	Data Model

#### 8.1.4.1	General

This subclause specifies the application data model supported by the API.

Table 8.1.4.1-1 specifies the data types defined for the CAPIF service based interface protocol.

Table 8.1.4.1-1: Specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| DiscoveredAPIs | 8.1.4.2.2 | Definition of the service API |  |

Table 8.1.4.1-2 specifies data types re-used by the CAPIF_Discover_Service_API service:

Table 8.1.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ServiceAPIDescription | Subclause 8.2.4.2.2 | Description of the service API |  |
| InterfaceDescription | Subclause 8.2.4.2.3 | Name of the protocol |  |
| Protocol | Subclause 8.2.4.3.3 | Protocol |  |
| DataFormat | Subclause 8.2.4.3.4 | Data format |  |
| CommunicationType | Subclause 8.2.4.3.5 | Communication type used by the API |  |
| Uri | 3GPP TS 29.122 [14] |  |  |
| ProblemDetails | 3GPP TS 29.122 [14] |  |  |

#### 8.1.4.2	Structured data types

##### 8.1.4.2.1	Introduction

8.1.4.2.2	Type: DiscoveredAPIs

Table 8.1.4.2.2-1: Definition of type DiscoveredAPIs

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| apiId | string | M | 1 | Identifier of the service API |  |
| serviceAPIDescription | ServiceAPIDescription | M | 1 | Description of the service API as published by the service. |  |

#### 8.1.4.3	Simple data types and enumerations

None.

### 8.1.5	Error Handling

General error responses are defined in subclause 7.7.

### 8.1.6	Feature negotiation

General feature negotiation procedures are defined in subclause 7.8.

Table 8.1.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| n/a |  |  |

## 8.2	CAPIF_Publish_Service_API

### 8.2.1	API URI

The request URI used in each HTTP request from the API publishing function towards the CAPIF core function shall have the structure as defined in subclause 7.5 with the following clarifications:

-	The {apiName} shall be "published-apis".

-	The {apiVersion} shall be "v1".

-	The {apiSpecificSuffixes} shall be set as described in subclause 8.2.2.

### 8.2.2	Resources

#### 8.2.2.1	Overview

Figure 8.2.2.1-1: Resource URI structure of the CAPIF_Publish_Service_API

Table 8.2.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.2.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| APF published APIs | {apiRoot} /published-apis/v1 /{apfId}/service-apis | POST | Publish a new API |
|  |  | GET | Retrieve all published service APIs |
| Individual APF published API | {apiRoot} /published-apis /v1 /{apfId}/service-apis /{serviceApiId} | GET | Retrieve a published service API |
|  |  | PUT | Update a published service API |
|  |  | DELETE | Unpublish a published service API |

#### 8.2.2.2	Resource: APF published APIs

##### 8.2.2.2.1	Description

The APF published APIs resource represents all published service APIs of a API publishing function.

##### 8.2.2.2.2	Resource Definition

Resource URI: {apiRoot}/published-apis/v1/{apfId}/service-apis

This resource shall support the resource URI variables defined in table 8.2.2.2.2-1.

Table 8.2.2.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 7.5 |
| apfId | String identifying the API publishing function |

##### 8.2.2.2.3	Resource Standard Methods

###### 8.2.2.2.3.1	POST

This method shall support the URI query parameters specified in table 8.2.2.2.3.1-1.

Table 8.2.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.2.2.2.3.1-2 and the response data structures and response codes specified in table 8.2.2.2.3.1-3.

Table 8.2.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ServiceAPIDescription | M | 1 | Definition of the service API being published |

Table 8.2.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ServiceAPIDescription | M | 1 | 201 Created | Service API published successfully. The URI of the created resource shall be returned in the "Location" HTTP header |

###### 8.2.2.2.3.2	GET

This method shall support the URI query parameters specified in table 8.2.2.2.3.2-1.

Table 8.2.2.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.2.2.2.3.2-2 and the response data structures and response codes specified in table 8.2.2.2.3.2-3.

Table 8.2.2.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.2.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(ServiceAPIDescription) | O | 0..N | 200 OK | Definition of all service API(s) published by the API publishing function. |

##### 8.2.2.2.4	Resource Custom Operations

None.

#### 8.2.2.3	Resource: Individual APF published API

##### 8.2.2.3.1	Description

The Individual APF published API resource represents an individual published service API.

##### 8.2.2.3.2	Resource Definition

Resource URI: {apiRoot}/published-apis/v1/{apfId}/service-apis/{serviceApiId}

This resource shall support the resource URI variables defined in table 8.2.2.3.2-1.

Table 8.2.2.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 7.5 |
| apfId | String identifying the API publishing function |
| serviceApiId | String identifying an individual published service API |

##### 8.2.2.3.3	Resource Standard Methods

###### 8.2.2.3.3.1	GET

This method shall support the URI query parameters specified in table 8.2.2.3.3.1-1.

Table 8.2.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.2.2.3.3.1-2 and the response data structures and response codes specified in table 8.2.2.3.3.1-3.

Table 8.2.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.2.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(ServiceAPIDescription) | O | 0..N | 200 OK | Definition of all service API published by the API publishing function. |

###### 8.2.2.3.3.2	PUT

This method shall support the URI query parameters specified in table 8.2.2.3.3.2-1.

Table 8.2.2.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.2.2.3.3.2-2 and the response data structures and response codes specified in table 8.2.2.3.3.2-3.

Table 8.2.2.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ServiceAPIDescription | M | 1 | Updated definition of the service API. |

Table 8.2.2.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ServiceAPIDescription | M | 1 | 200 OK | Definition of the service API updated successfully.. |

###### 8.2.2.3.3.3	DELETE

This method shall support the URI query parameters specified in table 8.2.2.3.3.3-1.

Table 8.2.2.3.3.3-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.2.2.3.3.3-2 and the response data structures and response codes specified in table 8.2.2.3.3.3-3.

Table 8.2.2.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.2.2.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The individual published service API matching the serviceApiId is deleted. |

##### 8.2.2.3.4	Resource Custom Operations

None.

### 8.2.3	Notifications

None.

### 8.2.4	Data Model

#### 8.2.4.1	General

This subclause specifies the application data model supported by the API.

Table 8.2.4.1-1 specifies the data types defined for the CAPIF service based interface protocol.

Table 8.2.4.1-1: Specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ServiceAPIDescription | 8.2.4.2.2 | Definition of the service API |  |
| InterfaceDescription | 8.2.4.2.3 | Description of the API interface |  |

Table 8.2.4.1-2 specifies data types re-used by the CAPIF_Publish_Service_API service:

Table 8.2.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Ipv4Addr | 3GPP TS 29.122 [14] |  |  |
| Ipv6Addr | 3GPP TS 29.122 [14] |  |  |
| Uri | 3GPP TS 29.122 [14] |  |  |
| Port | 3GPP TS 29.122 [14] |  |  |

#### 8.2.4.2	Structured data types

##### 8.2.4.2.1	Introduction

##### 8.2.4.2.2	Type: ServiceAPIDescription

Table 8.2.4.2.2-1: Definition of type ServiceAPIDescription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| apiName | string | M | 1 | API name |  |
| apiID | string | C | 0..1 | API identifier assigned by the CAPIF core function to the published service API. Shall not be present in the HTTP POST request from the API publishing function to the CAPIF core function. Shall be present in the HTTP POST response from the CAPIF core function to the API publishing function. |  |
| apiVersion | string | O | 0..1 | API version |  |
| domainName | string | O | 0..1 | Domain to which API belongs to |  |
| protocol | Protocol | O | 0..1 | Protocol used by the API. |  |
| serviceName | string | O | 0..1 | Name of the service to which the API belongs |  |
| commType | CommunicationType | O | 0..1 | Communication type used by the API |  |
| interfaceDescriptions | array(InterfaceDescription) | O | 0..N | Interface details |  |
| dataFormat | DataFormat | O | 0..1 | Data formats used by the API |  |
| description | string | O | 0..1 | Text description of the API |  |
| uris | array(Uri) | O | 0..N | Relative URI (s) of the API |  |

##### 8.2.4.2.3	Type: InterfaceDescription

Table 8.2.4.2.3-1: Definition of type InterfaceDescription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ipv4Addr | Ipv4Addr | O | 0..1 | String identifying an IPv4 address |  |
| ipv6Addr | Ipv6Addr | O | 0..1 | String identifying an IPv6 address |  |
| port | Port | O | 0..1 | Port |  |
| securityMethods | array(SecurityMethods) | M | 1..N | Security methods supported by the interface |  |

#### 8.2.4.3	Simple data types and enumerations

##### 8.2.4.3.1	Introduction

This subclause defines simple data types and enumerations that can be referenced from data structures defined in the previous subclauses.

##### 8.2.4.3.2	Simple data types

The simple data types defined in table 8.2.4.3.2-1 shall be supported.

Table 8.2.4.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

##### 8.2.4.3.3	Enumeration: Protocol

Table 8.2.4.3.3-1: Enumeration Protocol

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| HTTP_1_1 | HTTP version 1.1 |  |
| HTTP2 | HTTP version 2 |  |

##### 8.2.4.3.4	Enumeration: DataFormat

Table 8.2.4.3.4-1: Enumeration DataFormat

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| JSON | Serialization protocol: JavaScript Object Notation |  |

##### 8.2.4.3.5	Enumeration: CommunicationType

Table 8.2.4.3.5-1: Enumeration CommunicationType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| REQUEST_RESPONSE | The communication is of the type request-response. |  |
| SUBSCRIBE_NOTIFY | The communication is of the type subscribe-notify |  |

##### 8.2.4.3.6	Enumeration: SecurityMethods

Table 8.2.4.3.6-1: Enumeration SecurityMethods

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| PSK | Security method 1 (Using TLS-PSK) as described in 3GPP TS 33.122 [16]. |  |
| PKI | Security method 2 (Using PKI) as described in 3GPP TS 33.122 [16]. |  |
| OAUTH | Security method 3 (TLS with OAuth token) as described in 3GPP TS 33.122 [16]. |  |

### 8.2.5	Error Handling

General error responses are defined in subclause 7.7.

### 8.2.6	Feature negotiation

General feature negotiation procedures are defined in subclause 7.8.

Table 8.2.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| n/a |  |  |

## 8.3	CAPIF_Events_API

### 8.3.1	API URI

The request URI used in each HTTP request from the Subscriber towards the CAPIF core function shall have the structure as defined in subclause 7.5 with the following clarifications:

-	The {apiName} shall be "capif-events".

-	The {apiVersion} shall be "v1".

-	The {apiSpecificSuffixes} shall be set as described in subclause 8.3.2.

### 8.3.2	Resources

#### 8.3.2.1	Overview

Figure 8.3.2.1-1: Resource URI structure of the CAPIF_Events_API

Table 8.3.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.3.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| CAPIF Events Subscriptions | {apiRoot}/capif-events/v1/ {subscriberId}/subscriptions | POST | Creates a new individual CAPIF Event Subscription |
| Individual CAPIF Events Subscription | {apiRoot}/capif-events/v1/ {subscriberId}/subscriptions/ {subscriptionId} | DELETE | Deletes an individual CAPIF Event Subscription identified by the subscriptionId |

#### 8.3.2.2	Resource: CAPIF Events Subscriptions

##### 8.3.2.2.1	Description

The CAPIF Events Subscriptions resource represents all subscriptions of aSubscriber.

##### 8.3.2.2.2	Resource Definition

Resource URI: {apiRoot}/capif-events/v1/{subscriberId}/subscriptions

This resource shall support the resource URI variables defined in table 8.3.2.2.2-1.

Table 8.3.2.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 7.5 |
| subscriberId | ID of the Subscriber |

##### 8.3.2.2.3	Resource Standard Methods

###### 8.3.2.2.3.1	POST

This method shall support the URI query parameters specified in table 8.3.2.2.3.1-1.

Table 8.3.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.3.2.2.3.1-2 and the response data structures and response codes specified in table 8.3.2.2.3.1-3.

Table 8.3.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EventSubscription | M | 1 | Create a new individual CAPIF Events Subscription resource. |

Table 8.3.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EventSubscription | M | 1 | 201 Created | CAPIF Events Subscription resource created successfully. The URI of the created resource shall be returned in the "Location" HTTP header |

##### 8.3.2.2.4	Resource Custom Operations

None.

#### 8.3.2.3	Resource: Individual CAPIF Events Subscription

##### 8.3.2.3.1	Description

The Individual CAPIF Events Subscription resource represents an individual event subscription of aSubscriber.

##### 8.3.2.3.2	Resource Definition

Resource URI: {apiRoot}/capif-events/v1/{subscriberId}/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 8.3.2.3.2-1.

Table 8.3.2.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 7.5 |
| subscriberId | ID of the Subscriber |
| subscriptionId | String identifying an individual Events Subscription |

##### 8.3.2.3.3	Resource Standard Methods

###### 8.3.2.3.3.1	DELETE

This method shall support the URI query parameters specified in table 8.3.2.3.3.1-1.

Table 8.3.2.3.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.3.2.3.3.1-2 and the response data structures and response codes specified in table 8.3.2.3.3.1-3.

Table 8.3.2.3.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.3.2.3.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The individual CAPIF Events Subscription matching the subscriptionId is deleted. |

##### 8.3.2.3.4	Resource Custom Operations

None.

### 8.3.3	Notifications

#### 8.3.3.1	General

The delivery of notifications shall conform to subclause 7.6.

Table 8.3.3.1-1: Notifications overview

| Notification | Resource URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Event notification | {notificationDestination} | POST | Notifies Subscriber of a CAPIF Event |

#### 8.3.3.2	Event Notification

##### 8.3.3.2.1	Description

Event Notification is used by the CAPIF core function to notify a Subscriber of an Event. The Subscriber shall be subscribed to such Event Notification via the Individual CAPIF Events Subscription Resource.

##### 8.3.3.2.2	Notification definition

The POST method shall be used for Event notification and the URI shall be the one provided by the Subscriber during the on-boarding request.

Resource URI: {notificationDestination}

This method shall support the URI query parameters specified in table 8.3.3.2.2.1-1.

Table 8.3.3.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.3.3.2.2-2 and the response data structures and response codes specified in table 8.3.3.2.2-3.

Table 8.3.3.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EventNotification | M | 1 | Notification information of a CAPIF Event |

Table 8.3.3.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |

### 8.3.4	Data Model

#### 8.3.4.1	General

This subclause specifies the application data model supported by the API. Data types listed in subclause 7.2 apply to this API.

Table 8.3.4.1-1 specifies the data types defined specifically for the CAPIF_Events_API service.

Table 8.3.4.1-1: CAPIF_Events_API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| EventSubscription | 8.3.4.2.2 | Represents an individual CAPIF Event Subscription resource |  |
| EventNotification | 8.3.4.2.3 | Represents an individual CAPIF Event Subscription Notification resource |  |
| CAPIFEvent | 8.3.4.3.2 | Describes CAPIF events |  |

Table 8.3.4.1-2 specifies data types re-used by the CAPIF_Events_API service:

Table 8.3.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Uri | 3GPP TS 29.122 [14] |  |  |
| TestNotification | 3GPP TS 29.122 [14] | Following differences apply: - The SCEF is the CAPIF core function; and - The SCS/AS is the Subscribing functional entity. |  |
| WebsockNotifConfig | 3GPP TS 29.122 [14] | Following differences apply: - The SCEF is the CAPIF core function; and - The SCS/AS is the Subscribing functional entity. |  |

#### 8.3.4.2	Structured data types

##### 8.3.4.2.1	Introduction

This subclause defines the structures to be used in resource representations.

##### 8.3.4.2.2	Type: EventSubscription

Table 8.3.4.2.2-1: Definition of type EventSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| events | array(CAPIFEvent) | M | 1..N | Subscribed events |  |
| notificationDestination | Uri | M | 1 | URI where the notification should be delivered to. |  |
| requestTestNotification | boolean | O | 0..1 | Set to true by Subscriber to request the CAPIF core function to send a test notification as defined in subclause 7.6. Set to false or omitted otherwise. | Notification_test_event |
| websockNotifConfig | WebsockNotifConfig | O | 0..1 | Configuration parameters to set up notification delivery over Websocket protocol as defined in subclause 7.6. | Notification_websocket |

##### 8.3.4.2.3	Type: EventNotification

Table 8.3.4.2.4-1: Definition of type EventNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subscriptionId | string | M | 1 | Identifier of the subscription resource to which the notification is related – CAPIF resource identifier |  |
| events | array(CAPIFEvent) | M | 1..N | Notifications of individual events |  |

#### 8.3.4.3	Simple data types and enumerations

##### 8.3.4.3.1	Introduction

This subclause defines simple data types and enumerations that can be referenced from data structures defined in the previous subclauses.

##### 8.3.4.3.2	Simple data types

None.

The simple data types defined in table 8.3.4.3.2-1 shall be supported.

Table 8.3.4.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

##### 8.3.4.3.3	Enumeration: CAPIFEvent

Table 8.3.4.3.3-1: Enumeration CAPIFEvent

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| SERVICE_API_AVAILABLE | Events related to the availability of service APIs after the service APIs are published. |  |
| SERVICE_API_UNAVAILABLE | Events related to the unavailability of service APIs after the service APIs are unpublished. |  |
| SERVICE_API_UPDATE | Events related to change in service API information |  |
| API_INVOKER_ONBOARDED | Events related to API invoker onboarded to CAPIF |  |
| API_INVOKER_OFFBOARDED | Events related to API invoker offboarded from CAPIF |  |
| SERVICE_API_INVOCATION_SUCCESS | Events related to the successful invocation of service APIs |  |
| SERVICE_API_INVOCATION_FAILURE | Events related to the failed invocation of service APIs |  |
| ACCESS_CONTROL_POLICY_UPDATE | Events related to the update for the access control policy related to the service APIs |  |
| ACCESS_CONTROL_POLICY_UNAVAILABLE | Events related to the unavailability of the access control policy related to the service APIs |  |
| API_INVOKER_AUTHORIZATION_REVOKED | Events related to the revocation of the authorization of API invokers to access the service APIs. |  |

8.3.5	Error Handling

General error responses are defined in subclause 7.7.

### 8.3.6	Feature negotiation

General feature negotiation procedures are defined in subclause 7.8. Table 8.3.6-1 lists the supported features for CAPIF_Events_API.

Table 8.3.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_test_event | Testing of notification connection is supported according to subclause 7.6. |
| 2 | Notification_websocket | The delivery of notifications over Websocket is supported according to subclause 7.6. This feature requires that the Notification_test_event feature is also supported. |

Editor's Note:	Supporting features specific to Subscriber is for further study.

## 8.4	CAPIF_API_Invoker_Management_API

### 8.4.1	API URI

The request URI used in each HTTP request from the API invoker towards the CAPIF core function shall have the structure as defined in subclause 7.5 with the following clarifications:

-	The {apiName} shall be "api-invoker-management".

-	The {apiVersion} shall be "v1".

-	The {apiSpecificSuffixes} shall be set as described in subclause 8.4.2.

### 8.4.2	Resources

#### 8.4.2.1	Overview

Figure 8.4.2.1-1: Resource URI structure of the CAPIF_API_Invoker_Management_API

Table 8.4.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.4.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| On-boarded API Invokers | {apiRoot} /api-invoker-management/v1 /onboardedInvokers | POST | On-boards a new API invoker by creating a API invoker profile |
| Individual On-boarded API Invoker | {apiRoot} /api-invoker-management/v1 /onboardedInvokers/{onboardingId} | DELETE | Off-boards an individual API invoker by deleting the API invoker profile identified by {onboardingId} |

#### 8.4.2.2	Resource: On-boarded API invokers

##### 8.4.2.2.1	Description

The On-boarded API Invokers resource represents all the API invokers that are on-boarded at a given CAPIF core function.

##### 8.4.2.2.2	Resource Definition

Resource URI: {apiRoot}/api-invoker-management/v1/onboardedInvokers

This resource shall support the resource URI variables defined in table 8.4.2.2.2-1.

Table 8.4.2.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 7.5 |

##### 8.4.2.2.3	Resource Standard Methods

###### 8.4.2.2.3.1	POST

This method shall support the URI query parameters specified in table 8.4.2.2.3.1-1.

Table 8.4.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.4.2.2.3.1-2 and the response data structures and response codes specified in table 8.4.2.2.3.1-3.

Table 8.4.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| APIInvokerEnrolmentDetails | M | 1 | Enrolment details of the API invoker including notification destination URI for any on-boarding related notifications and an optional list of APIs the API invoker intends to invoke while on-board. |

Table 8.4.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| APIInvokerEnrolmentDetails | M | 1 | 201 Created | API invoker on-boarded successfully The URI of the created resource shall be returned in the "Location" HTTP header. A list of APIs the API invoker is allowed to invoke while on-board may also be included. |
| n/a |  |  | 202 Accepted | The CAPIF core has accepted the Onboarding request and is processing it. When processing is completed, the CAPIF core function will send a Notify_Onboarding_Completion notification to the requesting API Invoker. See subclause 8.4.3.2. |

8.4.2.2.4	Resource Custom Operations

None.

#### 8.4.2.3	Resource: Individual On-boarded API Invoker

##### 8.4.2.3.1	Description

The Individual On-boarded API Invokers resource represents an individual API invoker that is on-boarded at a given CAPIF core function.

##### 8.4.2.3.2	Resource Definition

Resource URI: {apiRoot}/api-invoker-management/v1/onboardedInvokers/{onboardingId}

This resource shall support the resource URI variables defined in table 8.4.2.3.2-1.

Table 8.4.2.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 7.5 |
| onboardingId | String identifying an individual on-boarded API invoker resource |

##### 8.4.2.3.3	Resource Standard Methods

###### 8.4.2.3.3.1	DELETE

This method shall support the URI query parameters specified in table 8.4.2.3.3.1-1.

Table 8.4.2.3.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the response codes specified in table 8.4.2.3.3.1-2 and the response data structures and response codes specified in table 8.4.2.3.3.1-3.

Table 8.4.2.3.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.4.2.3.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The individual on-boarded API invoker matching the onboardingId is deleted |

##### 8.3.2.3.4	Resource Custom Operations

None.

### 8.4.3	Notifications

#### 8.4.3.1	General

The delivery of notifications shall conform to subclause 7.6.

Table 8.4.3.1-1: Notifications overview

| Notification | Resource URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Notify_Onboarding_Completion | {notificationDestination} | POST | Notify API invoker of on-boarding result |

#### 8.4.3.2	Notify_Onboarding_Completion

##### 8.4.3.2.1	Description

Notify_Onboarding_Completion is used by the CAPIF core function to notify an API invoker of the on-boarding result.

##### 8.4.3.2.2	Notification definition

The POST method shall be used for Notify_Onboarding_Completion and the URI shall be the one provided by the API invoker during the on-boarding request.

Resource URI: {notificationDestination}

This method shall support the URI query parameters specified in table 8.4.3.2.2-1.

Table 8.4.3.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.4.3.2.2-2 and the response data structures and response codes specified in table 8.4.3.2.2-3.

Table 8.4.3.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| OnboardingNotification | M | 1 | Notification with on-boarding result |

Table 8.4.3.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |

### 8.4.4	Data Model

#### 8.4.4.1	General

This subclause specifies the application data model supported by the API. Data types listed in subclause 7.2 apply to this API.

Table 8.4.4.1-1 specifies the data types defined specifically for the CAPIF_API_Invoker_Management_API service.

Table 8.4.4.1-1: CAPIF_API_Invoker_Management_API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| APIInvokerEnrolmentDetails | 8.4.4.2.2 | API invoker's enrolment details |  |
| OnboardingNotificationDestination | 8.4.4.2.3 | Notification destination details. |  |
| APIList | 8.4.4.2.4 | List of APIs |  |
| OnboardingInformation | 8.4.4.2.5 | On-boarding information of the API invoker |  |
| OnboardingRequestAck | 8.4.4.2.6 | Acknowledgement to received request. |  |
| OnboardingNotification | 8.4.4.2.7 | Notification with on-boarding result |  |

Table 8.4.4.1-2 specifies data types re-used by the CAPIF_API_Invoker_Management_API service.

Table 8.4.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ServiceAPIDescription | Subclause 8.1.4.2.2 |  |  |
| Uri | 3GPP TS 29.122 [14] |  |  |
| TestNotification | 3GPP TS 29.122 [14] |  |  |
| WebsockNotifConfig | 3GPP TS 29.122 [14] |  |  |

#### 8.4.4.2	Structured data types

##### 8.4.4.2.1	Introduction

##### 8.4.4.2.2	Type: APIInvokerEnrolmentDetails

Table 8.4.4.2.2-1: Definition of type APIInvokerEnrolmentDetails

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| apiInvokerId | string | O | 0..1 | API invoker ID assigned by the CAPIF core function to the API invoker while on-boarding the API invoker. Shall not be present in the HTTP POST request from the API invoker to the CAPIF core function, to on-board itself. Shall be present in all other HTTP requests and responses. |  |
| onboardingInformation | OnboardingInformation | M | 1 | On-boarding information about the API invoker necessary for the CAPIF core function to on-board the API invoker. |  |
| onboardingNotificationDestination | OnboardingNotificationDestination | M | 1 | Onboarding notification destination information provided by the API invoker. |  |
| apiList | APIList | O | 0..1 | A list of APIs. When included by the API invoker in the HTTP request message, it lists the APIs that the API invoker intends to invoke while onboard. When included by the CAPIF core function in the HTTP response message, it lists the APIs that the API invoker is allowed to invoke while onboard. |  |
| apiInvokerInformation | string | O | 0..1 | Generic information related to the API invoker such as details of the device or the application. |  |

##### 8.4.4.2.3	Type: OnboardingNotificationDestination

Table 8.4.4.2.3-1: Definition of type OnboardingNotificationDestination

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notificationDestination | Uri | M | 1 | URI where the notification should be delivered to. |  |
| requestTestNotification | boolean | O | 0..1 | Set to true by Subscriber to request the CAPIF core function to send a test notification as defined in in subclause 7.6. Set to false or omitted otherwise. | Notification_test_event |
| websockNotifConfig | WebsockNotifConfig | O | 0..1 | Configuration parameters to set up notification delivery over Websocket protocol as defined in subclause 7.6. | Notification_websocket |

##### 8.4.4.2.4	Type: APIList

Table 8.4.4.2.4-1: Definition of type APIList

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| serviceAPIDescriptions | array(ServiceAPIDescription) | M | 1..N | Definition of the service API |  |

##### 8.4.4.2.5	Type: OnboardingInformation

Table 8.4.4.2.5-1: Definition of type OnboardingInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| apiInvokerPublicKey | string | M | 1 | Public Key of API Invoker |  |
| apiInvokerCertificate | string | O | 0..1 | API invoker's generic client certificate |  |

##### 8.4.4.2.6	Type: OnboardingRequestAck

Table 8.4.4.2.6-1: Definition of type OnboardingRequestAck

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| onboardingNotificationDestination | OnboardingNotificationDestination | M | 1 | On-boarding notification destination related details |  |

##### 8.4.4.2.7	Type: OnboardingNotification

Table 8.4.4.2.7-1: Definition of type OnboardingNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| result | boolean | M | 1 | Set to "true" indicate successful on-boarding. Otherwise set to "false" |  |
| resourceLocation | Uri | C | 1 | URI pointing to the new CAPIF resource created as a result of successful on-boarding. This attribute shall be present if 'result' attribute is set to "true". Otherwise it shall not be present. |  |
| apiInvokerEnrolmentDetails | APIInvokerEnrolmentDetails | C | 1 | Enrolment details of the API invoker which are verified by the CAPIF administrator or API management. This attribute shall be present if 'result' attribute is set to "true". Otherwise it shall not be present. |  |
| apiList | APIList | O | 0..1 | List of APIs API invoker is allowed to access. This attribute may be present if 'result' attribute is set to "true". Otherwise it shall not be present. |  |

#### 8.4.4.3	Simple data types and enumerations

None.

### 8.4.5	Error Handling

General error responses are defined in subclause 7.7.

### 8.4.6	Feature negotiation

General feature negotiation procedures are defined in subclause 7.8. Table 8.4.6-1 lists the supported features for CAPIF_API_Invoker_Management_API.

Table 8.4.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_test_event | Testing of notification connection is supported according to subclause 7.6. |
| 2 | Notification_websocket | The delivery of notifications over Websocket is supported according to subclause 7.6. This feature requires that the Notification_test_event feature is also supported. |

## 8.5	CAPIF_Security_API

### 8.5.1	API URI

The request URI used in each HTTP request from the API invoker or the API exposing function towards the CAPIF core function shall have the structure as defined in subclause 7.5 with the following clarifications:

-	The {apiName} shall be "capif-security".

-	The {apiVersion} shall be "v1".

-	The {apiSpecificSuffixes} shall be set as described in subclause 8.5.2.

### 8.5.2	Resources

#### 8.5.2.1	Overview

Figure 8.5.2.1-1: Resource URI structure of the CAPIF_Security_API

Table 8.5.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.5.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Trusted API invokers | {apiRoot} /capif-security/v1 /trustedInvokers | POST | All trusted API invokers |
| Individual trusted API invoker | {apiRoot} /capif-security/v1 /trustedInvokers/{apiInvokerId} | GET | Retrieve authentication information of an API invoker |
|  |  | DELETE | Revoke the authorization of the API invoker |

#### 8.5.2.2	Resource: Trusted API invokers

##### 8.5.2.2.1	Description

The Trusted API Invokers resource represents all the API invokers that are trusted by the CAPIF core function and have received authentication information from the CAPIF core function.

##### 8.5.2.2.2	Resource Definition

Resource URI: {apiRoot}/capif-security/v1/trustedInvokers

This resource shall support the resource URI variables defined in table 8.5.2.2.2-1.

Table 8.5.2.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 7.5 |

##### 8.5.2.2.3	Resource Standard Methods

###### 8.5.2.2.3.1	POST

This method shall support the URI query parameters specified in table 8.5.2.2.3.1-1.

Table 8.5.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.5.2.2.3.1-2 and the response data structures and response codes specified in table 8.5.2.2.3.1-3.

Table 8.5.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ServiceSecurity | M | 1 | Security method request from the API invoker to the CAPIF core function. The request indicates a list of service APIs and a preferred method of security for the service APIs. The request also includes a notification destination URI for security related notifications. |

Table 8.5.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ServiceSecurity | M | 1 | 201 Created | Security method from the CAPIF core function to the API invoker is based on the received request. The response indicates the security method to be used for the service APIs The URI of the created resource shall be returned in the "Location" HTTP header. |

##### 8.5.2.2.4	Resource Custom Operations

None.

#### 8.5.2.3	Resource: Individual trusted API invokers

##### 8.5.2.3.1	Description

The Individual trusted API Invokers resource represents an individual API invokers that is trusted by the CAPIF core function and have received security related information from the CAPIF core function.

##### 8.5.2.3.2	Resource Definition

Resource URI: {apiRoot}/capif-security/v1/trustedInvokers/{apiInvokerId}

This resource shall support the resource URI variables defined in table 8.5.2.3.2-1.

Table 8.5.2.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 7.5 |
| apiInvokerId | String identifying an individual API invoker |

##### 8.5.2.3.3	Resource Standard Methods

###### 8.5.2.3.3.1	GET

This method shall support the URI query parameters specified in table 8.5.2.3.3.1-1.

Table 8.5.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| authenticationInfo | boolean | O | 0..1 | When set to 'true', it indicates the CAPIF core function to send the authentication information of the API invoker. Set to false or omitted otherwise. |
| authorizationInfo | boolean | O | 0..1 | When set to 'true', it indicates the CAPIF core function to send the authorization information of the API invoker. Set to false or omitted otherwise. |

This method shall support the request data structures specified in table 8.5.2.3.3.1-2 and the response data structures and response codes specified in table 8.5.2.3.3.1-3.

Table 8.5.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.5.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ServiceSecurity | M | 1 | 200 OK | The security related information of the API Invoker based on the request from the API exposing function. |

###### 8.5.2.3.3.2	DELETE

This method shall support the URI query parameters specified in table 8.5.2.3.3.2-1.

Table 8.5.2.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.5.2.3.3.2-2 and the response data structures and response codes specified in table 8.5.2.3.3.2-3.

Table 8.5.2.3.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.5.2.3.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Authorization of the API invoker revoked, and a notification is sent to the API invoker as specified in subclause 8.5.3.2 |

##### 8.5.2.3.4	Resource Custom Operations

None.

### 8.5.3	Notifications

#### 8.5.3.1	General

The delivery of notifications shall conform to subclause 7.6.

Table 8.5.3.1-1: Notifications overview

| Notification | Resource URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Authorization revoked notification | {notificationDestination} | POST | Notify API invoker that the authorization rights are revoked by the API exposing function. |

#### 8.5.3.2	Authorization revoked notification

##### 8.5.3.2.1	Description

Authorization revoked notification is used by the CAPIF core function to notify an API invoker that the authorization rights are revoked by the API exposing function.

##### 8.5.3.2.2	Notification definition

The POST method shall be used for Authorization revoked notification and the URI shall be the one provided by the API invoker during the Obtain_Security_Method service operation.

Resource URI: {notificationDestination}

This method shall support the URI query parameters specified in table 8.5.3.2.2-1.

Table 8.5.3.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.5.3.2.2-2 and the response data structures and response codes specified in table 8.5.3.2.2-3.

Table 8.5.3.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SecurityNotification | M | 1 | Notification with information related to revoked authorization. |

Table 8.5.3.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |

### 8.5.4	Data Model

#### 8.5.4.1	General

This subclause specifies the application data model supported by the API.

Table 8.5.4.1-1 specifies the data types defined for the CAPIF service based interface protocol.

Table 8.5.4.1-1: Specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ServiceSecurity | 8.5.4.2.2 | Details of the security method for each service API interface. When included by the API invoker, it shall indicate the preferred method of security. When included by the CAPIF core function, it shall indicate the security method to be used for the service API interface. |  |
| SecurityMethod | 8.5.4.2.3 | Interface details and the security method |  |
| SecurityNotificationDestination | 8.5.4.2.4 | Notification destination details |  |
| SecurityNotification | 8.5.4.2.5 | Revoked authorization notification details |  |

Table 8.5.4.1-2 specifies data types re-used by the CAPIF_Security_API service based interface:

Table 8.5.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| InterfaceDescription | Subclause 8.2.4.2.3 | Details of the interface |  |
| Uri | 3GPP TS 29.122 [14] |  |  |
| TestNotification | 3GPP TS 29.122 [14] | Following differences apply: - The SCEF is the CAPIF core function; and - The SCS/AS is the Subscribing functional entity. |  |
| WebsockNotifConfig | 3GPP TS 29.122 [14] | Following differences apply: - The SCEF is the CAPIF core function; and - The SCS/AS is the Subscribing functional entity. |  |

#### 8.5.4.2	Structured data types

##### 8.5.4.2.1	Introduction

##### 8.5.4.2.2	Type: ServiceSecurity

Table 8.5.4.2.2-1: Definition of type ServiceSecurity

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| securityPreferences | array(SecurityMethod) | O | 0..N | API invoker's preference of the security method for a service API interface. May be present in the HTTP POST request from the API invoker to the CAPIF core function to negotiate the security mechanism. Shall not be present in any other HTTP request or HTTP response. |  |
| selectedSecurityMethods | array(SecurityMethod) | C | 0..N | Security method selected by the CAPIF core function for each service API interface. Shall be present in the HTTP POST response from the CAPIF core function to the API invoker. Shall not be present in any other HTTP request or HTTP response. |  |
| apiInvokerSecurityMethods | array(SecurityMethod) | O | 0..N | Security method for each service API interface selected for the API invoker by the CAPIF core function. May be present only in the HTTP GET response from the CAPIF core function to the API exposing function. Shall not be present in any other HTTP request or HTTP response. |  |
| securityNotificationDestination | SecurityNotificationDestination | M | 1 | Security notification destination information provided by the API invoker. |  |

##### 8.5.4.2.3	Type: SecurityMethod

Table 8.5.4.2.3-1: Definition of type SecurityMethod

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| interfaceDetails | InterfaceDescription | M | 1 | Details of the interface |  |
| securityMethod | string | M | 1 | Security method for the interface |  |
| authenticationInfo | string | O | 0..1 | Authentication related information |  |
| authorizationInfo | string | O | 0..1 | Authorization related information |  |

Editor's Note: The data models should be improved to accommodate changes or improvements done in SA3.

##### 8.5.4.2.4	Type: SecurityNotificationDestination

Table 8.5.4.2.4-1: Definition of type SecurityNotificationDestination

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notificationDestination | Uri | M | 1 | URI where the notification should be delivered to. |  |
| requestTestNotification | boolean | O | 0..1 | Set to true by API invoker to request the CAPIF core function to send a test notification as defined in in subclause 7.6. Set to false or omitted otherwise. | Notification_test_event |
| websockNotifConfig | WebsockNotifConfig | O | 0..1 | Configuration parameters to set up notification delivery over Websocket protocol as defined in subclause 7.6. | Notification_websocket |

##### 8.5.4.2.5	Type: SecurityNotification

Table 8.5.4.2.5-1: Definition of type SecurityNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| apiInvokerId | string | M | 1 | String identifying the API invoker assigned by the CAPIF core function |  |
| apiId | string | M | 1 | Identifier of the service API |  |
| cause | Cause | M | 1 | The cause for revoking the API invoker authorization to the service API |  |

#### 8.5.4.3	Simple data types and enumerations

8.5.4.3.1	Introduction

This subclause defines simple data types and enumerations that can be referenced from data structures defined in the previous subclauses.

##### 8.5.4.3.2	Simple data types

The simple data types defined in table 8.5.4.3.2-1 shall be supported.

Table 8.5.4.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

##### 8.5.4.3.3	Enumeration: Cause

Table 8.5.4.3.3-1: Enumeration Cause

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| OVERLIMIT_USAGE | The revocation of the authorization of the API invoker is due to the overlimit usage of the service API |  |
| UNEXPECTED_REASON | The revocation of the authorization of the API invoker is due to unexpected reason. |  |

### 8.5.5	Error Handling

General error responses are defined in subclause 7.7.

### 8.5.6	Feature negotiation

General feature negotiation procedures are defined in subclause 7.8. Table 8.5.6-1 lists the supported features for CAPIF_Security_API.

Table 8.5.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_test_event | Testing of notification connection is supported according to subclause 7.6. |
| 2 | Notification_websocket | The delivery of notifications over Websocket is supported according to subclause 7.6. This feature requires that the Notification_test_event feature is also supported. |

## 8.6	CAPIF_Access_Control_Policy_API

### 8.6.1	API URI

The request URI used in each HTTP request from the API exposing function towards the CAPIF core function shall have the structure as defined in subclause 7.5 with the following clarifications:

-	The {apiName} shall be "access-control-policy".

-	The {apiVersion} shall be "v1".

-	The {apiSpecificSuffixes} shall be set as described in subclause 8.6.2.

### 8.6.2	Resources

#### 8.6.2.1	Overview

This resource is created by the CAPIF administrator on the CAPIF core function.

NOTE:	The details of the mechanisms used to create the Access Control Policy List resource on the CAPIF core function is out of the scope of the present document.

Figure 8.6.2.1-1: Resource URI structure of the CAPIF_Access_Control_Policy_API

Table 8.6.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.6.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Access Control Policy List | {apiRoot} /access-control-policy/v1 /accessControlPolicyList/{serviceApiId} | GET | Retrieves the access control policy list for a published service API. |

#### 8.6.2.2	Resource: Access Control Policy List

##### 8.6.2.2.1	Description

The Access Control Policy List resource represents the access control information for all the service APIs per API invoker.

##### 8.6.2.2.2	Resource Definition

Resource URI: {apiRoot}/access-control-policy/v1/accessControlPolicyList/{serviceApiId}

This resource shall support the resource URI variables defined in table 8.6.2.2.2-1.

Table 8.6.2.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 7.5 |
| serviceApiId | String identifying an individual published service API |

##### 8.6.2.2.3	Resource Standard Methods

###### 8.6.2.2.3.1	GET

This method shall support the URI query parameters specified in table 8.6.2.2.3.1-1.

Table 8.6.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| apiInvokerId | string | O | 1 | String identifying the API invoker |

This method shall support the request data structures specified in table 8.6.2.2.3.1-2 and the response data structures and response codes specified in table 8.6.2.2.3.1-3.

Table 8.6.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.6.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| accessControlPolicyList | M | 1 | 200 OK | List of the access control policy applicable for the service API requested. |

##### 8.6.2.2.4	Resource Custom Operations

None.

### 8.6.3	Notifications

None.

### 8.6.4	Data Model

#### 8.6.4.1	General

This subclause specifies the application data model supported by the API. Data types listed in subclause 7.2 apply to this API.

Table 8.6.4.1-1 specifies the data types defined specifically for the CAPIF_Access_Control_Policy_API service.

Table 8.6.4.1-1: CAPIF_Access_Control_Policy_API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| AccessControlPolicyList | 8.6.4.2.2 | Access control policy list |  |

Table 8.6.4.1-2 specifies data types re-used by the CAPIF_Access_Control_Policy_API service.

Table 8.6.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

#### 8.6.4.2	Structured data types

##### 8.6.4.2.1	Introduction

This subclause defines data structures to be used in resource representations.

##### 8.6.4.2.2	Type: AccessControlPolicyList

Table 8.6.4.2.2-1: Definition of type AccessControlPolicyList

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| apiInvokerPolicies | array(ApiInvokerPolicy) | M | 0..N | Policy of each API invoker. |  |

##### 8.6.4.2.3	Type: ApiInvokerPolicy

Table 8.6.4.2.3-1: Definition of type ApiInvokerPolicy

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| apiInvokerId | string | M | 1 | API invoker ID assigned by the CAPIF core function |  |
| allowedTotalInvocations | integer | O | 0..1 | Total number of invocations allowed on the service API by the API invoker. |  |
| allowedInvocationsPerSecond | integer | O | 0..1 | Invocations per second allowed on the service API by the API invoker. |  |
| allowedInvocationTimeRangeList | array(TimeRangeList) | O | 1..N | The time ranges during which the invocations are allowed on the service API by the API invoker. |  |

##### 8.6.4.2.4	Type: TimeRangeList

Table 8.6.4.2.4-1: Definition of type TimeRangeList

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| startTime | DateTime | M | 1 | The start time for the invocations to be allowed on the service API by the API invoker. |  |
| endTime | DateTime | M | 1 | The end time for the invocations to be allowed on the service API by the API invoker. |  |

#### 8.6.4.3	Simple data types and enumerations

None.

### 8.6.5	Error Handling

General error responses are defined in subclause 7.7.

### 8.6.6	Feature negotiation

General feature negotiation procedures are defined in subclause 7.8.

Table 8.6.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| n/a |  |  |

## 8.7	CAPIF_Logging_API_Invocation_API

### 8.7.1	API URI

The request URI used in each HTTP request from the API exposing function towards the CAPIF core function shall have the structure as defined in subclause 7.5 with the following clarifications:

-	The {apiName} shall be "api-invocation-logs".

-	The {apiVersion} shall be "v1".

-	The {apiSpecificSuffixes} shall be set as described in subclause 8.7.2

### 8.7.2	Resources

#### 8.7.2.1	Overview

Figure 8.7.2.1-1: Resource URI structure of the CAPIF_Logging_API_Invocation_API

Table 8.7.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.7.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Logs | {apiRoot} /api-invocation-logs/v1 /{aefId}/logs | POST | Creates a new log entry for service API invocations |
| Individual log | {apiRoot} /api-invocation-logs/v1 /{aefId}/logs/{logId} | n/a | Individual log entry |

#### 8.7.2.2	Resource: Logs

##### 8.7.2.2.1	Description

The Logs resource represents all the log entries created by a API exposing function at CAPIF core function.

##### 8.7.2.2.2	Resource Definition

Resource URI: {apiRoot}/api-invocation-logs/v1/{aefId}/logs

This resource shall support the resource URI variables defined in table 8.7.2.2.2-1.

Table 8.7.2.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 7.5 |
| aefId | Identity of the API exposing function |

##### 8.7.2.2.3	Resource Standard Methods

###### 8.7.2.2.3.1	POST

This method shall support the URI query parameters specified in table 8.7.2.2.3.1-1.

Table 8.7.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.7.2.2.3.1-2 and the response data structures and response codes specified in table 8.7.2.2.3.1-3.

Table 8.7.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| InvocationLogs | M | 201 Created | Log of service API invocations provided by API exposing function to store on the CAPIF core function. |

Table 8.7.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| InvocationLogs | M | 1 | 201 Created | Log of service API invocations provided by API exposing function successfully stored on the CAPIF core function. The URI of the created resource shall be returned in the "Location" HTTP header. |

##### 8.7.2.2.4	Resource Custom Operations

None.

### 8.7.3	Notifications

None.

### 8.7.4	Data Model

#### 8.7.4.1	General

This subclause specifies the application data model supported by the API.

Table 8.7.4.1-1 specifies the data types defined for the CAPIF service based interface protocol.

Table 8.7.4.1-1: Specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| InvocationLogs | 8.7.4.2.2 | Set of Service API invocation logs to be stored on CAPIF core function |  |
| Log | 8.7.4.2.3 | Individual log entries |  |

Table 8.7.4.1-2 specifies data types re-used by the CAPIF_Logging_API_Invocation_API service:

Table 8.7.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Ipv4Addr | 3GPP TS 29.122 [14] |  |  |
| Ipv6Addr | 3GPP TS 29.122 [14] |  |  |
| Port | 3GPP TS 29.122 [14] |  |  |
| InterfaceDescription | Subclause 8.2.4.2.3 | Details of the interface |  |

#### 8.7.4.2	Structured data types

##### 8.7.4.2.1	Introduction

##### 8.7.4.2.2	Type: InvocationLogs

Table 8.7.4.2.2-1: Definition of type InvocationLogs

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| aefId | string | M | 1 | Identity information of the API exposing function requesting logging of service API invocations |  |
| apiInvokerId | string | M | 1 | Identity of the API invoker which invoked the service API |  |
| ipv4Addr | Ipv4Addr | C | 0..1 | String identifying an IPv4 address of the API invoker. This attribute shall not be present if ipv6Addr attribute is present. |  |
| ipv6Addr | Ipv6Addr | C | 0..1 | String identifying an IPv6 address of the API invoker. This attribute shall not be present if ipv4Addr attribute is present. |  |
| port | Port | C | 0..1 | Port. This attribute shall be present if either ipv4Addr or the ipv6Addr attribute is present. |  |
| logs | array(Log) | M | 1..N | Service API invocation log |  |

##### 8.7.4.2.3	Type: Log

Table 8.7.4.2.3-1: Definition of type Log

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| apiId | string | M | 1 | String identifying the API invoked. |  |
| apiInvoked | string | M | 1 | Name of the API which was invoked |  |
| version | number | M | 1 | Version of the API which was invoked |  |
| resourceName | string | M | 1 | Name of the specific resource invoked |  |
| operation | string | M | 1 | Operation that was invoked on the API |  |
| result | string | M | 1 | Result or output of the invocation |  |
| invocationTime | DateTime | O | 0..1 | Date on which it was invoked |  |
| parameters | string | O | 0..1 | List of input parameters |  |
| interfaceDescription | InterfaceDescription | O | 0..1 | Interface description of the API invoked. |  |

#### 8.7.4.3	Simple data types and enumerations

None.

### 8.7.5	Error Handling

General error responses are defined in subclause 7.7.

### 8.7.6	Feature negotiation

Table 8.7.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| n/a |  |  |

## 8.8	CAPIF_Auditing_API

### 8.8.1	API URI

The request URI used in each HTTP request from the API management function towards the CAPIF core function shall have the structure as defined in subclause 7.5 with the following clarifications:

-	The {apiName} shall be "logs".

-	The {apiVersion} shall be "v1".

-	The {apiSpecificSuffixes} shall be set as described in subclause 8.8.2.

### 8.8.2	Resources

#### 8.8.2.1	Overview

Figure 8.8.2.1-1: Resource URI structure of the CAPIF_Auditing_API

Table 8.8.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.8.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| All service API invocation logs (Store) | {apiRoot} /logs/v1/apiInvocationLogs | GET | Query and retrieve service API invocation logs stored on the CAPIF core function |

8.8.2.2	Resource: All service API invocation logs

##### 8.8.2.2.1	Description

The All service API invocation logs resource represents a collection of service API invocation logs stored on the CAPIF core function. The resource is modelled as a Store resource archetype (see annex C.3 of 3GPP TS 29.501 [18])

##### 8.8.2.2.2	Resource Definition

Resource URI: {apiRoot}/logs/v1/apiInvocationLogs

This resource shall support the resource URI variables defined in table 8.8.2.2.2-1.

Table 8.8.2.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 7.5 |

##### 8.8.2.2.3	Resource Standard Methods

8.8.2.2.3.1	GET

This method shall support the URI query parameters specified in table 8.8.2.2.3.1-1.

Table 8.8.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| aefId | string | O | 0..1 | String identifying the API exposing function |
| apiInvokerId | string | O | 0..1 | String identifying the API invoker which invoked the service API |
| ipv4Addr | Ipv4Addr | C | 0..1 | String identifying a IPv4 address of the API invoker. This attribute shall not be present if ipv6Addr attribute is present. |
| ipv6Addr | Ipv6Addr | C | 0..1 | String identifying a IPv6 address of the API invoker. This attribute shall not be present if ipv4Addr attribute is present. |
| port | Port | C | 0..1 | Port. This attribute shall be present if either ipv4Addr or the ipv6Addr attribute is present. |
| timeRangeStart | DateTime | O | 0..1 | Start time of the invocation time range |
| timeRangeEnd | DateTime | O | 0..1 | End time of the invocation time range |
| apiId | string | O | 0..1 | String identifying the API invoked. |
| apiName | String | O | 0..1 | Name of the API which was invoked |
| version | number | O | 0..1 | Version of the API which was invoked |
| operation | string | O | 0..1 | Operation that was invoked on the API |
| result | string | O | 0..1 | Result or output of the invocation |
| resourceName | string | O | 0..1 | Name of the specific resource invoked |
| interfaceDescription | InterfaceDescription | O | 0..1 | Interface description of the API invoked. |

This method shall support the request data structures specified in table 8.8.2.2.3.1-2 and the response data structures and response codes specified in table 8.8.2.2.3.1-3.

Table 8.8.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.8.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(InvocationLogs) | O | 1..N | 200 OK | Result of the query operation along with fetched service API invocation log data. |
| ProblemDetails | M | 1 | 414 URI Too Long | Indicates that the server is refusing to service the request because the request-target is too long. |

##### 8.8.2.2.4	Resource Custom Operations

None.

8.8.3	Notifications

None.

### 8.8.4	Data Model

#### 8.8.4.1	General

This subclause specifies the application data model supported by the API.

Table 8.8.4.1-1 specifies the data types defined for the CAPIF service based interface protocol.

Table 8.8.4.1-1: Specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.8.4.1-2 specifies data types re-used by the CAPIF_Auditing_API service:

Table 8.8.4.1-2: Re-used Data Types

| Data type |  | Reference |  | Comments |  | Applicability |
| --- | --- | --- | --- | --- | --- | --- |
| InvocationLogs |  | Subclause 8.7.4.2.2 |  | Logs of service API invocations stored on the CAPIF core function. |  |  |
| ProblemDetails | 3GPP TS 29.122 [14] |  |  |  |  |  |
| Ipv4Addr |  | 3GPP TS 29.122 [14] |  |  |  |  |
| Ipv6Addr |  | 3GPP TS 29.122 [14] |  |  |  |  |
| Uri |  | 3GPP TS 29.122 [14] |  |  |  |  |
| Port |  | 3GPP TS 29.122 [14] |  |  |  |  |

#### 8.8.4.2	Structured data types

None.

8.8.4.3	Simple data types and enumerations

None.

### 8.8.5	Error Handling

General error responses are defined in subclause 7.7.

### 8.8.6	Feature negotiation

General feature negotiation procedures are defined in subclause 7.8.

Table 8.8.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| n/a |  |  |

# 9	AEF API Definition

## 9.1	AEF_Authentication_API

9.1.1	API URI

The request URI used in each HTTP request from the API invoker towards the API exposing function shall have the following structure:

-	The {apiRoot} shall be as defined in the service API specification using CAPIF.

-	The {apiName} shall be "auth-initiation".

-	The {apiVersion} shall be "v1".

-	The {apiSpecificSuffixes} shall be set as described in subclause 9.1.2.

### 9.1.2	Resources

#### 9.1.2.1	Overview

Figure 9.1.2.1-1: Resource URI structure of the AEF_Authentication_API

Table 9.1.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 9.1.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| API invoker authentication profiles | {apiRoot} /auth-initiation/v1 /auth-profiles | GET | Checks if the API exposing function has credentials for API invoker authentication |

#### 9.1.2.2	Resource: API invoker authentication profiles

##### 9.1.2.2.1	Description

The API invoker authentication profiles represents all the authentication profiles of different API invokers available at the API exposing function.

##### 9.1.2.2.2	Resource Definition

Resource URI: {apiRoot}/auth-initiation /v1/auth-profiles

This resource shall support the resource URI variables defined in table 9.1.2.2.2-1.

Table 9.1.2.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | Defined in the service API specification using CAPIF |

##### 9.1.2.2.3	Resource Standard Methods

###### 9.1.2.2.3.1	GET

This method shall support the URI query parameters specified in table 9.1.2.2.3.1-1.

Table 9.1.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| apiInvokerId | string | M | 1 | API invoker ID assigned by the CAPIF core function to the API invoker while on-boarding the API invoker. |

This method shall support the request data structures specified in table 9.1.2.2.3.1-2 and the response data structures and response codes specified in table 9.1.2.2.3.1-3.

Table 9.1.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 9.1.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | API exposing function confirms that the authentication profile is available and it is ready for authentication of the API invoker. |

##### 9.1.2.2.4	Resource Custom Operations

None.

### 9.1.3	Notifications

None.

### 9.1.4	Data Model

#### 9.1.4.1	General

This subclause specifies the application data model supported by the API. Data types listed in subclause 7.2 apply to this API.

Table 9.1.4.1-1 specifies the data types defined specifically for the AEF_Authentication_API service.

Table 9.1.4.1-1: AEF_Authentication_API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 9.1.4.1-2 specifies data types re-used by the CAPIF_API_Invoker_Management_API service.

Table 9.1.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

#### 9.1.4.2	Structured data types

None.

9.1.4.3	Simple data types and enumerations

None.

### 9.1.5	Error Handling

General error responses are defined in the service API specification using CAPIF.

### 9.1.6	Feature negotiation

General feature negotiation procedures are defined in the service API specification using CAPIF.

Table 9.1.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| n/a |  |  |

# 10	Security

## 10.1	General

Security methods for CAPIF are specified in 3GPP TS 33.122 [16].

## 10.1	CAPIF-1/1e security

Secure communication between API invoker and CAPIF core function over CAPIF-1 or CAPIF-1e reference point, using a TLS protocol based connection is defined in 3GPP TS 33.122 [16].

For Onboard_API_Invoker service operation of the CAPIF_API_Invoker_Management_API, the TLS protocol based connection shall be established using server certificate as defined in 3GPP TS 33.122 [16].

For rest of the CAPIF APIs, the TLS protocol based connection shall be established with certificate based mutual authentication as defined in 3GPP TS 33.122 [16].

## 10.2	CAPIF-2/2e security and securely invoking service APIs

For secure communication between API invoker and API exposing function and ensuring secure invocations of service APIs, the API invoker:

-	shall negotiate the security method with the CAPIF core function using the Obtain_Security_Method service operation of the CAPIF_Security_API;

-	shall initiate the authentication with the API exposing function using the Authentication_Initiation_Request service operation of the AEF_Authentication _API; and

-	shall establish a secure connection with the API exposing function as defined in 3GPP TS 33.122 [16], using the method negotiated with the CAPIF core function.

Annex A (normative): OpenAPI specification

## A.1	General

This subclause will describe the purpose of the Annex.

## A.2	CAPIF_Discover_Service_API

openapi: 3.0.0

info:

description: Discover_Service API

title: CAPIF_Discover_Service_API

version: "1.PreR15.0.0"

servers:

- url: '{apiRoot}/service-apis/v1'

variables:

apiRoot:

default: https://demohost.com

description: apiRoot as defined in subclause 7.5 of 3GPP TS 29.222.

paths:

/allServiceAPIs:

get:

description: Discover published service APIs and retrieve a collection of APIs according to certain filter criteria.

parameters:

- name: apiInvokerId

in: query

description: String identifying the API invoker assigned by the CAPIF core function.

required: true

schema:

type: string

- name: serviceName

in: query

description: Name of the service.

schema:

type: string

- name: apiName

in: query

description: API name.

schema:

type: string

- name: apiVersion

in: query

description: API version.

schema:

type: string

- name: commType

in: query

description: Communication type used by the API (e.g. request/response or subscribe/notify).

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/CommunicationType'

- name: interfaceDescription

in: query

description: Interface details (e.g. domain name, ipv4/6Addr, port).

schema:

type: array

items:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/InterfaceDescription'

- name: dataFormat

in: query

description: Data formats used by the API (e.g. serialization protocol JSON used).

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/DataFormat'

responses:

'200':

description: The response body contains the result of the search over the list of registered APIs.

content:

application/json:

schema:

$ref: '#/components/schemas/DiscoveredAPIs'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'409':

$ref: 'TS29122_CommonData.yaml#/components/responses/409'

'412':

$ref: 'TS29122_CommonData.yaml#/components/responses/412'

'414':

description: URI Too Long

content:

application/problem+json:

schema:

$ref: 'TS29122_CommonData.yaml#/components/schemas/ProblemDetails'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

schemas:

DiscoveredAPIs:

type: array

properties:

apiId:

type: string

description: Identifier of the service API

serviceAPIDescription:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/ServiceAPIDescription'

required:

- apiId

- serviceAPIDescription

## A.3	CAPIF_Publish_Service_API

openapi: 3.0.0

info:

title: CAPIF_Publish_Service_API

version: "1.PreR15.0.0"

servers:

- url: '{apiRoot}/published-apis/v1'

variables:

apiRoot:

default: https://demohost.com

description: apiRoot as defined in subclause 7.5 of 3GPP TS 29.222.

paths:

# APF published API

/{apfId}/service-apis:

post:

description: Publish a new API.

parameters:

- name: apfId

in: path

required: true

schema:

$ref: '#/components/schemas/apfId'

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceAPIDescription'

responses:

'201':

description: Service API published successfully The URI of the created resource shall be returned in the "Location" HTTP header.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceAPIDescription'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'409':

$ref: 'TS29122_CommonData.yaml#/components/responses/409'

'412':

$ref: 'TS29122_CommonData.yaml#/components/responses/412'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

get:

description: Retrieve all published APIs.

parameters:

- name: apfId

in: path

required: true

schema:

$ref: '#/components/schemas/apfId'

responses:

'200':

description: Definition of all service API(s) published by the API publishing function.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceAPIDescription'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'409':

$ref: 'TS29122_CommonData.yaml#/components/responses/409'

'412':

$ref: 'TS29122_CommonData.yaml#/components/responses/412'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

# Individual APF published API

/{apfId}/service-apis/{serviceApiId}:

get:

description: Retrieve a published service API.

parameters:

- name: serviceApiId

in: path

required: true

schema:

$ref: '#/components/schemas/serviceApiId'

- name: apfId

in: path

required: true

schema:

$ref: '#/components/schemas/apfId'

responses:

'200':

description: Definition of all service API published by the API publishing function.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceAPIDescription'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'409':

$ref: 'TS29122_CommonData.yaml#/components/responses/409'

'412':

$ref: 'TS29122_CommonData.yaml#/components/responses/412'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

description: Update a published service API.

parameters:

- name: serviceApiId

in: path

required: true

schema:

$ref: '#/components/schemas/serviceApiId'

- name: apfId

in: path

required: true

schema:

$ref: '#/components/schemas/apfId'

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceAPIDescription'

responses:

'200':

description: Definition of service API updated successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceAPIDescription'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'409':

$ref: 'TS29122_CommonData.yaml#/components/responses/409'

'412':

$ref: 'TS29122_CommonData.yaml#/components/responses/412'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

description: Unpublish a published service API.

parameters:

- name: serviceApiId

in: path

required: true

schema:

$ref: '#/components/schemas/serviceApiId'

- name: apfId

in: path

required: true

schema:

$ref: '#/components/schemas/apfId'

responses:

'204':

description: The individual published service API matching the serviceAPiId is deleted.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'412':

$ref: 'TS29122_CommonData.yaml#/components/responses/412'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

# Components

components:

schemas:

# Data types uses as path variables

apfId:

type: string

description: Identification of the API publishing function.

serviceApiId:

type: string

description: String identifying an individual published service API.

# Data Type for representations

ServiceAPIDescription:

type: object

properties:

apiName:

type: string

description: API name

apiID:

type: string

description: API identifier assigned by the CAPIF core function to the published service API. Shall not be present in the HTTP POST request from the API publishing function to the CAPIF core function. Shall be present in the HTTP POST response from the CAPIF core function to the API publishing function.

apiVersion:

type: string

description: API version

serviceName:

type: string

description: Name of the service to which the API belongs

interfaceDescriptions:

type: array

items:

$ref: '#/components/schemas/InterfaceDescription'

minItems: 0

description: Interface details

commType:

$ref: '#/components/schemas/CommunicationType'

dataFormat:

$ref: '#/components/schemas/DataFormat'

description:

type: string

description: Text description of the API

uris:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

minItems: 0

description: Relative URI (s) of the API

domainName:

type: string

description: Domain to which API belongs to

protocol:

$ref: '#/components/schemas/Protocol'

required:

- apiName

InterfaceDescription:

type: object

properties:

ipv4Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv4Addr'

ipv6Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv6Addr'

port:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Port'

securityMethods:

type: array

items:

$ref: '#/components/schemas/SecurityMethods'

minItems: 1

description: Security methods supported by the interface

Protocol:

anyOf:

- type: string

enum:

- HTTP_1_1

- HTTP_2

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- HTTP_1_1: HTTP version 1.1

- HTTP_2: HTTP version 2

CommunicationType:

anyOf:

- type: string

enum:

- REQUEST_RESPONSE

- SUBSCRIBE_NOTIFY

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- REQUEST_RESPONSE: The communication is of the type request-response

- SUBSCRIBE_NOTIFY: The communication is of the type subscribe-notify

DataFormat:

anyOf:

- type: string

enum:

- JSON

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- JSON: JavaScript Object Notation

SecurityMethods:

anyOf:

- type: string

enum:

- PSK

- PKI

- OAUTH

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- PSK: Security method 1 (Using TLS-PSK) as described in 3GPP TS 33.122

- PKI: Security method 2 (Using PKI) as described in 3GPP TS 33.122

- OAUTH: Security method 3 (TLS with OAuth token) as described in 3GPP TS 33.122

## A.4	CAPIF_Events_API

openapi: 3.0.0

info:

title: CAPIF_Events_API

version: "1.PreR15.0.0"

servers:

- url: '{apiRoot}/capif-events/v1'

variables:

apiRoot:

default: https://demohost.com

description: apiRoot as defined in subclause 7.5 of 3GPP TS 29.222

paths:

/{subscriberId}/subscriptions:

post:

description: Creates a new individual CAPIF Event Subscription.

parameters:

- name: subscriberId

in: path

description: Identifier of the Subscriber

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EventSubscription'

callbacks:

notificationDestination:

'{request.body#/notificationDestination}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EventNotification'

responses:

'204':

description: No Content (successful notification)

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'409':

$ref: 'TS29122_CommonData.yaml#/components/responses/409'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'412':

$ref: 'TS29122_CommonData.yaml#/components/responses/412'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: Created (Successful creation of subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/EventSubscription'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'409':

$ref: 'TS29122_CommonData.yaml#/components/responses/409'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'412':

$ref: 'TS29122_CommonData.yaml#/components/responses/412'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/{subscriberId}/subscriptions/{subscriptionId}:

delete:

description: Deletes an individual CAPIF Event Subscription.

parameters:

- name: subscriberId

in: path

description: Identifier of the Subscriber

required: true

schema:

type: string

- name: subscriptionId

in: path

description: Identifier of an individual Events Subscription

required: true

schema:

type: string

responses:

'204':

description: The individual CAPIF Events Subscription matching the subscriptionId is deleted.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'412':

$ref: 'TS29122_CommonData.yaml#/components/responses/412'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

schemas:

EventSubscription:

type: object

properties:

events:

type: array

items:

$ref: '#/components/schemas/CAPIFEvent'

minItems: 1

description: Subscribed events

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

requestTestNotification:

type: boolean

description: Set to true by Subscriber to request the CAPIF core function to send a test notification as defined in in subclause 7.6. Set to false or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

required:

- events

- notificationDestination

EventNotification:

type: object

properties:

subscriptionId:

type: string

description: Identifier of the subscription resource to which the notification is related – CAPIF resource identifier

events:

$ref: '#/components/schemas/CAPIFEvent'

required:

- subscriptionId

- events

CAPIFEvent:

anyOf:

- type: string

enum:

- SERVICE_API_AVAILABLE

- SERVICE_API_UNAVAILABLE

- SERVICE_API_UPDATE

- API_INVOKER_ONBOARDED

- API_INVOKER_OFFBOARDED

- SERVICE_API_INVOCATION_SUCCESS

- SERVICE_API_INVOCATION_FAILURE

- ACCESS_CONTROL_POLICY_UPDATE

- ACCESS_CONTROL_POLICY_UNAVAILABLE

- API_INVOKER_AUTHORIZATION_REVOKED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- SERVICE_API_AVAILABLE: Events related to the availability of service APIs after the service APIs are published.

- SERVICE_API_UNAVAILABLE: Events related to the unavailability of service APIs after the service APIs are unpublished.

- SERVICE_API_UPDATE: Events related to change in service API information.

- API_INVOKER_ONBOARDED: Events related to API invoker onboarded to CAPIF.

- API_INVOKER_OFFBOARDED: Events related to API invoker offboarded from CAPIF.

- SERVICE_API_INVOCATION_SUCCESS: Events related to the successful invocation of service APIs.

- SERVICE_API_INVOCATION_FAILURE: Events related to the failed invocation of service APIs.

- ACCESS_CONTROL_POLICY_UPDATE: Events related to the update for the access control policy related to the service APIs.        - ACCESS_CONTROL_POLICY_UNAVAILABLE: Events related to the unavailability of the access control policy related to the service APIs.

- API_INVOKER_AUTHORIZATION_REVOKED: Events related to the revocation of the authorization of API invokers to access the service APIs.

## A.5	CAPIF_API_Invoker_Management_API

One subclause is introduced per Service, with the corresponding OpenAPI 3.0.0 Document.

openapi: 3.0.0

info:

title: CAPIF_API_Invoker_Management_API

version: "1.PreR15.0.0"

servers:

- url: '{apiRoot}/api-invoker-management/v1'

variables:

apiRoot:

default: https://demohost.com

description: apiRoot as defined in subclause 7.5 of 3GPP TS 29.222

paths:

/onboardedInvokers:

post:

description: Creates a new individual API Invoker profile.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/APIInvokerEnrolmentDetails'

callbacks:

notificationDestination:

'{request.body#/notificationDestination}':

post:

description: Notify the API Invoker about the onboarding completion

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/OnboardingNotification'

responses:

'204':

description: No Content (successful onboarding notification)

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'409':

$ref: 'TS29122_CommonData.yaml#/components/responses/409'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'412':

$ref: 'TS29122_CommonData.yaml#/components/responses/412'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: API invoker on-boarded successfully

content:

application/json:

schema:

$ref: '#/components/schemas/APIInvokerEnrolmentDetails'

'202':

description: The CAPIF core has accepted the Onboarding request and is processing it.

content:

application/json:

schema:

$ref: '#/components/schemas/OnboardingRequestAck'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'409':

$ref: 'TS29122_CommonData.yaml#/components/responses/409'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'412':

$ref: 'TS29122_CommonData.yaml#/components/responses/412'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/onboardedInvokers/{onboardingId}:

delete:

description: Deletes an individual API Invoker.

parameters:

- name: onboardingId

in: path

description: String identifying an individual on-boarded API invoker resource

required: true

schema:

type: string

responses:

'204':

description: The individual API Invoker matching onboardingId was offboarded.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'412':

$ref: 'TS29122_CommonData.yaml#/components/responses/412'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

schemas:

OnboardingInformation:

type: object

properties:

apiInvokerPublicKey:

type: string

description: The API Invoker’s public key

apiInvokerCertificate:

type: string

description: The API Invoker’s generic client certificate

required:

- apiInvokerPublicKey

OnboardingNotificationDestination:

type: object

properties:

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

requestTestNotification:

type: boolean

description: Set to true by Subscriber to request the CAPIF core function to send a test notification as defined in in subclause 7.6. Set to false or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

APIList:

type: array

items:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/ServiceAPIDescription'

minItems: 1

description: The list of service APIs that the API Invoker is allowed to invoke

APIInvokerEnrolmentDetails:

type: object

properties:

apiInvokerId:

type: string

description: API invoker ID assigned by the CAPIF core function to the API invoker while on-boarding the API invoker. Shall not be present in the HTTP POST request from the API invoker to the CAPIF core function, to on-board itself. Shall be present in all other HTTP requests and responses.

onboardingInformation:

$ref: '#/components/schemas/OnboardingInformation'

onboardingNotificationDestination:

$ref: '#/components/schemas/OnboardingNotificationDestination'

apiList:

$ref: '#/components/schemas/APIList'

apiInvokerInformation:

type: string

description: Generic information related to the API invoker such as details of the device or the application.

required:

- onboardingInformation

- onboardingNotificationDestination

description: Information about the API Invoker that requested to onboard

OnboardingRequestAck:

type: object

properties:

onboardingNotificationDestination:

$ref: '#/components/schemas/OnboardingNotificationDestination'

required:

- onboardingNotificationDestination

OnboardingNotification:

type: object

properties:

result:

type: boolean

description: Set to "true" indicate successful on-boarding. Otherwise set to "false"

resourceLocation:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

apiInvokerEnrolmentDetails:

$ref: '#/components/schemas/APIInvokerEnrolmentDetails'

apiList:

$ref: '#/components/schemas/APIList'

required:

- result

## A.6	CAPIF_Security_API

openapi: 3.0.0

info:

title: CAPIF_Security_API

version: "1.PreR15.0.0"

servers:

- url: '{apiRoot}/capif-security/v1'

variables:

apiRoot:

default: https://demohost.com

description: apiRoot as defined in subclause 7.5 of 3GPP TS 29.222.

paths:

/trustedInvokers:

post:

requestBody:

description: All trusted API invokers

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSecurity'

callbacks:

notificationDestination:

'{request.body#/notificationDestination}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SecurityNotification'

responses:

'204':

description: No Content (successful notification)

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'409':

$ref: 'TS29122_CommonData.yaml#/components/responses/409'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'412':

$ref: 'TS29122_CommonData.yaml#/components/responses/412'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: Successful created.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSecurity'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'409':

$ref: 'TS29122_CommonData.yaml#/components/responses/409'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'412':

$ref: 'TS29122_CommonData.yaml#/components/responses/412'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/trustedInvokers/{apiInvokerId}:

get:

parameters:

- name: apiInvokerId

in: path

description: Identifier of an individual API invoker

required: true

schema:

type: string

- name: authenticationInfo

in: query

description: When set to 'true', it indicates the CAPIF core function to send the authentication information of the API invoker. Set to false or omitted otherwise.

schema:

type: boolean

- name: authorizationInfo

in: query

description: When set to 'true', it indicates the CAPIF core function to send the authorization information of the API invoker. Set to false or omitted otherwise.

schema:

type: boolean

responses:

'200':

description: The security related information of the API Invoker based on the request from the API exposing function.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSecurity'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'412':

$ref: 'TS29122_CommonData.yaml#/components/responses/412'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

parameters:

- name: apiInvokerId

in: path

description: Identifier of an individual API invoker

required: true

schema:

type: string

responses:

'204':

description: No Content (Successful deletion of the existing subscription)

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'412':

$ref: 'TS29122_CommonData.yaml#/components/responses/412'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

schemas:

ServiceSecurity:

type: object

properties:

securityPreferences:

type: array

items:

$ref: '#/components/schemas/SecurityMethod'

minimum: 0

selectedSecurityMethods:

type: array

items:

$ref: '#/components/schemas/SecurityMethod'

minimum: 0

apiInvokerSecurityMethods:

type: array

items:

$ref: '#/components/schemas/SecurityMethod'

minItems: 0

securityNotificationDestination:

$ref: '#/components/schemas/SecurityNotificationDestination'

required:

- securityNotificationDestination

SecurityMethod:

type: object

properties:

interfaceDetails:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/InterfaceDescription'

securityMethod:

type: string

description: Security method for the interface

authenticationInfo:

type: string

description: Authentication related information

authorizationInfo:

type: string

description: Authorization related information

required:

- interfaceDetails

- securityMethod

SecurityNotificationDestination:

type: object

properties:

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

requestTestNotification:

type: boolean

description: Set to true by API invoker to request the CAPIF core function to send a test notification as defined in in subclause 7.6. Set to false or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

required:

- notificationDestination

SecurityNotification:

type: object

properties:

apiInvokerId:

type: string

description: String identifying the API invoker assigned by the CAPIF core function

apiId:

type: string

description: Identifier of the service API

cause:

$ref: '#/components/schemas/Cause'

required:

- apiInvokerId

- apiId

- cause

Cause:

anyOf:

- type: string

enum:

- OVERLIMIT_USAGE

- UNEXPECTED_REASON

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- OVERLIMIT_USAGE: The revocation of the authorization of the API invoker is due to the overlimit usage of the service API

- UNEXPECTED_REASON: The revocation of the authorization of the API invoker is due to unexpected reason.

## A.7	CAPIF_Access_Control_Policy_API

openapi: 3.0.0

info:

title: CAPIF_Access_Control_Policy_API

version: "1.PreR15.0.0"

servers:

- url: '{apiRoot}/access-control-policy/v1'

variables:

apiRoot:

default: https://demohost.com

description: apiRoot as defined in subclause 7.5 of 3GPP TS 29.222

paths:

/accessControlPolicyList/{serviceApiId}:

get:

description: Retrieves the access control policy list.

parameters:

- name: serviceApiId

in: path

description: Identifier of a published service API

required: true

schema:

type: string

- name: apiInvokerId

in: query

description: Identifier of the API invoker

schema:

type: string

responses:

'200':

description: OK.

content:

application/json:

schema:

$ref: '#/components/schemas/accessControlPolicyList'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'412':

$ref: 'TS29122_CommonData.yaml#/components/responses/412'

'414':

$ref: 'TS29122_CommonData.yaml#/components/responses/414'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

schemas:

accessControlPolicyList:

type: object

properties:

apiInvokerPolicies:

type: array

items:

$ref: '#/components/schemas/ApiInvokerPolicy'

minItems: 0

description: Policy of each API invoker.

ApiInvokerPolicy:

type: object

properties:

apiInvokerId:

type: string

description: API invoker ID assigned by the CAPIF core function

allowedTotalInvocations:

type: integer

description: Total number of invocations allowed on the service API by the API invoker.

allowedInvocationsPerSecond:

type: integer

description: Invocations per second allowed on the service API by the API invoker.

allowedInvocationTimeRangeList:

type: array

items:

$ref: '#/components/schemas/TimeRangeList'

minItems: 0

description: The time ranges during which the invocations are allowed on the service API by the API invoker.

required:

- apiInvokerID

TimeRangeList:

type: object

properties:

startTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

stopTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

## A.8	CAPIF_Logging_API_Invocation_API

openapi: 3.0.0

info:

title: CAPIF_Logging_API_Invocation_API

version: "1.PreR15.0.0"

servers:

- url: '{apiRoot}/api-invocation-logs/v1'

variables:

apiRoot:

default: https://demohost.com

description: apiRoot as defined in subclause 7.5 of 3GPP TS 29.222

paths:

/{aefId}/logs:

post:

description: Creates a new log entry for service API invocations.

parameters:

- name: aefId

in: path

description: Identifier of the API exposing function

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/InvocationLogs'

responses:

'201':

description: Log of service API invocations provided by API exposing function successfully stored on the CAPIF core function.

content:

application/json:

schema:

$ref: '#/components/schemas/InvocationLogs'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'409':

$ref: 'TS29122_CommonData.yaml#/components/responses/409'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'412':

$ref: 'TS29122_CommonData.yaml#/components/responses/412'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/{aefId}/logs/{logId}:

description: Creates a new log entry for service API invocations.

parameters:

- name: aefId

in: path

description: Identifier of the API exposing function

required: true

schema:

type: string

- name: logId

in: path

description: Identifier of individual log entry

required: true

schema:

type: string

components:

schemas:

InvocationLogs:

type: object

properties:

aefId:

type: string

description: Identity information of the API exposing function requesting logging of service API invocations

apiInvokerId:

type: string

description: Identity of the API invoker which invoked the service API

ipv4Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv4Addr'

ipv6Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv6Addr'

port:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Port'

logs:

type: array

items:

$ref: '#/components/schemas/Log'

minItems: 1

description: Service API invocation log

required:

- aefId

- apiInvokerId

- log

Log:

type: object

properties:

apiId:

type: string

description: String identifying the API invoked.

apiInvoked:

type: string

description: Name of the API which was invoked

version:

type: number

description: Version of the API which was invoked

resourceName:

type: string

description: Name of the specific resource invoked

operation:

type: string

description: Operation that was invoked on the API

result:

type: string

description: Result or output of the invocation

invocationTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

parameters:

type: string

description: List of input parameters

interfaceDescription:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml/InterfaceDescription'

required:

- apiId

- apiInvoked

- version

- resourceName

- operation

- result

## A.9	CAPIF_Auditing_API

openapi: 3.0.0

info:

description: Query_API_Invocation_Log

title: CAPIF_Auditing_API

version: "1.PreR15.0.0"

servers:

- url: '{apiRoot}/service-apis/v1'

variables:

apiRoot:

default: https://demohost.com

description: apiRoot as defined in subclause 7.5 of 3GPP TS 29.222.

paths:

/apiInvocationLogs:

get:

description: Query and retrieve service API invocation logs stored on the CAPIF core function.

parameters:

- name: aefId

in: query

description: String identifying the API exposing function.

schema:

type: string

- name: apiInvokerId

in: query

description: String identifying the API invoker which invoked the service API.

schema:

type: string

- name: ipv4Addr

in: query

description: String identifying a IPv4 address of the API invoker. This attribute shall not be present if ipv6Addr attribute is present.

schema:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv4Addr'

- name: ipv6Addr

in: query

description: String identifying a IPv4 address of the API invoker. This attribute shall not be present if ipv6Addr attribute is present.

schema:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv6Addr'

- name: port

in: query

description: Port. This attribute shall be present if either ipv4Addr or the ipv6Addr attribute is present.

schema:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Port'

- name: timeRangeStart

in: query

description: Start time of the invocation time range.

schema:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

- name: timeRangeEnd

in: query

description: End time of the invocation time range.

schema:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

- name: apiId

in: query

description: String identifying the API invoked.

schema:

type: string

- name: apiName

in: query

description: Name of the API which was invoked.

schema:

type: string

- name: version

in: query

description: Version of the API which was invoked.

schema:

type: number

- name: operation

in: query

description: Operation that was invoked on the API.

schema:

type: string

- name: result

in: query

description: Result or output of the invocation.

schema:

type: string

- name: resourceName

in: query

description: Name of the specific resource invoked.

schema:

type: string

- name: interfaceDescription

in: query

description: Interface description of the API invoked.

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/InterfaceDescription'

responses:

'200':

description: Result of the query operation along with fetched service API invocation log data.

content:

application/json:

schema:

$ref: '#/components/schemas/InvocationLogs'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'409':

$ref: 'TS29122_CommonData.yaml#/components/responses/409'

'412':

$ref: 'TS29122_CommonData.yaml#/components/responses/412'

'414':

description: URI Too Long

content:

application/problem+json:

schema:

$ref: 'TS29122_CommonData.yaml#/components/schemas/ProblemDetails'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

schemas:

InvocationLogs:

type: array

items:

$ref: 'TS29222_CAPIF_Logging_API_Invocation.yaml#/components/schemas/InvocationLogs'

## A.10	AEF_Authentication_API

openapi: 3.0.0

info:

title: AEF_Authentication_API

version: "1.PreR15.0.0"

servers:

- url: '{apiRoot}/auth-initiation/v1'

variables:

apiRoot:

default: https://demohost.com

description: apiRoot as defined in subclause 7.5 of 3GPP TS 29.222.

paths:

/auth-profiles:

get:

parameters:

- name: apiInvokerId

in: query

description: API invoker ID assigned by the CAPIF core function to the API invoker while on-boarding the API invoker.

schema:

type: string

responses:

'204':

description: API exposing function confirms that the authentication profile is available and it is ready for authentication of the API invoker.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'412':

$ref: 'TS29122_CommonData.yaml#/components/responses/412'

'414':

description: URI Too Long

content:

application/problem+json:

schema:

$ref: 'TS29122_CommonData.yaml#/components/schemas/ProblemDetails'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

Annex B (informative): Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2018-03 | CT3#95 | C3-181278 |  |  |  | TS skeleton of Common API Framework for 3GPP Northbound APIs | 0.0.0 |
| 2018-03 | CT3#95 | C3-181378 |  |  |  | Inclusion of documents agreed in CT3#95: C3-181281, C3-181282, C3-181283, C3-181284, C3-181285, C3-181286, C3-181287, C3-181321, C3-181322, Rapporteur changes | 0.1.0 |
| 2018-04 | CT3#96 | C3-182527 |  |  |  | Inclusion of documents agreed in CT3#96: C3-182204, C3-182387, C3-182393, C3-182395, C3-182468, C3-182469, C3-182470, C3-182483, C3-182484, C3-182485 | 0.2.0 |
| 2018-05 | CT3#97 |  |  |  |  | Inclusion of documents agreed in CT3#97: C3-183271, C3-183274, C3-183275, C3-183372, C3-183376, C3-183377, C3-183378, C3-183379, C3-183598, C3-183599, C3-183602, C3-183603, C3-183604, C3-183798, C3-183799, C3-183809, C3-183841, C3-183842 | 0.3.0 |
| 2018-06 | CT#80 | CP-181037 |  |  |  | TS sent to plenary for approval | 1.0.0 |
| 2018-06 | CT#80 | CP-181037 |  |  |  | TS approved by plenary | 15.0.0 |
| 2018-09 | CT#81 | CP-182016 | 0001 | 1 | F | Changes to clause 4 – Overview | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0003 | 2 | F | Changes to CAPIF Publish Service API subclause | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0004 | 2 | F | Changes to CAPIF Events API subclause | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0005 | 4 | F | Changes to CAPIF API Invoker Management API subclause | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0006 | 4 | F | Changes to CAPIF Authentication Authorization API subclause | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0007 | 3 | F | Update to data types for ServiceAPIDescription and APIQuery | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0008 | 5 | F | Definition of CAPIF_Access_Control_Policy_API, and OpenAPI schema | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0009 | 4 | F | CAPIF_Events_API OpenAPI schema | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0010 | 4 | F | AEF_Authentication_API OpenAPI schema | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0011 | 1 | F | CAPIF_Discover_Service API - Corrections | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0012 | 3 | F | CAPIF_discovery_service API OpenAPI file | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0013 | 4 | F | CAPIF_Publish_Service API - Corrections and OpenAPI file | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0014 | 4 | F | AEF_Authentication API - Editor's notes | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0015 | 4 | F | Corrections to data type | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0016 | 1 | F | API Invoker's Information in APIInvokerEnrolmentDetails | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0017 | 1 | F | Corrections to OnboardingInformation data type | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0018 | 2 | F | Security method preference | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0019 | 1 | F | Clarifications to Obtain_API_Invoker_Info service operation | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0020 | 1 | F | Subscribed and Subscribing functional entity | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0021 | 1 | F | Miscellaneous corrections | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0023 | 1 | F | Definitions and abbreviations | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0024 | 1 | F | Referenced data types and enumerations | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0025 | 2 | F | CAPIF_Security_API OpenAPI schema | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0026 | 1 | F | CAPIF discovery service API – API invoker retrieves API information using GET | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0028 | 2 | F | CAPIF_Auditing_API – API management function retrieves API information logs using GET – OpenAPI document | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0029 | 3 | F | API Names changes in clause 5 | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0030 | - | F | Change security-related API names in clause 8 and 10 | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0031 | 2 | F | Describe response code 202 for Onboard_API_Invoker POST method | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0032 | - | F | Correct cardinality for onboardingNotificationDestination | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0033 | - | F | Correct cardinality for securityNotificationDestination | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0034 | 1 | F | Correct protocol type in Interface Description | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0036 | 1 | F | Query parameter in retrieving access control | 15.1.0 |
| 2018-09 | CT#81 | CP-182037 | 0037 | 1 | F | Authorization endpoint and token request | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0038 | 1 | F | CAPIF Events | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0040 | 1 | F | Corrections to resource figures | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0041 | 1 | F | CAPIF_Auditing_API - 'query' custom operation | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0042 | 2 | F | OpenAPI - CAPIF_API_Invoker_Management API | 15.1.0 |
| 2018-09 | CT#81 | CP-182016 | 0043 | 2 | F | OpenAPI - CAPIF_Logging_API_Invocation API | 15.1.0 |