3GPP TS 29.214 V10.15.0 (2018-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

Policy and Charging Control over Rx reference point

(Release 10)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

UMTS, LTE, QoS, Charging, Policy

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2018, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI currently being registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions and abbreviations	7

3.1	Definitions	7

3.2	Abbreviations	8

4	Rx reference point	8

4.1	Overview	8

4.2	Rx reference model	9

4.3	Functional elements	10

4.3.1	AF	10

4.3.2	PCRF	11

4.4	PCC procedures over Rx reference point	11

4.4.1	Initial Provisioning of Session Information	11

4.4.2	Modification of Session Information	14

4.4.3	Gate Related Procedures	15

4.4.4	AF Session Termination	15

4.4.5	Subscription to Notification of Signalling Path Status	15

4.4.5a	Provisioning of AF Signalling Flow Information	16

4.4.6	Traffic Plane Events	16

4.4.6.1	IP-CAN Session Termination	16

4.4.6.2	Service Data Flow Deactivation	17

4.4.6.3	Notification of Signalling Path Status	17

4.4.6.4	IP-CAN type change Notification	17

4.4.6.5	Access Network Charging Information Notification	18

4.4.6.6	Reporting Usage for Sponsored Data Connectivity	18

5	Rx protocol	18

5.1	Protocol support	18

5.2	Initialization, maintenance and termination of connection and session	19

5.3	Rx specific AVPs	19

5.3.1	Abort-Cause AVP	20

5.3.2	Access-Network-Charging-Address AVP	21

5.3.3	Access-Network-Charging-Identifier AVP	21

5.3.4	Access-Network-Charging-Identifier-Value AVP	21

5.3.5	AF-Application-Identifier AVP	21

5.3.6	AF-Charging-Identifier AVP	21

5.3.7	Codec-Data AVP	21

5.3.8	Flow-Description AVP	22

5.3.9	Flow-Number AVP	23

5.3.10	Flows AVP	23

5.3.11	Flow-Status AVP	23

5.3.12	Flow-Usage AVP	23

5.3.13	Specific-Action AVP	24

5.3.14	Max-Requested-Bandwidth-DL AVP	26

5.3.15	Max-Requested-Bandwidth-UL AVP	26

5.3.16	Media-Component-Description AVP	26

5.3.17	Media-Component-Number AVP	27

5.3.18	Media-Sub-Component AVP	27

5.3.19	Media-Type AVP	28

5.3.20	RR-Bandwidth AVP	28

5.3.21	RS-Bandwidth AVP	28

5.3.22	SIP-Forking-Indication AVP	28

5.3.23	Service-URN AVP	28

5.3.24	Acceptable-Service-Info AVP	28

5.3.25	Service-Info-Status-AVP	29

5.3.26	AF-Signalling-Protocol-AVP	29

5.3.27	Sponsored-Connectivity-Data AVP	29

5.3.28	Sponsor-Identity AVP	30

5.3.29	Application-Service-Provider-Identity AVP	30

5.3.30	MPS-Identifier AVP	30

5.3.31	Min-Requested-Bandwidth-DL AVP	30

5.3.32	Min-Requested-Bandwidth-UL AVP	30

5.4	Rx re-used AVPs	30

5.4.1	 Use of the Supported-Features AVP on the Rx reference point	31

5.5	Rx specific Experimental-Result-Code AVP values	33

5.6	Rx messages	34

5.6.1	AA-Request (AAR) command	34

5.6.2	AA-Answer (AAA) command	34

5.6.3	Re-Auth-Request (RAR) command	35

5.6.4	Re-Auth-Answer (RAA) command	35

5.6.5	Session-Termination-Request (STR) command	36

5.6.6	Session-Termination-Answer (STA) command	36

5.6.7	Abort-Session-Request (ASR) command	36

5.6.8	Abort-Session-Answer (ASA) command	36

Annex A (normative):	IMS Related P-CSCF Procedures over Rx	38

A.1	Provision of Service Information at P-CSCF	38

A.2	Enabling of IP Flows	39

A.3	Support for SIP forking	40

A.3.1	PCC rule provisioning for early media for forked responses	40

A.3.2	Updating the provisioned PCC rules at the final answer	40

A.4	Notification of AF Signalling Transmission Path Status	41

A.5	Indication of Emergency Session	41

A.6	Notification IP-CAN Type Change	41

A.7	Support for Early Session disposition SDP	41

A.7.1	General	41

A.7.2	Service Information Provisioning for Early Media	42

A.7.3	Updating the Provisioned Service Information when Dialogue is established	43

A.8	Provision of Signalling Flow Information at P-CSCF	43

A.9	Handling of MPS Session	43

Annex B (normative):	Flow identifiers: Format definition and examples	45

B.1	Format of a flow identifier	45

B.1.1 	General	45

B.1.2 	Derivation of Flow Identifiers from SDP	46

B.1.2.1 	 Standard Procedure	46

B.1.2.2 	 SDP with "early session" disposition type	46

B.2	Example 1	46

B.3	Example 2	47

B.4	Example 3 without media components.	48

B.5	Example 4	49

Annex C (informative):	Void	51

Annex D (informative):	Change history	52

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document provides the stage 3 specification of the Rx reference point for the present release. The functional requirements and the stage 2 specifications of the Rx reference point are contained in 3GPP TS 23.203 [7]. The Rx reference point lies between the Application Function and the Policy and Charging Rule Function.

Whenever it is possible the present document specifies the requirements for the protocol by reference to specifications produced by the IETF within the scope of Diameter. Where this is not possible, extensions to Diameter are defined within the present document.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication and/or edition number or version number) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.203: "Policy and Charging Control architecture".

[3]	void

[4]	void

[5]	3GPP TS 29.209: "Policy control over Gq interface", latest Rel-6 version.

[6]	void

[7]	3GPP TS 29.211: "Rx Interface and Rx/Gx signalling flows", latest Rel-6 version.

[8]	3GPP TS 29.212: "Policy and Charging Control over Gx reference point".

[9]	3GPP TS 29.213: "Policy and Charging Control signalling flows and QoS parameter mapping".

[10]	IETF RFC 3588: "Diameter Base Protocol".

[11]	IETF RFC 3556: "Session Description Protocol (SDP) Bandwidth Modifiers for RTP Control Protocol (RTCP) Bandwidth".

[12]	IETF RFC 4005: "Diameter Network Access Server Application".

[13]	IETF RFC 4566: "SDP: Session Description Protocol".

[14]	IETF RFC 4006: "Diameter Credit Control Application".

[15]	ETSI TS 183 017 v3.2.1: "Telecommunications and Internet Converged Services and Protocols for Advanced Networking (TISPAN); Resource and Admission Control: DIAMETER protocol for session based policy set-up information exchange between the Application Function (AF) and the Service Policy Decision Function (SPDF); Protocol specification".

[16]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[17]	3GPP TS 24.229: "IP Multimedia Call Control Protocol based on SIP and SDP; Stage 3".

[18]	IETF RFC 3264: "An Offer/Answer Model with the Session Description Protocol (SDP)".

[19]	IETF RFC 4566: "SDP: Session Description Protocol".

[20]	IETF RFC 3162: "Radius and IPv6".

[21]	IETF RFC 5031: "A Uniform Resource Name (URN) for Emergency and Other Well-Known Services".

[22]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[23]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[24]	3GPP TS 32.299: "Telecommunication management; Charging management; Diameter charging applications".

[25]	3GPP TS 29.229: "Cx and Dx interfaces based on the Diameter protocol; Protocol details"

[26]	3GPP TS 24.292: "IP Multimedia (IM) Core Network (CN) subsystem Centralized Services (ICS); Stage 3".

[27]	IETF RFC 3959 (December 2004): "The Early Session Disposition Type for the Session Initiation Protocol (SIP)".

[28]		3GPP TS 23.380: "IMS Restoration Procedures".

[29]	3GPP TS 23.335: "User Data Convergence (UDC); Technical realization and information flows; Stage 2".

[30]	3GPP TS 29.335: "User Data Convergence (UDC); User Data Repository Access Protocol over the Ud interface; Stage 3".

[31]	3GPP TS 22.153: "Multimedia Priority Service".

[32]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS) Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH) ".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply:

Application Function (AF): element offering application(s) that use IP bearer resources

NOTE:	One example of an AF is the P-CSCF of the IM CN subsystem.

AF Session: application level session established by an application level signalling protocol offered by the AF that requires a session set-up with explicit session description before the use of the service.

NOTE:	One example of an application session is an IMS session.

Attribute-Value Pair (AVP): See RFC 3588 [5], corresponds to an Information Element in a Diameter message.

binding: PCRF process of associating IP flows described in AF Service Information with IP-CAN bearers.

IP-CAN bearer: IP transmission path of defined capacity, delay and bit error rate, etc.
See 3GPP TS 21.905 [1] for the definition of bearer.

IP-CAN session: association between a UE and an IP network (for GPRS, APN).
The association is identified by one UE Ipv4 address and/or one IPv6 prefix together with a UE identity information, if available, and a PDN represented by a PDN ID (e.g. an APN). An IP-CAN session incorporates one or more IP-CAN bearers. Support for multiple IP-CAN bearers per IP-CAN session is IP-CAN specific. An IP-CAN session exists as long as the related UE IPv4 address and/or IPv6 prefix are assigned and announced to the IP network.

IP flow: unidirectional flow of IP packets with the same source IP address and port number and the same destination IP address and port number and the same transport protocol
Port numbers are only applicable if used by the transport protocol.

MPS session: A session for which priority treatment is applied for allocating and maintaining radio and network resources to support the Multimedia Priority Service (MPS).  MPS is defined in 3GPP TS 22.153 [31].

packet flow: A specific user data flow carried through the PCEF. A packet flow can be an IP flow.

PCC rule: set of information enabling the detection of a service data flow and providing parameters for policy control and/or charging control

service information: set of information conveyed from the AF to the PCRF over the Rx interface to be used as a basis for PCC decisions at the PCRF, including information about the AF session (e.g. application identifier, type of media, bandwidth, IP address and port number)

service data flow: An aggregate set of packet flows.

## 3.2	Abbreviations

For the purpose of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply:

AF	Application Function

ASP	Application Service Provider

AVP	Attribute Value Pair

CRF	Charging Rules Function

IP-CAN	IP Connectivity Access Network

MPS	Multimedia Priority Service

PCC	Policy and Charging Control

PCEF	Policy and Charging Enforcement Function

PCRF	Policy and Charging Rule Function

PDF	Policy Decision Function

P-CSCF	Proxy-Call Session Control Function

QoS	Quality of Service

SDF	Service Data Flow

SPR	Subscriber Profile Repository

UDC	User Data Convergence

UE	User Equipment

UDR	User Data Repository

# 4	Rx reference point

## 4.1	Overview

The Rx reference point is used to exchange application level session information between the Policy and Charging Rules Function (PCRF) and the Application Function (AF). As defined in the stage 2 specifications (3GPP TS 23.203 [2]), this information is part of the input used by the PCRF for the Policy and Charging Control (PCC) decisions. The PCRF exchanges the PCC rules with the Policy and Charging Enforcement Function (PCEF) and QoS rules with the Bearer Binding and Event Reporting Function (BBERF) as specified in 3GPP TS 29.212 [8].

Signalling flows related to the both Rx and Gx interfaces are specified in 3GPP TS 29.213 [9].

## 4.2	Rx reference model

The Rx reference point is defined between the PCRF and the AF. The relationships between the different functional entities involved are depicted in figure 4.1. and 4.2.

Figure 4.1: Rx reference point at the Policy and Charging Control (PCC) architecture with SPR

With the UDC-based architecture, as defined in 3GPP TS 23.335 [29] and applied in 3GPP TS 23.203 [7], the UDR replaces SPR and the Ud reference point provides access to the subscription data in the UDR. The Ud interface as defined in 3GPP TS 29.335 [30] is the interface between the PCRF and the UDR.The relationships between the different functional elements are depicted in figure 4.2. When UDC architecture is used, SPR and Sp, whenever mentioned in this document, are replaced by UDR and Ud.

Figure 4.2: Rx reference point at the Policy and Charging Control (PCC) architecture with UDR

NOTE 1:	The details associated with the Sp reference point are not specified in this Release. The SPR's relation to existing subscriber databases is not specified in this Release.

NOTE 2:	The UDC Application Informational Model related to the PCRF is not specified in this Release.

NOTE 3:	PCEF is located in the Gateway node implementing the IP access to the PDN. Refer to Annexes of 3GPP TS 23.203 [2] for application to specific IP-CAN types.

NOTE 4:	Refer to Annexes A.5 and H.2 of 3GPP TS 23.203 [2] for application of AN-Gateways.

## 4.3	Functional elements

### 4.3.1	AF

The AF is an element offering applications that require the Policy and Charging Control of traffic plane resources (e.g. UMTS PS domain/GPRS domain resources). One example of an application function is the P-CSCF. The AF shall use the Rx reference point to provide session information to the PCRF.

NOTE: 	The AFs may be deployed by the same operator offering the IP-CAN or may be provided by external third party service provider.

### 4.3.2	PCRF

The PCRF (Policy Control and Charging Rules Function) is a functional element that encompasses policy control decision and flow based charging control functionalities. These 2 functionalities are the heritage of the release 6 logical entities PDF and CRF respectively. The PCRF provides network control regarding the service data flow detection, gating, QoS and flow based charging (except credit management) towards the PCEF. The PCRF receives session and media related information from the AF and informs AF of traffic plane events.

The PCRF may check that the service information provided by the AF is consistent with the operator defined policy rules before storing the service information. The service information shall be used to derive the QoS for the service. The PCRF may reject the request received from the AF and as a result the PCRF shall indicate, in the response to the AF, the service information that can be accepted by the PCRF.

The PCRF may use the subscription information as basis for the policy and charging control decisions. The subscription information may apply for both session based and non-session based services. The subscription specific information for each service may contain e.g. max QoS class and max bit rate.

If the AF requests it, the PCRF shall report IP-CAN session events (including bearer events and events on AF signalling transport) to the AF via the Rx reference point.

The PCRF PCC/QoS Rule decisions may be based on one or more of the following:

-	the session and media related information obtained from the AF via the Rx reference point;

-	the bearer and subscriber related information obtained from the PCEF over the Gx reference point;

-	the bearer and subscriber related information obtained from the BBERF over the Gxx reference point;

-	subscriber and service related data the PCRF may be aware of by configuration or through the Sp reference point;

-	pre-configured information in the PCRF.

NOTE:	The details associated with the Sp reference point are not specified in this Release. The SPR's relation to existing subscriber databases is not specified in this Release.

The PCRF shall provision PCC/QoS Rules to the PCEF/BBERF via the Gx/Gxx reference point.

## 4.4	PCC procedures over Rx reference point

### 4.4.1	Initial Provisioning of Session Information

When a new AF session is being established and media information for this AF session is available at the AF and the related media require PCC supervision, the AF shall open an Rx Diameter session with the PCRF for the AF session using an AA-Request command. The AF shall provide the full IP address of the UE using either Framed-IP-Address AVP or Framed-IPv6-Prefix AVP, and the corresponding Service Information within Media-Component-Description AVP(s). The AF shall not include circuit-switched bearer related media in the service information sent to the PCRF. The AF shall indicate to the PCRF as part of the Media-Component-Description whether the media IP flow(s) should be enabled or disabled with the Flow-Status AVP.

NOTE:	The AF does not need to open an Rx Diameter session with the PCRF, if the SDP payload is only proposing to use a circuit-switched bearer (i.e. "c=" line set to "PSTN" and an "m=" line set to "PSTN", refer to 3GPP TS 24.292 [26]).

NOTE:	The Rx Diameter session used for an AF session is different from the Rx Diameter session possibly used for the notifications of the status of the AF signalling transmission path. A new Rx Diameter session is established for each new AF session.

The AF may include the AF-Application-Identifier AVP into the AA-Request in order to indicate the particular service that the AF session belongs to. This AVP can be provided at both AF session level, and Media-Component-Description level. When provided at both levels, the AF-Application Identifier provided within the Media-Component-Description AVP will have precedence.

The AF may include the AF-Charging-Identifier AVP into the AA-Request for charging correlation purposes. The AF may also include the Specific-Action AVP to request notification for certain user plane events, e.g. bearer termination.

The AF may include the Service-URN AVP in order to indicate that the new AF session relates to emergency traffic. If the PCRF receives the Service-URN AVP indicating an emergency session, the PCRF may apply special policies, for instance prioritising service flows relating to the new AF session or allowing these service flows free of charge.

The AF may include the MPS-Identifier AVP in order to indicate that the new AF session relates to an MPS session. If the PCRF receives the MPS-Identifier AVP indicating an MPS session, the PCRF may take specific actions on the corresponding IP-CAN to ensure that the MPS session is prioritized as specified in 3GPP TS 29.212 [8]. For Multimedia Priority Sevice handling, see Annex A.9.

If the AF provides service information that has been fully negotiated (e.g. based on the SDP answer), the AF may include the Service-Info-Status AVP set to FINAL_SERVICE_INFORMATION. In this case the PCRF shall authorize the session and provision the corresponding PCC/QoS rules to the PCEF/BBERF.

The AF may additionally provide preliminary service information not fully negotiated yet (e.g. based on the SDP offer) at an earlier stage. To do so, the AF shall include the Service-Info-Status AVP with the value set to PRELIMINARY SERVICE INFORMATION. Upon receipt of such preliminary service information, the PCRF shall perform an early authorization check of the service information. For GPRS, the PCRF shall not provision PCC rules towards the PCEF unsolicitedly. However, the PCRF may authorize a PCC/QoS rule request received from the PCEF/BBERF as per 3GPP TS 29.212 [8].

For sponsored data connectivity, the AF shall provide the application service provider identity and the sponsor identity to the PCRF byincluding the Application-Service-Provider-Identity AVP and the Sponsor-Identity AVP in the Sponsored-Connectivity-Data AVP in the AA-Request.

NOTE:	The relationship between the AF and Sponsor is out of scope of this specification. A single AF can serve multiple ASPs and multiple sponsors, An ASP can also be a sponsor.

To support the usage monitoring of sponsored data connectivity, the AF may also include the Granted-Service-Unit AVP in the Sponsored-Connectivity-Data AVP and the Specific-Action AVP set to the value USAGE_REPORT in the AA-Request to request notification when the usage threshold has been reached.

NOTE:	If the AF is in the user plane, the AF can handle the usage monitoring and therefore it is not required to provide a usage threshold to the PCRF as part of the sponsored data connectivity information.

If the UE is roaming with the visited access case and the AF is located in the HPLMN or roaming with the home routed case and operator policies do not allow accessing the sponsored data connectivity with this roaming case, the H-PCRF shall reject the service request indicating UNAUTHORIZED_SPONSORED_DATA_CONNECTIVITY to the AF.

If the UE is roaming with the visited access case and the AF is located in the VPLMN, the V-PCRF shall reject the service request indicating UNAUTHORIZED_SPONSORED_DATA_CONNECTIVITY to the AF.

If the UE is in the non-roaming case or roaming with the home routed case and the operator policies allow accessing the sponsored data connectivity with this roaming case, the following procedures apply:

-	If the PCEF does not support sponsored connectivity and the required reporting level for that service indicates a sponsored connectivity level according to 3GPP TS 29.212 [8], clause 4.5.20, then the PCRF shall reject the request indicating REQUESTED_SERVICE_NOT_AUTHORIZED.

-	If the PCEF supports sponsored data connectivity feature or the required reporting level is different from sponsored connectivity level as described in 3GPP TS 29.212[8], then the PCRF, based on operator policies, shall check whether it is required to validate the sponsored connectivity data. If it is required, it shall perform the authorizations based on sponsored data connectivity profiles. If the authorization fails, the PCRF responds to the AF with an AA-Answer including the Experimental-Result-Code AVP set to the value UNAUTHORIZED_SPONSORED_DATA_CONNECTIVITY. The profile may include a list of Application Service Providers and their applications per sponsor.

NOTE:	If the AF is in the operator's network and is based on the OSA/Parlay-X GW, the PCRF is not required to verify that a trust relationship exists between the operator and the sponsors.

When the PCRF receives an initial AA-Request from the AF, the PCRF shall perform session binding as described in 3GPP TS 29.213 [9]. To allow the PCRF to identify the IP-CAN session for which this request applies, the AF shall provide either the Framed-IP-Address or the Framed-IPv6-Prefix containing the full IP address applicable to an IP flow or IP flows towards the UE. In case of private IP address being used, the AF may also provide PDN information if available in the Called-Station-Id AVP for session binding.

If the PCRF fails in executing session binding, the PCRF responds to the AF with an AA-Answer including the Experimental-Result-Code AVP set to the value IP-CAN_SESSION_NOT_AVAILABLE. Further details on how the PCRF identifies suitable IP-CAN sessions can be found in the binding mechanism described in 3GPP TS 29.213 [9].

If the request contains Media-Component-Description Attribute-Value Pair(s) (AVP(s)) the PCRF shall store the received Service Information. The PCRF shall process the received Service Information according to the operator policy and may decide whether the request is accepted or not. The PCRF may take the priority information within the Reservation-Priority AVP into account when making this decision. If the service information provided in the AA-Request command is rejected (e.g. the subscribed guaranteed bandwidth for a particular user is exceeded), the PCRF shall indicate in the AA-Answer the cause for the rejection with the Experimental-Result-Code AVP set to the value REQUESTED_SERVICE_NOT_AUTHORIZED. The PCRF may additionally provide the acceptable bandwidth within the Acceptable-Service-Info AVP.

To allow the PCRF and PCEF to perform PCC rule authorization and bearer binding for the described service IP flows, the AF shall supply both source and destination IP addresses and port numbers within the Flow-Description AVP, if such information is available.

NOTE:	In SDP source port information is usually not available.

The AF may specify the Reservation-Priority AVP at request level in the AA-Request in order to assign a priority to the AF Session as well as specify the Reservation-Priority AVP at the media-component-description AVP level to assign a priority to the IP flow. The presence of the Reservation-Priority in both levels does not constitute a conflict as they each represent different types of priority. Specifically the Reservation-Priority at the AA-Request level provides the relative priority for a session while the Reservation-Priority at the media-component-description level provides the relative priority for an IP flow within a session. If the Reservation-Priority AVP is not specified the requested priority is DEFAULT (0).

The AF may request notifications of specific IP-CAN session events through the usage of the Specific-Action AVP in the AA-Request command. The PCRF shall make sure to inform the AF of the requested notifications in the event that they take place.

The PCRF shall check whether the received Service Information requires PCC/QoS Rules to be created and provisioned and/or authorized QoS to be provisioned. Provisioning of PCC/QoS Rules and Authorized QoS to the PCEF/BBERF shall be carried out as specified at 3GPP TS 29.212 [8].

The PCRF shall reply with an AA-Answer to the AF. The acknowledgement towards the AF should take place before or in parallel with any required PCC Rule provisioning towards the PCEF and shall include the Access-Network-Charging-Identifier(s) and may include the Access-Network-Charging-Address AVP, if they are available. The AA-Answer message shall also include the IP-CAN-Type AVP, if such information is available. In that case, the AA-Answer message shall also include the RAT-Type AVP when applicable for the specific IP-CAN Type (e.g. 3GPP IP-CAN Type). In addition, if IP flow mobility applies to service data flows as specified in 3GPP TS 29.212 [8], such that a subset of the flows within the AF session are affected, the PCRF shall also include IP-CAN-type and RAT-type information (if applicable) to IP flow mobility related flows, if such information is available. The IP flow mobility affected service data flows are included within the Flows AVP at command level. If the PCRF needs to terminate the Rx session before it has sent the AA Answer, the PCRF shall send the AA Answer immediately and before the AS Request.

The behaviour when the AF does not receive the AA Answer, or when it arrives after the internal timer waiting for it has expired, or when it arrives with an indication different than DIAMETER_SUCCESS, are outside the scope of this specification and based on operator policy.

If the PCRF fails in installing PCC/QoS rules based on the provided service information due to resource allocation failure as specified in 3GPP TS 29.212 [8] and if requested by the AF, the PCRF shall send an RAR command to the AF with the Specific-Action AVP set to the value INDICATION_OF_FAILED_RESOURCES_ALLOCATION to report the resource allocation failure. The AF shall send an RAA command to acknowledge the RAR command.

### 4.4.2	Modification of Session Information

The AF may modify the session information at any time (e.g. due to an AF session modification or internal AF trigger) by sending an AA-Request command to the PCRF containing the Media-Component-Description AVP(s) with the updated Service Information. The AF shall send an AA-Request command to the PCRF, only after the previous AA-Request has been acknowledged.

If the AF provides service information that has been fully negotiated (e.g. based on the SDP answer), the AF may include the Service-Info-Status AVP set to FINAL_SERVICE_INFORMATION. In this case the PCRF shall authorize the session and provision the corresponding PCC rules to the PCEF.

The AF may additionally provide preliminary service information not fully negotiated yet (e.g. based on the SDP offer) at an earlier stage. To do so, the AF shall include the Service-Info-Status AVP with the value set to PRELIMINARY SERVICE INFORMATION. Upon receipt of such preliminary service information, the PCRF shall perform an early authorization check of the service information. For GPRS, the PCRF shall not provision PCC rules towards the PCEF unsolicitedly. However, the PCRF may authorize a PCC/QoS rule request received from the PCEF/BBERF as per 3GPP TS 29.212 [8].

The AF may include the MPS-Identifier AVP in order to indicate that the modified AF session relates to an MPS session. If the PCRF receives the MPS-Identifier AVP, it may take specific actions on the corresponding IP-CAN to ensure that the MPS session is prioritized as defined in 3GPP TS 29.212 [8]. For Multimedia Priority Sevice handling, see Annex A.9.

For sponsored data connectivity, the AF shall provide the application service provider identity and the sponsor identity to the PCRF by including Application-Service-Provider-Identity AVP and the Sponsor-Identity AVP in the Sponsored-Connectivity-Data AVP in the AA-Request.

To support the usage monitoring of sponsored data connectivity, the AF may also include the Granted-Service-Unit AVP in the Sponsored-Connectivity-Data AVP in the AA-Request.

NOTE:     If the AF is in the user plane, the AF can handle the usage monitoring and therefore it is not required to provide a usage threshold to the PCRF as part of the sponsored data connectivity information.

The PCRF shall process the received Service Information according the operator policy and may decide whether the request is accepted or not. If the updated Service Information is not acceptable (e.g. subscribed guaranteed bandwidth for a particular user is exceeded), the PCRF shall indicate in the AA-Answer the cause for the rejection with the Experimental-Result-Code AVP set to the value REQUESTED_SERVICE_NOT_AUTHORIZED. The PCRF may additionally provide the acceptable bandwidth within the Acceptable-Service-Info AVP.

If accepted, the PCRF shall update the Service Information with the new information received. Due to the updated Service Information, the PCRF may need to create, modify or delete the related PCC rules and provide the updated information towards the PCEF following the corresponding procedures specified at 3GPP TS 29.212 [8]. The procedures to update the Authorized QoS for the affected IP-CAN bearer are also specified at 3GPP TS 29.212 [8].

The PCRF shall reply with an AA-Answer to the AF. The acknowledgement towards the AF should take place before or in parallel with any required PCC Rule provisioning towards the PCEF and shall include the Access-Network-Charging-Identifier(s) and may include the Access-Network-Charging-Address AVP, if they are available at this moment and have not been yet supplied earlier to the AF. The AA-Answer message shall include the IP-CAN-Type AVP if such information is available and has not yet been supplied earlier to the AF. In that case, the AA-Answer message shall also include the RAT-Type AVP when applicable for the specific IP-CAN Type (e.g. 3GPP IP-CAN Type). In addition, if IP flow mobility applies to service data flows as specified in 3GPP TS 29.212 [8], such that a subset of the flows within the AF session are affected, the PCRF shall also include IP-CAN-type and RAT-type information (if applicable) to IP flow mobility related flows, if such information is available and has not yet been supplied earlier to the AF. The IP flow mobility affected service data flows are included within the Flows AVP at command level. If the PCRF needs to terminate the Rx session before it has sent the AA Answer, the PCRF shall send the AA Answer immediately and before the AS Request.

If the PCRF modifies existing PCC/QoS rules based on the updated service information and the modification fails due to resource allocation failure as specified in 3GPP TS 29.212 [8] and if requested by the AF, the PCRF shall send an RAR command to the AF with the Specific-Action AVP set to the value INDICATION_OF_FAILED_RESOURCES_ALLOCATION to report the modification failure. The AF shall send an RAA command to acknowledge the RAR command.

### 4.4.3	Gate Related Procedures

Depending on the application, in the Service Information provision, the AF may instruct the PCRF when the IP flow(s) are to be enabled or disabled to pass through the IP-CAN. The AF does this by sending the AA-Request message containing the Media-Component- Description AVP(s) that contains the flow status information (in the Flow-Status AVP) for the flows to be enabled or disabled.

In response to this action the PCRF shall set the appropriate gate status for the corresponding active PCC rule(s).

If a Media-Sub-Component AVP under a Media-Component-Description AVP contains a Flow-Usage AVP with the value RTCP, then the corresponding RTCP IP Flows in both directions shall be enabled even if the Flow-Status AVP under the Media-Sub-Component AVP is set to ENABLED-UPLINK, ENABLED-DOWNLINK, ENABLED, or DISABLED.

The PCRF shall reply with an AA-Answer and shall include the Access-Network-Charging-Identifier(s) available at this moment. The PCRF forwards the AF decision to enable or disable the authorized IP flows.

The behaviour when the AF does not receive the AAA, or when it arrives after the internal timer waiting for it has expired, or when it arrives with an indication different than DIAMETER_SUCCESS, are outside the scope of this specification and based on operator policy.

### 4.4.4	AF Session Termination

When an AF session is terminated, if the AF had received a successful AA-Answer for the initial AA-Request, the AF shall send a Session-Termination-Request command to the PCRF. Otherwise, the AF shall wait for the initial AA-Answer to be received prior to sending the Session-Termination-Request command to the PCRF.

When the PCRF receives a ST-Request from the AF, indicating an AF session termination, it shall acknowledge that request by sending a ST-Answer to the AF. Afterwards, it shall free the resources allocated for the corresponding Service Data Flow(s). In order to do that, the PCRF shall initiate the request for the removal of any related PCC/QoS rules from the PCEF/BBERF and for the update of the Authorized QoS for the affected IP-CAN bearer following the corresponding procedures specified at 3GPP TS 29.212 [8].

If the AF session being terminated corresponds to an MPS session, the PCRF may revoke the actions related to the prioritization of the MPS session in the corresponding IP-CAN as defined in 3GPP TS 29.212 [8]. For Multimedia Priority Sevice handling, see Annex A.9.

For sponsored data connectivity, and if a volume threshold was provided for the sponsored data connection at initial provisioning of session information (clause 4.4.1) or modification of session information (clause 4.4.2) procedures, the PCRF shall provide the volume consumed to the AF. For such purpose, the PCRF shall initiate the IP-CAN session modification procedure according 3GPP TS 29.212 [8] in order to obtain the consumed volume. The PCRF shall send then the ST-Answer to the AF including the Used-Service-Unit AVP for reporting accumulated usage within the Sponsored-Connectivity-Data AVP.

### 4.4.5	Subscription to Notification of Signalling Path Status

An AF may subscribe to notifications of the status of the AF Signalling transmission path. To do so, the AF shall open an Rx Diameter session with the PCRF for the AF signalling using an AA-Request command. The AF shall provide the UE's IP address (using either the Framed-IP-Address AVP or the Framed-IPv6-Prefix AVP) and the Specific-Action AVP requesting the subscription to "INDICATION_OF_LOSS_OF BEARER" and/or "INDICATION_OF_RELEASE_OF_BEARER". The AF shall additionally provide a Media-Component-Description AVP including a single Media-Sub-Component AVP with the Flow-Usage AVP set to the value "AF_SIGNALLING". The Media-Component-Description AVP shall contain the Media-Component-Number AVP set to "0".

If the procedures in Clause 4.4.5a are not applied, the Media-Sub-Component AVP shall contain the Flow-Number AVP set to "0", and the rest of AVPs within the Media-Component-Description and Media-Sub-Component AVPs shall not be used in this case.

When the PCRF receives an AA-Request as described in the preceding paragraph from the AF, the PCRF shall perform session binding as described in 3GPP TS 29.213 [9] and acknowledges the AAR command by sending an AA-Answer command to the AF.

PCC/QoS Rules related to AF Signalling IP Flows should be provisioned to PCEF/BBERF using the corresponding procedures specified at 3GPP TS 29.212 [8] at an earlier stage (e.g. typically at the establishment of the IP-CAN bearer dedicated for AF Signalling IP Flows). The PCRF may install the corresponding dynamic PCC/QoS rule for the AF signalling IP flows if none has been installed before.

If the Rx Diameter Session is only used for subscription to Notification of Signalling Path Status, the AF may cancel the subscription to notifications of the status of the AF Signalling transmission path. In that case, the AF shall use a Session-Termination-Request (STR) command to the PCRF, which shall be acknowledged with a Session-Termination-Answer (STA) command.

NOTE:	The Rx Diameter Session created for the AF signalling can also be used when the AF requests notifications of IP-CAN type change and/or when the AF provisions AF Signalling Flow Information.

### 4.4.5a	Provisioning of AF Signalling Flow Information

An AF may provision information about the AF signalling IP flows between the UE and the AF. To do so, the AF shall make use of an Rx Diameter session already opened with the PCRF if an Rx Diameter session related to the AF signalling is already established. The AF may modify an already open Rx Diameter session related to the AF signalling (e.g. an Rx Diameter session established for the purpose of subscription to notification of signalling path status as described in 4.4.5) or it may open a new Rx Diameter session related to the AF signalling if none exists.

To provision the AF signalling flow information the AF shall provide the UE's IP address using either Framed-IP-Address AVP or Framed-IPv6-Prefix AVP. The AF shall additionally provide a Media-Component-Description AVP including one or more Media-Sub-Component AVP(s) representing the AF signalling IP flows. The Media-Component-Description AVP shall contain the Media-Component-Number AVP set to "0". Each Media-Sub-Component AVP representing an AF signalling IP flow shall contain the Flow-Number AVP set according to the rules described in Annex B and one or two Flow-Description AVP(s) set to the IP flows of the AF signalling. Additionally, the Media-Sub-Component AVP shall include the Flow-Usage AVP set to the value "AF_SIGNALLING", the Flow-Status AVP set to "ENABLED" and the AF-Signalling-Protocol AVP set to the value corresponding to the signalling protocol used between the UE and the AF.

When the PCRF receives from the AF an AA-Request as described in the preceding paragraph, the PCRF shall perform session binding as described in 3GPP TS 29.213 [9] and shall acknowledge the AAR command by sending an AA-Answer command to the AF.

PCC/QoS Rules related to the AF signalling IP flows could have been provisioned to PCEF/BBERF using the corresponding procedures specified in 3GPP TS 29.212 [8] at an earlier stage (e.g. typically at the establishment of the IP-CAN bearer dedicated for AF Signalling IP Flows). The PCRF shall install the corresponding dynamic PCC/QoS rule for the AF signalling IP flows.

The AF may de-provision the information about the AF signalling IP flows at any time. To do that, if the Rx Diameter session is only used to provide information about the AF Signalling IP flows, the AF shall close the Rx Diameter session by sending a Session-Termination-Request (STR) command to the PCRF, which shall be acknowledged with a Session-Termination-Answer (STA) command. Otherwise, the AF shall remove the IP flows within the Media-Sub-Component- AVP by supplying the Flow-Status AVP with value "REMOVED".

### 4.4.6	Traffic Plane Events

#### 4.4.6.1	IP-CAN Session Termination

When an IP-CAN session is terminated, the PCRF shall inform the AF about the IP-CAN session termination by sending an ASR (abort session request) command to the AF on each active Rx Diameter session.

When the AF receives the ASR command, it shall acknowledge the command by sending an ASA (abort session answer) command to the PCRF and indicate the termination of the session by sending an STR (session termination request) command to the PCRF. The PCRF shall acknowledge the termination of the session by sending an STA (session termination answer) command to the AF.

Signalling flows for IP-CAN session termination cases are presented in 3GPP TS 29.213 [9].

#### 4.4.6.2	Service Data Flow Deactivation

It may happen that one or more PCC/QoS Rules (i.e. Service Data Flows) are deactivated at the PCEF/BBERF at a certain time, either permanently or temporarily. When the PCRF gets the knowledge that one or more SDFs have been deactivated, (e.g. due to a bearer release or loss of bearer or out of credit condition), the PCRF shall inform the AF accordingly if the AF has previously subscribed using the Specific-Action AVP in the AAR command.

When not all the service data flows within the AF session are affected, the PCRF shall inform the AF by sending an RAR (re-authorization request) command. The RAR command shall include the deactivated IP Flows encoded in the Flows AVP and the cause encoded in the Specific-Action AVP.

When the AF receives the RAR command, it shall acknowledge the command by sending an RAA (re-authorization answer) command to the PCRF. The AF may also update the session information by sending an AAR (AA-request) command to the PCRF.

If the PCRF receives the AAR command, it shall acknowledge the command by sending an AAA (AA-answer) command to the AF.

When all the service data flows within the AF session are affected, the PCRF shall inform the AF by sending an ASR command on the Rx Diameter session related to the AF session. When the AF receives the ASR command, it shall acknowledge the command by sending an ASA (abort session answer) command to the PCRF. After that the AF shall initiate an AF session termination procedure as defined in clause 4.4.4.

Signalling flows for Service Data Flow Deactivation cases are presented in 3GPP TS 29.213 [9].

#### 4.4.6.3	Notification of Signalling Path Status

In the event that the PCRF is notified of the loss or release of resources associated to the PCC/QoS Rules corresponding with AF Signalling IP Flows, the PCRF shall inform the AF about the Loss of the Signalling Transmission path by sending a Re-Authorization Request (RAR) command to the AF. The RAR shall include the Specific-Action AVP set to the value "INDICATION_OF_LOSS_OF_BEARER" or "INDICATION_OF_RELEASE_OF_BEARER" and the deactivated IP Flow encoded in the Flows AVP.

NOTE:   If the IMS signalling specific PCC rules include a QCI corresponding to a non-GBR bearer, the INDICATION_OF_LOSS_OF_BEARER will not be reported.

When the AF receives the RAR command, it shall acknowledge the command by sending an RAA command to the PCRF.

The AF may then decide to terminate the Rx Diameter session used for the notification of the status of the AF Signalling transmission path. The AF may also decide to terminate any other active Rx Diameter session with the PCRF related to the AF Signalling which is not available any longer. In that case, the AF shall then initiate the AF Termination procedure towards the PCRF as defined in clause 4.4.4.

#### 4.4.6.4	IP-CAN type change Notification

If the AF has successfully subscribed to change notifications in UE's IP-CAN type and RAT type, the PCRF shall provide the UE’s IP-CAN type and RAT type in the AA-Answer if already known by the PCRF. The PCRF shall also send an RAR command when a corresponding event occurs, i.e. when the UE's IP-CAN type or RAT type (if the IP-CAN type is GPRS), changes or becomes available. In this case the RAR from the PCRF shall include the Specific-Action AVP for the subscribed event and include the IP-CAN-Type AVP and RAT-Type AVP (in case of 3GPP IP-CAN) for the UE's new IP-CAN/RAT. If the PCRF is informed of an IP-CAN type change due to IP flow mobility as specified in 3GPP TS 29.212 [8], where a subset the flows within the AF session are affected, the PCRF shall include IP-CAN-type and RAT-type information (if applicable) to IP flow mobility affected service data flows. The IP flow mobility affected service data flows are included within the Flows AVP at command level.

NOTE:	The RAT type event is only applicable for IP-CAN type GPRS, the PCRF will provide the RAT type information to the AF not only in case of GPRS IP-CAN type, but also in case of other 3GPP IP-CAN types.

#### 4.4.6.5	Access Network Charging Information Notification

If the AF has subscribed to a notification about Access Network Charging Information, the PCRF shall provide the Access Network Charging Information in the response, if already known by the PCRF. If not available, the PCRF shall provide the Access Network Charging Information by sending a Re-Authorization-Request (RAR) command when the Access Network Charging Information is received from the PCEF. If different Access Network Charging Information is applicable to the IP-CAN session, the PCRF shall notify the AF about the Access Network Charging Information that applies to each authorized flow. The RAR shall include the Specific-Action AVP set to the value "CHARGING_CORRELATION_EXCHANGE" and shall include the assigned Access-Network-Charging-Identifier(s) and may include the Access-Network-Charging-Address AVP.

#### 4.4.6.6	Reporting Usage for Sponsored Data Connectivity

When the AF session is associated with a sponsor and the AF provided usage monitoring thresholds for such sponsor to the PCRF when the Rx Diameter session was established or modified, the PCRF shall report accumulated usage to the AF, when

-	the PCRF detects that the usage threshold provided by the AF has been reached; or

-	the AF session is terminated by the AF; or

-	the AF session is terminated by the PCRF due to the IP-CAN session termination, the termination of all the service data flows of the AF session or the home operator policy disallowing the UE accessing the sponsored data connectivity in the roaming case.

When the PCRF detects that the usage threshold has been reached, the PCRF shall report the accumulated usage as provided by the PCEF to the AF in a RA-Request (RAR) command with the Specific-Action AVP set to the value USAGE_REPORT Otherwise, when the AF session is terminated by the AF or by the PCRF, the PCRF shall report the accumulated usage as provided by the PCEF to the AF in ST-Answer (STA) command.

The accumulated usage shall be reported in the Used-Service-Unit AVP within the Sponsored-Connectivity-Data AVP.

If the AF receives a RAR command indicating the usage threshold is reached, the AF may terminate the AF session or provide a new usage threshold in the Granted-Service-Unit AVP within the Sponsored-Connectivity-Data AVP to the PCRF in the AAR comand. Alternatively, the AF may allow the session to continue without providing new usage threshold in the AAR command.

NOTE:	After the PCRF reports the accumulated usage to the AF, the AF can provide a new usage threshold to the PCRF. The monitoring will not start until the PCRF receives the new threshold from the AF and provide it to the PCEF

# 5	Rx protocol

## 5.1	Protocol support

The Rx interface in the present release is based on Rx and Gq protocols defined for Release 6 as specified in 3GPP TS 29.211 [7] and 3GPP TS 29.209 [5] respectively. However, to be able to separate the policy and charging rules function (PCRF) of the present release from the policy decision function (PDF) and charging rules function (CRF) of Release 6, the Rx application in the present release has an own vendor specific Diameter application.

The Rx application is defined as an IETF vendor specific Diameter application, where the vendor is 3GPP and the Application-ID for the Rx application in the present release is 16777236. The vendor identifier assigned by IANA to 3GPP (http://www.iana.org/assignments/enterprise-numbers) is 10415.

NOTE:	A route entry can have a different destination based on the application identification AVP of the message.  Therefore, Diameter agents (relay, proxy, redirection, translation agents) must be configured appropriately to identify the 3GPP Rx application within the Auth-Application-Id AVP in order to create suitable routeing tables.

Due to the definition of the commands used in Rx protocol, there is no possibility to skip the Auth-Application-Id AVP and use the Vendor-Specific-Application-Id AVP instead. Therefore the Rx application identification shall be included in the Auth-Application-Id AVP.

With regard to the Diameter protocol defined over the Rx reference point, the PCRF acts as a Diameter server, in the sense that it is the network element that handles AF session authorization requests for a particular realm. The AF acts as the Diameter client, in the sense that is the network element requesting the authorization of resources for an AF session.

## 5.2	Initialization, maintenance and termination of connection and session

The initialization and maintenance of the connection between each AF and PCRF pair is defined by the underlying protocol. Establishment and maintenance of connections between Diameter nodes is described in RFC 3588 [10].

After establishing the transport connection, the PCRF and the AF shall advertise the support of the Rx specific Application by including the value of the application identifier in the Auth-Application-Id AVP and the value of the 3GPP (10415) in the Vendor-Id AVP of the Vendor-Specific-Application-Id AVP contained in the Capabilities-Exchange-Request and Capabilities-Exchange-Answer commands. The Capabilities-Exchange-Request and Capabilities-Exchange-Answer commands are specified in the Diameter Base Protocol (RFC 3588 [10]).

The termination of the Diameter user session is specified in RFC 3588 [10] in clauses 8.4 and 8.5. The description of how to use of these termination procedures in the normal cases is embedded in the procedures description (clause 4.4).

## 5.3	Rx specific AVPs

Table 5.3.1 describes the Diameter AVPs defined for the Rx interface protocol, their AVP Code values, types, possible flag values, whether or not the AVP may be encrypted and which supported feature the AVP is applicable to. The Vendor-Id header of all AVPs defined in the present document shall be set to 3GPP (10415).

NOTE:	Most of these AVPs have already been defined in 3GPP TS 29.209 [5] for Rel-6. Their definition is based on the one used for Rel-6 with some possible modifications to be applied to the Rel-7 protocols.

Table 5.3.1: Rx specific Diameter AVPs

|  |  |  |  | AVP Flag rules (Note 1) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Attribute Name | AVP Code | Clause defined | Value Type (Note 2) | Must | May | Should not | Must not | May Encr. | Applicability (Note 3) |
| Abort-Cause | 500 | 5.3.1 | Enumerated | M,V | P |  |  | Y |  |
| Access-Network-Charging-Address | 501 | 5.3.2 | Address | M,V | P |  |  | Y |  |
| Access-Network-Charging-Identifier | 502 | 5.3.3 | Grouped | M,V | P |  |  | Y |  |
| Access-Network-Charging-Identifier-Value | 503 | 5.3.4 | OctetString | M,V | P |  |  | Y |  |
| Acceptable-Service-Info | 526 | 5.3.24 | Grouped | M,V | P |  |  | Y |  |
| AF-Application-Identifier | 504 | 5.3.5 | OctetString | M,V | P |  |  | Y |  |
| AF-Charging-Identifier | 505 | 5.3.6 | OctetString | M,V | P |  |  | Y |  |
| Application-Service-Provider-Identity | 532 | 5.3.29 | UTF8String | V | P |  | M | Y | SponsoredConnectivity |
| Codec-Data | 524 | 5.3.7 | OctetString | M,V | P |  |  | Y |  |
| Flow-Description | 507 | 5.3.8 | IPFilterRule | M,V | P |  |  | Y |  |
| Flow-Number | 509 | 5.3.9 | Unsigned32 | M,V | P |  |  | Y |  |
| Flows | 510 | 5.3.10 | Grouped | M,V | P |  |  | Y |  |
| Flow-Status | 511 | 5.3.11 | Enumerated | M,V | P |  |  | Y |  |
| Flow-Usage | 512 | 5.3.12 | Enumerated | M,V | P |  |  | Y |  |
| Service-URN | 525 | 5.3.23 | OctetString | M,V | P |  |  | Y |  |
| Specific-Action | 513 | 5.3.13 | Enumerated | M,V | P |  |  | Y |  |
| Max-Requested-Bandwidth-DL | 515 | 5.3.14 | Unsigned32 | M,V | P |  |  | Y |  |
| Max-Requested-Bandwidth-UL | 516 | 5.3.15 | Unsigned32 | M,V | P |  |  | Y |  |
| Media-Component-Description | 517 | 5.3.16 | Grouped | M,V | P |  |  | Y |  |
| Media-Component-Number | 518 | 5.3.17 | Unsigned32 | M,V | P |  |  | Y |  |
| Media-Sub-Component | 519 | 5.3.18 | Grouped | M,V | P |  |  | Y |  |
| Media-Type | 520 | 5.3.19 | Enumerated | M,V | P |  |  | Y |  |
| MPS-Identifier | 528 | 5.3.30 | OctetString | V | P |  | M | Y | Rel10 |
| Min-Requested-Bandwidth-DL | 534 | 5.3.31 | Unsigned32 | V | P |  | M | Y | Rel10 |
| Min-Requested-Bandwidth-UL | 535 | 5.3.32 | Unsigned32 | V | P |  | M | Y | Rel10 |
| RR-Bandwidth | 521 | 5.3.20 | Unsigned32 | M,V | P |  |  | Y |  |
| RS-Bandwidth | 522 | 5.3.21 | Unsigned32 | M,V | P |  |  | Y |  |
| Service-Info-Status | 527 | 5.3.25 | Enumerated | M,V | P |  |  | Y |  |
| SIP-Forking-Indication | 523 | 5.3.22 | Enumerated | M,V | P |  |  | Y |  |
| Sponsor-Identity | 531 | 5.3.28 | UTFString | V | P |  | M | Y | SponsoredConnectivity |
| Sponsored-Connectivity-Data | 530 | 5.3.27 | Grouped | V | P |  | M | Y | SponsoredConnectivity |
| AF-Signalling-Protocol | 529 | 5.3.26 | Enumerated | V | P |  | M | Y | ProvAFsignalFlow |
| NOTE 1: The AVP header bit denoted as 'M', indicates whether support of the AVP is required. The AVP header bit denoted as 'V', indicates whether the optional Vendor-ID field is present in the AVP header. For further details, see RFC 3588 [10]. NOTE 2: The value types are defined in RFC 3588 [10]. NOTE 3: AVPs marked with "ProvAFsignalFlow", "SponsoredConnectivity" or "Rel10" are applicable as described in clause 5.4.1 |  |  |  |  |  |  |  |  |  |

### 5.3.1	Abort-Cause AVP

The Abort-Cause AVP (AVP code 500) is of type Enumerated, and determines the cause of an abort session request (ASR) or of a RAR indicating a bearer release. The following values are defined:

BEARER_RELEASED (0)

This value is used when the bearer has been deactivated as a result from normal signalling handling. For GPRS the bearer refers to the PDP Context.

INSUFFICIENT_SERVER_RESOURCES (1)

This value is used to indicate that the server is overloaded and needs to abort the session.

INSUFFICIENT_BEARER_RESOURCES (2)

This value is used when the bearer has been deactivated due to insufficient bearer resources at a transport gateway (e.g. GGSN for GPRS).

PS_TO_CS_HANDOVER (3)

This value is used when the bearer has been deactivated due to PS to CS handover.SPONSORED_DATA_CONNECTIVITY_ DISALLOWED (4)

This value is used in the ASR when the PCRF needs to initiates the AF session termination due to the operator policy (e.g. disallowing the UE accessing the sponsored data connectivity in the roaming case).

### 5.3.2	Access-Network-Charging-Address AVP

The Access-Network-Charging-Address AVP (AVP code 501) is of type Address, and it indicates the IP Address of the network entity within the access network performing charging (e.g. the GGSN IP address). The Access-Network-Charging-Address AVP should not be forwarded over an inter-operator interface.

### 5.3.3	Access-Network-Charging-Identifier AVP

The Access-Network-Charging-Identifier AVP (AVP code 502) is of type Grouped, and contains a charging identifier (e.g. GCID) within the Access-Network-Charging-Identifier-Value AVP along with information about the flows transported within the corresponding bearer within the Flows AVP. If no Flows AVP is provided, the Access-Network-Charging-Identifier-Value applies for all flows within the AF session.

The Access-Network-Charging-Identifier AVP can be sent from the PCRF to the AF. The AF may use this information for charging correlation with session layer.

AVP Format:

Access-Network-Charging-Identifier ::= < AVP Header: 502 >

{ Access-Network-Charging-Identifier-Value}

*[ Flows ]

### 5.3.4	Access-Network-Charging-Identifier-Value AVP

The Access-Network-Charging-Identifier-Value AVP (AVP code 503) is of type OctetString, and contains a charging identifier (e.g. GCID).

### 5.3.5	AF-Application-Identifier AVP

The AF-Application-identifier AVP (AVP code 504) is of type OctetString, and it contains information that identifies the particular service that the AF service session belongs to. This information may be used by the PCRF to differentiate QoS for different application services.

For example the AF-Application-Identifier may be used as additional information together with the Media-Type AVP when the QoS class for the bearer authorization at the Gx interface is selected. The AF-Application-Identifier may be used also to complete the QoS authorization with application specific default settings in the PCRF if the AF does not provide full Session-Component-Description information.

### 5.3.6	AF-Charging-Identifier AVP

The AF-Charging-Identifier AVP (AVP code 505) is of type OctetString, contains the AF Charging Identifier that is sent by the AF. This information may be used for charging correlation with bearer layer.

### 5.3.7	Codec-Data AVP

The Codec-Data AVP (AVP code 524) is of type OctetString.

The Codec-Data AVP shall contain codec related information known at the AF. This information shall be encoded as follows:

-	The first line of the value of the Codec-Data AVP shall consist of either the word "uplink" or the word "downlink" (in ASCII, without quotes) followed by a new-line character. The semantics of these words are the following:

-	"uplink" indicates that the SDP was received from the UE and sent to the network.

-	"downlink" indicates that the SDP was received from the network and sent to the UE.

NOTE:	The first line indicates the direction of the source of the SDP used to derive the information. The majority of the information within the Codec-Data AVP indicating "downlink" describes properties, for instance receiver capabilities, of the sender of the SDP, the network in this case and is therefore applicable for IP flows in the uplink direction. Similarly, the majority of the information within the Codec-Data AVP indicating "uplink" describes properties, for instance receiver capabilities, of the sender of the SDP, the UE in this case and is therefore applicable for IP flows in the downlink direction.

-	The second line of the value of the Codec-Data AVP shall consist of either the word "offer" or the word "answer", or the word "description" (in ASCII, without quotes) followed by a new-line character. The semantics of these words are the following:

-	"offer" indicates that SDP lines from an SDP offer according to RFC 3264 [18] are being provisioned in the Codec-Data AVP;

-	"answer" indicates that SDP lines from an SDP answer according to RFC 3264 [18] are being provisioned in the Codec-Data AVP;

-	"description" indicates that SDP lines from a SDP session description in a scenario where the offer-answer mechanism of RFC 3264 [18] is not being applied are being provisioned in the Codec-Data AVP. For instance, SDP from an RTSP "Describe" reply may be provisioned.

-	The rest of the value shall consist of SDP line(s) in ASCII encoding separated by new-line characters, as specified in IETF RFC 4566 [13]. The first of these line(s) shall be an "m" line. The remaining lines shall be any available SDP "a" and "b" lines related to that "m" line. However, to avoid duplication of information, the SDP "a=sendrecv", "a=recvonly ", "a=sendonly", "a=inactive", "b:AS", "b:RS" and "b:RR" lines do not need to be included.

### 5.3.8	Flow-Description AVP

The Flow-Description AVP (AVP code 507) is of type IPFilterRule, and defines a packet filter for an IP flow with the following information:

-	Direction (in or out). The direction "in" refers to uplink IP flows, and the direction "out" refers to downlink IP flows.

-	Source and destination IP address (possibly masked).

-	Protocol.

-	Source and destination port.

The IPFilterRule type shall be used over Rx interface with the following restrictions:

-	The Source Port may be omitted to indicate that any source port is allowed. Lists or ranges shall not be used.

-	Only the Action "permit" shall be used.

-	No "options" shall be used.

-	The invert modifier "!" for addresses shall not be used.

-	The keyword "assigned" shall not be used.

NOTE:	For TCP protocol, destination port can also be omitted.

If any of these restrictions is not observed by the AF, the server shall send an error response to the AF containing the Experimental-Result-Code AVP with value FILTER_RESTRICTIONS.

For the Rx interface, the Flow description AVP shall be used to describe a single IP flow.

### 5.3.9	Flow-Number AVP

The Flow-Number AVP (AVP code 509) is of type Unsigned32, and it contains the ordinal number of the IP flow(s), assigned according to the rules in Annex B.

### 5.3.10	Flows AVP

The Flows AVP (AVP code 510) is of type Grouped, and it indicates IP flows via their flow identifiers.

When reporting an out of credit condition, the Final-Unit-Action AVP indicates the termination action applied to the impacted flows.

If no Flow-Number AVP(s) are supplied, the Flows AVP refers to all Flows matching the media component number.

AVP Format:

Flows::= < AVP Header: 510 >

{Media-Component-Number}

*[Flow-Number]

[Final-Unit-Action]

### 5.3.11	Flow-Status AVP

The Flow-Status AVP (AVP code 511) is of type Enumerated, and describes whether the IP flow(s) are enabled or disabled. The following values are defined:

ENABLED-UPLINK (0)

This value shall be used to enable associated uplink IP flow(s) and to disable associated downlink IP flow(s).

ENABLED-DOWNLINK (1)

This value shall be used to enable associated downlink IP flow(s) and to disable associated uplink IP flow(s).

ENABLED (2)

This value shall be used to enable all associated IP flow(s) in both directions.

DISABLED (3)

This value shall be used to disable all associated IP flow(s) in both directions.

REMOVED (4)

This value shall be used to remove all associated IP flow(s). The IP Filters for the associated IP flow(s) shall be removed. The associated IP flows shall not be taken into account when deriving the authorized QoS.

NOTE:	The interpretation of values for the RTCP flows in the Rx interface is described within the procedures in clause 4.4.3.

### 5.3.12	Flow-Usage AVP

The Flow-Usage AVP (AVP code 512) is of type Enumerated, and provides information about the usage of IP Flows. The following values are defined:

NO_INFORMATION (0)

This value is used to indicate that no information about the usage of the IP flow is being provided.

RTCP (1)

This value is used to indicate that an IP flow is used to transport RTCP.

AF_SIGNALLING (2)

This value is used to indicate that the IP flow is used to transport AF Signalling Protocols (e.g. SIP/SDP).

NO_INFORMATION is the default value.

NOTE:	An AF may choose not to identify RTCP flows, e.g. in order to avoid that RTCP flows are always enabled by the server.

### 5.3.13	Specific-Action AVP

The Specific-Action AVP (AVP code 513) is of type Enumerated.

Within a PCRF initiated Re-Authorization Request, the Specific-Action AVP determines the type of the action.

Within an initial AA request the AF may use the Specific-Action AVP to request specific actions from the server at the bearer events and to limit the contact to such bearer events where specific action is required. If the Specific-Action AVP is omitted within the initial AA request, no notification of any of the events defined below is requested.

The following values are defined:

Void (0)

CHARGING_CORRELATION_EXCHANGE (1)

Within a RAR, this value shall be used when the server reports the access network charging identifier to the AF. The Access-Network-Charging-Identifier AVP shall be included within the request. In the AAR, this value indicates that the AF requests the server to provide the access network charging identifier to the AF for each authorized flow, when the access network charging identifier becomes known at the PCRF.

INDICATION_OF_LOSS_OF_BEARER (2)

Within a RAR, this value shall be used when the server reports a loss of a bearer (e.g. in the case of GPRS PDP context bandwidth modification to 0 kbit) to the AF. The SDFs that are deactivated as a consequence of this loss of bearer shall be provided within the Flows AVP. In the AAR, this value indicates that the AF requests the server to provide a notification at the loss of a bearer.

INDICATION_OF_RECOVERY_OF_BEARER (3)

Within a RAR, this value shall be used when the server reports a recovery of a bearer (e.g. in the case of GPRS, PDP context bandwidth modification from 0 kbit to another value) to the AF. The SDFs that are re-activated as a consequence of the recovery of bearer shall be provided within the Flows AVP. In the AAR, this value indicates that the AF requests the server to provide a notification at the recovery of a bearer.

INDICATION_OF_RELEASE_OF_BEARER (4)

Within a RAR, this value shall be used when the server reports the release of a bearer (e.g. PDP context removal for GPRS) to the AF. The SDFs that are deactivated as a consequence of this release of bearer shall be provided within the Flows AVP. In the AAR, this value indicates that the AF requests the server to provide a notification at the removal of a bearer.

Void (5)

IP-CAN_CHANGE (6)

This value shall be used in RAR command by the PCRF to indicate a change in the IP-CAN type or RAT type (if the IP-CAN type is GPRS). When used in an AAR command, this value indicates that the AF is requesting subscription to IP-CAN change and RAT change notification. When used in RAR it indicates that the PCRF generated the request because of an IP-CAN or RAT change. IP-CAN-Type AVP and RAT-Type AVP (in case of 3GPP IP-CAN) shall be provided in the same request with the new/valid value(s).

If an IP-CAN type or RAT type change is due to IP flow mobility and a subset of the flows within the AF session is affected, the affected service data flows shall be provided in the same request.

INDICATION_OF_OUT_OF_CREDIT (7)

Within a RAR, this value shall be used when the PCRF reports to the AF that SDFs have run out of credit, and that the termination action indicated by the corresponding Final-Unit-Action AVP applies (3GPP TS 32.240 [23] and 3GPP TS 32.299 [24). The SDFs that are impacted as a consequence of the out of credit condition shall be provided within the Flows AVP. In the AAR, this value indicates that the AF requests the PCRF to provide a notification of SDFs for which credit is no longer available. Applicable to functionality introduced with the Rel8 feature as described in clause 5.4.1.

INDICATION_OF_SUCCESSFUL_RESOURCES_ALLOCATION (8)

Within a RAR, this value shall be used by the PCRF to indicate that the resources requested for particular service information have been successfully allocated. The SDFs corresponding to the resources successfully allocated shall be provided within the Flows AVP.

In the AAR, this value indicates that the AF requests the PCRF to provide a notification when the resources associated to the corresponding service information have been allocated.

Applicable to functionality introduced with the Rel8 feature as described in clause 5.4.1.

NOTE: 	This value applies to applications for which the successful resource allocation notification is required for their operation since subscription to this value impacts the resource allocation signalling overhead towards the PCEF/BBERF.

INDICATION_OF_FAILED_RESOURCES_ALLOCATION (9)

Within a RAR, this value shall be used by the PCRF to indicate that the resources requested for a particular service information cannot be successfully allocated. The SDFs corresponding to the resources that could not be allocated shall be provided within the Flows AVP.

In the AAR, this value indicates that the AF requests the PCRF to provide a notification when the resources associated to the corresponding service information cannot be allocated. Applicable to functionality introduced with the Rel8 feature as described in clause 5.4.1.

NOTE: 	This value applies to applications for which the unsuccessful resource allocation notification is required for their operation since subscription to this value impacts the resource allocation signalling overhead towards the PCEF/BBERF.

INDICATION_OF_LIMITED_PCC_DEPLOYMENT (10)

Within a RAR, this value shall be used when the PCRF reports the limited PCC deployment (i.e. dynamically allocated resources are not applicable) as specified at Annex K and Annex L in 3GPP TS 23.203 [2] to the AF. In the AAR, this value indicates that the AF requests the PCRF to provide a notification for the limited PCC deployment. Applicable to functionality introduced with the Rel8 feature as described in subclause 5.4.1.

USAGE_REPORT (11)

In the RA-Request (RAR), this value shall be used by the PCRF to report accumulated usage volume when the usage threshold provided by the AF has been reached.

In the AA-Request (AAR), this value indicates that the AF requests PCRF to report accumulated usage volume when it reaches the threshold.

Applicable to functionality introduced with the SponsoredConnectivity feature as described in clause 5.4.1.

INDICATION_OF_RECOVERY_FROM_LIMITED_PCC_DEPLOYMENT (13)

Within a RAR, this value shall be used when the PCRF reports the recovery from limited PCC deployment (i.e. the UE moves from the VPLMN to the HPLMN as specified at Annex K in 3GPP TS 23.203 [2]) to the AF. In the AAR, this value indicates that the AF requests the PCRF to provide a notification for the recovery from limited PCC deployment. Applicable to functionality introduced with the Rel8 feature as described in subclause 5.4.1.

NOTE:	This value is optional and only applicable to the scenario where PCC is deployed in the HPLMN but not in the VPLMN and dynamic policy provisioning only occurs in the home routed roaming cases if no BBERF is employed.

### 5.3.14	Max-Requested-Bandwidth-DL AVP

The Max-Requested-Bandwidth-DL AVP (AVP code 515) is of type Unsigned32, and it indicates the maximum bandwidth in bits per second for a downlink IP flow. The bandwidth contains all the overhead coming from the IP-layer and the layers above, e.g. IP, UDP, RTP and RTP payload.

When provided in an AA-Request, it indicates the maximum requested bandwidth. When provided in an AA-Answer, it indicates the maximum bandwidth acceptable by PCRF.

### 5.3.15	Max-Requested-Bandwidth-UL AVP

The Max -Bandwidth-UL AVP (AVP code 516) is of type Unsigned32, and it indicates the maximum requested bandwidth in bits per second for an uplink IP flow. The bandwidth contains all the overhead coming from the IP-layer and the layers above, e.g. IP, UDP, RTP and RTP payload.

When provided in an AA-Request, it indicates the maximum requested bandwidth. When provided in an AA-Answer, it indicates the maximum bandwidth acceptable by PCRF.

### 5.3.16	Media-Component-Description AVP

The Media-Component-Description AVP (AVP code 517) is of type Grouped, and it contains service information for a single media component within an AF session or the AF signalling information. The service information may be based on the SDI exchanged between the AF and the AF session client in the UE. The information may be used by the PCRF to determine authorized QoS and IP flow classifiers for bearer authorization and PCC rule selection.

Within one Diameter message, a single IP flow shall not be described by more than one Media-Component-Description AVP.

Bandwidth information and Flow-Status information provided within the Media-Component-Description AVP applies to all those IP flows within the media component, for which no corresponding information is being provided within Media-Sub-Component AVP(s).

If a Media-Component-Description AVP is not supplied by the AF, or if optional AVP(s) within a Media-Component-Description AVP are omitted, but corresponding information has been provided in previous Diameter messages, the previous information for the corresponding IP flow(s) remains valid.

All IP flows within a Media-Component-Description AVP are permanently disabled by supplying a Flow Status AVP with value "REMOVED". The server may delete corresponding filters and state information.

Reservation-Priority provided within the Media-Component-Description AVP in the request from the AF applies to all those IP flows within the media component and describes the relative importance of the IP flow as compared to other IP flows. The PCRF may use this value to implement priority based admission. If the Reservation-Priority AVP is not specified the IP flow priority is DEFAULT (0).

Each Media-Component-Description AVP shall contain either zero, or one, or two Codec-Data AVPs. In the case of conflicts, information contained in other AVPs either within this Media-Component-Description AVP, or within the corresponding Media-Component-Description AVP in a previous message, shall take precedence over information within the Codec-Data AVP(s). The AF shall provision all the available information in other applicable AVPs in addition to the information in the Codec-Data AVP, if such other AVPs are specified.

If the SDP offer-answer procedures of IETF RFC 3264 [18] are applicable for the session negotiation between the two ends taking part in the communication (e.g. for IMS), the following applies:

-	The AF shall provision information derived from an SDP answer and shall also provision information derived from the corresponding SDP offer.

-	If the Media-Component-Description AVP contains two Codec-Data AVPs, one of them shall represent an SDP offer and the other one the corresponding SDP answer.

-	If the Media-Component-Description AVP contains one Codec-Data AVP, and this AVP represents an SDP offer, the AF shall provision the corresponding SDP answer information in a Codec-Data AVP within a subsequent Rx message.

NOTE:	Some SDP parameters for the same codec in the SDP offer and answer are independent of each other and refer to IP flows in opposite directions, for instance some MIME parameters conveyed within "a=fmtp" SDP lines and the packetization time within the "a=ptime" line. Other parameters within the SDP answer take precedence over corresponding parameters within the SDP offer.

If SDP is applied without using the offer-answer procedures, zero or one Codec-Data AVP shall be provisioned.

The PCRF may provide the Media-Component-Description AVP(s) within the Acceptable-Service-Info AVP in the AA-Answer command if the service information received from the AF is rejected. For this usage, the Media-Component-Description AVP shall only include the appropriate Media-Component-Number AVP and the Max-Requested-Bandwidth-UL and/or Max-Requested-Bandwidth-DL AVPs indicating the maximum acceptable bandwidth.

AVP format:

Media-Component-Description ::= < AVP Header: 517 >

{ Media-Component-Number } ; Ordinal number of the media comp.

*[ Media-Sub-Component ]    ; Set of flows for one flow identifier

[ AF-Application-Identifier ]

[ Media-Type ]

[ Max-Requested-Bandwidth-UL ]

[ Max-Requested-Bandwidth-DL ]

[ Min-Requested-Bandwidth-UL ]

[ Min-Requested-Bandwidth-DL ]

[ Flow-Status ]

[ Reservation-Priority ]

[ RS-Bandwidth ]

[ RR-Bandwidth ]

*[ Codec-Data ]

### 5.3.17	Media-Component-Number AVP

The Media-Component-Number AVP (AVP code 518) is of type Unsigned32, and it contains the ordinal number of the media component, assigned according to the rules in Annex B.

When this AVP refers to AF signalling, this is indicated by using the value 0 according to the rules in Annex B.

### 5.3.18	Media-Sub-Component AVP

The Media-Sub-Component AVP (AVP code 519) is of type Grouped, and it contains the requested bitrate and filters for the set of IP flows identified by their common Flow-Identifier. The Flow-Identifier is defined in Annex B.

Possible Bandwidth information and Flow-Status information provided within the Media-Sub-Component AVP takes precedence over information within the encapsulating Media Component Description AVP. If a Media-Sub-Component- AVP is not supplied, or if optional AVP(s) within a Media-Sub-Component AVP are omitted, but corresponding information has been provided in previous Diameter messages, the previous information for the corresponding IP flow(s) remains valid, unless new information is provided within the encapsulating Media-Component-Description AVP. If Flow-Description AVP(s) are supplied, they replace all previous Flow-Description AVP(s), even if a new Flow-Description AVP has the opposite direction as the previous Flow-Description AVP.

The AF-Signalling-Protocol AVP may be included only if the Flow-Usage AVP has a value of 'AF_SIGNALLING'.

All IP flows within a Media-Sub-Component- AVP are permanently disabled by supplying a Flow Status AVP with value "REMOVED". The server may delete corresponding filters and state information.

AVP format:

Media-Sub-Component ::= < AVP Header: 519 >

{ Flow-Number }      		   ; Ordinal number of the IP flow

0*2[ Flow-Description ]        ; UL and/or DL

[ Flow-Status ]

[ Flow-Usage ]

[ Max-Requested-Bandwidth-UL ]

[ Max-Requested-Bandwidth-DL ]

[ AF-Signalling-Protocol ]

*[ AVP ]

### 5.3.19	Media-Type AVP

The Media-Type AVP (AVP code 520) is of type Enumerated, and it determines the media type of a session component. The media types indicate the type of media in the same way as the SDP media types with the same names defined in RFC 4566 [13]. The following values are defined:

-	AUDIO (0)

-	VIDEO (1)

-	DATA (2)

-	APPLICATION (3)

-	CONTROL (4)

-	TEXT (5)

-	MESSAGE (6)

-	OTHER (0xFFFFFFFF)

### 5.3.20	RR-Bandwidth AVP

The RR-Bandwidth AVP (AVP code 521) is of type Unsigned32, and it indicates the maximum required bandwidth in bits per second for RTCP receiver reports within the session component, as specified in RFC 3556 [11]. The bandwidth contains all the overhead coming from the IP-layer and the layers above, i.e. IP, UDP and RTCP.

### 5.3.21	RS-Bandwidth AVP

The RS-Bandwidth AVP (AVP code 522) is of type Unsigned32, and it indicates the maximum required bandwidth in bits per second for RTCP sender reports within the session component, as specified in RFC 3556 [11]. The bandwidth contains all the overhead coming from the IP-layer and the layers above, i.e. IP, UDP and RTCP.

### 5.3.22	SIP-Forking-Indication AVP

The SIP-Forking-Indication AVP (AVP code 523) is of type Enumerated, and describes if several SIP dialogues are related to one Diameter session:

SINGLE_DIALOGUE (0)

This value is used to indicate that the Diameter session relates to a single SIP dialogue.
This is the default value applicable if the AVP is omitted.

SEVERAL_DIALOGUES (1)

This value is used to indicate that the Diameter session relates to several SIP dialogues.

### 5.3.23	Service-URN AVP

The Service-URN AVP (AVP code 525) is of type OctetString, and it indicates that an AF session is used for emergency traffic.

It contains values of the service URN including subservices, as defined in [21] or registered at IANA. The string "urn:service:" in the beginning of the URN shall be omitted in the AVP and all subsequent text shall be included. Examples of valid values of the AVP are "sos", "sos.fire", "sos.police" and "sos.ambulance".

### 5.3.24	Acceptable-Service-Info AVP

The Acceptable-Service-Info AVP (AVP code 526) is of type Grouped, and contains the maximum bandwidth for an AF session and/or for specific media components that will be authorized by the PCRF. The Max-Requested-Bandwidth-DL AVP and Max-Requested-Bandwidth-UL AVP directly within the Acceptable-Service-Info AVP indicate the acceptable bandwidth for the entire AF session. The Max-Requested-Bandwidth-DL AVP and Max-Requested-Bandwidth-UL AVP within a Media-Component-Description AVP included in the Acceptable-Service-Info AVP indicate the acceptable bandwidth for the corresponding media component.

If the acceptable bandwidth applies to one or more media components, only the Media-Component-Description AVP will be provided. If the acceptable bandwidth applies to the whole AF session, only the Max-Requested-Bandwidth-DL AVP and Max-Requested-Bandwidth-UL AVP will be included.

Acceptable-Service-Info::= < AVP Header: 526 >

*[ Media-Component-Description]

[ Max-Requested-Bandwidth-DL ]

[ Max-Requested-Bandwidth-UL ]

*[ AVP ]

### 5.3.25	Service-Info-Status-AVP

The Service-Info-Status AVP (AVP code 527) is of type Enumerated, and indicates the status of the service information that the AF is providing to the PCRF. If the Service-Info-Status AVP is not provided in the AA request, the value FINAL SERVICE INFORMATION shall be assumed.

FINAL SERVICE INFORMATION (0)

This value is used to indicate that the service has been fully negotiated between the two ends and service information provided is the result of that negotiation.

PRELIMINARY SERVICE INFORMATION (1)

This value is used to indicate that the service information that the AF has provided to the PCRF is preliminary and needs to be further negotiated between the two ends (e.g. for IMS when the service information is sent based on the SDP offer).

### 5.3.26	AF-Signalling-Protocol-AVP

The AF-Signalling-Protocol AVP (AVP code 529) is of type Enumerated, and indicates the protocol used for signalling between the UE and the AF. If the AF-Signalling-Protocol AVP is not provided in the AA-Request, the value NO_INFORMATION shall be assumed.

NO_INFORMATION (0)

This value is used to indicate that no information about the AF signalling protocol is being provided.

SIP (1)

This value is used to indicate that the signalling protocol is Session Initiation Protocol.

### 5.3.27	Sponsored-Connectivity-Data AVP

The Sponsored-Connectivity-Data AVP (AVP code 530) is of type Grouped, and indicates the data associated with the sponsored data connectivity that the AF is providing to the PCRF.

The Sponsor-Identity AVP identifies the sponsor. It shall be included by the AF in the Sponsored-Connectivity-Data AVP.

The Application-Service-Provider-Identity AVP identifies the application service provider. It shall be included by the AF in the Sponsored-Connectivity-Data AVP.

The Granted-Service-Unit AVP shall be used by the AF to provide usage threshold level to the PCRF if the volume of traffic allowed during the sponsored data connectivity is to be monitored.

The Used-Service-Unit AVP shall be used by the PCRF to provide the measured usage to the AF. Reporting shall be done, as requested by the AF, in CC-Total-Octets, CC-Input-Octets or CC-Output-Octets of the Used-Service-Unit AVP.

AVP format:

Sponsored-Connectivity-Data::= < AVP Header: 530 >

[ Sponsor-Identity ]

[ Application-Service-Provider-Identity ]

[ Granted-Service-Unit ]

[ Used-Service-Unit ]

*[ AVP ]

### 5.3.28	Sponsor-Identity AVP

The Sponsor-Identity AVP (AVP code 531) is of type UTF8String and is used for sponsored data connectivity purposes as an identifier of the sponsor.

### 5.3.29	Application-Service-Provider-Identity AVP

The Application-Service-Provider-Identity AVP (AVP code 532) is of type UTF8String and is used for sponsored data connectivity purposes as an identifier of the application service provider.

### 5.3.30	MPS-Identifier AVP

The MPS-Identifier AVP (AVP code 528) is of type OctetString, and it indicates that an AF session relates to an MPS session.  It contains the national variant for MPS service name (e.g., NGN GETS).

### 5.3.31	Min-Requested-Bandwidth-DL AVP

The Min-Requested-Bandwidth-DL AVP (AVP code 534) is of type Unsigned32, and it indicates the minimum requested bandwidth in bits per second for a downlink IP flow. The bandwidth contains all the overhead coming from the IP-layer and the layers above, e.g. IP, TCP, UDP, HTTP, RTP and RTP payload.

When provided in an AA-Request, it indicates the minimum requested bandwidth.

### 5.3.32	Min-Requested-Bandwidth-UL AVP

The Min-Requested-Bandwidth-UL AVP (AVP code 535) is of type Unsigned32, and it indicates the minimum requested bandwidth in bits per second for an uplink IP flow. The bandwidth contains all the overhead coming from the IP-layer and the layers above, e.g. IP, TCP, UDP, HTTP, RTP and RTP payload.

When provided in an AA-Request, it indicates the minimum requested bandwidth.

## 5.4	Rx re-used AVPs

Table 5.4.1 lists the Diameter AVPs re-used by the Rx reference point from existing Diameter Applications, including a reference to their respective specifications and when needed, a short description of their usage within the Rx reference point. Other AVPs from existing Diameter Applications, except for the AVPs from Diameter Base Protocol, do not need to be supported. The AVPs from Diameter Base Protocol are not included in table 5.4.1, but they are re-used for the Rx protocol. Unless otherwise stated, re-used AVPs shall maintain their 'M', 'P' and 'V' flag settings. Where 3GPP Radius VSAs are re-used, unless otherwise stated, they shall be translated to Diameter AVPs as described in RFC 4005 [12] with the exception that the 'M' flag shall be set and the 'P' flag may be set.

Table 5.4.1: Rx re-used Diameter AVPs

| Attribute Name | Reference | Comments | Applicability (notes 1, 2) |
| --- | --- | --- | --- |
| Called-Station-Id | RFC 4005 [12] | The PDN the user is connected to. | Rel8 |
| Final-Unit-Action | RFC 4006 [14] | The action applied by the PCEF when the user's account cannot cover the service cost. | Rel8 |
| Framed-IP-Address | RFC 4005 [12] | The valid routable IPv4 address that is applicable for the IP Flows towards the UE at the PCEF. The PCRF shall use this address to identify the correct IP-CAN session (session binding). For example, the IP address may actually be that of the network interface of a NAT device between the UE and the GW. The values 0xFFFFFFFF and 0xFFFFFFFE are not applicable as described in RFC 4005 [12]. |  |
| Framed-IPv6-Prefix | RFC 4005 [12] | A valid full IPv6 address that is applicable to an IP flow or IP flows towards the UE at the PCEF. The PCRF shall use this address to identify the correct IP-CAN session (session binding, refer to 3GPP TS 29.213 [9]). For example, the IP address may actually be that of the network interface of a NAT device between the UE and the GW. The encoding of the value within this Octet String type AVP shall be as defined in RFC 3162 [20], clause 2.3. The "Reserved", "Prefix-Length" and "Prefix" fields shall be included in this order. The AF shall set the "Prefix Length" to 128 and encode the IPv6 address of the UE within the "Prefix" field. |  |
| Granted-Service-Unit | RFC 4006 [14] | The volume thresholds for sponsored data connectivity. Only CC-Total-Octets or one of the CC-Input-Octets and CC-Output-Octets AVPs are reused. This AVP shall have the 'M' bit cleared. | SponsoredConnectivity |
| IP-CAN-Type | 3GPP TS 29.212 [8] | IP-CAN type of the user. |  |
| RAT-Type | 3GPP TS 29.212[8] | Indicate which Radio Access Technology is currently serving the UE. | Rel8 |
| Reservation-Priority | TS 183.017 [15] | The vendor-id shall be set to ETSI (13019) [15]. The support of this AVP shall be advertised in the capabilities exchange mechanisms (CER/CEA) by including the ETSI parameter in the Supported-Vendor-Id AVP. |  |
| Subscription-Id | RFC 4006 [14] | The identification of the subscription (IMSI, MSISDN, etc.) |  |
| Supported-Features | 3GPP TS 29.229 [25] | If present, this AVP informs the destination host about the features that the origin host requires to successfully complete this command exchange. | Rel8 |
| Used-Service-Unit | RFC 4006 [14] | The measured volume for sponsored data connectivity. Only CC-Total-Octets or one of the CC-Input-Octets and CC-Output-Octets AVPs are reused. This AVP shall have the 'M' bit cleared. | SponsoredConnectivity |
| NOTE 1: AVPs marked with "Rel8" are applicable as described in clause 5.4.1. NOTE 2: AVPs marked with "SponsoredConnectivity" are applicable for sponsored data connectivity. |  |  |  |

### 5.4.1		Use of the Supported-Features AVP on the Rx reference point

The Supported-Features AVP is used during session establishment to inform the destination host about the required and optional features that the origin host supports. The client shall, in the first request of a Diameter session indicate the set of features required for the successul processing of the session. If there are features supported by the client that are not advertised as part of the required set of features, the client shall provide in the same request this set of features that are optional for the successful processing of the session. The server shall, in the first answer within the Diameter session indicate the set of features that it has in common with the client and that the server shall support within the same Diameter session. Any further command messages shall always be compliant with the list of supported features indicated in the Supported-Features AVPs during session establishment. Features that are not advertised as supported shall not be used to construct the command messages for that Diameter session. Unless otherwise stated, the use of the Supported-Features AVP on the Rx reference point shall be compliant with the requirements for dynamic discovery of supported features and associated error handling on the Cx reference point as defined in clause 7.2.1 of 3GPP TS 29.229 [25].

The base functionality for the Rx reference point is the 3GPP Rel-7 standard and a feature is an extension to that functionality. If the origin host does not support any features beyond the base functionality, the Supported-Features AVP may be absent from the Rx commands. As defined in clause 7.1.1 of 3GPP TS 29.229 [25], when extending the application by adding new AVPs for a feature, the new AVPs shall have the M bit cleared and the AVP shall not be defined mandatory in the command ABNF.

As defined in 3GPP TS 29.229 [25], the Supported-Features AVP is of type grouped and contains the Vendor-Id, Feature-List-ID and Feature-List AVPs. On the Rx reference point, the Supported-Features AVP is used to identify features that have been defined by 3GPP and hence, for features defined in this document, the Vendor-Id AVP shall contain the vendor ID of 3GPP (10415). If there are multiple feature lists defined for the Rx reference point, the Feature-List-ID AVP shall differentiate those lists from one another.

On receiving an initial request application message, the destination host shall act as defined in clause 7.2.1 of 3GPP TS 29.229 [25]. The following exceptions apply to the initial AAR/AAA command pair:

-	If the AF supports features that are required for the successful operation of the session, the AAR shall include these required features within Supported-Features AVP(s) with the ‘M’ bit set.

-	If the AF supports optional features that are not required for the successful operation of the session, the AAR shall include these optional features within Supported-Features AVP(s) with the ‘M’ bit cleared.

-	If the AAR command does not contain any Supported-Features AVP(s) and the PCRF supports Rel-7 Rx functionality, the AAA command shall not include the Supported-Features AVP. In this case, both AF and PCRF shall behave as specified in the Rel-7 version of this document.

-	If the AAR command contains the Supported-Features AVP(s) and the PCRF supports all the features advertised in the AAR command within Supported-Features AVP(s) with the ‘M’ bit set, the AAA command from the PCRF shall include the Supported-Features AVP(s), with the 'M' bit cleared, indicating only the features that both the PCRF and AF support.

Once the PCRF and AF have negotiated the set of supported features during session establishment, the set of common features shall be used during the lifetime of the Diameter session.

The table below defines the features applicable to the Rx interfaces for the feature list with a Feature-List-ID of 1.

Table 5.4.1.1: Features of Feature-List-ID 1 used in Rx

| Feature bit | Feature | M/O | Description |
| --- | --- | --- | --- |
| 0 | Rel8 | M | This feature indicates the support of the base 3GPP Rel-8 functionality, including the AVPs and corresponding procedures supported by the base 3GPP Rel-7 Rx standard, but excluding those features represented by separate feature bits. AVPs introduced with this feature are marked with "Rel8" in Table 5.4.1 |
| 1 | Rel9 | M | This feature indicates the support of the base 3GPP Rel-9 functionality, including the AVPs and corresponding procedures supported by the Rel8 feature bit, but excluding those features represented by separate feature bits. |
| 2 | ProvAFsignalFlow | O | This indicates support for the feature of provisioning of AF signalling flow information as described in subclause 4.4.5a. If the PCRF supports this feature the AF may provision AF signalling flow information. NOTE: This feature is used by the IMS Restoration Procedures to provide to the PDN-Gateway the address of the P-CSCF selected by the UE, refer to 3GPP TS 23.380 [28]. |
| 3 | SponsoredConnectivity | O | This feature indicates support for sponsored data connectivity feature. If the PCRF supports this feature, the AF may provide sponsored data connectivity to the subscriber. |
| 4 | Rel10 | M | This feature indicates the support of the base 3GPP Rel-10 functionality, including the AVPs and corresponding procedures supported by the Rel9 feature bit, but excluding those features represented by separate feature bits. |
| Feature bit: The order number of the bit within the Feature-List AVP where the least significant bit is assigned number "0". Feature: A short name that can be used to refer to the bit and to the feature, e.g. "EPS". M/O: Defines if the implementation of the feature is mandatory ("M") or optional ("O"). Description: A clear textual description of the feature. |  |  |  |

## 5.5	Rx specific Experimental-Result-Code AVP values

RFC 3588 [10] specifies the Experimental-Result AVP containing Vendor-ID AVP and Experimental-Result-Code AVP. The Experimental-Result-Code AVP (AVP Code 298) is of type Unsigned32 and contains a vendor-assigned value representing the result of processing a request. The Vendor-ID AVP shall be set to 3GPP (10415).

Specific values of the Rx specific Experimental-Result-Code AVP are:

INVALID_SERVICE_INFORMATION (5061)

The PCRF rejects new or modified service information the service information provided by the AF is invalid or insufficient for the server to perform the requested action.

FILTER_RESTRICTIONS (5062)

The PCRF rejects new or modified service information because the Flow-Description AVP(s) cannot be handled by the server because restrictions defined in clause 5.3.7 are not observed.

REQUESTED_SERVICE_NOT_AUTHORIZED (5063)

The PCRF rejects new or modified service information because the requested service, as described by the service information provided by the AF, is not consistent with either the related subscription information, operator defined policy rules and/or the supported features in the IP-CAN network.

DUPLICATED_AF_SESSION (5064)

The PCRF rejects a new Rx session setup because the new Rx session relates to an AF session with another related active Rx session, e.g. if the AF provided the same AF charging identifier for this new Rx session that is already in use for the other ongoing Rx session.

IP-CAN_SESSION_NOT_AVAILABLE (5065)

The PCRF rejects a new Rx session setup when it fails to associate the described service IP flows within the session information received from the AF to an existing IP-CAN session.

UNAUTHORIZED_NON_EMERGENCY_SESSION (5066)

The PCRF rejects a new Rx session setup because the session binding function associated a non-Emergency IMS session to an IP-CAN session established to an Emergency APN.

UNAUTHORIZED_SPONSORED_DATA_CONNECTIVITY (5067)

The PCRF rejects a new Rx session setup because the PCRF can’t authorize the sponsored data connectivity based on the sponsored data connectivity profile or the operator policy (e.g. sponsored data connectivity not authorised in the roaming case).

## 5.6	Rx messages

Existing Diameter command codes from the Diameter base protocol RFC 3588 [10] and the NASREQ Diameter application (RFC 4005 [12]) are used with the Rx specific AVPs. An Rx specific Auth-Application id is used together with the command code to identify the Rx messages.

NOTE 1:	The notion of NAS (Network Access Server) is not used here, NASREQ is just used for protocol purposes, not for its functional meaning.

NOTE 2:	Some of the AVPs included in the messages formats below are in bold to highlight that these AVPs are used by this specific protocol and do not belong to the original Diameter Base Protocol RFC 3588 [10].

### 5.6.1	AA-Request (AAR) command

The AAR command, indicated by the Command-Code field set to 265 and the 'R' bit set in the Command Flags field, is sent by an AF to the PCRF in order to provide it with the Session Information.

Message Format:

<AA-Request> ::= < Diameter Header: 265, REQ, PXY >

< Session-Id >

{ Auth-Application-Id }

{ Origin-Host }

{ Origin-Realm }

{ Destination-Realm }

[ Destination-Host ]

[ AF-Application-Identifier ]

*[ Media-Component-Description ]

[ Service-Info-Status ]

[ AF-Charging-Identifier ]

[ SIP-Forking-Indication ]

*[ Specific-Action ]

*[ Subscription-Id ]

*[ Supported-Features ]

[ Reservation-Priority ]

[ Framed-IP-Address ]

[ Framed-IPv6-Prefix ]

[ Called-Station-Id ]

[ Service-URN ]

[ Sponsored-Connectivity-Data ]

[ MPS-Identifier ]

[ Origin-State-Id ]

*[ Proxy-Info ]

*[ Route-Record ]

*[ AVP ]

### 5.6.2	AA-Answer (AAA) command

The AAA command, indicated by the Command-Code field set to 265 and the 'R' bit cleared in the Command Flags field, is sent by the PCRF to the AF in response to the AAR command.

Message Format:

<AA-Answer> ::=  < Diameter Header: 265, PXY >

< Session-Id >

{ Auth-Application-Id }

{ Origin-Host }

{ Origin-Realm }

[ Result-Code ]

[ Experimental-Result ]

*[ Access-Network-Charging-Identifier ]

[ Access-Network-Charging-Address ]

[ Acceptable-Service-Info ]

[ IP-CAN-Type ]

[ RAT-Type ]

*[ Flows ]

*[ Supported-Features ]

*[ Class ]

[ Error-Message ]

[ Error-Reporting-Host ]

*[ Failed-AVP ]

[ Origin-State-Id ]

*[ Redirect-Host ]

[ Redirect-Host-Usage ]

[ Redirect-Max-Cache-Time ]

*[ Proxy-Info ]

*[ AVP ]

### 5.6.3	Re-Auth-Request (RAR) command

The RAR command, indicated by the Command-Code field set to 258 and the 'R' bit set in the Command Flags field, is sent by the PCRF to the AF in order to indicate an Rx specific action.

Message Format:

<RA-Request> ::= < Diameter Header: 258, REQ, PXY >

< Session-Id >

{ Origin-Host }

{ Origin-Realm }

{ Destination-Realm }

{ Destination-Host }

{ Auth-Application-Id }

{ Specific-Action }

*[ Access-Network-Charging-Identifier ]

[ Access-Network-Charging-Address ]

*[ Flows ]

*[ Subscription-Id ]

[ Abort-Cause ]

[ IP-CAN-Type ]

[ RAT-Type ]

[ Sponsored-Connectivity-Data ]

[ Origin-State-Id ]

*[ Class ]

*[ Proxy-Info ]

*[ Route-Record ]

*[ AVP ]

### 5.6.4	Re-Auth-Answer (RAA) command

The RAA command, indicated by the Command-Code field set to 258 and the 'R' bit cleared in the Command Flags field, is sent by the AF to the PCRF in response to the RAR command.

Message Format:

<RA-Answer> ::=  < Diameter Header: 258, PXY >

< Session-Id >

{ Origin-Host }

{ Origin-Realm }

[ Result-Code ]

[ Experimental-Result ]

*[ Media-Component-Description ]

[ Service-URN ]

[ Origin-State-Id ]

*[ Class ]

[ Error-Message ]

[ Error-Reporting-Host ]

*[ Redirect-Host ]

[ Redirect-Host-Usage ]

[ Redirect-Max-Cache-Time ]

*[ Failed-AVP ]

*[ Proxy-Info ]

*[ AVP ]

### 5.6.5	Session-Termination-Request (STR) command

The STR command, indicated by the Command-Code field set to 275 and the 'R' bit set in the Command Flags field, is sent by the AF to inform the PCRF that an established session shall be terminated.

Message Format:

<ST-Request> ::= < Diameter Header: 275, REQ, PXY >

< Session-Id >

{ Origin-Host }

{ Origin-Realm }

{ Destination-Realm }

{ Auth-Application-Id }

{ Termination-Cause }

[ Destination-Host ]

*[ Class ]

[ Origin-State-Id ]

*[ Proxy-Info ]

*[ Route-Record ]

*[ AVP ]

### 5.6.6	Session-Termination-Answer (STA) command

The STA command, indicated by the Command-Code field set to 275 and the 'R' bit cleared in the Command Flags field, is sent by the PCRF to the AF in response to the STR command.

Message Format:

<ST-Answer>  ::= < Diameter Header: 275, PXY >

< Session-Id >

{ Origin-Host }

{ Origin-Realm }

[ Result-Code ]

[ Error-Message ]

[ Error-Reporting-Host ]

* [ Failed-AVP ]

[ Sponsored-Connectivity-Data ]

[ Origin-State-Id ]

*[ Class ]

*[ Redirect-Host ]

[ Redirect-Host-Usage ]

[ Redirect-Max-Cache-Time ]

*[ Proxy-Info ]

*[ AVP ]

### 5.6.7	Abort-Session-Request (ASR) command

The ASR command, indicated by the Command-Code field set to 274 and the 'R' bit set in the Command Flags field, is sent by the PCRF to inform the AF that bearer for the established session is no longer available.

Message Format:

<AS-Request>  ::= < Diameter Header: 274, REQ, PXY >

< Session-Id >

{ Origin-Host }

{ Origin-Realm }

{ Destination-Realm }

{ Destination-Host }

{ Auth-Application-Id }

{ Abort-Cause }

[ Origin-State-Id ]

*[ Proxy-Info ]

*[ Route-Record ]

*[ AVP ]

### 5.6.8	Abort-Session-Answer (ASA) command

The ASA command, indicated by the Command-Code field set to 274 and the 'R' bit cleared in the Command Flags field, is sent by the AF to the PCRF in response to the ASR command.

Message Format:

<AS-Answer>  ::=  < Diameter Header: 274, PXY >

< Session-Id >

{ Origin-Host }

{ Origin-Realm }

[ Result-Code ]

[ Origin-State-Id ]

[ Error-Message ]

[ Error-Reporting-Host ]

*[ Failed-AVP ]

*[ Redirect-Host ]

[ Redirect-Host-Usage ]

[ Redirect-Max-Cache-Time ]

*[ Proxy-Info ]

*[ AVP ]

Annex A (normative):
IMS Related P-CSCF Procedures over Rx

# A.1	Provision of Service Information at P-CSCF

The P-CSCF shall send service information to the PCRF upon every SIP message that includes an SDP answer payload for the purpose of authorizing the IP flows and the QoS resources required for a negotiated IMS session, unless the SDP payload only relates to a circuit-switched bearer (i.e. "c=" line set to "PSTN" and an "m=" line set to "PSTN", refer to 3GPP TS 24.292 [26]). The service information shall be derived both from the SDP offer and the SDP answer. This ensures that the PCRF receives proper information to perform media authorization for all possible IMS session set-up scenarios, and that the PCRF is also capable of handling session modifications. The P-CSCF may include the Service-Info-Status AVP with the value set to FINAL_SERVICE_INFORMATION.

Additionally, the P-CSCF may send service information to the PCRF when receiving a SIP message that includes an SDP offer payload for the purpose of performing an early bandwidth authorization check or for enabling pre-authorization for a UE terminated IMS session establishment or modification with UE initiated resource reservation. The P-CSCF shall send service information to the PCRF when receiving a SIP message that includes an SDP offer payload when the IMS session is an MPS session that requires priority treatment. For a UE terminated session the P-CSCF may send the service information derived from the SDP offer when the SDP offer either does not include any preconditions information or includes preconditions information indicating that the local preconditions (i.e. the preconditions related to the remote peer) are already met. In this case, the P-CSCF shall derive the service information only from the SDP offer and shall include the Service-Info-Status AVP with the value set to PRELIMINARY SERVICE INFORMATION.

NOTE: 	For a UE terminated session setup, when the SDP offer either does not include any preconditions information or includes preconditions information indicating that the local preconditions (i.e. the preconditions related to the remote peer) are already met, the terminating UE can request a resource modification prior to sending the SDP answer. Even if the IP address and port information in the session information derived from the SDP offer can be insufficient for PCC rule authorization, the policy to handle such UE initiated requests at the PCRF can take into account the fact that an IMS session establishment is ongoing, for instance in deciding whether to authorize the request and in selecting an appropriate charging key and a gating policy.

The P-CSCF shall derive Flow-Description AVP within the service information from the SDP as follows:

-	An uplink Flow-Description AVP shall be formed as follows: The destination address shall be taken from the SDP information received by the P-CSCF in downlink direction, while the source IP address may be formed from the address present in the SDP received by the P-CSCF in uplink direction (taking into account only the 64 bit prefix of the IPv6 address) Source and destination ports shall be derived according to rules provided in 3GPP TS 29.213 [9] clause 6.2.

EXAMPLE 1:	Assuming UE A sends an SDP to UE B, the PCRF of UE B uses the address present in this SDP for the destination address of UE B's uplink Flow-Description AVP, while the PCRF of the UE A uses the 64 bit prefix of the same address for the source address of UE A's uplink Flow-Description AVP. If the source address is not formed from the 64 bit prefix, the source address shall be wildcarded.

-	A downlink Flow-Description AVP shall be formed as follows: The destination address shall be taken from the SDP information received by the P-CSCF in uplink direction, while the source IP address may be formed (in order to reduce the possibilities of bearer misuse) from the destination address in the SDP received by the P-CSCF in downlink direction (taking into account only the 64 bit prefix of the IPv6 address) Source and destination ports shall be derived according to rules provided in 3GPP TS 29.213 [9] clause 6.2.

EXAMPLE 2:	Assuming UE A sends an SDP to UE B, the PCRF of UE A uses the address present in this SDP for the destination address of UE A's downlink Flow-Description AVP, while the PCRF of UE B uses the 64 bit prefix of the same address for the source address of UE B's downlink Flow-Description AVP. If the source address is not formed from the 64 bit prefix, the source address shall be wildcarded.

The P-CSCF shall derive the bandwidth information within the service information, from the "b=AS" SDP parameter, as detailed in 3GPP TS 29.213 [9] clause 6.2. For the possibly associated RTCP IP flows, the P-CSCF shall use the SDP "b=RR" and "b=RS" parameters, if present, as specified in 3GPP TS 29.213 [9] clause 6.2. The "b=AS", "b=RR" and "b=RS" parameters in the SDP contain all the overhead coming from the IP-layer and the layers above, e.g. IP, UDP, RTP and RTCP payload, or IP, UDP and RTCP.

However, if service information is received containing the "b=TIAS" SDP parameter that corresponds to an SDP answer payload, and if the P-CSCF supports this parameter, the P-CSCF may derive the bandwidth from this parameter rather than from the "b=AS" SDP parameter, as detailed in 3GPP TS 29.213 [9] clause 6.2.

When available, the P-CSCF shall also indicate to PCRF, as a complement to the Service Information, the IMS Communication Service Identifier within the AF-Application-Identifier AVP. The P-CSCF shall take the IMS Communication Service Identifier value from the SIP response for which the corresponding SIP request included the same IMS Communication Service Identifier value. Otherwise, the P-CSCF may not be able to provide an IMS Communication Service Identifier value to the PCRF. The format and specific headers where IMS communication service identifiers are transported within SIP are defined in 3GPP TS 24.229 [17].

If the Service-URN AVP does not include an emergency service URN, i.e. a top-level service type of "sos" as specified in IETF RFC 5031 [21] and possibly additional sub-service information on the type of the emergency service and the PCRF binds the IMS service session to an IP-CAN session established to an Emergency APN, the PCRF shall return an AAA command with Experimental-Result-Code AVP set to the value UNAUTHORIZED_NON_EMERGENCY_SESSION to the P-CSCF. Upon receiving an AAA with Experimental-Result-Code AVP set to the value UNAUTHORIZED_NON_EMERGENCY_SESSION the P-CSCF shall apply the procedures defined in 3GPP TS 24.229 [17].

NOTE: 	The PCRF determines whether an IP-CAN session is established to an Emergency APN based on the information received over Gx and operator configuration.

The PCRF may decide not to authorize requested service information. The PCRF will indicate it to the P-CSCF by sending an AA-Answer with Experimental-Result-Code AVP set to the value REQUESTED_SERVICE_NOT_AUTHORIZED. Upon receiving an AA-Answer with Experimental-Result-Code AVP set to the value REQUESTED_SERVICE_NOT_AUTHORIZED the P-CSCF shall apply the procedures defined in 3GPP TS 24.229 [17].

# A.2	Enabling of IP Flows

Prior to the completion of the SIP session set-up, i.e. until the 200 OK (INVITE) or any other 2xx response is received, the P-CSCF may enable or disable media IP flows depending on operator policy, thus allowing or forbidding early media in forward and/or backward direction. Only to disable early media, the P-CSCF may modify the values of the Flow-Status AVPs derived from SDP according to 3GPP TS 29.213 [9] clause 6.2. If the P-CSCF chooses to modify the values, the P-CSCF shall store the last received SDP.

When a 2xx response is received, the P-CSCF shall enable all media IP flows according to the direction attribute within the last received SDP, as specified in 3GPP TS 29.213 [9] clause 6.2. When a 2xx response is received and the P-CSCF previously provided modified values of the Flow-Status AVPs in the session information, the P-CSCF shall provide service information with values of the Flow-Status AVPs corresponding to the last received SDP.

NOTE:	In most cases a 2xx response is a 200 OK.

If the P-CSCF receives SDP answers after the completion of the SIP session set-up, i.e. after the 200 OK (INVITE) or any other 2xx response is received, the P-CSCF shall provide the Flow-Status AVP, based on the last received SDP answer. The Flow-Status AVP as derived from the SDP according to 3GPP TS 29.213 [9] clause 6.2.

# A.3	Support for SIP forking

The P-CSCF shall be able to handle forking when PCC is applied. Forking can occur as specified in 3GPP TS 23.228 [16]. The related UE procedures are described in 3GPP TS 24.229 [17].

## A.3.1	PCC rule provisioning for early media for forked responses

When a SIP session has been originated by a connected UE, the P-CSCF may receive multiple provisional responses due to forking before the first final answer is received. Multiple early media session may be established during this process.

The UE and the P-CSCF become aware of the forking only when a subsequent provisional response arrives for a new early dialogue. After the first early media session is established, for each subsequent provisional response establishing an additional early media session,, the P-CSCF shall use an AA request within the existing Diameter session containing the SIP-Forking-Indication AVP with value SEVERAL_DIALOGUES and include the service information derived from the latest provisional response.

The P-CSCF shall also provision the service information derived from any subsequent SDP offer-answer exchange within an early dialogue (e.g. in PRACK and OK(PRACK), or UPDATE and OK(UPDATE) ) using an AA request within the existing Diameter session containing the SIP-Forking-Indication AVP with value SEVERAL_DIALOGUES and the derived service information.

When receiving an AA request containing the SIP-Forking-Indication AVP with value SEVERAL_DIALOGUES, the PCRF shall identify the existing authorization information for that AF session. The PCRF shall send additional PCC Rules or individual service data flow filters to already provide PCC rules as required by the Flow Description AVPs within the session information to the PCEF. The PCRF shall authorize any additional media components and any increased QoS requirements for the previously authorized media components, as requested within the service information. The PCRF shall authorize the maximum bandwidth required by any of the dialogues, but not the sum of the bandwidths required by all dialogues. Thus, the QoS authorized for a media component is equal to the highest QoS requested for that media component by any of the forked responses. The PCRF shall open or close the gates for service flows depending on the flow status that is being provisioned. However, if a flow ID has been enabled in uplink or downlink direction or both way within previous service information, it shall remain enabled even if the PCRF receives service information that disable this flow ID within an AA request containing the SIP-Forking-Indication AVP with value SEVERAL_DIALOGUES.

## A.3.2	Updating the provisioned PCC rules at the final answer

The P-CSCF shall store the SDP information for each early dialogue separately till the first final SIP answer is received. Then the related early dialogue is progressed to an established dialogue to establish the final SIP session. All the other early dialogues are terminated. The service information for the SIP session is updated to match the requirements of the remaining early dialogue only.

When receiving the first final SIP response, the P-CSCF shall send an AA request without the SIP-Forking-Indication AVP and include the service information derived from the SDP corresponding to the dialogue of the final response. The P-CSCF shall provision the full service information including the applicable Flow-Description AVP(s) and Flow-Status AVP(s).

When receiving an AA request with no SIP-Forking-Indication AVP or with a SIP-Forking-Indication AVP with value SINGLE_DIALOGUE, the PCRF shall update installed PCC Rules information and Authorized-QoS information to match only the requirements of the service information within this AA request. The PCRF should immediately remove PCC Rule(s) or individual service data flow filters not matching IP flow(s) in the updated Service Information, to reduce the risk for initial clipping of the media stream, and to minimize possible misuse of resources. The PCRF shall also open or close the gates for service flows according to the flow status in the received service information.

# A.4	Notification of AF Signalling Transmission Path Status

When the P-CSCF receives an initial REGISTER SIP message from an attached UE, the P-CSCF may subscribe to notifications of the status of the AF Signalling transmission path using the procedures specified in clause 4.4.5. Once the P-CSCF has subscribed, the P-CSCF may receive notifications from the PCRF according to clause 4.4.6.3.

NOTE: This procedure is not applicable for IMS registrations for Emergency sessions.

The P-CSCF shall cancel the subscription to notification of the status of the AF Signalling transmission path when the AF Signalling to that particular user is terminated (i.e. when the user is de-REGISTERED from the IM CN subsystem).

When the P-CSCF receives a notification of loss of signalling connectivity from the PCRF, the P-CSCF shall behave as defined in 3GPP TS 24.229 [17].

# A.5	Indication of Emergency Session

A SIP INVITE request can contain a service URN as defined in IETF RFC 5031 [21] within the request URI. If the service within this URN is "sos", possibly with additional sub-service information, the P-CSCF shall provision this service and sub-service information within the Service-URN AVP towards the PCRF. The P-CSCF may also provision possible information about other services received within the service URN.

# A.6	Notification IP-CAN Type Change

When the P-CSCF receives an initial REGISTER SIP message from an attached UE, the P-CSCF mayrequest from the PCRF the information about the type of IP-CAN the UE is attached to using the procedure specified in subclause 4.4.1.

NOTE: This procedure is not applicable for IMS registrations for Emergency sessions.

If the P-CSCF requests information about the type of IP-CAN, the P-CSCF shall also subscribe within the same AAR command operation to notifications for changes of the IP-CAN type used by the UE. The P-CSCF shall include a Specific-Action AVP in the AAR that is set to the value of IP-CAN_CHANGE.

When the P-CSCF receives the AA-Answer or RA-Request from the PCRF, the P-CSCF stores the IP-CAN type information and the RAT type information (if also provided by the PCRF) and behaves as defined in 3GPP TS 24.229 [17].

The P-CSCF may receive notifications for changes of the IP-CAN type from the PCRF according to clause 4.4.6.4. When the P-CSCF receives a notification of the change of the IP-CAN used by the UE, the P-CSCF stores the new IP-CAN type information and behaves as defined in 3GPP TS 24.229 [17].

The P-CSCF shall cancel the subscription to notification for changes of the IP-CAN type used by the UE when the user is de-REGISTERED from the IM CN subsystem.

# A.7	Support for Early Session disposition SDP

## A.7.1	General

As a network option, the P-CSCF may support the PCC procedures in the present Clause to handle "early session" disposition type SDP, as standardised in IETF RFC 3959 [27].

## A.7.2	Service Information Provisioning for Early Media

The P-CSCF can receive "early session" disposition SDP in addition to "session" disposition SDP in SIP early dialogues.

The P-CSCF shall then provision service information derived both from the "early session" disposition SDP and "session" disposition SDP applying the procedures in Clauses A.1, A.2, and A.3, and in the present Clause.

The P-CSCF shall apply the mapping rules in Annex B.1.2.2.2 to derive the flow identifiers from "early session" disposition SDP.

If a single media line with one media type (e.g. "audio" or "video") is contained in "early session" disposition SDP and a single media line with the same media type is contained in the "session" disposition SDP of the same SIP dialogue, and both media lines describe service flows of the same directionality (uplink, downlink, or bidirectional), the P-CSCF should describe those SDP media lines in the same session information media component (with the same flow ID).

The "early session" disposition SDP can also contain media lines of a type not included in the "session" disposition SDP, or several media lines of the same type. Such media components shall be described in own media components in the service information.

If the P-CSCF desires to invoke special policies or separate event notifications for an "early session" disposition media line, it may choose to provision a separate session information media component even if a media line with the same media type and directionality is contained in "session" disposition SDP.

NOTE 1:	A PCRF is then likely to supply separate PCC rules for early media and the corresponding final media. This may lead to an over provisioning of resources during call establishment and a subsequent reconfiguration of the radio bearer, or even to a call failure if the extra resources are not authorized or available.

If the P-CSCF receives "early session" disposition SDP before any "session" disposition SDP and supplies service information derived from the "early session" disposition SDP at this point of time, it shall use dedicated media components relating only to the "early session" disposition SDP in the service information.

NOTE 2: 	The "session" disposition SDP offer will frequently occur before the "early session" disposition SDP offer, but can also occur in parallel or in exceptional cases afterwards. The "session" disposition SDP answer can be contained in the same SIP message as the "early session" disposition SDP offer, or can be sent in a 200 OK (INVITE), i.e. after the "early session" disposition SDP answer.

If the P-CSCF includes any media component relating both to "early session" disposition SDP and "session" disposition SDP in the service information, the P-CSCF shall:

-	provision the service information derived from "early session" disposition SDP and the service information derived from "session" disposition SDP in separate Rx AA-Requests, and shall send a new Rx AA-Request only after any previous Rx AA-Request has been acknowledged; and

-	provision the first service information (either derived from "early session" disposition SDP or "session" disposition SDP) without the SIP-Forking-Indication AVP, or with the SIP-Forking-Indication AVP with value SINGLE_DIALOGUE; and

-	provision all subsequent service information during ongoing call establishment with the SIP-Forking-Indication AVP with value SEVERAL_DIALOGUES; and

-	if an SDP answer has been received and Codec-Data AVPs are provisioned within a Media-Component-Description AVP, provision a Codec-Data AVP derived from the corresponding offer together with a Codec-Data AVP derived from the SDP answer.

NOTE 3: 	The P-CSCF needs to provision the service information derived from "early session" disposition SDP and the service information derived from "session" disposition SDP in separate Rx AA-Requests because the encoding of the media-component does not allow for the simultaneous provisioning of two corresponding filters.

NOTE 4: 	The PCRF will treat service information containing the SIP-Forking-Indication AVP as described in Clause A.3.

## A.7.3	Updating the Provisioned Service Information when Dialogue is established

The P-CSCF shall store the SDP information for the "session" disposition type until the first final SIP answer is received. Then the early media described in the "early session" disposition type SDP are terminated.

The P-CSCF shall then update the service information to match the requirements of the media described in the "session" disposition type SDP only:

-	If the P-CSCF included any media component relating both to "early session" disposition SDP and "session" disposition SDP in the service information, the P-CSCF shall send an AA request without the SIP-Forking-Indication AVP or with a SIP-Forking-Indication AVP with value SINGLE_DIALOGUE and shall include the service information derived from the "session" disposition SDP. The P-CSCF shall provision the full service information including the applicable Flow-Description AVP(s) and Flow-Status AVP(s).

-	The P-CSCF shall disable any media component(s) in the service information that relate to early media only by setting their flow status to "removed".

## A.8	Provision of Signalling Flow Information at P-CSCF

When the P-CSCF has successfully concluded the initial registration of an attached UE, i.e., when the P-CSCF has sent to the UE a SIP 200 (OK) response to the SIP REGISTER request, the P-CSCF may provision information about the SIP signalling flows between the UE and itself using the procedure specified in subclause 4.4.5a. If the P-CSCF already has an open Rx Diameter session with the PCRF related to the signalling with the UE, e.g. one that has been opened according to the procedure described in subclause A.6 and/or in subclause A.4, the P-CSCF shall reuse the already open session to provision the SIP Signalling IP Flow information.

NOTE: This procedure is not applicable for IMS registrations for Emergency sessions.

If the P-CSCF provisions information about the SIP signalling flows,the P-CSCF shall ensure that for each signalling IP flow information it provides, the Flow-Description AVP shall accurately reflect the IP flow information as seen in the IP header ‘on the wire’. The P-CSCF shall set the value of the AF-Signalling-Protocol AVP to 'SIP'.

When the P-CSCF de-registers the UE and terminates SIP Signalling to the UE, the P-CSCF shall de-provision the SIP Signalling IP flow information from the PCRF as described in subclause 4.4.5a.

# A.9	Handling of MPS Session

When the P-CSCF receives an authorised Resource-Priority header field or a temporarily authorised Resource-Priority header field containing an appropriate namespace and priority value in SIP signaling, and recognizes the need for priority treatment as specified in 3GPP TS 24.229 [17], the P-CSCF shall include the MPS-Identifier AVP and Reservation-Priority AVP in the AAR command towards the PCRF.  The MPS-Identifier AVP shall contain the national variant for MPS service name indicating an MPS session.  The Reservation-Priority AVP shall include the priority value of the related priority service. The Reservation-Priority AVP shall be populated with a default value if the priority value is unknown.

NOTE 1: 	Various mechanisms can be applied to recognize the need for priority treatment in the P-CSCF (e.g., based on the dialled digits), according to national regulation and network configuration, as stated in 3GPP TS 24.229 [17].

NOTE 2:	Lowest user priority value shall be mapped in the corresponding lowest enumerated value of the Reservation-Priority AVP.

Upon reception of a request that requires MPS treatment, the PCRF shall derive the PCC/QoS Rules corresponding to the MPS session, as appropriate. The PCRF shall take specific actions on the corresponding IP-CAN to ensure that the MPS session is prioritized, as described in 3GPP TS 29.212 [8], clause 4.5.19.1.3.

When the P-CSCF terminates the MPS session, the PCRF shall delete the PCC/QoS Rules corresponding to the MPS session. The PCRF shall revoke the actions related to the prioritization of the MPS session in the corresponding IP-CAN, as described in 3GPP TS 29.212 [8], subclause 4.5.19.1.3.

Annex B (normative):
Flow identifiers: Format definition and examples

# B.1	Format of a flow identifier

## B.1.1	General

A flow identifier is expressed as a 2-tuple as follows:

<The ordinal number of the position of the media component description in the SDI , The ordinal number of the IP flow(s) within the media component description assigned in the order of increasing downlink port numbers as detailed below >

where both are numbered starting from 1. The encoding of the flow identifier is as indicated in 3GPP TS 24.008 [12].

If UE and AF share an algorithm for a given application, which guarantees that UE and AF assign the same ordinal number to each media component, the ordinal numbers of the IP Flows within a media component shall be assigned according to the following rules:

-	All IP flow(s) or bidirectional combinations of two IP flow(s) within the media component, for which a downlink destination port number is available, shall be assigned ordinal numbers in the order of downlink destination port numbers.

-	All IP flows, where no downlink destination port number is available, shall be assigned the next higher ordinal numbers in the order of uplink destination port numbers.

The ordinal number of a media component shall not be changed when the session description information is modified.

If no SDI with fixed and unique positions for media components is exchanged between UE and AF, the UE and AF may assign the ordinal numbers of the media components in another application-dependent algorithm which guarantees that UE and AF assign the same ordinal number to each media component.

If UE and AF do not share an algorithm for a given application, which guarantees that UE and AF assign the same ordinal number to each media component, the ordinal number of the media component shall be set to zero and the ordinal number of the IP flows shall be assigned according to the following rules:

1.	If ordinal numbers for several IP flows are assigned at the same time, all uplink IP flows shall be assigned lower ordinal number than all downlink IP flows.

2.	If ordinal numbers for several IP flows are assigned at the same time, all uplink and all downlink IP flows shall separately be assigned ordinal numbers according to increasing internet protocol number assigned by IANA (e.g. 6 for TCP and 17 for UDP)

3.	If ordinal numbers for several IP flows are assigned at the same time, for each internet protocol with a port concept, all uplink and all downlink IP flows of this internet protocol shall separately be assigned ordinal numbers according to increasing port numbers.

4.	If IP flows are removed from an existing session, the previously assigned binding info shall remain unmodified for the remaining IP flows.

5.	If IP flows are added to an existing session, the previously assigned binding info shall remain unmodified and the new IP flows shall be assigned ordinal numbers following the rules 1. to 3., starting with the first previously unused ordinal number. The numbers freed in step 4. shall not be reused.

If the IP flows correspond to AF signalling (e.g. SIP signalling IP Flows), and no IP flow information is provided both the ordinal number of the media component and the IP flows shall be set to zero. In the case when the IP flow information of the AF signalling is provided, the ordinal number of the media component will be set to zero, and the ordinal number(s) of the IP flows shall follow the rules set above for the case when the UE and AF do not share an algorithm for a given application.

## B.1.2	Derivation of Flow Identifiers from SDP

### B.1.2.1	Standard Procedure

The procedure in Table B.1.2.1.1 shall be applied for SDP of "session" disposition type within SIP and for all SDP outside SIP.

Table B.1.2.1.1: Standard Procedure to derive Flow Identifiers from SDP

| The ordinal number of the position of the media component | The ordinal number of the IP flow(s) within the media component description |
| --- | --- |
| The ordinal number of the position of the "m=" line in the SDP | The ordinal number of the IP flow(s) within the "m=" line assigned in the order of increasing downlink destination port numbers, if downlink destination port numbers are available. For uplink or inactive unicast media IP flows, a downlink destination port number is nevertheless available, if SDP offer-answer according to RFC 3264 is used. The ordinal number of the IP flow(s) within the "m=" line assigned in the order of increasing uplink destination port numbers, if no downlink destination port numbers are available. |

### B.1.2.2	SDP with "early session" disposition type

The procedure in Table B.1.2.2.1 shall be applied for SDP of "early session" disposition type within SIP. The "early session" disposition type is specified within IETF RFC 3959 [27].

Table B.1.2.2.1: Procedure to derive Flow Identifiers from "early session" disposition SDP

| The ordinal number of the position of the media component | The ordinal number of the IP flow(s) within the media component description |
| --- | --- |
| If a single media component with one media type (e.g. "audio" or "video") is contained in "early session" disposition SDP and a single media component with the same media type is contained in the "session" disposition SDP of the same SIP dialogue, and if both SDP media components are described in a single Rx service information media component, that Rx service information media component shall obtain the media component number derived from "session" disposition SDP following the rules in table B.1.2.1.1. Otherwise each media component shall obtain the ordinal number of the position of the "m=" line in the "early session" disposition SDP plus 1000 as media component ordinal number. | As in table B.1.2.1.1. |

# B.2	Example 1

An UE, as the offerer, sends a SDP session description, as shown in table B.2.1, to an application server (only relevant SDP parameters are shown):

Table B.2.1: The values of the SDP parameters sent by the UE in example 1.

v=0 o=ecsreid 3262464865 3262464868 IN IP6 2001:0646:00F1:0045:02D0:59FF:FE14:F33A s=MM01 i=One unidirectional audio media and one unidirectional video media and one bidirectional application media t=3262377600 3262809600 m=video 50230 RTP/AVP 31 c=IN IP6 2001:0646:00F1:0045:02D0:59FF:FE14:F33A a=recvonly m=audio 50330 RTP/AVP 0 c=IN IP6 2001:0646:00F1:0045:02D0:59FF:FE14:F33A a=sendonly m=application 50430 udp wb c=IN IP6 2001:0646:00F1:0045:02D0:59FF:FE14:F33A a=sendrecv

and receives the SDP parameters, as shown in table B.2.2, from the application server:

Table B.2.2: The values of the SDP parameters sent by the application server in example 1.

v=0 o=ecsreid 3262464865 3262464868 IN IP6 2001:0646:00F1:0045:02D0:59FF:FE14:F33A s=MM01 i=One unidirectional audio media and one unidirectional video media and one bidirectional application media t=3262377600 3262809600 m=video 51372 RTP/AVP 31 c=IN IP6 2001:0646:000A:03A7:02D0:59FF:FE40:2014 a=sendonly m=audio 49170 RTP/AVP 0 c=IN IP6 2001:0646:000A:03A7:02D0:59FF:FE40:2014 a=recvonly m=application 32416 udp wb c=IN IP6 2001:0646:000A:03A7:0250:DAFF:FE0E:C6F2 a=sendrecv

From this offer–answer exchange of SDP parameters the UE and the PCRF each creates a list of flow identifiers comprising the IP flows as shown in table B.2.3:

Table B.2.3: Flow identifiers in example 1.

| Order of 'm='-line | Type of IP flows | Destination IP address / Port number of the IP flows | Flow identifier |
| --- | --- | --- | --- |
| 1 | RTP (Video) DL | 2001:0646:00F1:0045:02D0:59FF:FE14:F33A / 50230 | <1,1> |
| 1 | RTCP DL | 2001:0646:00F1:0045:02D0:59FF:FE14:F33A / 50231 | <1,2> |
| 1 | RTCP UL | 2001:0646:000A:03A7:02D0:59FF:FE40:2014 / 51373 | <1,2> |
| 2 | RTP (Audio) UL | 2001:0646:000A:03A7:02D0:59FF:FE40:2014 / 49170 | <2,1> |
| 2 | RTCP DL | 2001:0646:00F1:0045:02D0:59FF:FE14:F33A / 50331 | <2,2> |
| 2 | RTCP UL | 2001:0646:000A:03A7:02D0:59FF:FE40:2014 / 49171 | <2,2> |
| 3 | UDP (application) DL | 2001:0646:00F1:0045:02D0:59FF:FE14:F33A / 50430 | <3,1> |
| 3 | UDP (application) UL | 2001:0646:000A:03A7:0250:DAFF:FE0E:C6F2 / 32416 | <3,1> |

# B.3	Example 2

In the general case, multiple ports may be specified with a "number of ports" qualifier as follows, RFC 2327 [17]:

m=<media> <port>/<number of ports> <transport> <fmt list>

An UE, as the offerer, sends a SDP session description, as shown in table B.3.1, to an application server (only relevant SDP parameters are shown):

Table B.3.1: The values of the SDP parameters sent by the UE in example 2.

v=0 o=ecsreid 3262464321 3262464325 IN IP6 2001:0646:00F1:0045:02D0:59FF:FE14:F33A s=MM02 i=One unidirectional audio media consisting of two media IP flows described by one media component t=3262377600 3262809600 m=audio 50330/2 RTP/AVP 0 c=IN IP6 2001:0646:00F1:0045:02D0:59FF:FE14:F33A a=recvonly

and receives the SDP parameters, as shown in table B.3.2, from the application server:

Table B.3.2: The values of the SDP parameters sent by the application server in example 2.

v=0 o=ecsreid 3262464321 3262464325 IN IP6 2001:0646:00F1:0045:02D0:59FF:FE14:F33A s=MM02 i=One unidirectional audio media consisting of two media IP flows described by one media component t=3262377600 3262809600 m=audio 49170/2 RTP/AVP 0 c=IN IP6 2001:0646:000A:03A7:02D0:59FF:FE40:2014 a=sendonly

From this offer–answer exchange of SDP parameters the UE and the PCRF each creates a list of flow identifiers comprising the IP flows as shown in table B.3.3:

Table B.3.3: Flow identifiers in example 2.

| Order of 'm='-line | Type of IP flows | Destination IP address / Port number of the IP flows | Flow identifier |
| --- | --- | --- | --- |
| 1 | RTP (audio) DL | 2001:0646:00F1:0045:02D0:59FF:FE14:F33A / 50330 | <1,1> |
| 1 | RTCP DL | 2001:0646:00F1:0045:02D0:59FF:FE14:F33A / 50331 | <1,2> |
| 1 | RTCP UL | 2001:0646:000A:03A7:02D0:59FF:FE40:2014 / 49171 | <1,2> |
| 1 | RTP (audio) DL | 2001:0646:00F1:0045:02D0:59FF:FE14:F33A / 50332 | <1,3> |
| 1 | RTCP DL | 2001:0646:00F1:0045:02D0:59FF:FE14:F33A / 50333 | <1,4> |
| 1 | RTCP UL | 2001:0646:000A:03A7:02D0:59FF:FE40:2014 / 49173 | <1,4> |

# B.4	Example 3 without media components.

The UE and AF do not exchange SDP for an application and do not share an algorithm, which guarantees that UE and AF assign the same ordinal number to each media component.

At the AF session initiation, the UE and AF agree to set up the following IP flows:

-	Uplink UDP flow with destination port 100.

-	Downlink UDP flow with destination port 100.

-	Downlink TCP flow with destination port 100.

-	Uplink TCP flow with destination port 100.

-	Uplink UDP flow with destination port 200.

The following binding info is assigned to these IP flows.

-	Uplink UDP flow with destination port 100:	(0, 2)

-	Downlink UDP flow with destination port 100: 	(0, 5)

-	Downlink TCP flow with destination port 100: 	(0, 4)

-	Uplink TCP flow with destination port 100: 	(0, 1)

-	Uplink UDP flow with destination port 200: 	(0, 3)

At a later stage in the session, the TCP IP flows are removed and the following IP flows are added:

-	Uplink UDP flow with destination port 150.

-	Downlink UDP flow with destination port 50.

The following binding info is assigned to the IP flows existing at this stage:

-	Uplink UDP flow with destination port 100:	(0, 2)

-	Downlink UDP flow with destination port 100: 	(0, 5)

-	Uplink UDP flow with destination port 200: 	(0, 3)

-	Uplink UDP flow with destination port 150: 	(0, 6)

-	Downlink UDP flow with destination port 50: 	(0, 7)

# B.5	Example 4

In this example, the SDP "a=rtcp" attribute defined in IETF RFC 3605 is used.

An UE, as the offerer, sends a SDP session description, as shown in table B.5.1, to an application server (only relevant SDP parameters are shown):

Table B.5.1: The values of the SDP parameters sent by the UE in example 1.

v=0 o=ecsreid 3262464865 3262464868 IN IP6 2001:0646:00F1:0045:02D0:59FF:FE14:F33A s=MM01 i=One unidirectional video media t=3262377600 3262809600 m=video 50230 RTP/AVP 31 c=IN IP6 2001:0646:00F1:0045:02D0:59FF:FE14:F33A a=recvonly a=rtcp: 49320

and receives the SDP parameters, as shown in table B.5.2, from the application server:

Table C.5.2: The values of the SDP parameters sent by the application server in example 1.

v=0 o=ecsreid 3262464865 3262464868 IN IP6 2001:0646:00F1:0045:02D0:59FF:FE14:F33A s=MM01 i=One unidirectional video media t=3262377600 3262809600 m=video 51372 RTP/AVP 31 c=IN IP6 2001:0646:000A:03A7:02D0:59FF:FE40:2014 a=sendonly a=rtcp:53020

From this offer–answer exchange of SDP parameters the UE and the PCRF each creates a list of flow identifiers comprising the IP flows as shown in table B.5.3:

Table B.5.3: Flow identifiers in example 4.

| Order of 'm='-line | Type of IP flows | Destination IP address / Port number of the IP flows | Flow identifier |
| --- | --- | --- | --- |
| 1 | RTP (Video) DL | 2001:0646:00F1:0045:02D0:59FF:FE14:F33A / 50230 | <1,2> |
| 1 | RTCP DL | 2001:0646:00F1:0045:02D0:59FF:FE14:F33A / 49320 | <1,1> |
| 1 | RTCP UL | 2001:0646:000A:03A7:02D0:59FF:FE40:2014 / 53020 | <1,1> |

Annex C (informative):
Void

Annex D (informative):
Change history

| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 11/11/2005 |  |  |  |  | Includes the following TDOCs agreed at CT3#38: C3-050693, C3-050837, C3-050838, C3-050844, C3-050845 | 0.0.0 | 0.1.0 |
| 17/02/2006 |  |  |  |  | Includes the following TDOCs agreed at CT3#39: C3-060101, C3-060102, C3-060125, C3-060112, C3-060133, C3-060114, C3-060136, C3-060116 | 0.1.0 | 0.2.0 |
| 13/09/2006 |  |  |  |  | Includes the following TDOCs agreed at CT3#41: C3-060446, C3-060447, C3-060556, C3-060538, C3-060451, C3-C3-060453, C3-060540, C3-060496, C3-060544 | 0.2.0 | 0.3.0 |
| 15/09/2006 |  |  |  |  | Implementation of C3-060539 missing in former version and correction of an editorial error in C3-060496 implementation. | 0.3.0 | 0.4.0 |
| 11/11/2006 |  |  |  |  | Includes the following TDOCs agreed at CT3#42: C3-060605, C3-060639, , C3-060643, C3-060655, C3-060780, C3-060783, C3-060784, C3-060826, C3-060827, C3-060852, C3-060853 | 0.4.0 | 0.5.0 |
| 01/12/2006 | TSG#33 | CP-060637 |  |  | Editorial update by MCC for presentation to TSG CT for information | 0.5.0 | 1.0.0 |
| 10/01/2007 | TSG#33 |  |  |  | The misimplemented style of editor's notes in clause 4.3.2 was corrected to normal by MCC. | 1.0.0 | 1.0.1 |
| 15/02/2007 |  |  |  |  | Includes the following TDOCs agreed at CT3#43: C3-070063, C3-070086, C3-070118, C3-070139, C3-070151, C3-070189, C3-070217, C3-070218, C3-070221, C3-070236, C3-070254 | 1.0.1 | 1.1.0 |
| 28/02/2007 | TSG#35 | CP-070099 |  |  | Editorial update by MCC for presentation to TSG CT for approval | 1.1.0 | 2.0.0 |
| 09/03/2007 | TSG#35 |  |  |  | References to annex B in chapter 5 were replaced by "annex tbd" by Ragnar Huslende (CT3 Chairman), as agreed by CT#35 | 2.0.0 | 2.1.0 |
| 03-2007 |  |  |  |  | MCC update to version 7.0.0 after approval at TSG CT#35 | 2.1.0 | 7.0.0 |
| 06-2007 | TSG#36 | CP-070423 | 001 | 2 | PCRF assure emergency bearer for emergency service | 7.0.0 | 7.1.0 |
| 06-2007 | TSG#36 | CP-070423 | 002 | 7 | Signalling Path Status detected | 7.0.0 | 7.1.0 |
| 06-2007 | TSG#36 | CP-070423 | 003 |  | AF-Application Identifier at AF session level | 7.0.0 | 7.1.0 |
| 06-2007 | TSG#36 | CP-070423 | 005 | 2 | Alignment of PCRF Acknowledgment to AF | 7.0.0 | 7.1.0 |
| 06-2007 | TSG#36 | CP-070423 | 007 | 2 | Addition of Flow identifier annex | 7.0.0 | 7.1.0 |
| 06-2007 | TSG#36 | CP-070494 | 008 | 3 | Routeing of Diameter commands - Rx | 7.0.0 | 7.1.0 |
| 06-2007 | TSG#36 | CP-070423 | 013 | 1 | Various Minor Corrections | 7.0.0 | 7.1.0 |
| 06-2007 | TSG#36 | CP-070423 | 017 | 2 | Handling of disabled flows for forking | 7.0.0 | 7.1.0 |
| 06-2007 | TSG#36 | CP-070423 | 020 |  | Diameter application ID | 7.0.0 | 7.1.0 |
| 09-2007 | TSG#37 | CP-070558 | 022 | 1 | The input for PCC decisions | 7.1.0 | 7.2.0 |
| 09-2007 | TSG#37 | CP-070558 | 023 | 1 | Session Binding Error | 7.1.0 | 7.2.0 |
| 09-2007 | TSG#37 | CP-070559 | 025 | 2 | Service Information accepted by PCRF | 7.1.0 | 7.2.0 |
| 09-2007 | TSG#37 | CP-070558 | 026 |  | TS Clean-Up | 7.1.0 | 7.2.0 |
| 09-2007 | TSG#37 | CP-070558 | 028 | 1 | Origin-State-Id AVP missing in AA commands | 7.1.0 | 7.2.0 |
| 09-2007 | TSG#37 | CP-070558 | 030 | 2 | Clarification of PCRF Procedures | 7.1.0 | 7.2.0 |
| 09-2007 | TSG#37 | CP-070558 | 033 | 1 | Correction of implementation error of CR0013 | 7.1.0 | 7.2.0 |
| 12-2007 | TSG#38 | CP-070728 | 032 | 5 | Providing User’s IP-CAN Type and subscription for IP-CAN Type change notification to AF | 7.2.0 | 7.3.0 |
| 12-2007 | TSG#38 | CP-070728 | 037 | 1 | Provisioning of Service Information at P-CSCF | 7.2.0 | 7.3.0 |
| 12-2007 | TSG#38 | CP-070728 | 038 | 3 | Clarification on IMS Communication Service Identifier over Rx | 7.2.0 | 7.3.0 |
| 12-2007 | TSG#38 | CP-070728 | 039 |  | Removal of Emergency Services | 7.2.0 | 7.3.0 |
| 12-2007 | TSG#38 | CP-070728 | 041 | 1 | Adding Destination-Host to AAR | 7.2.0 | 7.3.0 |
| 03-2008 | TSG#39 | CP-080040 | 042 | 1 | Adding missing AVPs to Rx commands ABNF | 7.3.0 | 7.4.0 |
| 03-2008 | TSG#39 | CP-080046 | 044 | 1 | Clarification on Notification in Rx Interface | 7.4.0 | 8.0.0 |
| 05-2008 | TSG#40 | CP-080298 | 048 | 3 | AVP and Command Code updates for Common IMS | 8.0.0 | 8.1.0 |
| 05-2008 | TSG#40 | CP-080296 | 049 | 1 | Supporting third party application server for AF | 8.0.0 | 8.1.0 |
| 05-2008 | TSG#40 | CP-080292 | 051 | 1 | Support of TCP-based media in PCC | 8.0.0 | 8.1.0 |
| 05-2008 | TSG#40 | CP-080299 | 052 | 3 | IP-CAN Type verification | 8.0.0 | 8.1.0 |
| 05-2008 | TSG#40 | CP-080292 | 054 | 1 | Notification of Relase of Signalling Bearer | 8.0.0 | 8.1.0 |
| 05-2008 | TSG#40 | CP-080299 | 055 | 2 | IP CAN session definition update | 8.0.0 | 8.1.0 |
| 05-2008 | TSG#40 | CP-080299 | 057 | 2 | RAT type AVP | 8.0.0 | 8.1.0 |
| 09-2008 | TSG#41 | CP-080487 | 068 | 3 | PDN information in Rx interface | 8.1.0 | 8.2.0 |
| 12-2008 | TSG#42 | CP-080751 | 072 |  | Update of IETF references | 8.2.0 | 8.3.0 |
| 12-2008 | TSG#42 | CP-080768 | 076 | 1 | Out of credit indication, OFF only | 8.2.0 | 8.3.0 |
| 12-2008 | TSG#42 | CP-080760 | 081 | 1 | Update Reference Model | 8.2.0 | 8.3.0 |
| 12-2008 | TSG#42 | CP-080751 | 085 |  | Network-only bearer control mode | 8.2.0 | 8.3.0 |
| 12-2008 | TSG#42 | CP-080751 | 087 |  | Service information request | 8.2.0 | 8.3.0 |
| 12-2008 | TSG#42 | CP-080923 | 088 | 1 | Support of TIAS parameter | 8.2.0 | 8.3.0 |
| 03-2009 | TSG#43 | CP-090074 | 091 | 1 | Experimental-Result AVP usage on Rx | 8.3.0 | 8.4.0 |
| 03-2009 | TSG#43 | CP-090082 | 093 | 1 | Adding Supported-Features AVP to Rx commands | 8.3.0 | 8.4.0 |
| 03-2009 | TSG#43 | CP-090096 | 095 | 2 | Rx Diameter Sessions | 8.3.0 | 8.4.0 |
| 03-2009 | TSG#43 | CP-090096 | 096 | 2 | IMS session setup with UE initiated resource reservation | 8.3.0 | 8.4.0 |
| 05-2009 | TSG#44 | CP-090339 | 098 | 3 | Resource allocation subscription in Rx | 8.4.0 | 8.5.0 |
| 05-2009 | TSG#44 | CP-090346 | 100 | 1 | ICS impact on PCC | 8.4.0 | 8.5.0 |
| 05-2009 | TSG#44 | CP-090339 | 101 | 1 | Correction to feature bit definition | 8.4.0 | 8.5.0 |
| 05-2009 | TSG#44 | CP-090334 | 103 | 2 | Deleting INDICATION_OF_LOSS_OF_BEARER Event | 8.4.0 | 8.5.0 |
| 05-2009 | TSG#44 | CP-090339 | 104 | 3 | Limited PCC Deployment | 8.4.0 | 8.5.0 |
| 05-2009 | TSG#44 | CP-090353 | 099 | 1 | Preventing the use of an Emergency APN for Non-emergency calls | 8.5.0 | 9.0.0 |
| 09-2009 | TSG#45 | CP-090573 | 112 | 2 | Limited PCC deployment | 9.0.0 | 9.1.0 |
| 12-2009 | TSG#46 | CP-090842 | 116 | 3 | Access Network Charging Identifier handling in Rx | 9.1.0 | 9.2.0 |
| 12-2009 | TSG#46 | CP-090857 | 117 | 2 | PCC handling of SDP of early session disposition type | 9.1.0 | 9.2.0 |
| 12-2009 | TSG#46 | CP-090838 | 120 | 1 | Use of IPv6 Prefix | 9.1.0 | 9.2.0 |
| 12-2009 | TSG#46 | CP-090842 | 122 | 3 | Corrections for PCC/QoS rule modification failure | 9.1.0 | 9.2.0 |
| 12-2009 | TSG#46 | CP-090857 | 127 | 4 | Supported Features for Rel-9 Rx | 9.1.0 | 9.2.0 |
| 12-2009 | TSG#46 | CP-090838 | 130 | 1 | Rx request handling | 9.1.0 | 9.2.0 |
| 03-2010 | TSG#47 | CP-100070 | 137 | 1 | Correcting the reference of Annex 5 | 9.2.0 | 9.3.0 |
| 03-2010 | TSG#47 | CP-100076 | 139 | 2 | Correction of Flow-Description AVP description | 9.2.0 | 9.3.0 |
| 03-2010 | TSG#47 | CP-100086 | 140 | 3 | IMS Restoration procedure over Rx | 9.2.0 | 9.3.0 |
| 03-2010 | TSG#47 | CP-100070 | 143 | 1 | Notification about RAT-type change | 9.2.0 | 9.3.0 |
| 03-2010 | TSG#47 | CP-100076 | 145 | 2 | Supported-Feature feature bit definitions | 9.2.0 | 9.3.0 |
| 09-2010 | TSG#49 | CP-100626 | 147 | 1 | 3GPP References to TISPAN specifications | 9.3.0 | 9.4.0 |
| 09-2010 | TSG#49 | CP-100561 | 148 | 2 | Impacts of IFOM on Rx interface | 9.4.0 | 10.0.0 |
| 09-2010 | TSG#49 |  |  |  | Editorial correction by MCC | 10.0.0 | 10.0.1 |
| 12-2010 | TSG#50 | CP-100791 | 150 | 3 | Additions to support sponsored connectivity | 10.0.1 | 10.1.0 |
| 12-2010 | TSG#50 | CP-100771 | 153 | 1 | Specific-Action AVP to report specific actions | 10.0.1 | 10.1.0 |
| 12-2010 | TSG#50 | CP-100791 | 154 | 2 | UDR and Ud in PCC Architecture | 10.0.1 | 10.1.0 |
| 03-2011 | TSG#51 | CP-110125 | 159 | 3 | Authorization of the sponsored data connectivity | 10.1.0 | 10.2.0 |
| 03-2011 | TSG#51 | CP-110125 | 161 | 4 | ASP operators information for sponsored data connectivity | 10.1.0 | 10.2.0 |
| 03-2011 | TSG#51 | CP-110125 | 162 | 2 | Usage for sponsored connectivity | 10.1.0 | 10.2.0 |
| 03-2011 | TSG#51 | CP-110114 | 165 | 1 | Fixing Supported-Feature feature bit definitions | 10.1.0 | 10.2.0 |
| 03-2011 | TSG#51 | CP-110125 | 167 | 1 | Correction to feature bit for sponsored data connectivity | 10.1.0 | 10.2.0 |
| 03-2011 | TSG#51 | CP-110125 | 168 | 5 | Correction to the Reporting Usage for Sponsored Data Connectivity | 10.1.0 | 10.2.0 |
| 03-2011 | TSG#51 | CP-110121 | 171 | 1 | Support of MPS over Rx | 10.1.0 | 10.2.0 |
| 03-2011 | TSG#51 | CP-110125 | 172 | 2 | Some editorial corrections for sponsored data connectivity | 10.1.0 | 10.2.0 |
| 03-2011 | TSG#51 | CP-110125 | 173 |  | Requesting Usage Monitoring Control for sponsored connectivity | 10.1.0 | 10.2.0 |
| 06-2011 | TSG#52 | CP-110405 | 179 |  | PS-to-CS HO indicator in Rx | 10.2.0 | 10.3.0 |
| 06-2011 | TSG#52 | CP-110413 | 181 | 1 | Sponsor-Identity AVP & Application-Service-Provider-Identity AVP format. | 10.2.0 | 10.3.0 |
| 06-2011 | TSG#52 | CP-110409 | 184 | 1 | Some corrections for IFOM | 10.2.0 | 10.3.0 |
| 06-2011 | TSG#52 | CP-110411 | 186 |  | Supported-Features bit for eMPS | 10.2.0 | 10.3.0 |
| 06-2011 | TSG#52 | CP-110413 | 188 | 2 | Limit the sponsored data connectivity in the roaming case | 10.2.0 | 10.3.0 |
| 09-2011 | TSG#53 | CP-110608 | 194 | 2 | UE IPv6 prefix allocation clean-up | 10.3.0 | 10.4.0 |
| 09-2011 | TSG#53 | CP-110608 | 198 |  | Correction to the architecture figure(Rel-10) | 10.3.0 | 10.4.0 |
| 12-2011 | TSG#54 | CP-110935 | 203 |  | Alignment of PCC reference architecture with Stage 2 specification | 10.4.0 | 10.5.0 |
| 12-2011 | TSG#54 | CP-110827 | 207 |  | Correction in AVP names | 10.4.0 | 10.5.0 |
| 12-2011 | TSG#54 | CP-110835 | 209 |  | Flows AVP in AAA command | 10.4.0 | 10.5.0 |
| 03-2012 | TSG#55 | CP-120066 | 213 | 1 | QoS support for 3GP-DASH Services | 10.5.0 | 10.6.0 |
| 09-2012 | TSG#57 | CP-120511 | 232 | 1 | Correction of assigned internet protocol number | 10.6.0 | 10.7.0 |
| 09-2012 | TSG#57 | CP-120514 | 241 | 2 | Applicable IP-CAN Type | 10.6.0 | 10.7.0 |
| 12-2012 | TSG#58 | CP-120824 | 250 | 1 | PCC procedures for Rx sessions with no service information | 10.7.0 | 10.8.0 |
| 03-2013 | TSG#59 | CP-130059 | 266 | 1 | Cancelling subscription to notification of signaling path status | 10.8.0 | 10.9.0 |
| 06-2013 | TSG#60 | CP-130316 | 277 | 3 | Correction to the limited PCC deployment report | 10.9.0 | 10.10.0 |
| 09-2013 | TSG#61 | CP-130539 | 287 | 1 | Specific Action of recovery from limited PCC deployment correction | 10.10.0 | 10.11.0 |
| 09-2013 | TSG#61 | CP-130543 | 289 | 1 | Correction to resource allocation handling | 10.10.0 | 10.11.0 |
| 12-2013 | TSG#62 | CP-130663 | 301 | 2 | IP-CAN type notification correction | 10.11.0 | 10.12.0 |
| 12-2013 | TSG#62 | CP-130669 | 311 |  | Correction on Sponsored-Data-Connectivity AVP | 10.11.0 | 10.12.0 |
| 03-2014 | TSG#63 | CP-140066 | 320 | 1 | Correction of IMS emergency procedures for SIP Registration | 10.12.0 | 10.13.0 |
| 03-2015 | TSG#67 | CP-150101 | 377 |  | Correction to the usage monitoring control for sponsored data connectivity | 10.13.0 | 10.14.0 |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 12-2018 | CT #82 | CP-183119 | 1621 | - | A | Removal of editor's note on SIP forking | 10.15.0 |