3GPP TS 29.522 V16.0.0 (2019-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

5G System; Network Exposure Function Northbound APIs;

Stage 3

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Contents	3

Foreword	5

1	Scope	6

2	References	6

3	Definitions and abbreviations	7

3.1	Definitions	7

3.2	Abbreviations	7

4	NEF Northbound Interface	7

4.1	Overview	7

4.2	Reference model	8

4.3	Functional elements	9

4.3.1	NEF	9

4.3.2	AF	9

4.4	Procedures over NEF Northbound Interface	10

4.4.1	Introduction	10

4.4.2	Procedures for Monitoring	10

4.4.3	Procedures for Device Triggering	11

4.4.4	Procedures for resource management of Background Data Transfer	11

4.4.5	Procedures for CP Parameters Provisioning	11

4.4.6	Procedures for PFD Management	11

4.4.7	Procedures for Traffic Influence	12

4.4.7.1	General	12

4.4.7.2	AF request identified by UE address	12

4.4.7.3	AF request not identified by UE address	13

4.4.7.4	Handling of UP path management event notification	13

4.4.8	Procedures for changing the chargeable party at session set up or during the session	14

4.4.9	Procedures for setting up an AF session with required QoS	14

4.4.10	Procedures for MSISDN-less Mobile Originated SMS	14

4.4.11	Procedures for Network Configuration Parameters Provisioning	14

4.4.12	Procedures for Non-IP data delivery	15

4.4.12.1	General	15

4.4.12.2	NIDD configuration Triggered by the NEF	15

4.4.12.3	NIDD configuration triggered by the AF and NIDD delivery	15

5	NEF Northbound APIs	15

5.1	Introduction	15

5.2	Information applicable to several APIs	15

5.3	Reused APIs	16

5.4	TrafficInfluence API	16

5.4.1	Resources	16

5.4.1.1	Overview	16

5.4.1.2	Resource: Traffic Influence Subscription	17

5.4.1.2.1	Introduction	17

5.4.1.2.2	Resource Definition	17

5.4.1.2.3	Resource Methods	17

5.4.1.2.3.1	General	17

5.4.1.2.3.2	GET	18

5.4.1.2.3.3	POST	18

5.4.1.3	Resource: Individual Traffic Influence Subscription	19

5.4.1.3.1	Introduction	19

5.4.1.3.2	Resource Definition	19

5.4.1.3.3	Resource Methods	19

5.4.1.3.3.1	General	19

5.4.1.3.3.2	GET	19

5.4.1.3.3.3	PUT	20

5.4.1.3.3.4	PATCH	20

5.4.1.3.3.5	DELETE	20

5.4.2	Notifications	21

5.4.2.1	Introduction	21

5.4.2.2	Event Notification	21

5.4.2.3	Operation Definition	21

5.4.2.3.1	Notification via HTTP POST	21

5.4.2.3.2	Notification via Websocket	22

5.4.3	Data Model	22

5.4.3.1	General	22

5.4.3.2	Reused data types	22

5.4.3.3	Structured data types	22

5.4.3.3.1	Introduction	22

5.4.3.3.2	Type: TrafficInfluSub	23

5.4.3.3.3	Type: TrafficInfluSubPatch	25

5.4.3.3.4	Type: EventNotification	27

5.4.3.3.5	Type: AfResultInfo	27

5.4.3.4	Simple data types and enumerations	28

5.4.3.4.1	Introduction	28

5.4.3.4.2	Simple data types	28

5.4.3.4.3	Enumeration: SubscribedEvent	28

5.4.3.4.4	Enumeration: AfResultStatus	28

5.4.4	Used Features	28

5.5	NiddConfigurationTrigger API	29

5.5.1	Resources	29

5.5.2	Notifications	29

5.5.2.1	Introduction	29

5.5.2.2	Event Notification	29

5.5.2.3	Operation Definition	29

5.5.2.3.1	Notification via HTTP POST	29

5.5.2.3.2	Notification via Websocket	29

5.5.3	Data Model	29

5.5.3.1	General	29

5.5.3.2	Reused data types	30

5.5.3.3	Structured data types	30

5.5.3.3.1	Introduction	30

5.5.3.3.2	Type: NiddConfigTrigger	30

5.5.3.3.3	Type: NiddConfigTriggerReply	30

5.5.3.4	Simple data types and enumerations	31

5.5.3.4.1	Introduction	31

5.5.3.4.2	Simple data types	31

5.5.4	Used Features	31

6	Security	31

7	Using Common API Framework	31

7.1	General	31

7.2	Security	32

Annex A (normative):	OpenAPI representation for NEF Northbound APIs	33

A.1	General	33

A.2	TrafficInfluence API	33

A.3	NiddConfigurationTrigger API	40

Annex B (informative):	Change history	42

Annex B (informative):	Change history	31

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present specification describes the protocol for the NEF Northbound interface between the NEF and the AF. The NEF Northbound interface and the related stage 2 functional requirements are defined in 3GPP TS 23.502 [2].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.502: "Procedures for the 5G system".

[3]	3GPP TS 23.501: "System Architecture for the 5G".

[4]	3GPP TS 29.122: "T8 reference point for northbound Application Programming Interfaces (APIs)".

[5]	Open API Initiative, "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

[6]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[7]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[8]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[9]	3GPP TS 29.521: "5G System; Binding Support Management Service; Stage 3".

[10]	Void.

[11]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[12]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3".

[13]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[14]	3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[15]	Void.

[16]	IETF RFC 5246: "The Transport Layer Security (TLS) Protocol Version 1.2".

[17]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[18]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[19]	3GPP TS 29.554: "5G System; Background Data Transfer Policy Control Service; Stage 3".

[20]	3GPP TS 29.504: "5G System; Unified Data Repository Services; Stage 3".

[21]	3GPP TR 21.900: "Technical Specification Group working methods".

[22]	3GPP TS 29.523: "5G System; Policy Control Event Exposure Service; Stage 3".

[23]	3GPP TS 29.519: "5G System; Usage of the Unified Data Repository service for Policy Control Data, Application Data and Structured Data for Exposure; Stage 3".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AF	Application Function

BDT	Background Data Transfer

CAPIF	Common API Framework

CP	Communication Pattern

DN	Data Network

DNAI	DN Access Identifier

DNN	Data Network Name

GPSI	Generic Public Subscription Identifier

NEF	Network Exposure Function

PCF	Policy Control Function

PCRF	Policy and Charging Rule Function

PFD	Packet Flow Description

PFDF	Packet Flow Description Function

REST	Representational State Transfer

SCEF	Service Capability Exposure Function

S-NSSAI	Single Network Slice Selection Assistance Information

UDR	Unified Data Repository

UP	User Plane

# 4	NEF Northbound Interface

## 4.1	Overview

The NEF Northbound interface is between the NEF and the AF. It specifies RESTful APIs that allow the AF to access the services and capabilities provided by 3GPP network entities and securely exposed by the NEF.

This document also specifies the procedures triggered at the NEF by API requests from the AF and by event notifications received from 3GPP network entities.

The stage 2 level requirements and signalling flows for the NEF Northbound interface are defined in 3GPP TS 23.502 [2].

The NEF Northbound interface supports the following procedures:

1)	Procedures for Monitoring

2)	Procedures for Device Triggering

3)	Procedures for resource management of Background Data Transfer

4)	Procedures for CP Parameters and Network Configuration Parameters Provisioning

5)	Procedures for PFD Management

6)	Procedures for Traffic Influence

7)	Procedures for changing the chargeable party at session set up or during the session

8)	Procedures for setting up an AF session with required QoS

9)	Procedures for MSISDN-less Mobile Originated SMS

10)	Procedures for non-IP data delivery

Which correspond to the following services respectively, supported by the NEF as defined in 3GPP TS 23.502 [2]:

1)	Nnef_EventExposure service

2)	Nnef_Trigger service

3)	Nnef_BDTPNegotiation service

4)	Nnef_ParameterProvision service

5)	Nnef_PFDManagement service

6)	Nnef_TrafficInfluence service

7)	Nnef_ChargeableParty service

8)	Nnef_AFsessionWithQoS service

9)	Nnef_MSISDN-less_MO_SMS service

10)	Nnef_NIDDConfiguration and Nnef_NIDD services

NOTE 1:	For Nnef_PFDManagement service, only the Nnef_PFDManagement_Create/Update/Delete service operations are applicable for the NEF Northbound interface.

NOTE 2:	For Nnef_NIDD service, NF consumer other than the AF does not use the NEF Northbound interface.

NOTE 3:	For Nnef_NIDDConfiguration service, the Nnef_NIDDConfiguration_Trigger service operation is only applicable for the NEF Northbound interface.

## 4.2	Reference model

The NEF Northbound interface resides between the NEF and the AF as depicted in figure 4.2.1. The overall NEF architecture is depicted in 3GPP TS 23.502 [2]. An AF can get services from multiple NEFs, and an NEF can provide service to multiple AFs.

NOTE:	The AF can be provided by the third party.

Figure 4.2-1: Reference Architecture for the Nnef Service; SBI representation

Figure 4.2-2: Reference Architecture for the Nnef Service; reference point representation

## 4.3	Functional elements

### 4.3.1	NEF

The Network Exposure Function (NEF) is a functional element that supports the following functionalities:

-	The NEF shall securely expose network capabilities and events provided by 3GPP NFs to AF.

-	The NEF shall provide a means for the AF to securely provide information to 3GPP network and may authenticate, authorize and assist in throttling the AF.

-	The NEF shall be able to translate the information received from the AF to the one sent to internal 3GPP NFs, and vice versa.

-	The NEF shall support to expose information (collected from other 3GPP NFs) to the AF.

-	The NEF may support a PFD Function which allows the AF to provision PFD(s) and may store and retrieve PFD(s) in the UDR. The NEF further provisions PFD(s) to the SMF.

A specific NEF instance may support one or more of the functionalities described above and consequently an individual NEF may support a subset of the APIs specified for capability exposure.

NOTE:	The NEF can access the UDR located in the same PLMN as the NEF.

### 4.3.2	AF

The Application Function (AF) may interact with the 3GPP Core Network via the NEF in order to access network capabilities.

## 4.4	Procedures over NEF Northbound Interface

### 4.4.1	Introduction

All procedures that operate across the NEF Northbound interface, as specified in 3GPP TS 23.502 [2], are specified in the following subclauses.

### 4.4.2	Procedures for Monitoring

The procedures for monitoring as described in subclause 4.4.2 of 3GPP TS 29.122 [4] shall be applicable in 5GS with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF;

-	description of the HSS applies to the UDM, and the NEF shall interact with the UDM by using Nudm_EventExposure service as defined in 3GPP TS 29.503 [17];

-	description of the MME/SGSN applies to the AMF, and the NEF shall interact with the AMF by using Namf_EventExposure service as defined in 3GPP TS 29.518 [18];

-	description about the PCRF is not applicable.

-	description about the change of IMSI-IMEI(SV) association monitoring event applies to the change of SUPI-PEI association monitoring event; and

-	when sending the UDM/AMF event report to the AF, the NEF may store the event data in the report in the UDR as part of the data for exposure as specified in 3GPP TS 29.519 [23] by using Nudr_DataRepository service as specified in 3GPP TS 29.504 [20].

-	If the "Downlink_data_delivery_status_5G" as defined in subclause 5.3.4 of 3GPP TS 29.122 [4] is supported, in order to support the downlink data delivery status notification,

-	the AF shall send an HTTP POST message to the NEF to the resource "Monitoring Event Subscriptions" as defined in 5.3.3.2 of 3GPP TS 29.122 [4] with the following difference:

-	within the MonitoringEventSubscription data structure the AF may additionally include packet filter descriptions within the "dddTraDes" attribute and the list of monitoring downlink data delivery status event(s) within the "dddStati" attribute;

-	the NEF shall subscribe the events to the appropriate PCF(s) within the network by invoking the Npcf_EventExposure_Subscribe service operation as defined in subclause 4.2.2.2 of 3GPP TS 29.523 [22].

Editor's note:	It is FFS whether the update of the subscription is supported.

Editor's note:	references of TS 29.523 need to be updated later.

-	when the NEF receives the event notification as defined in subclause 4.2.4 of 3GPP TS 29.523 [22], the NEF shall send an HTTP POST message to the AF as defined in subclause 4.4.2.3 of 3GPP TS 29.122 [4] with the difference that within each MonitoringEventReport data structure, the NEF shall include:

-	the downlink data delivery status within the "dddStatus" attribute;

-	the estimated buffering time within the "maxWaitTime" attribute if the downlink data delivery status is set to "DATA_PACKETS_BUFFERED";

Editor´s note: If multiple traffic filters are permissable in the subscription, Flow IDs may be required.

-	If the "Availability_after_DDN_failure_notification_enhancement" feature as defined in subclause 5.3.4 of 3GPP TS 29.122 [4] is supported, AF may subscribe the event to the UDM as defined in subclause 4.4.2.2.2 of 3GPP TS 29.122 [4] with the difference that within the MonitoringEventSubscription data structure, the AF shall include packet filter descriptions within the "dddTraDes" attribute.

Editor´s note: If multiple traffic filters are permissable in the subscription, Flow IDs may be required.

### 4.4.3	Procedures for Device Triggering

The procedures for device triggering as described in subclause 4.4.6 of 3GPP TS 29.122 [4] shall be applicable in 5G with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF;

-	description of the HSS applies to the UDM;

-	the NEF shall interact with the UDM by using the Nudm_SubscriberDataManagement service and the Nudm_UEContextManagement service as defined in 3GPP TS 29.503 [17]; and

-	the NEF acts as MTC-IWF.

### 4.4.4	Procedures for resource management of Background Data Transfer

The procedures for resource management of Background Data Transfer (BDT) in 5GS are described in subclause 4.4.3 3GPP TS 29.122 [4] with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF;

-	if the external group Id is sent from the AF to the NEF, the NEF shall interact with the UDM by using Nudm_SubscriberDataManagement service as defined in 3GPP TS 29.503 [17] to translate the external group identifier into the corresponding internal group identifier;

-	description of the PCRF applies to the PCF; and

-	the NEF shall interact with the PCF by using Npcf_BDTPolicyControl service as defined in 3GPP TS 29.554 [19].

-	If the "WarningNotification_5G" is supported, the AF may include a notification URI within the "notificationDestination" attribute in the BDT data type during the background data transfer policy negotiation. When the NEF receives the BDT warning notification from the PCF as defined in clause 4.2.4.2 of 3GPP TS 29.554 [19], the NEF shall send an HTTP POST message including the ExNotification data structure to the AF identified by the notification destination URI received during the background data transfer policy negotiation. The AF shall respond with an HTTP response to confirm the received notification.

### 4.4.5	Procedures for CP Parameters Provisioning

The procedures for CP parameters provisioning as described in subclause 4.4.9 of 3GPP TS 29.122 [4] shall be applicable in 5G with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF;

-	description of the HSS applies to the UDM;

-	the NEF shall interact with the UDM by using Nudm_ParameterProvision service as defined in 3GPP TS 29.503 [17].

### 4.4.6	Procedures for PFD Management

The procedures for PFD management as described in subclause 4.4.10 of 3GPP TS 29.122 [4] shall be applicable for 5GS with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF; and

-	the NEF (PFDF) shall interact with the UDR for PFD management by using Nudr_DataRepository service as defined in 3GPP TS 29.504 [20]. The PFDF is functionality within the NEF.

-	If the PFDs are provisioned to at least one of the subscribed SMFs (but not all) within the allowed delay, the NEF (PFDF) may notify the AF about the failed PFD provisioning with the HTTP POST message using the failure code "PARTIAL_FAILURE" as defined in Table 5.11.2.2.3-1 of 3GPP TS 29.122 [4]. If the PFDs are provisioned to none of the subscribed SMFs within the allowed delay, the NEF (PFDF) shall notify the AF about the failed PFD provisioning with the HTTP POST message using appropriate failure code as defined in Table 5.11.2.2.3-1 of 3GPP TS 29.122 [4].

### 4.4.7	Procedures for Traffic Influence

#### 4.4.7.1	General

In order to create a resource for the Traffic Influence, the AF shall send an HTTP POST message to the NEF to the resource "Traffic Influence Subscription", the body of the HTTP POST message may include the AF Service Identifier, external Group Identifier, external Identifier, any UE Indication, the UE IP address, GPSI, DNN, S-NSSAI, Application Identifier or traffic filtering information, Subscribed Event, Notification destination address, a list of geographic zone identifier(s), AF Transaction Identifier, a list of DNAI(s), routing profile ID(s) or N6 traffic routing information, Indication of application relocation possibility, type of notifications, Temporal, spatial validity conditions, Indication of AF acknowledgement to be expected if the AFRelocationAck feature is supported and Indication of UE IP address preservation if the AddrPreservation feature is supported. The Notification destination address shall be included if the Subscribed Event is included in the HTTP request message.

In order to update an existing traffic influence subscription, the AF shall send an HTTP PUT or PATCH message to the resource "Individual Traffic Influence Subscription" requesting to change the traffic influence parameters.

In order to delete an existing traffic influence subscription, the AF shall send an HTTP DELETE message to the NEF to the resource "Individual Traffic Influence Subscription".

Upon receipt of the HTTP request from the AF, if the AF is authorized, the NEF shall perform the mapping as described in 3GPP TS 23.501 [3], and then perform as described in subclause 4.4.7.2 if the request is for an individual UE or perform as described in subclause 4.4.7.3 if the request is for multiple UEs.

4.4.7.2	AF request identified by UE address

Upon receipt of the above AF request which is for an individual UE identified by IP or Ethernet address, the NEF may interact with the BSF to retrieve the related PCF information by invoking the Nbsf_Management_Discovery service operation as described in 3GPP TS 29.521 [9], if the NEF receives an error code from the BSF, the NEF shall not create, update or delete the resource and shall respond to the SCS/AS with a status code set to 500 Internal Server Error.

After receiving a successful response from the BSF, the NEF shall interact with the PCF by invoking the Npcf_PolicyAuthorization service as described in 3GPP TS 29.514 [7]. After receiving a successful response from the PCF, the NEF shall,

-	for the HTTP POST request, create a resource "Individual Traffic Influence Subscription" which represents the traffic influence subscription, addressed by a URI that contains the AF Identifier and an NEF-created subscription identifier, and shall respond to the AF with a 201 Created status code, including a Location header field containing the URI for the created resource. The AF shall use the URI received in the Location header in subsequent requests to the NEF to refer to this traffic influence subscription.

-	for the HTTP PUT or PATCH request, update a resource "Individual Traffic Influence Subscription" which represents the traffic influence subscription, and shall responds to the AF with a 200 OK status code.

-	for the HTTP DELETE request, remove all properties of the resource and delete the corresponding active resource "Individual Traffic Influence Subscription" which represents the traffic influence subscription, then shall responds to the AF with a 204 No Content status code.

If the NEF receives a response with an error code from the PCF, the NEF shall not create, update or delete the resource and shall respond to the SCS/AS with a status code set to 500 Internal Server Error.

#### 4.4.7.3	AF request not identified by UE address

For AF request not identified by UE address, it may target an individual UE, a group of UEs or any UE. For an individual UE identified by GPSI, or a group of UEs identified by External Group Identifier, the NEF shall interact with the UDM by invoking the Nudm_SubscriberDataManagement service as described in 3GPP TS 29.503 [17] to retrieve the SUPI or Internal Group Identifier.,

The NEF may interact with the UDR by invoking the Nudr_DataRepository service as described in 3GPP TS 29.504 [20], if the NEF receives an error code from the UDR, the NEF shall not create, update or delete the resource and shall respond to the SCS/AS with a status code set to 500 Internal Server Error.

After receiving a successful response from the UDR, the NEF shall,

-	for the HTTP POST request, create a resource "Individual Traffic Influence Subscription" which represents the traffic influence subscription, addressed by a URI that contains the AF Identifier and an NEF-created subscription identifier, and shall respond to the AF with a 201 Created status code, including a Location header field containing the URI for the created resource. The AF shall use the URI received in the Location header in subsequent requests to the NEF to refer to this traffic influence subscription.

-	for the HTTP PUT or PATCH request, update a resource "Individual Traffic Influence Subscription" which represents the traffic influence subscription, and shall responds to the AF with a 200 OK status code.

-	for the HTTP DELETE request, delete the corresponding active resource "Individual Traffic Influence Subscription" which represents the traffic influence subscription, and shall responds to the AF with a 204 No Content status code.

#### 4.4.7.4	Handling of UP path management event notification

If the NEF receives a UP path management event notification from the SMF indicating that the subscribed event has been detected, then the NEF shall provide a notification by sending an HTTP POST message that shall include the EventNotification data type at least with the subscribed event (e.g. UP Path has changed) to the AF identified by the notification destination received during creation of the Individual Traffic Influence Subscription. If the AFRelocationAck feature is supported and the indication of AF acknowledgement to be expected within the "afAckInd" attribute was provided during creation and/or update of the Individual Traffic Influence Subscription, the NEF shall also provide a URI for AF acknowledgement in the EventNotification data.

Editor’s Note:	The attribute as the URI for AF acknowledgement needs to be defined in the EventNotification data.

Upon receipt of the event notification, the AF shall respond with a "204 No Content" status code to confirm the received event notification.

Afterwards, if the indication of AF acknowledgement to be expected within the "afAckInd" attribute was provided during creation and/or update of the Individual Traffic Influence Subscription, the AF may determine that an application layer relocation is needed, and may then send an HTTP POST request as acknowledgement for the UP path management event notification to inform the NEF about the result of application layer relocation. If the application layer is ready and/or the application relocation is completed, within the payload of the HTTP POST request, the AF shall include the AfAckInfo data type with the "afStatus" attribute sets to "SUCCESS" and may provide the N6 traffic routing information associated to the target DNAI as a "trafficRoute" attribute within the AfResultInfo data; otherwise, the AF shall indicate the failure by including the AfAckInfo data type in the payload with the "afStatus" attribute sets to the corresponding failure cause. The NEF Northbound interface transaction identifier generated by the AF shall also be provided within the AfAckInfo data if available.

Editor’s Note:	Definition of the HTTP POST message for AF acknowledgement (i.e. Nnef_TrafficInfluence_AppRelocationInfo service operation) and the AfAckInfo data structure are FFS.

Editor’s Note:	If the N6 traffic routing information is omitted, it’s FFS that whether it means PSA change is not allowed.

Upon receipt of the AF acknowledgement, the NEF shall respond with a "204 No Content" status code to confirm the received acknowledgement.

### 4.4.8	Procedures for changing the chargeable party at session set up or during the session

The procedures for changing the chargeable party at session set up or during the session in 5GS are described in subclause 4.4.4 of 3GPP TS 29.122 [4] with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF;

-	description of the PCRF applies to the PCF; and

-	the NEF may interact with BSF by using Nbsf_Management_Discovery service (as defined in 3GPP TS 29.521 [9]) to retrieve the PCF address; and

-	the NEF shall interact with the PCF by using Npcf_PolicyAuthorization service as defined in 3GPP TS 29.514 [7].

### 4.4.9	Procedures for setting up an AF session with required QoS

The procedures for setting up an AF session with required QoS in 5GS are described in subclause 4.4.13 of 3GPP TS 29.122 [4] with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF;

-	description of the PCRF applies to the PCF; and

-	the NEF may interact with BSF by using Nbsf_Management_Discovery service  (as defined in 3GPP TS 29.521 [9]) to retrieve the PCF address; and

-	the NEF shall interact with the PCF by using Npcf_PolicyAuthorization service as defined in 3GPP TS 29.514 [7].

### 4.4.10	Procedures for MSISDN-less Mobile Originated SMS

The procedures are used by the NEF to send the MSISDN-less MO-SMS to the AF in 5GS are described in subclause 4.4.14 of 3GPP TS 29.122 [4] with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF; and

-	the NEF shall interact with UDM by using Nudm_SubscriberDataManagement service (as defined in 3GPP TS 29.503 [17]) to retrieve the external identifier.

### 4.4.11	Procedures for Network Configuration Parameters Provisioning

The procedures for network configuration parameters provisioning as described in subclause 4.4.12 of 3GPP TS 29.122 [4] shall be applicable in 5GS with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF;

-	description of the HSS applies to the UDM;

-	the NEF shall interact with the UDM by using Nudm_ParameterProvision service as specified in 3GPP TS 29.503 [17].

Editor's note:	It is FFS whether the two existing 4G APIs (CP provisioning and network parameter Provisioning) can be enhanced to satisfy 5G provisioning service without introducing new 5G provisioning API.

### 4.4.12	Procedures for Non-IP data delivery

#### 4.4.12.1	General

The procedures are used by the NEF to send/receive the non-IP data to/from the AF. It comprises NIDD configuration and NIDD delivery.

The NIDD configuration may be triggered by the NEF or the AF. If it is triggered by the NEF, the NiddConfigurationTrigger API described in subclause 5.5 is used and the procedure is described in subclause 4.4.12.2.

#### 4.4.12.2	NIDD configuration Triggered by the NEF

If the NEF receives a NIDD connection establishment request from the SMF and if there is no NIDD configuration for the UE, the NEF may send a NIDD configuration trigger to the AF. The NEF determines the destination URI by local configuration. The NEF shall send to the determined destination URL an HTTP POST request that shall include a NiddConfiguarationTrigger data type with:

-	the NEF identifier,

-	the AF identifier, and

-	GPSI as UE identity.

The AF shall acknowledge the HTTP POST request with an HTTP 200 OK response. Then the AF may start NIDD configuration procedure as described in subclause 4.4.12.3.

#### 4.4.12.3	NIDD configuration triggered by the AF and NIDD delivery

The procedures for NIDD configuration triggered by the AF and NIDD delivery are described in subclause 4.4.5 of 3GPP TS 29.122 [4] with the following differences:

-	description of the SCS/AS applies to the AF;

-	description of the SCEF applies to the NEF;

-	description of the MME/SGSN applies to the SMF;

-	for the connection establishment, the interaction between the NEF and the SMF shall use Nnef_SMContext service as specified in 3GPP TS 29.xyz [xx];

-	for MO/MT NIDD, the interaction between the SMF and the NEF shall use Nnef_NIDD service as specified in 3GPP TS 29.xyz [xx].

Editor's note:	New TS number is not ready yet.

# 5	NEF Northbound APIs

## 5.1	Introduction

The NEF Northbound APIs are a set of APIs defining the related procedures and resources for the interaction between the NEF and the AF.

## 5.2	Information applicable to several APIs

The usage of HTTP and content type, as specified in subclauses 5.2.2 and 5.2.3 of 3GPP TS 29.122 [4] respectively, shall be applicable for NEF Northbound APIs.

The notification, error handling, feature negotiation, HTTP custom headers as specified in subclauses 5.2.5, 5.2.6, 5.2.7, 5.2.8 of 3GPP TS 29.122 [4] respectively, shall be applicable for NEF Northbound APIs except that the SCEF is replaced by the NEF and the SCS/AS by the AF.

The conventions for Open API specification files as specified in subclause 5.2.10 of 3GPP TS 29.122 [4] shall be applicable for NEF Northbound APIs.

## 5.3	Reused APIs

This subclause describes the northbound APIs which are applicable for both EPS and 5GS.

Table 5.3.1-1: Reused APIs applicable for both EPS and 5GS

| API Name | Differences |
| --- | --- |
| ResourceManagementOfBdt | - The "LocBdt_5G" feature as described in subclause 5.4.4 of 3GPP TS 29.122 [4] may only be supported in 5G. - The "Group_Id" feature as described in subclause 5.4.4 of 3GPP TS 29.122 [4] may be supported in 5G. - The "WarningNotification_5G" feature as described in subclause 5.4.4 of 3GPP TS 29.122 [4] may only be supported in 5G. |
| PfdManagement |  |
| MonitoringEvent | The "Number_of_UEs_in_an_area_notification_5G" feature as described in subclause 5.3.4 of 3GPP TS 29.122 [4] may only be supported in 5G. |
| DeviceTriggering |  |
| CpProvisioning | The "ExpectedUMT_5G" feature as described in subclause 5.10.4 of 3GPP TS 29.122 [4] may only be supported in 5G. |
| ChargeableParty | - The "EthChgParty_5G" feature as described in subclause 5.5.4 of 3GPP TS 29.122 [4] may only be supported in 5G. - The events (i.e. LOSS_OF_BEARER, RECOVERY_OF_BEARER and RELEASE_OF_BEARER) do not apply for 5G. |
| AsSessionWithQoS | - The "EthAsSessionQoS_5G" feature as described in subclause 5.14.4 of 3GPP TS 29.122 [4] may only be supported in 5G. - The events (i.e. LOSS_OF_BEARER, RECOVERY_OF_BEARER and RELEASE_OF_BEARER) do not apply for 5G. |
| MsisdnLessMoSms |  |
| NpConfiguration | - The "NpExpiry_5G” feature as described in subclause 5.13.4 of 3GPP TS 29.122 [4] may only be supported in 5G. |
| NIDD |  |

## 5.4	TrafficInfluence API

### 5.4.1	Resources

#### 5.4.1.1	Overview

This subclause describes the structure for the Resource URIs as shown in figure 5.4.1.1-1 and the resources and HTTP methods used for the TrafficInfluence API.

Figure 5.4.1.1-1: Resource URI structure of the TrafficInfluence API

Table 5.4.1.1-1 provides an overview of the resources and HTTP methods applicable for the TrafficInfluence API.

Table 5.4.1.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Traffic Influence Subscription | {apiRoot}/3gpp-traffic-Influence/v1/{afId}/subscriptions | GET | Read all subscriptions for a given AF |
|  |  | POST | Create a new subscription to traffic influence |
| Individual Traffic Influence Subscription | {apiRoot}/3gpp-traffic-Influence/v1/{afId}/subscriptions/{subscriptionId} | GET | Read the subscription to the traffic influence |
|  |  | PUT | Modify all of the properties of an existing subscription to an traffic influence |
|  |  | PATCH | Modify part of the properties of an existing subscription to an traffic influence |
|  |  | DELETE | Delete the subscription to the traffic influence |

#### 5.4.1.2	Resource: Traffic Influence Subscription

##### 5.4.1.2.1	Introduction

This resource allows a AF to read all active traffic influence subscribtions for the given AF.

##### 5.4.1.2.2	Resource Definition

Resource URI: {apiRoot}/3gpp-traffic-influence/v1/{afId}/subscriptions

This resource shall support the resource URI variables defined in table 5.4.1.2.2-1.

Table 5.4.1.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | Subclause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | Identifier of the AF of type string. |

##### 5.4.1.2.3	Resource Methods

###### 5.4.1.2.3.1	General

The following subclauses specify the resource methods supported by the resource as described in subclause 5.4.1.2.3.

###### 5.4.1.2.3.2	GET

The GET method allows to read all active subscriptions for a given AF. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.4.1.2.3.2-1.

Table 5.4.1.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.4.1.2.3.2-2 and the response data structures and response codes specified in table 5.4.1.2.3.2-3.

Table 5.4.1.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.4.1.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(TrafficInfluSub) |  | 0..N | 200 OK | The subscription information for the AF in the request URI are returned. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

###### 5.4.1.2.3.3	POST

The POST method creates a new subscription resource to traffic influence subscription for a given AF. The AF shall initiate the HTTP POST request message and the NEF shall respond to the message. The NEF shall construct the URI of the created resource.

This method shall support the request data structures specified in table 5.4.1.2.3.3-1 and the response data structures and response codes specified in table 5.4.1.2.3.3-2.

Table 5.4.1.2.3.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TrafficInfluSub | M | 1 | Parameters to register a subscription to influencing traffic routing and/or notification about UP management events with the NEF. |

Table 5.4.1.2.3.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TrafficInfluSub | M | 1 | 201 Created | The subscription was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

#### 5.4.1.3	Resource: Individual Traffic Influence Subscription

##### 5.4.1.3.1	Introduction

This resource allows a AF to register a subscription to influencing traffic routing and/or notification about UP management events with the NEF.

##### 5.4.1.3.2	Resource Definition

Resource URI: {apiRoot}/3gpp-traffic-influence/v1/{afId}/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 5.4.1.3.2-1.

Table 5.4.1.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | Subclause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | Identifier of the AF of type string. |
| subscriptionId | Identifier of the subscription resource of type string. |

##### 5.4.1.3.3	Resource Methods

###### 5.4.1.3.3.1	General

The following subclauses specify the resource methods supported by the resource as described in subclause 5.4.1.3.3.

###### 5.4.1.3.3.2	GET

The GET method allows to read the active subscription for a given AF and subscription Id. The AF shall initiate the HTTP GET request message and theNEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.4.1.3.3.2-1.

Table 5.4.1.3.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.4.1.3.3.2-2 and the response data structures and response codes specified in table 5.4.1.3.3.2-3.

Table 5.4.1.3.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.4.1.3.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TrafficInfluSub | M | 1 | 200 OK | The subscription information for the AF in the request URI are returned. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

###### 5.4.1.3.3.3	PUT

The PUT method modifies an existing subscription resource to update a subscription. The AF shall initiate the HTTP PUT request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.4.1.3.3.3-1 and the response data structures and response codes specified in table 5.4.1.3.3.3-2.

Table 5.4.1.3.3.3-1: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TrafficInfluSub | M | 1 | Modify an existing subscription to influencing traffic routing and/or notification about UP management events with the NEF. |

Table 5.4.1.3.3.3-2: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TrafficInfluSub | M | 1 | 200 OK | The subscription was updated successfully. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

###### 5.4.1.3.3.4	PATCH

The PATCH method allows to change some properties of an existing traffic influence subscription. The AF shall initiate the HTTP PATCH request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.4.1.3.3.4-1 and the response data structures and response codes specified in table 5.4.1.3.3.4-2.

Table 5.4.1.3.3.4-1: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TrafficInfluSubPatch | M | 1 | Partial update of a subscription to influencing traffic routing and/or notifications about UP management events with the NEF. |

Table 5.4.1.3.3.4-2: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TrafficInfluSub | M | 1 | 200 OK | The subscription was modified successfully. |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

###### 5.4.1.3.3.5	DELETE

The DELETE method deletes the traffic influence subscription for a given AF. The AF shall initiate the HTTP DELETE request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.4.1.3.3.5-1.

Table 5.4.1.3.3.5-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.4.1.3.3.5-2 and the response data structures and response codes specified in table 5.4.1.3.3.5-3.

Table 5.4.1.3.3.5-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.4.1.3.3.5-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The subscription was terminated successfully. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

### 5.4.2	Notifications

#### 5.4.2.1	Introduction

Upon receipt of a UP management event notification from the SMF indicating the subscribed event (e.g. a DNAI has changed) is detected, the NEF shall send an HTTP POST message including the notified event to the AF. The NEF and the AF shall support the notification mechanism as described in subclause 5.2.5 of 3GPP TS 29.122 [4].

#### 5.4.2.2	Event Notification

URI: {notificationDestination}

The operation shall support the URI variables defined in table 5.4.2.2-1.

Table 5.4.2.2-1: URI variables

| Name | Definition |
| --- | --- |
| notificationDestination | Callback reference provided by the AF during creation of the subscription within the TrafficInfluSub data type as defined in Table 5.4.3.3.2-1. |

#### 5.4.2.3	Operation Definition

##### 5.4.2.3.1	Notification via HTTP POST

This method shall support the request data structures specified in table 5.4.2.3.1-1 and the response data structures and response codes specified in table 5.4.2.3.1-2.

Table 5.4.2.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EventNotification | M | 1 | The UP management event notification is provided by the NEF to the AF. |

Table 5.4.2.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The event notification is received successfully. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

##### 5.4.2.3.2	Notification via Websocket

If supported by both AF and NEF and successfully negotiated, the EventNotification may alternatively be delivered through the Websocket mechanism as defined in subclause 5.2.5.4 of 3GPP TS 29.122 [4].

### 5.4.3	Data Model

#### 5.4.3.1	General

This subclause specifies the application data model supported by the TrafficInfluence API.

#### 5.4.3.2	Reused data types

The data types reused by the TrafficInfluence API from other specifications are listed in table 5.4.3.2-1.

Table 5.4.3.2-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Dnai | 3GPP TS 29.571 [8] | Identifies a DNAI. |
| DnaiChangeType | 3GPP TS 29.571 [8] | Describes the types of DNAI change. |
| Dnn | 3GPP TS 29.571 [8] | Identifies a DNN. |
| EthFlowDescription | 3GPP TS 29.514 [7] | Contains the Ethernet data flow information. |
| ExternalGroupId | 3GPP TS 29.122 [4] | External Group Identifier for a user group. |
| FlowInfo | 3GPP TS 29.122 [4] | Contains the IP data flow information. |
| Gpsi | 3GPP TS 29.571 [8] | Identifies a GPSI. |
| Ipv4Addr | 3GPP TS 29.122 [4] | Identifies an IPv4 address. |
| Ipv6Addr | 3GPP TS 29.122 [4] | Identifies an IPv6 address. |
| Ipv6Prefix | 3GPP TS 29.571 [8] | Identifies an IPv6 Prefix. |
| Link | 3GPP TS 29.122 [4] | Identifies a referenced resource. |
| MacAddr48 | 3GPP TS 29.571 [8] | Identifies a MAC address. |
| Port | 3GPP TS 29.122 [4] | Identifies a port number. |
| RouteToLocation | 3GPP TS 29.571 [8] | Describes the traffic routes to the locations of the application. |
| Snssai | 3GPP TS 29.571 [8] | Identifies the S-NSSAI. |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features defined in table 5.4.4-1. |
| TemporalValidity | 3GPP TS 29.514 [7] | Indicates the time interval(s) during which the AF request is to be applied |
| WebsockNotifConfig | 3GPP TS 29.122 [4] | Contains the configuration parameters to set up notification delivery over Websocket protocol. |

#### 5.4.3.3	Structured data types

##### 5.4.3.3.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.4.3.3.2	Type: TrafficInfluSub

This type represents a traffic influence subscription. The same structure is used in the subscription request and subscription response.

Table 5.4.3.3.2-1: Definition of type TrafficInfluSub

| Attribute name | Data type | P | Cardinality | Description | Applicability (NOTE 1) |
| --- | --- | --- | --- | --- | --- |
| afServiceId | string | O | 0..1 | Identifies a service on behalf of which the AF is issuing the request. |  |
| afAppId | string | O | 0..1 | Identifies an application. (NOTE 3) |  |
| afTransId | string | O | 0..1 | Identifies an NEF Northbound interface transaction, generated by the AF. |  |
| appReloInd | boolean | O | 0..1 | Identifies whether an application can be relocated once a location of the application has been selected. Set to "true" if it can be relocated; otherwise set to "false". Default value is "false" if omitted. |  |
| dnn | Dnn | O | 0..1 | Identifies a DNN. |  |
| snssai | Snssai | O | 0..1 | Identifies an S-NSSAI. |  |
| externalGroupId | ExternalGroupId | O | 0..1 | Identifies a group of users. (NOTE 2) |  |
| anyUeInd | boolean | O | 0..1 | Identifies whether the AF request applies to any UE. This attribute shall set to "true" if applicable for any UE, otherwise, set to "false". (NOTE 2) |  |
| subscribedEvents | array(SubscribedEvent) | O | 1..N | Identifies the requirement to be notified of the event(s). |  |
| gpsi | Gpsi | O | 0..1 | Identifies a user. (NOTE 2) |  |
| ipv4Addr | Ipv4Addr | O | 0..1 | Identifies the IPv4 address. (NOTE 2) |  |
| ipv6Addr | Ipv6Addr | O | 0..1 | Identifies the IPv6 address. (NOTE 2) |  |
| macAddr | MacAddr48 | O | 0..1 | Identifies the MAC address. |  |
| dnaiChgType | DnaiChangeType | O | 0..1 | Identifies a type of notification regarding UP path management event. |  |
| notificationDestination | Link | C | 0..1 | Contains the Callback URL to receive the notification from the NEF. It shall be present if the "subscribedEvents" is present. |  |
| requestTestNotification | boolean | O | 0..1 | Set to true by the SCS/AS to request the NEF to send a test notification as defined in subclause 5.2.5.3 of 3GPP TS 29.122 [4]. Set to false or omitted otherwise. | Notification_test_event |
| websockNotifConfig | WebsockNotifConfig | O | 0..1 | Configuration parameters to set up notification delivery over Websocket protocol. | Notification_websocket |
| self | Link | C | 0..1 | Link to the created resource. This parameter shall be supplied by the NEF in HTTP responses that include an object of TrafficInfluSub type |  |
| trafficFilters | array(FlowInfo) | O | 1..N | Identifies IP packet filters. (NOTE 3) |  |
| ethTrafficFilters | array(EthFlowDescription) | O | 1..N | Identifies Ethernet packet filters. (NOTE 3) |  |
| trafficRoutes | array(RouteToLocation) | O | 1..N | Identifies the N6 traffic routing requirement. |  |
| tempValidities | array(TemporalValidity) | O | 1..N | Indicates the time interval(s) during which the AF request is to be applied. |  |
| validGeoZoneIds | array(string) | O | 1..N | Identifies a geographic zone that the AF request applies only to the traffic of UE(s) located in this specific zone. |  |
| afAckInd | boolean | O | 0..1 | Identifies whether the AF acknowledgement of UP path event notification is expected. Set to "true" if the AF acknowledge is expected; otherwise set to "false". Default value is "false" if omitted. | AFRelocationAck |
| addrPreserInd | boolean | O | 0..1 | Indicates whether UE IP address should be preserved. This attribute shall set to "true" if preserved, otherwise, set to "false". Defalult value is "false" if omitted. | AddrPreservation |
| suppFeat | SupportedFeatures | C | 0..1 | Indicates the list of Supported features used as described in subclause 5.4.4. This attribute shall be provided in the POST request and in the response of successful resource creation. |  |
| NOTE 1: Properties marked with a feature as defined in subclause 5.4.4 are applicable as described in subclause 5.2.7 of 3GPP TS 29.122 [4]. If no feature is indicated, the related property applies for all the features. NOTE 2: One of individual UE identifier (i.e. "gpsi", "ipv4Addr" or "ipv6Addr"), External Group Identifier (i.e. "externalGroupId") or any UE indication "anyUeInd" shall be included. NOTE 3: One of "afAppId", "trafficFilters" or "ethTrafficFilters" shall be included. |  |  |  |  |  |

##### 5.4.3.3.3	Type: TrafficInfluSubPatch

This type represents a subscription of traffic influence parameters provided by the AF to the NEF. The structure is used for HTTP PATCH request.

Table 5.4.3.3.3-1: Definition of type TrafficInfluSubPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| appReloInd | boolean | O | 0..1 | Identifies whether an application can be relocated once a location of the application has been selected. (NOTE) |  |
| trafficFilters | array(FlowInfo) | O | 1..N | Identifies IP packet filters. |  |
| ethTrafficFilters | array(EthFlowDescription) | O | 1..N | Identifies Ethernet packet filters. |  |
| trafficRoutes | array(RouteToLocation) | O | 1..N | Identifies the N6 traffic routing requirement. (NOTE) |  |
| tempValidities | array(TemporalValidity) | O | 1..N | Indicates the time interval(s) during which the AF request is to be applied. (NOTE) |  |
| validGeoZoneIds | array(string) | O | 1..N | Identifies a geographic zone that the AF request applies only to the traffic of UE(s) located in this specific zone. (NOTE) |  |
| afAckInd | boolean | O | 0..1 | Identifies whether the AF acknowledgement of UP path event notification is expected. | AFRelocationAck |
| addrPreserInd | boolean | O | 0..1 | Indicates whether UE IP address should be preserved. (NOTE) | AddrPreservation |
| NOTE: The value of the property shall be set to NULL for removal. |  |  |  |  |  |

##### 5.4.3.3.4	Type: EventNotification

Table 5.4.3.3.4-1: Definition of type EventNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability (NOTE 1) |
| --- | --- | --- | --- | --- | --- |
| afTransId | string | O | 0..1 | Identifies an NEF Northbound interface transaction, generated by the AF. |  |
| dnaiChgType | DnaiChangeType | M | 1 | Identifies the type of notification regarding UP path management event. |  |
| sourceTrafficRoute | RouteToLocation | O | 0..1 | Identifies the N6 traffic routing information associated to the source DNAI. May be present if the "subscribedEvent" sets to "UP_PATH_CHANGE". (NOTE 3) |  |
| subscribedEvent | SubscribedEvent | M | 1 | Identifies a UP path management event the AF requested to be notified of. |  |
| targetTrafficRoute | RouteToLocation | O | 0..1 | Identifies the N6 traffic routing information associated to the target DNAI. May be present if the "subscribedEvent" sets to "UP_PATH_CHANGE". (NOTE 3) |  |
| sourceDnai | Dnai | O | 0..1 | Source DN Access Identifier. Shall be included for event "UP_PATH_CH" if the DNAI changed (NOTE 2, NOTE 3). |  |
| targetDnai | Dnai | O | 0..1 | Target DN Access Identifier. Shall be included for event "UP_PATH_CH" if the DNAI changed (NOTE 2, NOTE 3). |  |
| gpsi | Gpsi | O | 0..1 | Identifies a user. |  |
| srcUeIpv4Addr | Ipv4Addr | O | 0..1 | The IPv4 Address of the served UE for the source DNAI. |  |
| srcUeIpv6Prefix | Ipv6Prefix | O | 0..1 | The Ipv6 Address Prefix of the served UE for the source DNAI. |  |
| tgtUeIpv4Addr | Ipv4Addr | O | 0..1 | The IPv4 Address of the served UE for the target DNAI. |  |
| tgtUeIpv6Prefix | Ipv6Prefix | O | 0..1 | The Ipv6 Address Prefix of the served UE for the target DNAI. |  |
| ueMac | MacAddr48 | O | 0..1 | UE MAC address of the served UE. |  |
| NOTE 1: Properties marked with a feature as defined in subclause 5.4.4 are applicable as described in subclause 5.2.7 of 3GPP TS 29.122 [4]. If no feature is indicated, the related property applies for all the features. NOTE 2: If the DNAI is not changed while the N6 traffic routing information is changed, the "sourceDnai" attribute and "targetDnai" attribute shall not be provided. NOTE 3: The change from the UP path status where no DNAI applies to a status where a DNAI applies indicates the activation of the related AF request and therefore only the target DNAI and N6 traffic routing information is provided in the event notification; the change from the UP path status where a DNAI applies to a status where no DNAI applies indicates the de-activation of the related AF request and therefore only the source DNAI and N6 traffic routing information is provided in the event notification. |  |  |  |  |  |

##### 5.4.3.3.5	Type: AfResultInfo

Table 5.4.3.3.5-1: Definition of type AfResultInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| afStatus | AfResultStatus | M | 1 | Identifies the result of the application relocation. |
| trafficRoute | RouteToLocation | O | 0..1 | Identifies the N6 traffic routing information associated to the target DNAI. May only be present if the "afStatus" sets to "SUCCESS". |

#### 5.4.3.4	Simple data types and enumerations

##### 5.4.3.4.1	Introduction

This subclause defines simple data types and enumerations that can be referenced from data structures defined in the previous subclauses.

##### 5.4.3.4.2	Simple data types

The simple data types defined in table 5.4.3.4.2-1 shall be supported.

Table 5.4.3.4.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 5.4.3.4.3	Enumeration: SubscribedEvent

The enumeration SubscribedEvent represents the type of UP patch management events of which the AF requests to be notified. It shall comply with the provisions defined in table 5.4.3.4.3-1.

Table 5.4.3.4.3-1: Enumeration SubscribedEvent

| Enumeration value | Description |
| --- | --- |
| UP_PATH_CHANGE | The AF requests to be notified when the UP path changes for the PDU session. |

##### 5.4.3.4.4	Enumeration: AfResultStatus

The enumeration AfResultStatus represents the status of application handling result. It shall comply with the provisions defined in table 5.4.3.4.4-1.

Table 5.4.3.4.4-1: Enumeration AfResultStatus

| Enumeration value | Description |
| --- | --- |
| SUCCESS | The application layer is ready or the relocation is completed. |
| TEMP_CONGESTION | The application relocation fails due to temporary congestion. |
| RELOC_NO_ALLOWED | The application relocation fails because application relocation is not allowed. |
| OTHER | The application relocation fails due to other reason. |

### 5.4.4	Used Features

The table below defines the features applicable to the TrafficInfluence API. Those features are negotiated as described in subclause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.4.4-1: Features used by TrafficInfluence API

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_websocket | The delivery of notifications over Websocket is supported as described in 3GPP TS 29.122 [4]. This feature requires that the Notification_test_event feature is also supported. |
| 2 | Notification_test_event | The testing of notification connection is supported as described in 3GPP TS 29.122 [4]. |
| 3 | AFRelocationAck | This feature indicates support of AF application relocation acknowledgement requirement. |
| 4 | AddrPreservation | Indicates support of UE address(es) preservation requirement. |

Editor’s Note:	It’s FFS that whether the AddrPreservation can be merged with other Rel-16 feature.

## 5.5	NiddConfigurationTrigger API

### 5.5.1	Resources

There is no resource defined for this API.

### 5.5.2	Notifications

#### 5.5.2.1	Introduction

Upon receipt of a NIDD connection establishment request from the SMF and there is no NIDD configuration for the UE, the NEF may send an HTTP POST message in order to trigger the AF to start the NIDD configuration procedure as described in subclause 5.6.3.2.3.4 of 3GPP TS 29.122 [4].

#### 5.5.2.2	Event Notification

URI: {notificationUrI}

The operation shall support the URI variables defined in table 5.5.2.2-1.

Table 5.5.2.2-1: URI variables

| Name | Definition |
| --- | --- |
| notificationDestination | A URI indicating the notification destination where N33 notification requests shall be delivered to. This URI shall be preconfigured in the NEF. |

#### 5.5.2.3	Operation Definition

##### 5.5.2.3.1	Notification via HTTP POST

This method shall support the request data structures specified in table 5.5.2.3.1-1 and the response data structures and response codes specified in table 5.5.2.3.1-2.

Table 5.5.2.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| NiddConfigTrigger | M | 1 | The NIDD Configuration Trigger is provided by the NEF to the AF. |

Table 5.5.2.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| NiddConfigTriggerReply | M | 1 | 200 OK | The trigger is received successfully. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

##### 5.5.2.3.2	Notification via Websocket

Not specified in the present specification.

### 5.5.3	Data Model

#### 5.5.3.1	General

This subclause specifies the application data model supported by the NiddConfigurationTrigger API.

#### 5.5.3.2	Reused data types

The data types reused by the NiddConfigurationTrigger API from other specifications are listed in table 5.5.3.2-1.

Table 5.5.3.2-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Gpsi | 3GPP TS 29.571 [8] | Identifies a GPSI. |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features defined in table 5.5.4-1. |

#### 5.5.3.3	Structured data types

##### 5.5.3.3.1	Introduction

This clause defines the structured data types to be used in resource representations.

##### 5.5.3.3.2	Type: NiddConfigTrigger

This type represents a NIDD configuration trigger which is sent from the NEF to the AF.

Table 5.5.3.3.2-1: Definition of type NiddConfigTrigger

| Attribute name | Data type | P | Cardinality | Description | Applicability (NOTE) |
| --- | --- | --- | --- | --- | --- |
| afId | string | M | 1 | Identifies the trigger receiving entity. |  |
| nefId | string | M | 1 | Identifies the trigger sending entity. |  |
| gpsi | Gpsi | M | 1 | Identifies a user. |  |
| suppFeat | SupportedFeatures | M | 1 | Indicates the list of Supported features used as described in subclause 5.5.4. |  |
| NOTE: Properties marked with a feature as defined in subclause 5.5.4 are applicable as described in subclause 5.2.7 of 3GPP TS 29.122 [4]. If no feature is indicated, the related property applies for all the features. |  |  |  |  |  |

##### 5.5.3.3.3	Type: NiddConfigTriggerReply

This data type represents a reply to a NIDD configuration trigger and is sent from the AF to the NEF.

Table 5.15.2.1.3-1: Definition of type NiddConfigTriggerReply

| Attribute name | Data type | P | Cardinality | Description | Applicability (NOTE) |
| --- | --- | --- | --- | --- | --- |
| suppFeat | SupportedFeatures | M | 1 | Indicates the list of Supported features used as described in subclause 5.5.4. |  |
| NOTE: Properties marked with a feature as defined in subclause 5.5.4 are applicable as described in subclause 5.2.7 of 3GPP TS 29.122 [4]. If no feature is indicated, the related property applies for all the features. |  |  |  |  |  |

#### 5.5.3.4	Simple data types and enumerations

##### 5.5.3.4.1	Introduction

This subclause defines simple data types and enumerations that can be referenced from data structures defined in the previous subclauses.

##### 5.5.3.4.2	Simple data types

The simple data types defined in table 5.5.3.4.2-1 shall be supported.

Table 5.5.3.4.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

### 5.5.4	Used Features

The table below defines the features applicable to the NiddConfigurationTrigger API. Those features are negotiated as described in subclause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.5.4-1: Features used by NiddConfigurationTrigger API

| Feature number | Feature Name | Description |
| --- | --- | --- |

# 6	Security

TLS (IETF RFC 5246 [16]) shall be used to support the security communication between the NEF and the AF over NEF Northbound interface as defined in subclause 12 of 3GPP TS 33.501 [6]. The access to the SCEF northbound APIs shall be authorized by means of OAuth2 protocol (see IETF RFC 6749 [13]), based on local configuration, using the "Client Credentials" authorization grant. If OAuth2 is used, a client, prior to consuming services offered by the NEF Northbound APIs, shall obtain a "token" from the authorization server.

# 7	Using Common API Framework

## 7.1	General

When CAPIF is used with an NEF that is used for external exposure, the NEF shall support the following as defined in 3GPP TS 29.222 [12]:

-	the API exposing function and related APIs over CAPIF-2/2e and CAPIF-3 reference points;

-	the API publishing function and related APIs over CAPIF-4 reference point;

-	the API management function and related APIs over CAPIF-5 reference point; and

-	at least one of the security methods for authentication and authorization, and related security mechanisms.

In a centralized deployment as defined in 3GPP TS 23.222 [11], where the CAPIF core function and API provider domain functions are co-located, the interactions between the CAPIF core function and API provider domain functions may be independent of CAPIF-3, CAPIF-4 and CAPIF-5 reference points.

## 7.2	Security

When CAPIF is used for external exposure, before invoking the API exposed by the NEF, the AF as API invoker shall negotiate the security method (PKI, TLS-PSK or OAUTH2) with CAPIF core function and ensure the NEF has enough credential to authenticate the AF (see 3GPP TS 29.222 [12], subclause 5.6.2.2 and subclause 6.2.2.2).

If PKI or TLS-PSK is used as the selected security method between the AF and the NEF, upon API invocation, the NEF shall retrieve the authorization information from the CAPIF core function as described in 3GPP TS 29.222 [12], subclause 5.6.2.4.

As indicated in 3GPP TS 33.122 [14], the access to the NEF northbound APIs may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [13]), using the "Client Credentials" authorization grant, where the CAPIF core function (see 3GPP TS 29.222 [12]) plays the role of the authorization server.

NOTE 1:	In this release, only "Client Credentials" authorization grant is supported.

If OAuth2 is used as the selected security method between the AF and the NEF, the AF, prior to consuming services offered by the NEF northbound APIs, shall obtain a "token" from the authorization server, by invoking the Obtain_Authorization service, as described in 3GPP TS 29.222 [12], subclause 5.6.2.3.2.

The NEF northbound APIs do not define any scopes for OAuth2 authorization. It is the NEF responsibility to check whether the AF is authorized to use an API based on the "token". Once the NEF verifies the "token", it shall check whether the NEF identifier in the "token" matches its own published identifier, and whether the API name in the "token" matches its own published API name. If those checks are passed, the AF has full authority to access any resource or operation for the invoked API.

NOTE 2:	For aforementioned security methods, the NEF needs to apply admission control according to access control policies after performing the authorization checks.

NOTE 3:	The security requirement in the current subclause does not apply for the NiddConfigurationTrigger and the MsisdnLessMoSms APIs since they are the NEF initiated interaction with the AF. How the security scheme works for the NiddConfigurationTrigger and MsisdnLessMoSms APIs is left to configuration.

Annex A (normative):
OpenAPI representation for NEF Northbound APIs

# A.1	General

This Annex is based on the OpenAPI 3.0.0 specification [5] and provides corresponding representations of all APIs defined in the present specification.

NOTE 1:	An OpenAPIs representation embeds JSON Schema representations of HTTP message bodies.

This Annex shall take precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 2:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on the public 3GPP file server in the following locations (see clause 5B of the 3GPP TR 21.900 [21] for further information):

-	https://www.3gpp.org/ftp/Specs/archive/OpenAPI/<Release>/, and

-	https://www.3gpp.org/ftp/Specs/<Plenary>/<Release>/OpenAPI/.

NOTE 3:	To fetch the OpenAPI specification files after CT#83 plenary meeting for Release 15 in the above links <Plenary> must be replaced with the date the CT Plenary occurs, in the form of year-month (yyyy-mm), e.g. for CT#83 meeting <Plenary> must be replaced with value "2019-03" and <Release> must be replaced with value "Rel-15".

# A.2	TrafficInfluence API

openapi: 3.0.0

info:

title: 3gpp-traffic-influence

version: 1.1.0.alpha-1

description: |

API for AF traffic influence

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.522 V16.0.0; 5G System; Network Exposure Function Northbound APIs.

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.522/'

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/3gpp-traffic-influence/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 5.2.4 of 3GPP TS 29.122.

paths:

/{afId}/subscriptions:

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

get:

summary: read all of the active subscriptions for the AF

tags:

- TrafficInfluence API SCS/AS level GET Operation

responses:

'200':

description: OK.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/TrafficInfluSub'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Creates a new subscription resource

tags:

- TrafficInfluence API Subscription level POST Operation

requestBody:

description: Request to create a new subscription resource

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TrafficInfluSub'

callbacks:

notificationDestination:

'{request.body#/notificationDestination}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EventNotification'

responses:

'204':

description: No Content (successful notification)

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: Created (Successful creation of subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/TrafficInfluSub'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/{afId}/subscriptions/{subscriptionId}:

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

- name: subscriptionId

in: path

description: Identifier of the subscription resource

required: true

schema:

type: string

get:

summary: read an active subscriptions for the SCS/AS and the subscription Id

tags:

- TrafficInfluence API Subscription level GET Operation

responses:

'200':

description: OK (Successful get the active subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/TrafficInfluSub'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Updates/replaces an existing subscription resource

tags:

- TrafficInfluence API subscription level PUT Operation

requestBody:

description: Parameters to update/replace the existing subscription

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TrafficInfluSub'

responses:

'200':

description: OK (Successful update of the subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/TrafficInfluSub'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Updates/replaces an existing subscription resource

tags:

- TrafficInfluence API subscription level PATCH Operation

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/TrafficInfluSubPatch'

responses:

'200':

description: OK. The subscription was modified successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/TrafficInfluSub'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes an already existing subscription

tags:

- TrafficInfluence API Subscription level DELETE Operation

responses:

'204':

description: No Content (Successful deletion of the existing subscription)

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

TrafficInfluSub:

type: object

properties:

afServiceId:

type: string

description: Identifies a service on behalf of which the AF is issuing the request.

afAppId:

type: string

description: Identifies an application.

afTransId:

type: string

description: Identifies an NEF Northbound interface transaction, generated by the AF.

appReloInd:

type: boolean

description: Identifies whether an application can be relocated once a location of the application has been selected.

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

externalGroupId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/ExternalGroupId'

anyUeInd:

type: boolean

description: Identifies whether the AF request applies to any UE. This attribute shall set to "true" if applicable for any UE, otherwise, set to "false".

subscribedEvents:

type: array

items:

$ref: '#/components/schemas/SubscribedEvent'

minItems: 1

description: Identifies the requirement to be notified of the event(s).

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

ipv4Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv4Addr'

ipv6Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv6Addr'

macAddr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MacAddr48'

dnaiChgType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DnaiChangeType'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Link'

requestTestNotification:

type: boolean

description: Set to true by the SCS/AS to request the NEF to send a test notification as defined in subclause 5.2.5.3. Set to false or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

self:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Link'

trafficFilters:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/FlowInfo'

minItems: 1

description: Identifies IP packet filters.

ethTrafficFilters:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/EthFlowDescription'

minItems: 1

description: Identifies Ethernet packet filters.

trafficRoutes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RouteToLocation'

minItems: 1

description: Identifies the N6 traffic routing requirement.

tempValidities:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/TemporalValidity'

validGeoZoneIds:

type: array

items:

type: string

minItems: 1

description: Identifies a geographic zone that the AF request applies only to the traffic of UE(s) located in this specific zone.

afAckInd:

type: boolean

addrPreserInd:

type: boolean

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

allOf:

- oneOf:

- required: [afAppId]

- required: [trafficFilters]

- required: [ethTrafficFilters]

- oneOf:

- required: [ipv4Addr]

- required: [ipv6Addr]

- required: [macAddr]

- required: [gpsi]

- required: [externalGroupId]

- required: [anyUeInd]

anyOf:

- not:

required: [subscribedEvents]

- required: [notificationDestination]

TrafficInfluSubPatch:

type: object

properties:

appReloInd:

type: boolean

description: Identifies whether an application can be relocated once a location of the application has been selected.

nullable: true

trafficFilters:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/FlowInfo'

minItems: 1

description: Identifies IP packet filters.

ethTrafficFilters:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/EthFlowDescription'

minItems: 1

description: Identifies Ethernet packet filters.

trafficRoutes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RouteToLocation'

minItems: 1

description: Identifies the N6 traffic routing requirement.

tempValidities:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/TemporalValidity'

minItems: 1

nullable: true

validGeoZoneIds:

type: array

items:

type: string

minItems: 1

description: Identifies a geographic zone that the AF request applies only to the traffic of UE(s) located in this specific zone.

nullable: true

afAckInd:

type: boolean

nullable: true

addrPreserInd:

type: boolean

nullable: true

EventNotification:

type: object

properties:

afTransId:

type: string

description: Identifies an NEF Northbound interface transaction, generated by the AF.

dnaiChgType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DnaiChangeType'

sourceTrafficRoute:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RouteToLocation'

subscribedEvent:

$ref: '#/components/schemas/SubscribedEvent'

targetTrafficRoute:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RouteToLocation'

sourceDnai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

targetDnai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

srcUeIpv4Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv4Addr'

srcUeIpv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

tgtUeIpv4Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv4Addr'

tgtUeIpv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

ueMac:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MacAddr48'

required:

- dnaiChgType

- subscribedEvent

AfResultInfo:

type: object

properties:

afStatus:

$ref: '#/components/schemas/AfResultStatus'

trafficRoute:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RouteToLocation'

required:

- appStatus

SubscribedEvent:

anyOf:

- type: string

enum:

- UP_PATH_CHANGE

- type: string

description: >

Possible values are

- CHANGE_OF_DNAI: The AF requests to be notified when the UP path changes for the PDU session.

AfResultStatus:

anyOf:

- type: string

enum:

- SUCCESS

- TEMPORARY_CONGESTION

- RELOC_NO_ALLOWED

- OTHER

- type: string

description: >

Possible values are

- SUCCESS: The application layer is ready or the relocation is completed.

- TEMPORARY_CONGESTION: The application relocation fails due to temporary congestion.

- RELOC_NO_ALLOWED: The application relocation fails because application relocation is not allowed.

- OTHER: The application relocation fails due to other reason.

# A.3	NiddConfigurationTrigger API

openapi: 3.0.0

info:

title: 3gpp-nidd-configuration-trigger

version: 1.0.0.alpha-1

description: |

API for NIDD Configuration Trigger.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.522 V16.0.0; 5G System; Network Exposure Function Northbound APIs.

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.522/'

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 5.2.4 of 3GPP TS 29.122.

paths:

/:

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NiddConfigurationTrigger'

responses:

'200':

description: Success

content:

application/json:

schema:

$ref: '#/components/schemas/NiddConfigurationTriggerReply'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

NiddConfigurationTrigger:

type: object

properties:

afId:

type: string

description: Identifies the trigger receiving entity.

nefId:

type: string

description: Identifies the trigger sending entity.

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- afId

- nefId

- gpsi

- suppFeat

NiddConfigurationTriggerReply:

type: object

properties:

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- suppFeat

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc. | CR | Rev | Cat | Subject/Comment | New |
| 2018-03 | CT3#95 |  |  |  |  | TS Skeleton | 0.0.0 |
| 2018-03 | CT3#95 |  |  |  |  | Inclusion of C3-181332 and TS skeleton of Network Exposure Function Northbound APIs in C3-181362. | 0.1.0 |
| 2018-04 | CT3#96 |  |  |  |  | Inclusion of C3-182407, C3-182408, C3-182504, C3-182418, C3-182505, C3-182443, C3-182421, C3-182422, C3-182501 and editorial changes from Rapporteur. | 0.2.0 |
| 2018-05 | CT3#97 |  |  |  |  | Inclusion of C3-183187, C3-183773, C3-183774, C3-183553, C3-183826, C3-183329, C3-183776, C3-183827, C3-183778, C3-183605 and editorial changes from Rapporteur. | 0.3.0 |
| 2018-06 | CT#80 |  |  |  |  | TS sent to plenary for approval | 1.0.0 |
| 2018-06 | CT#80 |  |  |  |  | TS approved by plenary | 15.0.0 |
| 2018-09 | CT#81 | CP-182015 | 0001 | 1 | F | DNAI change notification type | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0002 | - | F | Corrections on NEF Northbound interface | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0003 | 1 | F | TrafficInfluence API OpenAPI schema | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0004 | 1 | F | AF influence traffic routing cleanup | 15.1.0 |
| 2018-09 | CT#81 | CP-182031 | 0005 | 1 | F | Definition of Changing the Chargeable Party procedures and API | 15.1.0 |
| 2018-09 | CT#81 | CP-182031 | 0006 | 1 | F | Definition of setting up an AS session with required QoS procedure and API | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0007 | 2 | F | Resource structure update | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0008 | - | F | Procedures for monitoring – Reference | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0009 | - | F | Ethernet packet filter for AF traffic influence API | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0010 | 3 | F | Removable attribute definition for AF traffic influence | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0011 | - | F | Supported feature for AF traffic influence | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0012 | - | F | Version numbering change | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0013 | - | F | Removal of externaldocs field | 15.1.0 |
| 2018-09 | CT#81 | CP-182035 | 0014 | 1 | F | PFD Management Service Operation | 15.1.0 |
| 2018-12 | CT#82 | CP-183205 | 0015 | 2 | F | ExternalDocs field | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0019 | - | F | Default value for apiRoot | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0021 | 4 | F | Correct traffic route and Ethernet flow data type | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0022 | 1 | F | Event correction for AF influence traffic routing | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0024 | 1 | F | Supporting Ethernet UE in Chargeable Party and AF session with QoS | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0025 | 1 | F | Add AF application ID for traffic influence | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0026 | 1 | F | Add BSF interaction for Chargeable Party and Required QoS | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0028 | 2 | F | Security field | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0029 | 1 | F | Corrections on subscribed event | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0030 | 1 | F | Status code update for TrafficInfluence API | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0031 | 3 | F | UE information during notification | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0017 | 2 | F | Error status codes for HTTP response | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0016 | 3 | F | Support of 5G location requirement | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0023 | 2 | F | Correction to the AF influence traffic steering control | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0032 | - | F | Location header | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0033 | 1 | F | API Version Update | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0034 | 1 | F | Support of 5G SUPI-PEI association | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0035 | 1 | F | Clarification of default value for boolean data type | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0027 | 2 | F | Security adaptation for Nnef northbound APIs with CAPIF | 15.2.0 |
| 2019-03 | CT#83 | CP-190116 | 0037 | 2 | F | Event notification | 15.3.0 |
| 2019-03 | CT#83 | CP-190116 | 0038 | 1 | F | Correction on MacAddr48 and RouteToLocation data type reference in the OpenAPI file | 15.3.0 |
| 2019-03 | CT#83 | CP-190116 | 0040 | 1 | F | Correction on mandatory 5G features | 15.3.0 |
| 2019-03 | CT#83 | CP-190116 | 0041 | - | F | OpenAPI Version number update | 15.3.0 |
| 2019-06 | CT#84 | CP-191080 | 0042 | 4 | F | Resource structure and AF Identifier | 15.4.0 |
| 2019-06 | CT#84 | CP-191080 | 0048 | 2 | F | UDM interaction for AF influence traffic | 15.4.0 |
| 2019-06 | CT#84 | CP-191080 | 0049 | 2 | F | Correct condition for DNAI in UP path change | 15.4.0 |
| 2019-06 | CT#84 | CP-191080 | 0053 | 1 | F | Precedence of OpenAPI file | 15.4.0 |
| 2019-06 | CT#84 | CP-191080 | 0059 | 1 | F | Copyright Note in YAML file | 15.4.0 |
| 2019-06 | CT#84 | CP-191090 | 0047 | 1 | B | Support of external group Id | 16.0.0 |
| 2019-06 | CT#84 | CP-191070 | 0043 | 2 | B | Nnef_MSISDN-less_MO_SMS service | 16.0.0 |
| 2019-06 | CT#84 | CP-191070 | 0044 | 2 | B | Application function notification of downlink data delivery status | 16.0.0 |
| 2019-06 | CT#84 | CP-191070 | 0045 | 2 | B | Availability after DDN failure notification for multiple Afs | 16.0.0 |
| 2019-06 | CT#84 | CP-191070 | 0050 | 2 | B | Network parameter provisioning support | 16.0.0 |
| 2019-06 | CT#84 | CP-191070 | 0051 | 3 | B | NIDD configuration and delivery in 5G | 16.0.0 |
| 2019-06 | CT#84 | CP-191229 | 0054 | 5 | B | AF acknowledgement of UP path event notification | 16.0.0 |
| 2019-06 | CT#84 | CP-191071 | 0055 | 2 | B | UE IP address preservation indication | 16.0.0 |
| 2019-06 | CT#84 | CP-191104 | 0056 | 1 | B | PFD management notification | 16.0.0 |
| 2019-06 | CT#84 | CP-191100 | 0057 | 1 | B | NEF stored exposure data | 16.0.0 |
| 2019-06 | CT#84 | CP-191105 | 0058 | 1 | B | BDT Warning Notification Support | 16.0.0 |
| 2019-06 | CT#84 | CP-191101 | 0061 | 1 | F | API version update | 16.0.0 |