3GPP TS 29.591 V16.0.0 (2020-03)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

5G System; Network Exposure Function Southbound Services

Stage 3

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions, symbols and abbreviations	8

3.1	Definitions	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Services offered by the NEF	8

4.1	Introduction	8

4.2	Nnef_EventExposure Service	9

4.2.1	Service Description	9

4.2.1.1	Overview	9

4.2.1.2	Service Architecture	9

4.2.1.3	Network Functions	10

4.2.1.3.1	Network Exposure Function (NEF)	10

4.2.1.3.2	NF Service Consumers	10

4.2.2	Service Operations	11

4.2.2.1	Introduction	11

4.2.2.2	Nnef_EventExposure_Subscribe service operation	11

4.2.2.2.1	General	11

4.2.2.2.2	Creating a new subscription	12

4.2.2.2.3	Modifying an existing subscription	13

4.2.2.3	Nnef_EventExposure_UnSubscribe service operation	14

4.2.2.3.1	General	14

4.2.2.3.2	Unsubscription from event notifications	14

4.2.2.4	Nnef_EventExposure_Notify service operation	15

4.2.2.4.1	General	15

4.2.2.4.2	Notification about subscribed events	15

5	API Definitions	16

5.1	Nnef_EventExposure Service API	16

5.1.1	Introduction	16

5.1.2	Usage of HTTP	16

5.1.2.1	General	16

5.1.2.2	HTTP standard headers	17

5.1.2.2.1	General	17

5.1.2.2.2	Content type	17

5.1.2.3	HTTP custom headers	17

5.1.3	Resources	17

5.1.3.1	Overview	17

5.1.3.2	Resource: Network Exposure Event Subscriptions	18

5.1.3.2.1	Description	18

5.1.3.2.2	Resource Definition	18

5.1.3.2.3	Resource Standard Methods	18

5.1.3.2.3.1	POST	18

5.1.3.3	Resource: Individual Network Exposure Event Subscription	19

5.1.3.3.1	Description	19

5.1.3.3.2	Resource Definition	19

5.1.3.3.3	Resource Standard Methods	19

5.1.3.3.3.1	GET	19

5.1.3.3.3.2	PUT	20

5.1.3.3.3.3	DELETE	20

5.1.4	Custom Operations without associated resources	20

5.1.5	Notifications	21

5.1.5.1	General	21

5.1.5.2	Network Exposure Event Notification	21

5.1.5.2.1	Description	21

5.1.5.2.2	Target URI	21

5.1.5.2.3	Standard Methods	21

5.1.5.2.3.1	POST	21

5.1.6	Data Model	21

5.1.6.1	General	21

5.1.6.2	Structured data types	22

5.1.6.2.1	Introduction	22

5.1.6.2.2	Type: NefEventExposureSubsc	23

5.1.6.2.3	Type: NefEventExposureNotif	23

5.1.6.2.4	Type: NefEventNotification	24

5.1.6.2.5	Type NefEventSubs	24

5.1.6.2.6	Type UeCommunicationInfo	24

5.1.6.2.7	Type NefEventFilter	25

5.1.6.2.8	Type TargetUeIdentification	25

5.1.6.2.9	Type: ServiceExperienceInfo	25

5.1.6.2.10	Type: UeMobilityInfo	26

5.1.6.2.11	Type: UeTrajectoryInfo	26

5.1.6.3	Simple data types and enumerations	26

5.1.6.3.1	Introduction	26

5.1.6.3.2	Simple data types	26

5.1.6.3.3	Enumeration: NefEvent	26

5.1.7	Error Handling	26

5.1.7.1	General	26

5.1.7.2	Protocol Errors	27

5.1.7.3	Application Errors	27

5.1.8	Feature negotiation	27

5.1.9	Security	27

Annex A (normative):	OpenAPI specification	28

A.1	General	28

A.2	Nnef_EventExposure API	28

Annex B (informative):	Change history	34

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, certain modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

NOTE 1:	The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

NOTE 2:	The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

NOTE 3:	The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

NOTE 4:	The constructions "can" and "cannot" shall not to be used as substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

NOTE 5:	The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the stage 3 protocol and data model for the Nnef southbound Service Based Interface. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the Network Exposure Function (NEF), Intermediate NEF (I-NEF).

The 5G System stage 2 architecture and procedures are specified in 3GPP TS 23.501 [2], 3GPP TS 23.502 [3] and 3GPP TS 23.288 [14].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition are specified in 3GPP TS 29.500 [4] and 3GPP TS 29.501 [5].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	OpenAPI: "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

[7]	3GPP TR 21.900: "Technical Specification Group working methods".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[11]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[12]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[13]	IETF RFC 7807: "Problem Details for HTTP APIs".

[14]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[15]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[16]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[17]	3GPP TS 29.520: "5G System; Network Data Analytics Services; Stage 3".

[18]	3GPP TS 29.517: "Application Function Event Exposure Service".

[19]	3GPP TS 29.551: "5G System; Packet Flow Description Management Service; Stage 3".

[20]	3GPP TS 29.541: "5G System; Network Exposure Function Services for Non-IP Data Delivery (NIDD); Stage 3".

[21]	3GPP TS 29.554: "5G System; Background Data Transfer Policy Control Service; Stage 3".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AF	Application Function

AMF	Access and Mobility Management Function

API	Application Programming Interface

I-NEF	Intermediate NEF

NEF	Network Exposure Function

NF	Network Function

NWDAF	Network Data Analytics Function

SMF	Session Management Function

# 4	Services offered by the NEF

## 4.1	Introduction

The NEF offers to other NFs the following southbound services:

-	Nnef_EventExposure

-	Nnef_PFDManagement

-	Nnef_SMContext

NOTE 1:	The northbound services offered by the NEF are defined in 3GPP TS 29.522 [15], e.g. the northbound requirement of Nnef_EventExposure service.

NOTE 2:	The services offered by the NEF (e.g. Nnef_EventExposure service) as specified in the present specification are only applicable for Nnef southbound services.

NOTE 3:	The Nnef_PFDManagement service offered by the NEF southbound is defined in 3GPP TS 29.551 [19].

NOTE 4:	The Nnef_SMContext service offered by the NEF southbound is defined in 3GPP TS 29.541 [20].

## 4.2	Nnef_EventExposure Service

### 4.2.1	Service Description

#### 4.2.1.1	Overview

The Nnef_EventExposure service, as defined in 3GPP TS 23.502 [3], is provided by the Network Exposure Function (NEF). The I-NEF acts as NEF for the roaming scenario.

This service:

-	allows NF service consumers to subscribe, modify and unsubscribe for application events; and

-	notifies NF service consumers with a corresponding subscription about observed events on the NEF.

The types of observed events applicable for NEF include:

-	Service experience;

-	UE mobility;

-	UE communication; and

-	Exceptions.

The types of observed events applicable for I-NEF include:

- Loss of connectivity;

- UE reachability;

- Location Reporting;

- Communication failure;

- Availability after Downlink Data Notification failure; and

- Downlink data delivery status.

The target of the event reporting may include one or more UE(s), a group of UEs or any UE (i.e. all UEs). When the event occurs, to which the NF service consumer has subscribed to, the NEF reports the requested information to the NF service consumer based on the event reporting information definition requested by the NF service consumer.

#### 4.2.1.2	Service Architecture

The 5G System Architecture is defined in 3GPP TS 23.501 [2]. The Network Data Analytics Exposure architecture is defined in 3GPP TS 23.288 [14].

The Nnef_EventExposure service is part of the Nnef service-based interface exhibited by the Network Exposure Function (NEF). The I-NEF acts as NEF for the roaming scenario.

Known consumers of the Nnef_EventExposure service in non-roaming scenario are:

-	Network Data Analytics Function (NWDAF)

Known consumers of the Nnef_EventExposure service in roaming scenario are:

-	Access and Mobility Management Function (AMF)

-	Session Management Function (SMF)

-	Network Exposure Function (NEF)

Figure 4.2.1.2-1: Reference Architecture for the Nnef_EventExposure Service; SBI representation

Figure 4.2.1.2-2: Reference Architecture for the Nnef_EventExposure Service: reference point representation

#### 4.2.1.3	Network Functions

##### 4.2.1.3.1	Network Exposure Function (NEF)

The Network Exposure Function (NEF) is a functional element that provides application or user related information to the NF service consumers as defined in current specification. It acts as I-NEF which is used for interworking between NFs in the VPLMN and the NEF. The I-NEF is optional and is only used in roaming scenarios. NFs in the VPLMN are configured with the I-NEF identity.

The (I-)NEF allows the NF consumer(s) to (un)subscribe to notification of monitoring observed event, and forwards the notification from the NF consumer(s) to the NEF when the event is detected by the NF consumer(s).

##### 4.2.1.3.2	NF Service Consumers

The known NF service consumers are as follows:

The Access and Mobility Management Function (AMF):

- supports (un)subscribe to monitoring event notification at I-NEF when UE is in roaming.

The Session Management Function (SMF):

- supports (un)subscribe to monitoring event notification at I-NEF when UE is in roaming.

The Network Data Analytics Function (NWDAF):

- supports (un)subscribing to notification of subscribed event(s) from the NEF;

- supports receiving the notification of subscribed event(s) from the NEF.

The Network Exposure Function (NEF):

- supports receiving monitored event report from the I-NEF when UE is in roaming.

### 4.2.2	Service Operations

#### 4.2.2.1	Introduction

Service operations defined for the Nnef_EventExposure Service are shown in table 4.2.2.1-1.

Table 4.2.2.1-1: Nnef_EventExposure Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| Nnef_EventExposure_Subscribe | This service operation is used by an NF service consumer to subscribe to, or modify a subscription in the (I-)NEF for event notifications on a specified application or user related event for one or more UE(s) or any UE. | NF service consumer |
| Nnef_EventExposure_Unsubscribe | This service operation is used by an NF service consumer to unsubscribe from event notifications. | NF service consumer |
| Nnef_EventExposure_Notify | This service operation is used by the (I-)NEF to report application or user related event(s) to the NF service consumer which has subscribed to the event report service. | NEF, I-NEF |

#### 4.2.2.2	Nnef_EventExposure_Subscribe service operation

##### 4.2.2.2.1	General

This service operation is used by an NF service consumer to subscribe for policy events notifications on a specified context for one or more UE(s) or any UE, or to modify an existing subscription.

The following are the types of events for which a subscription can be made:

-	Service experience;

-	UE mobility;

-	UE communication;

-	Exceptions;

-	Loss of connectivity;

-	UE reachability;

-	Location Reporting;

-	Communication failure;

-	Availability after Downlink Data Notification failure; and

-	Downlink data delivery status

The following procedures using the Nnef_EventExposure_Subscribe service operation are supported:

-	creating a new subscription;

-	modifying an existing subscription.

##### 4.2.2.2.2	Creating a new subscription

Figure 4.2.2.2.2-1 illustrates the creation of a subscription.

Figure 4.2.2.2.2-1: Creation of a subscription

To subscribe to event notifications, the NF service consumer shall send an HTTP POST request to the NEF with: "{apiRoot}/nnef-eventexposure/{apiVersion}/subscriptions/" as request URI as shown in step 1 of figure 4.2.2.2.2-1, and the "NefEventExposureSubsc" data structure as request body.

The "NefEventExposureSubsc" data structure shall include:

-	a URI where to receive the requested notifications as "notifUri" attribute; and

-	a Notification Correlation Identifier assigned by the NF service consumer for the requested notifications as "notifId" attribute.

For non-roaming case:

The "NefEventExposureSubsc" data structure shall include:

-	description of subscribed event information as "eventsSubs" attribute by using one or more "NefEventSubs" data;

-	description of the event reporting information as "eventsRepInfo" attribute;

The "NefEventSubs" data shall include:

-	a event to subscribe as a "event" attribute; and

-	event filter information as "eventFilter" attribute associated with the event.

The "eventsRepInfo" attribute may include:

-	event notification method (periodic, one time, on event detection) as "notifMethod" attribute;

-	Maximum Number of Reports as "maxReportNbr" attribute;

-	Monitoring Duration as "monDur" attribute;

-	repetition period for periodic reporting as "repPeriod" attribute;

-	immediate reporting indication as "immRep" attribute;

-	sampling ratio as "sampRatio" attribute; and/or

-	group reporting guard time as "grpRepTime" attribute.

If the NEF cannot successfully fulfil the received HTTP POST request due to the internal error or an error in the HTTP POST request, the NEF shall send the HTTP error response as specified in clause 5.1.7.

Upon successful reception of the HTTP POST request with "{apiRoot}/nnef-eventexposure/{apiVersion}/subscriptions/" as request URI and "NefEventExposureSubsc" data structure as request body, the NEF shall create a new "Individual Event Exposure Subscription" resource, shall store the subscription and shall send a HTTP "201 Created" response as shown in step 2 of figure 4.2.2.2.2-1. The NEF shall include in the "201 Created" response:

-	a Location header field; and

-	an "NefEventExposureSubsc" data type in the payload body.

The Location header field shall contain the URI of the created individual application session context resource i.e. "{apiRoot}/nnef-eventexposure/{apiVersion}/subscriptions/{subscriptionId}".

The "NefEventExposureSubsc" data type payload body shall contain the representation of the created "Individual Event Event Subscription".

When the "monDur" attribute is included in the response, it represents NEF selected expiry time that is equal or less than the received expiry time in the request.

When the "immRep" attribute is included and sets to "true" in the subscription and the subscribed events are available, the NEF shall immediately notify the NF service consumer using the Nnef_EventExposure_Notify service operation, as described in clause 4.2.2.4.

When the sampling ratio as the "sampRatio" attribute is included in the subscription, the NEF shall select a random subset of UEs among target UEs according to the sampling ratio and only report the event(s) related to the selected subset UEs.

When the group reporting guard time as the "grpRepTime" attribute is included in the subscription, the NEF shall accumulate all the event reports for the target UEs until the group reporting guard time expires. Then the NEF shall notify the NF service consumer using the Nnef_EventExposure_Notify service operation, as described in clause 4.2.2.4.

##### 4.2.2.2.3	Modifying an existing subscription

Figure 4.2.2.2.3-1 illustrates the modification of an existing subscription.

Figure 4.2.2.2.3-1: Modification of an existing subscription

To modify an existing subscription to event notifications, the NF service consumer shall send an HTTP PUT request with: "{apiRoot}/nnef-eventexposure/{apiVersion}/subscriptions/{subscriptionId}" as request URI, as shown in step 1 of figure 4.2.2.2.3-1, where "{subscriptionId}" is the subscription correlation ID of the existing subscription. The "NefEventExposureSubsc" data structure is included as request body as described in clause 4.2.2.2.2.

NOTE 1:	An alternate NF service consumer than the one that requested the generation of the subscription resource can send the PUT.

NOTE 2:	The "notifUri" attribute within the NefEventExposureSubsc data structure can be modified to request that subsequent notifications are sent to a new NF service consumer.

NOTE 3:	The "monDur" attribute within the NefEventExposureSubsc data structure can be modified to extend the expiry time to keep receiving notifications.

If the NEF cannot successfully fulfil the received HTTP PUT request due to the internal error or an error in the HTTP PUT request, the NEF shall send the HTTP error response as specified in clause 5.1.7.

Upon successful reception of an HTTP PUT request with: "{apiRoot}/nnef-eventexposure/{apiVersion}/subscriptions/{subscriptionId}" as request URI and "NefEventExposureSubsc" data structure as request body, the NEF shall store the subscription and shall send a HTTP "200 OK" response as shown in step 2 of figure 4.2.2.2.3-1, with the "NefEventExposureSubsc" data structure as response body.

The "NefEventExposureSubsc" data structure payload body shall contain the representation of the modified "Individual Application Event Subscription".

When the "monDur" attribute is included in the response, it represents NEF selected expiry time that is equal or less than the received expiry time in the request.

When the "immRep" attribute is included and sets to "true" in the subscription and the subscribed events are available, the NEF shall immediately notify the NF service consumer using the Nnef_EventExposure_Notify service operation, as described in clause 4.2.2.4.

When the sampling ratio as the "sampRatio" attribute is included in the subscription, the NEF shall select a random subset of UEs among target UEs according to the sampling ratio and only report the event(s) related to the selected subset UEs.

When the group reporting guard time as the "grpRepTime" attribute is included in the subscription, the NEF shall accumulate all the event reports for the target UEs until the group reporting guard time expires. Then the NEF shall notify the NF service consumer using the Nnef_EventExposure_Notify service operation, as described in clause 4.2.2.4.

#### 4.2.2.3	Nnef_EventExposure_UnSubscribe service operation

##### 4.2.2.3.1	General

This service operation is used by an NF service consumer to unsubscribe from event notifications.

The following procedure using the Nnef_EventExposure_UnSubscribe service operation is supported:

-	unsubscription from event notifications.

##### 4.2.2.3.2	Unsubscription from event notifications

Figure 4.2.2.3.2-1 illustrates the unsubscription from event notifications.

Figure 4.2.2.3.2-1: Unsubscription from event notifications

To unsubscribe from event notifications, the NF service consumer shall send an HTTP DELETE request with "{apiRoot}/nnef-eventexposure/{apiVersion}/subscriptions/{subscriptionId}" as request URI, as shown in step 1 of figure 4.2.2.3.2-1, where "{subscriptionId}" is the subscription correlation identifier of the existing resource subscription that is to be deleted.

If the NEF cannot successfully fulfil the received HTTP DELETE request due to the internal error or the error in the HTTP DELETE request, the NEF shall send the HTTP error response as specified in clause 6.1.7.

Upon successful reception of the HTTP DELETE request with: "{apiRoot}/nnef-eventexposure/{apiVersion}/subscriptions/{subscriptionId}" as request URI, the NEF shall remove the corresponding subscription and shall send an HTTP "204 No Content" response as shown in step 2 of figure 4.2.2.3.2-1.

#### 4.2.2.4	Nnef_EventExposure_Notify service operation

##### 4.2.2.4.1	General

The Nnef_EventExposure_Notify service operation enables the NEF to notify to the NF service consumer(s) that the previously subscribed application related event occurred.

The following procedure using the Nnef_EventExposure_Notify service operation is supported:

-	notification about subscribed events.

##### 4.2.2.4.2	Notification about subscribed events

Figure 4.2.2.4.2-1 illustrates the notification about subscribed events.

Figure 4.2.2.4.2-1: Notification about subscribed events

If the AF observes application related event(s) for which an NF service consumer has subscribed to, the AF shall send an HTTP POST request as shown in step 1 of figure 4.2.2.4.2-1, with the "{notifUri}" as request URI with the value previously provided by the NF service consumer within the corresponding subscription, and the "NefEventExposureNotif" data structure.

The "NefEventExposureNotif" data structure shall include:

-	notification correlation ID provided by the NF service consumer during the subscription as "notifId" attribute; and

-	information about the observed event(s) within the "eventNotifs" attribute that shall contain for each observed event an "NefEventNotification" data structure that shall include:

-	the application related event as "event" attribute;

-	the time at which the event was observed encoded as "timeStamp" attribute;

-	if the "event" attribute is "SVC_EXPERIENCE",

-	service experience information about the application involved in the reported event in the "svcExprcInfos" attribute;

-	if the "event" attribute is "UE_MOBILITY",

-	UE mobility information assoicated with the application as "ueMobilityInfos" attribute;

-	if the "event" attribute is "UE_COMM",

-	application communication information assoicated with the application as "ueCommuInfos" attribute;

and

-	if the "event" attribute is "EXCEPTIONS",

-	exceptions information associated with a service flow as "excepInfos" attribute.

If the NF service consumer cannot successfully fulfil the received HTTP POST request due to the internal error or an error in the HTTP POST request, the NF service consumer shall send the HTTP error response as specified in clause 5.1.7.

Upon successful reception of the HTTP POST request with "{notifUri}" as request URI and a "NefEventExposureNotif" data structure as request body, the NF service consumer shall send a "204 No Content" HTTP response, as shown in step 2 of figure 4.2.2.4.2-1, for a successful processing.

# 5	API Definitions

## 5.1	Nnef_EventExposure Service API

### 5.1.1	Introduction

The Nnef_EventExposure service shall use the Nnef_EventExposure API.

The request URI used in HTTP request from the NF service consumer towards the NF service producer shall have the structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/{apiVersion}/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "nnef-eventexposure".

-	The {apiVersion} shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 5.1.3.

### 5.1.2	Usage of HTTP

#### 5.1.2.1	General

HTTP/2, IETF RFC 7540 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Nnef_EventExposure API is contained in Annex A.

#### 5.1.2.2	HTTP standard headers

##### 5.1.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

##### 5.1.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [13].

#### 5.1.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be applicable.

In this Release of the specification, no specific custom headers are defined for the Nnef_EventExposure API.

### 5.1.3	Resources

#### 5.1.3.1	Overview

Figure 5.1.3.1-1: Resource URI structure of the Nnef_EventExposure API

Table 5.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 5.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Network Exposure Event Subscriptions | {apiRoot}/nnef-eventexposure/ {apiVersion}/subscriptions | POST | Subscription to the notification of application or user related event and creation of an Individual Network Exposure Event Subscription resource. |
| Individual Network Exposure Event Subscription | {apiRoot}/nnef-eventexposure/ {apiVersion}/subscriptions/ {subscriptionId} | GET | Reads an Individual Network Exposure Event Subscription resource. |
|  |  | PUT | Modifies an Individual Network Exposure Event Subscription. |
|  |  | DELETE | Cancels an individual subscription to notifications of subscribed event. |

#### 5.1.3.2	Resource: Network Exposure Event Subscriptions

##### 5.1.3.2.1	Description

The resource represents all subscriptions of the Nnef_EventExposure service. It allows the NF service consumers to create an new subscription to the notification of application or user related event.

##### 5.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/nnef-eventexposure/{apiVersion}/subscriptions

This resource shall support the resource URI variables defined in table 5.1.3.2.2-1.

Table 5.1.3.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 5.1.1 |
| apiVersion | See clause 5.1.1 |

##### 5.1.3.2.3	Resource Standard Methods

###### 5.1.3.2.3.1	POST

This method shall support the URI query parameters specified in table 5.1.3.2.3.1-1.

Table 5.1.3.2.3.1-1: URI query parameters supported by the <method 1> method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 5.1.3.2.3.1-2 and the response data structures and response codes specified in table 5.1.3.2.3.1-3.

Table 5.1.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| NnefEventExposureSubsc | M | 1 | Contains the information required for the creation of a new Individual Network Exposure Event Subscription resource. |

Table 5.1.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| NefEventExposureSubsc | M | 1 | 201 Created | Contains the representation of the Individual Network Exposure Event Subscription resource. |
| NOTE: The manadatory HTTP error status code for the <method 1> method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

#### 5.1.3.3	Resource: Individual Network Exposure Event Subscription

##### 5.1.3.3.1	Description

The resource represents all subscriptions of the Nnef_EventExposure service. It allows the NF service consumers to create an new subscription to the notification of application or user related event.

##### 5.1.3.3.2	Resource Definition

Resource URI: {apiRoot}/nnef-eventexposure/{apiVersion}/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 5.1.3.3.2-1.

Table 5.1.3.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 5.1.1 |
| apiVersion | See clause 5.1.1 |
| subscriptionId | String identifying a subscription to the NEF event exposure service. |

##### 5.1.3.3.3	Resource Standard Methods

###### 5.1.3.3.3.1	GET

This method shall support the URI query parameters specified in table 5.1.3.3.3.1-1.

Table 5.1.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.1.3.3.3.1-2 and the response data structures and response codes specified in table 5.1.3.3.3.1-3.

Table 5.1.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.1.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| NefEventExposureSubsc | M | 1 | 200 OK | Contains the representation of the Individual Network Exposure Event Subscription resource. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [5] also apply. |  |  |  |  |

###### 5.1.3.3.3.2	PUT

This method shall support the URI query parameters specified in table 5.1.3.3.3.2-1.

Table 5.1.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.1.3.3.3.2-2 and the response data structures and response codes specified in table 5.1.3.3.3.2-3.

Table 5.1.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| NefEventExposureSubsc | M | 1 | Modifies the existing Individual Network Exposure Event Subscription resource. |

Table 5.1.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| NefEventExposureSubsc | M | 1 | 200 OK | Successful case. The Individual Network Exposure Event Subscription resource was modified and a representation is returned. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [5] also apply. |  |  |  |  |

###### 5.1.3.3.3.3	DELETE

This method shall support the URI query parameters specified in table 5.1.3.3.3.3-1.

Table 5.1.3.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.1.3.3.3.3-2 and the response data structures and response codes specified in table 5.1.3.3.3.3-3.

Table 5.1.3.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.1.3.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The Individual Network Exposure Event Subscription resource matching the subscriptionId was deleted. |
| NOTE: The mandatory HTTP error status code for the DELETE method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [5] also apply. |  |  |  |  |

### 5.1.4	Custom Operations without associated resources

None.

### 5.1.5	Notifications

#### 5.1.5.1	General

Notifications shall comply to clause 6.2 of 3GPP TS 29.500 [4] and clause 4.6.2.3 of 3GPP TS 29.501 [5].

#### 5.1.5.2	Network Exposure Event Notification

##### 5.1.5.2.1	Description

The Network Exposure Event Notification is used by the NEF to report one or several observed Events to a NF service consumer that has subscribed to such Notifications.

##### 5.1.5.2.2	Target URI

The Notification URI "{notifUri}" shall be used with the resource URI variables defined in table 5.1.5.2.2-1.

Table 5.1.5.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| notifUri | String formatted as URI with the Notification Uri |

##### 5.1.5.2.3	Standard Methods

###### 5.1.5.2.3.1	POST

This method shall support the request data structures specified in table 5.1.5.2.3.1-1 and the response data structures and response codes specified in table 5.1.5.2.3.1-1.

Table 5.1.5.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| NefEventExposureNotif | M | 1 | Provides Information about observed events |

Table 5.1.5.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

### 5.1.6	Data Model

#### 5.1.6.1	General

This clause specifies the application data model supported by the API.

Table 5.1.6.1-1 specifies the data types defined for the Nnef_EventExposure service based interface protocol.

Table 5.1.6.1-1: Nnef_EventExposure specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| NefEvent | 5.1.6.3.3 |  |  |
| NefEventExposureNotif | 5.1.6.2.3 |  |  |
| NefEventExposureSubsc | 5.1.6.2.2 |  |  |
| NefEventFilter | 5.1.6.2.7 |  |  |
| NefEventNotification | 5.1.6.2.4 |  |  |
| NefEventSubs | 5.1.6.2.5 |  |  |
| ServiceExperienceInfo | 5.1.6.2.9 |  |  |
| TargetUeIdentification | 5.1.6.2.8 |  |  |
| UeCommunicationInfo | 5.1.6.2.6 |  |  |
| UeMobilityInfo | 5.1.6.2.10 |  |  |
| UeTrajectoryInfo | 5.1.6.2.11 |  |  |

Table 5.1.6.1-2 specifies data types re-used by the Nnef_EventExposure service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nnef_EventExposure service based interface.

Table 5.1.6.1-2: Nnef_EventExposure re-used Data Types

| Data type | Reference | Comments |  | Applicability |
| --- | --- | --- | --- | --- |
| ApplicationId | 3GPP TS 29.571 [16] |  |  |  |
| EventReportingRequirement | 3GPP TS 29.520 [17] |  |  |  |
| CommunicationCollection | 3GPP TS 29.517 [18] |  |  |  |
| DateTime | 3GPP TS 29.571 [16] |  |  |  |
| ExceptionInfo | 3GPP TS 29.517 [18] |  |  |  |
| GroupId | 3GPP TS 29.571 [16] |  |  |  |
| NetworkAreaInfo | 3GPP TS 29.554 [21] |  |  |  |
| Supi | 3GPP TS 29.571 [16] |  |  |  |
| SupportedFeatures | 3GPP TS 29.571 [16] |  |  |  |
| ServiceExperienceInfoPerFlow | 3GPP TS 29.517 [18] |  |  |  |
| UserLoaction | 3GPP TS 29.571 [16] |  |  |  |
| Uri | 3GPP TS 29.571 [16] |  |  |  |

#### 5.1.6.2	Structured data types

##### 5.1.6.2.1	Introduction

This subclause defines the structures to be used in resource representations.

##### 5.1.6.2.2	Type: NefEventExposureSubsc

Table 5.1.6.2.2-1: Definition of type NnefExposureSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventsSubs | array(NefEventSubs) | C | 1..N | Subscribed events and the related event filters. Shall be present if "event" sets to "SVC_EXPERIENCE", "UE_MOBILITY", "UE_COMM" or "EXCEPTIONS". | ServiceExperience UeCommunication UeMobility Exceptions |
| eventsRepInfo | EventReportingRequirement | C | 0..1 | Represents the reporting requirements of the subscription. Shall be present if "event" sets to "SVC_EXPERIENCE", "UE_MOBILITY", "UE_COMM" or "EXCEPTIONS". | ServiceExperience UeCommunication UeMobility Exceptions |
| notifUri | Uri | M | 1 | Notification URI for event reporting. |  |
| notifId | string | M | 1 | Notification Correlation ID assigned by the NF service consumer. |  |
| suppFeat | SupportedFeatures | M | 1 | This IE represents a list of Supported features used as described in subclause 5.8. (NOTE) |  |
| rspNotifUri | Uri | O | 0..1 | Notification URI of received notifcations to be forwarded. May only be present in the HTTP response for roaming case. | InefEventExposure |
| NOTE: In the HTTP POST request it represents the set of NF service consumer supported features. In the HTTP POST and GET responses it represents the set of features supported by both the NF service consumer and the NEF. |  |  |  |  |  |

Editor’s Note:	Whether more information will be added to support I-NEF Event Exposure is FFS.

##### 5.1.6.2.3	Type: NefEventExposureNotif

Table 5.1.6.2.3-1: Definition of type NefEventExposureNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifId | string | M | 1 | Notification Correlation ID assigned by the NF service consumer. |  |
| eventNotifs | array(NefEventNotification) | M | 1..N | Represents the Events to be reported according to the subscription corresponding to the Notification Correlation ID. |  |

##### 5.1.6.2.4	Type: NefEventNotification

Table 5.1.6.2.4-1: Definition of type NefEventNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | NefEvent | M | 1 | Reported application related event. |  |
| timeStamp | DateTime | M | 1 | Time at which the event is observed. |  |
| svcExprcInfos | array(ServiceExperienceInfo) | C | 1..N | Contains the service experience information. Shall be present if the "event" attribute sets to "SVC_EXPERIENCE" | ServiceExperience |
| ueMobilityInfos | array(UeMobilityInfo) | C | 1..N | Contains the UE mobility information. Shall be present if the "event" attribute sets to "UE_MOBILITY" | UeMobility |
| ueCommInfos | array(UeCommunicationInfo) | C | 1..N | Contains the application communication information. Shall be present if the "event" attribute sets to "UE_COMM" | UeCommunication |
| excepInfos | array(ExceptionInfo) | C | 1..N | Each element represents the exception information for a service flow. Shall be present if the "event" attribute sets to "EXCEPTIONS". | Exceptions |

Editor’s Note:	More information related to more event types will be defined later.

##### 5.1.6.2.5	Type NefEventSubs

Table 5.1.6.2.5-1: Definition of type NefEventSubs

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | NefEvent | M | 1 | Subscribed event. |  |
| eventFilter | NefEventFilter | C | 1 | Represents the event filter information associated with each event. Shall be present if "event" sets to "SVC_EXPERIENCE", "UE_MOBILITY", "UE_COMM" or "EXCEPTIONS". | ServiceExperience UeCommunication UeMobility Exceptions |

##### 5.1.6.2.6	Type UeCommunicationInfo

Table 5.1.6.2.6-1: Definition of type UeCommunicationInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supi | Supi | C | 0..1 | Identifies an UE. Shall be present if the event exposure request applies to more than one UE. |  |
| interGroupId | GroupId | O | 0..1 | Identifies an UE group. |  |
| appId | ApplicationId | O | 0..1 | Identifies an application identifier. |  |
| comms | array(CommunicationCollection) | M | 1..N | This attribute contains a list of communication information. |  |

##### 5.1.6.2.7	Type NefEventFilter

Table 5.1.6.2.7-1: Definition of type NefEventFilter

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| tgtUe | TargetUeIdentification | M | 1 | Represents the UE information to which the request applies. | (NOTE 1) |
| appIds | array(AppliationId) | C | 1..N | Each element indicates an application identifier. Shall be present if "event" sets to "SVC_EXPERIENCE" or "EXCEPTIONS". (NOTE 2) | ServiceExperience Exceptions |
| locArea | NetworkAreaInfo | O | 0..1 | Represents an area of interest. May only be present if "event" sets to "SVC_EXPERIENCE" or "EXCEPTIONS". | ServiceExperience Exceptions |
| NOTE 1: Applicability is further described in the corresponding data type. NOTE 2: For event "EXCEPTIONS", the "appIds" attribute shall include only one element. |  |  |  |  |  |

##### 5.1.6.2.8	Type TargetUeIdentification

Table 5.1.6.2.8-1: Definition of type TargetUeIdentification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supis | array(Supi) | O | 1..N | Each element identifies a SUPI for an UE. | ServiceExperience Exceptions UeMobility UeCommunication |
| interGroupIds | array(GroupId) | O | 1..N | Each element represents an internal group identifier which identifies a group of UEs. | ServiceExperience Exceptions UeMobility UeCommunication |
| anyUeId | boolean | O | 0..1 | Identifies whether the AF request applies to any UE. This attribute shall set to "true" if applicable for any UE, otherwise, set to "false". | ServiceExperience Exceptions |
| NOTE: For an applicable feature, only one attribute identifying the target UE shall be provided. |  |  |  |  |  |

##### 5.1.6.2.9	Type: ServiceExperienceInfo

Table 5.1.6.2.9-1: Definition of type ServiceExperienceInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| appId | ApplicationId | C | 0..1 | Identifies an application identifier. Shall be present if the event exposure service request applies to more than one application. | ServiceExperience |
| supis | array(Supi) | C | 1..N | Each element represents the internal UE identifier. | ServiceExperience |
| svcExpPerFlows | array(ServiceExperienceInfoPerFlow) | M | 1..N | Each element indicates service experience for each service flow. | ServiceExperience |

##### 5.1.6.2.10	Type: UeMobilityInfo

Table 5.1.6.2.10-1: Definition of type UeMobilityInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supi | Supi | M | 1 | Identifies an UE. Shall be present if the event exposure request applies to more than one UE. |  |
| appId | ApplicationId | O | 0..1 | Identifies an application identifier. |  |
| ueTrajs | array(UeTrajectoryInfo) | M | 1..N | Identifies an UE moving trajectory. |  |

##### 5.1.6.2.11	Type: UeTrajectoryInfo

Table 5.1.6.2.11-1: Definition of type UeTrajectoryInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ts | DateTime | M | 1 | Identifies the timestamp when the UE enteres this area. |  |
| location | UserLocation | M | 1 | Includes the location of the UE. |  |

#### 5.1.6.3	Simple data types and enumerations

##### 5.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 5.1.6.3.2	Simple data types

The simple data types defined in table 5.1.6.3.2-1 shall be supported.

Table 5.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 5.1.6.3.3	Enumeration: NefEvent

The enumeration NefEvent represents the observed event requested by the NF service consumer to be monitored. It shall comply with the provisions defined in table 5.1.6.3.3-1.

Table 5.1.6.3.3-1: Enumeration NefEvent

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| SVC_EXPERIENCE | Indicates that the observed event is service experience. | ServiceExperience |
| UE_COMM | Indicates that the observed event is UE communication. | UeCommunication |
| UE_MOBILITY | Indicates that the observed event is UE mobility. | UeMobility |
| EXCEPTIONS | Indicates that the observed event is exceptions information. | Exceptions |

### 5.1.7	Error Handling

#### 5.1.7.1	General

For the Nnef_EventExposure API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Nnef_EventExposure API.

#### 5.1.7.2	Protocol Errors

No specific procedures for the Nnef_EventExposure service are specified.

#### 5.1.7.3	Application Errors

The application errors defined for the Nnef_EventExposure service are listed in Table 5.1.7.3-1.

Table 5.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

### 5.1.8	Feature negotiation

The optional features in table 5.1.8-1 are defined for the Nnef_EventExposure API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 5.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | ServiceExperience | This feature indicates support for the "SVC_EXPERIENCE" event. |
| 2 | UeMobility | This feature indicates support for the "UE_MOBILITY" event. |
| 3 | UeCommunication | This feature indicates support for the "UE_COMM" event. |
| 4 | Exceptions | This feature indicates support for the "EXCEPTIONS" event. |
| 5 | InefEventExposure | This feature indicates support I-NEF event exposure in roaming status. |

### 5.1.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Nnef_EventExposure API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Nnef_EventExposure API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nnef_EventExposure service.

The Nnef_EventExposure API defines a single scope "nnef-eventexposure" for the entire service, and it does not define any additional scopes at resource or operation level.

Annex A (normative):
OpenAPI specification

## A.1	General

This Annex specifies the formal definition of the API(s) defined in the present specification. It consists of OpenAPI 3.0.0 specifications in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 1:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on the public 3GPP file server in the following locations (see clause 5B of the 3GPP TR 21.900 [7] for further information):

-	https://www.3gpp.org/ftp/Specs/archive/OpenAPI/<Release>/, and

-	https://www.3gpp.org/ftp/Specs/<Plenary>/<Release>/OpenAPI/.

NOTE 2:	To fetch the OpenAPI specification file after CT#83 plenary meeting for Release 15 in the above links <Plenary> must be replaced with the date the CT Plenary occurs, in the form of year-month (yyyy-mm), e.g. for CT#87 meeting <Plenary> must be replaced with value "2020-03" and <Release> must be replaced with value "Rel-16".

## A.2	Nnef_EventExposure API

openapi: 3.0.0

info:

title: Nnef_EventExposure

version: 1.0.0.alpha-1

description: |

NEF Event Exposure Service.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.591 V16.0.0; 5G System; Application Function (AF) event exposure service; Stage 3.

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.591/

servers:

- url: '{apiRoot}/nnef-eventexposure/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- nnef-eventexposure

paths:

/subscriptions:

post:

summary: subscribe to notifications

operationId: CreateIndividualSubcription

tags:

- Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NefEventExposureSubsc'

responses:

'201':

description: Success

content:

application/json:

schema:

$ref: '#/components/schemas/NefEventExposureSubsc'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nnef-eventexposure/{apiVersion}/subscriptions/{subscriptionId}'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NefEventExposureNotif'

responses:

'204':

description: No Content, Notification was succesfull

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

get:

summary: retrieve subscription

operationId: GetIndividualSubcription

tags:

- IndividualSubscription (Document)

parameters:

- name: subscriptionId

in: path

description: Event Subscription ID

required: true

schema:

type: string

responses:

'200':

description: OK. Resource representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/NefEventExposureSubsc'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: update subscription

operationId: ReplaceIndividualSubcription

tags:

- IndividualSubscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NefEventExposureSubsc'

parameters:

- name: subscriptionId

in: path

description: Event Subscription ID

required: true

schema:

type: string

responses:

'200':

description: OK. Resource was succesfully modified and representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/NefEventExposureSubsc'

'204':

description: No Content. Resource was succesfully modified

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: unsubscribe from notifications

operationId: DeleteIndividualSubcription

tags:

- IndividualSubscription (Document)

parameters:

- name: subscriptionId

in: path

description: Event Subscription ID

required: true

schema:

type: string

responses:

'204':

description: No Content. Resource was succesfully deleted

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nnef-eventexposure: Access to the Nnef_EventExposure API

schemas:

NefEventExposureSubsc:

type: object

properties:

eventsSubs:

type: array

items:

$ref: '#/components/schemas/NefEventSubs'

minItems: 1

eventsRepInfo:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/EventReportingRequirement'

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifId:

type: string

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

rspNotifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

required:

- notifId

- notifUri

- suppFeat

NefEventExposureNotif:

type: object

properties:

notifId:

type: string

eventNotifs:

type: array

items:

$ref: '#/components/schemas/NefEventNotification'

minItems: 1

required:

- notifId

- eventNotifs

NefEventNotification:

type: object

properties:

event:

$ref: '#/components/schemas/NefEvent'

timeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

svcExprcInfos:

type: array

items:

$ref: '#/components/schemas/ServiceExperienceInfo'

minItems: 1

ueMobilityInfos:

type: array

items:

$ref: '#/components/schemas/UeMobilityInfo'

minItems: 1

ueCommInfos:

type: array

items:

$ref: '#/components/schemas/UeCommunicationInfo'

minItems: 1

excepInfos:

type: array

items:

$ref: 'TS29517_Naf_EventExposure.yaml#/components/schemas/ExceptionInfo'

minItems: 1

required:

- event

- timeStamp

NefEventSubs:

type: object

properties:

event:

$ref: '#/components/schemas/NefEvent'

eventFilter:

$ref: '#/components/schemas/NefEventFilter'

required:

- event

NefEventFilter:

type: object

properties:

tgtUe:

$ref: '#/components/schemas/TargetUeIdentification'

appIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

minItems: 1

locArea:

$ref: 'TS29554_Npcf_BDTPolicyControl.yaml#/components/schemas/NetworkAreaInfo'

required:

- tgtUe

TargetUeIdentification:

type: object

properties:

supis:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

minItems: 1

interGroupIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

minItems: 1

anyUeId:

type: boolean

ServiceExperienceInfo:

type: object

properties:

appId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

supis:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

minItems: 1

svcExpPerFlows:

type: array

items:

$ref: 'TS29517_Naf_EventExposure.yaml#/components/schemas/ServiceExperienceInfoPerFlow'

minItems: 1

required:

- svcExpPerFlows

UeMobilityInfo:

type: object

properties:

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

appId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

ueTrajs:

type: array

items:

$ref: '#/components/schemas/UeTrajectoryInfo'

minItems: 1

required:

- supi

- ueTrajs

UeCommunicationInfo:

type: object

properties:

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

interGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

appId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

comms:

type: array

items:

$ref: 'TS29517_Naf_EventExposure.yaml#/components/schemas/CommunicationCollection'

minItems: 1

required:

- comms

UeTrajectoryInfo:

type: object

properties:

ts:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

location:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

required:

- ts

- location

# Simple data types and Enumerations

NefEvent:

anyOf:

- type: string

enum:

- SVC_EXPERIENCE

- UE_MOBILITY

- UE_COMM

- EXCEPTIONS

- type: string

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-10 | CT3#106 |  |  |  |  | TS Skeleton | 0.0.0 |
| 2019-10 | CT3#106 | C3-194392 |  |  |  | Removed some subclauses and editorial changes | 0.1.0 |
| 2019-10 | CT3#106 |  |  |  |  | Inclusion of C3-194271, C3-194396, C3-194397 and C3-194399. | 0.2.0 |
| 2019-11 | CT3#107 |  |  |  |  | Inclusion of C3-195234, C3-195235, C3-195236 and C3-195274. | 0.3.0 |
| 2020-02 | CT3#108e |  |  |  |  | Inclusion of C3-201284, C3-201288, C3-201367, C3-201368, C3-201370, C3-201407, C3-201409, C3-201413 and C3-201516. | 0.4.0 |
| 2020-03 | CT#87e | CP-200187 |  |  |  | TS sent to plenary for approval | 1.0.0 |
| 2020-03 | CT#87e | CP-200187 |  |  |  | TS approved by plenary | 16.0.0 |