3GPP TSG-CT3 Meeting #105	C3-193047

# Wroclaw, Poland, 26th Aug 2019 - 30th Aug 2019                                           (revision of …)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.512 | CR | 0302 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of requests colliding with an existing context |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc16 |  |  |  |  |  | Date: |  |  | 2019-08-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Requests may arrive late at upstreams entities, e.g. in networks experiencing processing or transport delays. Race scenarios similar to those studied for EPC (e.g. see clause 4.4.1.2 from TR 29.811) can also occur in 5GC. More details can be found in document C4-193186, which proposes to define a new Origination Time Stamp header in HTTP signalling to establish a PDU session and the related policy association to N7, and require the receiver of a request colliding with an existing context to accept the request only if it is newer than the timestamp of the request that created the existing context (as specified in EPC). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The forwarding of the Origination Time Stamp to the PCF is introduced. - Detection and handling of late arrival requests at the PCF, which shall accept the new request only if it contains a more recent 3gpp-Sbi-Origination-Timestamp than the timestamp stored for the existing context. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Receipt of a late request to establish a PDU session can result in tearing down a valid PDU session context. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.2, 4.2.y(new), 5.2.3.x(new), 5.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 29.502 CR 0182 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not have an impact on the OpenAPI file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

#### 4.2.2.2	SM Policy Association establishment

Figure 4.2.2.2-1: SM Policy Association establishment

When the SMF receives the Nsmf_PDUSession_CreateSMContext Request as defined in subclause 5.2.2.2 of 3GPP TS 29.502 [22], if the SMF was requested not to interact with the PCF, the SMF shall not interact with the PCF; otherwise, the SMF shall send the POST method as step 1of the figure 4.2.2.2-1 to request to create an "Individual SM Policy".

NOTE 1:	The decision to not interact with PCF applies for the life time of the PDU session.

NOTE 2:	The indicator to not interact with PCF is configured in the UDM. It is delivered by the UDM to the SMF within the Charging Characteristics using the Session Management Subscription Data Retrieval service operation as described in 3GPP TS 29.503 [34]. The indicator is operator specific, therefore it can only be used in non-roaming and home routed roaming cases.

The SMF shall include SmPolicyContextData data structure in the payload body of the HTTP POST to request a creation of representation of the "Individual SM Policy" resource. The "Individual SM Policy" resource is created as described below.

The SMF shall include (if available) in SmPolicyContextData data structure:

-	SUPI of the user within the "supi" attribute;

-	PDU Session Id within the "pduSessionId" attribute;

-	DNN within the "dnn" attribute;

-	URL identifying the recipient of SM policies update notification within the "notificationUri" attribute;

-	PDU Session Type within the "pduSessionType" attribute;

-	PEI within the "pei" attribute;

-	Internal Group Id(s) within the "InterGrpIds" attribute;

-	type of access within the "accessType" attribute;

-	type of the radio access technology within the "ratType" attribute;

-	the UE Ipv4 address within the "ipv4Address" attribute and/or the UE Ipv6 prefix within the "ipv6AddressPrefix" attribute;

-	the UE time zone information within "ueTimeZone" attribute;

-	the UDM subscribed Session-AMBR or, if the "DN-Authorization" feature is supported, the DN-AAA authorized Session-AMBR within "subsSessAmbr" attribute;

NOTE 3: 	When both, the UDM subscribed Session-AMBR and the DN-AAA authorized Session-AMBR are available in the SMF, the SMF includes the DN-AAA authorized Session-AMBR.

-	if the "DN-Authorization" feature is supported, the DN-AAA authorization profile index within the "authProfIndex" attribute;

-	subscribed Default QoS Information within "subsDefQos" attribute;

-	the number of supported packet filters for signalled QoS rules within the "numOfPackFilter";

-	the online charging status within "online" attribute;

-	the offline charging status within "offline" attribute;

-	the charging characteristics within "chargingCharacteristics" attribute;

-	access network charging identifier within the " accNetChId" attribute;

-	the address of the network entity performing charging within the "chargEntityAddr" attribute;

-	3GPP PS data off status within the "3gppPsDataOffStatus" attribute;

-	indication of UE supporting reflective QoS within the "refQosIndication" attribute;

-	user location information within the "userLocationInfo" attribute;

-	the S-NSSAI corresponding to the network slice the PDU session is allocated within the "sliceInfo" attribute.

-	the QoS flow usage required of the default QoS flow within the "qosFlowUsage" attribute;

-	identifier of the serving network within the "servingNetwork" attribute;

-	serving network function identifier within the "servNfId" attribute; and

-	trace control and configuration parameters information encoded as "traceReq" attribute.

Editor’s note:		It is FFS whether a new attribute "maPduInd" to indicate MA PDU request is needed.

Editor’s note:		In stage 2, the ATSSS Capability is not provided to the PCF. But if it is not provided to the PCF, the PCF can’t determine the ATSSS functionality. It is FFS whether the ATSSS Capability needs to be provided the PCF.

The SMF may include in "SmPolicyContextData" data structure the IPv4 address domain identity within the "ipDomain" attribute.

NOTE 4:	The "ipDomain" attribute is helpful when within a network slice instance, there are several separate IP address domains, with SMF/UPF(s) that allocate Ipv4 IP addresses out of the same private address range to UE PDU Sessions. The same IP address can thus be allocated to UE PDU sessions served by SMF/UPFs in different IPv4 address domains. If one PCF controls several SMF/UPFs in different IP address domains, the UE IP address is thus not sufficient for the AF session binding procedure, as described in 3GPP TS 29.514 [17]. The SMF assits the PCF in the session binding supplying an "ipDomain" attribute denoting the IPv4 address domain identity of the allocated UE IPv4 address.

When the PCF receives the HTTP POST request from the SMF, the PCF shall make an authorization based on the information received from the SMF and, if available, AMF, CHF, AF, UDR, NWDAF and operator policy pre-configured at the PCF. If the authorization is successful, the PCF shall create a new resource, which represents "Individual SM Policy", addressed by a URI as defined in subclause 5.3.3.2 and contains a PCF created resource identifier. The PCF shall respond to the SMF with a 201 Created message, including:

-	Location header field containing the URI for the created resource; and

-	a response body providing session management related policies, e.g. provisioning of PCC rules as defined in subclause 4.2.6.2, provisioning of policy control request triggers as defined in subclause 4.2.6.4.

The SMF shall use the URI received in the Location header in subsequent requests to the PCF to refer to the "Individual SM Policy".

It the PCF received a "traceReq" attribute, it shall perform trace procedures as defined in 3GPP TS 32.422 [24].

If errors occur when processing the HTTP POST request, the PCF shall apply error handling procedures as specified in subclause 5.7.

If the user information received within the "supi" attribute is unknown, the PCF shall reject the request and include in an HTTP "400 Bad Request" response message the "cause" attribute of the ProblemDetails data structure set to "USER_UNKNOWN".

If the PCF is, due to incomplete, erroneous or missing information (e.g. QoS, RAT type, subscriber information) not able to provision a policy decision as response to the request for PCC rules by the SMF, the PCF may reject the request and include in an HTTP "400 Bad Request" response message the "cause" attribute of the ProblemDetails data structure set to "ERROR_INITIAL_PARAMETERS".

If the PCF, based on local configuration and/or operator policies, denies the creation of the Individual SM Policy resource, the PCF may reject the request and include in an HTTP "403 Forbidden" response message the "cause" attribute of the ProblemDetails data structure set to "POLICY_CONTEXT_DENIED". Based on configured failure action, the SMF at reception of this error code may reject the PDU session establishment or allow the PDU session establishment applying local policies.

If the SMF receives HTTP response with these codes, the SMF shall reject the PDU session establishment that initiated the HTTP POST Request.

The forwarding of the Origination Time Stamp parameter shall apply as described hereafter, if the SMF supports the detection and handling of late arriving requests as specified in 3GPP TS 29.502, subclause 5.2.3.x [22] and the procedure is enabled by the operator. If the SMF receives a request to create an SM Context or a PDU session context, which includes the 3gpp-Sbi-Origination-Timestamp header as defined in subclause 5.2.3.x, the SMF shall forward this header to the PCF as HTTP custom header. See also subclause 4.2.y for the handling at the PCF, when the PCF receives the 3gpp-Sbi-Origination-Timestamp header.

# *** 2nd Change ***

### 4.2.y	Detection and handling of late arriving requests

### 4.2.y.1	Handling of requests which collide with an existing Individual SM Policy Association

The following principles shall apply, if this supported and end enabled by operator policies.

The PCF may receive an Originating Time Stamp parameter, which is set by the AMF, by the Npcf_SMPolicyControl_Create service request.

Upon receipt of a Npcf_SMPolicyControl_Create service request which collides with an existing Individual SM Policy Association for the same UE and the same PDU session Id the PCF shall accept the new request only if it contains a more recent 3gpp-Sbi-Origination-Timestamp than the timestamp stored for the existing Individual SM Policy Association. An incoming SM Policy Association request shall be considered as more recent than an existing Individual SM Policy Association and accepted if no 3gpp-Sbi-Origination-Timestamp information was provided for at least one of the two SM Policy Association. The PCF shall reject an incoming request whose timestamp is less recent than the timestamp of the existing Individual SM Policy Association with the HTTP status code "403 Forbidden" and the application error "LATE_OVERLAPPING_REQUEST".

NOTE:	When the PCF accepts the new request that contains a more recent 3gpp-Sbi-Origination-Timestamp than the timestamp stored for the Individual SM Policy Association, the PCF locally deletes the existing Individual SM Policy Association.

# *** 3rd Change ***

#### 5.2.3.x	3gpp-Sbi-Origination-Timestamp

The header contains the date and time (with a millisecond granularity) when the originating entity initiated the request as specified in 3GPP TS 29.502, subclause 6.1.2.3.x [22].

# *** 4th Change ***

### 5.7.3	Application Errors

The application errors defined for the Npcf_SMPolicyControl API are listed in table 5.7.3-1 and 5.7.3-2. The PCF shall include in the HTTP status code a "ProblemDetails" data structure with the "cause" attribute indicating the application error as listed in table 5.7.3-1 when PCF acts as a server. The SMF shall include in the HTTP status code a "ProblemDetails" data structure with the "cause" attribute indicating the application error as listed in table 5.7.3-2 when SMF acts as a server.

Table 5.7.3-1: Application errors when PCF acts as a server

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| USER_UNKNOWN | 400 Bad Request | The HTTP request is rejected because the end user specified in the request is unknown to the PCF. (NOTE 1) |
| ERROR_INITIAL_PARAMETERS | 400 Bad Request | The HTTP request is rejected because the set of session or subscriber information needed by the PCF for rule selection is incomplete or erroneous or not available for the decision to be made. (E.g. QoS, , RAT type, subscriber information) (NOTE 1) (NOTE 2) |
| ERROR_TRIGGER_EVENT | 400 Bad Request | The HTTP request is rejected because the set of session information sent the message originated due to a trigger been met is incoherent with the previous set of session information for the same session. (E.g. trigger met was RAT changed, and the RAT notified is the same as before) (NOTE 2) |
| ERROR_TRAFFIC_MAPPING_INFO_REJECTED | 403 Forbidden | The HTTP request is rejected because the PCF does not accept one or more of the traffic mapping filters provided by the SMF in a PCC Request. (NOTE 2) |
| ERROR_CONFLICTING_REQUEST | 403 Forbidden | The HTTP request is rejected because the PCF cannot accept the UE-initiated resource request as a network-initiated resource allocation is already in progress that has packet filters that cover the packet filters in the received UE-initiated resource request. The SMF shall reject the attempt for UE-initiated resource request. (NOTE 2) |
| LATE_OVERLAPPING_REQUEST | 403 Forbidden | The request is rejected because it collides with and exiting SM context or PDU session context with a more recent originating timestamp. (NOTE 1) |
| POLICY_CONTEXT_DENIED | 403 Forbidden | The HTTP request is rejected because the PCF does not accept the SMF request due to operator policies and/or local configuration. (NOTE 1) |
| PENDING_TRANSACTION | 400 Bad Request | This error shall be used when the PendingTransaction feature is supported and the PCF receives an incoming request on a policy association while it has an ongoing transaction on the same policy association and cannot handle the request as described in clause 9.2 of 3GPP TS 29.513 [7]. (NOTE 2) |
| NOTE 1: These application errors are used by the create service operation (see subclause 4.2.2.2) and included in the responses to the POST request. NOTE 2: These application errors are used by the update service operation (see subclause 4.2.4.2) and included in the responses to the POST request. |  |  |

Table 5.7.3-2: Application errors when SMF acts as a server to receive a notification

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| PCC_RULE_EVENT | 400 Bad Request | The HTTP request is rejected because all the PCC rules provisioned by the PCF in the request cannot be installed/activated. It is used to inform the PCF that the request failed, and should not be attempted again. (NOTE) |
| PCC_QOS_FLOW_EVENT | 400 Bad Request | The HTTP request is rejected because for some reason all the PCC rules provisioned by the PCF in the request cannot be enforced or modified successfully in a network initiated procedure. It is used to inform the PCF that the request could not be satisfied at the time it was received, but may be able to satisfy the request in the future. (NOTE) |
| UE_STATUS_SUSPEND | 400 Bad Request | The HTTP request is rejected because the UE’s status is suspended and the policy decisions received from the PCF cannot be enforced by the SMF. Applicable only to functionality introduced with the PolicyUpdateWhenUESuspends feature as described in subclause 5.8. (NOTE) |
| RULE_PERMANENT_ERROR | 400 Bad Request | The HTTP request is rejected because all the session rules provisioned by the PCF in the request cannot be installed/activated. It is used to inform the PCF that the request failed, and should not be attempted again. Applicable only to functionality introduced with the SessionRuleErrorHandling feature as described in subclause 5.8. (NOTE) |
| RULE_TEMPORARY _ERROR | 400 Bad Request | The HTTP request is rejected because for some reason all the session rules provisioned by the PCF in the request cannot be enforced or modified successfully in a network initiated procedure. It is used to inform the PCF that the request could not be satisfied at the time it was received, but may be able to satisfy the request in the future. Applicable only to functionality introduced with the SessionRuleErrorHandling feature as described in subclause 5.8. (NOTE) |
| PENDING_TRANSACTION | 400 Bad Request | This error shall be used when the PendingTransaction feature is supported and the SMF receives an incoming request on a policy association while it has an ongoing transaction on the same policy association and cannot handle the request as described in clause 9.2 of 3GPP TS 29.513 [7]. (NOTE) |
| NOTE: These application errors are used by the UpdateNotify service operation (see subclause 4.2.3.2) and included in the responses to the POST request. |  |  |

*** End of Changes ***