3GPP TSG-CT3 Meeting #106	C3-194159

# Portoroz, Slovenia, 07th – 11th  October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.222 | CR | 0098 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | API invoker details update – API Definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCAPIF |  |  |  |  |  | Date: |  |  | 2019-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 23.222 (Rel-16), clause 8.26 specifies the procedure and information flows related to update of API invoker’s API list on CAPIF core function. This procedure defines new CAPIF services that API invoker consumes to update the API list that the API invoker wishes to consume. API invoker uses HTTP PATCH method to udpate the onboarded resource of API invoker on CAPIF core function. The service architecture and functional entities description needs to be aligned in 29.222 (Rel-16) with TS 23.222 (Rel-16). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the definition of CAPIF_API_Invoker_Management_API with PATCH method and the new data types (APIInvokerDetailsPatch and APIInvokerUpdateNotification) for API invoker update and notification service operations. Update the CAPIF_API_Invoker_Management_API OpenAPI specification (Annex A.5) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | API definition and related data types are unspecified for API invoker update and notification service operations for CAPIF_API_Invoker_Management_API. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.1, 8.4.2.3.3.2 (new), 8.4.3.1, 8.4.3.3 (new), 8.4.3.3.1 (new), 8.4.3.3.2 (new), 8.4.4.1, 8.4.4.2.8 (new), 8.4.4.2.9 (new), A.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR … CR … TS/TR … CR … |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This contribution updates the API with backward incompatible change, affect the Open API. The version should be updated accordingly. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

#### 8.4.2.1	Overview

Figure 8.4.2.1-1: Resource URI structure of the CAPIF_API_Invoker_Management_API

Table 8.4.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.4.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| On-boarded API Invokers | {apiRoot} /api-invoker-management/v1 /onboardedInvokers | POST | On-boards a new API invoker by creating a API invoker profile |
| Individual On-boarded API Invoker | {apiRoot} /api-invoker-management/v1 /onboardedInvokers/{onboardingId} | DELETE | Off-boards an individual API invoker by deleting the API invoker profile identified by {onboardingId} |
|  |  | PATCH | Updates the API invoker details of an individual API invoker identified by the {onboardingId} |

2nd change

###### 8.4.2.3.3.2	PATCH

This method shall support the URI query parameters specified in table 8.4.2.3.3.2-1.

Table 8.4.2.3.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in the table 8.4.2.3.3.2-2 and the response data structures and response codes specified in the table 8.4.2.3.3.2-3.

Table 8.4.2.3.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| APIInvokerDetailsPatch | M | 1 | Updated details of the API invoker and a notification destination URI for any update request related notifications. |

Table 8.4.2.3.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| APIInvokerDetailsPatch | M | 1 | 200 OK | API invoker’s information updated successfully. Updated details of the API invoker as part of the APIInvokerDetailsPatch which is provided in the response body. |
| n/a |  |  | 202 Accepted | The CAPIF core has accepted the Update details request and is processing it. When processing is completed, the CAPIF core function will send a Notify_Update_Completion notification to the requesting API invoker. See sub clause 8.4.3.3. |

3rd change

#### 8.4.3.1	General

The delivery of notifications shall conform to subclause 7.6.

Table 8.4.3.1-1: Notifications overview

| Notification | Resource URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Notify_Onboarding_Completion | {notificationDestination} | POST | Notify API invoker of on-boarding result |
| Notify_Update_Completion | {notificationDestination} | POST | Notify API invoker of API invoker details update result. |

4th change

#### 8.4.3.3	Notify_Update_Completion

##### 8.4.3.3.1	Description

Notify_Update_Completion is used by the CAPIF core function to notify an API invoker of the update of API Invoker’s details result.

##### 8.4.3.3.2	Notification definition

The POST method shall be used for Notify_Update_Completion and the URI shall be the one provided by the API invoker during the API invoker details update request.

Resource URI: {notificationDestination}

This method shall support the URI query parameters specified in table 8.4.3.3.2-1.

Table 8.4.3.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.4.3.3.2-2 and the response data structures and response codes specified in table 8.4.3.3.2-3.

Table 8.4.3.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| APIInvokerUpdateNotification | M | 1 | Notification with API Invoker’s details update result. |

Table 8.4.3.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |

5th change

#### 8.4.4.1	General

This subclause specifies the application data model supported by the API. Data types listed in subclause 7.2 apply to this API.

Table 8.4.4.1-1 specifies the data types defined specifically for the CAPIF_API_Invoker_Management_API service.

Table 8.4.4.1-1: CAPIF_API_Invoker_Management_API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| APIInvokerEnrolmentDetails | 8.4.4.2.2 | API invoker's enrolment details |  |
| APIList | 8.4.4.2.4 | List of APIs |  |
| OnboardingInformation | 8.4.4.2.5 | On-boarding information of the API invoker |  |
| OnboardingNotification | 8.4.4.2.7 | Notification with on-boarding result |  |
| APIInvokerDetailsPatch | 8.4.4.2.8 | API invoker’s updated details |  |
| APIInvokerUpdateNotification | 8.4.4.2.9 | Notification with API invoker details update result |  |

Table 8.4.4.1-2 specifies data types re-used by the CAPIF_API_Invoker_Management_API service.

Table 8.4.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ServiceAPIDescription | Subclause 8.1.4.2.2 |  |  |
| Uri | 3GPP TS 29.122 [14] |  |  |
| TestNotification | 3GPP TS 29.122 [14] |  |  |
| WebsockNotifConfig | 3GPP TS 29.122 [14] |  |  |
| SupportedFeatures | 3GPP TS 29.571 [19] | Used to negotiate the applicability of optional features defined in table 8.4.6-1. |  |

6th change

##### 8.4.4.2.8	Type: APIInvokerDetailsPatch

Table 8.4.4.2.8-1: Definition of type APIInvokerDetailsPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| apiInvokerId | string | M | 1 | API invoker ID assigned by the CAPIF core function to the API invoker while on-boarding the API invoker. |  |
| notificationDestination | Uri | M | 1 | URI where the notification should be delivered to. |  |
| requestTestNotification | boolean | O | 0..1 | Set to true by Subscriber to request the CAPIF core function to send a test notification as defined in in subclause 7.6. Set to false or omitted otherwise. | Notification_test_event |
| websockNotifConfig | WebsockNotifConfig | O | 0..1 | Configuration parameters to set up notification delivery over Websocket protocol as defined in subclause 7.6. | Notification_websocket |
| apiList | APIList | O | 0..1 | In the PATCH request, it lists the updated set of APIs that the API invoker intends to invoke. When included by the CAPIF core function in the HTTP response message, it lists the APIs that the API invoker’s profile is updated with and is allowed to invoke. |  |
| apiInvokerInformation | string | O | 0..1 | Generic information related to the API invoker such as details of the device or the application. This is the information that the API invoker intends to update with in PATCH message. When included by the CAPIF core function in the HTTP response message, it consists of API invoker information that the API invoker profile is updated with. |  |
| supportedFeatures | SupportedFeatures | O | 0..1 | Used to negotiate the supported optional features of the API as described in subclause 7.8. This attribute shall be provided in the HTTP PATCH request and in the response of successful update. |  |

##### 8.4.4.2.9	Type: APIInvokerUpdateNotification

Table 8.4.4.2.7-1: Definition of type APIInvokerUpdateNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| result | boolean | M | 1 | Set to "true" indicate successful update. Otherwise set to "false" |  |
| apiInvokerDetailsPatch | APIInvokerDetailsPatch | C | 0..1 | Updated details of the API invoker, verified by the CAPIF administrator or API management. The details may include updated list of APIs the API invoker is allowed to access, API invoker information and supported features. This attribute shall be present if 'result' attribute is set to "true". Otherwise it shall not be present. |  |

7th change

## A.5	CAPIF_API_Invoker_Management_API

openapi: 3.0.0

info:

title: CAPIF_API_Invoker_Management_API

description: |

API for API invoker management.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.0.1"

externalDocs:

description: 3GPP TS 29.222 V15.4.0 Common API Framework for 3GPP Northbound APIs

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/api-invoker-management/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 7.5 of 3GPP TS 29.222

paths:

/onboardedInvokers:

post:

description: Creates a new individual API Invoker profile.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/APIInvokerEnrolmentDetails'

callbacks:

notificationDestination:

'{request.body#/notificationDestination}':

post:

description: Notify the API Invoker about the onboarding completion

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/OnboardingNotification'

responses:

'204':

description: No Content (successful onboarding notification)

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: API invoker on-boarded successfully

content:

application/json:

schema:

$ref: '#/components/schemas/APIInvokerEnrolmentDetails'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/api-invoker-management/v1/onboardedInvokers/{onboardingId}'

required: true

schema:

type: string

'202':

description: The CAPIF core has accepted the Onboarding request and is processing it.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/onboardedInvokers/{onboardingId}:

delete:

description: Deletes an individual API Invoker.

parameters:

- name: onboardingId

in: path

description: String identifying an individual on-boarded API invoker resource

required: true

schema:

type: string

responses:

'204':

description: The individual API Invoker matching onboardingId was offboarded.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

description: Updates an individual API invoker details.

parameters:

- name: onboardingId

in: path

description: String identifying an individual on-boarded API invoker resource

required: true

schema:

type: string

requestBody:

description: representation of the API invoker details to be updated in CAPIF core function

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/APIInvokerDetailsPatch'

callbacks:

notificationDestination:

'{request.body#/notificationDestination}':

post:

description: Notify the API Invoker about the API invoker update completion

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/APIInvokerUpdateNotification'

responses:

'204':

description: No Content (successful API invoker update notification)

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'200':

description: API invoker details updated successfully

content:

application/json:

schema:

$ref: '#/components/schemas/APIInvokerDetailsPatch'

'202':

description: The CAPIF core has accepted the API invoker update details request and is processing it.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

schemas:

OnboardingInformation:

type: object

properties:

apiInvokerPublicKey:

type: string

description: The API Invoker’s public key

apiInvokerCertificate:

type: string

description: The API Invoker’s generic client certificate, provided by the CAPIF core function.

onboardingSecret:

type: string

description: The API Invoker’s onboarding secret, provided by the CAPIF core function.

required:

- apiInvokerPublicKey

APIList:

type: array

items:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/ServiceAPIDescription'

minItems: 1

description: The list of service APIs that the API Invoker is allowed to invoke

APIInvokerEnrolmentDetails:

type: object

properties:

apiInvokerId:

type: string

description: API invoker ID assigned by the CAPIF core function to the API invoker while on-boarding the API invoker. Shall not be present in the HTTP POST request from the API invoker to the CAPIF core function, to on-board itself. Shall be present in all other HTTP requests and responses.

readOnly: true

onboardingInformation:

$ref: '#/components/schemas/OnboardingInformation'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

requestTestNotification:

type: boolean

description: Set to true by Subscriber to request the CAPIF core function to send a test notification as defined in in subclause 7.6. Set to false or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

apiList:

$ref: '#/components/schemas/APIList'

apiInvokerInformation:

type: string

description: Generic information related to the API invoker such as details of the device or the application.

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- onboardingInformation

- notificationDestination

description: Information about the API Invoker that requested to onboard

OnboardingNotification:

type: object

properties:

result:

type: boolean

description: Set to "true" indicate successful on-boarding. Otherwise set to "false"

resourceLocation:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

apiInvokerEnrolmentDetails:

$ref: '#/components/schemas/APIInvokerEnrolmentDetails'

apiList:

$ref: '#/components/schemas/APIList'

required:

- result

APIInvokerDetailsPatch:

type: object

properties:

apiInvokerId:

type: string

description: API invoker ID assigned by the CAPIF core function to the API invoker after successfully on-boarding the API invoker.

readOnly: true

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

requestTestNotification:

type: boolean

description: Set to true by Subscriber to request the CAPIF core function to send a test notification as defined in in subclause 7.6. Set to false or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

apiList:

$ref: '#/components/schemas/APIList'

apiInvokerInformation:

type: string

description: Generic information related to the API invoker such as details of the device or the application.

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- apiInvokerId

- notificationDestination

description: Information about the API Invoker that is requested to update

APIInvokerUpdateNotification:

type: object

properties:

result:

type: boolean

description: Set to "true" indicate successful update. Otherwise set to "false"

resourceLocation:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

apiInvokerDetailsPatch:

$ref: '#/components/schemas/APIInvokerDetailsPatch'

required:

- result

End of changes