3GPP TSG-CT WG3 Meeting #108-e	C3-201166

- E-Meeting, 19th – 28th February 2020                                               (revision of C3-201abc)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.519 | CR | 0173 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Subscription and notification to data changes related to a subset of resource data, Policy Data set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | en5GPccSer |  |  |  |  |  | Date: |  |  | 2020-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The subscription to data changes within the Policy Data is solved in an homogeneus and simple way for all the involved resources: the attribute “monitoredResourceUris” within the PolicyDataSubscription data type represents the set of URIs that identify the resources for which a modification triggers a notification. While this approach works fine for simple resources, as e.g. the ones related to UE procedures (as MM, UE Policies) because the NF service consumer always requires the subscription to the complete representation of the resource, and the notification of any change in such resource, it is suboptimum for complex resources, as e.g. sm-data, used in PDU Session related procedures, because they only require a partial representation of the resource, specifically, the subresource data corresponding to a given S-NSSAI and DNN combination. The notification to data changes within the Policy Data is also solved in an homogeneus and simple way for all the involved resources: the PolicyDataChangeNotification data type contains an attribute for every resource that shall include the complete resource representation when such resource is modified. The approach works fine for simple and small resources, as e.g. the ones related to UE procedures (as MM, UE Policies) because the NF service consumer requires the complete representation of the resource, it is suboptimum for complex and large resources, as e.g. sm-data, used for PDU Session related procedures, because they only require the notification of fragment of the resource, of a partial representation of the resource (i.e., the PCF, because of the subscription on sm-data changes for the update of a SM Policy Association, may be receiving useless notifications on changes on sm-data for any S-NSSAI and DNN combination). |  |  |  |  |  |  |  |  |
| Summary of change: |  | For the Subscription to data changes data types: A new attribute called “monResItems”, whose handling depends on the feature “ConditionalSubscription” is supported, is defined to indicate that the subscription to notification of data changes is conditioned to the modification occurs in one or more of the resource fragments included in the “monResItems” attribute. This IE may also include the list of resource fragmens that may be included in the notification of a modification in a given resource fragment when the “NotificationResourceFragment” feature is supported. The identification of the subset of resource data to monitor for data changes is defined in the SubscriptionItem data type, which contains two attributes: - “monResourceUri”, that identifies to which resource the conditional subscription referes to; and - “monFragments”, that contains the details of the subscription to notification on changes on a resource fragment, i.e., it contains: - the target location of the resource fragment in the “fragmentPath” attribute, and, - when the “NotificationResourceFragment” feature is supported, the “notifiedItems” attribute with the list of resource fragments to include in the notification triggered by the modification of the resource fragment indicated in the “fragmentPath” attribute. For the Notification of data changes data types: A new attribute called “reportedFragments” encoded as an array of “NotificationItem” types is defined within the PolicyDataChangeNotification type and represents the list of resource fragments notified as result of the modification of a given resource fragment. This attribute is included when the “NotificationResourceFragment” feature is supported, and the subscription requested the notification of resource fragments. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Subscription to notification of data changes functionality is not efficiently adapted to the network scenarios that require such subscriptions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.4.1, 5.4.2.10, 5.4.2.11, 5.4.2.x1 (new), 5.4.2.x2 (new), 5.4.2.x3 (new), 5.4.3.2, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces a backward compatible feature to the Policy Data API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

- Additional discussion(if needed):W

- Proposed changes:

- *** 1st Change ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Void.

[3]	OpenAPI: "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	3GPP TS 29.504:"5G System; Unified Data Repository Services; Stage 3".

[7]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

[8]	3GPP TS 29.551: "5G System; Packet Flow Description Management Service; Stage 3".

[9]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[10]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[11]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[12]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[13]	3GPP TS 29.554: "5G System; Background Data Transfer Policy Control Service; Stage 3".

[14]	Void.

[15]	3GPP TS 29.505: "5G System; Usage of the Unified Data Repository services for Subscription Data; Stage 3".

[16]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[17]	IETF RFC 4122: "A Universally Unique IDentifier (UUID) URN Namespace".

[18]	3GPP TR 21.900: "Technical Specification Group working methods".

[19]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[r1]	IETF RFC 6901: "JavaScript Object Notation (JSON) Pointer".

- *** 2nd Change ***

### 5.4.1	General

This subclause specifies the application data model supported by the API.

Table 5.4.1-1 specifies the data types defined for the Nudr_DataRepository for Policy Data service based interface protocol.

Table 5.4.1-1: Nudr_DataRepository specific Data Types for Policy Data

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| AmPolicyData | 5.4.2.2 | Contains the AM policy data for a given subscriber. |  |
| BdtData | 5.4.2.9 | Contains the background data transfer data. |  |
| DnnRouteSelectionDescriptor | 5.4.2.20 | Contains the route selector parameters per DNN. |  |
| UpdatedItem | 5.4.2.x3 | An updated resource fragment, represented by its location in a resource and its data type. | ConditionalSubscriptionwithPartialNotification |
| LimitIdToMonitoringKey | 5.4.2.16 | Contains the limit identifier and the corresponding monitoring key for a given S-NSSAI and DNN. |  |
| NotificationItem | 5.4.2.x2 | Contains the list of resource fragments included in the notification triggered by the modification of a given resource fragment. | ConditionalSubscriptionwithPartialNotification |
| OsId | 5.4.3.2 | Operating System supported by the UE. |  |
| Periodicity | 5.4.3.4 | Indicates a type of time period. |  |
| PlmnRouteSelectionDescriptor | 5.4.2.18 | Contains the route selectors for a serving PLMN. |  |
| PolicyDataChangeNotification | 5.4.2.11 | Contains changed policy data for which notification was requested. |  |
| PolicyDataSubscription | 5.4.2.10 | Identifies a subscription to policy data change notification. |  |
| SmPolicyData | 5.4.2.5 | Contains SM policy data for a subscriber. |  |
| SmPolicyDataPatch | 5.4.2.21 | Contains modifiable SM policy data for a subscriber. | SessionManagementPolicyDataPatch |
| SmPolicyDnnData | 5.4.2.15 | Contains SM policy data for a DNN and S-NSSAI. |  |
| SmPolicyDnnDataPatch | 5.4.2.23 | Contains modifiable SM policy data for a DNN and S-NSSAI. | SessionManagementPolicyDataPatch |
| SmPolicySnssaiData | 5.4.2.14 | Contains SM policy data for a S-NSSAI. |  |
| SmPolicySnssaiDataPatch | 5.4.2.22 | Contains modifiable SM policy data for a S-NSSAI. | SessionManagementPolicyDataPatch |
| SnssaiRouteSelectionDescriptor | 5.4.2.19 | Contains the route selector parameters per SNSSAI. |  |
| SponsorConnectivityData | 5.4.2.8 | Contains the sponsored data connectivity related information for a sponsor identifier. |  |
| ResourceItem | 5.4.2.x1 | Represents list of fragments of a resource, i.e. a list of subsets of resource data monitored for notification of data changes. | ConditionalSubscriptionwithPartialNotification |
| ItemPath | 5.4.3.2 | Represents a fragment of a resource, i.e. a subset of resource data. | ConditionalSubscriptionwithPartialNotification |
| TimePeriod | 5.4.2.12 | Contains the periodicity for the defined usage monitoring data limits. |  |
| UePolicySetPatch | 5.4.2.17 | Contains the UE policy data that can be modified by the PCF. |  |
| UePolicySection | 5.4.2.3 | Contains the UE policy section. |  |
| UePolicySet | 5.4.2.4 | Contains the UE policy set for a given subscriber. |  |
| UsageMonLevel | 5.4.3.3 | Indicates the level of the usage monitoring instance (PDU Session level or per Service) |  |
| UsageMonData | 5.4.2.7 | Contains remain allowed usage data for a subscriber. |  |
| UsageMonDataLimit | 5.4.2.6 | Contains usage monitoring control data for a subscriber. |  |
| UsageMonDataScope | 5.4.2.13 | Contains SNSSAI and DNN combinations to which the UsageMonData instance belongs to. |  |

Table 5.4.1-2 specifies data types re-used by the Nudr_DataRepository for Policy Data service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nudr_DataRepository for Policy Data service based interface.

Table 5.4.1-2: Nudr_DataRepository re-used Data Types for Policy Data

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| BdtReferenceId | 3GPP TS 29.122 [9] | Indicates the background data transfer reference ID for the transfer policy. |  |
| BitRate | 3GPP TS 29.571 [7] | String representing a bit rate that shall be formatted as follows: pattern: "^\d+(\.\d+)? (bps¦Kbps¦Mbps¦Gbps¦Tbps)$" Examples: "125 Mbps", "0.125 Gbps", "125000 Kbps". |  |
| Bytes | 3GPP TS 29.571 [7] | String with format "byte" as defined in OpenAPI Specification [3], i.e., base64-encoded characters. |  |
| ChargingInformation | 3GPP TS 29.512 [12] | The address of the Charging Function. |  |
| DateTime | 3GPP TS 29.571 [7] | String with format "date-time" as defined in OpenAPI Specification [3]. |  |
| Dnn | 3GPP TS 29.571 [7] | Data Network Name. |  |
| NetworkAreaInfo | 3GPP TS 29.554 [13] | Represents a network area information. |  |
| OperatorSpecificDataContainer | 3GPP TS 29.505 [15] | Container for operator specific data |  |
| PatchResult | 3GPP TS 29.571 [7] |  |  |
| PatchItem | 3GPP TS 29.571 [7] | Data structure used for JSON patch. |  |
| PduSessionType | 3GPP TS 29.571 [7] | PDU Session Type. |  |
| Pei | 3GPP TS 29.571 [7] | Personal Equipment Identifier. |  |
| PlmnId | 3GPP TS 29.571 [7] | PLMN Identity. |  |
| PresenceInfo | 3GPP TS 29.571 [7] | Presence Reporting Area Information. |  |
| ProblemDetails | 3GPP TS 29.571 [7] | Used in error responses to provide more detailed information about an error. |  |
| Snssai | 3GPP TS 29.571 [7] | Identifies the S-NSSAI. |  |
| SscMode | 3GPP TS 29.571 [7] | SSC mode. |  |
| SupportedFeatures | 3GPP TS 29.571 [7] | Used to negotiate the applicability of the optional features |  |
| TransferPolicy | 3GPP TS 29.554 [13] | Represents a transfer policy. |  |
| Uinteger | 3GPP TS 29.571 [7] | Unsigned Integer, i.e. only value 0 and integers greater than 0 are allowed. |  |
| Uri | 3GPP TS 29.571 [7] | String providing an URI. |  |
| UsageThreshold | 3GPP TS 29.122 [9] | Usage Thresholds (a data volume expected to be transferred per UE and/or time duration in seconds). |  |
| VarUeId | 3GPP TS 29.571 [7] | String represents the SUPI or GPSI. |  |

- *** 3rd Change ***

#### 5.4.2.10	Type PolicyDataSubscription

Table 5.4.2.10-1: Definition of type PolicyDataSubscription

| Attribute name | Data type | P | Cardinality |  | Description | Applicability |
| --- | --- | --- | --- | --- | --- | --- |
| notificationUri | Uri | M |  | 1 | URI provided by the NF service consumer indicating where to receive the subscribed notifications from the UDR. |  |
| notifId | string | C |  | 0..1 | Notification Correlation ID assigned by the NF service consumer. It shall be included when the "ConditionalSubscriptionwithPartialNotification" feature is supported. | ConditionalSubscriptionwithPartialNotification |
| monitoredResourceUris | array(Uri) | M |  | 1..N | A set of URIs that identify the resources as defined in table 5.2.2-1 for which a modification triggers a notification. (NOTE) |  |
| monResItems | array(ResourceItem) | O |  | 1..N | When present, this IE indicates the trigger of the notification is conditioned to the data change occurs in one or more of the attributes contained in a resource fragment as defined by the ResourceItem data type, and that the triggered notification shall only include the resource fragment as defined by the ResourceItem data type. | ConditionalSubscriptionwithPartialNotification |
| expiry | DateTime | C |  | 0..1 | This IE shall be included in a subscription response if, based on operator policy and taking into account the expiry time included in the request, the UDR needs to include an expiry time. This IE may be included in a subscription request. When present, this IE shall represent the time after which the subscription becomes invalid. The absence of this attribute in the subscription response means the subscription to be valid without an expiry time. |  |
| supportedFeatures | SupportedFeatures | C |  | 0..1 | Used to negotiate the applicability of the optional features. This attribute shall be provided in the POST request and in the response of successful resource creation. |  |
| NOTE: Neither the resource URI of the IndividualPolicyDataSubscription resource nor the resource URI of the PolicyDataSubscriptions resource shall be included in the "monitoredResourceUris" attribute. |  |  |  |  |  |  |

- *** 4th Change ***

#### 5.4.2.11	Type PolicyDataChangeNotification

Table 5.4.2.11-1: Definition of type PolicyDataChangeNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| amPolicyData | AmPolicyData | O | 0..1 | Access and Mobility Policy Data, if changed and notification was requested. |  |
| uePolicySet | UePolicySet | O | 0..1 | UE Policy Set, if changed and notification was requested. |  |
| plmnUePolicySet | UePolicySet | O | 0..1 | PLMN UE Policy Set, if changed and notification was requested. |  |
| smPolicyData | SmPolicyData | O | 0..1 | Session Management Policy Data, if changed and notification was requested. |  |
| usageMonData | UsageMonData | O | 0..1 | Usage Monitoring Data, if changed and notification was requested. |  |
| sponsorConnectivityData | SponsorConnectivityData | O | 0..1 | Sponsor data connectivity profile information, if changed and notification was requested. |  |
| bdtData | BdtData | O | 0..1 | Background Data Transfer Data, if changed and notification was requested. |  |
| opSpecData | OperatorSpecificDataContainer | O | 0..1 | Operator Specific Data, if changed and notification was requested. |  |
| ueId | VarUeId | C | 0..1 | Represents the UE subscription identifier SUPI or GPSI. It shall only be present when the "amPolicyData", "uePolicySet", "smPolicyData", "opSpecData" and/or "usageMonData" attribute is present. |  |
| sponsorId | string | C | 0..1 | Represents the sponsor identity. It shall only be present when the "sponsorConnectivityData" attribute is present. |  |
| bdtRefId | string | C | 0..1 | Represents the BDT reference identifier. It shall only be present when the "bdtData" attribute is present. |  |
| usageMonId | string | C | 0..1 | Represents the unique identifier of the individual SM Policy usage monitoring resource. It shall only be present when the "usageMonData" attribute is present. |  |
| plmnId | PlmnId | C | 0..1 | Represents the PLMN identifier. It shall only be present when the "plmnUePolicySet" attribute is present. |  |
| delResources | array(Uri) | O | 1..N | The resources, as defined in table 5.2.2-1, if removed from UDR and notification on resource data change was requested. (NOTE 2) | ResourceRemovalNotificationPolicyData |
| notifId | string | C | 0..1 | Notification Correlation ID assigned by the NF service consumer. It shall be included when the "ConditionalSubscriptionwithPartialNotification" feature is supported. | ConditionalSubscriptionwithPartialNotification |
| reportedFragments | array(NotificationItem) | O | 0..1 | This attribute contains the resource fragments indicated in the "monResItems" attribute of the PolicyDataSubscription data type. | ConditionalSubscriptionwithPartialNotification |
| NOTE 1: When the "ResourceRemovalNotificationPolicyData" feature and/or “ConditionalSubscriptionwithPartialNotification” feature is not supported or supported but the applicable attribute(s) are not provided, at least one of the "amPolicyData", "uePolicySet", "smPolicyData", "usageMonData", "sponsorConnectivityData", "bdtData", "opSpecData" or "plmnUePolicySet" shall be present. NOTE 2: When the "ResourceRemovalNotificationPolicyData" feature is supported, and the "delResources" attribute is present, the attributes that correspond with the values included in the "delResources" attribute shall be omitted. E.g., when the "delResources" attribute includes the value "../policy-data/ues/{ueId}/am-data" the attribute "amPolicyData" shall be omitted. |  |  |  |  |  |

- *** 5th Change ***

#### 5.4.2.x1	Type ResourceItem

Table 5.4.2.x1-1: Definition of type ResourceItem

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| monResourceUri | Uri | M | 1 | Represents the URI that identifies the monitored resource as defined in table 5.2.2-1. (NOTE) |  |
| items | array(ItemPath) | M | 1..N | Represents fragments of the resource identified by the "monResource" attribute, i.e. a subset of resource data, for which a modification triggers a notification. |  |
| NOTE: The resource URI included in the "monResourceUri" attribute shall be one of the resource URIs included in the "monitoredResourceUris" attribute of the PolicyDataSubscription data type. |  |  |  |  |  |

- *** 6th Change ***

#### 5.4.2.x2	Type NotificationItem

Table 5.4.2.x2-1: Definition of type NotificationItem

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| resourceId | Uri | M | 1 | This IE contains the URI of the resource which has been changed. |  |
| notifItems | array(UpdatedItem) | M | 1..N | This IE contains the fragments of the resource identified by the "resourceId" attribute which have been updated or requested to be notified. |  |

- *** 7th Change ***

#### 5.2.4.x3	Type UpdatedItem

Table 5.2.4.x3-1: Definition of type UpdatedItem

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| item | ItemPath | M | 1 | This IE contains a JSON pointer value (as defined in IETF RFC 6901 [r1]) that references a target location within the resource that identifies the resource fragment. |  |
| value | Any type | M | 1 | This IE indicates the value of the resource fragment located at the target location within the resource specified in the "fragmentPath" attribute. The data type of this attribute shall be the same as the type of the attribute or element located in the path. The null value shall be allowed. |  |

- *** 8th Change ***

#### 5.4.3.2	Simple data types

The simple data types defined in table 5.4.3.2-1 shall be supported.

Table 5.4.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| IpIndex | integer | Information that identifies which IP pool or external server is used to allocate the IP address. |  |
| ItemPath | string | It contains one JSON pointer value (as defined in IETF RFC 6901 [r1]) that references a target location within the resource represented in the "monResource" attribute and "notifiedItems" attribute. E.g. "/smPolicySnssaiData/<snssai>" represents the subscription to data changes in the SmPolicyData data document, in particular to the element of the "smPolicySnssaiData" map whose key is identified by the <snssai> value. E.g. 2 "/smPolicySnssaiData/<snssai>/smPolicyDnnData/<dnn>" represents the subscription to data changes in the SmPolicyData data document, in particular to the element of the "smPolicySnssaiData" map whose key is identified by the <snssai> value and, within this one, to the element of the "smPolicyDnnData" map whose key is identified by the <dnn> value. | ConditionalSubscriptionwithPartialNotification |
| OsId | string | Operating System of the served UE. It contains a Universally Unique IDentifier (UUID) as specified in IETF RFC 4122 [17]. The format of the OS Id is represented in 3GPP TS 24.501 [11]. |  |

- *** 9th Change ***

# A.2	Nudr_DataRepository API for Policy Data

For the purpose of referencing entities in the Open API file defined in this Annex, it shall be assumed that this Open API file is contained in a physical file named "TS29519_Policy_Data.yaml".

openapi: 3.0.0

info:

version: '-'

title: Unified Data Repository Service API file for policy data

description: |

The API version is defined in 3GPP TS 29.504

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.519 V16.2.0; 5G System; Usage of the Unified Data Repository Service for Policy Data, Application Data and Structured Data for Exposure.

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.519/'

paths:

/policy-data/ues/{ueId}/am-data:

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

get:

summary: Retrieves the access and mobility policy data for a subscriber

operationId: ReadAccessAndMobilityPolicyData

tags:

- AccessAndMobilityPolicyData (Document)

responses:

'200':

description: Upon success, a response body containing access and mobility policies shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/AmPolicyData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/ues/{ueId}/ue-policy-set:

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

get:

summary: Retrieves the UE policy set data for a subscriber

operationId: ReadUEPolicySet

tags:

- UEPolicySet (Document)

responses:

'200':

description: Upon success, a response body containing UE policies shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/UePolicySet'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Create or modify the UE policy set data for a subscriber

operationId: CreateOrReplaceUEPolicySet

tags:

- UEPolicySet (Document)

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/UePolicySet'

responses:

'201':

description: Successful case. The resource has been successfully created and a response body containing a representation of the created UEPolicySet resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/UePolicySet'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'200':

description: Successful case. The resource has been successfully created and a response body containing UE policies shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/UePolicySet'

'204':

description: Successful case. The resource has been successfully updated and no additional content is to be sent in the response message.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Modify the UE policy set data for a subscriber

operationId: UpdateUEPolicySet

tags:

- UEPolicySet (Document)

requestBody:

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/UePolicySetPatch'

responses:

'204':

description: Successful case. The resource has been successfully updated and no additional content is to be sent in the response message.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/ues/{ueId}/sm-data:

get:

summary: Retrieves the session management policy data for a subscriber

operationId: ReadSessionManagementPolicyData

tags:

- SessionManagementPolicyData (Document)

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: snssai

in: query

required: false

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

- name: dnn

in: query

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

- name: supp-feat

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Upon success, a response body containing SmPolicyData shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SmPolicyData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Modify the session management policy data for a subscriber

operationId: UpdateSessionManagementPolicyData

tags:

- SessionManagementPolicyData (Document)

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/SmPolicyDataPatch'

responses:

'204':

description: Successful case. The resource has been successfully updated and no additional content is to be sent in the response message.

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SmPolicyData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/ues/{ueId}/sm-data/{usageMonId}:

get:

summary: Retrieve a usage monitoring resource

operationId: ReadUsageMonitoringInformation

tags:

- UsageMonitoringInformation (Document)

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: usageMonId

in: path

required: true

schema:

type: string

responses:

'200':

description: Successful case. The usage monitoring data is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/UsageMonData'

'204':

description: The resource was found but no usage monitoring data is available.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Create a usage monitoring resource

operationId: CreateUsageMonitoringResource

tags:

- UsageMonitoringInformation (Document)

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: usageMonId

in: path

required: true

schema:

type: string

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/UsageMonData'

responses:

'201':

description: Successful case. The resource has been successfully created and a response body is returned containing a representation of the resource.

content:

application/json:

schema:

$ref: '#/components/schemas/UsageMonData'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete a usage monitoring resource

operationId: DeleteUsageMonitoringInformation

tags:

- UsageMonitoringInformation (Document)

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: usageMonId

in: path

required: true

schema:

type: string

responses:

'204':

description: Successful case. The resource has been successfully deleted.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/sponsor-connectivity-data/{sponsorId}:

parameters:

- name: sponsorId

in: path

required: true

schema:

type: string

get:

summary: Retrieves the sponsored connectivity information for a given sponsorId

operationId: ReadSponsorConnectivityData

tags:

- SponsorConnectivityData (Document)

responses:

'200':

description: Upon success, a response body containing Sponsor Connectivity Data shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SponsorConnectivityData'

'204':

description: The resource was found but no Sponsor Connectivity Data is available.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/bdt-data:

get:

summary: Retrieves the BDT data collection

operationId: ReadBdtData

tags:

- BdtData (Store)

parameters:

- name: bdt-ref-ids

in: query

description: List of the BDT reference identifiers.

schema:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/BdtReferenceId'

minItems: 1

style: form

explode: false

responses:

'200':

description: Upon success, a response body containing the BDT data shall be returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/BdtData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/bdt-data/{bdtReferenceId}:

parameters:

- name: bdtReferenceId

in: path

required: true

schema:

type: string

get:

summary: Retrieves the BDT data information associated with a BDT reference Id

operationId: ReadIndividualBdtData

tags:

- IndividualBdtData (Document)

responses:

'200':

description: Upon success, a response body containing the BDT data shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/BdtData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Creates an BDT data resource associated with an BDT reference Id

operationId: CreateIndividualBdtData

tags:

- IndividualBdtData (Document)

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/BdtData'

responses:

'201':

description: Successful case. The resource has been successfully created.

content:

application/json:

schema:

$ref: '#/components/schemas/BdtData'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes an BDT data resource associated with an BDT reference Id

operationId: DeleteIndividualBdtData

tags:

- IndividualBdtData (Document)

responses:

'204':

description: Successful case. The resource has been successfully deleted.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/subs-to-notify:

post:

summary: Create a subscription to receive notification of policy data changes

operationId: CreateIndividualPolicyDataSubscription

tags:

- PolicyDataSubscriptions (Collection)

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyDataSubscription'

responses:

'201':

description: Upon success, a response body containing a representation of each Individual subscription resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyDataSubscription'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

policyDataChangeNotification:

'{$request.body#/notificationUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/PolicyDataChangeNotification'

minItems: 1

responses:

'204':

description: No Content, Notification was successful

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/subs-to-notify/{subsId}:

parameters:

- name: subsId

in: path

required: true

schema:

type: string

put:

summary: Modify a subscription to receive notification of policy data changes

operationId: ReplaceIndividualPolicyDataSubscription

tags:

- IndividualPolicyDataSubscription (Document)

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyDataSubscription'

responses:

'200':

description: The individual subscription resource was updated successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyDataSubscription'

'204':

description: The individual subscription resource was updated successfully and no additional content is to be sent in the response message.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete the individual Policy Data subscription

operationId: DeleteIndividualPolicyDataSubscription

tags:

- IndividualPolicyDataSubscription (Document)

responses:

'204':

description: Upon success, an empty response body shall be returned.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/ues/{ueId}/operator-specific-data:

get:

summary: Retrieve the operator specific policy data of an UE

operationId: ReadOperatorSpecificData

tags:

- OperatorSpecificData (Document)

parameters:

- name: ueId

in: path

description: UE Id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

- name: supp-feat

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

type: object

additionalProperties:

$ref: 'TS29505_Subscription_Data.yaml#/components/schemas/OperatorSpecificDataContainer'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Modify the operator specific policy data of an UE

operationId: UpdateOperatorSpecificData

tags:

- OperatorSpecificData (Document)

parameters:

- name: ueId

in: path

description: UE Id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

required: true

responses:

'204':

description: No content. Response to successful modification.

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Modify the operator specific policy data of an UE

operationId: ReplaceOperatorSpecificData

tags:

- OperatorSpecificData (Document)

parameters:

- name: ueId

in: path

description: UE Id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

required: true

content:

application/json:

schema:

type: object

additionalProperties:

$ref: 'TS29505_Subscription_Data.yaml#/components/schemas/OperatorSpecificDataContainer'

responses:

'200':

description: OK

content:

application/json:

schema:

type: object

additionalProperties:

$ref: 'TS29505_Subscription_Data.yaml#/components/schemas/OperatorSpecificDataContainer'

'204':

description: The resource has been successfully updated.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/plmns/{plmnId}/ue-policy-set:

parameters:

- name: plmnId

in: path

required: true

schema:

$ref: 'TS29505_Subscription_Data.yaml#/components/schemas/VarPlmnId'

get:

summary: Retrieve the UE policy set data for an H-PLMN

operationId: ReadPlmnUePolicySet

tags:

- PlmnUePolicySet (Document)

responses:

'200':

description: Upon success, a response body containing UE policies shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/UePolicySet'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'412':

$ref: 'TS29571_CommonData.yaml#/components/responses/412'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

schemas:

AmPolicyData:

description: Contains the AM policy data for a given subscriber.

type: object

properties:

praInfos:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 1

subscCats:

type: array

items:

type: string

minItems: 1

UePolicySet:

description: Contains the UE policy data for a given subscriber.

type: object

properties:

praInfos:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 1

subscCats:

type: array

items:

type: string

minItems: 1

uePolicySections:

type: object

additionalProperties:

$ref: '#/components/schemas/UePolicySection'

minProperties: 1

upsis:

type: array

items:

type: string

minItems: 1

allowedRouteSelDescs:

type: object

additionalProperties:

$ref: '#/components/schemas/PlmnRouteSelectionDescriptor'

minProperties: 1

andspInd:

type: boolean

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

osIds:

type: array

items:

$ref: '#/components/schemas/OsId'

minItems: 1

UePolicySetPatch:

description: Contains the UE policy set for a given subscriber.

type: object

properties:

uePolicySections:

type: object

additionalProperties:

$ref: '#/components/schemas/UePolicySection'

minProperties: 1

upsis:

type: array

items:

type: string

minItems: 1

andspInd:

type: boolean

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

osIds:

type: array

items:

$ref: '#/components/schemas/OsId'

minItems: 1

UePolicySection:

description: Contains the UE policy section.

type: object

properties:

uePolicySectionInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

upsi:

type: string

required:

- uePolicySectionInfo

- upsi

SmPolicyData:

description: Contains the SM policy data for a given subscriber.

type: object

properties:

smPolicySnssaiData:

type: object

additionalProperties:

$ref: '#/components/schemas/SmPolicySnssaiData'

minProperties: 1

umDataLimits:

type: object

additionalProperties:

$ref: '#/components/schemas/UsageMonDataLimit'

minProperties: 1

umData:

type: object

additionalProperties:

$ref: '#/components/schemas/UsageMonData'

minProperties: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- smPolicySnssaiData

SmPolicySnssaiData:

description: Contains the SM policy data for a given subscriber and S-NSSAI.

type: object

properties:

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

smPolicyDnnData:

type: object

additionalProperties:

$ref: '#/components/schemas/SmPolicyDnnData'

minProperties: 1

required:

- snssai

SmPolicyDnnData:

description: Contains the SM policy data for a given DNN (and S-NSSAI).

type: object

properties:

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

allowedServices:

type: array

items:

type: string

minItems: 1

subscCats:

type: array

items:

type: string

minItems: 1

gbrUl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

gbrDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

adcSupport:

type: boolean

subscSpendingLimits:

type: boolean

ipv4Index:

$ref: '#/components/schemas/IpIndex'

ipv6Index:

$ref: '#/components/schemas/IpIndex'

offline:

type: boolean

online:

type: boolean

chfInfo:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/ChargingInformation'

refUmDataLimitIds:

type: object

additionalProperties:

$ref: '#/components/schemas/LimitIdToMonitoringKey'

minProperties: 1

mpsPriority:

type: boolean

mcsPriority:

type: boolean

imsSignallingPrio:

type: boolean

mpsPriorityLevel:

type: integer

mcsPriorityLevel:

type: integer

praInfos:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 1

bdtRefIds:

type: object

additionalProperties:

$ref: 'TS29122_CommonData.yaml#/components/schemas/BdtReferenceId'

nullable: true

minProperties: 1

nullable: true

required:

- dnn

UsageMonDataLimit:

description: Contains usage monitoring control data for a subscriber.

type: object

properties:

limitId:

type: string

scopes:

type: object

additionalProperties:

$ref: '#/components/schemas/UsageMonDataScope'

minProperties: 1

umLevel:

$ref: '#/components/schemas/UsageMonLevel'

startDate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

endDate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

usageLimit:

$ref: 'TS29122_CommonData.yaml#/components/schemas/UsageThreshold'

resetPeriod:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

required:

- limitId

UsageMonData:

description: Contains remain allowed usage data for a subscriber.

type: object

properties:

limitId:

type: string

scopes:

type: object

additionalProperties:

$ref: '#/components/schemas/UsageMonDataScope'

minProperties: 1

umLevel:

$ref: '#/components/schemas/UsageMonLevel'

allowedUsage:

$ref: 'TS29122_CommonData.yaml#/components/schemas/UsageThreshold'

resetTime:

$ref: '#/components/schemas/TimePeriod'

required:

- limitId

LimitIdToMonitoringKey:

description: Contains the limit identifier and the corresponding monitoring key for a given S-NSSAI and DNN.

type: object

properties:

limitId:

type: string

monkey:

type: array

items:

type: string

minItems: 1

required:

- limitId

nullable: true

UsageMonDataScope:

description: Contains a SNSSAI and DNN combinations to which the UsageMonData instance belongs to.

type: object

properties:

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

dnn:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

minItems: 1

required:

- snssai

TimePeriod:

description: Contains the periodicity for the defined usage monitoring data limits.

type: object

properties:

period:

$ref: '#/components/schemas/Periodicity'

maxNumPeriod:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

required:

- period

SponsorConnectivityData:

description: Contains the sponsored data connectivity related information for a sponsor identifier.

type: object

properties:

aspIds:

type: array

items:

type: string

required:

- aspIds

BdtData:

description: Contains the background data transfer data.

type: object

properties:

aspId:

type: string

transPolicy:

$ref: 'TS29554_Npcf_BDTPolicyControl.yaml#/components/schemas/TransferPolicy'

bdtRefId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/BdtReferenceId'

nwAreaInfo:

$ref: 'TS29554_Npcf_BDTPolicyControl.yaml#/components/schemas/NetworkAreaInfo'

numOfUes:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

volPerUe:

$ref: 'TS29122_CommonData.yaml#/components/schemas/UsageThreshold'

required:

- aspId

- transPolicy

PolicyDataSubscription:

description: Identifies a subscription to policy data change notification.

type: object

properties:

notificationUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifId:

type: string

monitoredResourceUris:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

monResItems:

type: array

items:

$ref: '#/components/schemas/ResourceItem'

minItems: 1

expiry:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- notificationUri

- monitoredResourceUris

PolicyDataChangeNotification:

description: Contains changed policy data for which notification was requested.

type: object

properties:

amPolicyData:

$ref: '#/components/schemas/AmPolicyData'

uePolicySet:

$ref: '#/components/schemas/UePolicySet'

plmnUePolicySet:

$ref: '#/components/schemas/UePolicySet'

smPolicyData:

$ref: '#/components/schemas/SmPolicyData'

usageMonData:

$ref: '#/components/schemas/UsageMonData'

SponsorConnectivityData:

$ref: '#/components/schemas/SponsorConnectivityData'

bdtData:

$ref: '#/components/schemas/BdtData'

opSpecData:

$ref: 'TS29505_Subscription_Data.yaml#/components/schemas/OperatorSpecificDataContainer'

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

sponsorId:

type: string

bdtRefId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/BdtReferenceId'

usageMonId:

type: string

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

delResources:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

minItems: 1

notifId:

type: string

reportedFragments:

type: array

items:

$ref: '#/components/schemas/NotificationItem'

minItems: 1

PlmnRouteSelectionDescriptor:

description: Contains the route selection descriptors (combinations of SNSSAI, DNNs, PDU session types, and SSC modes) allowed by subscription to the UE for a serving PLMN

type: object

properties:

servingPlmn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

snssaiRouteSelDescs:

type: array

items:

$ref: '#/components/schemas/SnssaiRouteSelectionDescriptor'

minItems: 1

required:

- servingPlmn

SnssaiRouteSelectionDescriptor:

description: Contains the route selector parameters (DNNs, PDU session types and SSC modes) per SNSSAI

type: object

properties:

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

dnnRouteSelDescs:

type: array

items:

$ref: '#/components/schemas/DnnRouteSelectionDescriptor'

minItems: 1

required:

- snssai

DnnRouteSelectionDescriptor:

description: Contains the route selector parameters (PDU session types and SSC modes) per DNN

type: object

properties:

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

sscModes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SscMode'

minItems: 1

pduSessTypes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

minItems: 1

required:

- dnn

SmPolicyDataPatch:

description: Contains the SM policy data for a given subscriber.

type: object

properties:

umData:

type: object

additionalProperties:

$ref: '#/components/schemas/UsageMonData'

minProperties: 1

nullable: true

smPolicySnssaiData:

type: object

additionalProperties:

$ref: '#/components/schemas/SmPolicySnssaiDataPatch'

minProperties: 1

SmPolicySnssaiDataPatch:

description: Contains the SM policy data for a given subscriber and S-NSSAI.

type: object

properties:

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

smPolicyDnnData:

type: object

additionalProperties:

$ref: '#/components/schemas/SmPolicyDnnDataPatch'

minProperties: 1

required:

- snssai

SmPolicyDnnDataPatch:

description: Contains the SM policy data for a given DNN (and S-NSSAI).

type: object

properties:

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

bdtRefIds:

type: object

additionalProperties:

$ref: 'TS29122_CommonData.yaml#/components/schemas/BdtReferenceId'

nullable: true

minProperties: 1

nullable: true

required:

- dnn

#

ResourceItem:

description: Identifies a subscription to policy data change notification when the change occurs in a fragment (subset of resource data) of a given resource.

type: object

properties:

monResourceUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

items:

type: array

items:

$ref: '#/components/schemas/ItemPath'

minItems: 1

required:

- monResourceUri

- items

#

NotificationItem:

description: Identifies a data change notification when the change occurs in a fragment (subset of resource data) of a given resource.

type: object

properties:

resourceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifItems:

type: array

items:

$ref: '#/components/schemas/UpdatedItem'

minItems: 1

required:

- resourceId

- notifItems

#

UpdatedItem:

description: Identifies a fragment of a resource.

type: object

properties:

item:

$ref: '#/components/schemas/ItemPath'

value:{}

required:

- item

- value

#

IpIndex:

type: integer

OsId:

type: string

format: uuid

UsageMonLevel:

anyOf:

- type: string

enum:

- SESSION_LEVEL

- SERVICE_LEVEL

- type: string

Periodicity:

anyOf:

- type: string

enum:

- YEARLY

- MONTHLY

- WEEKLY

- DAILY

- HOURLY

- type: string

#

ItemPath:

description: Identifies a fragment (subset of resource data) of a given resource.

type: string

*** End of Changes ***