TSG-CT WG3 Meeting #111-e	C3-204301

E-Meeting, 19th – 28th August 2020												(Revision of C3-204133)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.512 | CR | 0546 | rev | 1 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to policy control functions for TSN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17, Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | The SMF shall support the functions for TSN |  |  |  |  |  |  |  |  |
| Summary of change: |  | SMF transfers the DS-TT PMIC transparently towards/from the UE/DS-TT and transferring the B/PMIC transparently towards/from the UPF/NW-TT, if applicable; SMF adapts received TSCAI to 5GS GM and transferring to the AN-RAN; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how the SMF supports the TSN |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.3.1, 4.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact the OpenAPI file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

…

# Proposed changes:

# *** 1st Change ***

4.1.3.1	Policy Control Function (PCF)

The PCF is responsible for policy control decisions and flow based charging control functionalities. The PCF provides the following:

-	policies for application and service data flow detection, gating, QoS, flow based charging, traffic steering control, usage monitoring control, access traffic steering, switching and steering within a MA PDU Session, access network information report, TSN BMIC, TSN PMIC and TSN TSCAI input container and RAN support information to the SMF.

The policy decisions made by the PCF may be based on one or more of the following:

-	Information obtained from the AF, e.g. the session, media and subscriber related information;

-	Information obtained from the UDR;

-	Information obtained from the AMF, e.g. UE related and access related information;

-	Information obtained from the SMF;

-	Information obtained from the NWDAF;

-	Information obtained from the NEF;

-	information from CHF; and

-	PCF pre-configured policy context.

*** Next Change ***

4.1.3.2	NF Service Consumers

The SMF is responsible for the enforcement of session management related policy decisions from the PCF, related to service flow detection, QoS, charging, gating, traffic usage reporting, traffic steering and access traffic steering, switching and splitting within a MA PDU Session.

The SMF shall support:

-	sending the PDU session related attributes to the PCF;

-	requesting and receiving the PCC rule(s) from the PCF;

-	binding of service data flows to QoS flow as defined in 3GPP TS 29.513 [7];

-	deriving rule(s) from the PCC rule(s) and then providing those rules to the user plane function or remove the rule(s) from the user plane as defined in 3GPP TS 29.244 [13];

-	deriving the QoS rules towards the UE;

-	deriving the QoS profile towards the access network;

-	deriving the ATSSS rules towards the UE if applicable;

-	Transferring the DS-TT PMIC transparently towards/from the UE/DS-TT and transferring the B/PMIC transparently towards/from the UPF/NW-TT, if applicable;

-	Adapting received TSCAI to 5GS GM and transferring to the AN-RAN;

-	handling the policy control request trigger; and

-	handling the PDU session related policy information.

NOTE:	SMF functionality related to event exposure is defined in 3GPP TS 29.508 [12].

*** End of Changes ***