3GPP TSG-CT3 Meeting #118e	C3-215332

# E-Meeting, 11th – 15th October 2021																		C3-215300

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.522 | CR | 0438 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New Nnef_MBSTMGI service definition – Procedures part |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SA2 has further progressed the Stage 2 normative work on 5MBS WI and specified in 3GPP TS 23.247 (clause 9.4) the new NEF services that need to be supported. The new Nnef_MBSTMGI service exposed by the NEF needs hence to be defined in TS 29.522. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to: - Define the procedures part of the new Nnef_MBSTMGI service exposed by the NEF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - 5MBS requirements from Stage 2 on Nnef_MBSTMGI service is not implemented in Stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 4.1, 4.4.1, 4.4.x (new subclause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact OpenAPI specification files. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of changes * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.502: "Procedures for the 5G system".

[3]	3GPP TS 23.501: "System Architecture for the 5G".

[4]	3GPP TS 29.122: "T8 reference point for northbound Application Programming Interfaces (APIs)".

[5]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[6]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[7]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[8]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[9]	3GPP TS 29.521: "5G System; Binding Support Management Service; Stage 3".

[10]	Void.

[11]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[12]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3".

[13]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[14]	3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[15]	Void.

[16]	IETF RFC 5246: "The Transport Layer Security (TLS) Protocol Version 1.2".

[17]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[18]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[19]	3GPP TS 29.554: "5G System; Background Data Transfer Policy Control Service; Stage 3".

[20]	3GPP TS 29.504: "5G System; Unified Data Repository Services; Stage 3".

[21]	3GPP TR 21.900: "Technical Specification Group working methods".

[22]	3GPP TS 29.523: "5G System; Policy Control Event Exposure Service; Stage 3".

[23]	3GPP TS 29.519: "5G System; Usage of the Unified Data Repository service for Policy Control Data, Application Data and Structured Data for Exposure; Stage 3".

[24]	3GPP TS 29.541: "5G System; Network Exposure (NE) function services for Non-IP Data Delivery (NIDD); Stage 3".

[25]	3GPP TS 29.542: "5G System, Session management services for Non-IP Data Delivery (NIDD); Stage 3".

[26]	3GPP TS 29.508: "5G System; Session Management Event Exposure Service; Stage 3".

[27]	3GPP TS 29.520: "5G System; Network Data Analytics Services; Stage 3".

[28]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G system (5GS)".

[29]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[30]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[31]	Void

[32]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[33]	3GPP TS 24.588: "Vehicle-to-Everything (V2X) services in 5G System (5GS); User Equipment (UE) policies; Stage 3".

[34]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[35]	3GPP TS 29.515: "5G System; Gateway Mobile Location Services; Stage 3".

[36]	3GPP TS 23.273: "5G System Location Services (LCS)".

[37]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[38]	3GPP TS 29.535: "5G System; AKMA Anchor Services; Stage 3".

[39]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[40]	IETF RFC 7542: "The Network Access Identifier".

[41]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[42]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[43]	3GPP TS 29.534: "5G System; Access and Mobility Policy Authorization Service; Stage 3".

[44]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[45]	IEEE Std 1588-2019: "IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control".

[46]	IEEE Std 802.1AS-2020: "IEEE Standard for Local and metropolitan area networks--Timing and Synchronization for Time-Sensitive Applications".

[47]	3GPP TS 29.536: "5G System; Network Slice Admission Control Services; Stage 3".

[48]	3GPP TS 24.526: "User Equipment (UE) policies for 5G System (5GS); Stage 3".

[49]	3GPP TS 24.555: "Proximity based services (ProSe) in 5G system (5GS); User Equipment (UE) policies; Stage 3".

[50]	3GPP TS 29.565: "5G System; Time Sensitive Communication and Time Synchronization Function Services; Stage 3".

[51]	IEEE 802.1Q: "Virtual Bridged Local Area Networks".

[aa]	3GPP TS 23.247: " Architectural enhancements for 5G multicast-broadcast services; Stage 2".

* * * Next changes * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5MBS	5G Multicast/Broadcast Services

A-KID	AKMA Key IDentifier

A-TID	AKMA Temporary UE IDentifier

AAnF	AKMA Anchor Function

ACS	Auto-Configuration Server

AF	Application Function

AKMA	Authentication and Key Management for Applications

AM	Access and Mobility management

BDT	Background Data Transfer

CAPIF	Common API Framework

CP	Communication Pattern

DN	Data Network

DNAI	DN Access Identifier

DNN	Data Network Name

ECS	Edge Configuration Server

GMLC	Global Mobile Location Centre

GPSI	Generic Public Subscription Identifier

IPTV	Internet Protocol Television

KAF	AKMA Application Key

MBS	Multicast/Broadcast Service.

MB-SMF	Multicast/Broadcast Session Management Function.

MO-LR	Mobile Originated Location Request

NEF	Network Exposure Function

NSAC	Network Slice Admission Control

NSACF	Network Slice Admission Control Function

PCF	Policy Control Function

PCRF	Policy and Charging Rule Function

PFD	Packet Flow Description

PFDF	Packet Flow Description Function

REST	Representational State Transfer

SCEF	Service Capability Exposure Function

S-NSSAI	Single Network Slice Selection Assistance Information

TMGI	Temporary Mobile Group Identity

TSC	Time Sensitive Communication

TSCAI	Time Sensitive Communication Assistance Information

TSCTSF	Time Sensitive Communication and Time Synchronization Function

UDR	Unified Data Repository

UP	User Plane

URSP	UE Route Selection Policy

WB	Wide Band

* * * Next changes * * * *

## 4.1	Overview

The NEF Northbound interface is between the NEF and the AF. It specifies RESTful/RPC APIs that allow the AF to access the services and capabilities provided by 3GPP network entities and securely exposed by the NEF.

This document also specifies the procedures triggered at the NEF by API requests from the AF and by event notifications received from 3GPP network entities.

The stage 2 level requirements and signalling flows for the NEF Northbound interface are defined in 3GPP TS 23.502 [2], and in 3GPP TS 23.247 [aa] for 5MBS specific aspects.

The NEF Northbound interface supports the following procedures:

1)	Procedures for Monitoring

2)	Procedures for Device Triggering

3)	Procedures for resource management of Background Data Transfer

4)	Procedures for CP Parameters, Network Configuration Parameters Provisioning, 5G LAN Parameters Provisioning, ACS Configuration Parameter Provisioning, Location Privacy Indication Parameters Provisioning, Time Synchronization Exposure and ECS address provisioning

5)	Procedures for PFD Management

6)	Procedures for Traffic Influence

7)	Procedures for changing the chargeable party at session set up or during the session

8)	Procedures for setting up an AF session with required QoS

9)	Procedures for MSISDN-less Mobile Originated SMS

10)	Procedures for non-IP data delivery

11)	Procedures for analytics information exposure

12)	Procedure for applying BDT policy

13)	Procedures for Enhanced Coverage Restriction Control

14)	Procedures for IPTV Configuration

15)	Procedures for Service Parameter Provisioning

16)	Procedures for RACS Parameter Provisioning

17)	Procedures for Mobile Originated Location Request

18)	Procedures for AKMA

19)	Procedures for AF triggered Access and Mobility Influence

20)	Procedures for AF triggered Access and Mobility Policy Authorization

yy)	Procedures for TMGI allocation, deallocation, expiry timer refresh and timer expiry notification

Which correspond to the following services respectively, supported by the NEF as defined in 3GPP TS 23.502 [2]:

1)	Nnef_EventExposure service and Nnef_APISupportCapability service

2)	Nnef_Trigger service

3)	Nnef_BDTPNegotiation service

4)	Nnef_ParameterProvision service

5)	Nnef_PFDManagement service

6)	Nnef_TrafficInfluence service

7)	Nnef_ChargeableParty service

8)	Nnef_AFsessionWithQoS service

9)	Nnef_MSISDN-less_MO_SMS service

10)	Nnef_NIDDConfiguration and Nnef_NIDD services

11)	Nnef_AnalyticsExposure service

12)	Nnef_ApplyPolicy service

13)	Nnef_ECRestriction service

14)	Nnef_IPTVConfiguration service

15)	Nnef_ServiceParameter service

16)	Nnef_UCMFProvisioning service

17)	Nnef_Location service

18)	Nnef_AKMA service

19)	Nnef_AMInfluence service

20)	Nnef_AMPolicyAuthorization service

bb)	Nnef_MBSTMGI service

NOTE 1:	For Nnef_PFDManagement service, only the Nnef_PFDManagement_Create/Update/Delete service operations are applicable for the NEF Northbound interface.

NOTE 2:	For Nnef_NIDD service, NF consumer other than the AF does not use the NEF Northbound interface.

NOTE 3:	For Nnef_NIDDConfiguration service, the Nnef_NIDDConfiguration_Trigger service operation is only applicable for the NEF Northbound interface.

NOTE 4:	The Nnef_APISupportCapability service is only applicable in the MonitoringEvent API when the monitoring type sets to "API_SUPPORT_CAPABILITY".

*** Next Changes ***

### 4.4.1	Introduction

All procedures that operate across the NEF Northbound interface, as specified in 3GPP TS 23.502 [2], and in 3GPP TS 23.247 [aa] for 5MBS specific aspects, are specified in the following subclauses.

* * * Next changes * * * *

### 4.4.x	Procedures for MBS Session Management for 5MBS

#### 4.4.x.1	General

The procedures described in the subclauses below are used by an AF to interact with the 5GC for 5G MBS session(s) management as defined in 3GPP TS 23.247 [aa].

#### 4.4.x.2	Procedures for TMGI management for 5MBS

#### 4.4.x.2.1	General

The procedures described in the subclauses below are used by an AF to request and manage TMGI(s) for 5G MBS session(s) as defined in subclause 7.1 of 3GPP TS 23.247 [aa].

#### 4.4.x.2.2	Procedures for TMGI(s) allocation or TMGI(s) expiry time refresh

This procedure is used by an AF to request the allocation of TMGI(s) for new 5G MBS session(s) or the refresh of the expiry time of already allocated TMGI(s).

In order to request the allocation of TMGI(s) for new 5G MBS session(s) or the refresh of the expiry time of already allocated TMGI(s), an AF sends a Nnef_MBSTMGI_Allocation Request message to the NEF. The request body shall include the TmgiAllocRequest data structure which shall contain:

-	within the "afId" attribute, the identifier of the AF that is sending the request;

-	within the "tmgiParams" attribute, the parameters (e.g. number of TMGI(s) to be allocated, etc.) to request the allocation of TMGI(s) for new 5G MBS session(s) or the refresh of the expiry time of already allocated TMGI(s);

and may contain:

-	within the "notificationUri" attribute, the notification URI via which the AF desires to receive notifications on timer expiry for TMGI(s);

-	within the "requestTestNotification" attribute, an indication on whether the NEF should send a test notification;

-	within the "websockNotifConfig" attribute, the configuration parameters to set up notification delivery over Websocket protocol; and/or

-	within the "suppFeat" attribute, the features supported by the AF.

The NEF checks whether the AF is authorized or not as defined in subclause 6.1.1 of 3GPP TS 23.247 [aa]. If the AF is authorized, the NEF may query the NRF to discover and select an MB-SMF (service) instance that can handle this request. Then, the NEF conveys this TMGI(s) allocation or expiry time refresh request to the MB-SMF using the Nmbsmf_TMGI service.

Editor's Note:	The Nmbsmf_TMGI service will be defined by CT4.

Upon reception of a reply from the MB-SMF, the NEF forwards the received information (e.g. allocated TMGI(s), expiry time or updated expiry time, etc.) in a Nnef_MBSTMGI_Allocation Response message with a "200 OK" status code to the AF. The response body shall include the TmgiAllocResponse data structure which shall contain:

-	within the "tmgiInfo" attribute, the TMGI(s) allocation information or the refreshed expiry time for already allocated TMGI(s);

and may contain:

-	within the "suppFeat" attribute, the features supported by both the AF and the NEF.

Editor's note:	Error cases and the related responses are FFS.

#### 4.4.x.2.3	Procedures for TMGI(s) deallocation

This procedure is used by an AF to request the deallocation of previously allocated TMGI(s).

In order to request the deallocation of previously allocated TMGI(s), an AF sends a Nnef_MBSTMGI_Deallocation Request message to the NEF. The request body shall include:

-	within the "afId" attribute, the identifier of the AF that is sending the request; and

-	within the "tmgis" attribute, the the list of TMGI(s) for which deallocation is requested;

The NEF checks whether the AF is authorized or not as defined in subclause 6.1.1 of 3GPP TS 23.247 [aa]. If the AF is authorized, the NEF may query the NRF to discover and select an MB-SMF (service) instance that can handle this request. Then, the NEF conveys this TMGI(s) deallocation request to the MB-SMF using the Nmbsmf_TMGI service.

Editor's Note:	The Nmbsmf_TMGI service will be defined by CT4.

Upon reception of a reply from the MB-SMF confirming the deallocation of the TMGI(s), the NEF forwards this confirmation in a Nnef_MBSTMGI_Deallocation Response message with a "204 No Content" status code to the AF.

Editor's note:	Error cases and the related responses are FFS.

#### 4.4.x.2.4	Procedures for TMGI(s) timer expiry notification

This procedure is used by the NEF to notify an AF of timer expiry for previously allocated TMGI(s).

In order to notify an AF of timer expiry for previously allocated TMGI(s), the NEF sends a Nnef_MBSTMGI_ExpiryNotify Request message to the AF. The request body shall include the list of TMGI(s) whose timer expired within the "tmgis" attribute of the ExipryNotify data structure.

Upon reception of this notification request, the AF acknowledges its successful reception by sending a Nnef_MBSTMGI_ExpiryNotify Response message with an HTTP "204 No Content" status code.

Editor's note:	Error cases and the related responses are FFS.

* * * End of changes * * * *