3GPP TSG-CT3 Meeting #120-e	C3-221081

# Online, , 17th Feb 2022 - 25th Feb 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.522 | CR | 0500 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolutions related to URSP guidance inputs and procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The descriptions related to URSP guidance in 23.502 4.15.6.10 are meanwhile clear about the following aspects: - It is the NEF (and not the PCF) that converts the spatial validity (before storing it in the UDR), whle the spatial validity is a geographic area that is converted to a list of TAIs and it cannot contain DNAIs. (The PCF converts this to URSP Location criteria then as per 23.548). - The ServiceSpecificAuthorization with the UDM takes place after receiving the AF request and before interacting with the UDR. - The AF provides no precedence for its high-level URSP guidance requests, but only for the Route Selection Parameter Sets within each request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addressed and resolved ENs related to the above points in the URSP guidance procedure and data model. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with stage 2 requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 4.4.20, 5.11.2.3.4, 5.11.2.3.5, A.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR contains a backwards compatible correction to the OpenAPI file of the ServiceParameter API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5MBS	5G Multicast/Broadcast Services

A-KID	AKMA Key IDentifier

A-TID	AKMA Temporary UE IDentifier

AAnF	AKMA Anchor Function

ACS	Auto-Configuration Server

AF	Application Function

AKMA	Authentication and Key Management for Applications

AM	Access and Mobility management

BDT	Background Data Transfer

CAPIF	Common API Framework

CP	Communication Pattern

DN	Data Network

DNAI	DN Access Identifier

DNN	Data Network Name

EAS	Edge Application Server

ECS	Edge Configuration Server

GMLC	Global Mobile Location Centre

GPSI	Generic Public Subscription Identifier

IPTV	Internet Protocol Television

KAF	AKMA Application Key

MBS	Multicast/Broadcast Service

MB-SMF	Multicast/Broadcast Session Management Function

MO-LR	Mobile Originated Location Request

NEF	Network Exposure Function

NSAC	Network Slice Admission Control

NSACF	Network Slice Admission Control Function

PCF	Policy Control Function

PCRF	Policy and Charging Rule Function

PFD	Packet Flow Description

PFDF	Packet Flow Description Function

REST	Representational State Transfer

SCEF	Service Capability Exposure Function

S-NSSAI	Single Network Slice Selection Assistance Information

SSM	Source Specific IP Multicast address

TAI	Traffic Area Identity

TMGI	Temporary Mobile Group Identity

TSC	Time Sensitive Communication

TSCAI	Time Sensitive Communication Assistance Information

TSCTSF	Time Sensitive Communication and Time Synchronization Function

UDR	Unified Data Repository

UP	User Plane

URSP	UE Route Selection Policy

WB	Wide Band

# * * * * Next change * * * *

### 4.4.20	Procedures for service specific parameter provisioning

These procedures are used by an AF to provide service specific parameters to the 5G system via the NEF.

In order to provision service specific parameters to the 5G system, the AF shall send an HTTP POST message to the NEF targetting the resource "Service Parameter Subscriptions", the HTTP POST message shall include the ServiceParameterData data structure as request body. The ServiceParameterData data structure shall include:

-	service description via:

a)	a combination of DNN and S-NSSAI within the "dnn" attribute and the "snssai" attribute respectively;

b)	an AF service identifier within the "afServiceId" attribute; or

c)	an application identifier within the "appId" attribute;

-	indication of the UEs to which the subscription applies via:

a)	identification of an individual UE within the "gpsi" attribute;

b)	an IPv4 address of the UE within the "ueIpv4" attribute;

c)	an IPv6 address of the UE within the "ueIpv6" attribute;

d)	a MAC address of the UE within the "ueMac" attribute;

e)	an identification of a group of UE(s) within the "exterGroupId" attribute; or

f)	an identification of any UE within the "anyUeInd" attribute.

-	service parameters for at least one of the following:

-	V2X service parameters via:

a)	configuration parameters for V2X communications over PC5 within the "paramOverPc5" attribute;

b)	configuration parameters for V2X communications over Uu within the "paramOverUu" attribute;

-	if the "ProSe" feature is supported, 5G ProSe service parameters via:

a)	configuration parameters for 5G ProSe direct discovery within the "paramForProSeDd" attribute;

b)	configuration parameters for 5G ProSe direct communication within the "paramForProSeDc" attribute;

c)	configuration parameters for 5G ProSe UE-to-network relay, including configuration parameters for 5G ProSe UE-to-network relay UE within the "paramForProSeU2NRelUe" attribute and configuration parameters for 5G ProSe remote UE within the "ParamForProSeRemUe" attribute;

-	If the "EnEDGE" feature is supported, URSP service parameters via:

a)	contents for the AF influence on URSP within the "urspInfluence" attribute, which shall include one or more URSP rule requests. Each URSP rule request may include a traffic descriptor within the "trafficDesc" attribute and one or more route selection parameter sets within the "routeSelParamSets" attribute. Each route selection parameter set may include a precedence value within the "precedence" attribute, a DNN within the "dnn" attribute, an S-NSSAI within the "snssai" attribute, and a spatial validity condition within the "spatialValidity" attribute. If the request contains only one route selection parameter set, each of the optional attributes "dnn", "snssai", "precedence", and "spatialValidity" that is missing from the request may be complemented by the NEF based on local configuration for the provided AF service identifier. It is up to the NEF to transform the information of the "spatialValidity" attribute into a list of TAIs.

Editor's Note:	It is FFS to consider if "urspInfluence" should be encoded exactly in the same way that URSP rules are encoded in NAS messages (see TS 24.526).

Editor's Note:	It is FFS to consider if a separate feature should be used for "urspInfluence" instead of the EnEDGE feature.

and may include:

-	If the "AfNotifications" feature is supported,

a)	subscription to event notification of the outcome related to invocation of service parameter provisioning within the "subNotifEvents" attribute;

b)	notification URI within the "notificationDestination" attribute;

In order to update an existing service parameter subscription, the AF shall send an HTTP PUT or HTTP PATCH message to the NEF targetting the resource "Individual Service Parameter Subscription" and requesting to change the subscription with the Transaction Reference ID in the "afTransId" attribute which is assigned by NEF in the HTTP POST response message of service parameter provision subscription.

In order to delete an existing service parameter subscription, the AF shall send an HTTP DELETE message to the NEF targetting the resource "Individual Service Parameter Subscription".

Upon receipt of the HTTP request from the AF, and if the AF is authorized, the NEF shall interact with the UDM by invoking the Nudm_SubscriberDataManagement service as described in 3GPP TS 29.503 [17] to retrieve the SUPI or Internal Group Identifier. NEF may, based on local configuration, complement missing service parameters. Additionally, based on operator's local policy, NEF may support service specific authorization as described in subclause 4.15.6.10 in 3GPP TS 23.502 [2]. The NEF shall assign a Transaction Reference ID to the Nnef_ServiceParameter_Create request, then the NEF shall interact with the UDR to create, update or delete the associated service parameters including the Notification Correlation Id (referred as "policDelivNotifCorreId" attribute in 3GPP TS 29.519 [23]) which is derived from the previously assigned Transaction Reference ID by using the Nudr_DataRepository service as defined in 3GPP TS 29.519 [23]. If the NEF receives an error code from the UDR, the NEF shall not create, update or delete the resource and shall respond to the AF with a proper error status code.

After receiving a successful response from the UDR, the NEF shall,

-	for an HTTP POST request, create an "Individual Service Parameter Subscription" resource which represents the Service Parameter provisioning request, addressed by a URI that contains the AF Identifier and a NEF-created configuration identifier, and shall respond to the AF with a 201 Created status code, including a Location header field containing the URI for the created resource. The AF shall use the URI received in the Location header in subsequent requests to the NEF to refer to this Service Parameter Subscription.

-	for an HTTP PUT or HTTP PATCH request, update the "Individual Service Parameter Subscription" resource which represents the service parameter provisioning request, and respond to the AF with a 200 OK or 204 No Content status code.

-	for an HTTP DELETE request, remove all properties of the resource and delete the corresponding active "Individual Service Parameter Subscription" resource, then respond to the AF with a 204 No Content status code.

When the NEF receives the Service Specific Authorization Update information from the UDM by Nudm_ServiceSpecificAuthorization_UpdateNotify service operation defined in 3GPP TS 29.503 [17], if the authorization is revoked, the NEF shall provide a notification to AF by sending HTTP POST message that include the AfNotification data structure and shall include the "afTransId" attribute, "authResult" attribute and one of the target UE in the request body. Upon receipt of the notification, the AF shall respond with a "204 No Content" status code to confirm the received notification.

When the NEF receives the notification of the outcome of invocation related to AF provisioned service parameters from the PCF by Npcf_EventExposure_Notify service operation defined in 3GPP TS 29.523 [22], the NEF shall provide a notification to AF by sending HTTP POST message that include the AfNotification data structure and shall include the "afTransId" attribute, "reportEvent" attribute and one of the targeted UE in the request body. Upon receipt of the notification, the AF shall respond with a "204 No Content" status code to confirm the received notification.

# * * * * Next change * * * *

#### 5.11.2.2	Reused data types

The data types reused by the ServiceParameter API from other specifications are listed in table 5.9.2.2-1.

Table 5.11.2.2-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Dnn | 3GPP TS 29.571 [8] | Identifies a DNN. |
| ExternalGroupId | 3GPP TS 29.122 [4] | External Group Identifier for a user group. |
| Gpsi | 3GPP TS 29.571 [8] | Identifies a GPSI. |
| IpAddr | 3GPP TS 29.571 [8] | UE IP Address. |
| IPv4Addr | 3GPP TS 29.571 [8] | Identifies an IPv4 address. |
| IPv6Addr | 3GPP TS 29.571 [8] | Identifies an IPv6 address. |
| Link | 3GPP TS 29.122 [4] |  |
| MacAddr48 | 3GPP TS 29.571 [8] | Identifies an MAC address. |
| MtcProviderInformation | 3GPP TS 29.571 [8] | Indicates MTC provider information. |
| Snssai | 3GPP TS 29.571 [8] | Identifies the S-NSSAI. |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features defined in table 5.11.3-1. |
| Tai | 3GPP TS 29.571 [8] | Tracking Area Identity information. |
| TrafficDescriptor | 3GPP TS 29.122 [4] | Identifies a traffic descriptor as defined in Table 5.2.1 of 3GPP TS 24.526 [48], octets v+5 to w. |
| Uinteger | 3GPP TS 29.571 [8] | Unsigned integer. |

# * * * * Next change * * * *

##### 5.11.2.3.4	Type: UrspRuleRequest

Table 5.11.2.3.4-1: Definition of type UrspRuleRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| trafficDesc | TrafficDescriptor | O | 1 | Traffic descriptor for the requested URSP |  |
| routeSelParamSets | array(RouteSelectionParameterSet) | O | 1..N | Route Selection Parameter Sets, i.e. sets of parameters that may be used to influence the Route Selection Descriptors of the URSP. |  |

# * * * * Next change * * * *

##### 5.11.2.3.5	Type: RouteSelectionParameterSet

Table 5.11.2.3.5-1: Definition of type RouteSelectionParameterSet

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dnn | Dnn | O | 0..1 | DNN to be matched with the DNN of the PDU Session. |  |
| snssai | Snssai | O | 0..1 | S-NSSAI to be matched with the S-NSSAI of the PDU Session. |  |
| precedence | Uinteger | O | 0..1 | Determines the order in which the Route Selection Descriptors are to be applied. |  |
| spatialValidity | array(string) | O | 1..N | Indicates where the route selection parameters apply. It may correspond to a geographical area (e.g. a geographic zone identifier that is known to the AF and is configured by the operator to correspond to a list of TAIs). |  |
| spatialValidityTais | array(Tai) | O | 1..N | Indicates the TAIs in which the route selection parameters apply. |  |

# * * * * Next change * * * *

# A.9	ServiceParameter API

openapi: 3.0.0

info:

title: 3gpp-service-parameter

version: 1.1.0-alpha.2

description: |

API for AF service paramter

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.522 V17.4.0; 5G System; Network Exposure Function Northbound APIs.

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.522/'

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/3gpp-service-parameter/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 5.2.4 of 3GPP TS 29.122.

paths:

/{afId}/subscriptions:

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

get:

summary: read all of the active subscriptions for the AF

tags:

- Service Parameter Subscriptions

parameters:

- name: gpsi

in: query

description: The GPSI of the requested UE(s).

required: false

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

minItems: 1

- name: ip-addrs

in: query

description: The IP address(es) of the requested UE(s).

required: false

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

minItems: 1

- name: ip-domain

in: query

description: The IPv4 address domain identifier. The attribute may only be provided if IPv4 address is included in the ip-addrs query parameter.

required: false

schema:

type: string

- name: mac-addrs

in: query

description: The MAC address(es) of the requested UE(s).

required: false

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MacAddr48'

minItems: 1

responses:

'200':

description: OK.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/ServiceParameterData'

minItems: 0

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Creates a new subscription resource

tags:

- Service Parameter Subscriptions

requestBody:

description: Request to create a new subscription resource

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceParameterData'

responses:

'201':

description: Created (Successful creation of subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceParameterData'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

notificationDestination:

'{$request.body#/notificationDestination}':

post:

requestBody:

description: Notifications upon AF Service Parameter Authorization Update, and/or AF subscribed event notification of the outcome related to the invocation of service parameters provisioning.

required: true

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/AfNotification'

minItems: 1

responses:

'204':

description: Expected response to a successful callback processing without a body

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/{afId}/subscriptions/{subscriptionId}:

parameters:

- name: afId

in: path

description: Identifier of the AF

required: true

schema:

type: string

- name: subscriptionId

in: path

description: Identifier of the subscription resource

required: true

schema:

type: string

get:

summary: read an active subscriptions for the SCS/AS and the subscription Id

tags:

- Individual Service Parameter Subscription

responses:

'200':

description: OK (Successful get the active subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceParameterData'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Updates/replaces an existing subscription resource

tags:

- Individual Service Parameter Subscription

requestBody:

description: Parameters to update/replace the existing subscription

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceParameterData'

responses:

'200':

description: OK (Successful update of the subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceParameterData'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Updates/replaces an existing subscription resource

tags:

- Individual Service Parameter Subscription

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/ServiceParameterDataPatch'

responses:

'200':

description: OK. The subscription was modified successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceParameterData'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes an already existing subscription

tags:

- Individual Service Parameter Subscription

responses:

'204':

description: No Content (Successful deletion of the existing subscription)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

ServiceParameterData:

description: Represents an individual Service Parameter subscription resource.

type: object

properties:

afServiceId:

type: string

description: Identifies a service on behalf of which the AF is issuing the request.

appId:

type: string

description: Identifies an application.

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

afTransId:

type: string

description: AF Transaction Reference Id assigned by the NEF for the AF invoked ServiceParameter_Create request. Shall be present by the NEF in HTTP responses and by the AF in HTTP update request.

externalGroupId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/ExternalGroupId'

anyUeInd:

type: boolean

description: Identifies whether the AF request applies to any UE. This attribute shall set to "true" if applicable for any UE, otherwise, set to "false".

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

ueIpv4:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

ueIpv6:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

ueMac:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MacAddr48'

self:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Link'

subNotifEvents:

type: array

items:

$ref: '#/components/schemas/Event'

minItems: 1

notificationDestination:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

requestTestNotification:

type: boolean

description: Set to true by the AF to request the NEF to send a test notification as defined in subclause 5.2.5.3 of 3GPP TS 29.122. Set to false or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

paramOverPc5:

$ref: '#/components/schemas/ParameterOverPc5'

paramOverUu:

$ref: '#/components/schemas/ParameterOverUu'

paramForProSeDd:

$ref: '#/components/schemas/ParamForProSeDd'

paramForProSeDc:

$ref: '#/components/schemas/ParamForProSeDc'

paramForProSeU2NRelUe:

$ref: '#/components/schemas/ParamForProSeU2NRelUe'

paramForProSeRemUe:

$ref: '#/components/schemas/ParamForProSeRemUe'

urspInfluence:

type: array

items:

$ref: '#/components/schemas/UrspRuleRequest'

minItems: 1

description: Contains the service parameter used to influence the URSP.

mtcProviderId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MtcProviderInformation'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

ServiceParameterDataPatch:

description: Represents the parameters to request the modification of a service parameter subscription resource.

type: object

properties:

paramOverPc5:

$ref: '#/components/schemas/ParameterOverPc5Rm'

paramOverUu:

$ref: '#/components/schemas/ParameterOverUuRm'

paramForProSeDd:

$ref: '#/components/schemas/ParamForProSeDdRm'

paramForProSeDc:

$ref: '#/components/schemas/ParamForProSeDcRm'

paramForProSeU2NRelUe:

$ref: '#/components/schemas/ParamForProSeU2NRelUeRm'

paramForProSeRemUe:

$ref: '#/components/schemas/ParamForProSeRemUeRm'

urspInfluence:

type: array

items:

$ref: '#/components/schemas/UrspRuleRequest'

minItems: 1

description: Contains the service parameter used to influence the URSP.

notificationDestination:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

ParameterOverPc5:

description: Represents configuration parameters for V2X communications over PC5 reference point.

type: string

ParameterOverPc5Rm:

description: Represents the same as the ParameterOverPc5 data type but with the nullable:true property.

type: string

nullable: true

ParameterOverUu:

description: Represents configuration parameters for V2X communications over Uu reference point.

type: string

ParameterOverUuRm:

description: Represents the same as the ParameterOverUu data type but with the nullable:true property.

type: string

nullable: true

ParamForProSeDd:

description: Represents the service parameters for 5G ProSe direct discovery.

type: string

ParamForProSeDdRm:

description: This data type is defined in the same way as the ParamForProSeDd data type, but with the OpenAPI nullable property set to true.

type: string

nullable: true

ParamForProSeDc:

description: Represents the service parameters for 5G ProSe direct communications.

type: string

ParamForProSeDcRm:

description: This data type is defined in the same way as the ParamForProSeDc data type, but with the OpenAPI nullable property set to true.

type: string

nullable: true

ParamForProSeU2NRelUe:

description: Represents the service parameters for 5G ProSe UE-to-network relay UE.

type: string

ParamForProSeU2NRelUeRm:

description: This data type is defined in the same way as the ParamForProSeU2NRelay data type, but with the OpenAPI nullable property set to true.

type: string

nullable: true

ParamForProSeRemUe:

description: Represents the service parameters for 5G ProSe Remate UE.

type: string

ParamForProSeRemUeRm:

description: This data type is defined in the same way as the ParamForProSeRemUe data type, but with the OpenAPI nullable property set to true.

type: string

nullable: true

UrspRuleRequest:

description: Contains parameters that can be used to influence the URSP.

type: object

properties:

trafficDesc:

$ref: 'TS29122_ResourceManagementOfBdt.yaml#/components/schemas/TrafficDescriptor'

routeSelParamSets:

type: array

items:

$ref: '#/components/schemas/RouteSelectionParameterSet'

minItems: 1

description: Sets of parameters that may be used to influence the Route Selection Descriptors of the URSP.

RouteSelectionParameterSet:

description: Contains parameters that can be used to influence the Route Selection Descriptors of the URSP.

type: object

properties:

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

precedence:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

spatialValidity:

type: array

items:

type: string

minItems: 1

description: Indicates where the route selection parameters apply. It may correspond to a geographical area, for example using a geographic zone identifier that is known to the AF and is configured by the operator to correspond to a list of TAIs.

spatialValidityTais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

minItems: 1

description: Indicates the TAIs in which the route selection parameters apply.

Event:

anyOf:

- type: string

enum:

- SUCCESS_UE_POL_DEL_SP

- UNSUCCESS_UE_POL_DEL_SP

- type: string

description: >

This string identifies AF subscribe to event(s) notifications related to AF provisioned service parameters.

description: >

Possible values are

- SUCCESS_UE_POL_DEL_SP: Successful UE Policy Delivery related to the invocation of AF provisioned Service Parameters.

- UNSUCCESS_UE_POL_DEL_SP: Unsuccessful UE Policy Delivery related to the invocation of AF provisioned Service Parameters.

AfNotification:

description: Notifications upon AF Service Parameter Authorization Update e.g. to revoke the authorization, and/or AF subscribed event notification of the outcome related to the invocation of service parameter provisioning.

type: object

properties:

reportEvent:

$ref: '#/components/schemas/Event'

afTransId:

type: string

description: AF transaction identifier.

authResult:

$ref: '#/components/schemas/AuthorizationResult'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

externalGroupId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/ExternalGroupId'

anyUeInd:

type: boolean

description: Identifies whether the AF request applies to any UE. This attribute shall set to "true" if applicable for any UE, otherwise, set to "false".

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

eventInfo:

$ref: '#/components/schemas/EventInfo'

required:

- afTransId

oneOf:

- required: [gpsi]

- required: [externalGroupId]

- required: [anyUeInd]

anyOf:

- required: [reportEvent]

- required: [authResult]

AuthorizationResult:

anyOf:

- type: string

enum:

- AUTH_REVOKED

- type: string

description: >

This string indicates NEF notify the AF about the service parameters authorization updates result.

description: >

Possible values are

- AUTH_REVOKED: Indicated the service parameters authorization is revoked.

EventInfo:

description: Indicates the event information.

type: object

properties:

failureCause:

$ref: '#/components/schemas/Failure'

Failure:

oneOf:

- type: string

enum:

- UNSPECIFIED

- UE_NOT_REACHABLE

- UNKNOWN

- type: string

description: >

This string represents the failure reason for the unsuccessful result. May present if the reported afSubEvent attribute is "UNSUCCESS_UE_POL_DEL_SP".

description: >

Possible values are

- UNSPECIFIED: Indicates the PCF received the UE sent UE policy delivery service cause #111 (Protocol error, unspecified).

- UE_NOT_REACHABLE: Indicates the PCF received the notification from the AMF that the UE is not reachable.

- UNKNOWN: Indicates unknown reasons upon no response from the UE, e.g. UPDS message type is not defined or not implemented by the UE, or not compatible with the UPDS state, in which the UE shall ignore the UPDS message.

# * * * * End of changes * * * *