3GPP TSG-CT3 Meeting #120e	C3-221383

# E-Meeting, 17th – 25th February 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.214 | CR | 1675 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EPS Fallback |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | en5GPccSer |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature) C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Subscription to events, as specified in TS 29.214, clause 5.3.13, can only be done in the AAR-I, and cannot be done during the modification of session information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removal from E.3 the specification of the possibility to request EPS fallback during the modification of session information. |  |  |  |  |  |  |  |  |
| U |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification leading to misinterpretations and implementation mistakes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | E.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

…

# Proposed changes:

# *** 1st Change ***

# E.3	Reporting EPS Fallback

When the feature "EPSFallbackReport" is supported, if the AF requests the PCF to report EPS Fallback, the AF shall subscribe to the "EPS_FALLBACK" within the Specific-Action AVP as described in subclause 5.3.13. The AF shall request the PCF to report the EPS Fallback in conjuction with providing the PCF with the AF session information as described in subclause 4.4.1 for the initial provisioning of session information, and the subscription to "EPS_FALLBACK" shall apply for voice media type requests along the AF session lifetime.

When the PCF then receives the EPS Fallback indication from the SMF, the PCF shall provide the corresponding EPS Fallback indication to the AF by sending an RAR (re-authorization request) command. The RAR command shall include the EPS_FALLBACK cause encoded in the Specific-Action AVP.

When the AF receives the RAR command, it shall acknowledge the command by sending an RAA (re-authorization answer) command to the PCF.

** End of Changes ***