3GPP TSG-CT3 Meeting #120e	C3-22xxxx

- E-Meeting, 17th – 25th February 2022						                      (Revision of C3-221079)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.222 | CR | 0222 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Obtain security info with API ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NBI17 |  |  |  |  |  |  | Date: |  | 2022-02-03 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In CAPIF_Security_API, for API invoker interaction with CCF, the service operation Obtain_Security_Method (PUT) response provides SecurityInformation per API interface, but in case several APIs expose the same API interface (most likely in a real deployment) with different security methods, the current data structure cannot tell for which API the security info is provided. Hence, it needs to find a way to provide security info to API invoker per service API. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add API id in SecurityInformation data type to allow per API discrimination. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CAPIF cannot support security info provisioning to API invoker properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.4.2.3. A.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | This CR introduces a backward compatible feature for CAPIF_Security_API. |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

- Additional discussion(if needed):

…

- Proposed changes:

- *** 1st Change ***

##### 8.5.4.2.3	Type: SecurityInformation

Table 8.5.4.2.3-1: Definition of type SecurityInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| interfaceDetails | InterfaceDescription | O | 1 | Details of the interface (NOTE) |  |
| aefId | string | O | 0..1 | AEF identifier (NOTE) |  |
| apiId | string | C | 0..1 | API identifier. If API invoker supplies this IE in the PUT request, CCF shall respond back with this IE and its associated security information. | SecurityInfoPerAPI |
| prefSecurityMethods | array(SecurityMethod) | M | 1..N | Security methods preferred by the API invoker for the API interface |  |
| selSecurityMethod | SecurityMethod | O | 0..1 | Supplied by the CAPIF core function, it indicates the selected security method for the API interface. If it is not provided, it means no common supported security method by the API invoker and the AEF, or the selected security method is not allowed by the local policy in the CAPIF core function. |  |
| authenticationInfo | string | O | 0..1 | Authentication related information |  |
| authorizationInfo | string | O | 0..1 | Authorization related information |  |
| NOTE: Only one of the attributes "aefId" or "interfaceDetails" shall be included. |  |  |  |  |  |

- *** Next Change ***

### 8.5.6	Feature negotiation

General feature negotiation procedures are defined in subclause 7.8. Table 8.5.6-1 lists the supported features for CAPIF_Security_API.

Table 8.5.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_test_event | Testing of notification connection is supported according to subclause 7.6. |
| 2 | Notification_websocket | The delivery of notifications over Websocket is supported according to subclause 7.6. This feature requires that the Notification_test_event feature is also supported. |
| X | SecurityInfoPerAPI | Indicates the support of negotiating and obtaining service API security method information per API. |

- *** Next Change ***

## A.6	CAPIF_Security_API

openapi: 3.0.0

info:

title: CAPIF_Security_API

description: |

API for CAPIF security management.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.2.0-alpha.1"

externalDocs:

description: 3GPP TS 29.222 V17.1.0 Common API Framework for 3GPP Northbound APIs

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/capif-security/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 7.5 of 3GPP TS 29.222.

paths:

/trustedInvokers/{apiInvokerId}:

get:

parameters:

- name: apiInvokerId

in: path

description: Identifier of an individual API invoker

required: true

schema:

type: string

- name: authenticationInfo

in: query

description: When set to 'true', it indicates the CAPIF core function to send the authentication information of the API invoker. Set to false or omitted otherwise.

schema:

type: boolean

- name: authorizationInfo

in: query

description: When set to 'true', it indicates the CAPIF core function to send the authorization information of the API invoker. Set to false or omitted otherwise.

schema:

type: boolean

responses:

'200':

description: The security related information of the API Invoker based on the request from the API exposing function.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSecurity'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29122_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

parameters:

- name: apiInvokerId

in: path

description: Identifier of an individual API invoker

required: true

schema:

type: string

requestBody:

description: create a security context for an API invoker

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSecurity'

callbacks:

notificationDestination:

'{request.body#/notificationDestination}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SecurityNotification'

responses:

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: Successful created.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSecurity'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/capif-security/v1/trustedInvokers/{apiInvokerId}'

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'414':

$ref: 'TS29122_CommonData.yaml#/components/responses/414'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

parameters:

- name: apiInvokerId

in: path

description: Identifier of an individual API invoker

required: true

schema:

type: string

responses:

'204':

description: No Content (Successful deletion of the existing subscription)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/trustedInvokers/{apiInvokerId}/update:

post:

parameters:

- name: apiInvokerId

in: path

description: Identifier of an individual API invoker

required: true

schema:

type: string

requestBody:

description: Update the security context (e.g. re-negotiate the security methods).

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSecurity'

responses:

'200':

description: Successful updated.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSecurity'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/trustedInvokers/{apiInvokerId}/delete:

post:

parameters:

- name: apiInvokerId

in: path

description: Identifier of an individual API invoker

required: true

schema:

type: string

requestBody:

description: Revoke the authorization of the API invoker for APIs.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SecurityNotification'

responses:

'204':

description: Successful revoked.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/securities/{securityId}/token:

post:

parameters:

- name: securityId

in: path

description: Identifier of an individual API invoker

required: true

schema:

type: string

requestBody:

required: true

content:

application/x-www-form-urlencoded:

schema:

$ref: '#/components/schemas/AccessTokenReq'

responses:

'200':

description: Successful Access Token Request

content:

application/json:

schema:

$ref: '#/components/schemas/AccessTokenRsp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

description: Error in the Access Token Request

content:

application/json:

schema:

$ref: '#/components/schemas/AccessTokenErr'

components:

schemas:

ServiceSecurity:

type: object

description: Represents the details of the security method for each service API interface. When included by the API invoker, it indicates the preferred method of security. When included by the CAPIF core function, it indicates the security method to be used for the service API interface.

properties:

securityInfo:

type: array

items:

$ref: '#/components/schemas/SecurityInformation'

minimum: 1

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

requestTestNotification:

type: boolean

description: Set to true by API invoker to request the CAPIF core function to send a test notification as defined in in subclause 7.6. Set to false or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- securityInfo

- notificationDestination

SecurityInformation:

type: object

description: Represents the interface details and the security method.

properties:

interfaceDetails:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/InterfaceDescription'

aefId:

type: string

description: Identifier of the API exposing function

apiId:

type: string

description: API identifier

prefSecurityMethods:

type: array

items:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/SecurityMethod'

minItems: 1

description: Security methods preferred by the API invoker for the API interface.

selSecurityMethod:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/SecurityMethod'

authenticationInfo:

type: string

description: Authentication related information

authorizationInfo:

type: string

description: Authorization related information

required:

- prefSecurityMethods

oneOf:

- required: [interfaceDetails]

- required: [aefId]

SecurityNotification:

type: object

description: Represents revoked authorization notification details.

properties:

apiInvokerId:

type: string

description: String identifying the API invoker assigned by the CAPIF core function.

aefId:

type: string

description: String identifying the AEF.

apiIds:

type: array

items:

type: string

minItems: 1

description: Identifier of the service API

cause:

$ref: '#/components/schemas/Cause'

required:

- apiInvokerId

- apiIds

- cause

AccessTokenReq:

format: x-www-form-urlencoded

description: Represents the access token request information.

properties:

grant_type:

type: string

enum:

- client_credentials

client_id:

type: string

client_secret:

type: string

scope:

type: string

required:

- grant_type

- client_id

AccessTokenRsp:

type: object

description: Represents the access token response information.

properties:

access_token:

type: string

description: JWS Compact Serialized representation of JWS signed JSON object (AccessTokenClaims)

token_type:

type: string

enum:

- Bearer

expires_in:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

scope:

type: string

required:

- access_token

- token_type

- expires_in

AccessTokenClaims:

type: object

description: Represents the claims data structure for the access token.

properties:

iss:

type: string

scope:

type: string

exp:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

required:

- iss

- scope

- exp

AccessTokenErr:

type: object

description: Represents an error in the access token request.

properties:

error:

type: string

enum:

- invalid_request

- invalid_client

- invalid_grant

- unauthorized_client

- unsupported_grant_type

- invalid_scope

error_description:

type: string

error_uri:

type: string

required:

- error

Cause:

anyOf:

- type: string

enum:

- OVERLIMIT_USAGE

- UNEXPECTED_REASON

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- OVERLIMIT_USAGE: The revocation of the authorization of the API invoker is due to the overlimit usage of the service API

- UNEXPECTED_REASON: The revocation of the authorization of the API invoker is due to unexpected reason.

*** End of Changes ***