3GPP TSG-CT3 Meeting #121-e	C3-222573

# E-Meeting, 6th – 12th April 2022													(Revision of C3-222043)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.552 | CR | 0001 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding “Prepared analytics subscription transfer” signalling flows. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2022-04-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Providing the missing procedure for “Prepared analytics subscription transfer”.as per Stage 2 requirement |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Define procedure for “Prepared analytics subscription transfer”. - Refer newly defined procedure in step1 of “Analytics Subscription Transfer initiated by source NWDAF” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedures for Analytics Transferring would be incomplete. Stage 2 requirement is not met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2, 5.4.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR adds procedure for missing “Prepared analytics subscription transfer” singalling flow. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

### 5.4.2	Analytics Subscription Transfer initiated by source NWDAF

The procedure in below figure is used by an source NWDAF instance to request the transfer of analytics subscription(s) to another target NWDAF instance, using the Nnwdaf_EventsSubscription_Transfer service operation.

If the source NWDAF discovers that the NWDAF service consumer may change concurrently to this procedure, the source NWDAF should not perform the procedure. In such a case, the source NWDAF may send a message to indicate to the NWDAF service consumer that it will not serve this subscription anymore.

NOTE 1:	To discover the possible change of NWDAF service consumer, if the Analytics Event is UE related, the source NWDAF takes actions responding to external trigger (such as UE mobility), for example, checking if the Target of Analytics Reporting is still within the serving area of the analytics consumer, if the serving area information is available.

NOTE 2:	Handling of overload situation or preparation for a graceful shutdown are preferably executed inside an NWDAF Set, when available, therefore, not requiring an analytics subscription transfer as described in this clause. Below procedure is applicable for analytics subscription transfer across NF Sets or if the NWDAF is not deployed in a Set.

Figure 5.4.2-1: Analytics subscription transfer initiated by source NWDAF

0.	The NWDAF service consumer subscribes to analytics from source NWDAF. The NWDAF service consumer may send its NF ID or serving area, enabling the source NWDAF to determine whether the following analytics subscription transfer procedure is applicable. Optionally the source NWDAF subscribes to UE mobility events.

1.	[Optional] Source NWDAF determines, e.g. triggered by a UE mobility event notification, to prepare an analytics subscription transfer to target NWDAF(s) as described in clause 5.4.X with steps 0-1, 5- 6 will be followed.

2.	Source NWDAF determines, e.g. based on the UE location information received and the analytics consumer's serving area either directly received in step 0 or indirectly received via NRF, to perform an analytics subscription transfer to target NWDAF(s). Therefore, the source NWDAF determines the analytics subscription(s) to be transferred to a target NWDAF.

3.	Source NWDAF performs an NWDAF discovery and selects the target NWDAF. NWDAF discovery may be skipped if the target NWDAF had already been discovered as part of a prepared analytics subscription transfer. In the case of aggregated analytics from multiple NWDAFs, the source NWDAF may use the set of NWDAF identifiers related to aggregated analytics to preferably select a target NWDAF that is already serving the consumer.

4.	Source NWDAF requests, using Nnwdaf_EventsSubscription_Transfer service operation, a transfer of the analytics subscription(s) determined in step 2 to the target NWDAF as described in clause 4.2.2.5 of 3GPP TS 29.520 [5], the response message in step 4b is optional.

5.	Target NWDAF accepts the analytics subscription transfer and takes over the analytics generation based on the information received from the source NWDAF.

NOTE 3:	If not yet done during a prepared analytics subscription transfer, the target NWDAF allocates a new Subscription Correlation ID to the received analytics subscriptions.

NOTE 4:	The target NWDAF can already have received information on some/all of the analytics subscriptions as part of the prepared analytics subscription transfer request received in step 1 and, thus, can already have started to prepare for the analytics generation, e.g. by having already subscribed to relevant event notifications.

6.	Target NWDAF informs the NWDAF service consumer about the successful analytics subscription transfer using a Nnwdaf_EventsSubscription_Notify request message. A new Subscription Correlation ID, which was assigned by the target NWDAF, is provided in the Subscription Correlation ID and the old Subscription Correlation Id, which was allocated by the source NWDAF, is provided in the Subscription Change Notification Correlation ID parameter of this message.

NOTE 5:	Notification correlation information in the Nnwdaf_EventsSubscription_Notify message allows the NWDAF service consumer to correlate the notifications to the initial subscription request made with the source NWDAF in step 0.

NOTE 6:	The existing Analytics context in the source NWDAF is not deleted directly but will be purged first when it was collected by the target NWDAF.

NOTE 7:	If this subscription is used as input for analytics aggregation by the NWDAF service consumer, the NWDAF service consumer may inform the other NWDAFs instance participating in this analytics aggregation that the Set of NWDAF identifiers of NWDAF instances used by the NWDAF service consumer for this analytics aggregation has changed using the Nnwdaf_EventsSubscription_Subscribe service operation as described in clause 4.2.2.2 of 3GPP TS 29.520 [5].

7.	[Conditional] If "analytics context identifier(s)" had been included in the Nnwdaf_AnalyticsSubscription_Transfer Request received in step 4, the target NWDAF requests the "analytics context" by involking the Nnwdaf_AnalyticsInfo_ContextTransfer service operation as described in clause 4.3.2.3 of 3GPP TS 29.520 [5].

8.	[Optional] Target NWDAF subscribes to relevant data source(s), if it is not yet subscribed to the data source(s) for the data required for the Analytics.

9.	[Optional] Target NWDAF confirms the analytics subscription transfer to the source NWDAF by sending Nnwdaf_EventsSubscription_Transfer response message as described in clause 4.2.2.5 of 3GPP TS 29.520 [5], if the response has not been sent in step 4b.

10.	[Optional] Source NWDAF unsubscribes with the data source(s) that are no longer needed for the remaining analytics subscriptions. In addition, Source NWDAF unsubscribes with the NWDAF(s) containing MTLF, if exist, which are no longer needed for the remaining analytics subscriptions.

NOTE 8:	At this point, the analytics subscription transfer is deemed completed, i.e. the source NWDAF can delete all information related to the successfully transferred analytics subscription.

11-12.	Target NWDAF at some point derives new output analytics based on new input data and notifies the NWDAF server consumer about the new analytics using a Nnwdaf_EventsSubscription_Notify message as described in clause 4.3.2.3 of 3GPP TS 29.520 [5].

# *** Next Change ***

### 5.4.X	Prepared analytics subscription transfer

The procedure in below figure is used by a NWDAF instance to request another NWDAF instance for a prepared analytics subscription transfer from the source NWDAF instance, using the Nnwdaf_EventsSubscription_Transfer service operation.

NOTE 1:	The source NWDAF might determine that it needs to prepare to transfer analytics to another NWDAF instance, e.g. when the source NWDAF estimates for UE related analytics subscription that the UE might enter an area which is not covered by the source NWDAF (e.g., by subscribing to AMF event exposure service for UE mobility event notifications, by performing UE mobility analytics, or by subscribing to another NWDAF providing UE mobility analytics). If the source NWDAF discovers that the analytics consumer may change concurrently to this procedure, the source NWDAF does not perform the procedure. If the procedure makes use of predictions to determine the candidate NWDAFs, care must be taken with regards to load and signalling cost when sending data to an NWDAF that will not eventually start serving the UE.

NOTE 2:	The source NWDAF might also determine that it needs to prepare to transfer analytics subscriptions to another NWDAF instance, as the source NWDAF wants to resolve an internal load situation or prepare for a graceful shutdown.

NOTE 3:	Handling of overload situation or preparation for a graceful shutdown are preferably executed inside an NWDAF Set, when available.

Figure 5.4.X-1: Prepared analytics subscription transfer

- The NWDAF service consumer subscribes to analytics from source NWDAF. The NWDAF service consumer may send its NF ID or serving area, enabling the source NWDAF to determine whether the following analytics subscription transfer procedure is applicable.

0a. Source NWDAF starts data collection from relevant data source(s) (e.g. NFs or OAM) as specified in clause 5.3.2. Source NWDAF starts generating requested analytics.

0b. Optionally the source NWDAF subscribes to UE mobility events.

1.	[Conditional] Source NWDAF determines, (e.g. triggered by a UE mobility event notification) to prepare an analytics subscription transfer to target NWDAF(s).

2.	The source NWDAF discovers candidate target NWDAF instances (e.g. Target NWDAFy and Target NWDAFz) supporting the requested analytics information for the predicted target area(s). NWDAF discovery and selection is specified in clause 6.3.13 of TS 23.501 [2]. In the case of aggregated analytics from multiple NWDAFs, the source NWDAF may use the set of NWDAF identifiers related to aggregated analytics to preferably select a target NWDAF that is already serving the consumer.

NOTE 4:	In this procedure, Target NWDAFy and Target NWDAFz are examples for target NWDAF instances that are candidates to take over those analytic subscriptions.

3a.	The source NWDAF invokes Nnwdaf_EventsSubscription_Transfer Request service operation by sending the HTTP POST request message targeting the resource “NWDAF Event Subscription Transfers" to the NWDAFy as described in clause 4.2.2.5 of 3GPP TS 29.520 [5].

3b. The candidate target NWDAFy responds with HTTP "201 Created" to the source NWDAF as described in clause 4.2.2.5 of 3GPP TS 29.520 [5].

4a.	The source NWDAF invokes Nnwdaf_EventsSubscription_Transfer Request service operation by sending the HTTP POST request message targeting the resource “NWDAF Event Subscription Transfers" to the NWDAFz as described in clause 4.2.2.5 of 3GPP TS 29.520 [5].

4b. The candidate target NWDAFz responds with HTTP "201 Created" to the source NWDAF as described in clause 4.2.2.5 of 3GPP TS 29.520 [5].

5a.	[Conditional] If "analytics context identifier(s)" had been included in the Nnwdaf_EventSubscription_Transfer received in step 3a, the NWDAFy may request the "analytics context" from the source NWDAF by invoking the Nnwdaf_AnalyticsInfo_ContextTransfer service operation by sending the HTTP GET request message targeting the resource “NWDAF Context" as described in clause 4.3.2.3 of 3GPP TS 29.520 [5].

6b. The source NWDAF respond with “200 OK” to the NWDAFy as described in clause 4.3.2.3 of 3GPP TS 29.520 [5].

6a.	[Conditional] If "analytics context identifier(s)" had been included in the Nnwdaf_EventsSubscription_Transfer Request received in step 4a, the NWDAFz may request the "analytics context" from the source NWDAF by invoking the Nnwdaf_AnalyticsInfo_ContextTransfer service operation by sending the HTTP GET request message targeting the resource “NWDAF Context" as described in clause 4.3.2.3 of 3GPP TS 29.520 [5].

6b. The source NWDAF respond with “200 OK” to the NWDAFz as described in clause 4.3.2.3 of 3GPP TS 29.520 [5].

NOTE 5:	The target NWDAFs (e.g. Taget NWDAFy and Target NWDAFz) can allocate a new Subscription Correlation ID to the received analytics subscriptions.

7.	[Optional] Based on the information received from the source NWDAF, the target NWDAFs (e.g. Target NWDAFy and Target NWDAFz) start data collection from NFs or OAM (as specified in clause 5.5.1) and analytics generation for the indicated analytics subscriptions.

NOTE 6:	After step 7, the source NWDAF initiates the analytics subscription transfer to the target NWDAF as specified in steps 4 to 12 of the analytics subscription transfer procedure illustrated in Figure 5.4.2-1.

8a. The source NWDAF cancels the prepared analytics subscription transfer to a candidate target NWDAF (e.g. NWDAFz), by invoking Nnwdaf_EventsSubscription_Transfer Request service operation by sending the HTTP DELETE request message targeting the resource “Individual NWDAF Event Subscription Transfers" to the NWDAFz as described in clause 4.2.2.5 of 3GPP TS 29.520 [5].

8b. The target NWDAF (e.g. Target NWDAFz) confirms the cancelation to the source NWDAF by responding with "204 No Content" and, if applicable, deletes any analytics data that is no longer needed. If the target NWDAF (e.g. Target NWDAFz), as part of the analytics subscription preparation, had already subscribed to entities to collect data or acquire ML model, it unsubscribes to those entities if the subscriptions are not needed for other active analytics subscriptions with the target NWDAF.

Step 8 may take place any time after step 4 if the NWDAF determines that the candidate target NWDAF (e.g. Target NWDAFz) does no longer need to prepare for the analytics subscription transfer. In particular, the source NWDAF shall cancel the prepared analytics subscription transfer to all remaining candidate target NWDAFs after one target NWDAF has accepted the analytics subscription transfer (see NOTE 6).

*** End of Changes ***