3GPP TSG CT WG3 Meeting #137	C3-245525

# Hefei, CN, 14 - 18 October, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.520 | CR | 0948 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to anaMetaInfo attribute |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNetAE19 |  |  |  |  |  | Date: |  |  | 2024-10-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In 5.1.6.2.5 and 5.2.6.2.2, the description of anaMetaInfo attribute: It shall be present if the "anaMeta" attribute was included in the request/subscription, containing the information indicated by the "anaMeta" attribute. This description is incorrect, as it is the "anaMetaInd" attribute contains the information indicated by the "anaMeta" attribute. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear description leads to misunderstanding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6.2.5, 5.2.6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not have any impact in the OpenAPI specification. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

5.1.6.2.5	Type EventNotification

Table 5.1.6.2.5-1: Definition of type EventNotification

| Attribute name |  |  | Data type |  |  |  |  | P | Cardinality |  |  | Description |  |  | Applicability |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| event |  |  | NwdafEvent |  |  |  |  | M | 1 |  |  | Event that is notified. |  |  |  |  |  |
| start |  |  | DateTime |  |  |  |  | O | 0..1 |  |  | It defines the start time of which the statistics analytics information is applicable or predictions analytics information is valid. (NOTE 1) (NOTE 4) |  |  |  |  |  |
| dataVlTrnsTmInfos |  |  | array(E2eDataVolTransTimeInfo) |  |  |  |  | C | 1..N |  |  | E2E data volume transfer time information. Shall be present if the subscribed event is "E2E_DATA_VOL_TRANS_TIME". (NOTE 5) |  |  | E2eDataVolTransTime |  |  |
| expiry |  |  | DateTime |  |  |  |  | O | 0..1 |  |  | It defines the expiration time after which the statistics analytics information is not applicable or predictions analytics information is invalid. (NOTE 1) (NOTE 4) |  |  |  |  |  |
| timeStampGen |  |  | DateTime |  |  |  |  | C | 0..1 |  |  | It defines the timestamp of analytics generation. (NOTE 3) |  |  |  |  |  |
| failNotifyCode |  |  | NwdafFailureCode |  |  | C |  | 0..1 |  |  |  | Identifies the failure reason for the event notification. It shall only be included if the event notification is failed or the analytics information is not ready. (NOTE 2) |  |  | EneNA |  |  |
| rvWaitTime |  |  | DurationSec |  |  | O |  | 0..1 |  |  |  | Indicate a recommended time interval (in seconds) which is used to determine the time when analytics information is needed in similar future event subscriptions. It may only be included if the "failNotifyCode" attribute sets to "UNSATISFIED_REQUESTED_ANALYTICS_TIME". |  |  | EneNA |  |  |
| anaMetaInfo |  |  | AnalyticsMetadataInfo |  |  | C |  | 0..1 |  |  |  | Contains information about analytics metadata required to aggregate the analytics. It shall be present if the "anaMeta" attribute was included in the subscription, and contains the information requested by the "anaMeta" attribute. |  |  | Aggregation |  |  |
| nwPerfs |  |  | array(NetworkPerfInfo) |  |  |  |  | C | 1..N |  |  | The network performance information. Shall be present when subscribed even is "NETWORK_PERFORMANCE". (NOTE 5) |  |  | NetworkPerformance |  |  |
| nfLoadLevelInfos |  |  | array(NfLoadLevelInformation) |  |  |  |  | C | 1..N |  |  | The NF load level information. When subscribed event is "NF_LOAD", the nfLoadLevelInfos shall be included. (NOTE 5) |  |  | NfLoad |  |  |
| nsiLoadLevelInfos |  |  | array(NsiLoadLevelInfo) |  |  |  |  | C | 1..N |  |  | Each element identifies the load level information for each S-NSSAI and the optionally associated network slice instance. Shall be included when subscribed event is "NSI_LOAD_LEVEL". (NOTE 5) |  |  | NsiLoad |  |  |
| pfdDetermInfos |  |  | array(PfdDeterminationInfo) |  |  |  |  | C | 1..N |  |  | Represents the PFD Determination information for a known application identifier. Shall be included when subscribed event is "PFD_DETERMINATION". (NOTE 5) |  |  | PfdDetermination |  |  |
| qosSustainInfos |  |  | array(QosSustainabilityInfo) |  |  |  |  | C | 1..N |  |  | The QoS sustainability information. When subscribed event is "QOS_SUSTAINABILITY", the qosSustainInfos shall be included. (NOTE 5) |  |  | QoSSustainability |  |  |
| sliceLoadLevelInfo |  |  | SliceLoadLevelInformation |  |  |  |  | C | 0..1 |  |  | The slices and the load level information. When subscribed event is "SLICE_LOAD_LEVEL", the sliceLoadLevelInfo shall be included. (NOTE 5) |  |  |  |  |  |
| svcExps |  |  | array(ServiceExperienceInfo) |  |  |  |  | C | 1..N |  |  | The service experience information. When subscribed event is "SERVICE_EXPERIENCE", the svcExps shall be included. (NOTE 5) |  |  | ServiceExperience |  |  |
| ueComms |  |  | array(UeCommunication) |  |  |  |  | C | 1..N |  |  | The UE communication information. When subscribed event is "UE_COMM", the ueComms shall be included. (NOTE 5) |  |  | UeCommunication |  |  |
| ueMobs |  |  | array(UeMobility) |  |  |  |  | C | 1..N |  |  | The UE mobility information. When subscribed event is "UE_MOBILITY", the ueMobs shall be included. (NOTE 5) |  |  | UeMobility |  |  |
| abnorBehavrs |  |  | array(AbnormalBehaviour) |  |  |  |  | C | 1..N |  |  | The Abnormal Behaviour information. When subscribed event is "ABNORMAL_BEHAVIOUR", the abnorBehavrs shall be included. (NOTE 5) |  |  | AbnormalBehaviour |  |  |
| userDataCongInfos |  |  | array(UserDataCongestionInfo) |  |  |  |  | C | 1..N |  |  | The location and user data congestion information. Shall be present if the subscribed event is "USER_DATA_CONGESTION". (NOTE 5) |  |  | UserDataCongestion |  |  |
| dnPerfInfos |  |  | array(DnPerfInfo) |  |  |  |  | C | 1..N |  |  | The DN performance information. Shall be present if the subscribed event is "DN_PERFORMANCE". (NOTE 5) |  |  | DnPerformance |  |  |
| disperInfos |  |  | array(DispersionInfo) |  |  |  |  | C | 1..N |  |  | The Dispersion information. When subscribed event is "DISPERSION", the "disperInfos" attribute shall be included. (NOTE 5) |  |  | Dispersion |  |  |
| redTransInfos |  |  | array(RedundantTransmissionExpInfo) |  |  |  |  | C | 1..N |  |  | The redundant transmission experience related information. When subscribed event is "RED_TRANS_EXP", the "redTransInfos" attribute shall be included. (NOTE 5) |  |  | RedundantTransmissionExp |  |  |
| wlanInfos |  |  | array(WlanPerformanceInfo) |  |  |  |  | C | 1..N |  |  | The WLAN performance related information. When subscribed event is "WLAN_PERFORMANCE", the "wlanInfos" attribute shall be included. (NOTE 5) |  |  | WlanPerformance |  |  |
| smccExps |  |  | array(SmcceInfo) |  |  |  |  | C | 1..N |  |  | The Session Management Congestion Control Experience information. Shall be present when the requested event is "SM_CONGESTION". (NOTE 5) |  |  | SMCCE |  |  |
| pduSesTrafInfos |  |  | array(PduSesTrafficInfo) |  |  |  |  | C | 1..N |  |  | The PDU Session traffic related information. When subscribed event is "PDU_SESSION_TRAFFIC", the "pduSesTrafInfos" attribute shall be included. (NOTE 5) |  |  | PduSesTraffic |  |  |
| accuInfo |  |  | AccuracyInfo |  |  |  |  | C | 0..1 |  |  | The analytics accuracy information. Shall be provided if the analytics accuracy requirement was subscribed in the "accuReq" attribute and the "cancelAccuInd" attribute is set to "false" or omitted. |  |  | AnalyticsAccuracy |  |  |
| cancelAccuInd |  |  | boolean |  |  | O |  | 0..1 |  |  |  | Indicates cancelled subscription of the analytics accuracy information. Set to "true" indicates the NWDAF cancelled subscription of analytics accuracy information as the NWDAF does not support the accuracy checking capability. Otherwise set to "false". Default value is "false" if omitted. |  |  | AnalyticsAccuracy |  |  |
| pauseInd |  |  | boolean |  |  | O |  | 0..1 |  |  |  | Pause analytics consumption indication applicable on analytics ID level. Set to "true" to indicate the consumer to stop the consumption of the analytics because the accuracy level needs to be increased. Default value is "false" if omitted. |  |  | AnalyticsAccuracy |  |  |
| resumeInd |  |  | boolean |  |  | O |  | 0..1 |  |  |  | Resume analytics consumption indication applicable on analytics ID level. Set to "true" to indicate the consumer to resume the consumption of the analytics because the accuracy has been improved. Default value is "false" if omitted. |  |  | AnalyticsAccuracy |  |  |
| movBehavInfos |  |  | array(MovBehavInfo) |  |  | C |  | 1..N |  |  |  | The Movement Behaviour information. When subscribed event is "MOVEMENT_BEHAVIOUR", the "movBehavInfos" attribute shall be included. (NOTE 5) |  |  | MovementBehaviour |  |  |
| relProxInfos |  |  | array(RelProximityInfo) |  |  | C |  | 1..N |  |  |  | The Relative Proximity information. When subscribed event is "RELATIVE_PROXIMITY", the "relProxInfos" attribute shall be included. (NOTE 5) |  |  | RelativeProximity |  |  |
| locAccInfos |  |  | array(LocAccuracyInfo) |  |  | C |  | 1..N |  |  |  | The Location Accuracy related information. It shall be present when the subscribed event is "LOC_ACCURACY". (NOTE 5) |  |  | LocAccuracy |  |  |
| NOTE 1: If the "start" attribute and the "expiry" attribute are both provided, the DateTime of the "expiry" attribute shall not be earlier than the DateTime of the "start" attribute. NOTE 2: The value of "PREDICTION_NOT_ALLOWED" and "BOTH_STAT_PRED_NOT_ALLOWED" of the NwdafFailureCode data type is not applicable for the "failNotifyCode" attribute. The value of "UNAVAILABLE_DATA" of the NwdafFailureCode data type is applicable for the the "failNotifyCode" attribute only when the "StatisticsFailure" feature is supported. NOTE 3: This attribute shall be included when ADRF is deployed. NOTE 4: The validity period specified by "start" and "expiry" attributes is determined by NWDAF internal logic, and is a subset of the analytics target period indicated by "startTs" and "endTs", or "offsetPeriod" attributes contained in "extraReportReq" attribute of the subscription. If the analytics target period refers to the past, the period specified by these two attributes indicate the time period over which the statistics are applicable. If the analytics target period refers to the future, the period specified by these two attributes indicate the time period over which the predictions are valid. NOTE 5: If the AnalyticsAccuracy feature is supported and the notification is only for notifying about the accuracy information of subscribed events, this attribute is not required to be included even if the respective event was subscribed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# *** 2nd Change ***

5.2.6.2.2	Type AnalyticsData

Table 5.2.6.2.2-1: Definition of type AnalyticsData

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| start |  | DateTime |  | O |  | 0..1 |  | It defines the start time of which the statistics analytics information is applicable or predictions analytics information is valid. (NOTE 1) (NOTE 7) |  |  |  |
| expiry |  | DateTime |  | O |  | 0..1 |  | It defines the expiration time after which the statistics analytics information is applicable or predictions analytics information is invalid. (NOTE 1) (NOTE 7) |  |  |  |
| timeStampGen |  | DateTime |  | C |  | 0..1 |  | It defines the timestamp of analytics generation. (NOTE 3) |  |  |  |
| anaMetaInfo |  | AnalyticsMetadataInfo |  | C |  | 0..1 |  | Contains information about analytics metadata required to aggregate the analytics. It shall be present if the "anaMeta" attribute was included in the request, and contains the information requested by the "anaMeta" attribute. |  | Aggregation |  |
| sliceLoadLevelInfos |  | array(SliceLoadLevelInformation) |  | C |  | 1..N |  | The slices and the load level information. Shall be present when the requested event is "LOAD_LEVEL_INFORMATION". |  |  |  |
| nsiLoadLevelInfos |  | array(NsiLoadLevelInfo) |  | C |  | 1..N |  | Each element identifies the load level information for an S-NSSAI and the optionally associated network slice instance. Shall be presented when the requested event is "NSI_LOAD_LEVEL" |  | NsiLoad |  |
| nwPerfs |  | array(NetworkPerfInfo) |  | C |  | 1..N |  | The network performance information. Shall be present when the requested event is "NETWORK_PERFORMANCE". |  | NetworkPerformance |  |
| nfLoadLevelInfos |  | array(NfLoadLevelInformation) |  | C |  | 1..N |  | The NF load information. When the requestedevent is "NF_LOAD", the nfLoadLevelInfos shall be included. |  | NfLoad |  |
| qosSustainInfos |  | array(QosSustainabilityInfo) |  | C |  | 1..N |  | The QoS sustainability informations in the certain geographic areas. It shall be present if the requested eventis "QOS_SUSTAINABILITY". (NOTE 2) |  | QoSSustainability |  |
| ueMobs |  | array(UeMobility) |  | C |  | 1..N |  | The UE mobility information. When the requested event is "UE_MOBILITY", the "ueMobs" attribute shall be included. (NOTE 5) (NOTE 8) |  | UeMobility |  |
| ueComms |  | array(UeCommunication) |  | C |  | 1..N |  | The UE communication information. When the requested event is "UE_COMM", the "ueComms" attribute shall be included. (NOTE 9) |  | UeCommunication |  |
| userDataCongInfos |  | array(UserDataCongestionInfo) |  | C |  | 1..N |  | The user data congestion information. Shall be present when the requested event is "USER_DATA_CONGESTION". |  | UserDataCongestion |  |
| suppFeat |  | SupportedFeatures |  | C |  | 0..1 |  | List of Supported features used as described in clause 5.2.8. This parameter shall be supplied by NWDAF in the reply of GET request that request the analytics resource, if the consumer includes "supported-features" in the GET request. |  |  |  |
| svcExps |  | array(ServiceExperienceInfo) |  | C |  | 1..N |  | The service experience information. Shall be present when the requested event is "SERVICE_EXPERIENCE". |  | ServiceExperience |  |
| abnorBehavrs |  | array(AbnormalBehaviour) |  | C |  | 1..N |  | The abnormal behaviour information. Shall be present when the requested event is "ABNORMAL_BEHAVIOUR". |  | AbnormalBehaviour |  |
| smccExps |  | array(SmcceInfo) |  | C |  | 1..N |  | The Session Management congestion control experience information. Shall be present when the requested event is "SM_CONGESTION". |  | SMCCE |  |
| disperInfos |  | array(DispersionInfo) |  | C |  | 1..N |  | The Dispersion information. Shall be present when the requested event is "DISPERSION". |  | Dispersion |  |
| redTransInfos |  | array(RedundantTransmissionExpInfo) |  | C |  | 1..N |  | The Redundant Transmission Experience analytics information. Shall be present when the requested event is "RED_TRANS_EXP". |  | RedundantTransmissionExp |  |
| wlanInfos |  | array(WlanPerformanceInfo) |  | C |  | 1..N |  | The WLAN performance related information. When requested event is "WLAN_PERFORMANCE", the "wlanInfos" attribute shall be included. (NOTE 6) |  | WlanPerformance |  |
| dnPerfInfos |  | array(DnPerfInfo) |  | C |  | 1..N |  | The DN performance information. Shall be present when the requested event is "DN_PERFORMANCE". (NOTE 4) |  | DnPerformance |  |
| pduSesTrafInfos |  | array(PduSesTrafficInfo) |  | C |  | 1..N |  | The PDU Session traffic related information. Shall be present when the requested event is "PDU_SESSION_TRAFFIC". |  | PduSesTraffic |  |
| dataVlTrnsTmInfos |  | array(E2eDataVolTransTimeInfo) |  | C |  | 1..N |  | E2E data volume transfer time information. Shall be present when the requested event is "E2E_DATA_VOL_TRANS_TIME". |  | E2eDataVolTransTime |  |
| locAccInfos |  | array(LocAccuracyInfo) |  | C |  | 1..N |  | The Location Accuracy related information. It shall be present when the requested event is "LOC_ACCURACY". |  | LocAccuracy |  |
| accuInfo |  | AccuracyInfo |  | C |  | 0..1 |  | The analytics accuracy information. Shall be provided if the analytics accuracy requirement was requested in the "accuReq" attribute and the "cancelAccuInd" attribute is set to "false" or omitted. (NOTE 10) |  | AnalyticsAccuracy |  |
| cancelAccuInd |  | boolean |  | O |  | 0..1 |  | Indicates cancelled request of the analytics accuracy information. Set to "true" indicates the NWDAF cancelled request of analytics accuracy information as the NWDAF does not support the accuracy checking capability. Otherwise set to "false". Default value is "false" if omitted. |  | AnalyticsAccuracy |  |
| movBehavInfos |  | array(MovBehavInfo) |  | C |  | 1..N |  | The Movement Behaviour information. Shall be present when the requested event is "MOVEMENT_BEHAVIOUR". |  | MovementBehaviour |  |
| relProxInfos |  | array(RelProxInfo) |  | C |  | 1..N |  | The Relative Proximity information. Shall be present when the requested event is "RELATIVE_PROXIMITY", |  | RelativeProximity |  |
| NOTE 1: If the "start" attribute and the "expiry" attribute are both provided, the DateTime of the "expiry" attribute shall not be earlier than the DateTime of the "start" attribute. NOTE 2: The "qosFlowRetThd" and "ranUeThrouThd" attributes in QosSustainabilityInfo data type are not applicable. NOTE 3: This attribute shall be included when ADRF is deployed. NOTE 4: The "minTrafficRate", "aggTrafficRate", "varTrafficRate", "trafRateUeIds", "avePacketDelay", "maxPacketDelay", "varPacketDelay", "packDelayUeIds", "maxPacketLossRate", "varPacketLossRate" and "packetLossUeIds" attribute(s) within the DnPerfInfo data type is applicable only if the "DnPerformanceExt_AIML" feature is supported. NOTE 5: The "directionInfos" attribute and the "geoDistrInfos" attribute in the "locInfos" attribute within the UeMobility data type are applicable only if the "UeMobilityExt_AIML" feature is supported. NOTE 6: The "wlanPerUeIdInfos" attribute may be included within the "wlanInfos" attribute only if the "WlanPerformanceExt_AIML" feature is supported. NOTE 7: The validity period specified by "start" and "expiry" attributes is determined by NWDAF internal logic, and is a subset of the analytics target period indicated by "startTs" and "endTs", or "offsetPeriod" attributes contained in "ana-req" attribute. If the analytics target period refers to the past, the period specified by these two attributes indicate the time period over which the statistics are applicable. If the analytics target period refers to the future, the period specified by these two attributes indicate the time period over which the predictions are valid. NOTE 8: If the "UeMobilityExt2_eNA" feature is supported and the "locationGranReq" attribute value "LON_AND_LAT_LEVEL" is requested, the "geoLoc" attribute within the "locInfos" attribute in the "UeMobility" type shall be provided to report the geographical location (longitude and latitude level). NOTE 9: The "perioCommInd", "anaOfAppList" and "sessInactTimer" attributes within the UeCommunication data type are applicable only if the "UeCommunicationExt" feature is supported. NOTE 10: Only the "accuracyVal" and "accuSampleNbr" attributes within the AccuracyInfo data type are applicable. |  |  |  |  |  |  |  |  |  |  |  |

*** End of Changes ***