3GPP TSG-CT WG4 Meeting #101e	C4-205248

# E-Meeting, 03rd – 13th November 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.171 | CR | 0abc | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | High Accuracy Uncertainty |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | ************************************************************************ NOTE: This document is submitted FOR INFORMATION to CT4#101e for comparison with the non-backward compatible corrections submitted in C4-205246 / C4-205247 ************************************************************************ The current definition of the IE type "High Accuracy Uncertainty Ellipse" (clause 7.4.42) has an incorrect encoding of the "Uncertainty Code", because it re-uses the old range (non-high accuracy) of 0..127, when it should be 0..255, according to TS 23.032. Similarly, the IE "High Accuracy Uncertainty Altitude" has been coded with the old range 0..127 defined in type "Uncertainty Altitude"; it should use the same high accuracy uncertainty code range of 0..255. These types are used in 2 shape definitions: - High Accuracy Ellipsoid point with uncertainty Ellipse - High Accuracy Ellipsoid point with altitude and uncertainty Ellipsoid To fix this problem in a backwards-compatible way, and given that the type "High Accuracy Uncertainty Ellipse" has not been made extensible in the current ASN.1 definition, a new type needs to be defined: "Fixed High Accuracy Uncertainty Ellipse". This new type is used in the protocol extension IEs in the affected shapes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the 2 shapes that use a wrong range of the "High Accuracy Uncertainty Code". - Add a missing import of "id-Additional-PositioningDataSet" |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The shapes cannot be used to provide high-accuracy location estimates. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.12, 7.4.xx (new), 7.5.5. 7.5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 7.4.12	Geographic Area

This parameter provides a location estimate for the target UE in the case of a successful location attempt.

Table 7.4.12-1: Geographic Area

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Geographical Area |  |  |  |  |
| >Point |  |  |  | Ellipsoid point |
| >>Geographical Coordinates | M |  | 7.4.23 |  |
| >Point With Uncertainty |  |  |  | Ellipsoid point with uncertainty circle |
| >>Geographical Coordinates | M |  | 7.4.23 |  |
| >>Uncertainty Code | M |  | INTEGER ( 0..127) | The uncertainty "r" expressed in meters is derived from the "Uncertainty Code" k by r = 10x(1.1k-1) |
| >Ellipsoid point with uncertainty Ellipse |  |  |  |  |
| >>Geographical Coordinates | M |  | 7.4.23 |  |
| >>Uncertainty Ellipse | M |  | 7.4.24 |  |
| >>Confidence | M |  | INTEGER ( 0..100) | In percentage |
| >Polygon |  |  |  | List of Ellipsoid points |
| >>Polygon Point |  | 1..<maxnoofPoints> |  | The minimum number of points allowed is 3. |
| >>>Geographical Coordinates | M |  | 7.4.23 |  |
| >Ellipsoid point with altitude |  |  |  |  |
| >>Geographical Coordinates | M |  | 7.4.23 |  |
| >>Altitude and direction | M |  | 7.4.22 |  |
| >Ellipsoid point with altitude and uncertainty Ellipsoid |  |  |  |  |
| >>Geographical Coordinates | M |  | 7.4.23 |  |
| >>Altitude and direction | M |  | 7.4.22 |  |
| >>Uncertainty Ellipse | M |  | 7.4.24 |  |
| >>Uncertainty Altitude | M |  | INTEGER ( 0..127) | The uncertainty altitude "h" expressed in metres is derived from the "Uncertainty Altitude" k, by: h=45x(1.025k-1) |
| >>Confidence | M |  | INTEGER ( 0..100) | In percentage |
| >Ellipsoid Arc |  |  |  |  |
| >>Geographical Coordinates | M |  | 7.4.23 |  |
| >>Inner radius | M |  | INTEGER ( 0..216-1) | The relation between the value (N) and the radius (r) in meters it describes is 5N r <5(N+1), except for N=216-1 for which the range is extended to include all grater values of (r). |
| >>Uncertainty radius | M |  | INTEGER ( 0..127) | The uncertainty "r" is derived from the "Uncertainty radius" k by r = 10x(1.1k-1) |
| >>Offset angle | M |  | INTEGER ( 0..179) | The relation between the value (N) and the angle (a) in degrees it describes is 2N a <2(N+1) |
| >>Included angle | M |  | INTEGER ( 0..179) | The relation between the value (N) and the angle (a) in degrees it describes is 2N< a 2(N+1) |
| >>Confidence | M |  | INTEGER ( 0..100) |  |
| >High Accuracy Ellipsoid point with uncertainty Ellipse |  |  |  |  |
| >>High Accuracy Geographical Coordinates | M |  | 7.4.41 |  |
| >>High Accuracy Uncertainty Ellipse | M |  | 7.4.42 |  |
| >>Confidence | M |  | INTEGER ( 0..100) | In percentage |
| >>Fixed High Accuracy Uncertainty Ellipse | O |  | 7.4.xx | If this IE is present, the IE "High Accuray Uncertainty Ellipse" shall be ignored |
| >High Accuracy Ellipsoid point with altitude and uncertainty Ellipsoid |  |  |  |  |
| >>High Accuracy Geographical Coordinates | M |  | 7.4.41 |  |
| >>High Accuracy Altitude | M |  | 7.4.43 |  |
| >>High Accuracy Uncertainty Ellipse | M |  | 7.4.42 |  |
| >>Confidence | M |  | INTEGER ( 0..100) | In percentage |
| >>High Accuracy Uncertainty Altitude | M |  | INTEGER ( 0..127) | The uncertainty altitude "h" expressed in metres is derived from the "Uncertainty Altitude" k, by: h=0.3x(1.02k-1) |
| >>Vertical Confidence | M |  | INTEGER ( 0..100) | In percentage |
| >> Fixed High Accuracy Uncertainty Ellipse | O |  | 7.4.xx | If this IE is present, the IE "High Accuray Uncertainty Ellipse" shall be ignored |
| >> Fixed High Accuracy Uncertainty Altitude | O |  | INTEGER ( 0..255) | The high accuracy uncertainty altitude "h" expressed in metres is derived from the high accuracy uncertainty code "k", by: h=0.3x(1.02k-1) If this IE is present, the IE "High Accuray Uncertainty Altitude" shall be ignored |

Table 7.4.12-2: Range bound

| Range bound | Explanation |
| --- | --- |
| maxnoofPoints | Maximum no. of points in polygon. Value is 15. |

* * * Next Change * * * *

### 7.4.xx	Fixed High Accuracy Uncertainty Ellipse

This IE contains a corrected version of the high accuracy uncertainty ellipse of a geographical area, using the range (0..255) for high-accuracy uncertainty code "k", as specified in 3GPP TS 23.032 [6].

Table 7.4.xx-1: Fixed High Accuracy Uncertainty Ellipse

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| High Accuracy Uncertainty semi-major | M |  | INTEGER (0..255) | The high accuracy uncertainty "r" is derived from the high accuracy uncertainty code "k" by: r = 0.3x(1.02k-1) |
| High Accuracy Uncertainty semi-minor | M |  | INTEGER (0..255) | The high accuracy uncertainty "r" is derived from the high accuracy uncertainty code "k" by: r = 0.3x(1.02k-1) |
| Orientation of major axis | M |  | INTEGER (0..179) | Angle in degrees between the major axis and north |

* * * Next Change * * * *

### 7.5.5	Information element definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

LCS-AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) lcs-AP (4) version1 (1) lcs-AP-IEs (2)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

max-No-Of-Points,

max-Set,

max-GNSS-Set,

max-Add-Pos-Set,

max-Cipher-Set,

id-Additional-PositioningDataSet,

id-Fixed-High-Accuracy-Uncertainty-Ellipse,

id-Fixed-High-Accuracy-Uncertainty-Altitude

FROM LCS-AP-Constants

Criticality,

ProcedureCode,

TriggeringMessage,

ProtocolIE-ID

FROM LCS-AP-CommonDataTypes

ProtocolExtensionContainer{},

LCS-AP-PROTOCOL-EXTENSION

FROM LCS-AP-Containers;

-- A

APDU		::=	OCTET STRING

Accuracy-Fulfillment-Indicator ::= ENUMERATED {

requested-accuracy-fulfilled,

requested-accuracy-not-fulfilled,

...

}

Additional-PositioningDataSet ::= SEQUENCE(SIZE(1..max-Add-Pos-Set)) OF Additional-PositioningMethodAndUsage

Additional-PositioningMethodAndUsage ::= OCTET STRING (SIZE(1))

Altitude ::= INTEGER (0..65535)

-- The valid value for this IE is 0 to 32767.

-- Reception of value greater than 32767 shall be mapped to the value 32767.

Altitude-And-Direction ::= SEQUENCE {

direction-Of-Altitude	Direction-Of-Altitude,

altitude		Altitude,

...

}

Angle ::= INTEGER (0..179)

-- B

Barometric-Pressure ::= INTEGER (30000..115000)

Bearing ::= INTEGER (0..359)

-- C

C0                  ::= BIT STRING (SIZE (1..128))

CellIdentity	::= BIT STRING (SIZE (28))

Cell-Portion-ID	::= INTEGER (0..255,..., 256..4095)

Ciphering-Data		::= SEQUENCE(SIZE(1..max-Cipher-Set)) OF Ciphering-Data-Set

Ciphering-Data-Ack	::= SEQUENCE(SIZE(1..max-Cipher-Set)) OF Ciphering-Set-ID

Ciphering-Data-Error-Report	::= SEQUENCE(SIZE(1..max-Cipher-Set)) OF Ciphering-Data-Error-Report-Contents

Ciphering-Data-Set	::= SEQUENCE {

ciphering-Set-ID	Ciphering-Set-ID,

ciphering-Key		Ciphering-Key,

c0			C0,

sib-Types		SIB-Types,

validity-Start-Time	Validity-Start-Time,

validity-Duration	Validity-Duration,

tais-List		TAIs-List,

...

}

Ciphering-Data-Error-Report-Contents	::= SEQUENCE {

ciphering-Set-ID	Ciphering-Set-ID,

storage-Outcome		Storage-Outcome,

...

}

Ciphering-Set-ID ::= INTEGER (0..65535)

Ciphering-Key ::= OCTET STRING (SIZE (16))

Civic-Address	::=	OCTET STRING

Confidence		::= INTEGER (0..100)

Correlation-ID	::=	OCTET STRING (SIZE (4))

-- D

DegreesLatitude	::= INTEGER (0..8388607)	-- 23 bit field

DegreesLongitude	::= INTEGER (-8388608..8388607)	-- 24 bit field

Direction-Of-Altitude ::= ENUMERATED {

height,

depth

}

-- E

E-CGI			::= SEQUENCE {

pLMNidentity		PLMN-ID,

cell-ID			CellIdentity,

iE-Extensions		ProtocolExtensionContainer { { E-CGI-ExtIEs} } OPTIONAL,

...}

E-CGI-ExtIEs LCS-AP-PROTOCOL-EXTENSION ::= {

...

}

Coverage-Level		::= ENUMERATED {

extendedcoverage,

...

}

Ellipsoid-Point-With-Uncertainty-Ellipse  ::= SEQUENCE {

geographical-Coordinates	Geographical-Coordinates,

uncertainty-Ellipse		Uncertainty-Ellipse,

confidence			Confidence,

iE-Extensions		ProtocolExtensionContainer { { Ellipsoid-Point-With-Uncertainty-Ellipse-ExtIEs} } OPTIONAL,

...}

Ellipsoid-Point-With-Uncertainty-Ellipse-ExtIEs LCS-AP-PROTOCOL-EXTENSION ::= {

...

}

Ellipsoid-Point-With-Altitude	::= SEQUENCE {

geographical-Coordinates	Geographical-Coordinates,

altitude-And-Direction	Altitude-And-Direction,

iE-Extensions		ProtocolExtensionContainer { { Ellipsoid-Point-With-Altitude-ExtIEs} } OPTIONAL,

...}

Ellipsoid-Point-With-Altitude-ExtIEs LCS-AP-PROTOCOL-EXTENSION ::= {

...

}

Ellipsoid-Point-With-Altitude-And-Uncertainty-Ellipsoid	::= SEQUENCE {

geographical-Coordinates	Geographical-Coordinates,

altitude-And-Direction	Altitude-And-Direction,

uncertainty-Ellipse		Uncertainty-Ellipse,

uncertainty-Altitude	Uncertainty-Altitude,

confidence			Confidence,

iE-Extensions		ProtocolExtensionContainer { { Ellipsoid-Point-With-Altitude-And-Uncertainty-Ellipsoid-ExtIEs} } OPTIONAL,

...}

Ellipsoid-Point-With-Altitude-And-Uncertainty-Ellipsoid-ExtIEs LCS-AP-PROTOCOL-EXTENSION ::= {

...

}

Ellipsoid-Arc			::= SEQUENCE {

geographical-Coordinates	Geographical-Coordinates,

inner-Radius		Inner-Radius,

uncertainty-Radius		Uncertainty-Code,

offset-Angle		Angle,

included-Angle		Angle,

confidence			Confidence,

iE-Extensions		ProtocolExtensionContainer { { Ellipsoid-Arc-ExtIEs} } OPTIONAL,

...

}

Ellipsoid-Arc-ExtIEs LCS-AP-PROTOCOL-EXTENSION ::= {

...

}

ENB-ID		::= CHOICE {

macro-eNB-ID	Macro-eNB-ID,

home-eNB-ID	Home-eNB-ID,

...,

short-macro-eNB-ID	Short-Macro-eNB-ID,

long-macro-eNB-ID	Long-Macro-eNB-ID

}

E-SMLC-ID ::= INTEGER (0..255)

-- F

Fixed-High-Accuracy-Uncertainty-Ellipse  ::= SEQUENCE {

high-Accuracy-Uncertainty-SemiMajor		High-Accuracy-Uncertainty-Code,

high-Accuracy-Uncertainty-SemiMinor		High-Accuracy-Uncertainty-Code,

orientation-Major-Axis	INTEGER (0..179),

iE-Extensions  			ProtocolExtensionContainer	{ {Fixed-High-Accuracy-Uncertainty-Ellipse-ExtIEs} } OPTIONAL,

...

}

Fixed-High-Accuracy-Uncertainty-Ellipse-ExtIEs LCS-AP-PROTOCOL-EXTENSION ::= {

...

}

-- G

Geographical-Area		::= CHOICE {

point							Point,

point-With-Uncertainty					Point-With-Uncertainty,

ellipsoidPoint-With-Uncertainty-Ellipse			Ellipsoid-Point-With-Uncertainty-Ellipse,

polygon							Polygon,

ellipsoid-Point-With-Altitude				Ellipsoid-Point-With-Altitude,

ellipsoid-Point-With-Altitude-And-Uncertainty-Ellipsoid	Ellipsoid-Point-With-Altitude-And-Uncertainty-Ellipsoid,

ellipsoid-Arc						Ellipsoid-Arc,

...,

high-Accuracy-Ellipsoid-Point-With-Uncertainty-Ellipse	High-Accuracy-Ellipsoid-Point-With-Uncertainty-Ellipse,

high-Accuracy-Ellipsoid-Point-With-Altitude-And-Uncertainty-Ellipsoid	High-Accuracy-Ellipsoid-Point-With-Altitude-And-Uncertainty-Ellipsoid

}

Geographical-Coordinates	::= SEQUENCE {

latitudeSign		LatitudeSign,

degreesLatitude		DegreesLatitude,

degreesLongitude	DegreesLongitude,

iE-Extensions		ProtocolExtensionContainer { {Geographical-Coordinates-ExtIEs} } OPTIONAL,

...

}

Geographical-Coordinates-ExtIEs LCS-AP-PROTOCOL-EXTENSION ::= {

...

}

Global-eNB-ID	::= SEQUENCE {

pLMN-ID	PLMN-ID,

eNB-ID	ENB-ID

}

GNSS-Positioning-Method-And-Usage ::= OCTET STRING (SIZE (1))

GNSS-Positioning-Data-Set ::= SEQUENCE (SIZE (1..max-GNSS-Set)) OF GNSS-Positioning-Method-And-Usage

-- H

High-Accuracy-Altitude ::= INTEGER (-64000..128000)

High-Accuracy-DegreesLatitude	::= INTEGER(-2147483648..2147483647)

High-Accuracy-DegreesLongitude	::= INTEGER(-2147483648..2147483647)

High-Accuracy-Ellipsoid-Point-With-Uncertainty-Ellipse  ::= SEQUENCE {

high-Accuracy-Geographical-Coordinates	High-Accuracy-Geographical-Coordinates,

high-Accuracy-Uncertainty-Ellipse		High-Accuracy-Uncertainty-Ellipse,

confidence			Confidence,

iE-Extensions		ProtocolExtensionContainer { { High-Accuracy-Ellipsoid-Point-With-Uncertainty-Ellipse-ExtIEs} } OPTIONAL,

...}

High-Accuracy-Ellipsoid-Point-With-Uncertainty-Ellipse-ExtIEs LCS-AP-PROTOCOL-EXTENSION ::= {

{ ID id-Fixed-High-Accuracy-Uncertainty-Ellipse	CRITICALITY reject	EXTENSION Fixed-High-Accuracy-Uncertainty-Ellipse	PRESENCE optional },

...

}

High-Accuracy-Ellipsoid-Point-With-Altitude-And-Uncertainty-Ellipsoid	::= SEQUENCE {

high-Accuracy-Geographical-Coordinates	High-Accuracy-Geographical-Coordinates,

high-Accuracy-Altitude	High-Accuracy-Altitude,

high-Accuracy-Uncertainty-Ellipse	High-Accuracy-Uncertainty-Ellipse,

confidence			Confidence,

high-Accuracy-Uncertainty-Altitude	Uncertainty-Altitude,

vertical-Confidence	Confidence,

iE-Extensions		ProtocolExtensionContainer { { High-Accuracy-Ellipsoid-Point-With-Altitude-And-Uncertainty-Ellipsoid-ExtIEs} } OPTIONAL,

...}

High-Accuracy-Ellipsoid-Point-With-Altitude-And-Uncertainty-Ellipsoid-ExtIEs LCS-AP-PROTOCOL-EXTENSION ::= {

{ ID id-Fixed-High-Accuracy-Uncertainty-Ellipse	CRITICALITY reject	EXTENSION Fixed-High-Accuracy-Uncertainty-Ellipse	PRESENCE optional } |

{ ID id-Fixed-High-Accuracy-Uncertainty-Altitude	CRITICALITY reject 	EXTENSION High-Accuracy-Uncertainty-Code	PRESENCE optional },

...

}

High-Accuracy-Geographical-Coordinates	::= SEQUENCE {

high-Accuracy-DegreesLatitude	High-Accuracy-DegreesLatitude,

high-Accuracy-DegreesLongitude	High-Accuracy-DegreesLongitude,

iE-Extensions		ProtocolExtensionContainer { { High-Accuracy-Geographical-Coordinates-ExtIEs} } OPTIONAL,

...

}

High-Accuracy-Geographical-Coordinates-ExtIEs LCS-AP-PROTOCOL-EXTENSION ::= {

...

}

High-Accuracy-Uncertainty-Code ::= INTEGER (0..255)

High-Accuracy-Uncertainty-Ellipse	::= SEQUENCE {

high-Accuracy-Uncertainty-SemiMajor	Uncertainty-Code,

high-Accuracy-Uncertainty-SemiMinor	Uncertainty-Code,

orientation-Major-Axis	INTEGER (0..179)

}

Home-eNB-ID ::= BIT STRING (SIZE (28))

Horizontal-Accuracy ::= INTEGER (0..127)

Horizontal-Speed-And-Bearing	::= SEQUENCE {

bearing			INTEGER (0..359),

horizontal-Speed		INTEGER (0..2047)

}

Horizontal-Velocity	::= SEQUENCE {

horizontal-Speed-And-Bearing	Horizontal-Speed-And-Bearing,

iE-Extensions	ProtocolExtensionContainer { { Horizontal-Velocity-ExtIEs} } OPTIONAL,

...

}

Horizontal-Velocity-ExtIEs LCS-AP-PROTOCOL-EXTENSION ::= {

...

}

Horizontal-With-Vertical-Velocity	::= SEQUENCE {

horizontal-Speed-And-Bearing	Horizontal-Speed-And-Bearing,

vertical-Velocity		Vertical-Velocity,

iE-Extensions			ProtocolExtensionContainer { { Horizontal-With-Vertical-Velocity-ExtIEs} } OPTIONAL,

...

}

Horizontal-With-Vertical-Velocity-ExtIEs LCS-AP-PROTOCOL-EXTENSION ::= {

...

}

Horizontal-Velocity-With-Uncertainty	::= SEQUENCE {

horizontal-Speed-And-Bearing	Horizontal-Speed-And-Bearing,

uncertainty-Speed			INTEGER (0..255),

iE-Extensions			ProtocolExtensionContainer { { Horizontal-Velocity-With-Uncertainty-ExtIEs} } OPTIONAL,

...

}

Horizontal-Velocity-With-Uncertainty-ExtIEs LCS-AP-PROTOCOL-EXTENSION ::= {

...

}

Horizontal-With-Vertical-Velocity-And-Uncertainty ::= SEQUENCE {

horizontal-Speed-And-Bearing	Horizontal-Speed-And-Bearing,

vertical-Velocity		Vertical-Velocity,

horizontal-Uncertainty-Speed	INTEGER (0..255),

vertical-Uncertainty-Speed	INTEGER (0..255),

iE-Extensions			ProtocolExtensionContainer { { Horizontal-With-Vertical-Velocity-And-Uncertainty-ExtIEs} } OPTIONAL,

...

}

Horizontal-With-Vertical-Velocity-And-Uncertainty-ExtIEs LCS-AP-PROTOCOL-EXTENSION ::= {

...

}

-- I

IMSI		::=	OCTET STRING (SIZE (3..8))

IMEI		::=	OCTET STRING (SIZE (8))

Include-Velocity	::= ENUMERATED {

requested,

not-Requested,

...

}

Inner-Radius	::= INTEGER (0..65535)	-- 16 bit field

-- J

-- K

-- L

LatitudeSign	::= ENUMERATED {

north,

south

}

Location-Type	::= ENUMERATED {

geographic-Information,

assistance-Information,

...,

last-known-location

}

LCS-Cause		::= CHOICE {

radio-Network-Layer	Radio-Network-Layer-Cause,

transport-Layer	Transport-Layer-Cause,

protocol		Protocol-Cause,

misc		Misc-Cause

}

LCS-Client-Type	::= ENUMERATED {

emergency-Services,

value-Added-Services,

pLMN-Operator-Services,

lawful-Intercept-Services,

pLMN-Operator-broadcast-Services,

pLMN-Operator-OM,

pLMN-Operator-Anonymous-Statistics,

pLMN-Operator-Target-MS-Service-Support,

...

}

LCS-Priority	::=	OCTET STRING (SIZE (1))

LCS-QoS			::= SEQUENCE {

horizontal-Accuracy		Horizontal-Accuracy	OPTIONAL,

vertical-Requested		Vertical-Requested	OPTIONAL,

vertical-Accuracy		Vertical-Accuracy	OPTIONAL,

response-Time		Response-Time		OPTIONAL,

...

}

LCS-Service-Type-ID	::= INTEGER (0..127)

Long-Macro-eNB-ID ::= BIT STRING (SIZE (21))

-- M

Macro-eNB-ID ::= BIT STRING (SIZE (20))

Message-Identifier ::= BIT STRING (SIZE (16))

Misc-Cause	::= ENUMERATED {

processing-Overload,

hardware-Failure,

o-And-M-Intervention,

unspecified,

...,

ciphering-key-data-lost

}

MultipleAPDUs ::= SEQUENCE (SIZE (1.. 3)) OF APDU

-- N

Network-Element		::= CHOICE {

global-eNB-ID		Global-eNB-ID,

e-SMLC-ID		E-SMLC-ID

}

-- O

Orientation-Major-Axis ::= INTEGER (0..89)

-- P

Payload-Type ::= ENUMERATED {

lPP,

lPPa,

...

}

PLMN-ID ::= TBCD-STRING

Point				::= SEQUENCE {

geographical-Coordinates	Geographical-Coordinates,

iE-Extensions		ProtocolExtensionContainer { {Point-ExtIEs} } OPTIONAL,

...

}

Point-ExtIEs LCS-AP-PROTOCOL-EXTENSION ::= {

...

}

Point-With-Uncertainty		::= SEQUENCE {

geographical-Coordinates	Geographical-Coordinates,

uncertainty-Code		Uncertainty-Code,

iE-Extensions		ProtocolExtensionContainer { {Point-With-Uncertainty-ExtIEs} } OPTIONAL,

...}

Point-With-Uncertainty-ExtIEs LCS-AP-PROTOCOL-EXTENSION ::= {

...

}

Polygon ::= SEQUENCE (SIZE (1..max-No-Of-Points)) OF Polygon-Point

Polygon-Point ::= SEQUENCE {

geographical-Coordinates	Geographical-Coordinates,

iE-Extensions		ProtocolExtensionContainer { {Polygon-Point-ExtIEs} } OPTIONAL,

...

}

Polygon-Point-ExtIEs LCS-AP-PROTOCOL-EXTENSION ::= {

...

}

Positioning-Data		::= SEQUENCE {

positioning-Data-Set	Positioning-Data-Set  OPTIONAL,

gNSS-Positioning-Data-Set	GNSS-Positioning-Data-Set OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { Positioning-Data-ExtIEs} } OPTIONAL,

...

}

Positioning-Data-ExtIEs LCS-AP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PositioningDataSet		CRITICALITY ignore	EXTENSION Additional-PositioningDataSet		PRESENCE optional },

...

}

Positioning-Data-Set  ::= SEQUENCE (SIZE (1..max-Set)) OF Positioning-Method-And-Usage

Positioning-Method-And-Usage	::=	OCTET STRING (SIZE (1))

Protocol-Cause	::= ENUMERATED {

transfer-Syntax-Error,
abstract-Syntax-Error-Reject,
abstract-Syntax-Error-Ignore-And-Notify,
message-Not-Compatible-With-Receiver-State,

semantic-Error,
unspecified,

abstract-Syntax-Error,

...

}

-- Q

-- R

Radio-Network-Layer-Cause	::= ENUMERATED {

unspecified,

...

}

RAT-Type	::= ENUMERATED {

lte-wb,

nb-iot,

...,

lte-m

}

Response-Time		::= ENUMERATED {

low-Delay(0),

delay-Tolerant(1),

...

}

Return-Error-Type ::= ENUMERATED {

yes,

no

}

Return-Error-Cause ::= ENUMERATED {

system-Failure,

protocol-Error,

destination-Unknown,

destination-Unreachable,

congestion,

...

}

-- S

Short-Macro-eNB-ID ::= BIT STRING (SIZE (18))

SIB-Types ::= OCTET STRING (SIZE (4))

Storage-Outcome ::= ENUMERATED {

successful,

failed

}

-- T

TAIs-List ::= OCTET STRING (SIZE (7..97))

TBCD-STRING ::= OCTET STRING (SIZE (3))

Transport-Layer-Cause	::= ENUMERATED {

tranport-Resource-Unavailable,

unspecified,

...

}

-- U

Uncertainty-Altitude ::= INTEGER (0..127)

Uncertainty-Code  ::= INTEGER (0..127)

Uncertainty-Ellipse	::= SEQUENCE {

uncertainty-SemiMajor	Uncertainty-Code,

uncertainty-SemiMinor	Uncertainty-Code,

orientation-Major-Axis	Orientation-Major-Axis

}

UE-Positioning-Capability		::= SEQUENCE {

lPP				BOOLEAN,

...

}

-- V

Validity-Duration ::= INTEGER (0..65535)

Validity-Start-Time ::= OCTET STRING (SIZE (4))

Velocity-Estimate ::= CHOICE {

horizontal-Velocity					Horizontal-Velocity,

horizontal-With-Vertical-Velocity			Horizontal-With-Vertical-Velocity,

horizontal-Velocity-With-Uncertainty		Horizontal-Velocity-With-Uncertainty,

horizontal-With-Vertical-Velocity-And-Uncertainty	Horizontal-With-Vertical-Velocity-And-Uncertainty,

...

}

Vertical-Requested	::= ENUMERATED {

vertical-coordinate-Is-Not-Requested(0),

vertical-coordinate-Is-Requested(1)

}

Vertical-Accuracy ::= INTEGER (0..127)

Vertical-Velocity ::= SEQUENCE {

vertical-Speed					INTEGER (0..255),

vertical-Speed-Direction				Vertical-Speed-Direction

}

Vertical-Speed-Direction ::= ENUMERATED {

upward,

downward

}

-- W

-- X

-- Y

END

* * * Next Change * * * *

### 7.5.7	Constant definitions

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

LCS-AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) lcs-AP (4) version1 (1) lcs-AP-Constants (4)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- Elementary Procedures

--

-- **************************************************************

id-Location-Service-Request			INTEGER ::= 0

id-Connection-Oriented-Information-Transfer	INTEGER ::= 1

id-Connectionless-Information-Transfer		INTEGER ::= 2

id-Location-Abort				INTEGER ::= 3

id-Reset					INTEGER ::= 4

id-Ciphering-Key-Data-Delivery				INTEGER ::= 5

-- **************************************************************

--

-- IEs

--

-- **************************************************************

id-Accuracy-Fulfillment-Indicator	INTEGER ::= 0

id-APDU				INTEGER ::= 1

id-Correlation-ID			INTEGER ::= 2

id-Destination-ID			INTEGER ::= 3

id-E-UTRAN-Cell-Identifier		INTEGER ::= 4

id-Include-Velocity			INTEGER ::= 5

id-IMEI				INTEGER ::= 6

id-IMSI				INTEGER ::= 7

id-LCS-Client-Type			INTEGER ::= 8

id-LCS-Priority			INTEGER ::= 9

id-LCS-QOS				INTEGER ::= 10

id-LCS-Cause			INTEGER ::= 11

id-Location-Estimate		INTEGER ::= 12

id-Location-Type			INTEGER ::= 13

id-MultipleAPDUs			INTEGER ::= 14

id-Payload-Type			INTEGER ::= 15

id-Positioning-Data			INTEGER ::= 16

id-Return-Error-Request		INTEGER ::= 17

id-Return-Error-Cause		INTEGER ::= 18

id-Source-Identity			INTEGER ::= 19

id-UE-Positioning-Capability	INTEGER ::= 20

id-Velocity-Estimate		INTEGER ::= 21

id-LCS-Service-Type-ID		INTEGER ::= 22

id-Cell-Portion-ID			INTEGER ::= 23

id-Civic-Address			INTEGER ::= 24

id-Barometric-Pressure		INTEGER ::= 25

id-Additional-PositioningDataSet	INTEGER ::= 26

id-RAT-Type				INTEGER ::= 27

id-Ciphering-Data			INTEGER ::= 28

id-Ciphering-Data-Ack		INTEGER ::= 29

id-Ciphering-Data-Error-Report	INTEGER ::= 30

id-Coverage-Level			INTEGER ::= 31

id-Fixed-High-Accuracy-Uncertainty-Ellipse	INTEGER ::= 32

id-Fixed-High-Accuracy-Uncertainty-Altitude	INTEGER ::= 33

-- **************************************************************

--

-- Extension constants

--

-- **************************************************************

-- **************************************************************

--

-- Lists

--

-- **************************************************************

max-No-Of-Points		INTEGER ::= 15

max-Set			INTEGER ::= 9

max-GNSS-Set		INTEGER ::= 9

max-Add-Pos-Set		INTEGER ::= 8

max-Cipher-Set		INTEGER ::= 16

maxProtocolExtensions	INTEGER ::= 65535

maxProtocolIEs		INTEGER ::= 65535

END

* * * End of Changes * * * *