3GPP TSG-CT WG4 Meeting #102-e	C4-211203

# E-Meeting, 24th Feb – 05th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.333 | CR | 0111 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reference update: RFC 8851 and RFC 8853 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | MMCMH_Enh-CT |  |  |  |  |  | Date: |  |  | 2021-02-01 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | IETF draft-ietf-mmusic-rid has now been published as RFC 8851 and IETF draft-ietf-mmusic-sdp-simulcast has now been published as RFC 8853. Therefore the specification requires updating to the published versions. A number of editorial changes were implemented in draft-ietf-mmusic-rid-15 before it was published as RFC 8851 but there are no technical changes between the referenced draft version -15 and RFC 8851. A number of editorial changes were implemented in draft-ietf-mmusic-sdp-simulcast-14 before it was published as RFC 8853 (e.g. title was changed), but there are no technical changes between the referenced draft version -14 and RFC 8853. |  |  |  |  |  |  |  |  |
| Summary of change: |  | IETF draft-ietf-mmusic-rid is replaced with RFC 8851 and IETF draft-ietf-mmusic-sdp-simulcast is replaced with RFC 8853. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 3GPP specification is not aligned with IETF and will continue to refer to draft versions that are formally not available, leading to potential interoperability problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.15, 5.17.1, C.1, C.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*******

# *** First Change ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[2]	3GPP TS 23.002: "Network architecture".

[3]	ITU-T Recommendation H.248.1 (05/2002), Gateway control protocol: Version 2 + Corrigendum 1 (03/2004) and ITU-T Recommendation H.248.1 (09/2005), Gateway control protocol: Version 3 for Floor Control requirements.

[4]	ITU-T Recommendation H.248.4 (11/2000), Gateway control protocol: Transport over Stream Control Transmission Protocol (SCTP) + Corrigendum 1 (03/2004).

[5]	ITU-T Recommendation H.248.7 (03/2004), Gateway control protocol: Generic announcement package.

[6]	ITU-T Recommendation H.248.9 (03/2002), Gateway control protocol: Advanced media server package.

[7]	ITU-T Recommendation H.248.11 (11/2002), Gateway control protocol: Media gateway overload control package.

[8]	IETF RFC 2960: "Stream Control Transmission Protocol".

[9]	ITU-T Recommendation H.248.14 (03/2002), Gateway control protocol: Inactivity timer package.

[10]	ITU-T Recommendation H.248.16 (11/2002), Gateway control protocol: Enhanced digit collection packages and procedures + Corrigendum 1 (03/2004).

[11]	Void

[12]	ITU-T Recommendation H.248.27 (07/2003), Gateway control protocol: Supplemental Tones package

[13]	ITU-T Recommendation Q.1950 (12/2002), Bearer independent call bearer control protocol.

[14]	ITU-T Recommendation G.711 (11/1988), Pulse code modulation (PCM) of voice frequencies.

[15]	ITU-T Recommendation G.711 Appendix I (09/1999), A high quality low-complexity algorithm for packet loss concealment with G.711.

[16]	ITU-T Recommendation G.711 Appendix I (09/1999), A comfort noise payload definition for ITU-T G.711 use in packet-based multimedia communication systems.

[17]	ITU-T Recommendation E.180 (03/1998), Technical characteristics of tones for the telephone service.

[18]	TS 183 022: Telecommunication and Internet converged Services and Protocols for Advanced Networking (TISPAN); MGC Information Package.

[19]	ES 201 970 Access and Terminals (AT); Public Switched Telephone Networks (PSTN); Harmonized specification of physical and electrical characteristics at a 2-wire analogue presented Network Temination Point (NTP).

[20]	IETF RFC 2327 (1998): "SDP: Session Description Protocol".

[21]	IETF RFC 3551 (2003): "RTP Profile for Audio and Video Conferences with Minimal Control".

[22]	IETF RFC 2833 (2000): "RTP Payload for DTMF Digits, Telephony Tones and Telephony Signals".

[23]	IETF RFC 4040 (2005): "RTP payload format for a 64 kbit/s transparent call".

[24]	IETF RFC 3555 (2003): "MIME Type Registration of RTP Payload Formats".

[25]	3GPP TS 23.333: "Multimedia Resource Function Controller (MRFC) – Multimedia Resource Function Processor (MRFP) Mp interface: Procedures Descriptions".

[26]	ITU-T Recommendation H.248.9a1 (03/2007), "Gateway control protocol: Advanced media server package (draft work in progress)".

[27]	3GPP TS 29.163: "Interworking between the IM CN subsystem and CS networks – Stage 3".

[28]	W3C Recommendation (September 2004): "Speech Synthesis Markup Language (SSML) Version 1.0".

[29]	W3C Recommendation (September 2004): "Speech Recognition Grammar Specification (SRGS) Version 1.0".

[30]	ITU-T Recommendation H.248.36 (09/2005): "Hanging Termination Detection Package".

[31]	Void

[32]	IETF RFC 4583 (2006): "Session Description Protocol (SDP) Format for Binary Floor Control Protocol (BFCP) Streams".

[33]	ITU-T Recommendation H.248.19 (03/2013): "Gateway Control Protocol: Decomposed multipoint control unit, audio, video and data conferencing packages".

[34]	IETF RFC 4975 (2007): "The Message Session Relay Protocol (MSRP)".

[35]	ITU-T H.248.69 (03/2009): "Gateway control protocol: Packages for interworking between MSRP and H.248".

[36]	Void

[37]	Void

[38]	Void

[39]	IETF RFC 4145 (2005): "TCP-Based Media Transport in the Session Description Protocol (SDP)".

[40]	IETF RFC 4585 (2006): "Extended RTP Profile for Real-time Transport Control Protocol (RTCP) - Based Feedback (RTP/AVPF)".

[41]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[42]	3GPP TS 22.153: "Multimedia Priority Service".

[43]	ITU-T Recommendation H.248.52 (06/2008): "Gateway control protocol: QoS support packages".

[44]	ITU-T Recommendation H.248.82 (03/2013): "Gateway control protocol: Explicit Congestion Notification Support".

[45]	IETF RFC 5285: "A General Mechanism for RTP Header Extensions".

[46]	IETF RFC 6236: "Negotiation of Generic Image Attributes in the Session Description Protocol (SDP)".

[47]	ITU-T Recommendation H.248.50 (2010) and Corrigendum 1 (02/12): "Gateway control protocol: NAT traversal toolkit packages".

[48]	IETF RFC 5245: "Interactive Connectivity Establishment (ICE): A Protocol for Network Address Translator (NAT) Traversal for Offer/Answer Protocols".

[49]	3GPP TS 24.229: "IP Multimedia Call Control Protocol based on SIP and SDP".

[50]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[51]	IETF RFC 3830: "MIKEY: Multimedia Internet KEYing".

[52]	IETF RFC 793: "Transmission Control Protocol - DARPA Internet Program - Protocol Specification".

[53]	IETF RFC 4582: "The Binary Floor Control Protocol (BFCP)".

[54]	ITU-T Recommendation H.248.89 (10/2014): "Gateway control protocol: TCP support packages".

[55]	ITU-T Recommendation H.248.90 (10/2014): "Gateway control protocol: H.248 packages for control of transport security using TLS".

[56]	IETF RFC 6043: "MIKEY-TICKET: Ticket-Based Modes of Key Distribution in Multimedia Internet KEYing (MIKEY)".

[57]	3GPP TS 33.328: "IP Multimedia Subsystem (IMS) media plane security".

[58]	IETF RFC 4279: "Pre-Shared Key Ciphersuites for Transport Layer Security (TLS)".

[59]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[60]	3GPP TS 24.103: "Telepresence using the IP Multimedia (IM) Core Network (CN) Subsystem (IMS); Stage 3".

[61]	IETF draft-ietf-mmusic-sctp-sdp-26: "Session Description Protocol (SDP) Offer/Answer Procedures For Stream Control Transmission Protocol (SCTP) over Datagram Transport Layer Security (DTLS) Transport".

Editor's Note: The above document cannot be formally referenced until it is published as an RFC.

[62]	IETF draft-ietf-mmusic-data-channel-sdpneg-28: "SDP-based Data Channel Negotiation".

Editor's note: The above document cannot be formally referenced until it is published as an RFC.

[63]	ITU-T Recommendation H.248.93 (10/2014): "Gateway control protocol: ITU-T H.248 support for control of transport security using the datagram transport layer security (DTLS) protocol".

[64]	IETF RFC 8122: "Connection-Oriented Media Transport over the Transport Layer Security (TLS) Protocol in the Session Description Protocol (SDP)".

[65]	ITU-T Recommendation H.248.78 (11/2015): "Gateway control protocol: Bearer-level message backhauling and application level gateway".

[66]	IETF RFC 4573: "MIME Type Registration for RTP Payload Format for H.224".

[67]	ITU-T Recommendation H.224 (01/2005): "A real time control protocol for simplex applications using the H.221 LSD/HSD/MLP channels".

[68]	ITU-T Recommendation H.281 (11/1994): "A far end camera control protocol for videoconferences using H.224".

[69]	IETF RFC 5939: "Session Description Protocol (SDP) Capability Negotiation".

[70]	ITU-T Recommendation H.248.80 (01/2014): "Gateway control protocol: Usage of the revised SDP offer/answer model with ITU-T H.248".

[71]	IETF RFC 5104: "Codec Control Messages in the RTP Audio-Visual Profile with Feedback (AVPF)".

[72]	IETF RFC 4796: "The Session Description Protocol (SDP) Content Attribute".

[73]	IETF RFC 8853: "Using Simulcast in Session Description Protocol (SDP) and RTP Session".

[74]	IETF RFC 8851: "RTP Payload Format Restrictions".

[75]	IETF RFC 7728: "RTP Stream Pause and Resume".

[76]	ITU-T Recommendation H.248.98 (02/2016): "Gateway control protocol: Support of remote media pause and resume".

# *** Next Change ***

## 5.15	Mandatory Support of SDP and Annex C Information Elements

The v=, o=, s=, m=, c=, t=, a= and b= lines of the SDP [20] syntax shall be supported. All other lines should be ignored if received.

Table 5.15.1: Supported Annex C and SDP information elements

| Supported Annex C and SDP information elements: |  |  |
| --- | --- | --- |
| Information Element | Annex C Support | SDP Support |
| Protocol version (v=) | "SDP_V " | The protocol version (v=) line contains a single field: v= <version> and shall be used in accordance with IETF RFC 2327 [20] (i.e. v=0). |
| Origin (o=) | "SDP_O " | The origin line consists of 6 fields: o= <user name> <session ID> <version> <network type> <address type> <address>. The MRFC is not required to supply this line but shall accept it. The MRFP should populate this line as follows or use the value received from the MRFC: - <user name> should contain an hyphen - <session ID> and <version> should contain one or mode digits as described in IETF RFC 2327 [20] - <network type> shall be set to IN - <address type> shall be set to IP4 or IP6 The Address Type shall be set to "IP4" or "IP6" depending on the addressing scheme used by the network to which the MRFP is connected. - <address> should contain the fully qualified domain name of the gateway. |
| Session Name (s=) | "SDP_S" | The session name (s=) line contains a single field: s= <session-name>. The MRFC is not required to supply a session name but shall accept one. This line may be used to convey correlation information for use in CDRs. The MRFP shall use an hyphen "-" as a session name or the value received from the MRFC. |
| Connection data (c=) | "SDP_C " | The connection data line consists of 3 fields: c= <network-type> <address-type> <connection-address> - The <network-type> shall be set to "IN". - The <address-type> shall be set to "IP4" or "IP6" depending on the addressing scheme used by the network to which the MRFP is connected. - The <connection-address> sent by the MRFC in the remote descriptor is the address to which the MRFP shall send the media flows. - The <connection-address> sent by the MRFC in local descriptors may be a unicast IPv4 or IPv6 address or it may be wildcarded to allow the MRFP to choose an address. In the second case, MGs shall fill this field with a unicast IP address at which they will receive the media stream. Thus a TTL value shall not be present and a "number of addresses" value shall not be present. The field shall not be filled with a fully-qualified domain name instead of an IP address. When the <connection address> is wildcarded (i.e. choose wildcard) by the MRFC, the MRFP allocates an IP address based on the address type. The addressing space for which this address is taken may depend on the termination ID supplied by the MRFC. |
| Media announcements (m=) | "SDP_M " | Media Announcements (m=) lines consist of 3 fields: m= <media> <port> <transport> <format> - The <media> field shall be set to "audio"or "video" or "message" or "application" (NOTE 1). - The <port> field in remote descriptors is provided by the MRFC and represents the port to which the MRFP shall send the media flows. - The <port> field in local descriptors may be provided by the MRFC or wildcarded (i.e. choose wildcard) to allow the MRFP to choose a value for the port on which it wishes to receive the media stream - The <transport> field shall be according to table 5.15.2 - The <format> field may be explicitly supplied by the MRFC, wildcarded or overspecified. If the MRFC wishes to request the MRFP to choose which media formats it wishes to use for the call then the MRFC shall provide a "$" wildcard. If the MRFC wishes to suggest that the MRFP selects a media format from a list of possible media formats then it shall provide a list of appropriate media types in accordance with SDP. All conforming gateways shall support at least the default narrowband AMR codec as defined in 3GPP TS 26.114 [41]. Optionally, other codecs defined in 3GPP TS 26.114 [41] and format "8" for RTP/AVP (i.e. G.711 A-Law). Dynamic payloads shall not be used when a static RTP/AVP payload value is defined in IETF RFC 3551 [21]. |
| Bandwidth (b=) | "SDP_B " | The Bandwitdh (b=) line consists of 2 fields: b= <modifier>: <bandwidth-value> Bandwidth information shall be supplied by the MRFC if the required bandwidth cannot be immediately derived from the information contained in the m= line. If absent, the MRFP shall assume a reasonable default bandwidth value for well-known codecs and shall provide this value in the response sent to the MRFC. The Modifier field shall be set to "AS". The Bandwidth Value field shall be set to the maximum bandwidth requirement of the media stream in kbit/s. The bandwidth value shall take into account all headers down to the IP layer, including a 5% bandwidth for RTCP packets. |
| Time (t=) | "SDP_T " | The time (t=) line consists of two fields: t= <start-time> <stop-time>. This line is ignored by both the MRFC and the MRFP if received in local and remote descriptors. The MRFC is not required to supply a time description but shall accept one. When supplied, this line shall be set to 0 0. |
| Attributes (a=) | "SDP_A " | Attributes (a=) lines consist of two fields: a= <attribute>: <value> One or more of the "a" attribute lines specified below may be included, depending on the payload type. An attribute line not specified below should not be used. Only the following attributes are understood by the MRFP. Other attributes are ignored. a= rtpmap: <payload type> <encoding name>/<clock rate> [/<encoding parameters>] a= fmtp:<format> <format specific parameters> a= ptime: <time> a= userid: <token of user identifier> (NOTE 3) a= floorid: <token of Floor identifier> (NOTE 3) a= path:MSRP-URI (NOTE 4) a= rtcp-fb: <…> (NOTE 5, NOTE 13, NOTE 14) a= extmap:<x> <CVO-URN or ROI URN> (NOTE 6) a= imageattr: <payload type> <…> (NOTE 7) a= sctp-port: <port> (NOTE 8) a= max-message-size: <value> (NOTE 8) a= dcmap:< dcmap-stream-id> < subprotocol-opt> (NOTE 9) a= fingerprint: <certificate fingerprint> (NOTE 10) a=predefined_ROI: <…> (NOTE 11) a=bw-info: <payload type> <dir> <MaxSupBw>; <MaxDesBw>; <MinDesBw>; <MinSupBw>; <IpVer> (NOTE 12) a=content: <mediacnt> (NOTE 15) a=simulcast: <sc-dir> <rid-id-list> (NOTE 16) a=rid: <rid-id> <dir> <payload type> (NOTE 17) a=ccc_list: <codeclist> "¦" <ccc-prof> (NOTE 18) ICE support The attributes "a=candidate", "a=ice-pwd", and "a=ice-ufrag" (see IETF RFC 5245 [48]) may be provided for an SDP m-line in the local and remote descriptor if the MRFP supports ICE, see also 3GPP TS 24.229 [49]. In the local descriptor, the MRFC shall provide "a=ice-pwd", and "a=ice-ufrag" with wildcard sign "$" to request the allocation of a password and user name fragment, and the "a=candidate" of type "host" with the transport, port and priority parameters with wildcard sign "$" to request the allocation of a host candidate. The MRFP shall then reply with completed "a=ice-pwd", and "a=ice-ufrag" and "a=candidate" attributes in the local descriptor, and shall include "a=ice-lite" if it only supports ICE lite. In the remote descriptor, the MRFC may provide the "a=candidate", "a=ice-pwd", and "a=ice-ufrag". SDP Capability Negotiation support: the attributes of "a=acap", "a=tcap", "a=pcfg" and "a=acfg" (see IETF RFC 5939 [69]) may be provided in the local descriptor and/or remote descriptor. |
| NOTE 1: The "application" media is used to describe H.248 stream for a BFCP stream or H.248 stream for an UDP/DTLS/SCTP stream to be created for a CLUE data channel in telepresence using IMS as specified in 3GPP TS 24.103 [60]. The way to generate an "m" line for a BFCP stream follows the format specified in IETF RFC 4583 [32], where the port is always a TCP port, the transport field is "TCP/TLS/BFCP" if IMS media plane security is applied or otherwise "TCP/BFCP", the fmt (format) list is ignored. When a CLUE data channel is created, the "m" line for a UDP/DTLS/SCTP stream follows the format specified in IETF draft-ietf-mmusic-sctp-sdp [61] and IETF draft-ietf-mmusic-data-channel-sdpneg [62], where the transport field is "UDP/DTLS/SCTP", the fmt (format) indicates the usage of the SCTP association as "webrtc-datachannel". NOTE 2: Void NOTE 3: The "userid" and "floorid" are SDP media-level attributes. They are used in BFCP 'm' lines. The "floorid" defines a list of Floor identifiers, indicating the available Floor(s) for the user represented by the termination. The token representing the Floor identifier is the integer representation of the Floor ID. The "userid" attributes carry the integer representation of a user ID. NOTE 4: An MSRP-URI is an "msrp" or "msrps" URI defined as "MSRP-URI = msrp-scheme "://" authority ["/" session-id] ";" transport *(";" URI-parameter)". The authority component contains a numeric IP address and port. The session-id part identifies a particular session of the participant allowing multiple sessions to share the same TCP connection. NOTE 5: For AVPF transport, the "rtcp-fb" SDP attribute defined in IETF RFC 4585 [40] may be used to provide the feedback message types the MRFP is allowed to send and to indicate RTCP timing information. The support is optional and dependent on RTCP-fb support as described in 3GPP TS 26.114 [41]. The list of feedback messages supported by the MRFP is preconfigured in the MRFC. The "rtcp-fb" SDP attribute shall be sent from MRFC when applicable. NOTE 6: Support of the RTP header extension to signal CVO or Sent ROI is optional. The attribute "a=extmap" (see IETF RFC 5285 [45]) may be provided for an m-line in the local and remote descriptor. CVO-URN is "urn:3gpp:video-orientation" for a 2 bit granularity of rotation or "urn:3gpp:video-orientation:6" for a higher granularity of rotation, and ROI-URNs are "urn:3gpp:roi-sent" for arbitrary ROI information and "urn:3gpp:predefined-roi-sent" for predefined ROI information respectively, as specified in 3GPP TS 26.114 [41] and "x" represents the local identifier of the RTP header extension element as specified in IETF RFC 5285 [45] and is any number in a range [1 - 14]. NOTE 7: The support of the generic image attribute to negotiate the image size is optional. The attribute "a=imageattr" (see IETF RFC 6236 [46]) may be provided for an m-line in the local and remote descriptor if the MRFP supports the generic image attributes, see also 3GPP TS 26.114 [41]. The local descriptor indicates the image sizes which the MRFP supports in the receiving direction for the selected payload type and corresponds to the "recv" keyword (see IETF RFC 6236 [46]) in the "a=imageattr" that the MRFC will send within the SDP body on the Mr interface. The remote descriptor indicates the image sizes which the MRFP supports in the sending direction for the selected payload type and corresponds to the "send" keyword (see IETF RFC 6236 [46]) in the "a=imageattr" that the MRFC will send within the SDP body on the Mr interface. NOTE 8: The support of the "a=sctp-port" attribute to indicate the actual SCTP port is used only when the transport field of 'm' line is "UDP/DTLS/SCTP". The SDP "a=max-message-size" attribute may be used to indicate the maximum message size that an SCTP endpoint is willing to receive on the SCTP association associated with the 'm' line. NOTE 9: The support of the dcmap attribute to realize the CLUE data channel is used only when the transport field of 'm' line is "UDP/DTLS/SCTP", where the dcmap-stream-id field indicates the actual SCTP stream, and the subprotocol field indicates the protocol "CLUE". NOTE 10: The attribute(s) "a=fingerprint" (see IETF RFC 8122 [64]) shall be provided for an "m=" line in the local and remote descriptor if the MRFC requests the MRFP to establish the CLUE data channel. NOTE 11: The support of the predefined ROI attribute in the SDP is optional. The attribute "a=predefined_ROI" (see TS 26.114 [41]) may be provided for an m-line in the local and remote descriptor if the MRFP supports the predefined ROI attributes, see also 3GPP TS 26.114 [41]. NOTE 12: If the MRFP performs media transcoding and if the rate adaptation for media endpoints using the enhanced bandwidth negotiation is supported by the MRFP, attribute(s) "a=bw-info" (defined in 3GPP TS 26.114 [41], clause 19) with direction "send" or "sendrecv" may be provided for an m-line and the selected IP payload type and applicable IP version in the remote descriptor. NOTE 13: The support of the"RTCP Codec Control Commands and Indications" signalling is optional. The "rtcp-fb" SDP attribute with the "ccm" feedback parameter and the "fir" and/or "tmmbr" ccm parameters as defined in IETF RFC 5104 [71] may be provided for an m-line in the local and remote descriptor to indicate that the MRFP shall be prepared to receive and is allowed to send, respectively, the RTCP CCM feedback messages FIR, and/or TMMBR and TMMBN (the usage of the messages have been agreed in the SDP offer/answer negotiation between the MRFC and the end user). NOTE 14: The "rtcp-fb" SDP attribute with the "ccm" feedback parameter and the "pause" ccm parameter may be provided for an m-line in the local and remote descriptor to request the MRFP to apply "RTP-level pause and resume" procedures as defined in IETF RFC 7728 [75] and to indicate to the MRFP which RTCP feedback "CCM PAUSE-RESUME" messages the MRFP may send to the end user. NOTE 15: The "content" SDP attribute (see IETF RFC 4796 [72]) may be provided for an m-line in the local and remote descriptor to indicate a content of the media stream. NOTE 16: The "simulcast" SDP attribute (see IETF RFC 8853 [73]) may be provided for an m-line in the local and remote descriptor to indicate the list of the supported simulcast RTP formats in the receiving direction and/or in the sending direction. Each simulcast RTP format is identified by a simulcast stream identifier which has the form of the RTP stream identifier. NOTE 17: The "rid" SDP attribute (see IETF RFC 8851 [74]) may be provided for an m-line in the local and remote descriptor to indicate the identity, directionality and the payload type of the simulcast RTP stream. NOTE 18: The support of "Compact Concurrent Codec Negotiation and Capabilities" is optional. The "ccc_list" SDP attribute (defined in 3GPP TS 26.114 [41], clause S.5.7.2) may be provided in the remote descriptor to indicate to the MRFP the concurrent codec capabilities of an MMCMH conference participant in a compact representation. |  |  |

Table 5.15.2: Transport Protocol

| Transport Protocol <proto> in m-line: | If the MG does not support the requested transport protocol, it shall reject the command with error code 449. |
| --- | --- |
| RTP/AVP | RTP profile according IETF RFC 3551 [21]. For voice and video services |
| RTP/AVPF | Extended RTP profile for RTCP-based Feedback (RTP/AVPF) according IETF RFC 4585 [40]. For voice and video services (NOTE 1). |
| TCP/BFCP | For floor control service, see IETF RFC 4583 [32]. (NOTE 1) |
| TCP/MSRP | For message service, see IETF RFC 4975 [34]. (NOTE 1) |
| TCP/TLS/BFCP | For floor control service with IMS media plane security, see IETF RFC 4583 [32]). (NOTE 1) |
| TCP/TLS/MSRP | For message service with IMS media plane security, see IETF RFC 4975 [34]). (NOTE 1) |
| UDP/DTLS/SCTP | Data channel support using IETF draft-ietf-mmusic-sctp-sdp [61] and IETF draft-ietf-mmusic-data-channel-sdpneg [62]. |
| NOTE 1: support optional. NOTE 2: Upper case TCP is defined by IETF RFC 4145 [39] and registered by IANA. |  |

# *** Next Change ***

### 5.17.1	Formats and Codes

Table 5.17.1.1 shows the parameters which are required for the procedures defined in the following clauses.

The coding rules applied in ITU-T Recommendation H.248.1 [3] for the applicable coding technique shall be followed for the UMTS capability set.

The binary encoding rules which are applicable to the defined Abstract Syntaxes are the Basic Encoding Rules for Abstract Syntax Notation One, defined in ITU-T Recommendation X.690 [41].  Specifically in accordance with ITU-T Recommendation X.690 [41] clause 7.3, alternative encodings based on the definite and indefinite form of length are permitted by the basic encoding rules as a sender's option. Receivers shall support both alternatives.

Unsupported values of parameters or properties may be reported by the MGW and shall be supported by the MSC as such by using H.248.1 error code #449 "Unsupported or Unknown Parameter or Property Value". The unsupported or unknown value is included in the error text in the error descriptor.

Table 5.17.1.1: Information Elements Used in Procedures

| Signalling Object |  | H.248 Descriptor |  | Coding |  |
| --- | --- | --- | --- | --- | --- |
| Additional Bandwidth Properties |  | Remote Descriptor |  | The "a=bw-info" SDP attribute defined in 3GPP TS 26.114 [41], see table 5.15.1. |  |
| Allowed RTCP APP message types |  | Remote Descriptor |  | The "a=3gpp_mtsi_app_adapt" SDP attribute defined in 3GPP TS 26.114 [41]. |  |
| Announcement Cause |  | Events ObservedEvents |  | The "Meth" parameter in g/sc event per ITU-T Recommendation H.248.1 [3] Annex E.1.2 |  |
| Announcement Completed |  | Events ObservedEvents |  | The g/sc event per ITU-T Recommendation H.248.1 [3] Annex E.1.2 |  |
| Announcement Cycles |  | Signal |  | The "noc" parameter as per ITU-T Recommendation H.248.7 [5], Clause 4.3.1 |  |
| Announcement Direction |  | Signal |  | The "di" parameter as per ITU-T Recommendation H.248.7 [5], Clause 4.3.1 |  |
| Announcement Variant |  | Signal |  | The "av" parameter as per ITU-T Recommendation H.248.7 [5], Clause 4.3.1 |  |
| Arbitrary ROI Sent |  | Local Descriptor |  | The "rtcp-fb" SDP attribute defined in IETF RFC 4585 [30] to indicate the " Arbitrary ROI" RTCP feedback message expressed by the "3gpp-roi-arbitrary" parameter, as described in 3GPP TS 26.114 [41]. |  |
| Arbitrary ROI Received |  | Remote Descriptor |  | The "rtcp-fb" SDP attribute defined in IETF RFC 4585 [30] to indicate the " Arbitrary ROI" RTCP feedback message expressed by the "3gpp-roi-arbitrary" parameter, as described in 3GPP TS 26.114 [41]. |  |
| ASR Cause |  | Events ObservedEvents |  | The "rc" parameter in asr/asrfail event as per ITU-T Recommendation H.248.9a1 [26] Clause 12.2.1. |  |
| Autonomous response |  | LocalControl |  | Defined as "Autonomous Response" property ("rempr/ar") in ITU-T Recommendation H.248.98 [76]. |  |
| Autonomous request |  | LocalControl |  | Defined as "Autonomous Request" property ("rempr/aq") in ITU-T Recommendation H.248.98 [76]. |  |
| Cause |  | Events ObservedEvents |  | Encoded as "Meth" parameter in g/sc event per ITU-T Recommendation H.248.1 [3] Annex E.1.2 |  |
| CCM BASE |  | Local Descriptor or Remote Descriptor |  | "rtcp-fb" SDP attribute (defined in IETF RFC 4585 [30]) with the "ccm" feedback parameter and the "fir" and/or "tmmbr" ccm parameters as defined in IETF RFC 5104 [71]. |  |
| CCM pause-resume |  | Local Descriptor or Remote Descriptor |  | The "rtcp-fb" SDP attribute (defined in IETF RFC 4585 [40]) with the "ccm" feedback parameter (defined in IETF RFC 5104 [71]) and the "pause" ccm parameter as defined in IETF RFC 7728 [75]. |  |
| Certificate Fingerprint |  | Local Descriptor or Remote Descriptor |  | The "a=fingerprint" SDP attribute(s) as defined in IETF RFC 8122 [64], see table 5.15.1. |  |
| CLUE Message Send |  | Signal |  | Defined as the "mcbalg/sblm" signal with the application protocol indicating "CLUE" in ITU-T Recommendation H.248.78 [65]. |  |
| CLUE Message Received |  | Events ObservedEvents |  | Defined according to Detect Bearer Level Message event with the application protocol indicating "CLUE" in ITU-T Recommendation H.248.78 [65]. |  |
| Codec List |  | Local Descriptor or Remote Descriptor |  | <fmt list> in a single SDP m-line. For a static RTP payload type, the codec type should be implied by the RTP payload type, if not then each codec type shall be provided in a separate SDP "a=rtpmap"-line and possibly additional SDP "a=fmtp"-line(s). For a dynamic RTP payload type, for each codec information on the codec type shall be provided in a separate SDP "a=rtpmap"-line and possibly additional SDP "a=fmtp"-line(s). |  |
| Concurrent Codec Capabilities |  | Remote Descriptor |  | The "a=ccc_list" session level SDP attribute defined in 3GPP TS 26.114 [41], see table 5.15.1. |  |
| ConfID |  | ContextAttribute Descriptor |  | The "fconfid" parameter as per ITU-T Recommendation H.248.19 [33], Clause 10.6.1.1.It is defined as type integer as used over BFCP. |  |
| Context ID |  | NA |  | Binary Encoding: As per ITU-T Recommendation H.248.1 [3] Annex A. Textual Encoding: As per ITU-T Recommendation H.248.1 [3] Annex B. |  |
| ControlledByChair |  | TerminationState Descriptor |  | List of Floor Ids controlled by this termination as a chair, specified by "cfi" as defined in Clause 10.1.1.2 of ITU-T Recommendation H.248.19 [33]. |  |
| Diffserv Code Point |  | Local Control |  | Defined according to the Differentiated Services Code Point property in ITU-T Recommendation H.248.52 [43]. |  |
| Digit |  | Observed Events |  | Encoding as per ITU-T Recommendation H.248.1 Annex E.6.2. Digits are reported individually by the MRFP. |  |
| DTMFTrigger |  | Signal Descriptor |  | "endinputkey, eik" see H.248.9a1 [26] Clause 16.3.1.1.16. |  |
| ECN Enabled |  | Local Descriptor or Remote Descriptor |  | Defined according to the "ECN Enabled" property in ITU-T Recommendation H.248.82 [44]. |  |
| ECN Failure |  | Events, Observed Events |  | Defined according to the "ECN Failure" Event in ITU-T Recommendation H.248.82 [44]. |  |
| ECN Failure Type |  | ObservedEvents Descriptor |  | As for the ObservedEventsDescriptor Parameter "Failure Type" in ITU-T Recommendation H.248.82 [44]. |  |
| ECN Initiation Method |  | Local Descriptor or Remote Descriptor |  | Defined according to "Initiation Method" property in ITU-T Recommendation H.248.82 [44]. |  |
| End of Recording Notification |  | Events ObservedEvents |  | Enables the MRFC to be informed of the end of a recording. Corresponds to aasrec/audfail (mrp/audfail) and aasrec/precsucc, (mrp/precsucc) events see ITU-T Recommendation H.248.9a1 [26] 12.2. |  |
| Establish TCP Connection |  | Signals |  | Defined according to the Establish BNC signal (tcpbcc/EstBNC) in ITU-T Recommendation H.248.89 [54]. |  |
| Establish (D)TLS session |  | Signals |  | Defined according to the Establish BNC signal (tlsbsc/EstBNC) in ITU-T Recommendation H.248.90 [55] and for DTLS usage in ITU-T Recommendation H.248.93 [63]. |  |
| Extended Header for CVO |  | Local Descriptor or Remote Descriptor |  | "extmap" attribute in SDP a-line as defined in IETF RFC 5285 [45], see table 5.15.1. |  |
| Extended RTP Header for Sent ROI |  | Local Descriptor or Remote Descriptor |  | "extmap" attribute in SDP a-line to pass on the ROI extended RTP header as defined by IETF RFC 5285 [45] for carriage of predefined and/or arbitrary ROI information, see table 5.15.1 |  |
| FloorControlAlgorithm |  | Context Attrribute (NOTE 1) |  | Sub-list of (Floorid, Algorithm). "fca" as defined in Clause 10.4.1.2 of ITU-T Recommendation H.248.19 [33]. |  |
| FloorID |  | Local Descriptor |  | "a= floorid" SDP line as specified in Table 5.15.1. |  |
| FloorRequestResult |  | Signal Descriptor |  | The "res" parameter as per ITU-T Recommendation H.248.19 [33], Clause 10.5.3.1.1.2. It is defined as Boolean (success or fail) |  |
| FloorResAssociations |  | Context Attribute (NOTE 1) |  | The "fsa" parameter as per ITU-T Recommendation H.248.19 [33], Clause 10.6.1.2. It is defined as sub-list of (Floorid, StreamID). |  |
| FloorStatus |  | Observed Events |  | "Floor Status, fs" as defined in ITU-T Recommendation H.248.19 [33]. This is a list of FloorIds and status (e.g. granted, revoked) |  |
| Generic Image Attribute |  | Local Descriptor or Remote Descriptor |  | "imageattr" attribute in SDP a-line as defined in IETF RFC 6236 [46], see table 5.15.1. |  |
| ICE host candidate request |  | Local Descriptor |  | The "a=candidate" SDP attribute defined in IETF RFC 5245 [48] of type "host" with the transport, port and priority parameters with wildcard sign "$" to request the allocation of a host candidate |  |
| ICE host candidate |  | Local Descriptor |  | The "a=candidate" SDP attribute defined in IETF RFC 5245 [48] |  |
| ICE lite indication |  | Local Descriptor |  | The "a=ice-lite" SDP attribute defined in IETF RFC 5245 [48]. |  |
| ICE password request |  | Local Descriptor |  | The "a=ice-pwd" SDP attribute defined in IETF RFC 5245 [48] with wildcard sign "$". |  |
| ICE password |  | Local Descriptor |  | The "a=ice-pwd" SDP attribute defined in IETF RFC 5245 [48]. |  |
| ICE received candidate |  | Remote Descriptor |  | The "a=candidate" SDP attribute defined in IETF RFC 5245 [48] |  |
| ICE received password |  | Remote Descriptor |  | The "a=ice-pwd" SDP attribute defined in IETF RFC 5245 [48]. |  |
| ICE received Ufrag |  | Remote Descriptor |  | The "a=ice-ufrag" SDP attribute defined in IETF RFC 5245 [48]. |  |
| ICE Ufrag request |  | Local Descriptor |  | The "a=ice-ufrag" SDP attribute defined in IETF RFC 5245 [48] with wildcard sign "$". |  |
| ICE Ufrag |  | Local Descriptor |  | The "a=ice-ufrag" SDP attribute defined in IETF RFC 5245 [48]. |  |
| ICE Connectivity Check Result |  | Events, Observed Events |  | Defined according to Connectivity Check Result event in ITU-T Recommendation H.248.50 [47]. |  |
| ICE Send Connectivity Check |  | Signals |  | Defined as the ostuncc/scc signal in ITU-T Recommendation H.248.50 [47]. |  |
| ICE New Peer Reflexive Candidate |  | Events, Observed Events |  | Defined according to New Peer Reflexive Candidate event in ITU-T Recommendation H.248.50 [47]. |  |
| ICE Send Additional Connectivity Check |  | Signals |  | Defined as the ostuncc/sacc signal in ITU-T Recommendation H.248.50 [47]. |  |
| IncMessageFilters |  | LocalControl Descriptor |  | "Incoming Message Filters, imf" parameter in H.248.69 [35] Clause 13.1.1, which is defined as string and complies with Sieve [IETF RFC5228] with the exceptions described in H.248.69 [35] Clause 13.6. |  |
| IP Address |  | Local Descriptor or Remote Descriptor |  | <connection address> in SDP "c-line" |  |
| Iterations |  | Signal |  | " Iterations, it" parameter in H.248.9a1 [26] Clause 13.3.1.1.3 or Clause 13.3.2.1.3 |  |
| MaxFloorHolder |  | Context Attribute (NOTE 1) |  | Sub-list of (FloorID, Number). "mfu" as defined in Clause 10.4.1.2 of ITU-T Recommendation H.248.19 [33]. |  |
| Maximum Record Time |  | Signal |  | "Record Length Timer, rlt" parameter in H.248.9a1 [26] Clause 16.3.1.1.8 for multimedia recording or Clause 10.3.1.1.8 for audio recording |  |
| Media Identifier |  | Signal |  | TBD |  |
| Mediatype |  | Local Descriptor or Remote Descriptor |  | <media> in sdp m-line "audio" for voice service, and "image" for T.38 service. |  |
| MessageContentType |  |  |  | TBD as enumeration to indicate the content type of message. (e.g. video, audio) |  |
| MessageContentFmt |  |  |  | TBD as enumeration to indicate the content format (e.g. mpeg, jpeg for picture) |  |
| MessageIdentifier |  | Signal |  | "mcr" parameter in the mess/sm signal in H.248.69 [35] Clause 10.3.1.1.2, which is defined as URI that points to the message data that shall be sent. |  |
| MessagePlayResultReport |  | Signal |  | "fr" or "sr" parameter in the mess/sm signal in H.248.69 [35], which is defined as Enumeration to indicate the request of report result of message play (Success Report, Failure Report, Both or None) |  |
| MessagePlayCause |  | ObservedEvents |  | "sc" parameter in the mess/msrs event in H.248.69 [35] Clause 10.2.1.2.2 , which is defined as Enumeration to notify the result of the message play. |  |
| MessageRecordFileIdentifier |  | Signal |  | "sl" parameter in the recmess/rm signal in H.248.69 [35] Clause 15.3.1.1.1, which is defined as a URI where the messages are to be stored. |  |
| MessagesReceivedNumQuota |  | Events |  | "mrq" parameter in the msrpstat/mquota event in H.248.69 [35] Clause 8.2.1.1.2, which is defined as integer to define the quota for number of messages that may be received on the termination for the messaging Stream. |  |
| MessagesReceivedVolQuota |  | Events |  | "mrv" parameter in the msrpstat/mquota event in H.248.69 [35] Clause 8.2.1.1.4, which is defined as integer to define the quota for cumulative total size of messages that may be received on the Termination for the messaging Stream. |  |
| MessagesreceivedNum |  | ObservedEvents Statistics |  | "nmr" parameter in the msrpstat/mquota event or statistics in H.248.69 [35], which is defined as integer to define the number of messages that have been received on the termination for the messaging Stream. |  |
| MessagesReceivedVol |  | ObservedEvents Statistics |  | "vmr" parameter in the msrpstat/mquota event or statistics in H.248.69 [35], which is defined as integer to define the cumulative total size of messages that have been received on the Termination for the messaging Stream. |  |
| MessagesSentNumQuota |  | Events |  | "msq" parameter in the msrpstat/mquota event in H.248.69 [35] Clause 8.2.1.1.1, which is defined as integer to define the quota for number of messages that may be sent from the termination for the messaging Stream. |  |
| MessagesSentVolQuota |  | Events |  | "msv" parameter in the msrpstat/mquota event in H.248.69 [35] Clause 8.2.1.1.3, which is defined as integer to define the quota for cumulative total size of messages that may be sent from the Termination for the messaging Stream. |  |
| MessagesSentNum |  | ObservedEvents Statistics |  | "nms" parameter in the msrpstat/mquota event or or statistics in H.248.69 [35], which is defined as integer to define the number of messages that have been sent from the termination for the messaging Stream. |  |
| MessagesSentVol |  | ObservedEvents Statistics |  | "vms" parameter in the msrpstat/mquota event or statistics in H.248.69 [35], which is defined as integer to define the cumulative total size of messages that may be sent from the Termination for the messaging Stream. |  |
| MMCMH policy |  | Context Attrribute (NOTE 1) |  | Defined as "MMCMH policy" property in Annex C, clause C.2.2.1. |  |
| MSRP session identity |  | Local Descriptor or Remote Descriptor |  | <session-id> in SDP "a= path:MSRP-URI"-line. |  |
| Notify TCP Connection Establishment Failure Event |  | ObservedEvents |  | As for the ObservedEvent Parameter in clause E.1.2 of ITU-T Recommendation H.248.1 [3] "General cause". |  |
| Notify (D)TLS session establishment Failure Event |  | ObservedEvents |  | As for the ObservedEvent Parameter in clause E.1.2 of ITU-T Recommendation H.248.1 [3] "General cause". |  |
| OutMessageFilters |  | LocalControl Descriptor |  | "Outgoing Message Filters, omf" parameter in H.248.69 [35] Clause 13.1.3, which is defined as string and complies with Sieve [IETF RFC5228] with the exceptions described in H.248.69 [35] Clause 13.6. |  |
| Port |  | Local Descriptor or Remote Descriptor |  | <port> in SDP m-line. |  |
| Predefined ROI Sent |  | Local Descriptor |  | The "rtcp-fb" SDP attribute defined in IETF RFC 4585 [30] to indicate the "Predefined ROI" RTCP feedback message expressed by the "3gpp-roi-predefined" parameter, as described in 3GPP TS 26.114 [41]. |  |
| Predefined ROI Received |  | Remote Descriptor |  | The "rtcp-fb" SDP attribute defined in IETF RFC 4585 [30] to indicate the "Predefined ROI" RTCP feedback message expressed by the "3gpp-roi-predefined" parameter, as described in 3GPP TS 26.114 [41]. |  |
| Pre-Shared Key |  | LocalControl Descriptor |  | Traffic-Encrypting Key (TEK) associated with the Crypto Session (CS) as defined in IETF RFC 6043 [56] and Annex H of 3GPP TS 33.328 [57] that will be used in TLS handshake. (NOTE 2) |  |
| Priority Information |  | NA |  | Priority Indicator (clause 6.1.1 of ITU-T Recommendation H.248.1 [3]) Binary Encoding: Encoding as per ITU-T Recommendation H.248.1 [3] Annex A "priority" context attribute Textual Encoding: Encoding as per ITU-T Recommendation H.248.1 [3] Annex B "priority" context attribute |  |
| Recognition Result |  | ObservedEvents |  | "asrr" parameter to "asrsucc" event in H.248.9a1 [26] Clause 12.2.2.2.1. Each result may be able to be structured by multiple parts in time sequence with the input time, may be able to include the text token that the value will correspond to tokens as defined by the SRGS grammar, may be able to include the interpretation of application specific markup, may be able to include the confidence score that represents the recognition quality. |  |
| Record File Format |  | Signal |  | To Be Defined |  |
| Record File Identifier |  | Signal |  | "rid" parameter in playrec signal H.248.9a1 [26] Clause 16.3.1.1.9 for multimedia recording or Clause 10.3.1.1.9 for audio recording |  |
| Release TCP Connection |  | Signals |  | Defined according to the Release BNC signal (tcpbcc/RelBNC) in ITU-T Recommendation H.248.89 [54]. |  |
| Release TLS session |  | Signals |  | Defined according to the Release BNC signal (tlsbsc/RelBNC) in ITU-T Recommendation H.248.90 [55]. |  |
| Reserve_Value |  | Local Control |  | ITU-T Recommendation H.248.1 [3] Mode property. Binary Encoding: Encoding as per ITU-T Recommendation H.248.1 Annex A "reserveValue" Textual Encoding: Encoding as per ITU-T Recommendation H.248.1 Annex B "reservedValueMode". |  |
| RtcpbwRS |  | Local Descriptor or Remote Descriptor |  | <bandwidth> in SDP "b:RS"-line. |  |
| RtcpbwRR |  | Local Descriptor or Remote Descriptor |  | <bandwidth> in SDP "b:RR"-line. |  |
| RTPpayload |  | Local Descriptor or Remote Descriptor |  | <fmt list> in SDP m-line |  |
| SCTP Max Message Size |  | Local Descriptor or Remote Descriptor |  | The "a=max-message-size" SDP attribute as defined in IETF draft-ietf-mmusic-sctp-sdp [61], see table 5.15.1. |  |
| SCTP Port |  | Local Descriptor or Remote Descriptor |  | The "a=sctp-port" SDP attribute as defined in IETF draft-ietf-mmusic-sctp-sdp [61], see table 5.15.1. |  |
| SCTP Stream ID |  | Local Descriptor or Remote Descriptor |  | <dcmap-stream-id> in SDP "a=dcmap" line as defined in IETF draft-ietf-mmusic-data-channel-sdpneg [62], see table 5.15.1. |  |
| SCTP Subprotocol |  | Local Descriptor or Remote Descriptor |  | <subprotocol-opt> in SDP "a=dcmap" line as defined in IETF draft-ietf-mmusic-data-channel-sdpneg [62], see table 5.15.1. |  |
| SDPCapNeg configuration |  | Local Descriptor or Remote Descriptor |  | The SDP attributes for SDP capability negotiation according to IETF RFC 5939 [69]. |  |
| SDPCapNeg Supported Capabilities |  | Termination State |  | Defined according to SDPCapNeg Extensions property in ITU-T Recommendation H.248.80 [70]. |  |
| SenderAddr |  |  |  | TBD |  |
| Simulcast desc |  | Local Descriptor or Remote Descriptor |  | The "a=simulcast" SDP attribute as defined in IETF RFC 8853 [73], see table 5.15.1. |  |
| Simulcast format |  | Local Descriptor or Remote Descriptor |  | The "a=rid" SDP attribute as defined in IETF RFC 8851 [74], see table 5.15.1. |  |
| SRGS Grammar |  | Signal |  | "grammar file, gf" parameter in asr/asr signal in H.248.9a1 [26] Clause 12.3.1.1.2 |  |
| SRGS grammar URI |  | Signal |  | " Recognition grammar identifier, rgid" parameter in asr/ asrid signal in H.248.9a1 [26] Clause 12.3.2.1.2 |  |
| SSML |  | Signal |  | "an" parameter in the aastts/play signal in H.248.9a1 [26] Clause 14.3.1.1.1 |  |
| StatRepReason |  | ObservedEvents |  | "qreach" parameter in the msrpstat/mquota event in H.248.69 [35] Clause 8.2.1.2.1, which is defined as enumeration to indicate the quota that has triggered the reporting of the event. |  |
| StatValTime |  | Events |  | "tm" parameter in the msrpstat/mquota event in H.248.69 [35] Clause 8.2.1.1.5, which is defined as integer to define how long for the quotas associated are active for. |  |
| Stream content |  | Local Descriptor or Remote Descriptor |  | The "a=content" SDP attribute as defined in IETF RFC 4796 [72], see table 5.15.1. |  |
| Stream Number |  | Stream |  | Encoding as per ITU-T Recommendation H.248.1 Annex B "Stream"/"ST". For a single stream, this may be omitted by the MRFC. |  |
| STUN server request |  | LocalControl |  | Encoding as per ITU-T Recommendation H.248.50 [47] "MG Act-as STUN Server" (mgastuns) package "Act-as STUN Server" (astuns, 0x0001) property. |  |
| Termination heartbeat |  | Events ObservedEvents |  | The hangterm/thb event as per ITU-T Recommendation H.248.36 [30] Clause 5.2.1. |  |
| Termination ID |  | NA |  | Binary Encoding: As per ITU-T Recommendation H.248.1 [3] Annex A. Textual Encoding: As per ITU-T Recommendation H.248.1 [3] Annex B. |  |
| Timing |  | Events |  | As in dd package H.248.1 [3] Annex E.6.2, (end tone detected shall be used) |  |
| Tone Completed |  | Events ObservedEvents |  | "g/sc" see H.248.1 [3] Annex E.1.2 |  |
| Tone Duration |  | Signal |  | As in the respective tone package |  |
| Tone Identity |  | Signal |  | Encoding as per ITU-T Recommendation H.248.1 Annex B and the package which defines the tone (Tone Signal Ids only). |  |
| Transaction ID |  | NA |  | Binary Encoding: As per ITU-T Recommendation H.248.1 [3] Annex A. Textual Encoding: As per ITU-T Recommendation H.248.1 [3] Annex B. |  |
| TTS Completed |  | Events ObservedEvents |  | "g/sc" see H.248.1 [3] Annex E.1.2 if successful, aastts/ttsfail H.248.9a1 [26] Clause 14.2.1 if not successful. |  |
| Transport |  | Local Descriptor or Remote Descriptor |  | <transport> in SDP m-line, see 5.15 |  |
| UserID |  | Local Descriptor |  | "a= userid" SDP line as specified in Table 5.15.1. |  |
| NOTE 1: H.248.1 version 3 required. NOTE 2: Pre-Shared Key information element needs to be specified in ITU-T Recommendation H.248.90 [55]. |  |  |  |  |  |

# *** Next Change ***

# C.1	Introduction

This annex contains a Multi-party Multimedia Conference Media Handling Package that is required for the Multi-stream Multiparty Conferencing Media Handling (MMCMH) feature as specified in 3GPP TS 23.333 [25] clause 5.11.3. The MMCMH feature requires support of simulcast RTP media streams, see IETF RFC 8853 [73].

NOTE:	The ID value of Packages, Properties, Events, Parameters, Signals, etc. are designated below by "textID" (a string representing its text ID) and by "0x????" (the ID hexadecimal representation).

# *** Next Change ***

## C.2.7	Procedures

To enable the Multi-party Multimedia Conference Media Handling functionality, the MC:

a)	shall reserve a context and indicate the applicable MMCMH policies via the mmcmhp property, including at least the "mmcmhbp" value;

b)	for each conference participant, shall allocate a termination within that context and place all streams towards/from that participant within that termination; and

c)	for each media stream:

-	shall indicate the media type for each stream;

-	may indicate the video media type via the "a=content" SDP attribute (defined in IETF RFC 4796 [72]) in the local descriptor and the remote descriptor;

-	may provide the "a=simulcast" attribute (defined in IETF RFC 8853 [73]), and the corresponding "a=rid" attributes (defined in IETF RFC 8851 [74]) with the "pt" parameter defining the simulcast stream identification in the local descriptor and the remote descriptor; and

-	may provide the "a=rtcp-fb" line (see IETF RFC 4585 [40]) with the "pause" CCM parameter (defined in IETF RFC 5104 [71]), the "nowait" pause attribute and the "config" pause attribute (defined in IETF RFC 7728 [75]) in the local descriptor and the remote descriptor and the "Autonomous Response (rempr/ar)" and the "Autonomous Request" (rempr/aq)" properties defined in ITU-T Recommendation H.248.98 [76] in the LocalControl descriptor.

NOTE:	The SDP "a=rid" attribute lines with a "pt" parameter define the simulcast stream identifications within a single media description.

Upon reception of the mmcmhp property, the MP shall execute the policies defined for the received values.

Figure C.2.7.1:	Example of MMCMH switching

Figure C.2.7.1 shows an example of MMCMH switching where video media flows with different StreamIDs can be interconnected. The context level mmcmhp property is set to "mmcmhbp, vadv". Value "vadv" indicates that the MP shall detect voice activity on the incoming audio streams. The MP shall forward the main video received from the active speaker (i.e. from the media sender from which the audio stream is received where voice activity is currently detected) to all other conference participant. The MP should forward the main video of the previous speaker (i.e. received from the media sender from which an audio stream was received where the most recent past voice activity has been detected) to the active speaker (i.e. towards the media receiver associated with the media sender from which an audio stream is received where voice activity is currently detected). The MP should forward received thumbnail video streams from the most recent previous speaker(s) (i.e. from the media sender(s) from which audio stream(s) was/were received where the most recent past voice activities have been detected). The MP should select video streams received from the most recent speaker(s) (i.e. from the media sender(s) from which audio stream(s) are received where the most recent voice activities are or have been detected).

Stream ID = 1 is an audio stream whose volume level on each termination is being monitored. Main video stream Stream ID = 2 is configured with a simulcast property on each termination: two simulcast RTP video streams with "recv" property and one RTP video stream with "send" property. Stream ID = 3 is a screenshare video stream. StreamID =4 and StreamID = 5 are thumbnail video streams with the StreamMode property set to "SendOnly". On termination B only one thumbnail video stream StreamID =4 can be sent. Termination A is the active speaker. Termination C was the previous speaker. As the mmcmhp property is set to "mmcmhbp, vadv", the received incoming videos are sent as outgoing videos according to the figure C.2.7.1. Local image shows the simulcast streams of main video: one video stream in high resolution and the other video stream in low resolution (thumbnail-sized simulcast format of the main video) received by the MP on StreamID = 2 and the remote image shows the video streams sent to each user: main video stream of active speaker in high resolution on StreamID = 2, and thumbnail videos of the other participants on StreamID =4 and StreamID = 5. Active speaker A will receive the main video of the previous speaker C in high resolution on StreamID = 2. On termination B only the thumbnail video from the previous speaker C is sent.

# *** End of Changes ***