3GPP TSG-CT WG4 Meeting #103-e	C4-212610

# E-Meeting, 14th – 23th Apr 2021														    was C4-212110, 2431

Source:	ZTE, Samsung

Title:	TS29.536 – NSAC Service and Procedures

Spec:	3GPP TS29.536

Agenda item:	6.1.8 / eNS_Ph2

Document for:	Approval

1. Introduction

This contribution provides service and procedure description clauses to TS29.536 v0.0.1.

2. Proposal

It is proposed to agree the following changes to 3GPP TS29.536 v0.0.1.

* * * Begin of Changes * * * *

5.2	Nnsacf_NumOfUEsPerSlice Service

5.2.1	Service Description

The Nnsacf_NumOfUEsPerSlice service provides the service capability for the NF Service Consumer (e.g. AMF) to request admission control for UEs accessing a specific network slice. The following are the key functionalities of this NF service:

-	Request the NSACF to control the number of UEs registered to a specific slice, e.g. perform availability check and update the number of UEs registered to a specific slice;

-	Notify the NF Service Consumer (e.g. AMF) of the activation/deactivation of EAC (early admission control) mode for NSAC procedure;

The Nnsacf_NumOfUEsPerSlice service supports the following service operations.

Table 5.2.1-1: Service operations supported by the Nnsacf_NumOfUEsPerSlice service

| Service Operations | Description | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| AvailabilityCheckAndUpdate | Request the NSACF to perform admission control related to the number of UEs registered to a slice. | Request/Response | AMF |
| EACNotify | Notify the NF Service Consumer (e.g. AMF) of the activation/deactivation of EAC mode. | Subscribe/Notify | AMF |

Editor's Note:	It is FFS whether a better name for the "AvailabilityCheckAndUpdate" service operation should be considered. If so, an LS to SA2 is needed.

5.2.2	Service Operations

5.2.2.1	Introduction

This clause introduces the related procedures using Nnsacf_NumOfUEsPerSlice service operations to request the NSACF to control the number of UEs registered to a specific slice.

5.2.2.2	AvailabilityCheckAndUpdate

5.2.2.2.1	General

The AvailabilityCheckAndUpdate service operation shall be used by the NF Service Consumer (e.g. AMF) to request the NSACF to control the number of UEs registered to a specific slice, e.g. perform availability check and update the number of UEs registered to a slice. It is used in the following procedures:

-	AMF initiated network slice admission control procedure related to control the number of UEs registered to a slice (see clause 4.2.11.2 of 3GPP TS 23.502 [3]).

5.2.2.2.2	AMF initiated network slice admission control

The NF Service Consumer (e.g. AMF) shall invoke the AvailabilityCheckAndUpdate service operation to request the NSACF to perform network slice admission control procedure related to the number of UEs, by using the HTTP POST method as shown in Figure 5.2.2.2.2-1.

Figure 5.2.2.2.2-1: Availability Check for Number of UEs per Slice

1.	The NF Service Consumer (e.g. AMF) shall send a POST request to the resource representing the network slice admission control related to the number of UEs (i.e. …/slices/ue-ac) in the NSACF. The payload body of the POST request shall contain the input data structure for network slice admission control.

2a.	For each S-NSSAI included in UeACRequestData, the NSACF shall perform the following actions:

- if the update flag is set to "increase" and EAC mode for this S-NSSAI is enabled, the NSACF shall first check the total number of UEs to this slice will not exceed the maximum number of UEs allowed to be registered to this slice. When counting the total number of the UEs ( including the UEs indicated in the request and the UEs already stored in the NSACF), the NSACF shall check whether a UE indicated in the request is already in the UE ID list stored in the NSACF.

-	if no excess, the NSACF records the indicated UEs to the UE ID list stored in the NSACF, and updates the total number of UEs registered to this slice accordingly;

-	if the total number of UEs exceeds the maximum of UEs allowed to be registered to this slice, the NSACF shall not update the UE ID list stored in the NSACF, and not update the total number of UEs. Instead, the NSACF shall record this S-NSSAI in the failed list of S-NSSAI in the response message, together with a "MAXIMUM_NUM_REACHED" reason;

- if the update flag is set to "increase" and EAC mode for this S-NSSAI is disabled, the NSACF shall:

-	record the indicated UEs to the UE ID list stored in the NSACF, and updates the total number of UEs registered to this slice accordingly;

- if the update flag is set to "decrease", the NSACF decrease the total number of UEs registered to this slice, and removes the indicated UEs from the UE ID list stored in the NSACF;

If above NSACF handling is successful, "200 OK" shall be returned, with necessary response data, e.g. indicating the failed S-NSSAI.

Editor's Note:	It is decided to return "200 OK" at this time, as the NSACF handling is regarded successful even the total number of UEs for some S-NSSAI exceed the allowed maximum number.

2b.	On failure or redirection, the appropriate HTTP status code (e.g. "403 Forbidden") indicating the error shall be returned.

A ProblemDetails IE shall be included in the payload body of POST response, with the "cause" attribute of ProblemDetails set to application error codes specified in table 6.1.7.3-1.

5.2.2.3	EACNotify

5.2.2.3.1	General

The EACNotify service operation shall be used by the NSACF to inform the NF Service Consumer (e.g. AMF) of the activation/deactivation of EAC mode. It is used in the following procedures:

-	NSACF initiated configuration on EAC mode procedure (see clause 4.2.11.3 of 3GPP TS 23.502 [3]).

5.2.2.3.2	NSACF initiated EAC mode configuration

The EACNotify service operation shall be used by the NSACF to configure the EAC mode to the NF Service Consumer (e.g. AMF).

The NSACF shall notify the NF Service Consumer (e.g. AMF) by using the HTTP POST method as shown in Figure 5.2.2.2.3-1.

Figure 5.2.2.2.3-1: NSACF initiated EAC mode configuration procedure

1.	The NSACF shall send a POST request to the EAC Notification callback URI provided by the NF Service Consumer (e.g. AMF). The payload body of the POST request shall contain the notification payload.

The callback URI may be provided by the AMF during the first interaction with the NSACF, or be retrieved from the AMF profile during NF discovery from the NRF.

2a.	On success, "204 No Content" shall be returned and the payload body of the POST response shall be empty.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.5.2.3.1-2 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.7.3-1.

* * * Next Change * * * *

6.1	Nnsacf_NumOfUEsPerSlice Service API

6.1.1	Introduction

The Nnsacf_NumOfUEsPerSlice shall use the Nnsacf_NumOfUEsPerSlice API.

The API URI of the Nnsacf_NumOfUEsPerSlice API shall be:

{apiRoot}/<apiName>/<apiVersion>/

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "nnsacf-num-of-ues-per-slice".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 5.3.

6.1.2	Usage of HTTP

6.1.2.1	General

HTTP/2, IETF RFC 7540 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Nnsacf-NumOfUEsPerSlice API is contained in Annex A.

6.1.2.2	HTTP standard headers

6.1.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

6.1.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [13].

6.1.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be supported, and the optional HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [4] may be supported.

6.1.3	Resources

6.1.3.1	Overview

The figure 6.1.3.1-1 describes the resource URI structure of the Nnsacf-NumOfUEsPerSlice API.

Figure 6.1.3.1-1: Resource URI structure of the nnsacf-num-of-ues-per-slice API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Slice Collection Subject to NSAC (Collection) | /slices | POST | Request the NSACF to perform network slice admission control related to the number of UEs registered to a slice, for a group of slices. |

6.1.3.2	Resource: Slice Collection Subject to NSAC

6.1.3.2.1	Description

This resource represents the collection of slice subject to NSAC.

This resource is modelled with the Store resource archetype (see clause C.2 of 3GPP TS 29.501 [5]).

6.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/<apiName>/<apiVersion>/slices

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| apiVersion | string | See clause 6.1.1 |
| <name> | <type> | <definition> |

6.1.3.2.3	Resource Standard Methods

6.1.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UeACRequestData | M | 1 | Request data for NSAC procedure related to the number of UEs per slice. |

Table 6.1.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UeACResponseData | M | 1 | 200 OK | Response data for NSAC procedure related to the number of UEs per slice. |
|  |  |  | 307 Temporary Redirect | Temporary redirection, during a NSAC procedure. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the resource located on an alternative service instance within the NSACF that was selected by the AMF. |
|  |  |  | 308 Permanent Redirect | Permanent redirection, during a NSAC procedure. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the resource located on an alternative service instance within the NSACF that was selected by the AMF. |
| ProblemDetails | O | 0..1 | 404 Not Found | When used to represent the failure of NSAC procedure, the "cause" attribute of the "ProblemDetails" shall be set to one of the following application error codes: - SLICE_NOT_FOUND, if the given S-NSSAI is not found from the NSSAI which are subject to NSAC procedure; |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.1.3.2.3.1-4: Headers supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

Table 6.1.3.2.3.1-5: Headers supported by the 200 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

Table 6.1.3.2.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |

Table 6.1.3.2.3.1-7: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the NSACF that was selected by the AMF. |

Table 6.1.3.2.3.1-8: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the NSACF that was selected by the AMF. |

6.1.3.2.4	Resource Custom Operations

In this release of this specification, no custom operations associated to this resource is defined.

6.1.4	Custom Operations without associated resources

In this release of this specification, no custom operations without associated resources are defined.

6.1.5	Notifications

6.1.5.1	General

Notifications shall comply to clause 6.2 of 3GPP TS 29.500 [4] and clause 4.6.2.3 of 3GPP TS 29.501 [5].

Table 6.1.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| EAC Mode Notification | {EACNotificationUri} | POST | Notify the NF Service Customer (e.g. AMF) of the activation/deactivation of EAC mode. |

6.1.5.2	EAC Mode Notification

6.1.5.2.1	Description

The EAC Mode Notification is used by the NSACF to inform the NF Service Consumer (e.g. AMF) of the activation/deactivation of EAC mode.

6.1.5.2.2	Target URI

The Callback URI "{ EACNotificationUri}" shall be used with the callback URI variables defined in table 6.1.5.2.2-1.

Table 6.1.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| eacNotificationUri | Notification URI for receiving EAC mode notification. |

6.1.5.2.3	Standard Methods

6.1.5.2.3.1	POST

This method shall support the request data structures specified in table 6.1.5.2.3.1-1 and the response data structures and response codes specified in table 6.1.5.2.3.1-1.

Table 6.1.5.2.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EACNotification | M | 1 | EAC mode notification |

Table 6.1.5.2.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

* * * End of Changes * * * *