3GPP TSG-CT WG4 Meeting #104e	C4-213062

# E-Meeting, 19th May– 28th May 2021

Source:	China Telecom

Title:	Evaluation and Conclusion for KI#1

Spec:	3GPP TR 29.829 v0.5.0

Agenda item:	6.1.5

Document for:	Approval

1. Introduction

<Introduction part (optional)>

2. Reason for Change

Pseudo-CR on Evaluation and Conclusions for KI#1.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 29.829 v0.5.0.

* * * First Change * * * *

7.1	Evaluation and Conclusions of Solutions for Key Issue#1

7.1.1	Evaluation

For Key Issue #1 – "SBI-based SM MT message transfer", candidates Solution #1, Solution #13 and a solution group (including Solution#4, #6, #7, #8, #9, #10) have been proposed.

Candidate Solution #1 proposes to introduce new services for MT SMS Delivery, MT SMS Delivery Report, Routing Information, Alert SC and so on. These new services are registered in NRF and can be queried by NF service consumers. Procedures and node functionality enhancements for SBI-based MT SMS are also introduced in this solution.

Candidate Solution #13 introduces an alternative solution for MT SMS transfer in SMS Home routing scenario, where the SMS is first transferred to the SMSC/SMS-GMSC of the recipient's home network.

Candidate solution group (including Solution#4, #6, #7, #8, #9, #10) introduces SMS transfer in the cases of MT SM transfer, MT SM transfer via SMS Router/IP-SM-GW, unsuccessful MT SM transfer via SMS Router/IP-SM-GW, unsuccessful MT SM transfer, alert and alert the recovery of UE memory for SMS. In this solution group, new services are also defined.

In order to fulfil SBI-based SM MT, the three candidate solutions share the same method: new services for SM MT are defined and registered in NRF, and then service consumers can query NRF and invoke the services.

Editor's Note:	The three candidate solutions can be merged for conclusion.

7.1.2	Conclusion

The following conclusions are agreed:

-	New TS for SBI-based 5G SMS message transfer is needed, including:

-	Definitions of the services for SBI-based SM MT message transfer;

-	Procedures of MT SMS Delivery, MT SMS Delivery Report, Routing Information, Alert SC and so on;

-	Node functionality enhancements needed for SBI-based SM MT;

-	Interface enhancements for SBI-based SM MT;

-	Other enhancements for SBI-based SM MT.

* * * End of Changes * * * *